3GPP TSG RAN WG1 #105-e                                              R1-2106163

e-Meeting, May 10th – 27th, 2021

Source: 	Moderator (Nokia)

Title: 	Feature Lead Summary #3 for Potential multipath/NLOS mitigation

Agenda item:	8.5.5

Document for: 	Discussion and Decision

# Introduction

In the WID, [1], for ePos the following objective was added at RAN#91:

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

In this contribution, we provide summary of the potential enhancements for information reporting from UE and gNB for multipath/NLOS mitigation proposed by companies in contributions [2]-[21]. We also make some initial proposals to facilitate RAN1 discussion. This document provides the summary of the following email discussion in RAN1#105-e:

[105-e-NR-ePos-05] Email discussion/approval on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation with checkpoints for agreements on May 25, May 27 – Ryan (Nokia)

Overview of proposals in contributions

The following list of proposed enhancements/areas was identified based on submitted contributions [2]-[21]:

- Specification Change

- LoS/NLoS Indicator

- Additional Reporting from UE and TRP/gNB to LMF

- Soft/Hard Indicators

- LoS/NLoS Identification methods

- UL-AoA Related Topics

- DL-AoD Related Topics

- Specific PRS resources

- Additional Paths

- CIR reporting

- Measurement Time Window

- UE-based Proposals

- Others

Issues for discussion

## Issue #1: Specification Change

From the WID, [1], the objective has a study component to see if specification effort is needed on this topic. Based on the FL review of contributions almost all companies seem in favor of specification effort on this topic:

- Support for specification work/changes: Proposed by 17 companies

- Support for implementation-based solutions only (i.e., no specification change): Proposed by 2 companies ([4],[8])

- Proposals by companies supporting specification work/changes are captured in individual sub-sections later in this document. Proposals by companies supporting implementation-based solutions only are:

- [4]

  - Proposal 1: Implementation-based solution should be considered to solve NLOS problems.

- [8]

  - Proposal 1: For multipath/NLOS mitigation, only focus on the implementation-based solutions in Rel-17.

### Round #1 Discussion

Feature Lead View

Almost all companies seem to be supportive of some specification changes related to enhancements of information reporting from UE and gNB for multipath/NLOS mitigation. Two companies (vivo, OPPO) prefer implementation-based solutions but seem to agree that the NLOS/multipath is a problem that needs to be solved for positioning. Other companies point out in their contributions that some of the proposed enhancements can be combined with implementation-based solutions to further improve the performance. As such perhaps their concerns could be alleviated.

Proposal 1.1

- Enhancements of information reporting from UE and TRP/gNB for multipath/NLOS detection and mitigation will be supported.

- FFS: Specific enhancements including:

  - LoS/NLoS indicators

  - Additional reporting from UE and TRP/gNB to LMF

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | Support |
| vivo | Not support. First of all, we have concern on this proposal which seems too broad to cover any “enhancements of information reporting from UE and TRP/gNB for multipath/NLOS detection and mitigation”. On the performance benefits of LOS/NLOS detection, as we discussed and showed in our contribution, depends on the LOS/NLOS detection methods, there could be performance loss instead of gain compared to implementation based method. There’re also several other companies provide evaluation results on LOS/NLOS detection indicator in this meeting. Most of them actually show minor/marginal performance gain on top of UE implementation based method. We see little value for a standard solution on this matter. On the FFS bullet, we note LOS/NLOS indicator and additional reporting are discussed in proposal 2.2 and proposal 3.1 already. Furthermore, we’re not clear if any performance benefit verification of additional information reporting. We don’t think the sub-bullet of additional reporting is well justified. In summary, we object this proposal and don’t think such high level proposal is needed. |
| Qualcomm | We prefer not to agree on a so-broad blank statement; there needs to be a consensus what broad direction of enhancement we are going to specify. The LOS/NLOS vs the multipath reporting are very different features. |
| OPPO | Not support We share the same understanding as vivo and Qualcomm. We should not agree such a high level proposal. |
| CATT | Support. NR R17 should support reporting of LOS/NLOS identification information indicating whether a measurement is associated with LOS or NLOS, or the probability of the measurement being associated with LOS or NLOS. |
| ZTE | We think multipath mitigation and NLOS detection should be discussed separately. Since this is the first meeting to study multipath/NLOS mitigation, we’re OK to agree high level proposal first. At least we should decide whether to specify corresponding enhancements or not in this meeting. After we decide to enhance multipath/NLOS mitigation, we can further discuss what’s in scope for further study. From our point of view, we support to enhance multipath/NLOS mitigation. |
| China Telecom | We share the similar view as ZTE. Even in the LOS scenario, the multipath still exists. Multipath and LOS/NLOS are concepts of different dimension. We should support the detection of LOS/NLOS, and mitigation of the multipath. |
| Lenovo, Motorola Mobility | Support FL’s proposal. This proposal serves an initial basis for progress within the group to at least agree on the need for multipath/NLOS detection and mitigation enhancements in Rel-17 based on all previous discussions thus far including in other AIs on this aspect. |
| Samsung | Although this proposal seems no harm, but we can wait a bit further to see more details on the design. |
| Xiaomi | Support Information for multipath/NLOS detection and mitigation can improve the accuracy |
| LG | Support. |
| Intel | Support FL’s proposal. |
| InterDigital | We need to discuss details of solutions and their effectiveness (e.g., LOS/NLOS identification and reporting, multipath reporting, etc.) to agree on the first bullet. |
| Futurewei | We support in general but not sure having the various enhancements but not sure what we gain by agreeing on something this general. Perhaps it is better to agree on something more specifics. |
| SONY | Support. This is the first meeting, so naturally we make a high level agreement. |
| Nokia/NSB | Support. In our view the NLOS problem is critical to address or we will not meet the required accuracies. |

### Round #2 Discussion (Closed)

FL Summary of Round 1 Discussion

Companies supporting proposal 1.1 – 10

Companies okay in principle but prefer specifics – 2

Companies not supporting – 4

The majority of companies seem in favor of the proposal but some companies have concerns on how broad the statement is. From FL perspective the intention of the proposal was to agree that we will do some specification work for NLOS/multipath mitigation and then work on the details. Perhaps a small rewording and a note can be attempted to ease some concerns. If it is too difficult to make progress then FL suggest to move directly to the specifics. After the GTW it seems that companies prefer to discuss the solutions directly. I would still suggest to continue discussion on this proposal but we will only bring it for online discussion if views change.

Proposal 1.1.1

- Enhancements of information reporting from UE and TRP/gNB for multipath/NLOS detection and mitigation will be supported.

- FFS: Specific information including:

  - LoS/NLoS indicators

  - Additional multipath reporting from UE and TRP/gNB to LMF

- Note: This agreement is not meant to indicate that a specific solution or enhancement will be specified.

Companies views:

| Company Name | Comments |
| --- | --- |
| ZTE | We’re confused with this proposal since we already agreed to further study those methods. However, this proposal is saying we should support them right now. Our suggestion is that we should identify some possible solutions that needs to further study in next meeting. |
| CATT | Support. |
| vivo | Don’t understand why repeat this proposal again. It was discussed and not agreeable to us. |
| Ericsson | Support the proposal, at least for the sake of commiting to enabling NLOS detection/indication by specification. Our understanding is that the proposal is meant to be secure the intention. |
| OPPO | Do not support this proposal This proposal was discussed in last GTW session and we agreed to study detailed methods. Why is the same proposal repeated here again? |
| China Telecom | Support. |
| Lenovo, Motorola Mobility | Support in principle. |
| SONY | Support, with the following modification: - Enhancements of information reporting from UE and TRP/gNB for multipath/NLOS detection and mitigation will be is supported. |
| FL | Thanks for some companies to continue exchanging views but seems the spirit of the group is to not go for this proposal again at this meeting. As such let us consider this topic closed. |

## Issue #2: LoS/NLoS Indicator

One issue discussed by many companies is the introduction of an indicator of a LoS/NLoS indicator. These indicators could be reported by the UE and/or TRP to the LMF when making positioning measurement reports. Specific proposals from other companies are:

- [2]

  - Proposal 1: To improve positioning accuracy by regularization techniques, use of LOS indicators as soft values for each link for UE-assisted and UE-based positioning should be supported.

  - Proposal 1a: For UE-assisted positioning, the determination of the LOS indicator can be done at the gNB/LMF or at the UE. For the latter, the LOS indicator is additionally reported back to the gNB/LMF.

  - Proposal 1c: For UE-based positioning, the LOS indicator can be signaled to the UE. The UE can request the LOS indicator for specific path(s) associated with a gNB or TRP to be signaled to the UE by the LMF/gNB.

- [3]

  - Proposal 5:  Support reporting the NLOS identification results along with the corresponding measurement results.

- [5]

  - Proposal 1: NR R17 should at least support reporting of LOS/NLOS identification information indicating whether a measurement is associated with LOS or NLOS, or the probability of the measurement being associated with LOS or NLOS.

- [9]

  - Proposal 2: Support the UE to report LOS/NLOS indicator together with the RSRP measurement of first arriving path.

  - Proposal 4: The UE may take advantage of the measurement of NLOS scenario to mitigate the multi-path in LOS scenario.

- [11]

  - Proposal 1:	For the UL-TDOA / UL-AOA / Multi-RTT positioning method support introduction of the LOS/NLOS identifier associated with the UL-RTOA time / UL-AOA angle / gNB Rx-Tx time difference measurements

  - Proposal 2:	For the DL-AOD / Multi-RTT positioning method support introduction of the LOS/NLOS identifier associated with the RSRP / UE Rx-Tx time difference measurements

  - Proposal 3:	For the DL-TDOA positioning method support introduction of the LOS/NLOS identifier associated with the DL RSTD time measurement using the following format:

    - (LOS/NLOS identifier #1, LOS/NLOS identifier #2) – LOS/NLOS identifier #1 corresponds to the link associated with a reference cell and LOS/NLOS identifier #2 corresponds to the link associated with a neighbor cell

  - Proposal 4:	Support introduction of the LOS/NLOS identifier in the format:

    - Alt 1: LOS/NLOS identifier may be equal to 0 or 1, where 0 indicates the LOS channel and 1 indicates the NLOS channel

    - Alt 2: LOS/NLOS identifier may be equal to variable u distributed in the range from 0 to 1 and has a meaning of probability for NLOS link detection

    - The u = 0 corresponds to the case of a pure LOS channel with a single channel tap in time domain and zero NLOS components

    - Conversely, the u = 1 corresponds to the case of a pure NLOS channel with a multi-path channel structure and zero LOS component

- [13]

  - Proposal 1: Support UE positioning measurement report with LOS/NLOS identification.

- [14]

  - Proposal 1: Support UE/TRP sending to the LMF an NLOS/LOS indication associated with the measurements for positioning if the LOS/NLOS could be reliably differentiated.

- [16]

  - Proposal 2: RAN1 to study NLOS identification reporting from the UE/TRP to the LMF during at least UE-A positioning.

- [19]

  - Proposal 2: Support UE reporting of RSTD, UE Rx-Tx time difference and/or PRS RSRP associated with LOS/NLOS indicators. FFS further details such as how these indicators are mapped, e.g. per beam, etc and granularity of the indicators.

### Round #1 Discussion

Feature Lead View

Many companies (at least 9) seem interested in introducing LoS/NLoS indicators calculated by the measuring nodes (UE/TRP) and reporting to LMF as part of location reports. Some companies propose additional details related with these indicators and those are discussed in subsequent sections. With that in mind the follow proposal may be a good starting ground:

Proposal 2.2

- LoS/NLoS indicators should be reported for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

  - FFS: Details of indicators

  - FFS: UE based positioning

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | Can we clarify the LOS/NLOS indicator, it is not a 0/1 Flag since that is captured in P4.1. LoS/NLoS indicators: The LOS/NLOS identification can provide information (for example probability) that First Arriving Path (FAP) is in LOS or a NLOS condition. The LoS/NLoS indicator can include an indication on the quality of the FAP. The motivation is that for LOS scenarios, the FAP may be come from to the LOS in addition to other near reflection (referred to OLOS in [20]). A UE or TRP can identify the quality of the FAP which is useful for the LMF to identify TOA offsets (LOS with near reflections). |
| vivo | As we comment toward proposal 1.1, we’re not in favour of standard solution for LOS/NLOS mitigation to begin with. In order to obtain the performance benefit of reporting LOS/NLOS indicator, at least the associated LOS/NLOS identification method(s) and error rate should be studied before we agree to support LOS/NLOS indicator reporting. |
| Qualcomm | We are not supportive of this feature. Using multipath reporting would enable the LMF to do any method it wants, whereas having LOS/NLOS just restricts the applicability to a feature of low understanding on what is happening “under the hood”. To be more specific, there will not be a consensus on which/what LOS/NLOS algorithm is being used (and should not be such algorithm details in the spec), nor will there be any tests/minimum requirements, so the practicality of a feature where a device reports a 0/1 or a [0,1] number is debatable (it could even be coming from a random generator from what one could try to verify). Reporting multipath characteristics is a feature that includes “low-level” channel characteristics, it has already been supported up to an extend in LTE and rel-16, and can be better understood/verified how it works and what is supposed to be doing. |
| OPPO | Not support From our evaluation, it is observed that we shall focus on implementation-based method for NLOS mitigation. We have discussed a lot during study item and specification supported method does not have sufficient justification. |
| CATT | Support. LOS/NLOS indicator can be defined as a function of the Rice factor in the time domain, the variance of CFR in the frequency domain, or the combination of the above two parameters. |
| Huawei/HiSilicon | Support. We listed the following benefits in our contribution that cannot be achieved by implementation-based methods. - Improving the calibration performance based on the reference device. - Improving positioning accuracy for limited LOS measurement scenario. - Reducing the complexity at the LMF - Improving the uncertainty estimate accuracy Regarding the testability, we think this can be further studied whether we need such requirement for UE or TRP. In our view, LOS/NLOS identification can even be working with implementation-based methods to reduce the LMF computation complexity. |
| ZTE | This can be further details when high level proposal is agreed. |
| China Telecom | Support. The indicator of LOS/NLOS identification doesn’t affect the details for mitigation of multipath, conversely, it is kind of the fundamental of multipath mitigation. How to identify whether the scenario is LOS/NLOS needs further study, but the indicator itself should be support. |
| Lenovo, Motorola Mobility | Support, We view that the UE can use a variety of implementation and non-implementation based approaches to derive LOS/NLOS indicators. The LOS/NLOS indication increases the efficacy of the reported measurement and can be used by the LMF as assistance information. |
| Samsung | As we discussed in the tdoc, if the LOS/NLOS can accurately achieved, such indication could be very helpful. However, it seems the details on how it achieved can be FFS. |
| NTT DOCOMO | Support We think LOS/NLOS indicator is helpful regardless of LOS/NLOS detection method. |
| Intel | Support. As it was shown by simulation analysis in R1-2104909 introduction of LOS/NLOS indication allows to improve the performance of the implementation-based methods significantly and achieve the required performance accuracy for the I-IoT use case. At the same time, it does not introduce additional overhead associated with the multi-path information reporting. Note, that limiting the number of channel taps in the multi-path report compromises the ultimate performance. The required specification change is small, just introduction of the LOS/NLOS indicator associated with the measurement. The particular implementation of the LOS/NLOS classification algorithm can be left up to implementation. |
| InterDigital | Support |
| Futurewei | Support |
| SONY | Support |
| Nokia/NSB | Support |

### Round #2 Discussion (Closed)

FL Summary of Round 1

Companies supporting – 11

Companies opposed – 3

Majority of companies seem to be supportive while some companies have concerns on the benefits. Some benefits have been listed by the supporting companies. In addition some companies commented in Issue #4 that it should be merged here. As this is the first meeting of this AI we suggest the following updated proposal.

Proposal 2.2.1

- LoS/NLoS indicators should be reported for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options of LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: UE based positioning support of LoS/NLoS indicators

- Outcome of GTW:

Agreement:

- Study reporting of LoS/NLoS indicators for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options (or combinations of the following options) for LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: LoS/NLoS indicators for UE-based positioning

- Companies are encouraged to continue discussion on proposal 2.2.1 to see if further progress can be reached on the study.

Companies views:

| Company Name | Comments |
| --- | --- |
| ZTE | We have no other views. We can keep the agreement as it is. Then, interested companies can bring their evaluation results and corresponding benefits in next meeting. |
| Fraunhofer | The indication is normally derived from one or multiple metrics from the measurements. These metrics represent an expected behaviour which can is more probable in one state than the other but will there will always overlapping between the two LOS and NLOS states. This means that TRP/UE cannot normally determine with 100% if this a LOS or a NLOS, however it can say that a certain link is XX% LOS and YY%NLOS [in our understanding YY and XX are independent]; for example, the UE/TRP can report: - Channel state indication : LOS, NLOS - Confidence: Soft value And - Secondary channel state indication: LOS, NLOS - Secondary channel confidence: Soft value |
| CATT | We are fine with current agreement. |
| vivo | Current agreement is a good step for our study and discussion in next meeting. Don’t see we need any more progress. |
| Ericsson | Our preference for UE assisted is to enable NLOS detection with enhanced reporting. The LMF has access to additional information beside what the UE can compute, and this should be leveraged by placing the NLOS decision at the LMF. We are not against supporting indicators, but we see it as a secondary solution. |
| InterDigital | We are also ok with the current agreement. |
| OPPO | In our view, the current agreement made in last GTW it good enough for now. Detailed method need further study and discussion. |
| China Telecom | Support. |
| Xiaomi | We are fine with the current agreement and more discussion in next meeting. |
| NTT DOCOMO | We are fine with the current agreement. The details can be discussed further in the next meeting. |
| LG | We are fine with current agreement. |
| SONY | We are OK with the current agreement and further discussion in the next meeting. |
| FL | Thanks for some companies to continue exchanging views but seems the spirit of the group is to not go for this proposal again at this meeting. As such let us consider this topic closed. |

## Issue #3: Additional reporting from UE and TRP/gNB to LMF

Many companies also propose enhancing the reporting from UE/TRP during positioning sessions in order to enable the LMF to determine the LoS/NLoS status. Some companies seem to view this as an alternative to UE/TRP reporting LoS/NLoS indicators while other companies support both enhancements. Some specific proposals are included in other sub-sections, in particular those aimed at specific positioning methods, while other more general ones are captured here:

- [2]

  - Proposal 1b: For UE-assisted positioning, at least support the baseline case where needed measurements are reported back to the gNB/LMF for the determination of the LOS indicator at the gNB/LMF. The algorithm to compute the LOS indicator hence becomes a matter of network implementation.

- [7]

  - Proposal 4: In the multipath reporting framework, the UE/gNB can also include an indication of which additional path is the strongest path measured per PRS/SRS resource.

- [16]

  - Proposal 3: RAN1 to study NLOS enhanced reporting from the UE/TRP to the LMF to enable the LMF to calculate the probability of NLOS.

- [17]

  - Proposal 2: To report some information for LoS/NLoS identification from UE.

  - Proposal 3: To discuss which information will be reported from UE for LoS/NLoS identification.

- [20]

  - Proposal 2	Define list of metrics/signal quality data for LOS/NLOS detection by LMF

- [21]

  - Proposal 5	NLOS/LOS detection should be done by the LMF by using CIRs from the UE and gNBs.

  - Proposal 7	Following measurements should be specified in Rel-17 to support LOS identification methods. These measurements can be part of rich reporting.

    - a.	Location and magnitude of the first and additional paths.

    - b.	Location and magnitude of the highest peak.

    - c.	Components of PDP/CIR around first/highest peak.

### Round #1 Discussion

Feature Lead View

Overall, many companies seem to propose additional reporting elements as part of UE/TRP reports to LMF which will enable the LMF to determine LoS/NLoS status. The specific elements may need more time to converge and the details could potentially be discussed as part of some subsequent proposals in this document.

- Proposal 3.1

- Reporting enhancements for DL, UL, and DL+UL positioning should be supported to enable LoS/NLoS identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

Companies views:

| Company Name | Comments |
| --- | --- |
| vivo | Not support. First of all, is there any performance evaluation to justify/demonstrate the benefit(s) of additional information reporting on top of LOS/NLOS indicator? We don’t support this proposal without solid justification. |
| Qualcomm | Seems to general to me again. I thought the debate is about LOS/NLOS vs multipath reporting. This proposal does not provide a progress in either direction. |
| OPPO | Similar view as Qualcomm, this proposal is too general. |
| CATT | Support. |
| ZTE | This can be further details when high level proposal is agreed. |
| China Telecom | Similar view as ZTE. |
| Lenovo, Motorola Mobility | Support the intention of FL’s proposal, but it may still be too general at this stage for an agreement. |
| Samsung | Similar view as ZTE. |
| Xiaomi | We are not clear what enhancements will be. |
| LG | We are on the same page with Qualcomm. |
| Intel | These are the second order details, but if LOS/NLOS identification is agreed, we are open to discuss it. |
| InterDigital | Support |
| Futurewei | Needs to have more specifics on the FFS |
| Sony | Support in principle. It would be better if we can make a list of options. |
| Nokia/NSB | Support. To vivo, many companies have brought simulation results showing clear gains for the introduction of additional reporting. If we do nothing then NR positioning will not meet the requirements for IIoT. That is crystal clear. |

### Round #2 Discussion (Closed)

FL Summary of Round 1

The intention of the agreement was on multipath reporting to enable the LMF for LoS/NLoS identification and multipath mitigation. Perhaps discussing the number of paths and the measurements would be better but trying for a high-level view might be a good start. As such an updated proposal is

- Proposal 3.1.1

- Multipath reporting enhancements will be supported for DL, UL, and DL+UL positioning to enable LoS/NLoS identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

Outcome of GTW:

- Agreement:

- Study multipath reporting enhancements for DL, UL, and DL+UL positioning to enable LoS/NLoS/multipath identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

Companies are encouraged to continue discussion on proposal 3.1.1 to see if further progress can be reached on the study.

Companies views:

| Company Name | Comments |
| --- | --- |
| ZTE | The “FFS: Details of the enhancements” can be extended to include some concrete solutions (e.g. path power/delay/angle that is planned to be discussed in other issues) |
| CATT | We are fine with the current agreement. |
| vivo | It’s clear that we cannot agree proposal 3.1.1. Current agreement is a good step for our study and discussion in next meeting. Don’t see we need any more progress. |
| OPPO | Same view as CATT and vivo, the current agreement made in last GTW is good enough for current moment. |
| SONY | We are fine with the current progress and discuss the details in the next meeting. |
| FL | Thanks for some companies to continue exchanging views but seems the spirit of the group is to not go for this proposal again at this meeting. As such let us consider this topic closed. |

## Issue #4: Soft/Hard Indicators

- [14]

  - Proposal 2: The NLOS/LOS indicator can be either a hard decision (binary values) or a soft decision (real values such as the probability of the path being NLOS/LOS).

- [15]

  - Proposal 2: In order to increase the positioning accuracy, followings can be studied.

    - UE does not necessarily to report positioning measurement (e.g., RSTD(s), UE Rx-Tx time difference, and etc.) corresponding to certain TRP determined with NLOS, or

    - LOS-likelyhood value can be reported in conjunction with positioning measurement.

### Round #1 Discussion (Closed)

Feature Lead View

Some companies seem to implicitly discuss this topic in their contributions without specific proposals. It would be good for companies to exchange further views in order to move towards some consensus and better understand companies’ understanding of this topic.

Proposal 4.1

- Study the following options of LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | Do not support |
| vivo | Seems to us, this proposal should be part of proposal 2.2 and no need to have a separate proposal. |
| Qualcomm | Need a resolution on whether LOS/NLOS will be supported first. |
| OPPO | Need to discuss if we support LOS/NLOS reporting first |
| CATT | Support. To FL, CATT had also provided the discussions and observation about the hard and soft decision methods in our contribution [5], we prefer to include CATT’s scheme into the background descriptions of section 3.4, as follows, Observation 2: Based on the reported LOS/NLOS identification information corresponding to each measurement, the positioning engine (in LMF or UE) can reduce the influence of NLOS and multipath and achieve higher accuracy and reliability, by a soft decision method such as weighted measurement information, or a hard decision method where only the measurements with LOS/NLOS identification information higher than a certain threshold are used for positioning. |
| Huawei, HiSilicon | Agree to either to agree 3.2 first or merge the proposal in 3.2 |
| ZTE | This can be further details when high level proposal is agreed. |
| Lenovo, Motorola Mobility | Agree with Huawei that we could merge this proposal as FFS example points in Issue#2 of 3.2: - LoS/NloS indicators should be reported for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning. - FFS: Details of indicators (e.g. binary, soft-value) - FFS: UE based positioning |
| Xiaomi | Discuss it after proposal 2.2 |
| NTT DOCOMO | This proposal should be merged with section 3.2. In addition, if LOS/NLOS indictor is supported, the detail design (e.g. soft/hard indicators) should be discussed. |
| LG | We agree with the intention of the proposal. But, we also similar view with other companies. It seems proper to merge the proposal into 3.2. |
| Intel | Option 2. Agree to consider the proposal 3.2 first and then decide based on the outcome of discussion. |
| InterDigital | We need to discuss if we support LOS/NLOS reporting first. |
| Sony | Same view as Huawei. It is strongly related with proposal 3.2 |
| Nokia/NSB | Support as this is further study but okay to move into proposal 3.2 as suggested by other companies. |

FL Summary of Round 1

Seems that most companies prefer to discuss with proposal 2.2. It has been merged above. As such we can close this sub-section.

## Issue #5: LoS/NloS identification methods

Many different methods have been proposed for determining LoS status. The related proposals are discussed here. This section focuses on algorithms that would be run at UE/TRP to determine a LoS/NloS indicator. The proposals related to this topic are:

- [2]

  - Proposal 2: At least the gNB beam/antenna bores-sight information can be provided to the LMF by the gNB and to the UE for UE-based positioning.

- [4]

  - Proposal 3: Before discussing detailed enhancements of information reporting for supporting multipath/NLOS mitigation, the method of LOS identification and the LOS identification error should be studied.

  - Proposal 4: The intention and benefit of information to be reported for multipath/NLOS mitigation should be further clarified since the correct rate of LOS detection is different with different scenarios and different LOS identification methods.

  - Proposal 5: Whether different LOS identification method should be applied to different positioning method should be confirmed.

- [5]

  - Observation 1: LOS/NLOS identification information, which is defined as a function of the Rice factor in the time domain, the variance of CFR in the frequency domain, or the combination of the above two parameters, can help the positioning engine (in LMF or UE) to select LOS links between TRP and UE to obtain a more precise position by mitigating the influence of NLOS and multipath.

  - Observation 2: Based on the reported LOS/NLOS identification information corresponding to each measurement, the positioning engine (in LMF or UE) can reduce the influence of NLOS and multipath and achieve higher accuracy and reliability, by a soft decision method such as weighted measurement information, or a hard decision method where only the measurements with LOS/NLOS identification information higher than a certain threshold are used for positioning.

- [6]

  - Proposal 1：Support UE and TRP to report coherence bandwidth of measured reference signal to LMF for NLOS mitigation.

- [13]

  - Proposal 2: The method/techniques of obtaining LOS/NLOS identification is left for UE implementation.

  - Proposal 3: Support gNB to provide gNB/TRP antenna polarization to LMF and subsequently, LMF to provide gNB/TRP antenna polarization information to UE.

- [15]

  - Proposal 1: For LOS/NLOS identification, RAN1 needs to consider at least following method based on:

    - Polarization characteristic

    - Propagation time difference threshold/window between a reference and a target TRP.

- [16]

  - Proposal 5: RAN1 to study both LOS/NLOS identification methods computed in PHY layer processing and LMF localization processing.

- [19]

  - Proposal 1: Scope of Multipath/NLOS reporting should initially cover a general and a low complexity solution across DL-based, UL-based and (DL+UL) positioning methods to increase the quality of positioning measurements provided to the LMF.

- [21]

  - Proposal 1: Support NLOS/LOS detection mechanisms in release 17

  - Proposal 6: UEs and gNBs should indicate the method they are using to detect NLOS/LOS nature of the links to the LMF.

### Round #1 Discussion

Feature Lead View

Some of the proposal on this topic seem related also with the discussion in AI 8.5.3 on DL-AoD enhancements so alignment with that AI may be needed. There are quite a few proposals to study/support different LoS/NloS identification methods.

Proposal 5.1

- RAN1 to study the following options of LoS/NloS detection

  - Option 1: Polarization based detection.

  - Option 2: Coherence bandwidth based detection.

  - Option 3: Implementation based solutions

  - Option 4: Propagation time difference based

Companies views

| Company Name | Comments |
| --- | --- |
| vivo | We support to study LOS/NLOS detection methods in general as they have impacts on the detection error rate and hence positioning performance. |
| Qualcomm | Not support. The WID just talks about what reporting enhancements to do. There is no need to discuss methods of LOS/NLOS detection. |
| OPPO | Not support. The proposal is out of the scope of WID: - Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3] |
| CATT | Support to study LOS/NLOS detection methods, and we prefer Option 5 described in our contribution [5]. And the updated Proposal 5.1 as follows, Proposal 5.1 - RAN1 to study the following options of LoS/NloS detection - Option 1: Polarization based detection. - Option 2: Coherence bandwidth based detection. - Option 3: Implementation based solutions - Option 4: Propagation time difference based - Option 5: Rice factor and variance of CFR based solution. To FL: CATT had also provided the discussions and observation about the LOS/NLOS detection methods in our contribution [5], we prefer to include CATT’s scheme into the background descriptions of section 3.4, as follows, Observation 1: LOS/NLOS identification information, which is defined as a function of the Rice factor in the time domain, the variance of CFR in the frequency domain, or the combination of the above two parameters, can help the positioning engine (in LMF or UE) to select LOS links between TRP and UE to obtain a more precise position by mitigating the influence of NLOS and multipath. |
| ZTE | Support this proposal after we have decided to enhance NLOS detection. |
| China Telecom | Support. |
| Lenovo, Motorola Mobility | It is perhaps a bit early to agree to a general LOS/NLOS detection solution, we can rather focus on an agreeable reporting solution at this stage. |
| Samsung | Is the intention to find a method to be supported by specification? |
| LG | Support. |
| Intel | We believe that the particular implementation of the LOS/NLOS identification algorithm can be left up to implementation. |
| InterDigital | In multipath channels, channels become frequency selective and the PRS RSRP may change across frequencies. Thus, averaging across frequencies may not provide sufficient information for the network to determine characteristics of the multipath channel. Thus, we proposed to add another option in the proposal. We also prefer to leave the door open for other options to study. Proposal 5.1 - RAN1 to study the following options of LoS/NloS detection - Option 1: Polarization based detection. - Option 2: Coherence bandwidth based detection. - Option 3: Implementation based solutions - Option 4: Propagation time difference based - Option 5: Rice factor and variance of CFR based solution. - Option 6 : RSRP reporting with finer granularity - Other options are not precluded |
| Futurewei | It is more appropriate to discuss the spec changes e.g., measurements, reporting etc at this stage instead of the actual implementation techniques. |
| Sony | Support the proposal |
| Nokia/NSB | Support as this is “study” which is the WID objective after all. |

### Round #2 Discussion

FL Summary of Round 1

Some companies want to study and say we must do that first while other companies prefer not to study. As such it may be challenging to make progress. An update is suggested below.

Proposal 5.1.1

- RAN1 to study the following options of LoS/NloS detection

  - Option 1: Polarization based detection.

  - Option 2: Coherence bandwidth based detection.

  - Option 3: Implementation based solutions

  - Option 4: Propagation time difference based

  - Option 5: Rice factor and variance of CFR based solution.

  - Option 6: RSRP reporting with finer granularity

  - Other options are not precluded

- Note: Not all options necessarily imply specification impact

Companies views:

| Company Name | Comments |
| --- | --- |
| ZTE | OK with the proposal. |
| Fraunhofer | We don’t think listing all options helpful. By default identification is implementation based and when a spec impact is needed then this should be discussed case by case. |
| CATT | Support Proposal 5.1.1. |
| Huawei, HiSilicon | Not clear why those options are listed. Is the intention to evaluate the performance or are we discussing any spec impact? |
| FL | To Huawei, the intention is to list options which can be further evaluated/studied. From FL point of view this addition agreement would help to further focus the work given our agreement during the GTW to study LoS/NloS indicators. Our understanding is that some of the above techniques may have spec impact while others may be fully based on implementation. |
| Apple | Do Not support (out of scope) |
| vivo | We’re okay to study options and identify whether standard solution for LOS/NLOS detection is need or not. |
| Ericsson | Do not support. We can discuss methods on a high level in order to better understand what needs to be enhanced in the reporting, but the goal should not be to impact the spec with a specific algorithm. |
| InterDigital | Support |
| China Telecom | Support. |
| Xiaomi | We are ok to study these options. |
| Lenovo, Motorola Mobility | Generally ok, but it would be useful to indicate which of the options have measurement reporting spec impact aside from Option 3. |
| LG | Agree. |
| SONY | We support. I think the general idea is to let the proponent of each option to provide the details, showing the performance results (if any) and possible spec impact. |
| FL | To companies opposed to studying. Even if some options are based on implementation at the end of the day it seems reasonable given the WID study objective to study these and falls well within scope from FL perspective. Lenovo’s suggestion seems reasonable so an updated proposal is suggested here: - RAN1 to study the following options of LoS/NLoS detection - Option 1: Polarization based detection. - Option 2: Coherence bandwidth based detection. - Option 3: Implementation based solutions - Option 4: Propagation time difference based - Option 5: Rice factor and variance of CFR based solution. - Option 6: RSRP reporting with finer granularity - Other options are not precluded - FFS: Specification impact of Option 1, 2, 4, and 6 which may require additional assistance information signaled from LMF to UE/TRP. - Note: Not all options necessarily imply specification impact |
| Intel | We are OK to list the options, but we think that the identification method itself can be left up to implementation. |
| Qualcomm | Our understanding was that enhancement in assistance data was not in scope. It was only about enhancement of reporting. |
| CEWiT | The NLOS detection is implementation based, but we are okay if the objective is to study the impact of these options on the accuracy. |

### Round #3 Discussion

FL View

Many companies brought proposals for enhanced signaling to enable different LoS/NLoS schemes. While others feel that it can be fully up to implementation. Recommend the updated proposal to continue discussion. The intention of the proposal is to gain common understanding among companies. I have removed all options that are spec transparent and added option 5 to cover those options.

Proposal 5.1.2

- To enable LoS/NLoS indicator reporting, study at least the following options

  - Option 1: Polarization signaling from gNB/LMF to UE.

  - Option 2: Coherence bandwidth signaling from gNB/LMF to UE.

  - Option 3: Propagation time difference signaling from LMF to UE.

  - Option 4: RSRP reporting with finer granularity

  - Option 5: No specification impact outside of LoS/NLoS reporting

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | We are fine with the current formulation. Although the following for Options 2,3,4 is unclear: - Option 3: can’t this option already be achieved with expectedRSTD? - Option 2: okay to discuss however we understand the proposal suggests to report the coherence bandwidth. Is any signaling from LMF to the UE really needed? - Option 4: It’s not clear how RSRP reporting is related with the LOS/NLOS indicator, isn’t this part of issue-9 discussions? |
| Huawei, HiSilicon | Is it the correct signalling flow? - Option 1: LMF  UE would mean that the PRS configuration has polarization information? Not sure there should be signaling between gNB to UE on the positioning procedure. - Option 2: gNB/LMF  UE on the coherence bandwidth, which seem quite unclear. Is it using UL/DL reciprocity? - Option 3: LMF  UE on time difference signaling, in which we have the same question as FhG. Generally if the time difference signaling can be used to determine LOS/NLOS, the prior information of UE location should already be highly accurate. - Option 4: Is it from UE/gNB to the LMF? |
| ZTE | About the signaling flow: For Option 1-4, we think it should be that UE/gNB reports the additional measurements to LMF, so LMF can get additional information to decide whether the measurements from UE/gNB are based on LOS link. In short, it should be the enhancements of reporting rather than assistance data as clearly mentioned in the WID. |
| vivo | We have a concern about the description of the main bullet, we would like to understand is there any implication for “To enable LoS/NLoS indicator reporting”? In our view, LoS/NLoS indicator is not yet supported. But the main bullet may make people misunderstand that reporting the LoS/NLoS indicator already be supported and then the detailed option needs to be studied. So we prefer the previous description. |
| CATT | We share the same view with vivo that maybe it is better to use previous descriptions on the main bullet, and we add a new Option 5 (signaling of Rice factor and variance of CFR) in the proposal. For original Option 5 (No specification impact), maybe we don’t need it since it has no impact on the specs. We prefer the updated proposal as follows, Proposed Proposal 5.1.2 - Study the following options for signaling related to LoS/NLoS detection To enable LoS/NLoS indicator reporting, study at least the following options - Option 1: Polarization signaling from gNB/LMF to UE. - Option 2: Coherence bandwidth signaling from gNB/LMF to UE. - Option 3: Propagation time difference signaling from LMF to UE. - Option 4: RSRP reporting with finer granularity - Option 5: Signalling for information of Rice factor and the variance of Channel Frequency Response (CFR) from UE to LMF - Option 5: No specification impact outside of LoS/NLoS reporting |
| FL | To vivo, thanks for the comment and sorry for the confusion. The intention was not to say it was supported but to say we will study these options for LoS/NLoS detection which may need further discussion if LoS/NLoS indicators are supported. To CATT, from [5] I didn’t see you were proposing to add signaling and to just rely on implementation algorithsm using Rice factor and variance of CFR. If that was the wrong understanding then your update is okay from FL side. All, updated proposal here: - As part of studying LoS/NLoS indicator reporting, study at least the following options for signaling to enable LoS/NLoS dection: - Option 1: Polarization signaling from LMF to UE/TRP. - Option 2: Coherence bandwidth signaling from LMF to UE/TRP. - Option 3: Propagation time difference signaling from LMF to UE/TRP. - Option 4: RSRP reporting with finer granularity - Option 5: Rice factor and the variance of Channel Frequency Response (CFR) signaling from UE to LMF - Option 6: No specification impact outside of LoS/NLoS reporting |
| ZTE | To FL: As clearly stated in WID, - Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3] It should be the enhancements of reporting rather from UE/gNB rather than assistance data from LMF. Based on your latest proposal, we provide the following changes, - As part of studying LoS/NLoS information reporting, study at least the following options for information to enable/assist LoS/NLoS dection: - Option 1: Polarization information reporting from UE/gNB to LMF. - Option 2: Coherence bandwidth information reporting from UE/gNB to LMF. - Option 3: Propagation time difference information reporting from UE/gNB to LMF. - Option 4: RSRP reporting from UE/gNB to LMF with finer granularity - Option 5: Rice factor and the variance of Channel Frequency Response (CFR) information reporting from UE/gNB to LMF - Option 6: No specification impact outside of LoS/NLoS reporting |
| InterDigital | The modified proposal suggested by ZTE is ok with us. To HW: In our contribution, the intention is to enhance RSRP reporting from the UE to the LMF in hte presence of mulltpath channels. We are also open to gNB reporting RSRP with finer granurality to the LMF. To Fraunhofer : Issue 9 mainly discusses about value of N. Here the proposal is to provide more accurate view of RSRP. |
| CATT-2 | To FL: We are fine with the updated proposal from FL above. Regarding to the signaling aspect of Option 5 (Rice factor and CFR), in fact, we had mentioned it in our contribution [5] as follows ( i.e., UE reports the information to LMF). “Step 5: The UE generates the LOS/NLOS identification information by use of either soft decision or hard decision, and reports the information to the positioning engine in LMF or UE.” Therefore, we prefer to include Option 5 into the updated proposal, and the updated proposal from FL above is OK for us. |
| Lenovo, Motorola Mobility | Ok with FL’s updated proposal. |
| SONY | Support FL’s updated proposal. |
| Intel | OK to study, but tend to Option 6 in the latest FL’s summary (No specification impact outside of LoS/NLoS reporting). |
| Qualcomm | We should always study in relation/comparison to multipath reporting. Option 2/3/4/5 can all be determined from a rich enough multipath reporting (UE/gNB reporting enough additional paths, their relative TOAs and their relative RSRPs). We cannot argue that these are “new/different” information reporting, so we would like to ask the companies to compare/identify the incremental differences/gains over the multipath information reporting . - As part of studying LoS/NLoS information reporting, study at least the following options for information to enable/assist LoS/NLoS detection: - Option 1: Polarization information reporting from UE/gNB to LMF. - Option 2: Coherence bandwidth information reporting from UE/gNB to LMF. - Option 3: Propagation time difference information reporting from UE/gNB to LMF. - Option 4: RSRP reporting from UE/gNB to LMF with finer granularity - Option 5: Rice factor and the variance of Channel Frequency Response (CFR) information reporting from UE/gNB to LMF - Note: Companies are encouraged to identify differences in information reporting and any incremental performance gains from the multipath information reporting |
| FL | Thanks all for the constructive comments. The proposed updates from ZTE and QC look reasonable to me. Therefore the latest update (with small changes) is Proposal 5.1.3 - As part of studying LoS/NLoS information reporting, study at least the following options for information to enable/assist LoS/NLoS detection: - Option 1: Polarization information reporting from UE/gNB to LMF. - Option 2: Coherence bandwidth information reporting from UE/gNB to LMF. - Option 3: Propagation time difference information reporting from UE/gNB to LMF. - Option 4: RSRP reporting from UE/gNB to LMF with finer granularity - Option 5: Rice factor and the variance of Channel Frequency Response (CFR) information reporting from UE/gNB to LMF - Option 6: No specification impact outside of LoS/NLoS reporting - Note: Companies are encouraged to identify differences in information reporting and any performance gains compared with multipath information reporting |
| LG | We think we have been talking about just further study in wide variety of ways, not discuss whether to support or not. We are okay with the FL’s updated proposal. |
| CEWiT | Support FL proposal. |

## Issue #6: UL-AoA Related Topics

Quite a few companies brought proposals specific to enhancing reporting of UL-AoA for at least LoS/NLoS detection. Some of the proposals extend to more positioning techniques. Some alignment may be needed with AI 8.5.2 but the common view in RAN1#104-b seemed to be that these topics were more related to LoS/NLoS discussion. The specific proposals were:

- [2]

  - Proposal 4: NR supports reporting to LMF of N ≥ 1 UL-AOA measurement values per additional path for the same timestamp.

- [3]

  - Proposal 1:  Support the same number of UL AoA measurements per additional path.

  - Proposal 2:  Support gNB to report the path-specific association among TOA, AoA (multiple), and strength with same timestamp. Liaise RAN3 on the support of the feature with the following information.

    - RAN1 sees the necessity to introduce the information in the measurement response/report in NRPPa indicating that TOA, multiple AoAs, and path strength are measured from the same path, where different paths can be measured via the same SRS resource and associated with the same time stamp.

    - RAN3 is encouraged to provide solution in NRPPa.

- [7]

  - Proposal 3: Support a gNB to report multiple tuples (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) within a single report, such that

    - The UL-RSRP corresponds to a relative RSRP associated to the reported path in the angle/delay domain.

    - The RTOA/gNB-Rx-Tx corresponds to the delay of the associated reported path in the angle/delay domain

    - The UL-AoA corresponds to the received angle (potentially 2-dimensional) of the associated reported path in the angle/delay domain

    - FFS: Max number of  (UL-AoA, UL-RSRP, RTOA/gNB Rx-Tx) tuples that can be sent in a single report

- [13]

  - Proposal 4: Support UL-AoA measurement report from gNB to LMF that contain the statistical property (e.g., standard deviation of AoA) of the measured AoA for multipath/NLOS mitigation.

- [16]

  - Proposal 7: Further clarify and enhance the prior agreemtn of multiple measurements of M > 1 UL-AOA (AoA/ZoA) measurement values associated with the first arrival path and corresponding to the same timestamp by stating that:

    - M is the number of UL-AoA (AoA/ZoA) measuremnets that a UL receiver can measure in the same time stamp.

    - A UL receiver measures UL-AoA (AoA/ZoA) on a first arrival path at a measurement timing.

    - Corresponding to one UL-AoA measurement, a UL receiver may be requested to report additional information such as ToA of the measured path or beamforming to LMF.

    - Multiple measurements at a same time stamp are requested up to gNB measurement capability.

### Round #1 Discussion

Feature Lead View

It seems like many companies are interested in further enhancing at least the UL-AoA reports to help enable LoS/NLoS detection. It is not fully clear if there is common understanding how this should be done but the below proposal can be a starting point for discussion:

- Proposal 6.1

- For UL-AoA reporting from TRP to LMF support reporting angle, timing, and power for the first arrival path and additional N paths.

  - FFS: Value of N.

Companies views:

| Company Name | Comments |
| --- | --- |
| vivo | We’re open to discuss reporting enhancement for UL-AoA. However, we prefer not to duplicate discussion which happen in 8.5.2 as well especially the enhancement in this proposal is for the first arrival path. |
| Qualcomm | Support. To vivo: This proposal is for the additional paths. |
| OPPO | Shall this be discussed under sub-agenda 8.5.2? The additional path was also discussed in 8.5.2 too. |
| CATT | We share the same view with vivo and prefer this topic had better to be forwarded to 8.5.2. |
| Huawei, HiSilicon | Support to conclude that anything related to additional path should be discussed in 8.5.5 or discussed in each respective agenda (AoA, AoD). In our view, we think it should be better treated here, and the general increase of multi-path measurements including TOA, AoA, path strength can be resolved altogether. |
| Lenovo, Motorola Mobility | Prefer that these enhancements be discussed under a general measurement framework in AI 8.5.5. |
| LG | We are generally fine with FL’s proposal. But, to avoid duplicated discussion, we hope that the issue is discussed in either 8.5.2 or 8.5.5. |
| InterDigital | Share the view with Vivo and CATT that this feature can be discussed in 8.5.2. |
| Futurewei | No objections if this topic is moved to 8.5.2 |
| SONY | We think the statistical information of measurements that is obtained at TRP can be beneficial for LMF to mitigate multipath/NLOS. |
| Nokia/NSB | Support. In our understanding this topic was postponed from RAN1#104-b due to comments that it was related to multipath/NLOS so it should be discussed in this AI. |

### Round #2 Discussion (Closed)

FL Summary of Round 1

During RAN1#104-b it was commented by many companies that this issue should be discussed during RAN1#105 due to the relevance to multipath/NLOS. From FL perspective either AI is okay, but we should not waste time picking which AI to discuss a topic. Just conclude on one and then discuss technical aspects. This also applied to DL-AoD related proposals. We suggest a conclusion and then to continue discussion on Proposal 6.1.

- Proposal 6.1.1

- For UL-AoA reporting from TRP to LMF support reporting angle, timing, and power for the first arrival path and additional N paths.

  - FFS: Value of N.

Companies views:

| Company Name | Comments |
| --- | --- |
| ZTE | This should be discussed together with Proposal 3.1.1 as a possible solution for multipath reporting enhancements. Suggest to add “study” in main bullet as well. |
| CATT | We share the same view with ZTE that adding “Study” in the main bullet. |
| Huawei, HiSilicon | We do not think that the first arrival path should be discussed here, and timing of the first arrival path should not be associated with the UL-AoA method. |
| FL | To ZTE, Proposal 3.1.1 was already agreed with “study”. Not sure what you intention is but I think this proposal can be discussed separately. From FL point of view “study” is okay if companies are okay with it. |
| Apple | Support FL’s proposal. |
| vivo | As we commented in 1st round, we don’t think reporting for the first arrival path should be discussed here. We don’t agree to support additional N paths right now but are open to study as suggested by ZTE/CATT. |
| Ericsson | Support. |
| OPPO | We are ok with the proposal even though it looks 8.5.2 is the right sub-agenda for discussing this. |
| ZTE | To FL, We agree that this can be discussed separately. However, we should at least state that this is for “multipath reporting enhancements” that we have already agreed in 1st GTW. |
| Lenovo, Motorola Mobility | Generally fine with the proposal, but as ZTE mentioned could we add a mention in the proposal that this reporting is intended for multipath mitigation. |
| NTT DOCOMO | Support |
| LG | For clarification, in the previous 104bis-e meeting, only multiple UL-AoA/ZoA is agreed to be reported as shown below, Agreement: Reporting to LMF of M > 1 UL-AOA (AoA/ZoA) measurement values associated with the first arrival path and corresponding to the same timestamp is supported.. In our understanding, we have not discussed whether other information such as multiple timing and/or power also are reported or not. So, we need to discuss it firstly before discussion on the proposal. |
| SONY | We are OK to study (similar view as ZTE) |
| FL | The suggestion from ZTE is a good one. An updated proposal is added here: - For multipath reporting enhancements, study UL-AoA reporting from TRP to LMF support reporting angle, timing, and power for the first arrival path and additional N paths. - FFS: Value of N. To LG, our understanding is that the discussion on multiple timing/powers/angles is all being taken directly here and that is the intention of the proposal. |
| CATT | Support the updated proposal from FL above. We are fine with the addition of “multipath” and “study” in the updated proposal. |
| Intel | We think that it should be discussed together with the multi-path additional reporting discussed in 3.3.2. |
| Qualcomm | Support |
| CEWiT | We support the proposal. |

## Issue #7: DL-AoD Related Topics

Quite a few companies brought proposals specific to enhancing reporting of DL-AoD for at least LoS/NLoS detection. Some of the proposals extend to more positioning techniques. Some alignment may be needed with AI 8.5.3. The specific proposals brought were:

- [3]

  - Proposal 3:  For multi-path DL-AOD, support reporting for each path

    - TOA information defined relative to the first path (only for the additional paths)

    - A Rx beam index

    - A list of path powers measured from different PRS resources for the path measured via the Rx beam indicated by the Rx beam index

- [6]

  - Proposal 2: Rel-17 should support UE to report angular differences between Rx beams when receiving reference signals for UE-assisted DL-AOD.

- [9]

  - Proposal 1: Only the RSRP measurement of the LOS path can be used for DL-AoD positioning.

- [10]

  - Proposal 1: Support RSRP reporting with configured granularity of RSRP for timing/angle based solutions when additional paths observed by the UE are received within the cyclic prefix

  - Proposal 2: In the presence of multipath, uncertainty and expected AoD should be transferred to the UE from the LMF for DL based techniques

- [12]

  - Proposal 2: For both UE-based and UE-assisted methods of DL-AoD technique, the relative power of the first detected path to the measured RSRP is also measured and reported.

### Round #1 Discussion

Feature Lead View

The proposals are quite diverse, so the following 2 proposals are made. For the ExpectedAoD related proposal the feature lead view is that this should be discussed under 8.5.3 but if the proponent wishes to further explain the relation to LoS/NLoS that may be helpful.

Proposal 7.1

- Support the relative power and timing of multiple paths as part of DL-AoD reporting.

  - FFS: number of paths

Companies views.

| Company Name | Comments |
| --- | --- |
| vivo | We prefer not to duplicate discussion which happen in 8.5.3 as well. We’re open to discuss reporting for multipath, but think it’s too early to agree on relative power for multiple paths. On the timing part, our understanding is that timing information for DL-AoD is discussed in 8.5.3 and has not been agreed even for single path. |
| Qualcomm | This should be clarified a bit further. Does it refer to relative time of paths observed for a single PRS resource? (e.g. UE measures one PRS resources, goes in time domain and determines the main paths for the PRS resource, and then reports the time difference of the 2nd over the 1st and the 3rd of the 2nd, etc, etc). However, the absolute timing of the first path is not reported since it does not carry any information right? What matters is the relative time between paths and the relative power of those paths per PRS resource. |
| OPPO | Similar to proposal 6.1: shall this be discussed under sub agenda 8.5.3? |
| CATT | We share the same view with vivo and prefer this topic had better to be forwarded to 8.5.3. |
| Huawei, HiSilicon | Support to conclude that anything related to additional path should be discussed in 8.5.5 or discussed in each respective agenda (AoA, AoD). In our view, we think it should be better treated here, and the general increase of multi-path measurements including TOA, AoA, path strength can be resolved altogether. To QC: We think the intention here is that the first path TOA reporting is not supported for DL-AoD positioning, but serves as the common reference timing for the additional paths. |
| China Telecom | Support to discussion this proposal here. The reporting method and the first arriving path of DL-AoD method can be discussed in 8.5.3, but this proposal is about the multipath issue for DL-AoD, which should be concluded in 8.5.5. Also in the 104bis-e, the proposals about multipath were not discussed in 8.5.2/8.5.3, then in my understanding these proposals should be discussed in 8.5.5 naturally. |
| Lenovo, Motorola Mobility | Prefer that these enhancements be discussed under a general measurement framework in AI 8.5.5. |
| LG | It is similar to issue #6. We have same comment in 3.6.1. |
| InterDigital | Share the view with Vivo and CATT that this feature can be discussed in 8.5.3. |
| Futurewei | Have concern on the amount of reporting proposed. This should not be required per PRS resource unless indicated or requested. |
| Nokia/NSB | We are fine with the proposal. |

Proposal 7.2

- RAN1 to study angle difference reporting between UE Rx beams for NLOS identification in DL-AoD.

Companies views.

| Company Name | Comments |
| --- | --- |
| LG | We are fine to discuss it. |

### Round #2 Discussion

FL Summary of Round 1

See also summary in Section 3.6.1. For Proposal 7.1: this proposal seems to have some support but requires some further details. An update is proposed below. For Proposal 7.2 companies are invited to provide their views (seems maybe it was missed in round 1 discussion.

Proposal 7.1.1

- Support the relative power and relative timing of multiple paths per DL PRS resource as part of DL-AoD reporting.

  - FFS: number of paths

Companies views.

| Company Name | Comments |
| --- | --- |
| ZTE | Similar comment as Proposal 6.1.1. |
| Huawei, HiSilicon | In our understanding, the relative path timing of a path should be the same across multiple PRS resource, and the relative power for the path can be different for different PRS resource. One example can found in our contribution R1-2104281. Path #0; Path #1. Time info; Additional path delay. Rx beam info; Rx beam #1; Rx beam #0. Path power; Resource #2; Power 02; Resource #0; Power 10. Resource #1; Power 01; Resource #1; Power 11. Resource #3; Power 03; Resource #2; Power 12. Resource #0; Power 00; Resource #3; Power 13. Resource #4; Power 04; Resource #4; Power 14. The proposal can be revised as - Support relative timing of multiple additional paths and the relative power per DL PRS resource per additional path as part of DL-AoD reporting. - FFS: number of paths |
| FL | The updated proposal from Huawei seems reasonable and a bit more clear so let us use that for further discussion. |
| Apple | Support HW/HiSi’s proposal. |
| vivo | We’re not why relative timing for multiple paths should be for DL-AoD reporting. Isn’t that time information already contained in time stamp for each DL PRS resource? |
| Ericsson | Support, ok with HW/HiSi’s update. |
| OPPO | We are ok with the proposal |
| ZTE | Support Huawei’s proposal |
| Xiaomi | Ok with HW’s update |
| Lenovo, Motorola Mobility | Support HW’s revised proposal. |
| LG | Agree with HW’s revision. |
| Sony | We suggest the following modification: “Study Support the relative timing of multiple additional paths and the relative power…..” |
| Intel | We think that it should be discussed together with the multi-path additional reporting discussed in 3.3.2. |
| Nokia/NSB | We are fine with Sony’s proposal. |
| Qualcomm | Support HW’s proposal. |
| CEWiT | We support the proposal. |

Proposal 7.2.1

- RAN1 to study angle difference reporting between UE Rx beams for NLOS identification in DL-AoD.

Companies views.

| Company Name | Comments |
| --- | --- |
| ZTE | OK to agree this. |
| CATT | Support. |
| OPPO | Not support The description of 7.2.1 is not clear. What is “angle difference reporting”? Does it mean the phase difference of different Rx beam with respect to the same DL PRS resource? If so, it might not work because the absolute phase measured from different RF Rx chain do not give valid information. |
| LG | Support. |
| Nokia/NSB | Support proposal 7.2.1 To OPPO: This proposal suggests reporting of the angle difference between different reception beams. It is not related to the phase difference. The measurement would be between the boresight direction of the UE Rx beams used to receive multiple DL PRS resources. In DL-AoD positioning, reporting of the Rx beam indexes has been supported. In Rel-16 NR positioning, LMF only knew Rx beam index and there was no other information related to Rx beam index. By additionally supporting angle difference information between Rx beams, however, LMF can utilize the information for the accuracy improvement. |

### Round #3 Discussion

- Proposal 7.1.3

- For multipath reporting enhancements, study relative timing of multiple additional paths and the relative power per DL PRS resource per additional path for at least DL-AoD reporting.

  - FFS: number of paths

Proposal 7.2.2

- RAN1 to study angle difference reporting between UE Rx beams for NLOS identification in DL-AoD.

Companies views.

| Company Name | Comments |
| --- | --- |
| Huawei, HiSilicon | Support both. |
| ZTE | Support both. Small change for Proposal 7.1.3 to align the similar description in the agreement we made in last GTW. - For multipath reporting enhancements, study relative timing of multiple additional paths and the relative power per DL PRS resource per additional path for at least DL-AoD reporting (the number of paths is part of the study). - FFS: number of paths |
| vivo | We want to understand the difference between proposal 7.1.3 and the previous agreement. In our view, maybe only need to revise the Agreement as follows. - Agreement: For multipath reporting enhancements, study reporting from TRP / UE to LMF, angle, timing, phase (of additional paths) and power for the additional N paths (value of N is part of the study). - Note: Companies are not obligated to provide inputs for all parameters in their study , |
| CATT | We are fine to further study the schemes in both proposals. And we prefer the updated proposals as follows, - Proposal 7.1.3 - For multipath reporting enhancements, study reporting from UE to LMF, relative timing of multiple additional paths and the relative power per DL PRS resource per additional path for at least DL-AoD reporting. (the number of paths is part of the study) - FFS: number of paths Proposal 7.2.2 - RAN1 to study reporting from UE to LMF, angle difference reporting between UE Rx beams for NLOS identification in DL-AoD. About the question from vivo, the difference between proposal 7.1.3 and the previous agreement is that proposal 7.1.3 focuses on relative measurements reporting and related to UE reporting. |
| FL | To vivo, the difference is as CATT points out and also that we don’t have angle reporting from UE to LMF so it as simple as adding UE to the prior agreement, even though there are lots of similarity to the agreements. The updates from CATT/ZTE look reasonable and can be used for continued discussion. |
| Qualcomm | We do not support the 2nd proposal |
| IDC | We are ok to support both propposals. |
| Lenovo, Motorola Mobility | Support P7.1.3 , On P7.2.2, does the angle difference somehow take into account the UE’s relative orientation with respect to the gNB? |
| Nokia/NSB | Support FL’s proposal. To Lenovo: Proposal 7.2.2 would be a relative angle measurement between UE Rx beams at the UE and this value is not affected by UE’s orientation, so the UE orientation is not important of this proposal. |
| Apple2 | Support P7.1.3 |
| LG | We are ok to support both options and we also agree with Nokia’s comment. |
| CEWiT | We support both the proposals. |

.

## Issue #8: Specific PRS resources

Related proposals:

- [2]

  - Proposal 3: UE can be requested to measure and report on specific PRS resources by the gNB.

- [4]

  - Proposal 2: The potential impact of LOS/NLOS identification on UE selecting PRS resources/paths to report should be clarified.

- [20]

  - Proposal 1: Support signaling enhancements for the Tx/Rx LOS beam selection procedure in case of positioning OLOS and NLOS scenarios.

### Round #1 Discussion

Feature Lead View

The relevance of these proposals to mitigating NLOS/multipath is unclear to the feature lead. The proposals seem more related to beam management and PRS prioritization. Suggest the proponent to explain further the motivation and usefulness for NLOS/multipath mitigation.

- Proposal 8.1

- RAN1 to continue discussion on specific PRS resources and beam management related proposals for NLOS mitigation.

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | It’s not clear why this “continue discussion” mean. If the LMF knows that the PRS resources from multi-TRPs are in LOS conditions then the UE can directly use this information in UE-A as well in UE-B. Even environment related information may be available at the LMF from Ray-Tracing and positioning RF-planning tools. With this motivation, we propose to reformulate the proposal: “Support providing a UE with AD for the measurement of specific PRS resources for NLOS mitigation.” |
| vivo | We’re okay to study further on this aspect. |
| OPPO | We might not need this proposal. The proposal is not clear. And agree with the FL’s view, we do not see this proposal is relevant with the NLOS issue. |
| CATT | This topic seems to be more related to adjacent beams of PRS resource which is discussed in 8.5.3. We prefer to forward this topic into 8.5.3. |
| Xiaomi | We are fine to discuss it |
| Futurewei | Support, ok with revised wording from Fraunhofer |
| Nokia/NSB | Low priority in our view. |

### Round #2 Discussion (Closed)

FL Summary of Round 1

This proposal may need more time for discussion. Not clear that consensus could be reached with an update as Fraunhofer proposes to make the proposal stronger but other companies already are not so supportive of the prior one. As such companies are encouraged to continue exchanging views on this proposal.

- Proposal 8.1.1

- RAN1 to continue discussion on specific PRS resources and beam management related proposals for NLOS mitigation.

Companies views.

| Company Name | Comments |
| --- | --- |
| ZTE | The proposal is not clear to us. |
| Fraunhofer | We don’t want to make the proposal stronger but we cannot understand the progress if we agree on 8.1.1: so if we agree we can further discuss? We clearly support to identify signalling enhancements for NLOS mitigation and LOS detection if needed. As a second example to the above, in our FR2 experiment [Section 3; 20] we see that in the presence of obstacles (OLOS) the direct path can be very week and still useful (35dB attenuated compared with LOS link). The UE alone will not be able to detect such week signals without NW information. |
| CATT | We prefer to put this issue as low priority. |
| FL | To Fraunhofer, from FL point of view the proposals brought by companies did not have much support so this section could be used for companies to continue exchanging views to see if progress can be reached. Sorry if that was unclear. |
| Vivo | We’re open to study but we don’t see much value of this proposal. If the intention is to give guidance for next meeting(s), some options and/or proposals under this topic should be listed. |
| Ericsson | We think it is up to companies to clarify their intention in the next meeting. We suggest to close this item. |
| OPPO | We might not need agree on this proposal because the wording in the proposal is too general. We really do not know what is proposed. |
| ZTE | Agree with Ericsson. |
| Xiaomi | The proposal is not very clear |
| LG | We think the motivation/intention of the proposal is not clear. We prefer the issue as low priority. |
| SONY | We have similar view as Ericsson. |
| CEWiT | Low priority. |

FL summary of Round 2

Seems that this issue can be closed and interested companies can clarify their intended proposals in RAN1#106.

## Issue #9: Additional Paths

Many companies brought proposals related to extending the number of additional paths or related topics. The proposals were:

- [3]

  - Proposal 4:  Extend the number of paths for a measurement to 8, in which each path may be associated with its

    - TOA

    - Strength (Path RSRP)

      - Multiple values for DL, with each associated with a DL PRS resource

      - Single values for UL

    - Single or Multiple AoA values (UL)

    - Rx beam index (DL)

- [7]

  - Proposal 1: Support a UE to report to the LMF additional time-domain paths (beyond 2 paths which is already specified) and their corresponding relative powers

    - Applicable to both RTT and DL-TDOA methods

    - Support at least [8] total paths to be provided per PRS resource

  - Proposal 2: Support a gNB to report to the LMF additional time-domain paths (beyond 2 paths which is already specified) and their corresponding relative powers

    - Applicable to both RTT and UL-TDOA methods

    - Support at least [8] total paths to be provided per SRS resource

- [10]

  - Proposal 4: When additional paths are observed, support multiple SRSp resources associated with the reference PRS resource to compute multiple Rx-Tx values.

  - Proposal 5: An indication of multiple paths and number of paths (e.g., first path only, all of measured paths) used to compute location information for UE based DL positioning methods should be supported

- [17]

  - Proposal 1: To indicate the first arrival path by reporting the arrival time in the PRS measurement report when there is no LoS path.

  - Proposal 4: Support to reuse PRS for identifying LoS/NLoS.

- [21]

  - Proposal 8: Magnitude, SNR, Doppler frequency, angle of arrival of every path should be reported.

  - Proposal 9: It shall be unambiguously defined what additional paths a UE shall report.

  - Proposal 10: The UE shall always report both the first path and the strongest path

  - Proposal 11: For enabling good accuracy in Machine learning based LOS/NLOS detection, information of as many peaks as possible should be reported.

### Round #1 Discussion

Feature Lead View

The extension of the number of paths has some relation to the UL-AoA proposal discussed above and in 8.5.2. However, companies seem to want to extend the number of paths for all positioning techniques.

Proposal 9.1

- Support up to N additional paths in the measurement reports for at least DL-TDOA, UL-TDOA, and multi-RTT, where N>2.

  - FFS: Exact value of N.

  - FFS: reporting the power of the paths in addition to the timing.

Companies views:

| Company Name | Comments |  |
| --- | --- | --- |
| vivo | We’re not sure about the intention of N additional paths. Are they for the purpose of LOS/NLOS detection? Not agreeable without understanding the motivation. |  |
| Qualcomm | Support and include the power information (as it is done for issue 6). Merge with Issue 10, no need to discuss it separately. The feature is for multipath/LOS/NLOS mitigation. |  |
| OPPO | Not support. Reporting additional path is already supported in rel16, thus we do not need this proposal. Here is the specification in 37.355 of additional path reporting in Dl TDOA: |  |
| CATT | We can support this proposal in principle, since reporting the power of the paths for time-based positioning methods maybe benefits the identifications of NLOS/LOS. However, if current specs had supported this proposal as mentioned by OPPO, we don’t need to discuss this issue any more. |
| Qualcomm2 | To OPPO/CATT: yes up to 2 paths are already supported, that’s why the feature lead says: N>2 in the proposal. The topic here is to go beyond 2 and add the per-path powers also. |  |
| China Telecom | Similar view as CATT |  |
| Lenovo, Motorola Mobility | Generally supportive of reporting power (and existing timing) per path. |  |
| Xiaomi | We are fine to report the indication of the first arrival path. |  |
| LG | Agree. |  |
| InterDigital | We have a question for clarification. We understand the intention of the proposal to increase additional path reporting beyond N=2. Is the intention to increase N for the AdditionalPathList-r16 the main measurement? Or does the increase in N apply to the main measurement and AdditionalPathList-r16 under additional measurements also (e.g., highlighted in yellow in OPPO’s example)? |  |
| SONY | We still think the improvement from having N > 2 reported additional paths is very limited. |  |
| Nokia/NSB | Support. |  |

### Round #2 Discussion

FL Summary of Round 1

To OPPO, our understanding is that only up to 2 additional paths are supported in Rel-16 (as mentioned by Sony). The intention of the proposal is to go beyond those 2 paths. There is a proposal to merge this with Issue #10 but as that issue seems to be more controversial it may be better for progress to keep them separate for now. A small update is suggested below:

Proposal 9.1.1

- Support up to N additional paths in the measurement reports for at least DL-TDOA, UL-TDOA, and multi-RTT, where N>2.

  - FFS: Exact value of N.

  - FFS: reporting the power of the paths in addition to the timing.

Note 1: This agreement applies to additional paths (i.e., not including the “main” path).

Note 2: Rel-16 supports N=2 already.

Companies views.

| Company Name | Comments |
| --- | --- |
| ZTE | We prefer to keep current limit. We don’t see the necessity to extend the number of additional paths. |
| Fraunhofer | We see two scenarios for CIR reporting: - Scenario 1: reporting multiple additional paths in the measurement reports which includes the NLOS paths for advanced CIR fingerprinting approaches. - Scenario 2: reporting multiple additional paths around the FAP to allow the LMF determine a better measurement (for example with advanced algorithms such ML approaches) Scenario 2 is P10 in our view and we are fine to discuss together with P9 with the following changes: - Support up to N additional paths in the measurement reports for at least DL-TDOA, UL-TDOA, and multi-RTT, where N>2. - FFS: Exact value of N. - FFS: Number of additional paths around the first arriving path - FFS: reporting the power of the paths in addition to the timing. Note 1: This agreement applies to additional paths (i.e., not including the “main” path). Note 2: Rel-16 supports N=2 already |
| CATT | We are OK with the proposal, since current specs only support up to 2 additional paths. |
| vivo | We don’t agree to support N>2 additional paths without study. |
| Ericsson | Support. Ideally, the value for N should be aligned across methods. |
| OPPO | Do not support the proposal. We do not see justification to support > 2 additional paths yet. Suggest to study it first before we can agree to extend to beyond 2. |
| China Telecom | We support the proposal as long as the improvement of having N > 2 reported additional paths can be evaluated. |
| LG | We have same view with OPPO. |
| SONY | We still think the improvement from having N > 2 reported additional paths is very limited. |
| Intel | The reporting of N > 2 needs to be justified by performance gains. |
| Qualcomm | Support the proposal. From QC side, We have been providing results with N>2 for many meetings. We see clear gains. |
| CEWiT | We support the proposal. |

### Round #3 Discussion

Proposal 9.1.2

- Support up to N additional paths in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT, where N>2.

  - FFS: Exact value of N.

  - FFS: reporting the power of the paths in addition to the timing.

Note 1: This agreement applies to additional paths (i.e., not including the “main” path).

Note 2: Rel-16 supports N=2 already.

Companies views.

| Company Name | Comments |
| --- | --- |
| Fraunhofer | We suggest the following change: - Support up to N additional paths and M relative paths in the measurement reports for at least DL-TDOA, UL-TDOA, and multi-RTT, where N>2. - FFS: Exact value of N, M. - FFS: reporting the power of the paths in addition to the timing. Note 1: The N additional paths applies to additional paths (i.e., not including the “main” path). Note 2: Rel-16 supports N=2 already. Note 3: The M relative paths applies to the relative paths corresponding to the main path The near multipath components with low delay relative to the direct LOS path contribute heavily to the ToA estimation error especially for measurements within or below a 100MHz bandwidth. If the UE reports M relative paths preceding or following the FAP, then M results in significant TOA estimation improvements at the LMF since algorithms are unlikely to be implemented at the UE side due to complexity or (ML) scenario dependency. |
| Huawei, HiSilicon | We are OK with the FL original proposal. What FhG proposed seems quite unclear to us. Normally in the case FhG mentioned, UE/TRP may not be able to extract the path at all, because the relative delay is so small that it is within the time resolution given by a PRS bandwidth. We wonder what the intention is here. Is it about the CIR reporting around the main path (including amplitude and phase) so that LMF can use advanced algorithm to extract the useful path information from the cluster of the paths represented by the CIR? |
| ZTE | We don’t see the necessity to extend the number of additional paths. If some other companies want to further study the gains over this enhancement, we can accept that the main bullet should clearly say this is ” for studying multipath reporting enhancements” rather than decide this should be supported right now. Regarding the changes from Fraunhofer, we don’t see the benefits to report relative paths around a cluster. As mentioned by Huawei, UE may not be able to identify other paths in a cluster due to the time resolution limit. |
| vivo | Firstly, we don’t agree to support N>2 additional paths without study. In addition, the proposal seems a part of the following revised agreement in 3.7.3, we can only support it as the first progress - Agreement: For multipath reporting enhancements, study reporting from TRP / UE to LMF, angle, timing, phase (of additional paths) and power for the additional N paths (value of N is part of the study). - Note: Companies are not obligated to provide inputs for all parameters in their study |
| CATT | We prefer to study the N>2 additional paths measurement and reporting in current phase. Our suggested updated main bullet of the proposal as follws, - Study supporting up to N additional paths in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT, where N>2. |
| Fraunhofer_v02 | To Huawei: The proposed modification doesn’t intend to introduce new UE features. In fact it is only providing a clear UE reporting configuration of N paths within the PRS sampling rate. To address the concerns from HW we can further add that UE reports M /N within a granularity equals to or lower than the sampling frequency according the PRS bandwidth (No phase reporting is introduced by this proposal). In our view the additional path reporting has two main applications: - Reporting of additional paths including late multipath reflections which can be used for example in fingerprinting or when a cluster position is known - Reporting of additional paths near the FAP to correct the ToA offset introduced from the near multipath components using advanced ToA algorithims. Added following example for illustration: |
| Huawei, HiSilicon | To Fraunhofer: It is interesting to see the reporting without phase information can resolve the dense multi-path with very close TOA. Such a bias is also observed in our paper R1-2104297. |
| FL | To Fraunhofer, what is the different between additional paths and relative paths in your understanding. Seems clear that you want to have the UE report paths close to the LoS or “main” path but in my understanding there is no restriction on the additional paths (i.e., additional paths are already relative to the main path). So I don’t see the technical difference or need for a modification to the proposed wording. To vivo, this proposal is about DL-TDOA and multi-RTT reporting from the UE to the LMF. If companies prefer to merge this wth the proposal in section 3.7 on DL-AoD that is fine from FL perspective. Given how long we have discussed this issue it is probably just simpler to agree to them separately. Updated proposal: - Study supporting to N additional paths in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT, where N>2. - FFS: Exact value of N. - FFS: reporting the power of the paths in addition to the timing. Note 1: This agreement applies to additional paths (i.e., not including the “main” path). Note 2: Rel-16 supports N=2 already. |
| Qualcomm | What happened to the gNB reporting enhancement? This should be included in the same study (UL-TDOA & RTT) |
| ZTE | To Qualcomm, UL-TDOA&RTT have been covered by the following agreement. - Agreement: - For multipath reporting enhancements, study reporting from TRP to LMF, angle, timing, phase (of additional paths) and power for the additional N paths (value of N is part of the study). - Note: Companies are not obligated to provide inputs for all parameters in their study |
| InterDigital | We are ok with the proposal from the FL. |
| Lenovo, Motorola Mobility | Generally support FL’s proposal. |
| Fraunhofer | The LMF should be able to inform the UE which “N” paths are desired. With the current formulation we are only discussing the number of paths and associated power information. This can be captured as an additional FFS bullet or as a separate “M”. How about adding an additional FFS bullet on the update proposal to clarify: FFS: LMF reporting configuration for the selection of the N paths (observable distinct paths and/or relative paths corresponding to the main path). |
| Intel | OK with the latest FL’s proposal (with replaced “support” to “study”). |
| Nokia/NSB | Support FL’s updated proposal. For the suggestion’s from Fraunhofer, we need to consider the followings. If the UE detects multiple paths around the first paths, it is UE’s job to determine what is the first path considering those multiple paths around the first path. The UE might consider the reliability of multiple paths around the first path to determine timing of the first path. What is the advantage of the LMF determining the number of paths over UE deciding based on the reliability? This may lead to further discussion on the reliability reporting on those multiple paths around the first path. In addition, this discussion could be extended to multi-path reporting around each specific “n-th” path. Thus, we prefer not to extend the current proposal. |
| Qualcomm | Thanks to ZTE and Nokia for confirming that the previous agreement includes the gNB aspect. We support this proposal |
| Apple2 | Support the intention. Here we assume the “main” path is referring to the first arrival path. We would like to add as FFS relative power of the additional path to the first path, and impact on UE processing capability. |
| LG | We are agree with the FL’s original proposal. |
| Fraunhofer | To Nokia If the UE detects multiple paths around the first paths, it is UE’s job to determine what is the first path considering those multiple paths around the first path. The UE is not aware of the multipath near the FAP from the PRS bandlimited signal (see the CIR vs the UE detectable Correlation in the example above), the multipath will impact the correlation lobe of the FAP leading to early or late ToA detection at the UE: an advanced ToA algorithm at the LMF can achieve much better performance. The UE might consider the reliability of multiple paths around the first path to determine timing of the first path. What is the advantage of the LMF determining the number of paths over UE deciding based on the reliability? This may lead to further discussion on the reliability reporting on those multiple paths around the first path. Sorry if the proposal from Fraunhofer was not clear here: the N-path reporting is enables the LMF use information as we mentioned in previous comments to enable better (1)LOS detection OR (2) enable multipath positioning based algorithms. How shall the UE know if it LMF expects the additional path for (1) and/or (2) and report accordingly. It seems Nokia don’t share our views on the impact of (1), that’s why we think it should be part of the study proposal for the next meeting. In addition, this discussion could be extended to multi-path reporting around each specific “n-th” path. Thus, we prefer not to extend the current proposal. We don’t see the motivation for NLOS per path reporting, the reporting shouldn’t be equivalent to complete CIR reporting. The values of N in Fraunhofer view is below 10 paths. |
| CEWiT | We support FL’s proposal. |

## Issue #10: CIR reporting

Some proposals were related to the CIR. They were:

- [9]

  - Proposal 3: Support UE to measure and report the phase of the CIR corresponding to the LOS path to LMF.

- [20]

  - Proposal 4 Support the UE to measure and report phase information over multiple time instants

  - Proposal 5 Report a part of the complex valued CIR including the FAP with a resolution of 1/fs (fs is the sampling frequency according the bandwidth of the carrier)

- [21]

  - Proposal 2 The CIR generated at both gNB and at the UE should be corroborated using reciprocity principle for ensuring correct NLOS/LOS detection.

  - Proposal 3 The UE and the gNB report the impulse responses used in NLOS detection with many peaks to the LMF to validate or improve the detection.

- [16]

  - Proposal 6: Introduce LoS/NLoS identification assistance information for both DL and UL channels from LMF to UE/gNB.

    - Define both NRPPa and LPP messages

    - Format of the assistance information/status report is FFS (i.e. hard bit indication, quality metric or probability)

    - PHY/L1 can monitor CIR corresponding to the assistance information and report it back to LMF optionally.

### Round #1 Discussion

Feature Lead View

The CIR andS phase information has also been discussed under the 8.5.3 AI. In addition, it is not fully clear if these company proposals would be covered by reporting timing and power information (e.g., in proposal 9.1). Some further exchange of views is likely needed.

Proposal 10.1

- RAN1 to study reporting of CIR from the UE to the LMF for positioning.

  - FFS: which part of the CIR is reported.

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | Support |
| vivo | Not support. We prefer not to duplicate discussion in 8.5.3. Furthermore, we have strong concern on the feasibility of CIR/phase information reporting given the issues and performance evaluations we showed in our contribution to AI 8.5.3. |
| Qualcomm | It should be clarified whether this is different than Proposal 9. Is it really phase reporting? If yes, we are not supportive. If it is Power delay profile like Proposal 9, we are supportive. |
| OPPO | Not support Reporting CIR and phase information was already discussed in 8.5.3 for DL AoD. It is not feasible to report such information. For example, the phase information does not give us any meaningful information because the phase at the UE side contains many factors, including the RF hardware distortions. |
| CATT | This topic seems to be more related to “Aspect #1 reporting of first arrival path” which is discussed in 8.5.3. We prefer to forward this topic into 8.5.3. |
| LG | Do not support. The issue has been already discussed in 8.5.3. |
| Futurewei | Do not support. The FFS must be resolved prior to agreeing. |
| Nokia/NSB | We are okay to study further. |

### Round #2 Discussion

FL Summary of Round 1

Most companies seem to have concern and there may be some confusion on the intention. An update to further clarify the difference between this proposal and Proposal 9.1 is made below.

Proposal 10.1

- RAN1 to study reporting of the phase of CIR from the UE to the LMF for positioning.

Companies views.

| Company Name | Comments |
| --- | --- |
| ZTE | Not support. We don’t need to repeat discussion that we have already expressed our views in AI 8.5.3. |
| Fraunhofer | We don’t completely understand the phase of CIR probably it means the LOS phase from CIR which leads to a different proposal than original P10. We support reporting the complex part for the CIR around the FAP since the complex includes useful information for the LMF. |
| CATT | Do not support, since the signal phase is very sensitive to RF impairments, the benefits are not clear for us. |
| vivo | Don’t support. We have concern on the phase of CIR reporting. Detailed reasons stated in our contribution. |
| Ericsson | Support. The gains vs RSRP-only solution can be addressed next meeting to decide whether to support phase reporting. |
| OPPO | Do not support This has been discussed multiple times. The absolute value of phase detected at the UE side does not give valid information because it contains all kinds of phase shift imposed by all the Rx chain components. |
| LG | Same view with ZTE. |
| Nokia/NSB | Low priority. The benefit is unclear considering inherent phase shift errors from RF. |
| Qualcomm | Support |
| CEWiT | We don’t support. |

### Round #3 Discussion

Proposal 10.1.1

- For multipath reporting, study reporting of the phase of CIR for at least the first arrival path from the UE to the LMF for positioning.

Companies views.

| Company Name | Comments |
| --- | --- |
| Fraunhofer | Support |
| ZTE | As lot of companies have negative views on this. We don’t think we can get consensus at this meeting. Suggest to close the discussion. |
| vivo | Don‘t support the study of the first path here, it seems duplicate with 8.5.3 |
| CATT | Do not support, since the signal phase is very sensitive to RF impairments, the benefits are not clear for us. |
| Intel | We believe that the phase reporting of the first arrival path should be discussed in the AI 8.5.3 as it was originally discussed. |
| Nokia/NSB | Do not support. |
| LG | Do not support. |
| CEWiT | Do not support. |

## Issue #11: Measurement time window

This topic has some relation to the discussion in 8.5.1. Some companies propose using the time window to assist with LoS/NLoS identification. The proposals are:

- [12]

  - Proposal 1: For DL-AoD technique, support PRS-RSRP measurement within a configured time window wherein the power of paths out of the window, if any, does not contribute in PRS-RSRP.

    - Alternatively, or additionally, for DL-AoD technique, support PRS-RSRP for the first arrival path only that is measured within a configured time window.

- [14]

  - Proposal 3: Support the indication of the uncertainty range of the first arrival LOS path to UE/TRP.

- [15]

  - Proposal 1: For LOS/NLOS identification, RAN1 needs to consider at least following method based on:

    - Polarization characteristic

    - Propagation time difference threshold/window between a reference and a target TRP.

- [19]

  - Proposal 3: Specify a LOS/NLOS measurement window of duration, ‘T’, where the applicable LOS/NLOS positioning measurement is valid. FFS further details such as length of the measurement window(s), configuration of periodic/aperiodic measurement window(s).

- [21]

  - Proposal 4: The LMF can provide configuration for FFT window placement while doing positioning measurements or generating measurements for NLOS detection.

### Round #1 Discussion (Closed)

Feature Lead View

This discussion seems quite tied to any progress under AI 8.5.1 and the prior agreement there. As such it is proposed to discuss under 8.5.1 and wait to discuss under this AI if there are any LoS/NLoS specific details. Since there may be dependency on if RAN1 decides to introduce that feature.

Proposal 11.1

- Discuss measurement time window related proposals under 8.5.1 and wait for further progress.

Companies views:

| Company Name | Comments |
| --- | --- |
| vivo | OK to discuss under AI 8.5.1. |
| Qualcomm | OK |
| CATT | OK to discuss this issue in 8.5.1. |
| Huawei, HiSilicon | We would like to clarify the understanding on proposal from [12]. This measurement window should be treated as a small scale window to filter out the necessary additional path so that a path-specific power can be measured. It is not like the window we discussed in 8.5.1, which can last hundreds of milli-seconds; instead the window size can be couple of sampling period. If that is the case, we suggest not to mix those windows in the discussion. |
| China Telecom | OK |
| Lenovo, Motorola Mobility | Similar to Huawei we think that the discussed measurement window for LOS/NLOS is different to AI 8.5.1. Our understanding is the that this window is necessary to accurately capture LOS/NLOS behaviour over a configured duration of time, which is different from the measurement instance time stamp reporting discussed in AI 8.5.1. This separate LOS/NLOS measurement window should be discussed in AI 8.5.5. |
| Xiaomi | Support the proposal |
| LG | Agree. |
| Sony | OK to discuss it in 8.5.1 but preferably leave it as TRP/UE implementation. |
| Nokia/NSB | Support. |

FL Summary of Round 1

Seems that almost all companies are okay to support under 8.5.1 so we can consider this subsection closed and reopen if needed based on progress in 8.5.1.

## Issue #12: UE-based proposals

Two proposals specific to UE-based LoS/NLoS methods were brought:

- [10]

  - Proposal 6: Support reporting of multiple positing information derived based on different criteria for UE-based DL positioning methods

- [16]

  - Proposal 1: RAN1 to consider if enhanced signaling between LMF and UE is needed for BLADE or other NLOS mitigation techniques for UE-based operation

### Round #1 Discussion

Feature Lead View

More discussion seems to be needed. Suggest RAN1 to discuss this topic and companies to provide additional views

Companies views on the above UE-based proposals.

| Company Name | Comments |
| --- | --- |

## Issue #13: Others

Feature Lead View

The following topics were only brought by a single company. As this is the first meeting with this AI Suggest companies to provide feedback and input to the proposals to see if any consensus can be reached.

### Round #1 Discussion

- [10]

  - Proposal 3: Support spatial relationship where multiple SRSp resources can be associated with the one PRS resource

FL View

This seems already supported by the current spec. Perhaps the proponent can explain the intention below.

Companies views:

| Company Name | Comments |
| --- | --- |
| InterDigital | According to spatialRelationInfoPos in TS 38.331, spatial relationship ties one reference RS (which can be DL-PRS) and the target SRS. We believe what the FL meant by “This seems already supported by the current spec” is that the gNB can configure multiple spatial relationships, e.g., DL-PRS resource #1-SRS resource #2 and DL-PRS resource #1-SRS resource #3. In the presence of multipaths, the UE may observe multiple copies of PRS (one for LOS and another for NLOS) separated spatially. In that case, one PRS resource may be associated with more than one PRS resources. The motivation behind this proposal is to have an enhanced spatial relationship signalling to covey to the UE that one PRS beam may be spatially associated with multiple SRS beams due to the multipath channel. Thus, RRC can be enhanced to indicate DL-PRS resource #1 -{SRS resource #2, SRS resource #3}. |

- [19]

  - Proposal 4: Triggering criteria for the on-demand PRS procedure should be based on LOS/NLOS measurement quality indication. Requires further co-ordination with RAN2.

FL View

Suggest discussing this proposal under 8.5.6 with other on-demand PRS related proposals.

Companies views:

| Company Name | Comments |
| --- | --- |
| Lenovo, Motorola Mobility | The intention of this proposal is to include LOS/NLOS measurement quality indication (if agreed) as part of triggering procedure of on-demand PRS. Ok to further discuss in 8.5.6 once progress has been made. |

- [20]

  - Proposal 3: Support UE to provide the LMF with Motion Information reports with the same timestamp of the measurements or transmitted SRS.

FL View

Suggest proponent to explain how this information is useful for NLOS/multipath mitigation.

Companies views:

| Company Name | Comments |
| --- | --- |
| Fraunhofer | In reply on FL comment. The use of IMU information for multipath and NLOS mitigation is one of the main established approaches in GNSS. LPP supports IMU information since LTE and this information can be used identify for example if a change in orientation or displacement corresponds to the ToA/Phase/Doppler-measured displacement. For a NLOS the displacement will be as resulting from a virtual TRP. An example of the track information along with the measurements over a track in InF LOS scenarios is shown below: |

Proposals for GTW

Suggested Proposals for 1st GTW

Proposal 1.1.1

- Enhancements of information reporting from UE and TRP/gNB for multipath/NLOS detection and mitigation will be supported.

- FFS: Specific information including:

  - LoS/NLoS indicators

  - Additional multipath reporting from UE and TRP/gNB to LMF

- Note: This agreement is not meant to indicate that a specific solution or enhancement will be specified.

Proposal 2.2.1

- LoS/NLoS indicators should be reported for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options of LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: UE based positioning support of LoS/NLoS indicators

- Proposal 3.1.1

- Multipath reporting enhancements will be supported for DL, UL, and DL+UL positioning to enable LoS/NLoS identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

Proposal 5.1.1

- RAN1 to study the following options of LoS/NLoS detection

  - Option 1: Polarization based detection.

  - Option 2: Coherence bandwidth based detection.

  - Option 3: Implementation based solutions

  - Option 4: Propagation time difference based

  - Option 5: Rice factor and variance of CFR based solution.

  - Option 6: RSRP reporting with finer granularity

  - Other options are not precluded

- Note: Not all options necessarily imply specification impact

Proposed Conclusion:

Discuss all topics related to additional path reporting under AI 8.5.5.

Outcome of 1st GTW

Agreement:

- Study reporting of LoS/NLoS indicators for DL, UL, and DL+UL positioning measurements taken at both UE and TRP at least for UE assisted positioning.

- Study the following options (or combinations of the following options) for LoS/NLoS indicators

  - Option 1: Binary (i.e., hard) value indicators

  - Option 2: Soft value indicators (i.e., [0,1]).

    - FFS: Format and criteria for determination

  - FFS: additional information or options

- FFS: LoS/NLoS indicators for UE-based positioning

- Agreement:

- Study multipath reporting enhancements for DL, UL, and DL+UL positioning to enable LoS/NLoS/multipath identification and mitigation at the LMF for UE-assisted positioning.

  - FFS: Details of the enhancements.

Suggested Proposals for 2nd GTW

- Proposal 5.1.2

- RAN1 to study the following options of LoS/NLoS detection

  - Option 1: Polarization based detection.

  - Option 2: Coherence bandwidth-based detection.

  - Option 3: Implementation based solutions

  - Option 4: Propagation time difference based

  - Option 5: Rice factor and variance of CFR based solution.

  - Option 6: RSRP reporting with finer granularity

  - Other options are not precluded

  - FFS: Specification impact of Option 1, 2, 4, and 6 which may require additional assistance information signaled from LMF to UE/TRP.

- Note: Not all options necessarily imply specification impact

- Proposal 6.1.2

- For multipath reporting enhancements, study UL-AoA reporting from TRP to LMF support reporting angle, timing, and power for the first arrival path and additional N paths.

  - FFS: Value of N.

- Proposal 7.1.2

- For multipath reporting enhancements, study relative timing of multiple additional paths and the relative power per DL PRS resource per additional path as part of DL-AoD reporting.

  - FFS: number of paths

Proposal 9.1.2

- For multipath reporting enhancements, study up to N additional paths in the measurement reports for at least DL-TDOA, UL-TDOA, and multi-RTT, where N>2.

  - FFS: Exact value of N.

  - FFS: reporting the power of the paths in addition to the timing.

Note 1: This agreement applies to additional paths (i.e., not including the “main” path).

Note 2: Rel-16 supports N=2 already.

Outcome of 2nd GTW

- Agreement:

- For multipath reporting enhancements, study reporting from TRP to LMF, angle, timing, phase (of additional paths) and power for the additional N paths (value of N is part of the study).

- Note: Companies are not obligated to provide inputs for all parameters in their study

Suggested Proposals for 3rd GTW

- Proposal 7.1.3

- For multipath reporting enhancements, study reporting from UE to LMF, relative timing of multiple additional paths and the relative power per DL PRS resource per additional path for at least DL-AoD reporting (the number of paths is part of the study).

- Proposal 9.1.2

- Study supporting to N additional paths in the measurement reports from UE to LMF for at least DL-TDOA and multi-RTT, where N>2.

  - FFS: Exact value of N.

  - FFS: reporting the power of the paths in addition to the timing.

  - FFS: LMF requesting an additional M non-distinct paths corresponding to the first path.

Note 1: This agreement applies to N additional paths (i.e., not including the “main” path).

Note 2: Rel-16 supports N=2 already.

Proposal 5.1.3

- As part of studying LoS/NLoS information reporting, study at least the following options for information to enable/assist LoS/NLoS detection:

  - Option 1: Polarization information reporting from UE/gNB to LMF.

  - Option 2: Coherence bandwidth information reporting from UE/gNB to LMF.

  - Option 3: Propagation time difference information reporting from UE/gNB to LMF.

  - Option 4: RSRP reporting from UE/gNB to LMF with finer granularity

  - Option 5: Rice factor and the variance of Channel Frequency Response (CFR) information reporting from UE/gNB to LMF

  - Option 6: No specification impact outside of LoS/NLoS reporting

  - Note: Companies are encouraged to identify differences in information reporting and any performance gains compared with multipath information reporting

Proposal 7.2.2

- RAN1 to study reporting from UE to LMF, angle difference reporting between UE Rx beams for NLOS identification in DL-AoD.

Conclusion

In this contribution, we provided review of the submitted contributions for NR Positioning AI 8.5.5 on potential enhancements for information reporting from UE and gNB for multipath/NLOS mitigation and prepared initial set of proposals to facilitate further discussion/decision by RAN1 during the RAN1#105–e meeting.

References

- RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

- R1-2104198, NLOS Mitigation Enhancements, Futurewei.

- R1-2104281, Enhancements to support multi-path and NLOS mitigation, Huawei, HiSilicon.

- R1-2104363, Discussion on potential enhancements for multipath/NLOS mitigation, vivo.

- R1-2104524, Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, CATT.

- R1-2104594, Enhancements on NLOS mitigation for NR positioning, ZTE.

- R1-2104675, Multipath Reporting in NR Positioning, Qualcomm Incorporated.

- R1-2104743, Discussion on multipath/NLOS mitigation for NR positioning, OPPO.

- R1-2104856, Potential enhancements of information reporting from UE for multipath/NLOS mitigation, China Telecom.

- R1-2104875, Discussion on multipath/NLOS mitigation for positioning, InterDigital Inc.

- R1-2104909, Mitigation of NLOS Problem for NR Positioning, Intel Corporation.

- R1-2105109, Views on potential enhancements for NLOS mitigation in Rel-17 positioning, Apple.

- R1-2105172, Discussion on enhanced reporting from UE and gNB for Multipath/NLOS mitigation, Sony.

- R1-2105314, Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, Samsung.

- R1-2105486, Discussion on multipath/NLOS mitigation for positioning, LG Electronics.

- R1-2105516, Views on LoS/NLoS Identification and Mitigation, Nokia, Nokia Shanghai Bell.

- R1-2105565, Potential enhancements for multipath/NLOS mitigation, Xiaomi.

- R1-2105702, Discussion on multipath/NLOS mitigation for NR positioning, NTT DOCOMO, INC.

- R1-2105862, Accuracy enhancements based on NLOS/Multipath Information Reporting, Lenovo, Motorola Mobility.

- R1-2105865, Potential positioning enhancements for multipath/NLOS mitigation, Fraunhofer IIS, Fraunhofer HHI.

- R1-2105912, Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, Ericsson.