3GPP TSG RAN WG1 Meeting #105-e			R1-2106201

e-Meeting, May 10th – May 27th, 2021

Title:	Draft LS on support of UL-AOA/ZOA assistance information signalling for NR positioning

Response to:

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	Intel Corporation

To:	RAN WG3

Cc:	RAN WG2

Contact Person:

#### Name:	Alexey Khoryaev

E-mail Address:	alexey.khoryaev@intel.com

1. Overall Description:

RAN1 discussed support UL-AOA/ZOA assistance information signalling from LMF to gNB and made the following agreements.

RAN1 #104-e Meeting

Agreement:

NR supports at least the following additional assistance signaling from LMF to gNB/TRP to facilitate UL measurements of UL-AOA

  - Indication of expected AoA/ZoA value and uncertainty (of the expected AoA/ZoA value) range(s)

  - FFS: Details of procedure for providing the assistance

  - FFS: Reference angle of expected AoA/ZoA

RAN1 #104-bis-e Meeting

Agreement:

- Uncertainty range for expected UL AoA/ZoA is defined as follows

  - Expected azimuth angle of arrival as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2)

    - φAOA - expected azimuth angle of arrival, ΔφAOA – uncertainty range for expected azimuth angle of arrival

  - Expected zenith angle of arrival as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2)

    - θZOA - expected zenith angle of arrival, ΔθZOA – uncertainty range for expected zenith angle of arrival

- Select one of the following coordinate system alternatives for signaling UL AoA/ZoA assistance information

  - Alt.1: Only GCS is supported for AoA/ZoA assistance information indication

  - Alt.2: Both GCS and LCS are supported for AoA/ZoA assistance information indication

- FFS: Additional signaling for AoA/ZoA assistance information (expected value and uncertainty range)

RAN1 #105-e Meeting

Agreement:

- Both GCS and LCS are supported for UL AoA/ZoA assistance information indication.

- Note: Existing signaling can be used for obtaining LCS to GCS translation information

Agreement:

- Granularity of 0.1 degrees is applied for the expected AoA (φAOA), expected ZoA (θZOA) and the corresponding uncertainty values

Agreement:

- LMF to gNB signaling of UL AoA/ZoA assistance information (expected value and uncertainty range) is supported for UL-TDOA and Multi-RTT positioning methods

2. Actions:

To: RAN WG3

ACTION: 	RAN WG1 respectfully asks RAN WG3 to define UL-AOA/ZOA assistance signalling for NR positioning enhancements.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #106-e                       16th – 27th August, 2021		E-meeting

TSG RAN WG1 Meeting #106-bis-e                 11th – 19th October, 2021		E-meeting