3GPP TSG RAN WG1 Meeting #105-e

e-Meeting, May 10th – 27th, 2021

Title:	Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC)

Source:	Ad-hoc chair (Samsung)

## Rel-17 enhancements for NB-IoT and LTE-MTC

Please refer to RP-201306 for detailed scope of the WI

### Support of 16-QAM for unicast in UL and DL for NB-IoT

R1-2104288	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

R1-2104548	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

R1-2104716	Discussion on UL and DL 16QAM for NB-IoT	ZTE

R1-2104819	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

R1-2105374	Support 16QAM in NB-IOT Release 17	MediaTek Inc.

R1-2105622	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

R1-2105889	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

[105-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: May 24

- 2nd check point: May 27

R1-2106042	Feature lead summary #1 on 105-e-LTE-Rel17_NB_IoT_eMTC-01	Moderator (Huawei)

Agreement

Support 16-QAM for multi-TB scheduling.

Working Assumption

Support 16-QAM for NPUSCH in PUR procedure.

- FFS on support of 16-QAM for NPDSCH in PUR procedure.

Agreement

Confirm the working assumption.

The DCI size is not increased to support 16-QAM in uplink and downlink.

Agreement

For the indication of 16-QAM in downlink:

- The “Modulation and coding scheme” field in DCI Format N1 is utilized as in legacy for scheduling QPSK.

- One reserved state in the “Modulation and coding scheme” field in DCI Format N1 is utilized to indicate the use of 16QAM.

- The “Repetition number” field in DCI Format N1 is utilized to indicate the TBS indices for 16-QAM in DL when the reserved state in MCS field is indicated.

- FFS: The manner of distinguishing the different ranges of TBS indices for “Stand-alone/Guard-band” (i.e., I_TBS indices from 14 to 21) and “In-band” (i.e., I_TBS indices from 11 to 17) deployments.

Working Assumption

For downlink power allocation to support 16QAM:

- For standalone and guard-band deployments:

  - One power ratio is signaled optionally

    - NPDSCH EPRE to NRS EPRE in symbols without NRS

  - The same transmit power is assumed across different symbols.

  - If the signaling is not indicated, the legacy power allocation is used.

    - i.e., the ratio of NPDSCH EPRE to NRS EPRE is 0dB for one NRS antenna port, and -3dB for two NRS antenna ports

- UE specific signaling is used

Agreement

Introduce a new term in uplink power control of NPUSCH using 16-QAM. FFS on the details.

Agreement

When configured with downlink 16-QAM, the channel quality can be reported in MAC CE.

- FFS on support in Msg3 in connected mode

6104

Agreement

On the indication of downlink 16-QAM, when the reserved state in MCS field is indicated, the “Repetition number” field in DCI Format N1 is utilized to indicate the TBS indices

- From 14 to 21 for standalone/guardband deployments,

- From 11 to 17 for inband deployment.

- FFS: How UE distinguishes the deployment

6219

Working Assumption

For the indication of 16-QAM in uplink

- The “Modulation and coding scheme” field in DCI Format N0 is utilized as in legacy for scheduling QPSK.

- One reserved state in the “Modulation and coding scheme” field in DCI Format N0 is utilized to indicate the use of 16QAM.

- The “Repetition number” field in DCI Format N0 is utilized to indicate the TBS indices (i.e., I_TBS indices from 14 to 21) for 16-QAM in UL.

Agreement

For CQI table for downlink 16-QAM, down-select between following options in RAN1#106-e:

- Option 1: More than three candidate values for 16-QAM are added in the legacy table.

  - FFS: Which of the legacy entries are removed

- Option 2: Three candidate values for 16-QAM are added in the legacy table.

- Option 3: A new CQI table is defined for 16-QAM based on the eMTC table (CQI Tables in 36.213) as a starting point

Agreement

For downlink power allocation to support 16QAM:

- For inband deployments, a power ratio is signaled in addition to the signaling for standalone and guard-band deployments which in this case applies to “symbols with NRS” and “symbols without NRS nor CRS”. Down-select from the following options in RAN1#106-e:

  - Alt 1:

    - The existing parameter nrs-CRS-PowerOffset is reused for same PCI case, and is signaled for different PCI case.

    - The same transmit power of different symbols is assumed as to derive from nrs-CRS-PowerOffset the NPDSCH EPRE in symbols with CRS.

  - Alt 2:

    - the power ratio between NPDSCH EPRE and NRS EPRE in symbols with CRS is signaled

  - FFS: NPDSCH EPRE to NRS EPRE in symbols with NRS

FFS: Whether UE specific or cell-specific or carrier-specific signaling is used

### Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

R1-2104289	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

R1-2104549	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

R1-2104717	Remaining issues on 14-HARQ processes in DL for eMTC	ZTE

R1-2104821	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

R1-2105890	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

[105-e-LTE-Rel17_NB_IoT_eMTC-02] Email discussion on support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC – Gerardo (Ericsson)

- 1st check point: May 24

- 2nd check point: May 27

6028

Agreement

In Rel-17, for the 14 HARQ process feature the HARQ-ACK delay solution will be supported with multiple solutions: Alt-1 for full flexibility and Alt-2e for support of legacy delay

Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay).

  - Without using more than 6 bits

  - FFS: How to minimize the overhead by using joint encoding

Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

  - The HARQ-ACK delay values and the length of the HARQ-ACK delay set will be based on

    - Alt-2e: “3 bits (same as legacy)”

    - FFS: Whether HARQ delay set is to use range1 or range2

RRC signaling will be used to configure between Alt-1 and Alt-2e

FFS: Signaling details

FFS: Joint encoding

6029

Working Assumption

The PDSCH scheduling delay and HARQ-ACK delay are jointly encoded in a single DCI field:

•	The field uses no more than 7 bits if Alt-1 is configured.

•	The field is 5 bits if Alt-2e is configured.

•	FFS: Details of the joint encoding.

•	FFS: Legacy DCI fields that might be re-purposed for the jointly encoded solution of Alt-1 and Alt-2e respectively.

Note: Alt-1 expresses the HARQ-ACK delay as: (y) BL/CE DL subframe + 1 subframe + (z) BL/CE UL subframes, where y = {0, 1, 2, … 11} and z = {1, 2, 3}.

Conclusion:

In Rel-17, for the 14 HARQ processes feature:

When the HARQ-ACK delay is configured to use Alt-1 “PUCCH using Repetition = 1 is postponed”, whereas when the HARQ-ACK delay is configured to use Alt-2e “PUCCH using Repetition = 1 is not postponed (legacy behavior)”.

Agreement

In Rel-17, the 14 HARQ processes feature is applicable for HD-FDD Cat M1 UEs in CE Mode A only.

For discussion in future meetings:

Whether 14 HARQ processes feature can be enabled for PDSCH repetition case

### Others

Including any remaining issues in supporting a maximum DL TBS of 1736 bits as a Rel-17 optional UE capability

R1-2104718	Remaining issues on DL TBS of 1736 bits for CE mode A	ZTE

R1-2105891	On the L2 Buffer Size for NB-IoT and LTE-M UEs	Ericsson

R1-2105939	Discussion on DL PAPR for 16-QAM of NB-IoT	Huawei, HiSilicon

[105-e-LTE-Rel17_NB_IoT_eMTC-03] Email discussion on support of a max TBS of 1736 bits – Martin (Sony)

- 1st check point: May 24

- 2nd check point: May 27