3GPP TSG RAN WG1 #105-e	R1-210xxxx

e-Meeting, May 10th – 27th, 2021

Agenda Item:	6.2.2

Source:	Moderator (Huawei)

Title:	Feature lead summary #1 on [105-e-LTE-NB_IoTenh3-01]

Document for:	Discussion and Decision

Introduction

This contribution provides discussion on the following issue:

[105-e-LTE-NB_IoTenh3-01] Correction on DMRS cyclic shift for PUR in NB-IoT – Xiang (Huawei)

- Discussion and decision by May 24, TPs by May 27

Discussions

Issue

Description: Regarding NB-IoT PUR, contribution [1] points out that the parameter npusch-CyclicShift is only used for the DMRS in PUR transmission. However, the current spec does not limit the use case of this parameter. Therefore, [1] proposes to adopt the following TP#1 to specify that the parameter npusch-CyclicShift for DMRS cyclic shift is only for PUR transmission. Otherwise, the npusch-CyclicShift for DMRS cyclic shift dedicated for PUR may be wrongly used in some non-PUR scenarios (e.g., msg3).

TP#1: The following TP#1 for TS 36.211 is proposed by [1]:

#### --------------------------- Text starts (TS 36.211 clause 10.1.4.1.2)----------------------------- ##### 10.1.4.1.2 Reference signal sequence for <Unchanged parts are omitted> The reference signal sequences for is defined by a cyclic shift of a base sequence according to , where is given by Table 10.1.4.1.2-1 for , Table 10.1.4.1.2-2 for and Table 5.5.1.2-1 for . If group hopping is not enabled, the base sequence index is given by higher layer parameters threeTone-BaseSequence, sixTone-BaseSequence, and twelveTone-BaseSequence for , , and , respectively. If not signalled by higher layers, the base sequence is given by If group hopping is enabled, the base sequence index is given by clause 10.1.4.1.3. The cyclic shift for and is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 10.1.4.1.2-3. For , if npusch-CyclicShift is configured it provides the value of ncs and the cyclic shift α in a slot is given as α=2πncs/12 for NPUSCH (re)transmission corresponding to preconfigured uplink resource, otherwise . <Unchanged parts are omitted> #### --------------------------- Text ends (TS 36.211 clause 10.1.4.1.2)-----------------------------

Question

Question: Do you agree with TP#1 above?

| Company | Agree? | Comments |
| --- | --- | --- |
| Lenovo,MotoM | Agree | npusch-CyclicShift is configured in pur-PhysicalConfig, and the two parameters are only used for PUR other than non-PUR case. So the CR clarification is needed. |
| Qualcomm |  | Technically speaking, npusch-CyclicShift is configured only for PUR, so in all other cases “if npusch-CyclicShift is configured” will be false, and therefore we will use alpha = 0. But if a clear majority of companies feel this clarification would lead to better readability, we will not object. |
| Ericsson | See comment | We have a similar view as Qualcomm. We would be ok with the clarification, but perhaps the issue can be simply solved by appending “PUR-Config-NB” to “npusch-CyclicShift”. That is: <Unchanged parts are omitted> If group hopping is enabled, the base sequence index is given by clause 10.1.4.1.3. The cyclic shift for and is derived from higher layer parameters threeTone-CyclicShift and sixTone-CyclicShift, respectively, as defined in Table 10.1.4.1.2-3. For , if npusch-CyclicShift in PUR-Config-NB is configured it provides the value of ncs and the cyclic shift α in a slot is given as α=2πncs/12, otherwise . <Unchanged parts are omitted> |
| ZTE, Sanechips | Agree | From our understanding, the current specification indicates this parameter npusch-CyclicShift is assumed for all the NPUSCH transmission (e.g. the PUSCH in connected mode or msg3), before this parameter released or reconfiguration. It seems that the revision from Ericsson can not resolve the above issue. |

Summary

- Reference

- R1-2105269	Correction on DMRS cyclic shift for PUR in NB-IoT	ZTE