3GPP TSG RAN WG1 Meeting #105-e	  R1-21xxxxx

E-meeting, May 10 – May 27, 2021

Agenda Item:	7.2.5

Source:	Moderator (Huawei)

Title:	Summary #1 of email discussion [105-e-NR-L1enh-URLLC-01] on remaining issues on PDCCH enhancements

Document for:	Discussion and Decision

Introduction

The email discussion is to discuss the remaining issues on PDCCH enhancements.

[105-e-NR-L1enh-URLLC-01] Email discussion/approval on remaining issues on PDCCH enhancements – Chengyan (Huawei):

- Issue #2: Correction on relative SLIV reference for Type 1 HARQ codebook

- Issue #4: Remove DCI format 1_1 indicating SCell dormancy in case of 1-bit C-DAI

- Issue #6: Correction on the number of SRS resource set configuration

- Issue #3 (editor CR): Correction on RRC parameter in DCI formats when two HARQ-ACK codebooks are configured

- Issue #5 (editor CR): Editorial correction on VDAI,mlastDL

- Discussion and decision by May 24, TPs by May 27

This document summarizes the above issue and provide some initial proposals for discussion. Companies are encouraged to provide the first round views by 05/20, 11:00am UTC, then we can adjust the proposals and/or prepare the TPs for the next step discussions.

Issue #2:  Correction on relative SLIV reference for Type 1 HARQ codebook

In RAN1#104b-e meeting, the issue (details as copied below) was discussed but was not concluded yet.

Based on the current specification text in 38.213, if the UE is configured with referenceOfSLIVDCI-1-2, then for each row index with K0=0 and each PDCCH monitoring occasion with starting symbol S0>0, a new row index is added into the TDRA table. b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c a) if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for each PDCCH monitoring occasion in a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0. Based on the discussion in RAN1#104b-e, two different interpretations as below of the above descriptions were discussed and it can be seen that most companies agree with interpretation 1. - Interpretation 1: The extended SLIV applies in every slot (i.e. even for the slot(s) with no PDCCH monitoring occasion with starting symbol S0>0). Interpretation 1 ensures the same number of the set of row indexes of the TDRA tables for type-1 codebook construction for every slot. - Note: With this interpretation, there is no problem related to the Type 1 codebook size and at most some conclusion can be provided for clarification. - Support: ZTE, Ericsson, LG, Sharp, Huawei, HiSilicon, Qualcomm, Intel, CATT, NTT DCM, Vivo, OPPO - Interpretation 2: The extended SLIV is only applied to the slot(s) with PDCCH monitoring occasion with starting symbol S0>0. The candidate PDSCHs in a slot can be impacted by both PDCCH Mos and the number of PDSCH repetitions as discussed in Samsung paper. - Note: With this understanding, there is a problem with the Type-1 codebook size and further solutions need to be considered. - Support: LG, Samsung, It was proposed to put some restriction as below on top of interpretation 1, for simplicity of the specification and possible to address the concern of increasing the HARQ-ACK codebook size. =============== If the UE is provided referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all slots. =============== Therefore, the following proposal A-7 was proposed from the feature lead in RAN1#104-e. However, there was some concern raised that with this restriction, the feature is not that useful. Proposal A-7: It is recommended to conclude that, - If a UE is provided referenceOfSLIVDCI-1-2, R defined in section 9.1.2.1 of 38.213 is applicable for all slots, including the slot(s) with no PDCCH monitoring occasion with starting symbol S0>0. - If a UE is provided referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all the slots with PDCCH monitoring occasion with starting symbol S0>0.

In this meeting, several companies (i.e. Ericsson, Nokia, CATT, Samsung, Vivo and Huawei) provide further views on this issue. The following three options was observed from the views:

- Option 1:   Go with interpretation 1 with restriction.

  - If a UE is provided referenceOfSLIVDCI-1-2, R defined in section 9.1.2.1 of 38.213 is applicable for all slots, including the slot(s) with no PDCCH monitoring occasion with starting symbol S0>0.

  - If a UE is provided referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all the slots with PDCCH monitoring occasion with starting symbol S0>0.

Potential TP:

========

---------------------------------Start of Text Proposal to TS 38.213 v16.5.0-----------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

a)	if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for each PDCCH monitoring occasion in a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12  for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0. If the UE is provided with referenceOfSLIVDCI-1-2, the gNB configures Type-1 HARQ-ACK codebook only if the starting symbols of PDCCH monitoring occasions are configured the same in all slots.

--------------------------------- End of Text Proposal to TS 38.213 v16.5.0-----------------------

========

  - Pros: No increase of the HARQ-ACK codebook size compared to option 2.

  - Cons: Restriction applied which will limit the applicable scenario of the feature. Due to the configuration of search space sets may vary much, it may be difficult to always ensure same starting symbol for PDCCH monitoring occasions.

- Option 2:   Go with interpretation 1 without restriction.

  - If a UE is provided referenceOfSLIVDCI-1-2, R defined in section 9.1.2.1 of 38.213 is applicable for all slots, including the slot(s) with no PDCCH monitoring occasion with starting symbol S0>0.

  - If the starting symbols of PDCCH monitoring occasions are configured different in different slots with PDCCH monitoring occasion with starting symbol S0>0, R is determined based on the union of PDCCH monitoring configurations in all the slots.

Potential TP:

========

------------------------------------ Start of TP 38.213 V16.5.0 section 9.1.2.1---------------------------------

<unchanged text omitted>

b)	on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c

a)	if the UE is provided referenceOfSLIVDCI-1-2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for eachany PDCCH monitoring occasion in any slotin a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12  for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0

<unchanged text omitted>

------------------------------------ End of TP 38.213 V16.5.0 section 9.1.2.1---------------------------------

========

  - Pros: No restriction needed and meanwhile make the specification clearer, which will ensure more applicable scenarios for the new SLIV reference feature.

  - Cons: The HARQ-ACK codebook size may be larger compared to option 1.

- Option 3:   Slot boundary should be used as the reference of PDSCH SLIV if UE is configured with Type-1 HARQ-ACK codebook. Adopt the following TP#1 and TP#2.

TP #1

TS 38.214 - 5.1.2.1 Resource allocation in time domain When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception. Given the parameter values of the indexed row: - The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211]. - The reference point S0 for starting symbol S is defined as: - if configured with pdsch-HARQ-ACK-Codebook = dynamic and configured with referenceOfSLIVForDCI-Format1-2-r16, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the starting symbol S0 of the PDCCH monitoring occasion where DCI format 1_2 is detected; - otherwise, the starting symbol S is relative to the start of the slot using S0=0. …

TP #2

TS 38.213 - 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values K1 associated with the active UL BWP a) If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} b) If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK c) If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-ForDCIFormat1_2 d) If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2 b) on a set of row indexes R of a table that is associated with the active DL BWP and defining respective sets of slot offsets K0, start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by the union of row indexes of time domain resource allocation tables for DCI formats the UE is configured to monitor PDCCH for serving cell c a) if the UE is provided ReferenceofSLIV-ForDCIFormat1_2, for each row index with slot offset K0=0 and PDSCH mapping Type B in a set of row indexes of a table for DCI format 1_2 [6, TS 38.214], for each PDCCH monitoring occasion in a set of PDCCH monitoring occasions with different starting symbols within a slot where the UE monitors PDCCH for DCI format 1_2 and with starting symbol S0>0, if S+S0+L≤14 for normal cyclic prefix and S+S0+L≤12 for extended cyclic prefix, add a new row index in the set of row indexes of the table by replacing the starting symbol S of the row index by S+S0 …

Feature lead: Before making any proposal, companies views on the above three options are needed first.

Question 2-1: Which option (i.e. option 1, option 2 and option 3 above) do you prefer? If you have any suggestion on the TP for any of the option, please share also.

| Company | View |
| --- | --- |

Issue #3:  Correction on RRC parameter in DCI formats when two HARQ-ACK codebooks are configured

CATT R1-2104481 The RRC parameter pdsch-HARQ-ACK-CodebookList-r16 was introduced in Rel-16 for URLLC to generate two HARQ-ACK codebooks. The descriptions for this parameter in TS38.331 is as follows pdsch-HARQ-ACK-CodebookList A list of configuration for at least two simultaneously constructed HARQ-ACK codebooks. Each configuration in the list is defined in the same way as pdsch-HARQ-ACK-Codebook (see TS 38.212 [17], clause 7.3.1.2.2 and TS 38.213 [13], clauses 7.2.1, 9.1.2, 9.1.3 and 9.2.1). If this field is present, the field pdsch-HARQ-ACK-Codebook is ignored for the case at least two HARQ-ACK codebooks are simultaneously constructed.. According to the description, the HARQ-ACK codebook type is determined by pdsch-HARQ-ACK-CodebookList if configured, otherwise by field pdsch-HARQ-ACK-Codebook. However, only pdsch-HARQ-ACK-Codebook is considered for DCI formats in TS38.212 section 7.3.1. For example, for DCI format 1_1, the number of bits of DAI field is determined by the RRC parameter pdsch-HARQ-ACK-Codebook as shown below, but if pdsch-HARQ-ACK-CodebookList is configured, the bit of DAI field should be determined by the RRC parameter pdsch-HARQ-ACK-CodebookList. To resolve the issue, it should be clarified that pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-CodebookList-r16 when pdsch-HARQ-ACK-CodebookList-r16 is configured in TS38.212. - 7.3.1.2.2 Format 1_1 …… - Downlink assignment index – number of bits as defined in the following - 6 bits if more than one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included=true = enable. The 4 MSB bits are the counter DAI and the total DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group. - 4 bits if only one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included=true = enable. The 2 MSB bits are the counter DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group; - 4 bits if more than one serving cell are configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic, and nfi-TotalDAI-Included=true is not configured, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI; - 4 bits if one serving cell is configured in the DL, and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI; - 2 bits if only one serving cell is configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16=enhancedDynamic, and nfi-TotalDAI-Included=true is not configured, when the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided or the UE is not configured with ackNackFeedbackMode = joint, where the 2 bits are the counter DAI; - 0 bits otherwise.. Proposal 2: Adopt the following TP for TS38.212 section 7.3.1. -------------------------------------------------- Start of text proposal ------------------------------------------------------ ### 7.3.1 DCI formats The DCI formats defined in table 7.3.1-1 are supported. Table 7.3.1-1: DCI formats DCI format; Usage. 0_0; Scheduling of PUSCH in one cell. 0_1; Scheduling of one or multiple PUSCH in one cell, or indicating downlink feedback information for configured grant PUSCH (CG-DFI). 0_2; Scheduling of PUSCH in one cell. 1_0; Scheduling of PDSCH in one cell. 1_1; Scheduling of PDSCH in one cell, and/or triggering one shot HARQ-ACK codebook feedback. 1_2; Scheduling of PDSCH in one cell. 2_0; Notifying a group of UEs of the slot format, available RB sets, COT duration and search space set group switching. 2_1; Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE. 2_2; Transmission of TPC commands for PUCCH and PUSCH. 2_3; Transmission of a group of TPC commands for SRS transmissions by one or more UEs. 2_4; Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE. 2_5; Notifying the availability of soft resources as defined in Clause 9.3.1 of [10, TS 38.473]. 2_6; Notifying the power saving information outside DRX Active Time for one or more UEs. 3_0; Scheduling of NR sidelink in one cell. 3_1; Scheduling of LTE sidelink in one cell. The fields defined in the DCI formats below are mapped to the information bits to as follows. Each field is mapped in the order in which it appears in the description, including the zero-padding bit(s), if any, with the first field mapped to the lowest order information bit and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the number of information bits in a DCI format is less than 12 bits, zeros shall be appended to the DCI format until the payload size equals 12. The size of each DCI format is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary. If a UE is configured with pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-CodebookList-r16 in this clause. -------------------------------------------------------- End of text proposal ------------------------------------------------------

Feature lead view: The issues is valid. The TP from R1-2104481 can be taken as the starting point. Based on the rule in previous meetings, it is recommended to provide to editor directly.

Proposal 3-1: Provide the text proposal below to TS 38.212 editor.

---------------------------------Start of Text Proposal to TS 38.212 v16.5.0----------------------- ### 7.3.1 DCI formats The DCI formats defined in table 7.3.1-1 are supported. Table 7.3.1-1: DCI formats DCI format; Usage. 0_0; Scheduling of PUSCH in one cell. 0_1; Scheduling of one or multiple PUSCH in one cell, or indicating downlink feedback information for configured grant PUSCH (CG-DFI). 0_2; Scheduling of PUSCH in one cell. 1_0; Scheduling of PDSCH in one cell. 1_1; Scheduling of PDSCH in one cell, and/or triggering one shot HARQ-ACK codebook feedback. 1_2; Scheduling of PDSCH in one cell. 2_0; Notifying a group of UEs of the slot format, available RB sets, COT duration and search space set group switching. 2_1; Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE. 2_2; Transmission of TPC commands for PUCCH and PUSCH. 2_3; Transmission of a group of TPC commands for SRS transmissions by one or more UEs. 2_4; Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE cancels the corresponding UL transmission from the UE. 2_5; Notifying the availability of soft resources as defined in Clause 9.3.1 of [10, TS 38.473]. 2_6; Notifying the power saving information outside DRX Active Time for one or more UEs. 3_0; Scheduling of NR sidelink in one cell. 3_1; Scheduling of LTE sidelink in one cell. The fields defined in the DCI formats below are mapped to the information bits to as follows. Each field is mapped in the order in which it appears in the description, including the zero-padding bit(s), if any, with the first field mapped to the lowest order information bit and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the number of information bits in a DCI format is less than 12 bits, zeros shall be appended to the DCI format until the payload size equals 12. The size of each DCI format is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary. If a UE is configured with pdsch-HARQ-ACK-CodebookList-r16, pdsch-HARQ-ACK-Codebook is replaced by pdsch-HARQ-ACK-CodebookList-r16 in this clause. … < Unchanged parts are omitted > --------------------------------- End of Text Proposal to TS 38.212 v16.5.0-----------------------

| Company | View |
| --- | --- |

Issue #4: Remove DCI format 1_1 indicating SCell dormancy in case of 1-bit C-DAI

WILUS R1-2105867 Rel-16 URLLC supports the configuration of 1-bit or 2-bit counter DAI for DCI format 1_2 if one serving cell is configured in the DL. However, there is no such a configuration for DCI format 1_1 so that the DCI format 1_1 always includes 2-bit counter DAI. As a reference, we copy the relevant texts from TS38.212 v16.5.0 below. TS38.212 v16.5.0 [1] 7.3.1.2.2 Format 1_1 <…> - Downlink assignment index – number of bits as defined in the following - 6 bits if more than one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included=true = enable. The 4 MSB bits are the counter DAI and the total DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group. - 4 bits if only one serving cell are configured in the DL and the higher layer parameter nfi-TotalDAI-Included=true = enable. The 2 MSB bits are the counter DAI for the scheduled PDSCH group, and the 2 LSB bits are the total DAI for the non-scheduled PDSCH group; - 4 bits if more than one serving cell are configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16= enhancedDynamic, and nfi-TotalDAI-Included=true is not configured, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI; - 4 bits if one serving cell is configured in the DL, and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI; - 2 bits if only one serving cell is configured in the DL, the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic or pdsch-HARQ-ACK-Codebook-r16=enhancedDynamic, and nfi-TotalDAI-Included=true is not configured, when the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided or the UE is not configured with ackNackFeedbackMode = joint, where the 2 bits are the counter DAI; - 0 bits otherwise. <…> ##### 7.3.1.2.3 Format 1_2 <…> - Downlink assignment index – 0, 1, 2 or 4 bits - 0 bit if the higher layer parameter downlinkAssignmentIndexDCI-1-2 is not configured; - 1, 2 or 4 bits determined by higher layer parameter downlinkAssignmentIndexDCI-1-2 otherwise, - 4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI - 4 bits if one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI. - 1 or 2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, when the UE is not configured with coresetPoolIndex or the value of coresetPoolIndex is the same for all CORESETs if coresetPoolIndex is provided or the UE is not configured with ackNackFeedbackMode = joint, where the 1 bit or 2 bits are the counter DAI. <…>. Also, a UE does not multiplex with HARQ-ACK information of two DCI formats, where one DCI format has 2-bit counter DAI and another DCI format has 1-bit counter DAI based on the following highlighted text as yellow in TS38.213 v16.5.0 [2]. #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel … Denote by NC-DAIDL the number of bits for the counter DAI and set TD=2NC-DAIDL. Denote by VC-DAI,c,mDL the value of the counter DAI in a DCI format scheduling PDSCH reception, SPS PDSCH release or SCell dormancy indication on serving cell c in PDCCH monitoring occasion m according to Table 9.1.3-1 or Table 9.1.3-1A. Denote by VT-DAI,mDL the value of the total DAI in a DCI format in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats that include a total DAI field in PDCCH monitoring occasion m. A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field. …. Therefore, the UE generates one Type-2 HARQ-ACK CB based on 2-bit counter DAI in DCI format 1_0 and 1_1 and another Type-2 HARQ-ACK CB based on 1-bit counter DAI in DCI format 1_2 (if 1-bit counter DAI is configured). Also, the N-bit counter DAI only counts the number of {serving cell, PDCCH monitoring occasion}-pairs in DCI formats with the N-bit counter DAI. However, Table 9.1.3-1A, shown in below, says that the number of {serving cell, PDCCH monitoring occasion}-pair(s) in which DCI format 1_1 indicating SCell dormancy is also counted for the 1-bit counter DAI. Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1 DAI; VC-DAIDL; Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or DCI format 1_1 indicating SCell dormancy is present, denoted as Y and Y≥1. 0; 1; Y-1modTD+1=1. 1; 2; Y-1modTD+1=2. This is not aligned with the UE behavior “A UE does not expect to multiplex, in a same Type-2 HARQ-ACK codebook, HARQ-ACK information that is in response to detection of DCI formats with different number of bits for the counter DAI field.” So, we propose to adopt the following TP on the Clause 9.1.3.1 in TS38.213. - Proposal 1. Adopt the following TP for Clause 9.1.3.1 in TS38.213. #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1 DAI; VC-DAIDL; Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or DCI format 1_1 indicating SCell dormancy is present, denoted as Y and Y≥1. 0; 1; Y-1modTD+1=1. 1; 2; Y-1modTD+1=2. < Unchanged parts are omitted >.

Feature lead view: The issues is valid. The TP from R1-2105867 can be taken as the starting point.

Proposal 4-1: Endorse the text proposal in R1-2xxxxxx for TS 38.213 Section 9.1.3.1.

---------------------------------Start of Text Proposal to TS 38.213 v16.5.0----------------------- #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1 DAI; VC-DAIDL; Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH indicating SPS PDSCH release or DCI format 1_1 indicating SCell dormancy is present, denoted as Y and Y≥1. 0; 1; Y-1modTD+1=1. 1; 2; Y-1modTD+1=2. < Unchanged parts are omitted > --------------------------------- End of Text Proposal to TS 38.213 v16.5.0-----------------------

| Company | View |
| --- | --- |

Issue #5: Editorial corrections on VDAI,mlastDL

WILUS R1-2105867 Note that the following correction is not relevant to Rel-16 URLLC WI, but it is included in this contribution to facilitate the discussion. At the RAN1#103-e meeting, RAN1 endorsed the following alignment CR for TS38.213 (R1-2009742, CR0176) [3]. The alignment CR included editorial changes to improve readability and compatibility of equations. Among the editorial changes, the equation (highlighted as ‘yellow’) was changed from VDAI,mlastDL=0 to VDAI,mlastDL=1. Due to this change, even though the UE does not detect any DCI format scheduling CBG-based transmission, nHARQ-ACK,CBG is not 0, which makes wrong UE behaviour for obtaining a PUCCH transmission power. R1-2009742, CR0176 [3] 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel <…> <…>. Therefore, we propose to adopt the following TP on the Clause 9.1.3.1 in TS38.213. - Proposal 2. Adopt the following TP for Clause 9.1.3.1 in TS38.213. #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in Clause 7.2.1, with nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG where - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions - if NcellsDL>1,, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions - VDAI,mlastDL=10, if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions - UDAI,cCBG is the total number of DCI formats scheduling CBG-based PDSCH receptions that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cCBG=0 if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions - Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH scheduled by a DCI format that supports CBG-based PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH < Unchanged parts are omitted >.

Feature lead view: The issues is valid. The TP from R1-2105867 can be taken as the starting point. It seems the issue can be provided to editor directly.

Proposal 3-1: Provide the text proposal below to TS 38.213 editor.

---------------------------------Start of Text Proposal to TS 38.213 v16.5.0----------------------- #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel < Unchanged parts are omitted > If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in Clause 7.2.1, with nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG where - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions - if NcellsDL>1,, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions - VDAI,mlastDL=10, if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions - UDAI,cCBG is the total number of DCI formats scheduling CBG-based PDSCH receptions that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cCBG=0 if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions - Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH scheduled by a DCI format that supports CBG-based PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH < Unchanged parts are omitted > --------------------------------- End of Text Proposal to TS 38.213 v16.5.0-----------------------

| Company | View |
| --- | --- |

Issue #6: Correction on the number of SRS resource set configuration

Vivo R1-2105468 In Rel-15, only one SRS resource set can be configured with higher layer parameter usage in SRS-ResourceSet set to 'codebook' for codebook based transmission and only one SRS resource set to 'nonCodebook' for non-codebook based transmission. In Rel-16, two SRS resource sets can be configured, with each one of the sets used for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 respectively. However, the text in spec current version of spec seems to restrict only one SRS resource set for both usage ‘codebook’ and ‘non-codebook’, which is ambiguous. Draft CR as below: #### 6.1.1.1 Codebook based UL transmission For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission rank are given by DCI fields of SRS resource indicator and Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. Only one SRS resource set can be configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. For codebook based transmission, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 and codebookSubsetDCI-0-2 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability. When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' and the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetForDCI-Format0-2 is set to 'partialAndNonCoherent', and when the SRS-resourceSet with usage set to "codebook" includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 and maxRank-ForDCIFormat0_2 for PUSCH scheduled with DCI format 0_2. A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2 with 'fullyAndPartialAndNonCoherent'. A UE reporting its UE capability of 'nonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2 with 'fullyAndPartialAndNonCoherent' or with 'partialAndNonCoherent'. A UE shall not expect to be configured with the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetForDCI-Format0-2 set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that the maximum number of the configured SRS antenna ports in the SRS-ResourceSet is two. For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to 'codebook' and only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources. A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission set to 'fullpowerMode1' and codebookSubset or codebookSubsetDCI-0-2 set to 'fullAndPartialAndNonCoherent' simultaneously. The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3. The DM-RS antenna ports in Clause 6.4.1.1.3 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212]. Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources. When higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', - the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'. - up to 2 different spatial relations can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set. - subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'. #### 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder and transmission rank based on the SRI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource(s) identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' if configured. - If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, as well as DCI format 0_2 (if SRS request field is present) and DCI format 1_2 (if SRS request field is present), where AperiodicSRS-ResourceTrigger and AperiodicSRS-ResourceTriggerList (indicating the association between aperiodic SRS triggering state(s) and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. The SRS-ResourceSet(s) associated with the SRS request by DCI format 0_1 and 1_1 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and the SRS-ResourceSet(s) associated with the SRS request by DCI format 0_2 and 1_2 are defined by the entries of the higher layer parameter. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than 42 OFDM symbols. - If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with qcl-Type set to 'typeD'. - If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceId for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet. The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) and their corresponding PUSCH layers {0 … ν-1} given by DCI format 0_1 or by configuredGrantConfig according to clause 6.1.2.3 in increasing order. The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or by configuredGrantConfig according to clause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as . The DM-RS antenna ports in Clause 6.4.1.1.3 of [4, TS 38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212]. For non-codebook based transmission, the UE does not expect to be configured with both spatialRelationInfo for SRS resource and associatedCSI-RS in SRS-ResourceSet for SRS resource set. For non-codebook based transmission, the UE can be scheduled with DCI format 0_1 when at least one SRS resource is configured in SRS-ResourceSet with usage set to 'nonCodebook'.

Feature lead view: The issues is valid. The TP from R1-2105468 can be taken as the starting point. Note that the paper was submitted to MIMO, and chairman brought it us since the issue originates from the introduction of DCI format 0_2.

Proposal 6-1: Endorse the text proposal in R1-2xxxxxx for TS 38.214 Section 6.1.1.1 & 6.1.1.2.

---------------------------------Start of Text Proposal to TS 38.214 v16.5.0----------------------- #### 6.1.1.1 Codebook based UL transmission For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission rank are given by DCI fields of SRS resource indicator and Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. Only one SRS resource set can be configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook'. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. < Unchanged parts are omitted > For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to 'codebook' and only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources. < Unchanged parts are omitted > #### 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder and transmission rank based on the SRI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource(s) identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI. < Unchanged parts are omitted > --------------------------------- End of Text Proposal to TS 38.213 v16.5.0-----------------------

| Company | View |
| --- | --- |

- References

- R1-2104215 	Maintenance of PDCCH and SPS for Rel-16 NR URLLC (Ericcson)

- R1-2104312	Rel-16 URLLC/IIoT maintenance of PDCCH, Scheduling/HARQ and SPS enhancements (Nokia, Nokia Shanghai Bell)

- R1-2104323	Discussion on PDSCH processing timeline for DCI format 1_2 (ZTE)

- R1-2104410	Maintenance on PDCCH enhancements for Rel-16 NR URLLC (Spreadtrum Communications)

- R1-2104481	Remaining issues on HARQ codebook	 (CATT)

- R1-2104799	Maintenance on PDCCH for UE PDSCH processing time for DCI format 1_2 (OPPO)

- R1-2105286 	Maintanence on PDCCH as PDSCH SLIV reference (Samsung)

- R1-2105465	Maintenance on PDCCH enhancement (vivo)

- R1-2105867	Maintenance of PDCCH enhancements for Rel-16 URLLC (WILUS Inc.)

- R1-2105928	Remaining issues on PDCCH enhancements (Huawei, HiSilicon)

- R1-2105468    [Draft] CR on the number of SRS resource set configuration           vivo