3GPP TSG RAN WG1 Meeting #106-e	R1-2106443

E-meeting, August 16 –August 27, 2021

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	Enhancement on PDCCH monitoring

Document for:	Discussion and Decision

Introduction

In the previous meeting (RAN1#104b-e) [1], three alternatives for multi-slot PDCCH monitoring capability definition are further defined with more details as follows:

Agreement: Previous agreement is modified as follows: Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability - Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability. - Each slot group consists of X slots - Slot groups are consecutive and non-overlapping - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group - FFS: Further definition of capabilities - Alt 2: Use an (X, Y) span as the baseline to define the new capability - X is the minimum time separation between the start of two consecutive spans - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots] - Y <= X - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary. - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots. - FFS: Further definition of capabilities - Alt 3: Use a sliding window of X slots as the baseline to define the new capability. - The capability indicates the BD/CCE budget within the sliding window - The sliding unit of the sliding window is [1] slot. - FFS: Further definition of capabilities - Specific numbers for X, Y may depend on UE capability and gNB configuration - Examples: - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

In this contribution, we focus on the multi-slot PDCCH monitoring enhancement for new SCSs, including the monitoring capability definition, SS configuration enhancement, and cross-carrier scheduling enhancement.

Discussion

Multi-slot PDCCH monitoring capability

Three alternatives of PDCCH monitoring capability definition will be further discussed with pros and cons in this section.

Based on different limitations on value and location of Y, there are several interpretations for Alt-1:

- Y=X slots: PDCCH monitoring capability indicates the BD/CCE budget within Y consecutive slots as shown in Figure 1. Under this definition, the BD/CCE budget can be allocated anywhere within the fixed pattern, which may leads to some problematic scenarios:

  - The total BD/CCE budget is evenly distributed within X slots as shown in Figure 1 (a). It leads to more frequent PDCCH detection than that in FR2-1 if higher SCS is configured, which results in increased UE power consumption and restricted aggregation level per monitoring occasion;

  - PDCCH candidates is randomly distributed within each X slots as shown in Figure 1 (b). Then, UE might suffer from instantaneous increasing of blind detection within a slide window across two consecutive X slots (illustrated by red dotted box) in which capability is defined. Thus, higher capability is required for UE to cater to the worst case.

Figure 1. Two scenarios when Y=X of Alt-1 for PDCCH monitoring capability definition

- Y<X slots, there are further two alternatives:

  - Y is located at the first several [symbols or slots] of each X slots, as shown in Figure 2(a). The PDCCH candidates can only be located within the Y [symbols or slots]. Then the distance between two consecutive PDCCH monitoring occasions can be guaranteed. The only drawback is less flexibility on search space configuration. However, the less flexibility will not cause latency degradation considering the shorter slot duration of 480 kHz and 960 kHz SCS.

  - Y can be located at anywhere within each X slots, as shown in Figure 2(b). Similar to the case illustrated in Figure 1 (b), it might lead to an uneven BD/CCE allocation in a unit period, due to a gap between two adjacent MOs less than X slots (as illustrated by the red dotted box).

Figure 2. Three scenarios when Y<X slots of Alt-1 for PDCCH monitoring capability definition

Alt-2 is based on the capability definition per span introduced in Rel-16 URLLC WI. In Rel-16, a span is defined as the number of consecutive symbols in a slot for UE to monitor PDCCH. The maximum value of a span is Y, and the minimum gap between two consecutive spans is X symbols either within a slot or across slot boundaries. To extend such concept to multi-slot PDCCH monitoring, a straightforward approach is to replace X symbols in Rel-16 with X slots. Then, the capability is defined as Y consecutive [symbols or slots], as demonstrated in Figure 3. It will require considerable standard efforts for specification of PDCCH monitoring capability for 120kHz, 480 kHz and 960 kHz SCS in FR2-2, considering that only capability for 15 kHz SCS and 30 kHz SCS in FR1 have been specified for URLLC.

Figure 3. Alt-2 for PDCCH monitoring capability definition

To avoid the scenarios demonstrated in Figure 1 (b) and Figure 2 (b) for Alt-1, Alt-3 with a floating window was proposed. In Alt-3, the PDCCH monitoring capability is defined as a window with a floating start point and a fixed length of X slots. Within a given window, the PDCCH monitoring candidates are dynamically configured according to the latency requirement. A drawback of Alt3 is that the number of PDCCH candidates allocated per monitoring occasion may be too small to achieve high aggregation level to ensure coverage, and it may increase UE power consumption compared to Alt1 since with Alt3 the UE would have to monitor PDCCH in every slot, which goes against the starting motivation of using multi-slot PDCCH monitoring for large SCS values. The PDCCH latency of Alt1 is already good enough in our view.

Considering the tradeoff between flexibility and standard effort, we prefer Alt 1 with Y<X slots and Y located at the beginning of each X slots. Under this pattern, the location of CSS, USS per UE can be TDMed within Y symbols/slots, as shown in Figure 4. In addition, location of USS for a given UE is located at the same position within each X slots. Therefore, we can make sure the PDCCH candidates within any window of length X slots don’t exceed the PDCCH monitoring capability.

Figure 4. Search spacing location based on Alt-1 with Y < X

Using the capability definition of 120 kHz as a baseline, the value of X can be 4 and 8 for 480 kHz and 960 kHz, respectively. For the value of Y [symbols or slots], we suggest using a smaller number. Because a large time duration of Y introduces at least two problems. One is that there will be too much opportunities to configure the monitoring occasion within Y [symbols or slots], leading to a small number of PDCCH candidates configured per monitoring occasion, and the highest aggregation level cannot be achieved. The other is that it increases complexity of UE due to the frequent blind PDCCH detection. Therefore, a maximum value of Y can be 1 slot for 480 kHz and 2 slots for 960 kHz as a start point.

Observation 1:  For multi-slot PDSCH scheduling with 480 kHz and 960 kHz SCS, it is sufficient to configure a search space within Y consecutive [symbols or slots], where the Y [symbols or slot] are located at the beginning of each X slots. It is therefore sufficient to define the UE multi-slot PDCCH monitoring capability based on the fixed pattern of Y [symbols or slots] within each X slots, with Y < X.

In FR2, slot-level PDCCH monitoring is supported in terms of the maximum number of monitored PDCCH candidates and the maximum number of non-overlapped CCEs as shown in Table 1 and Table 2, respectively [2]. Typically, up to three OFDM symbols in one slot can be configured for slot-level PDCCH monitoring. In those symbols, UE blindly decodes the PDCCH candidates at different CCE aggregation levels.

Table 1. Maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of monitored PDCCH candidates per slot and per serving cell MPDCCHmax,slot,μ |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 2. Maximum number CPDCCHmax,slot,μ of non-overlapped CCEs per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of non-overlapped CCEs per slot and per serving cell CPDCCHmax,slot,μ |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

It can be observed from the tables that the maximum numbers of monitored PDCCH candidates and non-overlapped CCEs are 20 and 32 per slot for 120 kHz SCS, respectively. For 480 kHz and 960 kHz, the number of slots for multi-slot PDCCH monitoring can be 4 and 8, to keep the same monitoring duration as that of a slot under 120 kHz. Hence, to keep the same complexity as slot-level PDCCH monitoring of 120 kHz, the same maximum number of PDCCH candidates and non-overlapped CCEs can be applied to multi-slot PDCCH monitoring of 480 kHz and 960 kHz, as shown in Table 3 and Table 4. In the tables, X equals to 1 for μ≤3, equals to 4 for μ=5, and equals to 8 for μ=6.

Table 3. Maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per X slot(s) for a DL BWP with SCS configuration μ∈0, 1,2,3,5,6 for a single serving cell. X=4 forμ=5, X=8 for μ=6. X=1 otherwise.

| μ | Maximum number of monitored PDCCH candidates per X slot(s) and per serving cell MPDCCHmax,slot,μ |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |
| 5 | 20 |
| 6 | 20 |

Table 4. Maximum number CPDCCHmax,slot,μ of non-overlapped CCEs per X slot(s) for a DL BWP with SCS configuration μ∈0, 1,2,3,5,6 for a single serving cell. X=4 forμ=5, X=8 for μ=6. X=1 otherwise.

| μ | Maximum number of non-overlapped CCEs per X slot(s) and per serving cell CPDCCHmax,slot,μ |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |
| 5 | 32 |
| 6 | 32 |

Proposal 1: The BD/CCE budget for multi-slot PDCCH monitoring is defined within the first Y consecutive slots per fixed X consecutive slots, where the maximum number of PDCCH candidates and non-overlapped CCEs per X slots for 480 and 960 kHz SCS is the same as for 120 kHz with slot-level PDCCH monitoring:

  - for 480 kHz SCS: X is 4 slots, and Y = 1 slot

  - for 960 kHz SCS: X is 8 slots, and Y = 2 slots

Search Space enhancement

The time domain parameters include periodicity, offset, duration, and monitoring symbols within slot as shown in following table.

Table 5. Search space set configuration

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}

duration                                INTEGER (2..2559)

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))

…

Periodicity and offset: Assuming multi-slot PDCCH monitoring capability for 480 kHz and 960 kHz SCS is defined within the first Y [slots or symbols] per X-slot pattern as Alt 1(a) in section 2.1, a UE is usually configured with monitoring periodicity with integer multiple of X slots in order to ensure its monitoring occasion locates within the Y [slots or symbols]. Meanwhile, the number of slots will increase by 4 times and 8 times respectively for 480kHz SCS and 960kHz SCS if similar absolute monitoring periodicity as 120kHz SCS is maintained. In order not to increase the number of choices in monitoringSlotPeriodicityAndOffset too much, the unit of the periodicity could be X-slots corresponding to the multiple slot PDCCH monitoring capability, instead of one single slot. The offset values should also be scaled by X-slots considering the monitoring occasion should be located at Y [slots or symbols] from the beginning of X slots pattern. For example, “sl4” in Table 4 represents for the periodicity of 16 slots with 480 kHz SCS and 32 slots with 960 kHz SCS. The integers values of (0,1,2,3) for “sl4” correspond to slot#0, slot#3, slot#7, slot#11 in a 16-slot periodicity with 480kHz SCS and slot#0, slot#7, slot#15, slot#23 in a 32-slot periodicity with 960kHz SCS. If monitoring occasions are restricted within the first Y>1 slots in an X-slot pattern, additional slot-level offset should be added to the slot index derived from monitoringSlotPeriodicityAndOffset. Alternatively, the number of bits of monitoringSymbolsWithinSlot can be increased to cover Y slots, e.g. 28 bits for Y=2 slots. Or the number of bits of monitoringSymbolsWithinSlot could be extended to 56 bits or 112 bits as a simplest way to indicate the offset within X slots.

Duration: In Rel-15, the duration field in SearchSpace configuration denotes the number of consecutive slots that a SearchSpace lasts upon every period. The value of duration should be smaller than the periodicity indicated by monitoringSlotPeriodicityAndOffset. In order to reuse the field and the value range defined in Rel-15 for 480 kHz and 960 kHz, a new interpretation should be defined. If the PDCCH monitoring capability for 480 kHz and 960 kHz is the same as that of 120 kHz in a fixed time duration, the duration for 480 kHz and 960 kHz can be defined as a number of consecutive X-slots where SearchSpace can be located. Within each X-slots, the monitoring occasion can only be located in the first Y [symbols or slots] configured by monitoringSymbolsWithinSlot. An example is shown between 120 kHz and 480 kHz in Figure 5, wherein, Ts denotes the duration configured for SearchSpace (yellow box), ks denotes the period.

Figure 5. TDM-ed search space for different UEs within a monitoring span

Proposal 2: The time domain parameters of search space set configuration should be revised for multi-slot PDCCH monitoring for 480 kHz and 960 kHz, as follows:

  - The unit of monitoringSlotPeriodicityAndOffset is changed to X-slots, with X=4 for 480 kHz and X=8 for 960 kHz.

  - The unit of “duration” is changed to X-slots, with X=4 for 480 kHz and X=8 for 960 kHz.

  - Additional slot level offset or extension of monitoringSymbolsWithinSlot is required if monitoring occasions are within the first Y>1 slots in an X-slots pattern.

Cross-carrier scheduling

For cross-carrier scheduling, a gap in unit of PDCCH symbols between PDSCH and PDCCH are defined in Rel-16 for PDSCH resource allocation, which is associated with the SCS of PDCCH as shown in Table 6. All of them have a close relation with the PDCCH processing time and the time duration of a PDCCH symbol. For the newly introduced SCS of 480 kHz and 960 kHz SCS, it is not expected to have significant processing timeline reduction than 120 kHz SCS. The scaled values of Npdsch, from that for 120 kHz SCS can be applied as a start point, i.e., 56 and 112 for 480 kHz and 960 kHz, respectively.

Table 6. Npdsch as a function of the subcarrier spacing of the scheduling PDCCH

| µPDCCH | Npdsch [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | 14 |

Proposal 3: Npdsch for 480 kHz and 960 kHz SCS are scaled from that of 120 kHz SCS by 4 times and 8 times assuming similar PDCCH processing capability as 120 kHz SCS.

SSSG switching

In NRU Rel-16, search space set group switching is introduced to balance the UE power consumption on PDCCH monitoring and channel access flexibility from gNB side. The gNB can initiate DL transmission in the middle of slot after LBT by configuring mini slot PDCCH monitoring(SSSG#0) to UE. After detecting a scheduling PDCCH or DCI format 2-0 with SSSG switching trigger, UE may switch to SSSG#1 with longer monitoring period within a COT. UE switch back to SSSG#0 after end of COT. It takes Pswitch symbols as listed in Table 8 for UE to accomplish the SSSG switch procedure.

Table 7. Minimum value of Pswitch [symbols]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |

In FR2-2, a slot with 120 kHz SCS has similar time duration as 3 OFDM symbol mini-slot with 30 kHz SCS in FR1, which bring sufficient flexibility in channel access. Moreover, if slot-based PDCCH monitoring is supported for 480 kHz and 960 kHz SCS, the PDCCH monitoring periodicity is as small as about 15 us, which is very close to the CCA sensing slot duration. So it is not necessary to introduce SSSG switch between mini-slot monitoring and slot-based monitoring, and between slot-based monitoring and multi-slot monitoring. However, for 480 kHz and 960 kHz, SSSG switching can still bring benefit of UE power saving when SSSG switching occurs between multi-slot-based monitoring with a lower periodicity and a higher periodicity.

Proposal 4: SSSG switching can be supported between two different periodicities of multi-slot-based monitoring in order to save UE power consumption on PDCCH monitoring.

Conclusions

In this paper, we discuss the PDCCH monitoring enhancement for the new SCSs. Specifically, the definition and specification impacts of multi-slot PDCCH monitoring are considered, and we have the following observation and proposals:

Observation 1:  For multi-slot PDSCH scheduling with 480 kHz and 960 kHz SCS, it is sufficient to configure a search space within Y consecutive [symbols or slots], where the Y [symbols or slot] are located at the beginning of each X slots. It is therefore sufficient to define the UE multi-slot PDCCH monitoring capability based on the fixed pattern of Y [symbols or slots] within each X slots, with Y < X.

Proposal 1: The BD/CCE budget for multi-slot PDCCH monitoring is defined within the first Y consecutive slots per fixed X consecutive slots, where the maximum number of PDCCH candidates and non-overlapped CCEs per X slots for 480 and 960 kHz SCS is the same as for 120 kHz with slot-level PDCCH monitoring:

  - for 480 kHz SCS: X is 4 slots, and Y = 1 slot

  - for 960 kHz SCS: X is 8 slots, and Y = 2 slots

Proposal 2: The time domain parameters of search space set configuration should be revised for multi-slot PDCCH monitoring for 480 kHz and 960 kHz, as follows:

  - The unit of monitoringSlotPeriodicityAndOffset is changed to X-slots, with X=4 for 480 kHz and X=8 for 960 kHz.

  - The unit of “duration” is changed to X-slots, with X=4 for 480 kHz and X=8 for 960 kHz.

  - Additional slot level offset or extension of monitoringSymbolsWithinSlot is required if monitoring occasions are within the first Y>1 slots in an X-slots pattern.

Proposal 3: Npdsch for 480 kHz and 960 kHz SCS are scaled from that of 120 kHz SCS by 4 times and 8 times assuming similar PDCCH processing capability as 120 kHz SCS.

Proposal 4: SSSG switching can be supported between two different periodicities of multi-slot-based monitoring in order to save UE power consumption on PDCCH monitoring.

- References

3GPP TSG RAN WG1 Meeting #104-e , “RAN1 Chairman’s Notes”,  January 25th – February 5th, 2021

TS 38.213, “Physical layer procedures for control”, v16.4.0.