3GPP TSG-RAN WG1 Meeting #106-e	R1-2106503

E-meeting, August 16th – 27th, 2021

Agenda Item:	7.2.8

Source:	Huawei, HiSilicon

Title:	Discussion on clauses for positioning procedures in TS 38.214

Document for:	Discussion and Decision

Introduction

In the current TS 38.214 [2], the PRS reception procedures and positioning SRS transmission procedures are placed under Clause 5 (Physical downlink shared channel related procedures) and under Clause 6 (Physical uplink shared channel related procedure), respectively.

However, we observed that

DL PRS reception procedure is NOT related to PDSCH Rx procedure at all.

SRS for positioning Tx procedure is NOT related to PUSCH Tx procedure at all.

SRS for MIMO used for positioning is transparent to the UE.

- Given that in Rel-17, we may introduce a lot of new features for positioning, e.g. positioning measurement grouping in a measurement report, TEG reporting, DL-AoD enhancements, multi-path reporting, NLOS status reporting, PRS priority with other RS/channels, dynamic indication of measurement gap for PRS, etc., the specification will grow unexpectedly obscure, with a lengthy clause of 5.1.6.5 and 6.2.1/6.2.1.4.

- What makes it even worse is that currently the sections used to describe SRS for positioning include both the general common SRS aspects in the “hanging paragraphs” (not allowed by 3GPP specification drafting rule [1] with the definition of hanging paragraphs in Appendix) of clause 6.2.1, and the positioning SRS dedicated aspects of clause 6.2.1.4.

Even though we understand the logic of placing PRS Rx procedures in clause 5.1.6.5 and SRS Tx procedures in clauses 6.2.1/6.2.1.4 is that it follows the general framework of DL RS reception procedure and UL RS transmission procedure, we are now feeling a need to revisit such a decision.

- In additional to positioning, some other procedures for reference signals are also less relevant to PDSCH or PUSCH related procedures, including

5.1.6.1.3	CSI-RS for mobility

5.1.6.4	SRS reception procedure for CLI

5.1.6.5	PRS reception procedure

6.2.1.2	UE sounding procedure for DL CSI acquisition

6.2.1.3	UE sounding procedure between component carriers

6.2.1.4	UE sounding procedure for positioning purposes

Way-out

We want to discuss the following aspects with respect to the above problem.

A separate clause for positioning related procedures from PDSCH/PUSCH related procedures

We think that a separate clause for positioning is a proper solution to allow for any potential enhancements expected for positioning in future releases. For the current TS 38.214, we may append a new clause (Clause X) after Clause 8. Under this clause, we may at least add two subclauses X.1 and X.2 for PRS and positioning SRS related procedures, respectively. A more general solution would be separate some UE reference signal procedures from the PDSCH/PUSCH procedures, but it would require a lot of changes, which dates back to Rel-15, and is less favoured.

A refined set of subclauses for PRS

- The current PRS related procedure in 5.1.6.5 is growing too lengthy. Since it can be categorized into the following aspects, we think that it should be good to have the related subclauses, and either append the subclauses or add new subclauses in the future releases.

Assistance data

DL PRS measurement and reporting

UE PRS processing times

Contents for the positioning SRS subclause

For the current SRS related procedure, we could move clause 6.2.1.4 to the new section, keep the section 6.2.1, but also include the following paragraphs to the new section.

For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'periodic'. For operations in the same carrier, the UE is not expected to be activated or triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource with resourceType of the SRS resources as 'semi-persistent' or 'aperiodic'. For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource on different CCs, subject to UE's capability

Conclusion

In this contribution, we have the following observation and proposal regarding the specification writing for positioning related procedures.

Observation 1: The current arrangement of DL PRS reception procedure and positioning SRS transmission procedure in the clauses for PDSCH/PUSCH related procedures in TS 38.214 is not proper.

Observation 2: The clause for DL PRS reception procedure may grow obscure when new features are added.

- Proposal 1: RAN1 should decide whether

We should create a separate clause for positioning related procedures from PDSCH and PUSCH related procedures

We should create subclauses for DL PRS reception procedures

- Proposal 2: Consider adopting the accompanying draftCR (R1-2107682) to resolve the issue.

- Reference

- 3GPP TR 21.801, Specification drafting rules.

- 3GPP TS 38.214, NR; Physical layer procedures for data.

- Appendix

Definition of hanging paragraphs in TR 21.801.

### 5.2.4 Paragraph "Hanging paragraphs" such as those shown in the following example shall be avoided since reference to them is ambiguous. EXAMPLE: In the following example, the hanging paragraphs indicated cannot be uniquely identified as being in "clause 5" since strictly speaking the paragraphs in 5.1 and 5.2 are also in clause 5. To avoid this problem it would be necessary either to identify the unnumbered paragraphs as clause "5.1 Xxxxxxxxxxxx" and to renumber the existing 5.1 and 5.2 accordingly (as shown), or to add a new clause header immediately below the header for clause 5 numbered it 5.0. If hanging paragraphs are to be removed from a TS or TR already under change control, the renumbering of existing clauses being deprecated (see clause 5.2.1A), the newly introduced clause shall be numbered 5.0 (using the above example) and clauses 5.1 and 5.2 remain unchanged. Permitted; Not permitted. 5 Designation 5.1 Xxxxxxxxxxx xxxxxxx x xxxxxxxxxxxxx xxxxxxxxxxxxxxxxx xxxxxxx x xxxxxxxxxxxxx xxxxxxxxxxxxxxxxx 5.2 Xxxxxxxxxxx xxxxxxx x xxxxxxxxxxxxx xxxxxxxxxxxxxxxxx 5.3 Xxxxxxxxxxx xxxxxxx x xxxxxxxxxxxxx xxxxxxxxxxxxxxxxx xxxxxxxxxxxx xxxxxxxxxxxxx 6 Test report; 5 Designation xxxxxxx x xxxxxxxxxxxxx } xxxxxxx x xxxxxxxxxxxxx } hanging paragraphs xxxxxxx x xxxxxxxxxxxxx } 5.1 Xxxxxxxxxxx xxxxxxx x xxxxxxxxxxxxx xxxxxxxxxxxxxxxxx 5.2 Xxxxxxxxxxx xxxxxxx x xxxxxxxxxxxxx xxxxxxxxxxxxxxxxx xxxxxxxxxxxx xxxxxxxxxxxxx 6 Test report.