3GPP TSG RAN WG1 #106-e																   R1-2106643

e-Meeting, August 16th – 27th, 2021

Agenda Item:		8.1.2.3

Source:				InterDigital, Inc.

Title:	Further Details on Beam Management Enhancements for Multi-TRP

Document for:		Discussion and Decision

# Introduction

The work on the WID for Rel-17 eMIMO [1] was kicked off in the RAN1 #102-e meeting. This AI studies the enhancements to the beam management framework for multi-TRP:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In the past meetings, significant progress was achieved on group-based beam management and per-TRP BFR enhancements. In this contribution, we provide our views on the remaining issues for per-TRP BFR.

# Per-TRP BFR

PUCCH-SR resource selection rule

It’s agreed so far that up to two PUCCH-SR resources may be configured per cell group. One of the remaining open issues concerns the PUCCH-SR resource selection. The association between PUCCH-SR resources and TRP remains FFS which affects the UE’s choice of PUCCH-SR resource when a TRP fails. Three alternatives were initially discussed in RAN1#104b-e [2]:

For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs, - Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS - Alt-3: Leave it up to UE implementation - Note: PUCCH-SR resource is PUCCH resource carrying SR - FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1)

Later, several options were highlighted for down-selection in RAN1#105-e [3]:

Select one of the following alternatives with possible modification in RAN1#106-e - Alt 2.5.2 A: - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 B: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 C: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 D: - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

After the RAN1#105-e meeting, further discussions were made offline through e-mail discussions and the following proposals were drafted:

Proposal 1 For PUCCH-SR resource selection for TRP-specific BFR, - Support to configure an association between a TRP (e.g., BFD-RS set) on SpCell and a PUCCH-SR resource on SpCell. Proposal 2 - When 2 PUCCH-SR resources are configured on SpCell, if SR for BFR is triggered (e.g., by any TRP/cell failure in the cell group), the two PUCCH-SR resources are transmitted.

With respect to the Proposal 1, we agree that an association between a TRP and a PUCCH-SR resource is desirable for the SpCell case. It enables the UE to implicitly provide information on the failed TRP which is useful for the gNB’s scheduling decisions.

With respect to Proposal 2, the UE does not need to select a PUCCH-SR resource and sends on both. This is intended for reliability because the network has a better chance of receiving SR if more than one is sent. We don’t think it is necessary to send both. If increased reliability is needed, then there are other enhancements that can be used such as Rel-17 repetition enhancements.

Observation 1: Association between PUCCH-SR and BFD-RS set provides information to the network for its further scheduling decisions.

Proposal 1: Support Proposal 1 only.

For the different alternatives, one main issue raised during the e-mail discussion is for the inter-cell case. If different TRPs are configured in different cells, then there may be more TRPs than PUCCH-SR resources configured for the cell group since they are limited to 2. Then it is FFS which PUCCH-SR resource should be used for those TRPs without association.

Alt 2.5.2A does not specify a rule for choosing PUCCH-SR resources, and it is left up to UE implementation. We don’t see the benefit of leaving it up entirely to UE implementation as the association between failed TRP and PUCCH-SR provides useful information to the network on which TRP to use for the scheduling grant.

Alt 2.5.2B provides a solution when at most one BFD-RS set fails per CC. Then the UE selects the associated PUCCH-SR resource to the failed TRP. If more than one BFD-RS set fails across the CCs, then for TRPs in SCells, they may be associated with a PUCCH-SR resource in the SpCell. This ensures that the association between PUCCH-SR resource and failed TRP is configured.

Alt 2.5.2C is similar to Alt2.5.2B but the chosen PUCCH-SR resource is associated to the TRP that did not fail.

Alt 2.5.2D suggests to revert the past agreement and to reduce to 1 the number of PUCCH-SR resource configurable per cell group. This is not a desirable option because there is already significant support for 2 PUCCH-SR resources.

Based on this, we suggest that downs-election should be between Alt 2.5.2B and Alt 2.5.2C. We prefer Alt 2.5.2B.

Observation 2: The inter-cell case can be supported through associating PUCCH-SR resources from SpCell to TRPs in SCells.

Proposal 2: Support Alt 2.5.2B/Alt2.

An important special case to consider is the case when both TRPs declare BFR simultaneously. If BFR for both TRP1 and TRP2 are triggered at the same time, then it is not clear how UE should react to such situation. Such scenario represents a more severe case of beam failure, and hence it requires the quickest and most reliable response. Therefore, in our view, the BFR associated to TRP1 or PCell should be prioritized. Then, like Rel-15 PCell BFR, non-contention-based PRACH resource can be used.

Observation 3: BFR at both TRP is a severe beam failure case that needs to be considered.

Proposal 3: If beam failure is detected at both TRP simultaneously, the BFR associated to TRP1 or PCell should be prioritized.

# Conclusions

This contribution discussed the remaining issues on group-based beam management and per-TRP BFR. Based on the presented discussion, we make the following observations and proposals:

Observation 1: Association between PUCCH-SR and BFD-RS set provides information to the network for its further scheduling decisions.

Observation 2: The inter-cell case can be supported through associating PUCCH-SR resources from SpCell to TRPs in SCells.

Observation 3: BFR at both TRP is a severe beam failure case that needs to be considered.

Proposal 1: Support Proposal 1 only.

Proposal 2: Support Alt 2.5.2B/Alt2.

Proposal 3: If beam failure is detected at both TRP simultaneously, the BFR associated to TRP1 or PCell should be prioritized.

# References

- RP-193133, “New WI: Further enhancements on MIMO for NR”, Samsung, 3GPP RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104b-e, e-Meeting, April 2021

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #105-e, e-Meeting, May 2021