3GPP TSG RAN WG1 #106-e                                                     R1-2106702

E-Meeting, August 16th – 27th, 2021

Agenda Item:	8.4.2

Source:	Spreadtrum Communications

Title:	Discussion on enhancements on UL time and frequency synchronization for NTN

Document for:	Discussion and decision

Introduction

During the Rel-16 SI phase, a couple of agreements have been achieved to enhance the uplink timing and frequency synchronization for NTN scenarios. The technical issues and potential solutions, covering both GNSS UE and non-GNSS UE, are summarized in TR 38.821. The solutions are identified as follows

For the timing advance (TA) in the initial access and the subsequent TA maintenance

Option 1: Autonomous acquisition of the TA at UE with UE known location and satellite ephemeris.

Option 2: Timing advanced adjustment based on network indication

For the UL frequency compensation

Option 1: Both the estimation and pre-compensation of UE-specific frequency offset are conducted at the UE side. The acquisition of this value can be done by utilizing DL reference signals, UE location and satellite ephemeris.

Option 2: The required frequency offset for UL frequency compensation at least in LEO systems is indicated by the network to UE. The acquisition on this value can be done at the network side with detection of UL signals, e.g., preamble.

In this contribution, we discuss our views on the enhancement of the uplink timing and frequency synchronization for NTN scenarios.

UL timing synchronization for NTN

Granularity and signalling of Common TA

In RAN1#104b_e, the follow proposal on NTN UE Timing Advance formula had been achieved [1].

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e.

In one cell, different SCS may be supported simultaneously. A large SCS will result in a short CP. In the case of large SCS, UE is more sensitive to changes in TA. Therefore, large granularity is not preferred in the expression of NTA,common  , when the SCS is large. In our view, NTA,common should be expressed in the legacy granularity of Tc units.

Proposal 1: NTA,common should be expressed in the legacy granularity of Tc units.

TA update in RRC_CONNECTED state

In RAN1#105, the following proposal and FL recommendation on TA update in RRC_CONNECTED state had been achieved [2].

Updated Proposal 6: In RRC_CONNECTED state in NR NTN: - NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction as in NR Release 16. - FFS: Whether some enhancements are needed - NTA,UE-specific is updated autonomously by the UE based on its GNSS acquired position and satellite ephemeris - The UE calculates the NTA,UE-specific (in Tc units) as follows: - NTA, UE-specific=ds,DL+ds,UL/Tc where ds,DL is the DL service link delay of the signal to which the UE local time reference is synchronized and ds,UL is the UL service link delay of the UL signal to which the UE applies the TA. - NTA,common is updated autonomously by the UE based on the parameters acquired from SIB - FFS: How the UE calculates/update the NTA,common - UE always add the accumulated TA command and UE autonomous TA

FL recommendation 6: Companies are encouraged to review the Updated Proposal 6 and provide inputs to RAN1#106-e on the following aspects: - The formula to be used for UE-specific TA - The formula to be used for Common TA - Whether some enhancements are needed regarding NTA update - RAN1 working assumption would be that the UE always add the accumulated TA command and UE autonomous TA. BUT the group needs to check if there is any concern.

Regarding of the formula to be used for UE-specific TA, it is reasonable to leave it as an UE implementation. There is no needs to introduce additional description for DL and UL, separately.

Proposal 2: The calculation of UE-specific TA is up to UE implementation.

NTA,common is updated autonomously by the UE based on the parameters acquired from SIB. When SIB has not signaled, the NTA,common, NTA,common=0. The formula to be used for Common TA is based indication of common TA drift parameters.

For open loop method, NR NTN UE can use an acquired GNSS position and satellite ephemeris to calculate the UE specific TA for service link and apply the calculated values accordingly in RRC_CONNECTED states. For closed loop method, the same mechanism as existing specs shall be used, i.e., TAC provided in mgs2 and subsequent TACs provided within the MAC CE. If UE can obtain accurate TA based on GNSS capability and satellite ephemeris automatically, TA maintenance can be relying on open-loop method only. Otherwise, closed loop method can be considered to help UE to obtain accurate TA, i.e., received TA commands.

Proposal 3: Existing NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change.

UL Frequency adjustment for UE in RRC connected mode

For the UL frequency compensation, at least for LEO system, the following solutions are identified with consideration on the beam specific post-compensation of common frequency offset at the network side in R16 SI:

- Option-1: Both the estimation and pre-compensation of UE-specific frequency offset are conducted at the UE side. The acquisition of this value can be done by utilizing DL reference signals, UE location and satellite ephemeris.

- Option-2: The required frequency offset for UL frequency compensation at least in LEO systems is indicated by the network to UE. The acquisition on this value can be done at the network side with detection of UL signals, e.g., preamble.

Common frequency pre-compensation offset on DL service link

In RAN1#105e, the following FL recommendation is made [2].

FL recommendation 11: Companies are encouraged to study the pro and cons of support of Common DL frequency compensation for the service link Doppler. By taking into account the impact on the UE, gNB and the signalling overhead especially in case of the Earth fixed cell. - Is support of Common DL frequency compensation for the service link Doppler beneficial and needed? - Companies are encouraged to provide simulations on the benefits of support of Common DL frequency compensation for the service link Doppler: Note: the performance evaluation on the DL synchronization performance is conducted during the SI. The corresponding results are summarized in [R1-1909479]. But it seems that they do not clearly show the necessity of support of Common DL frequency compensation for the service link Doppler - What would be the reasonable Way forward? - WF1: Similar to Earth moving cell, the amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE. Note: In this case , the UE needs to frequently acquire the SIB to retrieve common pre-compensated FO parameters. every 2.3 seconds. Which make this is WF questionable. - WF 2: Indicate the beam-specific ECEF co-ordinates of a fixed Reference Point w.r.t the common Doppler shift experienced on the DL service link is pre-compensated by the gNB. Note: The UE will assume that the pre-compensation changes continuously with time. Thereby, the gNB DL frequency needs to be periodically updated so that the deviation between the UE self-calculated DL pre-compensation (based on indicated beam-specific ECEF co-ordinates and Satellite ephemeris) and actual pre-compensation remains within a defined threshold e.g. 10% of 0.1PPM. - WF3: deprioritize support of DL frequency compensation for the service link Doppler in Release 17.

In R16 SI, the performance evaluation on the DL synchronization performance is conducted. It is observed that for DL initial synchronization, robust performance can be provided by the SSB design in Rel-15 in case of GEO and LEO with beam specific pre-compensation of common frequency shift, e.g., conducted with respect to the spot beam center at network side, respectively. However, for the LEO without pre-compensation of the frequency offset, additional complexity is needed at UE receiver to achieve robust DL initial synchronization performance based on Rel-15 SSB. No further enhancement on the SSB is needed.

In our view, DL frequency compensation for the service link Doppler is beneficial to prevent the UEs from having to deal with a very large frequency offset in DL, which would add complexity to the DL synchronization and potentially delay the initial access procedure. If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

Proposal 4: The amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE.

Common post-compensation frequency offset for Uplink

In RAN1#105e, the following FL recommendation is made [2].

FL Recommendation 12: On the need and indication of common post-compensation frequency offset for Uplink companies are encouraged to provide inputs to next RAN1 meeting

For GNSS UE, UE only pre-compensates its estimated Doppler shift on the service link in its uplink transmissions and frequency correction signaling can be avoided. In our view, common frequency post-compensation for Uplink is not needed. Therefore, indication of common post-compensation frequency offset for Uplink is not needed.

Proposal 5: Indication of common post-compensation frequency offset for Uplink is not needed.

Close control loop for UL frequency alignment

In RAN1#105e, the following FL recommendation is made [2].

FL recommendation 13 RAN1 to further investigate the need and benefit to support closed-loop UL frequency compensation

For GNSS UE, both the estimation and UE-specific UL frequency compensation is conducted at the UE side, the accuracy of estimated frequency offset for UL frequency adjustment can be ensured. In our view, close control loop for UL frequency alignment is not needed.

Proposal 6: Close control loop for UL frequency alignment is not needed.

Conclusion

As summary, we have the following proposals.

Proposal 1: NTA, common should be expressed in the legacy granularity of Tc units.

Proposal 2: The calculation of UE-specific TA is up to UE implementation.

Proposal 3: Existing NTA update based on TA Command field in msg2/msgB and MAC CE TA command is used for UL timing alignment correction without specification change.

Proposal 4: The amount of frequency that has been pre-compensated in DL, relative to the nominal DL Tx frequency, shall be indicated to the UE.

Proposal 5: Indication of common post-compensation frequency offset for Uplink is not needed.

Proposal 6: Close control loop for UL frequency alignment is not needed.

Reference

- 3GPP RAN1#104b_e Chairman’s Notes, April 12 - April 20, 2021.

- R1-2106290, “FL Summary on enhancements on UL time and frequency synchronization for NR”, Moderator (Thales).