3GPP TSG RAN WG1 #106-e                                               R1-2106848

e-Meeting, August 16th –27th, 2021

Source:               ZTE, Sanechips

Title:                    Remaining issues for support of 14-HARQ processes in DL for eMTC

Agenda item:      8.9.2

Document for:    Discussion and Decision

Introduction

In RAN1 #105 e-meeting, the following agreements/working assumption/conclusion were achieved for 14-HARQ processes in DL for LTE-MTC [1]：

Agreement

In Rel-17, for the 14 HARQ process feature the HARQ-ACK delay solution will be supported with multiple solutions: Alt-1 for full flexibility and Alt-2e for support of legacy delay

Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay).

  - Without using more than 6 bits

  - FFS: How to minimize the overhead by using joint encoding

Alt-2e: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

  - The HARQ-ACK delay values and the length of the HARQ-ACK delay set will be based on

    - Alt-2e: “3 bits (same as legacy)”

    - FFS: Whether HARQ delay set is to use range1 or range2

RRC signaling will be used to configure between Alt-1 and Alt-2e

FFS: Signaling details

FFS: Joint encoding

Working Assumption

The PDSCH scheduling delay and HARQ-ACK delay are jointly encoded in a single DCI field:

• The field uses no more than 7 bits if Alt-1 is configured.

• The field is 5 bits if Alt-2e is configured.

• FFS: Details of the joint encoding.

•  FFS: Legacy DCI fields that might be re-purposed for the jointly encoded solution of Alt-1 and Alt-2e respectively.

Note: Alt-1 expresses the HARQ-ACK delay as: (y) BL/CE DL subframe + 1 subframe + (z) BL/CE UL subframes, where y = {0, 1, 2, … 11} and z = {1, 2, 3}.

Agreement

In Rel-17, the 14 HARQ processes feature is applicable for HD-FDD Cat M1 UEs in CE Mode A only.

For discussion in future meetings:

Whether 14 HARQ processes feature can be enabled for PDSCH repetition case

In this contribution, we discuss the detailed signaling of PDSCH scheduling delay and HARQ-ACK delay for LTE-MTC.

Discussion

In last RAN1 meeting, it was agreed that the 14 HARQ processes feature is applicable for HD-FDD Cat M1 UEs in CE Mode A only. From the perspective of peak data rate, feature of 14-HARQ processes is not enabled for PDSCH repetition case. In this case, the 2-bit ‘Repetition number’ field in DCI format 6-1A can be repurposed when 14-HARQ processes is enabled.

Proposal 1: The PDSCH repetition number is assumed to be 1 if 14-HARQ processes feature is enabled.

Details of signaling

For the 14 HARQ process feature, the HARQ-ACK delay solution will be supported with multiple solutions: Alt-1 for full flexibility and Alt-2e for support of legacy delay. Alt-1 or Alt-2e is configured by UE-specific high layer RRC signaling. For solution Alt-2e, both HARQ-ACK delay set range1 and range2 can be supported. The specific HARQ-ACK delay value is signaled by DCI.

Proposal 2: For HARQ-ACK solution Alt-2e, both HARQ-ACK delay set range1 and range2 can be supported. The specific HARQ-ACK delay value is signaled by DCI.

If percentage of invalid subframes is 0, when PDSCH scheduling delay is 2, the legacy HARQ-ACK delay value, i.e., 4~11, can be reused. When PDSCH scheduling delay is 7, considering that the minimum gap between PDSCH and corresponding UL subframe for HARQ-ACK transmission is 11 (as highlighted in yellow in Figure 1), the minimum HARQ-ACK delay value would be 12.

Figure 1

When percentage of invalid subframes is larger than 0, the HARQ-ACK delay value would be increased along with the increase of number of invalid subframes.

Observation 1: The minimum HARQ-ACK delay value would be 12 when PDSCH scheduling delay is 7.

The PDSCH scheduling delay and HARQ-ACK delay are jointly encoded in a single DCI field. The field is 5 bits if Alt-2e is configured. Legacy 2-bit ‘Repetition number’ field and 3-bit ‘HARQ-ACK delay field’ in DCI format 6-1A can be repurposed to indicate the 5-bit ‘PDSCH scheduling delay and HARQ-ACK delay’ field. One example of joint indication is shown in Table 1. Besides legacy HARQ-ACK delay values, larger HARQ-ACK delay values also can be supported in this example.

Table 1 Example of joint indication if Alt-2e is configured

| 'Repetition number' field in DCI | 'HARQ-ACK delay' field in DCI | Scheduling delay for PDSCH | HARQ-ACK delay value in absolute subframes |
| --- | --- | --- | --- |
| 00 | 000 | 2 | 4 |
|  | 001 | 2 | 5 |
|  | 010 | 2 | 6 |
|  | 011 | 2 | 7 |
|  | 100 | 2 | 8 |
|  | 101 | 2 | 9 |
|  | 110 | 2 | 10 |
|  | 111 | 2 | 11 |
| 01 | 000 | 2 | 12 |
|  | 001 | 2 | 13 |
|  | 010 | 2 | 14 |
|  | 011 | 2 | 15 |
|  | 100 | 2 | 16 |
|  | 101 | 2 | 17 |
|  | 110 | 7 (Type 1) | 12 |
|  | 111 | 7 (Type 2) | 12 |
| 10 | 000 | 7 (Type 1) | 13 |
|  | 001 | 7 (Type 2) | 13 |
|  | 010 | 7 (Type 1) | 14 |
|  | 011 | 7 (Type 2) | 14 |
|  | 100 | 7 (Type 1) | 15 |
|  | 101 | 7 (Type 2) | 15 |
|  | 110 | 7 (Type 1) | 16 |
|  | 111 | 7 (Type 2) | 16 |
| 11 | 000 | 7 (Type 1) | 17 |
|  | 001 | 7 (Type 2) | 17 |
|  | 010 | 7 (Type 1) | 18 |
|  | 011 | 7 (Type 2) | 18 |
|  | 100 | 7 (Type 1) | 19 |
|  | 101 | 7 (Type 2) | 19 |
|  | 110~111 | Reserved |  |

Proposal 3: For HARQ-ACK solution Alt-2e, legacy 2-bit ‘Repetition number’ field and 3-bit ‘HARQ-ACK delay field’ in DCI format 6-1A can be repurposed to indicate the 5-bit ‘PDSCH scheduling delay and HARQ-ACK delay’ field.

- HARQ-ACK delay value in absolute subframes would be {4~17} when PDSCH scheduling delay is 2.

- HARQ-ACK delay value in absolute subframes would be {12~19} when PDSCH scheduling delay is 7.

If Alt-1 is configured, for PDSCH scheduling delay of 2, the value set y can be fixed for a certain z.

- y would be {8, 9 10, 11} BL/CE DL subframes for z = 1 BL/CE UL subframe

- y would be {4, 5, 6, 7} BL/CE DL subframes for z = 2 BL/CE UL subframes

- y would be {0, 1, 2, 3} BL/CE DL subframe(s) for z = 3 BL/CE UL subframes

For PDSCH scheduling delay of 7, the value z would be 1 BL/CE UL subframe which corresponds to {8, 9 10, 11} BL/CE DL subframes in order to avoid earlier scheduling but later feedback .

‘PDSCH scheduling delay and HARQ-ACK delay’ joint encoding field can be indicated by 5 bits. Legacy 2-bit ‘Repetition number’ field and 3-bit ‘HARQ-ACK delay field’ in DCI format 6-1A can be repurposed for indication. One example of joint indication is shown in Table 2.

Table 2 Example of joint indication if Alt-1 is configured

| Joint encoding ‘PDSCH scheduling delay and HARQ-ACK delay’ field |  | Scheduling delay for PDSCH | HARQ-ACK delay {(y) BL/CE DL subframe + 1 subframe + (z) BL/CE UL subframes} |  |
| --- | --- | --- | --- | --- |
| repurposed legacy 'Repetition number' field | repurposed legacy 'HARQ-ACK delay' field |  | y | z |
| 00 | 000 | 2 | 8 | 1 |
|  | 001 | 2 | 9 | 1 |
|  | 010 | 2 | 10 | 1 |
|  | 011 | 2 | 11 | 1 |
|  | 100 | 2 | 4 | 2 |
|  | 101 | 2 | 5 | 2 |
|  | 110 | 2 | 6 | 2 |
|  | 111 | 2 | 7 | 2 |
| 01 | 000 | 2 | 0 | 3 |
|  | 001 | 2 | 1 | 3 |
|  | 010 | 2 | 2 | 3 |
|  | 011 | 2 | 3 | 3 |
|  | 100 | 7 (Type 1) | 8 | 1 |
|  | 101 | 7 (Type 2) | 8 | 1 |
|  | 110 | 7 (Type 1) | 9 | 1 |
|  | 111 | 7 (Type 2) | 9 | 1 |
| 10 | 000 | 7 (Type 1) | 10 | 1 |
|  | 001 | 7 (Type 2) | 10 | 1 |
|  | 010 | 7 (Type 1) | 11 | 1 |
|  | 011 | 7 (Type 2) | 11 | 1 |
|  | 100~111 | Reserved |  |  |
| 11 | 110~111 |  |  |  |

Proposal 4: For HARQ-ACK solution Alt-1, legacy 2-bit ‘Repetition number’ field and 3-bit ‘HARQ-ACK delay field’ in DCI format 6-1A can be repurposed to indicate the 5-bit ‘PDSCH scheduling delay and HARQ-ACK delay’ field.

- For PDSCH scheduling delay of 2, the value set y can be fixed for a certain z.

      - y would be {8, 9 10, 11} BL/CE DL subframes for z = 1 BL/CE UL subframe

      - y would be {4, 5, 6, 7} BL/CE DL subframes for z = 2 BL/CE UL subframes

      - y would be {0, 1, 2, 3} BL/CE DL subframe(s) for z = 3 BL/CE UL subframes

- For PDSCH scheduling delay of 7, the value z would be 1 BL/CE UL subframe which corresponds to {8, 9 10, 11} BL/CE DL subframes.

Conclusions

In this contribution, we have discussed the remaining issues to support DL 14-HARQ processes for H-FDD LTE-MTC. We make the following observations and proposals:

Observation 1: The minimum HARQ-ACK delay value would be 12 when PDSCH scheduling delay is 7.

Proposal 1: The PDSCH repetition number is assumed to be 1 if 14-HARQ processes feature is enabled.

Proposal 2: For HARQ-ACK solution Alt-2e, both HARQ-ACK delay set range1 and range2 can be supported. The specific HARQ-ACK delay value is signaled by DCI.

Proposal 3: For HARQ-ACK solution Alt-2e, legacy 2-bit ‘Repetition number’ field and 3-bit ‘HARQ-ACK delay field’ in DCI format 6-1A can be repurposed to indicate the 5-bit ‘PDSCH scheduling delay and HARQ-ACK delay’ field.

- HARQ-ACK delay value in absolute subframes would be {4~17} when PDSCH scheduling delay is 2.

- HARQ-ACK delay value in absolute subframes would be {12~19} when PDSCH scheduling delay is 7.

Proposal 4: For HARQ-ACK solution Alt-1, legacy 2-bit ‘Repetition number’ field and 3-bit ‘HARQ-ACK delay field’ in DCI format 6-1A can be repurposed to indicate the 5-bit ‘PDSCH scheduling delay and HARQ-ACK delay’ field.

- For PDSCH scheduling delay of 2, the value set y can be fixed for a certain z.

      - y would be {8, 9 10, 11} BL/CE DL subframes for z = 1 BL/CE UL subframe

      - y would be {4, 5, 6, 7} BL/CE DL subframes for z = 2 BL/CE UL subframes

      - y would be {0, 1, 2, 3} BL/CE DL subframe(s) for z = 3 BL/CE UL subframes

- For PDSCH scheduling delay of 7, the value z would be 1 BL/CE UL subframe which corresponds to {8, 9 10, 11} BL/CE DL subframes.

- References

- 3GPP, RAN1 Chairman’s Notes, RAN1 #105-e