3GPP TSG RAN WG1 Meeting #106-e					      R1-2107038

e-Meeting, August 16th – 27th, 2021

Source:	Fujitsu

Title:	Considerations on Inter-UE Coordination for Mode 2 Enhancements

Agenda item:	8.11.1.2

Document for:	Discussion and Decision

Introduction

In RAN1#104b, the following agreements have been achieved for inter-UE coordination [1]. In RAN1#105, no further agreements have been achieved.

Agreement: - Support the following schemes of inter-UE coordination in Mode 2: - Inter-UE Coordination Scheme 1: - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information - FFS condition(s) in which Scheme 1 is used - Inter-UE Coordination Scheme 2: - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict - FFS condition(s) in which Scheme 2 is used Agreements: - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination: - Details include applicable scenario(s)/inter-UE coordination scheme(s) - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc. - Including the possibility of being subject to certain conditions and/or capability Agreement: - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission - For scheme 1: - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information - For scheme 2: - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

According to the agreements so far, both Scheme 1 indicating a set of resources and Scheme 2 indicating the presence of conflict are candidates for inter-UE coordination. In this contribution, we express our views on more details of Scheme 1 and Scheme 2.

Discussions

Inter-UE coordination Scheme 1

The scenarios where Scheme 1 is used

Figure 1: Hidden-node problem.                                      Figure 2: Half-duplex problem.

As one type of Scheme 1, UE A can send UE B the set of resources preferred for UE B’s transmission. In our view, sending preferred resources is useful at least for solving the hidden-node and half-duplex problems. Furthermore, it is more suitable to be used in the scenario where TX UE B transmits to RX UE A.

In Figure 1, the hidden-node problem is illustrated. Although UE A is interfered by UE C, UE B is not aware of UE C since UE C is outside its sensing range. Therefore, UE B’s transmission cannot avoid the interference from UE C. The half-duplex problem is shown in Figure 2. UE B transmits to UE A in the slots where UE A is performing its own transmission. In this case, UE A cannot receive from UE B due to the half-duplex constraint. By using the mode-2 resource selection with potential enhancements, UE A can report a set of preferred resources for reception by UE A, SA, to UE B. On one hand, the hidden-node problem can be avoided since SA can be based on sensing results which cannot be obtained by UE B’s own sensing. On the other hand, the half-duplex problem can be solved since the slots used for UE A’s own transmission have already been excluded from SA.

Scheme 1 with preferred resources transmitted is supported to solve the hidden-node and half-duplex problems.

- The preferred resources at least include the set of candidate resources SA obtained based on mode-2 resource selection with potential enhancements.

Scheme 1 with preferred resources transmitted can be supported at least for unicast where only one RX UE reports the preferred resources. In Section 2.3.1, the PRR gain has been verified by the system-level simulations. For groupcast and broadcast, the transmitted preferred resources are from multiple RX UEs and their intersection may be empty. In this case, it should be further studied how UE B selects resources for transmission.

Scheme 1 with preferred resources transmitted is supported at least for unicast.

As another type of Scheme 1, UE A can send UE B the set of resources not preferred for UE B’s transmission. In our view, sending non-preferred resources is beneficial at least for solving the hidden-node, half-duplex, consecutive-packet-loss and exposed-node problems. In this case, UE A does not have to be RX UE of UE B.

As for solving the hidden-node problem, sending non-preferred resources can achieve a similar effect with that of sending preferred resources since one type can be derived from the other one. In this case, the non-preferred resources are the resources reserved and heavily interfered by other UEs.

In terms of solving the half-duplex problem in Figure 2, sending non-preferred resources can achieve the same effect as that of sending preferred resources but with different reports. Instead of reporting preferred resources, RX UE A can inform TX UE B a set of slots used for UE A’s own transmission. These slots are the resources not preferred for UE B’s transmission.

Figure 3: Consecutive packet loss.

In Figure 3, an example of consecutive packet loss is shown. This can happen even if UE A is not the intended receiver of UE B transmissions. For example, when different TX UEs perform resource selection independently, it is possible that UEs’ transmissions collide with each other on the selected resources. This could be even worse if UEs are transmitting periodic traffic. Due to half-duplex constraints, neither UE A nor UE B can identify the collision and even the re-evaluation or pre-emption specified in NR V2X cannot resolve this issue. Therefore, two UEs could keep colliding potentially for a long time. Scheme 1 with non-preferred resources transmitted can be used to avoid consecutive packet loss. For example, TX UE A can broadcast its TX resources (non-preferred resources) in slots other than those used for its periodic transmissions. By this way, another TX UE (UE B) have opportunities to receive UE-A’s non-preferred resources and thus avoid using those resources.

Figure 4: Exposed-node problem.

The exposed-node problem is illustrated in Figure 4. In Rel-16 NR V2X, if a candidate resource overlaps with another resource indicated by an SCI, and the corresponding RSRP measurement result is higher than a threshold, this resource shall be excluded from the candidate resource set. In some cases, such kind of exclusion is unnecessary. Figure 4 gives an example, where UE B transmits to UE A, and UE C transmits to UE D. If UE C is far from UE A, UE B may not actually be interfered even if UE B and UE C have selected overlapping resources. Then, UE C can be regarded as an exposed node for UE B. Type B can be used to identify the existence of exposed nodes. For example, UE A reports the resources reserved by UE C’s SCI and the measured RSRPs. Then UE B can identify UE C is far from UE A by comparing the RSRP values corresponding to the same reservation SCI. More specifically, the RSRPs reported from UE A can be compared with UE B’s measurements. Alternatively, such an identification can be realized by comparing the geographic locations of UE A and UE C after performing the sensing procedure or acquiring information reported from UE A which includes the geographic location.

Scheme 1 with non-preferred resources transmitted is supported to solve the hidden-node, half-duplex, consecutive-packet-loss, and exposed-node problems.

- The non-preferred resources at least include UE A’s TX resources and the resources reserved by other UEs.

Scheme 1 with non-preferred resources transmitted can be supported for any cast type. At least when UE A indicates its own TX resources in the coordination information, UE B can try to avoid colliding with UE A when performing resource selection, no matter what cast type is currently used by UE B.

Scheme 1 with non-preferred resources transmitted can be supported for any cast type.

The condition for UE to be UE A

For Scheme 1, at least the RX UE of UE B can be UE A. From the perspective of the RX UE, the preferred or non-preferred resources are the resources preferred or non-preferred for its reception. Reporting these resources is beneficial for improving the transmission reliability between TX UE B and RX UE A. Therefore, it should be supported that the RX UE is UE A.

A RX UE can become UE A in a triggering manner. In this case, UE B sends the request to a RX UE, and then the UE becomes UE A and passively reports the coordination information to UE B. Firstly, UE B which has an arrived packet for delivering sends a request to UE A by MAC-CE or SCI, in order to acquire the coordination information. The request may contain the information such as source ID, destination ID, QoS requirement, cast type, packet size, service type, traffic type, geographical information, and communication range associated with the arrived packet. Then, UE A informs the coordination information to UE B by MAC-CE or SCI, in consideration of what UE B exactly requested. The coordination information can be detectable by other non-requesting UEs for reference purposes.

For Scheme 1, it is at least supported that the RX UE of UE B is UE A.

- It is at least supported that UE B triggers a RX UE to send the coordination information.

When the report procedure is triggered by UE B, UE A can determine the resources included in the reported “resource set” based on the RSRP measurement and its comparison with a threshold. More specifically, for a resource, if the corresponding RSRP value measured by UE A is larger/smaller than a threshold, the resource should be included in the reported “resource set” and reported from UE A. To determine the threshold, it can be considered to reuse the priority combination pi, pj , where pi is the priority value received in the SCI format 1-A and  pj is the priority value indicated by the request signaling from UE B.

For Scheme 1, when the reporting is based on triggering or requesting from UE B, UE A determines the reported resources based on the comparison between the corresponding RSRP measurement and the RSRP threshold.

- The indicated priority in the triggering/requesting signaling should be used as prioTX to determine the RSRP threshold.

It can be considered to let a UE with high-priority periodic traffic become UE A. The UE can notify its selected periodic resources to other UEs is slots other than those used for the periodic transmission. By this way, the possibility of having consecutive packet loss can be reduced.  The issue of consecutive packet loss has been illustrated in Figure 3 where TX UE A and TX UE B transmit periodic traffics and collide with each other. Due to half-duplex constraints, neither UE A nor UE B can identify the collision. If UE A can broadcast its TX resources in slots other than those used for its periodic transmissions, UE B can receive UE-A’s notification and thus avoid the persistent collision.

For Scheme 1 with non-preferred resources transmitted, it can be considered that the UE with high-priority periodic traffic is UE A.

- To avoid consecutive packet loss, UE A notifies its own periodic transmission to other UEs in the slots other than those used for its periodic transmission.

The contents of the coordination information

The message associated with a set of resources provided by UE A to UE B may at least contain the information as follows:

- A set of resources in both time and frequency domains: This information can be informed by UE A to UE B, which may either directly select it as transmission resource, or select part of a set for its transmission resource. The acquiring of this information may reduce the burden in the resource selection procedure at TX UE, which is required for power saving, etc.

- Destination ID: UE A informs the coordination information associated with this destination ID to TX UE(s). This implies that only TX UE(s) interested in the destination ID takes the coordination information into consideration. The destination ID may be not necessary if the coordination information is shared by all the UEs.

- Source ID(s): The source ID belongs to TX UE, which is going to potentially utilize the given coordination information if UE A informs the coordination information to a specific TX UE. UE A could include multiple source IDs if the given coordination information is potentially utilized by multiple TX UEs.

- RSRP and priority associated with the set of the resources: Based on sensing, UE A can obtain the information of the reserved resources, the associated RSRPs and the associated priorities (prioRX). With this information, UE B can increase the RSRP threshold to generate more candidate resources when the number of candidate resources is too small. This provides UE B with more freedom to decide whether a reported resource is excluded or not during the resource selection. One example to report RSRP is shown in Figure 5 where RSRPs are quantified with N levels. More specifically, these N levels correspond to maximum N resource exclusions at the UE-B side. If Th1<RSRP≤Th2, the related candidate resource is excluded for the 1st and 2nd exclusions but not excluded for the 3rd and 4th exclusions. By this way, RSRP can be signalled with only a limited number of bits.

Figure 5: The reported RSRP and priority determines when the resource should be excluded.

For Scheme 1, the coordination information provided by UE A to UE B can contain at least the follows: a set of resources, destination ID, source ID, RSRPs and priorities associated with the resources.

The signalling of the coordination information

How to convey the coordination information should be specified. According to the previous analysis, many other information will also be transmitted by UE A together with the set of resources. In our view, either MAC-CE or SCI can be used to carry the coordination information. Similarly, when a request is sent by UE B to trigger UE-A report, it can be carried by either MAC-CE or SCI.

For Scheme 1, either MAC-CE or SCI can be utilized for the coordination information sent by UE A and the request sent by UE B.

Until now, we have not discussed how many resources can/should be reported to UE B. We believe that the more the resources reported, the better inter-UE coordination that can be realized. This, however, results in a higher signalling overhead. Thanks to the default functionality of resource selection in mode-2, UE B is always able to autonomously select the resource if necessary, and thus, UE B does not need to enforce the UE-A to report the resource sets or all the resources which meet the criteria for reporting.

A simple way is to impose a limitation on the number of reported resources; namely, the UE-A should skip to report the resources if the report condition hits the limitation ceiling. To this end, the resources potentially for reporting can be prioritized, whereby UE A could efficiently pick up the higher priority resources and finalize the report accordingly.

For Scheme 1, it should be studied whether/how to limit the number of resources transmitted from UE A to UE B.

UE-A behaviour to determine a set of resources

When UE A determines preferred resources SA by using mode-2 resource selection, the ratio of the preferred resources to the total resources will be larger than X%. Although a typical value for X% is 20%, it would be beneficial if X% at the UE-A side can be adjustable. In this way, the size of the intersection of the resources from UE B and UE A can be adjusted. In some cases, a smaller size (also smaller X%) is preferred to select more clean resources. In other cases, a larger size (also larger X%) is preferred to accommodate more HARQ retransmissions. In Section 2.3.1, the system-level simulation result shows the benefit to have adjustable X%.

For Scheme 1 where UE A signals the preferred resource set SA based on mode-2 resource selection, it should be considered that the ratio of the preferred resources to the total resources X% at UE A is adjustable.

For inter-UE coordination Scheme 1, it can be considered that SINR estimated at UE A (receiver side) is used to determine the set of resources reported to UE B. In conventional mode 2, for the same purpose, RSRP (interference) measured at UE B (transmitter side) is used but SINR is not used. This is because that mode-2 resource selection is determined by the transmitter which does not have the information on the signal part of SINR. Even if the signal part can be estimated based on the transmission power and the pathloss from power control, the applicability will be limited to unicast only since power control is not supported for groupcast or broadcast. As for RSRP-based resource selection, it is not accurate enough and may result in that a resource with low (high) SINR is mistakenly included (excluded). Thanks to the assistance from the receiver, inter-UE coordination provides an opportunity to use SINR in resource selection. More specifically, UE A can determine whether a resource is preferred or not based on the estimated SINR which provides a more accurate estimation. As for the signal part of SINR, it can be estimated based on the transmission from UE B which is already in place for inter-UE coordination. For example, the transmission from UE B would be available if UE B sends signalling to trigger UE A’s reporting, or if UE A performs reporting when identifying a potential conflict on UE B’s transmission. Based on the transmission from UE B, either L1-RSRP as used in mode-2 resource selection or L3-RSRP as used in power control can be used as the signal part of SINR. In Section 2.3.1, the system-level simulation result verified the effectiveness of determining the set of resources based on SINR.

For Scheme 1 where UE A signals a set of resources, it should be considered that the estimated SINR instead of the measured RSRP at UE-A side is used to determine the set of resources.

UE-B behaviour to utilize the coordination information

In the last RAN1#104b meeting, four options have been captured in the agreement w.r.t. the UE-B behaviour to utilize the coordination information. In our view, at least Option 1-1 should be supported where UE B uses both its own sensing results and the coordination information. By using UE B’s own sensing results, UE B can avoid producing too much interference to other UEs. By using the coordination information, e.g., preferred/non-preferred resources, UE B can select more desirable resources for UE A’s reception. Therefore, UE B should balance between causing interference and benefiting reception by using both information.

For Scheme 1, at least the following UE-B behaviour should be supported.

- Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information.

As for the behaviour of UE B, it should be studied how UE B performs resource selection when receiving the coordination information from multiple reporting UEs. For example, given a case where UE B transmits the packets to multiple UEs in a broadcast/groupcast manner by utilizing the sensing results (reserved resources by other UEs) reported from these UEs, it should be considered how to utilize these sensing results during resource selection. Since the different reporting UEs may have the different levels of tolerance for interference, it would be more desirable that the RSRP thresholds used in the resource selection are updated independently for different reporting UEs. One example is shown in Figure 6 where UE B receives coordination information from UE A and UE C. Updating the RSRP threshold potentially means the resources with higher interference can become candidate resources. Considering a “near” RX UE (UE A) can endure higher interference than that of a “far” RX UE (UE C), updating the RSRP thresholds simultaneously may introduce excessive interference to a “far” RX UE. By letting the RSRP thresholds be independently updated, UE B can select more compromised resources which are acceptable for both “near” and “far” UEs.

Figure 6: UE B receives coordination information from UE A and UE C.

For Scheme 1, if UE B receives coordination information from more than one reporting UE A indicating reserved resources by other UEs, the RSRP thresholds used in resource selection should be updated independently for different reporting UEs.

Another behaviour of UE B can be considered is, in both LTE Rel-14 and NR Rel-16 sensing procedure, all the candidate resources within the selection window shall be excluded as long as the corresponding slots have not been monitored. With the service of the periodic traffic implemented by the higher layer parameter sl-ResourceReservePeriodList, for example, the slots on which the TX UE performs transmission cannot be monitored due to the half-duplex issue. In this case, however, too many resources may be excluded if the UE has performed the frequent transmissions within the sensing window, especially when a large number of short period packets (e.g., less than 100ms) are allowed. A feasible method to reduce the excessive exclusion caused by unmonitored slots is, if the resources reported from UE A includes the sensing results in one or more unmonitored slots of UE B, UE B can take these sensing results of UE A into account during its own resource exclusion procedure instead of excluding all the candidate resources within the corresponding slots in the selection window. In our opinion, whether this mechanism can work should depend on the geographical relation between UE A and UE B.

Figure 7 illustrates an example, where UE B finds UE A in proximity, by the geographical location notified from UE A or the RSRP measurement result of UE A, UE B can take into the consideration of the sensing results in the unmonitored slots reported from UE A.

Figure 7: Illustration of UE B take the sensing results reported from UE A of the unmonitored slots into account.

For Scheme 1, UE B can take into the consideration of the sensing results in its unmonitored slots reported from UE A when UE B finds UE A in proximity.

- UE B can find whether UE A is adjacent by the geographical location notified by UE A or the RSRP measurement result of UE A.

The resource set which is not preferred for UE-B transmission can be used to avoid the half-duplex issue. For example, if a UE A can predict its own transmission time in the future, e.g., in the case where the transmission is periodic, it can generate a set of such resources and notify them to other UEs by broadcast. Then, as shown in Figure 8, UE B has packets to be transmitted to the intended receiver UE A, while UE A has reserved two sets of resources; the resource set #1 targeting for UE B and the resource set #2 targeting for UE C. In this case, UE B should avoid using the same slot in the resource set #1 and resource series #2 if reported from UE A, and mitigating the half duplex issue incurred at UE A.

Figure 8: UE A is the destination UE.

However, when UE B’s intended receiver is not UE A, a different behaviour of UE B should be considered. For example, as shown in Figure 9, only the same slot in resource set #1 targeting for UE B should be avoided so as to mitigate the half duplex issue incurred at UE B; Another case is shown in Figure 10, the half-duplex issue no longer occurs in both UE A and UE B, and in this case, then, the over-exclusion should be avoided.

Figure 9: UE A is not the destination UE.

Figure 10: UE A is not the destination UE.

For Scheme 1, the resource set which is not preferred for UE-A reception can be used to avoid the half duplex issue.

- FFS: the details on the behavior of UE B about how to take this kind of resource into account during the resource (re)selection procedure, in consideration of the different intended receiver(s).

Inter-UE coordination Scheme 2

The scenarios where Scheme 2 is used

As for Scheme 2, UE A notifies UE B the presence of the resource conflict. In our view, Scheme 2 can be at least used to solve the collision and half-duplex problems. More specifically, it can be used to assist TX UE B which experiences the collision or half-duplex impact posterior to the initial transmission. By this way, the resource collision and/or half-duplex impact in retransmission and/or next initial transmission can be avoided.

For Scheme 2, UE A can signal retransmission indication to UE B. This is one way to signal the detected conflict to UE B. One use case is shown in Figure 11 and Figure 12. Figure 11 exemplifies the half-duplex problem for groupcast with option-1 based HARQ, where UE A, UE B and UE C are group members. In phase 1, UE B and UE C simultaneously perform initial transmissions over PSSCH1 and PSSCH2 respectively. In this case, UE A successfully receives the TB from UE B but fails the TB reception from UE C. Meanwhile, UE B and UE C face the half-duplex issue due to the transmission in the same slot. In phase 2, UE A only reports NACK over PSFCH2 to UE-C and asks for its retransmission based on the option-1 HARQ process. In phase 3, UE C retransmits the TB over PSSCH2, and UE B and UE A succeed in TB reception. Consequently, UE C has no opportunity to receive the TB from UE B due to the half-duplex constraint.

Figure 11: Half-duplex problem for groupcast with option-1 HARQ.

Scheme 2 with retransmission indication, or with detected conflict indication, can be used to solve the half-duplex problem. As a coordinating UE, UE A needs to satisfy the following conditions:

- UE A is a group member, which enables it to receive groupcast transmissions.

- UE A is able to identify that both TX UEs are transmitting simultaneously in a given slot.

- UE A can transmit ACK/NACK over PSFCH regardless of the status of its own PSSCH reception as long as it can succeed in receiving the corresponding PSCCH.

Figure 12 depicts the details for the TB transmissions and retransmissions with the retransmission indication feedback process, completed with four phases. More specifically, Phase-1 is for the initial TB transmission, Phase-2 is for the NACK feedback(s), Phase-3 is for the TB1 retransmission, and Phase-4 is for the TB2 retransmission.

Figure 12: Type-C inter-UE coordination with inter-UE coordinative HARQ feedback.

More precisely, the procedures for the retransmission indication feedback process can be described as follows.

- UE B and the UE C groupcast the initial TBs in the group in the same slot.

- UE A receives both TBs and decodes PSCCH and then PSSCH in the same slot. By checking both L1 source IDs and L1 destination IDs, and the distance between UE B and UE C based on Zone-IDs of both UEs, UE A determines whether to trigger the retransmission indication feedback process.

- UE A transmits the generated retransmission indication (NACKs) to the corresponding UE(s) over PSFCH(s).

- UE B retransmits the TB if UE B receives NACK from PSFCH1.

- UE C retransmits the TB if UE C receives NACK from PSFCH2.

The retransmitted TB from UE C is received by both UE B and UE A, and UE A decodes the TB if it has also failed in the initial decoding process, otherwise, UE A ignores that TB reception. In Section 2.3.2, the PRR gain has been verified by the system-level simulations.

Scheme 2 with detected conflict indication is supported to solve the half-duplex problem.

- If UE A identifies that the half-duplex problem happens between UE B and UE C, it notifies UE B and UE C via PSFCH to perform retransmissions.

Scheme 2 with detected conflict indication is supported at least for groupcast with HARQ option 1.

Figure 13: Scheme 2 with expected conflict indication.

For Scheme 2, UE A can also signal expected conflict to UE B, or signal re-selection indication to UE B. One use case is shown in Figure 13. UE A identifies that the reserved resources by UE B and UE C will collide with each other in future. This can happen due to the hidden-node problem where UE B cannot receive the SCI transmitted by UE C, and thus cannot avoid the collision by its own sensing. However, UE A can receive from both UE B and UE C thus identifying the potential collision. After identifying the problem, UE A can notify UE B to perform resource re-selection via PSFCH. To distinguish from PSFCH conveying ACK/NACK, separate PSFCH resources can be used to convey the re-selection indication. By reusing the NACK-only mechanism in groupcast with HARQ option 1, re-selection indications from more than one UE A can be superposed on the same PSFCH resource. Also, PSFCH resources can be configured for all the cast types to convey the re-selection indication.

Scheme 2 with expected conflict indication is supported to solve the hidden-node problem.

- If UE A identifies that the resources reserved by UE B and UE C will collide with each other, it notifies UE B via PSFCH to perform resource re-selection.

Scheme 2 with expected conflict indication can be supported for any cast type.

The condition for UE to be UE A

- For Scheme 2 with either retransmission indication or re-selection indication transmitted, at least the RX UE of UE B can be UE A, and thus transmits the retransmission indication or re-selection indication. As an example, for groupcast with HARQ option 1, groupcast RX UE can identify that UE B and UE C simultaneously transmit to the same group by identifying the group destination ID in the 2nd stage SCI. In other words, RX UE can know that the half-duplex problem happens to UE B and UE C. Therefore, RX UE can be UE A and transmits the retransmission indications to UE B and UE C respectively. As another example, based on sensing, RX UE can know that its reception from UE B will be interfered by UE C in future. Then RX UE can be UE A and transmits the re-selection indication to UE B such that UE B can avoid the collision.

For Scheme 2, it is at least supported that the RX UE of UE B is UE A.

- For Scheme 2 with expected conflict indication, only under some conditions, a non-RX UE of UE B can be UE A, and transmits the re-selection indication to UE B. It is necessary to have some restrictions to guarantee that the observed collision will actually affect the reception. Consider a case where a UE is not the receiver of UE B or UE C. Even if the UE has observed the potential collision between UE B and UE C, the collision may not result in any decoding failure if the receivers of UE B and UE C are far from each other. This is because that the estimation at the UE does not accurately reflect the interference experienced by the receiver. In this case, the UE should not be UE A since there is no need to do any resource re-selection. However, if the UE is not the receiver of UE B but is the receiver of UE C, then the observed collision will have a real impact on the reception. In this case, the UE can be UE A and transmits the re-selection indication to UE B. Therefore, a non-RX UE of UE B can transmit re-selection indication to UE B only if some conditions are met.

For Scheme 2 with expected conflict indication, a non-RX UE of UE B can be UE A if the following conditions are satisfied.

- The UE has identified the potential collision between UE B and UE C.

- The UE is not the RX UE of UE B, but is the RX UE of UE C.

The signalling of the coordination information

- As aforementioned, PSFCH can be used to convey retransmission indication or re-selection indication. For PSFCH conveying ACK/NACK, Rel-16 V2X has defined the priority of PSFCH. The purpose is to determine the prioritization rule when a UE needs to simultaneously transmit (or simultaneously transmit and receive) PSFCH and UL, PSFCH and PSFCH etc. If PSFCH is used to convey the coordination information, the priority of PSFCH should also be specified to handle the related prioritization.

- For Scheme 2 with expected conflict indication, the priority of PSFCH can be determined as the highest priority of the PSSCHs having the conflict. This can be illustrated by using Figure 13. It is assumed that PSFCH is transmitted to lower-priority UE B in order to avoid the conflict between UE B and UE C. Conceptually, this is similar with pre-emption in the sense that lower-priority UE B performs re-selection to protect UE C which has a higher priority. Since the coordination information is for guaranteeing the successful transmission of UE C, the priority of the coordination information should follow the priority of UE C’s PSSCH.

- For Scheme 2 with detected conflict indication, the coordination information is for triggering re-transmission and thus is utilized as if it was NACK. In this sense, the priority of the coordination information should be determined in the same way as that of NACK.

For Scheme 2, it should be supported that PSFCH is used to convey the coordination information.

For Scheme 2 with expected conflict indication, the priority of the coordination information is the highest priority of the PSSCHs having the expected conflict. For Scheme 2 with detected conflict indication, the priority of the coordination information is the priority of PSFCH conveying NACK.

UE-A behaviour to transmit the coordination information

Figure 14: ACK transmitted before coordination information.

For Scheme 2 with expected conflict indication, generally UE A transmits the coordination information to UE B when identifying the future conflict. However, if UE A has reported ACK to UE B before, it can skip transmitting the coordination information. Even if PSFCH is used to convey the coordination information, the timing of the coordination information can be different from that of PSFCH conveying HARQ-ACK. This is because that HARQ-ACK timing is determined w.r.t. the prior-transmission, but the timing of the coordination information can be determined w.r.t. the reserved resource in future. This is illustrated in Figure 14 where the PSFCH slot conveying the coordination information is immediately before the pre-emption checking slot m-T3, and the PSFCH slot conveying ACK is the first PSFCH slot after UE B’s “blue” transmission. Therefore, it is possible that UE A transmits ACK before transmitting the coordination information to UE B. Since the ACKed UE B will not perform re-transmission, UE A does not need to transmit the coordination information to indicate the conflict of the re-transmission. However, if ACK is dropped due to prioritization, UE B will still perform retransmission. In this case, UE A needs to transmit the coordination information.

For Scheme 2 with expected conflict indication, once identifying the expected conflict, UE A transmits the coordination information to UE B, except when UE A has transmitted ACK to UE B for the same TB before.

For Scheme 2 with expected conflict indication, it would be beneficial for UE B to know whether the coordination information is from its intended receivers or not. For example, it can resolve the ambiguity which occurrs when UE B receives both ACK and the coordination information in groupcast with HARQ option 1. More specifically, consider the following two cases for groupcast with HARQ option 1.

- Case 1: A group member does not transmit NACK due to prioritization and transmits the coordination information. The other group members indicate ACK and do not transmit the coordination information.

- Case 2: All the group members indicate ACK and do not transmit the coordination information. Some UEs other than the group members transmit the coordination information.

In Case 1, the group member who drops NACK will transmit the coordination information. In Case 2, the UEs other than the group members transmit the coordination information. Ideally, UE B should perform resource re-selection for Case 1, but not perform resource re-selection for Case 2. However, in both cases, UE B will receive ACK and the coordination information. With the same observations, UE B cannot distinguish these two cases. To resolve this ambiguity, separate PSFCH resources can be allocated to RX UE (intended receiver) and non-RX UE. If UE A is the intended receiver of UE B, UE A transmits the coordination information on the 1st PSFCH resource. Otherwise, UE A transmits the coordination information on the 2nd PSFCH resource. By this way, UE B can know whether the coordination information is from the intended receivers or not, and thus distinguish Case 1 and Case 2. If UE B receives the coordination information from its intended receivers (Case 1), it will perform resource re-selection regardless of ACK/NACK. Otherwise, if UE B receives the coordination information from UEs other than its intended receivers and ACK (Case 2), it will not perform resource re-selection.

For Scheme 2 with expected conflict indication, if UE A is the intended receiver of UE B, UE A transmits the coordination information on the 1st PSFCH resource; Otherwise, UE A transmits the coordination information on the 2nd PSFCH resource.

Figure 15: No conflict between UE B and ACKed UE C.

For Scheme 2 with expected conflict indication, it is not desirable to trigger too many re-selections since re-selection may also lead to conflict. At least under the following circumstances, UE A does not consider the overlap of the reserved resources as an expected conflict and thus not transmitting the coordination information to UE B. An example is shown in Figure 15. If UE A has sent ACK to UE C, it will consider the resource reserved by UE C has been released and thus will not conflict with the resource reserved by UE B. In this case, UE A does not need to transmit the coordination information to UE B. As another example, UE A may not transmit the coordination information to UE B if the overlapping part between UE B and UE C is relatively small in terms of the time-frequency resource size.

For Scheme 2 with expected conflict indication, even if the overlapping of the reserved resources by UE B and UE C is identified, UE A does not consider it as an expected conflict at least when

- UE A has transmitted ACK to UE C, or

- The size of the overlapping part is smaller than a certain threshold.

For Scheme 2 with expected conflict indication, when identifying an expected collision between UE B and UE C, UE A determines whether UE B or UE C the coordination information is transmitted to. Furthermore, UE A should follow a pre-defined rule to avoid triggering re-selection at both UE B and UE C sides. For example, UE A transmits the coordination information to the UE with the lower priority. This is aligned with the principle of pre-emption where the low-priority UE performs re-selection to avoid interfering with the high-priority UE. Especially, if the priorities of UE B and UE C are the same, a tie-breaking rule is also needed to prevent that some UEs transmit the coordination information to UE B and some other UEs transmit the coordination to UE C.

For Scheme 2 with expected conflict indication, UE A follows a pre-defined rule to determine whether UE B or UE C the coordination information is transmitted to.

UE-B behaviour to utilize the coordination information

In the last RAN1#104b meeting, two options have been captured in the agreement w.r.t. the UE-B behaviour to utilize the coordination information. In our view, both Option 2-1 and Option 2-2 should be supported. In Section 2.2.1, we have shown the usage scenarios for both options. For Option 2-1, UE B can determine to perform resource re-selection to avoid potential collisions in future. For Option 2-2, UE B can determine to perform retransmission to avoid the existing half-duplex problem. In our view, these two types of UE-B behaviours for inter-UE coordination do not conflict with each other and thus can be both supported.

For Scheme 2, the following UE-B behaviours should be supported.

- Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information.

- Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information.

Figure 16: Re-selection before the timing of coordination information.

For Scheme 2 with expected conflict indication, UE B should avoid receiving the unnecessary coordination information, or avoid performing unnecessary re-selection. An example is shown in Figure 16. From UE B perspective, the PSFCH slots t1 and t2 are the opportunities for receiving coordination information 1 and coordination information 2 which indicate the expected conflicts for resource 1 and resource 2 respectively. UE B performs pre-emption checking in slot m1-T3 for resource 1 and resource 2. Then UE B identifies that resource 1 and resource 2 are pre-empted and should be re-selected. The re-selection will be done before slot m1 and thus being also ahead of PSFCH slot t2. Since resource 2 has already been re-selected, there is no need to receive coordination information 2 for resource 2, or to perform re-selection for resource 2 when receiving coordination information 2. Therefore, UE-B shall not receive coordination information 2 or does not perform re-selection based on the received coordination information 2 if it has determined to perform re-selection for resource 2.

For Scheme 2 with expected conflict indication, UE-B shall not receive the coordination information or does not perform re-selection when receiving the coordination information if it has determined to perform re-selection for the reserved resource associated with the coordination information.

Combination of Scheme 1 and Scheme 2

- For Scheme 1, the advantage is that UE A would clearly report its "preferred" or "non-preferred" resources, and then UE B can take this coordination information into account during the resource selection procedure, and preferentially select "preferred" or exclude "non-preferred" resources. Based on this, the selected resources have high reliabilities and have low probabilities to collide with the resources selected by other UEs. However, frequently using Scheme 1 for reporting or updating the coordination information may result in high system overhead. The advantage of Scheme 2 is that the number of bits required for this type of information is relatively small, and it will not cause excessive overhead. However, since it simply instructs UE B to do resource reselection, the re-selected resources of UE B may still overlap with other UEs’ reserved resources. In this respect, Scheme 2 is not as reliable as Scheme 1.

- Regarding the pros and cons of Scheme 1 and Scheme 2, a feasible method is to combine Scheme 1 and Scheme 2 as a joint solution. This is illustrated in Figure 17. When UE B performs initial resource selection, UE A is triggered to report the "preferred" or "non-preferred" resource set by using Scheme 1. After that, if UE A detects that UE B’s reserved resources collide with other UEs’ reserved resources, UE A can use Scheme 2 to send a re-selection indication to UE B. Then UE B can trigger the resource re-selection for the collided resources. This can be defined as a special UE B pre-emption detection procedure with additional cooperation from UE A. In this way, system reliability can be guaranteed without introducing too much overhead.

Figure 17 A joint working mechanism of Scheme 1 and Scheme 2

Considering the pros and cons of Scheme 1 and Scheme 2, a joint working mechanism can be supported for inter-UE coordination.

Evaluation results

Scheme 1 with preferred resources transmitted

A system-level simulation is performed to evaluate Scheme 1 with preferred resources transmitted. In the simulation, unicast with periodic traffic is simulated for highway and urban scenarios. For inter-UE coordination, RX UE A sends TX UE B an assistance table with the size of N×M, where N is the length of the selection window in slots (e.g., N=20 slots), and M is the number of candidate resources in the frequency domain (e.g., M=4 for sub-channel size = 15 RBs with 20MHz). Each element in the table simply indicates whether it is the preferred candidate or not with a single bit (0 or 1). The intersection of the resources recommended by UE A and pre-selected by UE B is used for UE-B transmission. Other simulation assumptions are listed in Table 1 in Annex.

The following three schemes S1, S2 and S3 are evaluated.

- S1 without inter-UE coordination (100 RBs)

- S2 without inter-UE coordination (90 RBs)

- S3 with inter-UE coordination (90 RBs)

For S1, one sub-channel includes 25 RBs, resulting in 4 sub-channels in the 20MHz bandwidth. UE B uses 2 sub-channels for each TB transmission. Totally 100 RBs can be used for PSSCH transmission.

For S3, one sub-channel includes 15 RBs, resulting in 6 sub-channels in the 20MHz bandwidth. UE B uses 3 sub-channels for each TB transmission. Only 90 RBs out of 100 RBs can be used for PSSCH transmission. The remaining 10 RBs are assumed as the permanent overhead of the coordination information. In this case, the number of candidates possible for TB transmission becomes 4 in each slot, requiring the assistance table with the size of 20×4=80. It is assumed that 10 RBs are dedicated for the transmission of two assistance tables in each slot. This further results in the code rate of 0.074, derived by the transmission parameters for the assistance table, such as 5 RBs, QPSK.

For S2, the sub-channel allocation is the same as that of S3.

Figure 18: PRR performance of Scheme 1 with preferred resources transmitted for highway.

Figure 19: PRR performance of Scheme 1 with preferred resources transmitted for urban.

The effectiveness of inter-UE coordination with signaling overhead can be verified by comparing S1 with S3. As shown in Figure 18 and Figure 19, S3 outperforms S1 when the distance becomes relatively large. For example, about 2% PRR gain can be achieved at 320m for highway and at 150m for urban. These results have verified the effectiveness of inter-UE coordination in the realistic scenario where the signaling overhead is taken into account. Actually, the overhead here is assumed to be relatively large and is dedicated for the use of the coordination information. However, even in this case, the PRR gain can still be observed for unicast. From our simulations, it seems that inter-UE coordination is more meaningful for unicast. In the contribution [2] for RAN1#104, we show a simulation result for inter-UE coordination in groupcast where only one randomly selected group member reports the coordination information. When the signaling overhead is considered, the gain of inter-UE coordination is not observed for groupcast. Compared with that, Figure 18 and Figure 19 show the positive results for unicast even if the overhead is taken into account. The effectiveness of inter-UE coordination without signaling overhead can be verified by comparing S2 with S3. In this ideal case, inter-UE coordination can achieve a higher PRR gain.

By using Scheme 1 for unicast, i.e., letting UE A report preferred RX resources, about 2% PRR gain is achieved at 300m for highway and at 100m for urban with the signalling overhead taken into account.

Figure 20: PRR performance of Scheme 1 with different X% at UE A for highway.

Figure 21: PRR performance of Scheme 1 with different X% at UE A for urban.

- In Section 2.1.5, it is proposed to have adjustable X% which is the ratio of the preferred resources to the total resources. The system-level simulation is performed to verify the improvement on PRR performance. For example, Figure 20 and Figure 21 provide the simulation results where X% at UE B is fixed to 20% and X% at UE A varies from 10% to 30%. It is assumed that the maximum number of HARQ (re)transmission is 2. Other simulation assumptions are listed in Table 1 in Annex. In this case, it can be observed that the scheme with X=10% has the best PRR performance for the larger distance. More specifically, 2% PRR gain is achieved at 300m for highway and 1% PRR gain is achieved at 100m for urban when comparing X=10% and X=20%. Therefore, it is beneficial to have some freedom to adjust X% at UE A.

By using Scheme 1 for unicast where UE A signals the preferred resources and the ratio of the preferred resources to the total resources X% is adjustable, 2% PRR gain is achieved at 300m for highway and 1% PRR gain is achieved at 100m for urban when comparing X=10% and X=20%.

Figure 22: PRR performance of Scheme 1 with SINR-based resource determination for highway.

Figure 23: PRR performance of Scheme 1 with SINR-based resource determination for urban.

- In Section 2.1.5, it is proposed that SINR is used to determine the set of resources. In this section, the system-level simulations are performed to verify the potential benefit. The simulation assumptions are listed in Table 1 in Annex. In Figure 22 and Figure 23, the PRR performances are compared between the conventional RSRP-based method A1 and the proposed SINR-based method A2. For A1, UE A determines the set of resources SA in the same way as that of mode 2. For A2, the set of resources is further refined on top of A1 by excluding any resource with SINR smaller than a threshold. With the distance increased, the proposed method A2 tends to have the better PRR performance. For example, about 1.5% PRR gain is observed at 300m for highway, and about 0.5% PRR gain is observed at 100m for urban.

By using Scheme 1 for unicast where SINR is used by UE A to determine the set of resources, about 1.5% PRR gain is achieved at 300m for highway and about 0.5% PRR gain is achieved at 100m for urban.

Scheme 2 with detected conflict (half-duplex) indication

In order to clarify how much impact occurs due to the half-duplex constraint, we perform the system level simulation based on the simulation assumptions, listed in Table 2, in Annex. Four scenarios are taken into account:

- Scenario-1: Periodic traffic in highway.

- Scenario-2: Periodic traffic in urban.

- Scenario-3: Aperiodic traffic in highway.

- Scenario-4: Aperiodic traffic in urban.

Figure 24, Figure 25, Figure 26, and Figure 27 show the PRR as a function of the distance between TX UE and RX UE, with 100m communication range, in consideration of groupcast with option-1 HARQ, for the scenario-1, 2, 3 and 4, respectively. It can be observed that, in any scenario, the PRR for inter-UE coordination Scheme 2 with detected indication (HARQ assist) always outperforms that without inter-UE coordination. Consequently, the PRR degradation due to the half-duplex issue is about 1~2% in groupcast, depending on the how many TX UEs simultaneously transmit TBs in a slot. It is observed that, the larger the number of TX UEs in a slot, the bigger the impact of half-duplex. This implies that, the half-duplex impact could fatally block the realization of stringent PRR reliability in groupcast with option-1 HARQ.

Figure 24: PRR vs. distance for periodic traffic in highway and 100m communication range in scenario-1.

Figure 25: PRR vs. distance for periodic traffic in urban and 100m communication range in scenario-2.

Figure 26: PRR vs. distance for aperiodic traffic in highway and 100m communication range in scenario-3.

Figure 27: PRR vs. distance for aperiodic traffic in urban and 100m communication range in scenario-4.

By using Scheme 2 for groupcast with HARQ option 1, i.e., letting UE A notify UE B via PSFCH to perform retransmission when identifying half-duplex, about 1~2% PRR gain is achieved with the signalling overhead and latency taken into account.

Conclusions

In this contribution, we have expressed our views on potential solutions towards inter-UE coordination. In summary, we have the following list of the observations and the proposals:

- By using Scheme 1 for unicast, i.e., letting UE A report preferred RX resources, about 2% PRR gain is achieved at 300m for highway and at 100m for urban with the signalling overhead taken into account.

By using Scheme 1 for unicast where UE A signals the preferred resources and the ratio of the preferred resources to the total resources X% is adjustable, 2% PRR gain is achieved at 300m for highway and 1% PRR gain is achieved at 100m for urban when comparing X=10% and X=20%.

By using Scheme 1 for unicast where SINR is used by UE A to determine the set of resources, about 1.5% PRR gain is achieved at 300m for highway and about 0.5% PRR gain is achieved at 100m for urban.

By using Scheme 2 for groupcast with HARQ option 1, i.e., letting UE A notify UE B via PSFCH to perform retransmission when identifying half-duplex, about 1~2% PRR gain is achieved with the signalling overhead and latency taken into account.

- Scheme 1 with preferred resources transmitted is supported to solve the hidden-node and half-duplex problems.

- The preferred resources at least include the set of candidate resources SA obtained based on mode-2 resource selection with potential enhancements.

Scheme 1 with preferred resources transmitted is supported at least for unicast.

Scheme 1 with non-preferred resources transmitted is supported to solve the hidden-node, half-duplex, consecutive-packet-loss, and exposed-node problems.

- The non-preferred resources at least include UE A’s TX resources and the resources reserved by other UEs.

Scheme 1 with non-preferred resources transmitted can be supported for any cast type.

For Scheme 1, it is at least supported that the RX UE of UE B is UE A.

- It is at least supported that UE B triggers a RX UE to send the coordination information.

For Scheme 1, when the reporting is based on triggering or requesting from UE B, UE A determines the reported resources based on the comparison between the corresponding RSRP measurement and the RSRP threshold.

- The indicated priority in the triggering/requesting signaling should be used as prioTX to determine the RSRP threshold.

For Scheme 1 with non-preferred resources transmitted, it can be considered that the UE with high-priority periodic traffic is UE A.

- To avoid consecutive packet loss, UE A notifies its own periodic transmission to other UEs in the slots other than those used for its periodic transmission.

For Scheme 1, the coordination information provided by UE A to UE B can contain at least the follows: a set of resources, destination ID, source ID, RSRPs and priorities associated with the resources.

For Scheme 1, either MAC-CE or SCI can be utilized for the coordination information sent by UE A and the request sent by UE B.

For Scheme 1, it should be studied whether/how to limit the number of resources transmitted from UE A to UE B.

For Scheme 1 where UE A signals the preferred resource set SA based on mode-2 resource selection, it should be considered that the ratio of the preferred resources to the total resources X% at UE A is adjustable.

For Scheme 1 where UE A signals a set of resources, it should be considered that the estimated SINR instead of the measured RSRP at UE-A side is used to determine the set of resources.

For Scheme 1, at least the following UE-B behaviour should be supported.

- Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information.

For Scheme 1, if UE B receives coordination information from more than one reporting UE A indicating reserved resources by other UEs, the RSRP thresholds used in resource selection should be updated independently for different reporting UEs.

For Scheme 1, UE B can take into the consideration of the sensing results in its unmonitored slots reported from UE A when UE B finds UE A in proximity.

- UE B can find whether UE A is adjacent by the geographical location notified by UE A or the RSRP measurement result of UE A.

For Scheme 1, the resource set which is not preferred for UE-A reception can be used to avoid the half duplex issue.

- FFS: the details on the behavior of UE B about how to take this kind of resource into account during the resource (re)selection procedure, in consideration of the different intended receiver(s).

Scheme 2 with detected conflict indication is supported to solve the half-duplex problem.

- If UE A identifies that the half-duplex problem happens between UE B and UE C, it notifies UE B and UE C via PSFCH to perform retransmissions.

Scheme 2 with detected conflict indication is supported at least for groupcast with HARQ option 1.

Scheme 2 with expected conflict indication is supported to solve the hidden-node problem.

- If UE A identifies that the resources reserved by UE B and UE C will collide with each other, it notifies UE B via PSFCH to perform resource re-selection.

Scheme 2 with expected conflict indication can be supported for any cast type.

For Scheme 2, it is at least supported that the RX UE of UE B is UE A.

For Scheme 2 with expected conflict indication, a non-RX UE of UE B can be UE A if the following conditions are satisfied.

- The UE has identified the potential collision between UE B and UE C.

- The UE is not the RX UE of UE B, but is the RX UE of UE C.

For Scheme 2, it should be supported that PSFCH is used to convey the coordination information.

For Scheme 2 with expected conflict indication, the priority of the coordination information is the highest priority of the PSSCHs having the expected conflict. For Scheme 2 with detected conflict indication, the priority of the coordination information is the priority of PSFCH conveying NACK.

For Scheme 2 with expected conflict indication, once identifying the expected conflict, UE A transmits the coordination information to UE B, except when UE A has transmitted ACK to UE B for the same TB before.

For Scheme 2 with expected conflict indication, if UE A is the intended receiver of UE B, UE A transmits the coordination information on the 1st PSFCH resource; Otherwise, UE A transmits the coordination information on the 2nd PSFCH resource.

For Scheme 2 with expected conflict indication, even if the overlapping of the reserved resources by UE B and UE C is identified, UE A does not consider it as an expected conflict at least when

- UE A has transmitted ACK to UE C, or

- The size of the overlapping part is smaller than a certain threshold.

For Scheme 2 with expected conflict indication, UE A follows a pre-defined rule to determine whether UE B or UE C the coordination information is transmitted to.

For Scheme 2, the following UE-B behaviours should be supported.

- Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information.

- Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information.

For Scheme 2 with expected conflict indication, UE-B shall not receive the coordination information or does not perform re-selection when receiving the coordination information if it has determined to perform re-selection for the reserved resource associated with the coordination information.

Considering the pros and cons of Scheme 1 and Scheme 2, a joint working mechanism can be supported for inter-UE coordination.

References

- 3GPP TSG RAN1#104b, RAN1 Chair’s Notes, April 12th – 20th, 2021.

- 3GPP TSG RAN1#104, R1-2100746, Considerations on Inter-UE Coordination for Mode 2 Enhancements, Fujitsu, January 25th – February 5th, 2021.

Annex

The system level simulation assumptions are listed in Table 1 and Table 2.

Table 1: SLS simulation assumptions for unicast.

| Attributes | Values or Assumptions |
| --- | --- |
| Scenario | Base on cases of highway and urban in TR 37.885 |
| Speed of vehicle | 140km/h and 60km/h for highway and urban |
| Carrier frequency | 5.9[GHz] |
| Bandwidth | 20[MHz] (100RBs, 1200subcarriers) |
| Subcarrier spacing | 15[kHz] |
| Slot length | 1[ms] (14symbols) |
| Transmission power | 23[dBm] |
| TX Antenna Configuration | 1 antenna |
| RX Configuration | 4 antennas with λ/2 spacing |
| Antenna pattern | Omnidirectional |
| Antenna height | 1.6 [m] (option A, type 2) |
| Antenna gain | 3 [dBi] |
| Noise figure | 9 [dB] |
| Number of DMRS | 4 |
| Size of sub-channel | 25RB, 15RB |
| Modulation and Code rate Error curve type of PSCCH | QPSK, Polar coding |
| Modulation and Code rate Error curve type of PSSCH | 16QAM, LDPC 800byte: 50RB, 45RB 1200byte: 50RB, 45RB |
| Traffic mode | Periodic traffic: Model 2 (Medium traffic intensity) (Inter-packet arrival time: 50ms) |
| Threshold for excluding SCI decoded resources | -128[dBm] |
| Repetition | Chase combining with the same number of sub-channels as initial Tx |

Table 2: SLS simulation assumptions for groupcast.

| Attributes | Values or Assumptions |
| --- | --- |
| Scenario | Base on cases of highway and urban in TR 37.885 |
| Speed of vehicle | 140km/h and 60km/h for highway and urban |
| Carrier frequency | 5.9[GHz] |
| Bandwidth | 40[MHz] (200RBs, 2400subcarriers) |
| Subcarrier spacing | 15[kHz] |
| Slot length | 1[ms] (14symbols) |
| Transmission power | 23[dBm] |
| TX Antenna Configuration | 1 antenna |
| RX Configuration | 4 antennas with λ/2 spacing |
| Antenna pattern | Omnidirectional |
| Antenna height | 1.6 [m] (option A, type 2) |
| Antenna gain | 3 [dBi] |
| Noise figure | 9 [dB] |
| Number of DMRS | 4 |
| Size of sub-channel | 50RB |
| Modulation and Code rate Error curve type of PSCCH | QPSK, Polar coding |
| Modulation and Code rate Error curve type of PSSCH | 16QAM, LDPC 800byte: 50RB 1200byte: 50RB |
| Traffic mode | Periodic traffic: Model 2 (Medium traffic intensity) (Inter-packet arrival time: 10ms) Aperiodic traffic: Model 1 (Medium traffic intensity) (Inter-packet arrival time: 50ms + random) |
| Threshold for excluding SCI decoded resources | -128[dBm] |
| Repetition | Chase combining with the same number of sub-channels as initial Tx |