3GPP TSG RAN WG1 #106-e	                                       	  R1-2107185

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.3.1.2

Source:	Lenovo, Motorola Mobility

Title:	CSI feedback enhancements for URLLC/IIoT

Document for:	Discussion

# Introduction

This contribution expresses our views on CSI feedback enhancements as part of the objectives of the related Work Item [1]:

- CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

Based on the RAN plenary guidance [2] the work is focused as follows:

- Revised Recommendation1: Provide the following RAN guidance on CSI feedback enhancement [RAN1]

  - Focus subsequent working group discussions on the schemes proposed in RP-211297.

    - Details (e.g. how to calculate delta-MCS) are up to further working group discussions.

From RP-211297 [3]:

RAN1 to further investigate the following for CSI enhancements for IIoT/URLLC:

- Increasing the number of bits used for the reported subband CQI (3-bits differential subband CQI or 4-bits CQI)

- Reporting of delta-MCS:

  - Report consists of delta-MCS for a TB received with MCS index IMCS:

    - delta-MCS is calculated from the difference between IMCS_tgt and IMCS, where IMCS_tgt is the largest MCS index such that the estimated BLER for a TB received with this MCS index would be smaller than or equal to a BLER target, and IMCS is the MCS index of the received TB.

In this contribution, we provide our views on the focused schemes.

# Discussion

## Increasing number of subband CQI bits

This scheme (having low anticipated specification impact especially for 4-bit absolute CQI) provides more accurate information for the network to get statistics of the CQI distribution at the cost of significant increase in reporting overhead. The simulation results provided in the last meeting mostly do not show noticeable gains in terms of improving number of satisfied UEs/RU, and hence further evaluations are needed.

## Delta-MCS Reporting

### When Delta-MCS Reporting is useful? Initial Transmission or Retransmission

From the simulation results provided in the last meeting, it seems that the gain of such reporting is realized mostly for retransmissions [4] and not much for initial transmissions [5], at least without significant increase in RU [6].

Besides, if the scheduled resources are very different in size and/or location between two consecutive PDSCHs, it is not clear how much benefit a delta-MCS report provides.

### Specification Impacts

To support such a delta-MCS scheme, it seems that the following aspects are needed:

#### Determining target BLER

The following options listed in RAN1#105e’s FL summary [7]:

- Solution 1: Semi-static configuration: Ericsson [3], Sony [14] (per SPS config)

  - Using values from configured CQI tables (1e-1 or 1e-5) not flexible enough for gNB [3]

- Solution 2: Dynamically from RNTI of the DL assignment: Sony [14]

- Solution 3: Same BLER as previous CQI report: Huawei [4] (“option 1”)

- Solution 4: Tied to the MCS table used for PDSCH: Samsung [16]

We refer to the above schemes as solutions 1-4. In our view, using a BLER target associated with the MCS table corresponding to the received TB (Solution 4) is a simple and reasonable choice. For solution 4, at least two RRC indications for target BLER determination might be needed if the UE can be scheduled with PDSCHs with different MCS tables.

Proposal 1: The BLER target for case 2 reporting is determined based on the MCS table corresponding to the received TB.

#### Whether to report for every PDSCH

The following options listed in RAN1#105e’s FL summary [7]:

-	High-priority codebook [3][19], SPS [3], HARQ process [19], configured TBS [19], MCS threshold [19]

-	Multiple PDSCH to one delta-MCS/CQI to reduce overhead [7]

-	May use time window [8]

-	Per-CC reporting [8]

-	Dynamically indicated from RNTI of DL assignment [14]

-	May take an average/filter from multiple PDSCHs [14], [16] (type 1 codebook)

-	gNB indicate number of TBs for which UE provides delta-MCS value [16]

-	Should not report for every ACK/NACK position in codebook [21]

In our view, associating multiple PDSCHs to one delta-MCS (e.g., via averaging) could potentially reduce MCS accuracy, which is not well aligned with the objective of the work item. Mechanisms to skip delta-MCS reporting for some PDSCHs can be further discussed. For instance, the UE can report delta_MCS if the number of PDSCH REs in the PDSCH transmission (such as ones not cancelled or not indicated by interruption indication) is large enough.

Also, based on the granularity of the report/MCS step size in the report, there could be cases where even if delta_MCS is reported, it doesn’t make the intended difference. For instance, if the scheduled MCS is quite large and if to achieve the target BLER, a small MCS is needed, in case the delta_MCS cannot report the correct MCS difference to achieve a reasonably lower BLER than that of the currently scheduled PDSCH, the report may not be useful and can be skipped. However, we expect this situation to be a rare case (deep fade/severe instantaneous interference), for which delta_MCS procedure optimization might not be needed.

Observation 1: Associating multiple PDSCHs to one delta-MCS (e.g., via averaging) could potentially reduce MCS accuracy.

Proposal 2: Further discuss mechanisms to skip delta-MCS reporting for some PDSCHs.

Proposal 3: The UE can report delta_MCS if the number of PDSCH REs in the PDSCH transmission (such as ones not cancelled or not indicated by interruption indication) is large enough.

#### Reporting resource

The following options listed in RAN1#105e’s FL summary [7]:

-	Within updated HARQ-ACK codebook: Ericsson [3], Spreadtrum [7], (CATT [8]), Apple [13], Samsung [16]

o	No need to send earlier than HARQ-ACK: Spreadtrum [7]

o	Type 2 codebook only (too much information otherwise in Type 1): Ericsson [3]

o	Avoid modification to R16 HARQ-ACK codebook construction [16]

-	Outside HARQ-ACK codebook: Ericsson [3], (Huawei [4]), (Spreadtrum [7]) (CATT [8]), LG [17], Nokia [19] (?)

o	May require less resources (control when and how often to report) [3]

o	Reuse current CSI framework [17]

o	On PUCCH only: Ericsson [3], Huawei [4]

o	Requires mapping to a reference PDSCH: Ericsson [3], LG [17]

o	On semi-statically configured resource: (CATT [8])

In our view, if multiple PDSCHs are not associated to a delta_MCS report (e.g., a single bit delta_MCS), the delta_MCS can be reported together with HARQ-ACK at least for codebook type 2.

We note that determining a reference PDSCH for sending delta_MCS report outside of HARQ-ACK codebook could complicate the specifications, e.g., when the UE misses the DCI scheduling the supposedly ‘reference’ PDSCH.

Proposal 4: For no delta_MCS bundling case, the delta_MCS can be reported together with HARQ-ACK at least for codebook type 2.

Observation 2: Determining a reference PDSCH for sending delta_MCS report outside of HARQ-ACK codebook could complicate the specifications, e.g., when the UE misses the DCI scheduling the supposedly ‘reference’ PDSCH.

#### UCI multiplexing in PUSCH

If HARQ-ACK and delta_MCS are multiplexed in a PUSCH, whether the HARQ-ACK and delta_MCS are to be jointly encoded or separately encoded can be further discussed.

Proposal 5: Further discuss joint encoding vs. separate encoding of HARQ-ACK and delta_MCS when multiplexed in a PUSCH.

# Conclusions

This contribution provided our views regarding CSI feedback enhancements for URLLC as follows:

Proposal 1: The BLER target for case 2 reporting is determined based on the MCS table corresponding to the received TB.

Proposal 2: Further discuss mechanisms to skip delta-MCS reporting for some PDSCHs.

Proposal 3: The UE can report delta_MCS if the number of PDSCH REs in the PDSCH transmission (such as ones not cancelled or not indicated by interruption indication) is large enough.

Proposal 4: For no delta_MCS bundling case, the delta_MCS can be reported together with HARQ-ACK at least for codebook type 2.

Proposal 5: Further discuss joint encoding vs. separate encoding of HARQ-ACK and delta_MCS when multiplexed in a PUSCH.

Observation 1: Associating multiple PDSCHs to one delta-MCS (e.g., via averaging) could potentially reduce MCS accuracy.

Observation 2: Determining a reference PDSCH for sending delta_MCS report outside of HARQ-ACK codebook could complicate the specifications, e.g., when the UE misses the DCI scheduling the supposedly ‘reference’ PDSCH.

References

[1] RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, RAN 88e.

[2] RP-211569, “Moderator's summary for email discussion [92-e-15-IIoT-URLLC-Scope]”, RAN 92e.

[3] RP-211297, “Way forward on CSI feedback enhancements for enhanced URLLC/IIoT”, RAN1 105-e, InterDigital Inc., Ericsson, Motorola Mobility, OPPO, Qualcomm, Samsung, SONY, Spreadtrum.

[4] R1-2104664, “CSI enhancement for IOT and URLLC”, Qualcomm

[5] R1-2105958, “Selection of enhanced CSI feedback schemes”, Intel

[6] R1-2104327, “Discussion on CSI feedback enhancements for eURLLC”,	 ZTE

[7] R1-2005870, “Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT”, RAN1 105e, InterDigital.