3GPP TSG-RAN WG1 Meeting #106-e	R1-2107222

e-Meeting, August 16th – 27th, 2021

Title:	[Draft] Reply LS on resource reselection trigger sl-reselectAfter

Response to:	R2-2106625

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	OPPO [to be RAN1]

To:	RAN2

Cc:	-

Contact Person:

Name:

Tel. Number:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 thank RAN2’s LS in R2-2106625 (R1-2106406) and would like to provide our response to the following questions asked by RAN2.

Question 1: In NR V2X, whether “unused retransmission opportunities in case of HARQ feedback is enabled” shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

RAN1 response: No. Considering up to 32 transmission times per TB is supported in NR SL, if all of the unused retransmission opportunities is counted toward the counter, that may cause unnecessary resource reselection in case ACK is received and the following retransmission opportunities are not used. Hence it is RAN1’s understanding that only unused transmission opportunities for initial transmissions should be counted towards “consecutive unused transmission opportunities” to trigger resource reselection.

Question 2: In NR V2X, according to RAN1 agreement, is it the correct understanding that only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

RAN1 response: No, same reason as the response for Question 1.

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#106b-e	11-19 October 2021   	E-Meeting

RAN1#107-e	11-19 November 2021   	E-Meeting