3GPP TSG-RAN WG1 #106e	R1- 2107334

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.2.5

Source: 	Qualcomm Incorporated

Title: 	PDSCH and PUSCH enhancements for 52.6-71GHz band

Document for:	Discussion/Decision

# Introduction

In RAN #90e, a Rel-17 work item for NR operation in a frequency regime between 52.6GHz and 71GHz has been approved [1]. As a part of the work item, the study on the PDSCH/PUSCH potential enhancements with new agreed numerologies, i.e., 120kHz, 480kHz, and 960kHz. Throughout the study item [2], it was clear that some PTRS enhancements are needed to facilitate ICI compensation for the phase noise impact, while DMRS enhancements are need for larger SCSs, 480kHz, and 960kHz, with channels with high frequency selectivity. In RAN1 #104-bis-e and RAN1 #105-e, [9] and [10], it was agreed to continue studying different PTRS and DMRS patterns for the higher band and other enhancements needed for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI.

In this contribution, we continue to discuss some key aspects of waveform design and present our views on some potential issues for the high frequency regime supported by link-level evaluation based on the evaluation methodology and assumptions for the link-level simulation were discussed in RAN1 #104-bis-e [9]. Additionally, we provide proposals for the different aspects of the reference signals and PDSCH/PUSCH for the high frequency regime.

# Discussion

## Link-level performance evaluation

This section provides link-level evaluation results as per the agreements on the simulation methodology and assumptions in RAN1 #104-e. Some of the key parameter settings for the link-level evaluation are listed in Table 1, yet the complete set of parameters are available in [8]. For further reference, the entire set of link performance plots are included in the Appendix.

Table 1: Link-level simulation parameters

| Parameters |  | Values |
| --- | --- | --- |
| Carrier frequency |  | 60 GHz |
| Antenna config (CDL) | Config 1 | (Mg,Ng,M,N,P) = (1,1,8,16,2) @gNB (Mg,Ng,M,N,P)=(1,1,4,4,2) @UE |
| Doppler |  | 167Hz (3km/h at 60GHz) |
| Channel Model and pre-beamforming DS |  | TDL-A with 5ns/10ns delay spread CDL-B with 20ns/50ns delay spread |
| Phase noise mask |  | 3GPP TR 38.803, example 2 (BS and UE) |
| SCS/Bandwidth |  | 120kHz/400MHz (256RBs) 480kHz/1600MHz (256RBs) 960kHz/2000MHz (160RBs) |
| CP config |  | Normal CP |
| PDSCH/PUSCH symbol index |  | (S=2, L=12) |
| PDSCH/PUSCH waveform |  | CP-OFDM for PDSCH DFT-s-OFDM for PUSCH |
| DMRS symbol index |  | 2 (front-loaded) for 1 DMRS |
| MCS |  | MCS Table 1 (TS 38.214) |

## PDSCH/PUSCH Enhancements

### PTRS design and phase noise compensation

Phase noise (PN) has been regarded as a source of nonideality during FR2 waveform and signal/channel design. For the high frequency regime between 52.6GHz and 71GHz, however, the impact of phase noise would be exacerbated, and further investigation is required in relation to the study on the candidate numerologies of the high frequency regime. The time varying PN induces both common phase error (CPE) and inter-carrier interference (ICI) in the frequency domain. Although the CPE compensation is quite straightforward, the ICI compensation may need a special PTRS pattern and a more involved algorithm. In this regard, in RAN1 #103-e, it was agreed to study any potential modification of the existing PTRS design/configuration, as well as the ICI compensation methods, to aid performance improvement for CP-OFDM and DFT-s-OFDM waveforms.

#### CP-OFDM

It is widely believed and actually proven by link-level evaluation in [4] that a higher SCS is more robust to PN when the CPE compensation is applied. In particular, since the impact of ICI is more pronounced with low SCSs, non-negligible performance degradation is observed with 120kHz SCS for high MCSs with 64QAM modulation. However, in RAN1 #102-e and RAN1 #104-e, it was also pointed out by several companies that, with a proper PTRS pattern design and an ICI compensation algorithm, the performance losses at low SCSs and high MCSs can largely be recovered.

For the ICI compensation algorithm, we consider the two algorithms proposed in [3], i.e., direct de-ICI filtering (Algorithm 1) and ICI filter approximation approaches (Algorithm 2). For new candidates PTRS design, we consider several block PTRS patterns, where the PTRS tones are mapped to one or more equal-size clusters of contiguous REs that are evenly distributed in the allocated PDSCH. As the baseline, we also consider the legacy Rel-15 PTRS pattern. Note that Algorithm 1 can be applied for both the block and legacy PTRS patterns, while Algorithm 2 can only be used with the block PTRS pattern.

One of the drawbacks of block PTRS patterns is ICI leakage from PTRS tones to the neighboring data tones. One proposal was to consider block PTRS patterns with cyclic nature to reduce the ICI leakage as in [5] and [6]. Another way is modifying the structure of the block PTRS to contain zero-power tones at the edges of each cluster of PTRS tones as shown in Fig. 1. These tones can act as guard tones as well as being used for ICI filter calculations. For fair comparison, we boosted the power of the non-zero power PTRS tones based on the number of zero-power tones in each cluster to match the total power of the PTRS cluster with all non-zero power tones. It is worth mentioning that such a structure can reduce the complexity of the filter calculations, by converting the matrix of the PTRS tones, for Algorithm 2, into a diagonal matrix for each cluster.

Fig. 1: Block PTRS pattern with ZP tones: 7 tones per cluster and 6 ZP tones

For 120kHz SCS and TDL-A with 5ns DS, the PDSCH (256 RBs) BLER performances for different PTRS patterns and ICI compensation algorithms are evaluated. For the block PTRS pattern, we considered three different clustering schemes (9, 7, and 6 clusters) with similar overheads to the legacy PTRS pattern with K=4. The cyclic block is based on the description in [5]. The PTRS tones are constructed based on the corresponding DMRS tones, i.e., a pn-sequence modulated by QPSK in the frequency domain is used to construct the PTRS clusters. The CINR points corresponding to 10%/1% BLER performance are summarized in Table 2. From the results, the following observation is made in general:

Observation 1: Comparing the different PTRS patterns

- The best performance is obtained with block PTRS pattern with zero-power tones.

- The legacy pattern performance is very close to block PTRS pattern with zero-power tones.

Next, we compare the performance of the legacy pattern vs the clustered cyclic pattern proposed in [6] with different circular sequences lengths (3 and 5), we choose p=q, i.e., the two circular parts have the same lengths. For the cyclic patterns, we consider 4 clusters each with 15 tones where the base sequence is a Zadoff-Chu sequence. The performance is summarized in Table 3.

Table 2: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 120kHz SCS, Rank 1, TDL-A 5ns DS and 5-tap filters

| MCS | Compensation Scheme | PTRS Pattern |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 9 Clusters, 7 tones each (No ZP tones) | 9 Clusters, 7 tones each (6 ZP tones) | 9 Clusters, 7 tones each (Cyclic Block) | 6 Clusters, 11 tones each (Cyclic Block) | Legacy Pattern with K=4 |
| 22 | Algorithm 1 | 11.6/12.2 | 10.4/11.0 | 10.8/11.7 | 10.9/11.8 | 10.7/11.4 |
|  | Algorithm 2 | 13.6/15.0 |  |  | 11.0/12.0 |  |
| 26 | Algorithm 1 | 16.8/18.4 | 16.0/17.6 | 16.6/18.3 | 16.7/18.0 | 16.3/17.8 |
|  | Algorithm 2 | 20.3/- |  |  | 17.0/18.9 |  |

Table 3: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 120kHz SCS, Rank 1, TDL-A 5ns DS and 5-tap filters

| MCS | Compensation Scheme | PTRS Pattern |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 4 Clusters, 15 tones each (P=5) | 4 Clusters, 15 tones each (P=3) | 9 Clusters, 7 tones each (6 ZP tones) | Legacy Pattern with K=4 |
| 22 | Algorithm 1 | 10.8/11.8 | 10.9/11.8 | 10.4/11.0 | 10.7/11.4 |
|  | Algorithm 2 | 11.9/15.2 | 11.7/12.7 |  |  |
| 24 | Algorithm 1 | 13.3/14.0 | 13.4/14.4 | 12.9/13.8 | 12.9/13.9 |
|  | Algorithm 2 | 14.3/17.7 | 13.9/15.0 |  |  |

Observation 2: The block PTRS pattern with ZP tones and the legacy pattern outperform the cyclic patterns based on ZC sequences.

It should be mentioned that if we use a cluster of 7 tones, with a single NZP tone in the middle, using Algorithm 2 directly, only a 3-tap filter can be estimated, i.e., we cannot estimate a 5-tap filter. Next, in Table 4, for CDL-B channel, we compare the performance of Algorithms 1 and 2 with 7 clusters, with 9 PTRS tones per cluster, 8 of them are ZP tones. For this configuration, we can estimate a 5-tap filter with Algorithm 2.

Table 4: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 120kHz SCS, Rank 1, CDL-B 20ns DS and 5-tap filters

| MCS | Compensation Scheme | PTRS Pattern |  |
| --- | --- | --- | --- |
|  |  | 7 Clusters, 9 tones each (8 ZP tones) | Legacy Pattern with K=4 |
| 22 | Algorithm 1 | 6.7/13.9 | 6.8/14.8 |
|  | Algorithm 2 | 6.7/13.9 |  |
| 24 | Algorithm 1 | 9.2/16.7 | 9.4/16.9 |
|  | Algorithm 2 | 9.3/16.9 |  |

Observation 3: With 7 clusters each with 9 tones, 8 ZP PTRS tones and a single NZP tone in the centre, the two algorithms with the clustered pattern have almost the same performance, and they slightly outperform the legacy pattern.

In RAN1 #104-bis-e, we made the following agreements on further studying PTRS enhancements for the CP-OFDM waveform.

Agreement: - It is recommended to strictly follow and evaluate at least based on assumptions which are not optional in previous agreed LLS assumptions for study of potential RS enhancements for NR operation in 52.6 to 71 GHz. - Note: evaluation based on optional model/scenario/parameter values are not precluded from being considered for discussion and decisions - Companies are encouraged to report results (along with previously reported aspects and cubic metric for power boosting aspects) at least for SINR in dB achieving PDSCH/PUSCH BLER of 10% in a numerical and tabular way (e.g. adapted from LLS result report template in SI). - Note: other ways of presentation of results (e.g. BLER curve) is not precluded Agreement: - In Rel-17, for NR operation in 52.6 – 71 GHz, conclude that increased PTRS frequency density is not supported for CP-OFDM at least for Rel-15 PTRS pattern when the allocated number of RB > 32 - Companies are encouraged to study whether to increase PTRS frequency density for small RB allocations for CP-OFDM for NR operation in 52.6 to 71 GHz with respect to phase noise compensation performance - CPE and ICI PN compensation - Note: Results for CPE compensation-only are to be reported for reference - (K = 0.5, L = 1), (K = 1, L = 1), (K = 2, L = 1), - Note: PTRS per K number of PRBs, and PTRS every L number of OFDM symbols - Number of RBs: 8, 16, 32 - Other values of K and number of RBs are not precluded - Study on other aspects of potential PTRS enhancement (e.g., decreased PTRS frequency density) is not precluded

One concern was raised by some companies was that fully power boosting a single tone in the PTRS block may cause issues due to the large difference between the power levels of different frequency tones. In table 5, we compare the BLER performance if we limit the power of the NZP tone in the cluster. We considered 7 clusters each with 9 tones, where each cluster has only a single NZP tone at its centre. Different power levels were considered: 1) Fully power boost, i.e., maintain the total power of the cluster as if all tones are NZP, 2) No power boost, 3) 3dB power boost, 4) 6 dB power boost, 5) 5dB power boost.

Table 5: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 120kHz SCS, Rank 1, TDL-A 5ns DS and 5-tap filters

| MCS | Compensation Scheme | PTRS Pattern |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 Clusters, 9 tones (8 ZP tones in each cluster) |  |  |  |  | Legacy Pattern with K=4 |
|  |  | Full power boost | No power boost | 3dB power boost | 6dB power boost | 8dB power boost |  |
| 22 | Algorithm 1 | 6.7/13.9 | 7.9/15.2 | 7.3/14.7 | 6.9/14.2 | 6.7/13.9 | 6.8/14.2 |
|  | Algorithm 2 | 6.7/13.9 | 9.4/16.8 | 8.0/15.1 | 7.1/14.5 | 6.8/14.0 |  |
| 24 | Algorithm 1 | 9.2/16.7 | 10.5/17.8 | 9.9/17.3 | 9.5/16.9 | 9.4/16.7 | 9.4/16.9 |
|  | Algorithm 2 | 9.3/16.9 | 12.0/19.5 | 10.5/17.9 | 9.6/16.9 | 9.5/16.6 |  |

Observation 4:  With 7 clusters each with 9 tones, 8 ZP PTRS tones and a single NZP tone in the centre, power boosting the NZP tone by 6 dB can matches the performance in of the legacy PTRS pattern.

Next, we evaluate the impact of power boosting on the PAPR and CM of the transmitted signals. We compare these metrics for the transmitted time domain signals under 4 levels of power boosting: 1) Full power boost, 2) No power boost, 3) 3dB power boost, and 4) 6dB power boost. Fig.2 shows the PAPR and CM, in dB, for these cases, and it is clear that the frequency domain power boosting will not affect the PAPR nor CM of the transmitted signals.

Fig. 2: PAPR and CM for different levels of power boosting

Observation 5: For clustered PTRS pattern with ZP tones, power boosting NZP tones will not increase CM or PAPR.

It is noteworthy that the ICI compensation algorithm may require a larger number of PTRS REs than CPE-only compensation and, thus, it may not be the case when the PTRS frequency density is low and the number of allocated RBs is small. In Fig. 3 and 4 and Table 6, we show the performance with different PTRS frequency densities for RB allocation 16 and 8, respectively.

Fig. 3: 16 RBs: Legacy PTRS with different K

Fig. 4: 8 RBs: Legacy PTRS with different K

Table 6: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 120kHz SCS, TDL-A 5ns DS, PTRS (L=1), Rank 1

| MCS | RB allocation | K=0.5 |  | K=1 |  | K=2 |  | K=4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CPE compensation | ICI compensation | CPE compensation | ICI compensation | CPE compensation | ICI compensation | CPE compensation |
| 22 | 8 RBs | 14.6/17.7 | 14.0/16.6 | 13.6/17.6 | 13.7/16.4 | 14.1/18.9 | 18.9/- | 15.5/- |
|  | 16 RBs | 14.1/16.4 | 13.4/15.7 | 12.6/15.3 | 12.5/14.6 | 12.6/15.9 | 13.0/15.2 | 13.5/17.5 |
| 24 | 8 RBs | 19.6/- | 18.0/20.4 | 16.5/20.6 | 16.5/19.0 | 16.7/21.9 | -/- | 18.2/- |
|  | 16 RBs | 20.7/- | 17.9/20.2 | 16.3/19.3 | 15.5/17.9 | 15.9/19.7 | 16.1/18.6 | 17.0/21.3 |

Observation 6: For small RB allocation, e.g., 16 or 8 RBs, sending the PTRS over every RB, i.e., K=1, enhances the performance as it helps in having more accurate ICI filter calculations or CPE estimates. In addition, the performance is degraded when the density is increased with K=0.5 due to the large overhead and increase coding rate.

Therefore, the PTRS enhancement for the high frequency regime may consider increasing the PTRS density when the number of allocated RBs is small.

Now, it is well understood that SCS 120kHz needs ICI compensation for the phase noise to achieve comparable performance with SCS 960kHz for high MCSs, e.g., 64QAM. Since ICI compensation algorithms, such as Algorithm 1 (direct de-ICI filtering) requires matrix multiplication and matrix inversion operations to calculate the filter coefficients, these algorithms might be computationally expensive with the current NR timeline for SCS 120kHz. In addition, the RF module at the UE is expected to be the main source of the phase noise, which will determine whether the ICI compensation is needed or not, and if needed, how many filter taps are required. From our evaluations, performance-wise, Rel. 15 PTRS pattern seems to be sufficient. However, low-complexity ICI compensation techniques may be needed to simplify the UE processing, given the current timeline, and such techniques may require adding an additional PTRS pattern, e.g., block-based patterns, to allow time domain estimation of the phase noise or a specific structure for PTRS tones matrix, e.g., diagonal matrix, which reduces the complexity of the matrix inversion for ICI filter calculations. Another alternative solution could be to define a UE capability to support the high MCS that requires ICI compensation.

Proposal 1: For SCS 120kHz, the complexity of phase noise ICI compensation and its effects on the UE processing timeline need to be considered.

Proposal 2: Support introducing a new PTRS pattern, clustered based with ZP tones, to reduce the ICI compensation complexity.

Proposal 3: As PTRS enhancement for assisting ICI compensation, increasing the frequency domain density, of Rel. 15 PTRS, for small RB allocation can be considered.

#### DFT-s-OFDM

One of the agreements of RAN1 #104-bis-e is to continue studying whether increasing the number of PTRS samples, with SC-FDM waveform, is needed for SCS 120kHz or not.

Agreement: Continue study at least the following aspects for potential PTRS enhancement for DFT-s-OFDM for NR operation in 52.6 to 71 GHz - The need of potential PTRS enhancement - PTRS pattern with more PTRS groups within one DFT-s-OFDM symbol when a large number of PRBs is scheduled - (Ng = 8, Ns = 4, L = 1), (Ng = 16, Ns = 2, L = 1), (Ng = 16, Ns = 4, L = 1), - Note: Ng number of PT-RS groups, Ns number of samples per PT-RS group, and PTRS every L number of DFT-s-OFDM symbols - Other patterns are not precluded - Other aspects of PTRS enhancements are not precluded from further study

Currently, Rel. 15 supports up to 32 samples per symbol. We evaluated the performance with different numbers of PTRS samples. In addition to the legacy pattern, 16 samples (4 chunks each with 4 samples) and 32 samples (8 chunks each with 4 samples), we considered 64 samples (16 chunks each with 4 samples), 128 samples (32 chunks each with 4 samples) and 16 samples (8 chunks each with 2 samples). Table 5 contains the CINR points corresponding to 10%/1% BLER points with different PTRS patterns.

Table 5: CINR in dB achieving PUSCH iBLER of 10% ∕ 1%: 120kHz SCS, PTRS (K=4, L=1), Rank 1

| MCS | Channel | PTRS number of samples |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 16 |  | 32 | 64 | 128 |
|  |  | 4 Chunks | 8 Chunks |  |  |  |
| 22 | TDL-A, 5ns | 15.9/20.4 | 14.8/19.8 | 13.8/18.5 | 13.7/17.9 | 13.8/17.4 |
| 24 | TDL-A, 5ns | 23.0/- | 17.9/23.6 | 16.1/21.5 | 15.9/20.9 | 16.8/21.0 |

Observation 7: For MCS 22, there is no significant enhancement for increasing the total number of PTRS samples to 64, while small gains can be observed with MCS 24. In addition, increasing the number of PTRS samples beyond 64 is not helping the performance.

Observation 8: With total of 16 samples, the pattern with 8 chunks outperforms the one with 4 chunks, i.e., distributing samples over time domain enhances the performance, but still the legacy pattern with 32 samples is outperforming.

Proposal 4: No need to increase the PTRS sample density for the SC-FDM waveform.

### DMRS enhancements

For a given channel, the frequency-domain channel correlation between adjacent REs declines as the SCS increases. With the existing DMRS patterns (i.e., PDSCH/PUSCH DMRS configuration type 1 or 2 in Rel-15), the enhanced frequency selectivity by a large SCS may result in poor channel estimation performance, particularly when the channel delay spread is large.
In Fig. 5, (b) a new candidate DMRS pattern is illustrated. In comparison with existing DMRS patterns, (a) config type 1 and (c) config type 2, the new DMRS pattern is featured by high frequency density (i.e., every RE) and 2-FD-OCC across adjacent REs. In RAN1 #104-e, we have shown that, for rank 1, the gain from introducing the full frequency DMRS pattern is not significant compared with Config. Type 1 with FD-OCC OFF. In Table 6, the PDSCH performances of the different DMRS patterns with 960kHz SCS are compared for TDL-A and CDL-B channels with DS 20ns and 50ns, respectively, using the updated configuration as discussed in RAN1 #104-e. Similarly, Table 7 shows the performance for SCS 480kHz.

| (a) | (b) | (c) |
| --- | --- | --- |
| Fig. 5: 1-symbol front-loaded DMRS patterns: (a) Rel-15 config type 1, (b) new configuration for high SCSs, (c) Rel-15 config type 2 |  |  |

Table 6: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 960kHz SCS, PTRS (K=4, L=1), Rank 1

| MCS | Channel | Every other RE (config type 1) |  | Every RE (new pattern) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CDM | No CDM | CDM | No CDM |
| 16 | TDL-A, 20ns | 7.4/8.7 | 6.9/8.0 | 7.2/8.4 | 6.9/8.0 |
| 16 | CDL-B, 50ns | 1.1/11.5 | 1.0/9.7 | 1.1/10.1 | 1.0/9.6 |
| 22 | TDL-A, 20ns | 13.1/14.5 | 12.2/13.7 | 12.7/13.9 | 12.3/13.6 |
| 22 | CDL-B, 50ns | 6.8/- | 6.8/15.8 | 6.9/16.3 | 6.8/15.7 |

Table 7: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 480kHz SCS, PTRS (K=4, L=1), Rank 1

| MCS | Channel | Every other RE (config type 1) |  | Every RE (new pattern) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CDM | No CDM | CDM | No CDM |
| 22 | TDL-A, 20ns | 12.9/15.1 | 12.7/14.9 | 12.9/15.2 | 12.7/14.7 |
| 22 | CDL-B, 50ns | 7.1/15.8 | 6.9/15.2 | 7.0/15.9 | 7.2/15.5 |
| 26 | TDL-A, 20ns | -/- | 27.1/- | -/- | 27.9/- |
| 26 | CDL-B, 50ns | 24.27/- | 21.76/- | 25.83/- | 24.9/- |

Observation 9: For rank 1, with SCS 960kHz, for channels with larger DS, the main reason of performance degradation with the larger SCS is the loss of orthogonality, and the gain from increasing the frequency density of the DMRS tones is limited, i.e., the performances of Config.1 with no CDMing and the new configuration with no CDMing are very close to each other.

Observation 10: For rank 1, with SCS 480kHz, even for channels with larger DS, the different DMRS patterns have very comparable performances, and the best performance is obtained Config.1 with no CDMing.

Next, we evaluated the performance with rank 2 transmissions. We considered CDL-B channel with DS 20ns and 50 ns to compare the performance of three configurations: 1) config. type 1 with ports 1000 and 1001, i.e., two ports from the same CDM group, 2) config. type 1 with ports 1000 and 1002, i.e., two ports from the different CDM groups (no CDMing), 3) the new pattern with DMRS sent over every RE, with ports 1000 and 1001, i.e., two ports from the same CDM group. In Tables 8 and 9, the CINR values in dB, which achieve 10% and 1% PDSCH iBLER, are summarized.

Table 8: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 960kHz SCS, PTRS (K=4, L=1), Rank 2

| MCS | Channel | Every other RE (config type 1) |  | Every RE (new pattern) |
| --- | --- | --- | --- | --- |
|  |  | CDM | No CDM | CDM |
| 16 | CDL-B, 20ns | 8.6/17.1 | 8.5/16.9 | 8.6/17.1 |
|  | CDL-B, 50ns | 9.4/- | 9.1/19.8 | 9.5/20.7 |
| 22 | CDL-B, 20ns | 15.0/24.1 | 14.7/23.9 | 15.2/23.9 |
|  | CDL-B, 50ns | 18.7/- | 17.4/- | 18.4/- |

Table 9: CINR in dB achieving PDSCH iBLER of 10% ∕ 1%: 480kHz SCS, PTRS (K=4, L=1), Rank 2

| MCS | Channel | Every other RE (config type 1) |  | Every RE (new pattern) |
| --- | --- | --- | --- | --- |
|  |  | CDM | No CDM | CDM |
| 22 | CDL-B, 20ns | 15.9/25.8 | 15.7/24.8 | 15.9/25.8 |
|  | CDL-B, 50ns | 17.5/- | 17.0/36.7 | 17.5/- |

Observation 11: For rank 2, with SCS 960kHz, with DS 20ns all DMRS patterns perform the same, while with DS 50ns, there is a significant performance loss when FD-OCC is ON, and the best performance is obtained with config type 1 with ports from different CDM groups.

Observation 12: For rank 2, with SCS 480kHz, all DMRS patterns perform the same.

Proposal 5: Do not introduce a new pattern with DMRS tones sent over every RE, for the higher band.

It should be noted that when the DMRS tones are sent over every other RE, Configuration type 1, the DMRS tones are power boosted by 3dB compared with the new pattern where the DMRS tones are sent over every RE. As mentioned above, the main cause of the performance loss for NLOS channels with large DS is the despreading step at the receiver due to the loss of orthogonality.

gNB can assign the UE with port(s) from different CDM groups and the remaining ports from each CDM group should not be assigned for other UEs. Such information should be signaled to the UE to avoid the despreading step. For DM-RS configuration type 1, with rank 2 transmission, if the UE is scheduled with ports 0 and 2, i.e., index 11 in Table 7.3.1.2.2-1 for one codeword and Table 7.3.1.2.2-2 for two codewords of Clause 7.3.1.2 of [7, TS 38.212], the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE. Therefore, no spec changes are needed for rank 2. However, the aforementioned assumption is not ensured by the current specs for rank 1 transmission. Therefore, we need to signal to the UE, with rank 1 transmission, when it can assume that the remaining ports from the same CDM group, of its assigned DMRS port, are not used by another UE. Such signaling can be dependent on the channel conditions and the operating MCS, i.e., for low MCS values, such as QPSK and 16QAM modulations, the performance is less sensitive to channel estimation errors. Several options can be considered such as explicit signaling in the scheduling DCI or a rule can be defined to prevent FD-OCC for high MCS such as 64QAM modulation so that the UE can assume the FD-OCC is turned off when it is scheduled with such MCS.

Proposal 6: For DMRS enhancement for high SCSs, while communicating over channels with large DS and using high MCS, for rank 1, a single port should be used from one CDM group and the remaining ports from the same group should not be assigned to other UEs.

There are several options to signal such information to the UE in order to simplify the processing and avoid the need of applying blind detection for the other DMRS ports before performing the channel estimation. If the UE is configured with an RRC configuration that indicates whether the FD-OCC is either ON or OFF, not based on the MCS or channel conditions, then many multiplexing opportunities will be lost, as the performance hit due to orthogonality loss is only evident for the channels with high delay spread when operating with high MCS, e.g., 64QAM. The most efficient way will be conveying such information via the DCI, in the DMRS related field. The reserved entries in the DMRS antenna ports tables, e.g., 7.3.1.2.2.1-1, can be modified to indicate that the FD-OCC is OFF, and gNB will have the flexibility to assign different configurations based on the MCS, and the estimated channel conditions. A middle ground could be making the RRC configuration tied to at least the MCS, i.e., if the MCS is higher than a certain threshold, then the UE can assume that the FD-OCC is OFF, i.e., no DMRE of other UEs are CDMed with his own DMRS tones.

Proposal 7: Consider the following options to signal the information of FD-OCC OFF/ON

- Option 1: RRC configuration tied to an MCS threshold

- Option 2: DCI indication by introducing new entries to the DMRS antenna ports tables, e.g., reuse the reserved entries

### Multi-PDSCH/PUSCH with single grant

#### Maximum Number of PDSCHs/PUSCHs per single DCI

In RAN1 #104-bis-e, we made the following agreement

Agreement: - The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz. - FFS: Further restrictions for 480 kHz to 4 - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided. - The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8. - FFS: Further restrictions for 120 kHz and 480 kHz SCS - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS

The multi-PUSCH scheduling is supported as one of the features for Rel. 16 in NR-U WI, where the SCS is 120 kHz or lower, and it was agreed to maximize the similarities between multi-PDSCH and multi-PUSCH designs, therefore, we believe that SCS 120kHz should be considered as well as SCS 480 and 960kHz for multi-PDSCH/PUSCH grants with as a single DCI. In addition, multi-PDSCH grants should help in reducing the UE processing time as it will reduce the frequent re-tuning of the UE hardware, e.g., the RF module. We had more discussion on the per-slot and multi-slot PDCCH monitoring in [3]. Also, supporting a multi-PDSCH grant may require the UE to buffer a significant amount of time domain samples especially with smaller SCS, e.g., 120kHz and 480kHz, thus the maximum number of PDSCHs/PUSCHs to be scheduled by the same DCI for these SCSs should be based on the UE capabilities.

Proposal 8: Multi-PDSCH or multi-PUSCH scheduling with the same DCI should be applicable to 120kHz as well as 480 and 960kHz, though we don’t need to introduce multi-slot monitoring capability for 120KHz.

Proposal 9: A UE capability to be defined per SCS, to indicate the maximum number of supported PDSCHs/PUSCHs per single DCI for SCS 120kHz and 480kHz.

#### HARQ-ACK enhancements

In the current NR specs, there are up to 16 HARQ processes, except for the NTN use case, there are up to 32 HARQ processes.

With larger SCS, such as 480 and 960KHz, given 5ms maximum COT, each COT can cover 160 slots of SCS 480kHz or 320 slots of SCS 960kHz.With 16 HARQ processes, it might be needed to insert PUCCH every 5 slots, which will lead to 32/64 UL/DL switching within the 5ms, i.e., large overhead.  To allow transmission for a larger number of slots without feedback, one proposed solution is to increase the number of HARQ processes to 32. However, this may require the UE to maintain a larger soft buffer which may be problematic based on the UE capabilities. Therefore, a higher number of HARQ processes (X) can be supported based on the UE capability, and also a subset of supported HARQ processes can be supported with soft combining (Y), where Y ≤ X. For the subset of HARQ processes without soft combining, gNB can always transmit a self-decodable redundancy version of the code, e.g., one with RVID 0 or 3.

Proposal 10: In the case of increasing the HARQ processes to 32 for SCSs 480kHz and 960kHz, a UE capability should be defined such that X HARQ processes can be supported, and Y of them can do soft combining where X and Y ≥ 16.

In order to simplify the UE processing, we do not support allowing HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI to be carried by different PUCCHs. In addition, we do not believe there will be a significant reduction in the latency by allowing some of the PDSCHs to be feedbacked before the remaining PDSCHs scheduled by the same DCI, as the timeline N1 is expected to cover multiple slots for SCS 480kHz and 960kHz. For example, with 960kHz, N1 will be around 11 slots, using SCS 120kHz absolute timeline as a baseline. With 8 PDSCHs at maximum scheduled by the same DCI, the offset between any PDSCH reception and its feedback has to be larger than N1, i.e., 11 slots, i.e., counting this offset from a slot of PDCSH at the middle of the allocation or the slot of the last PDSCH will not lead to a significant reduction in the latency of the feedback. For example, if we allow the first 4 PDSCHs to be feedback separately before the last 4 PDSCHs, the first feedback will be transmitted at most 4 slots before the second feedback, this will count for ~56us latency enhancement for SCS 960kHz which is not significant. Therefore, we do not see a substantial gain from allowing the HARQ-ACK information corresponding to different PDSCHs scheduled by the same DCI to be carried by more than one PUCCH, especially when additional control overhead (more fields in DCI, and more PUCCH transmission) is needed to support the feature.

Proposal 11: All HARQ-ACK information corresponding to different PDSCHs scheduled by the same DCI to be carried by the same PUCCH.

Based on the PDSCH processing timeline for the new SCS, i.e., 480kHz and 960kHz, it will be needed to extend the range of values of K1 for both the default and RRC configured values. For example, if the absolute timeline for 120kHz is applied to 480kHz and 960kHz, where N1 is 20 symbols for frontloaded DMRS, then N1 for 480kHz and 960kHz can cover around 80 and 160 symbols, respectively. Before RRC configuration, the K1 values for fallback DCI are {1,2,3,4,5,6,7,8}, the maximum value 8 can be shorter than the N1 timeline for the new SCS. In addition, for RRC configuration, K1 ranges up to 15 slots, but most of the values within the range will be less than the N1 values for 480kHz and 960kHz. Therefore, the values of K1 should be revisited based on the PDSCH timeline. One simple solution could be shifting the values of K1 based on the N1 value, e.g., shift range by floor(N1/14).

Proposal 12: Revisit K1 values for SCS 480kHz and 960kHz after the discussions on the PDSCH processing timeline.

##### Type-2 HARQ-ACK codebook enhancements

For the HARQ-ACK timing information and type-2 HARQ-ACK codebook enhancements for multi-PDSCH scheduling, we reached the following conclusion in RAN1 #105-e.

Agreement: - If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - At least two sub-codebooks are generated for a PUCCH cell group where - The first sub-codebook is for the following cases: - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH - The second sub-codebook is for the following case: - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH - FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group - FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI Agreement: - If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly. - The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR. - Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR - This shall not impose additional gNB’s scheduling restriction. - In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on - The number of SLIVs associated with the row indexes in TDRA table - FFS: details - FFS: the case with configuration of CBG retransmission - FFS: the number of sub-codebooks - FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

For Alt 1, with time domain bundling, a single codebook can be sufficient whenever all the feedback of the PDSCHs scheduled by the same DCI are combined into a single bit. In the case of two sub-codebooks, we need to have a fixed number of A/N bits per DCI to ensure proper alignment of gNB and UE sub-codebook sizes. Additional flexibility can be achieved by allowing different number A/N bits per multi-PDSCH grant, such that for each A/N occasion all the corresponding multi-PDSCH grants will have the same number of A/N bits, however, from one A/N occasion to another we can allow different numbers of A/N bits per grant. Similarly, if the time domain bundling is used, the bundling pattern can be changed from one A/N occasion to another. The time domain bundling pattern can be defined via RRC configuration or MAC-CE. For example, the number of A/N bits can be defined per grant and if the number of PDSCHs is less than the number of A/N bits, then zero-padding can be used to ensure equal codebook sizes, e.g., if the number of A/N bits per grant is 4, and only 3 PDSCHs were granted then the UE will add 0 to the feedback codebook. On the other hand, if the number of PDSCHs is greater than the number of A/N bits per DCI, then some of PDSCHs can be bundled together, e.g., if number A/N bits per grant is 4 and 8 PDSCHs were granted, then A/N of each two PDSCHs will be bundled together. A more dynamic approach can be by allowing the PDCCH to carry an explicit indication of the number of A/N bits for all upcoming grants within one A/N occasion. 
Proposal 13: With Alt 1, in the case of time domain bundling of A/N bits corresponding to PDSCHs scheduled by the same DCI into one bit, a single codebook should be defined.

Proposal 14: Allowing different numbers of A/N bits per multi-PDSCH grant, such that for each A/N occasion all the corresponding multi-PDSCH grants will have the same A/N bits, however, from one A/N occasion to another we can allow different number A/N bits per grant

- If time domain bundling is enabled, then the bundling pattern can be changed from one A/N occasion to another.

  - Time-domain bundling patterns to be defined via RRC configuration and the active pattern can be changed by MAC-CE or PDCCH.

We prefer Alt 2, i.e., the DAI to be counted per PDSCH. With Alt 1, we need to ensure that a fixed number of A/N bits is transmitted for the different grants corresponding to the same A/N occasion. That is, for fallback DCI or SPS PDSCH, the same number of A/N bits need to be added to the codebook similar to a multi-PDSCH grant to avoid the misalignment between the codebooks at gNB and UE. With a multi-PDSCH grant with a single DCI, the DAI field size needs to be increased.

We want to stress the point that with a careful configuration of the TDRA table, the current DAI filed size can be sufficient. For example, if the configured TDRA contains both single PDSCHs and 7 PDSCHs per a single DCI, then missing a grant of size 7 can be detected by the total DAI. On the other hand, the DAI field needs to be increased if gNB wants to configure TDRA table with 4 or 8 PDSCHs allocations per a single DCI as with 2-bit DAI (i.e., we apply mod 4), the total DAI increase will be 0 and the missed DCI will not be detected. This problem will be there, similar to legacy if multiple DCIs are missed. Therefore, the DAI filed size of DCIs for multi-PDSCH grants can be variable and it should be up to gNB to configure this field size via RRC.

In addition, the resolution of the DAI counter can be adjusted based on the greatest common divisor of the number of SLIVs in each row of the TDRA table. For example, consider a TDRA table that only contains rows with 4 or 8 SLIVs, then every four PDSCHs can increment the DAI counter by 1, i.e., a grant of 4 PDSCHs will increment the DAI once, and a grant of 8 PDSCHs will increment the DAI twice. By doing this, there is no need to increase the number of DAI bits in the DCI based on the maximum number of the SLIVs in the TDRA table, which can save some DCI overhead while maintaining the same level of reliability of the missed grants. If a single PDSCH is scheduled using DCI format 1_0, the DAI can be incremented, then UE can pad some NAKs to ensure the correct codebook size. In the previous example, if the resolution is 4, then the UE will pad 3 NAKs for each of the fallback DCI grants. By changing the resolution of the DAI increments, gNB can determine the DAI field size based on the greatest common divisor of the number of the SLIVs not the maximum number of the SLIVs among the different rows in the TDRA table which can lead to a significant DCI overhead saving while maintaining the same level of reliability.

Proposal 15:

- Support increasing the field size of the DAI based on RRC configuration to increase the reliability against the missed DCIs. However, the field size increase should be subject to gNB configuration.

- Allow adjusting the resolution of the DAI counter based on the greatest common divisor of the number of the SLIVs, among the rows of the TDRA, i.e., each increment of the DAI indicates that a number of PDSCHs equal to the greatest common divisor has been sent.

##### DCI contents for multi-PDSCH grants

For the DCI contents for multi-PDSCH grants, the following agreement was made in RAN1 #104-bis-e and RAN1 #105-e.

### Agreement: For a DCI that can schedule multiple PDSCHs, - MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH - HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed) - FFS: - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements Agreement: - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. - FFS: - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

Similar to the 1st TB related fields, the 2nd TB can be configured, i.e., MCS appears once and applies to the second TB of each PDSCH, NDI is signaled per PDSCH and applied to the second TB of each PDSCH, RV is signaled per PDSCH with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the second TB of each PDSCH. We do not see a need to limit the specifications from supporting a second TB, while it will be based on the UE capability.

The CBG retransmission is not supported for the Rel-16 multi-PUSCH grant, for more than one PUSCH scheduled by the same DCI, CBG retransmission should not be supported for the 60GHz band for both multi-PDSCH and multi-PUSCH grants. Otherwise, we can end up with up to four sub-codebooks based on the discussions on dynamic codebook enhancements which is too complicated and is not justified.

Regarding URLLC related fields, we support applying them for all PUSCHs/ or PUCCHs corresponding to the PDSCHs scheduled by the same DCI, otherwise, this will require defining multiple PUCCHs for the same multi-PDSCH grant which will complicate the UE processing and we do not support.

The current range of values for RBG sizes (up to 16) for FDRA, seems to be sufficient for the higher band as the FDRA field will be shared by all the PUSCHs/PDSCHs scheduled by the same DCI and we still maintain 275RBs as the maximum allowed BWP. The discussion on the FDRA optimization can be deprioritized. 
The VRB-to-PRB mapping and PRB bundling size indicator should be singled once per DCI and be applied for all granted data allocations by the same DCI. Similarly, the ZP CSI trigger should be applied for all the slots of the granted PDSCHs.

Proposal 16: For multi-PDSCH/PUSCH DCI fields enhancements:

- Second TB can be supported for each PDSCH

- MCS for the 2nd TB: This appears only once in the DCI and applies commonly to the second TB of each PDSCH

- NDI for the 2nd TB: This is signaled per PDSCH and applies to the second TB of each PDSCH

- RV for the 2nd TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the second TB of each PDSCH

- CBGTI: Not to be supported for more than one PDSCH/PUSCH for SCS 480kHz and 960kHz

- FDRA optimization can be deprioritized

- VRB-to-PRB mapping and PRB bundling size indicator: to be applied for all granted data allocations by the same DCI

- ZP CSI trigger: to be applied to all the slots granted by the same DCI.

Regarding the frequency hopping, with a single PUSCH grant both inter and intra-slot hopping should be supported similar to the legacy, while for more than one PUSCH, Rel. 16 does not support frequency hopping. Given the time frame for discussions, intra-slot hopping can be discussed for a multi-PUSCH grant with two or more PUSCHs where the hopping pattern can be repeated for all the granted slots. With larger SCS such as 960kHz, the symbol length may be very short and the time needed for frequency retuning at each frequency hop may cause some performance loss due to missing a subset of the time domain samples. One solution can be by allowing only frequency hopping based on the gaps defined by the TDRA table, i.e., frequency hops occur over the gaps between the different repetitions.  

Proposal 17: Consider the impact of RF retuning delay on the frequency hopping when operating over larger SCS

- Frequency hopping discussion can be deprioritized

#### TDRA for multi-PDSCH/PUSCH grants

For the multi-PUSCH, we agreed to use Rel-16 multi-PUSCH scheduling as a baseline, in addition to some enhancements for the DCI fields. In RAN1 #105 -e, the following agreement was made.

Agreement: - For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs), - A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots. - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

One way of configuring the TDRA table with multiple SLIVs over non-consecutive slots can be as follows. Each row in the TDRA table can be extended to include multiple SLIVs, and the SLIVs can be discontinuous in the time domain and keep a single value for k0/k2. For example, a row in the TDRA table can include k0=0, and two SLIVs, SLIV1: (S, L)=(1,4) and SLIV2: (S, L)=(4,5). Now, the second SLIV starts at symbol 4 which is covered by the first SLIV, therefore the UE can interpret by assuming the second SLIV to be granted in the slot k0+1. As illustrated in Fig. 6.

Fig. 6: Multi-PDSCH/PUSCH grant with overlapped SLIVs

With the aforementioned rule, we can have at most 13-symbol gap between two adjacent PDSCH/PUSCH allocations. To allow gaps longer than one slot, a new rule is needed. For example, we can allow SLIV value to be “0” and use it as an indication that the next SLIV will be shifted by one slot. Consider the case where a row in the TDRA table includes k0=0, and three SLIVs: and SLIV1: (S,L)=(1,4), SLIV2: 0, and SLIV3: (S,L)=(9,5). The UE can interpret this, by shifting the third SLIV to the next available slot as illustrated in Fig. 7.

Fig. 7: Multi-PDSCH/PUSCH grant with slot-level gap

Another alternative to configure the TDRA table with gaps can be by indicating the slot offset value of each SLIV, i.e., the column of k0/k2 will have multiple values equal to the number of the SLIVs in the SLIV column. This can be achieved in two ways. The first way is to directly configure a k0/k2 value for each SLIV. The second way is to configure a single value of k0/k2 for the first SLIV and to include a number of parameters (di) equals to the number of SLIVs -1 to indicate the slot offset between any consecutive time domain allocations. The two ways achieve the same goal with the same level of flexibility; however, the second way requires less overhead when configuring the TDRA table, as the range of values of di’s will be significantly smaller than the range of values of k0/k2, as di can be at most equal to the maximum value of the gap that is allowed between any two consecutive SLIVs.

Proposal 18:  For configuring a TDRA table that supports multi-PDSCH/PUSCH grants with a single DCI, two options can be considered:

- Option 1: each row in the TDRA table specifies explicitly only the slot offset of the first SLIV, i.e., a single value for defining the slot offset of the first allocation, i.e., k0/k2, and define a set of new rules

  - For overlapping SLIVs: the second SLIV to be allocated in the next slot.

  - Allow SLIV ‘0’ to indicate slot level gaps between the adjacent allocations.

- Option 2: each row specifies explicitly the slot offset of each SLIV,

  - Option 2-1: multiple values of k0/k2 equal to the number of the SLIVs

  - Option 2-2: A single value of k0/k2 to indicate the slot offset of the first SLIV and number of parameters (di)’s equals to the number of SLIVs -1, to define the slot offsets between any two adjacent SLIVs

  - Note: Option 2-2 has less configuration overhead

The gap between two consecutive allocations should be defined, however, we need also to define the maximum span of the allocations granted by a single DCI.  For example, suppose the gap between two allocations is defined not to exceed two slots, then without specifying the total span of the allocations granted with the same DCI, a single DCI can schedule 8 PDSCHs that may span 22 slots. To avoid this issue a limit should be imposed on the maximum allocation including the gaps. There are several options to impose such limit. Option 1 can be defining a total gap limit, e.g., a total gap of x slots is allowed for any multi-PDSCH/PUSCH grant. Option 2 can be defining a maximum size of each gap, and a maximum number of gaps per grant. Option 3 can be defining the total allowed span, data allocations and the gaps, that can be granted by a single DCI. This total span can take multiple values based on the number of active SLIVs per DCI.

Proposal 19: Define the maximum slot gap between any two SLIVs, it can be either SCS dependent or fixed values for all SCSs.

Proposal 20: Define a maximum allowed span per single DCI as X slots, where X >= 8.

Regarding the number of SLIVs that are allowed to occupy the same slot, we prefer to allow only a single SLIV per slot to reduce the processing complexity at least for DL transmission.

Proposal 21: The TDRA configuration should not allow scheduling more than one PDSCH per slot with a single DCI.

Before the RRC configuration, the TDRA is defined based on the default tables of PDSCH and PUSCH. The default tables of PDSCH have k0 with values 0 and 1, i.e., gNB can only schedule the UE within the same PDCCH slot or the next slot. For PUSCH default tables, the k2 values are limited based on the minimum PUSCH slot offset and it can take values j,j+1,j+2,j+3, where j depends on the SCS, i.e., gNB can only schedule the UE with PUSCH within 4 slots ahead from the minimum PUSCH scheduling offset. With multi-slot PDCCH monitoring, gNB should be able to schedule all the resources between any two adjacent PDCCH monitoring occasions. To do so, the ranges of the values of the slot offsets in the default TDRA tables need to be extended to the periodicity of the PDCCH monitoring. For example, if the UE will monitor the PDCCH every M slots, then the default TDRA tables should support slot offsets up to M-1. Therefore, the default TDRA tables need to be defined per each SCS for the new band. In addition, for the slots without PDCCH monitoring, the SLIV can be allowed to cover the whole slot, i.e., L=14.

Proposal 22: Introduce new default TDRA PDSCH and PUSCH tables depending on the used SCS, e.g., 960kHz and 480kHz SCS, to be able to schedule all the resources between any two adjacent PDCCH monitoring occasions. The slot offsets in these tables should cover up to the PDCCH monitoring periodicity. For the slots without PDCCH monitoring, L=14 can be considered.

As discussed in the scope of WI, scheduling a single TB that is repeated over multiple allocations is allowed in the new band.  To simultaneously support the ability to schedule a single TB to be repeated over multiple allocations and multiple TBs, with no repetitions, with the same TDRA table, a new signaling technique is needed. Either by modifying the TDRA table such that different rows can schedule either repetitions of the same TB, or multiple TBs without repetition or an additional bit in the DCI can indicate whether the scheduled allocations are corresponding to the same TB or different TBs. If the SLIVs have different lengths and the transmission mode will be a single TB with repetition, then a new rule is needed to determine which SLIV will be used for calculating the transport block size. For example, the TB size can be calculated either based on the shortest SLIV or the first SLIV.

Proposal 23: Support the ability to schedule a single TB to be repeated over multiple allocations and multiple TBs, with no repetitions, using the same DCI format.

- FFS: signaling details and TB size calculations.

### Timeline aspects

In the high band, multi-slot PDCCH monitoring as well as per-slot monitoring may be supported for the same SCS, e.g., 120kHz. Similarly, multi-PUSCH/PDSCH will be supported to reduce the control overhead. Such flexibility of the scheduling may help UE to save some processing by reducing the frequent retuning of its hardware, e.g., RF module. This can allow the UE to have an adaptive timeline based on the nature of the granted DCIs, e.g., multi-PDSCH or single PDSCH and the PDCCH monitoring periodicity, e.g., per-slot or multi-slot. Therefore, it will be beneficial to consider a dynamic processing timeline adaptation to be applied at gNB and UE, otherwise the timeline should be calculated based on the worst case, e.g., per-slot PDCCH monitoring.

Proposal 24: The timeline calculations need to take into account the different cases for PDCCH monitoring, i.e., per-slot or multi-slot. This will require either one of the following

- Timeline is calculated based on the worst case

- gNB and UE applies different processing timeline depending on PDCCH monitoring periodicity

Before the RRC configuration, the TDRA is defined based on the default tables of PDSCH and PUSCH. The default tables of PDSCH have k0 with values 0 and 1, i.e., gNB can only schedule the UE within the same PDCCH slot or the next slot. For PUSCH default tables, the k2 values are limited based on the minimum PUSCH slot offset and it can take values j,j+1,j+2,j+3, where j depends on the SCS, i.e., gNB can only schedule the UE with PUSCH within 4 slots ahead from the minimum PUSCH scheduling offset. With multi-slot PDCCH monitoring, gNB should be able to schedule all the resources between any two adjacent PDCCH monitoring occasions. To do so, the ranges of the values of the slot offsets in the default TDRA tables need to be extended to the periodicity of the PDCCH monitoring. For example, if the UE will monitor the PDCCH every M slots, then the default TDRA tables should supports slot offsets up to M-1. Therefore, the default TDRA tables need to be defined per each SCS for the new band. For the slots without PDCCH monitoring, the SLIV can be allowed to cover the whole slot, i.e., L=14.

Proposal 25: Introduce new default TDRA PDSCH and PUSCH tables depending on the used SCS, e.g., 960kHz and 480kHz SCS, to be able to schedule all the resources between any two adjacent PDCCH monitoring occasions. The slot offsets in these tables should cover up to the PDCCH monitoring periodicity. For the slots without PDCCH monitoring, L=14 can be considered.

In RAN1 #104-e, it was agreed to use the absolute time duration for 120 kHz SCS as the upper bound for the discussion of UE processing timelines (not related to PDCCH monitoring) for 480 kHz and 960 kHz SCS for NR operation in 52.6 to 71 GHz. However, we do not expect a significant decrease from 120kHz timeline. Thus, RAN1 should provide a valid design under this assumption.

Proposal 26: RAN1 design for 480kHz and 960kHz SCS, should assume a timeline similar to the absolute timeline of 120kHz.

# Conclusion

Observation 1: Comparing the different PTRS patterns

- The best performance is obtained with block PTRS pattern with zero-power tones.

- The legacy pattern performance is very close to block PTRS pattern with zero-power tones.

Observation 2: The block PTRS pattern with ZP tones and the legacy pattern outperform the cyclic patterns based on ZC sequences.

Observation 3: With 7 clusters each with 9 tones, 8 ZP PTRS tones and a single NZP tone in the centre, the two algorithms with the clustered pattern have almost the same performance, and they slightly outperform the legacy pattern.

Observation 4:  With 7 clusters each with 9 tones, 8 ZP PTRS tones and a single NZP tone in the centre, power boosting the NZP tone by 6 dB can matches the performance in of the legacy PTRS pattern.

Observation 5: For clustered PTRS pattern with ZP tones, power boosting NZP tones will not increase CM or PAPR.

Observation 6: For small RB allocation, e.g., 16 or 8 RBs, sending the PTRS over every RB, i.e., K=1, enhances the performance as it helps in having more accurate ICI filter calculations or CPE estimates. In addition, the performance is degraded when the density is increased with K=0.5 due to the large overhead and increase coding rate.

Observation 7: For MCS 22, there is no significant enhancement for increasing the total number of PTRS samples to 64, while small gains can be observed with MCS 24. In addition, increasing the number of PTRS samples beyond 64 is not helping the performance.

Observation 8: With total of 16 samples, the pattern with 8 chunks outperforms the one with 4 chunks, i.e., distributing samples over time domain enhances the performance, but still the legacy pattern with 32 samples is outperforming.

Observation 9: For rank 1, with SCS 960kHz, for channels with larger DS, the main reason of performance degradation with the larger SCS is the loss of orthogonality, and the gain from increasing the frequency density of the DMRS tones is limited, i.e., the performances of Config.1 with no CDMing and the new configuration with no CDMing are very close to each other.

Observation 10: For rank 1, with SCS 480kHz, even for channels with larger DS, the different DMRS patterns have very comparable performances, and the best performance is obtained Config.1 with no CDMing.

Observation 11: For rank 2, with SCS 960kHz, with DS 20ns all DMRS patterns perform the same, while with DS 50ns, there is a significant performance loss when FD-OCC is ON, and the best performance is obtained with config type 1 with ports from different CDM groups.

Observation 12: For rank 2, with SCS 480kHz, all DMRS patterns perform the same.

Proposal 1: For SCS 120kHz, the complexity of phase noise ICI compensation and its effects on the UE processing timeline need to be considered.

Proposal 2: Support introducing a new PTRS pattern, clustered based with ZP tones, to reduce the ICI compensation complexity.

Proposal 3: As PTRS enhancement for assisting ICI compensation, increasing the frequency domain density, of Rel. 15 PTRS, for small RB allocation can be considered.

Proposal 4: No need to increase the PTRS sample density for the SC-FDM waveform.

Proposal 5: Do not introduce a new pattern with DMRS tones sent over every RE, for the higher band.

Proposal 6: For DMRS enhancement for high SCSs, while communicating over channels with large DS and using high MCS, for rank 1, a single port should be used from one CDM group and the remaining ports from the same group should not be assigned to other UEs.

Proposal 7: Consider the following options to signal the information of FD-OCC OFF/ON

- Option 1: RRC configuration tied to an MCS threshold

- Option 2: DCI indication by introducing new entries to the DMRS antenna ports tables, e.g., reuse the reserved entries.

Proposal 8: Multi-PDSCH or multi-PUSCH scheduling with the same DCI should be applicable to 120kHz as well as 480 and 960kHz, though we don’t need to introduce multi-slot monitoring capability for 120KHz.

Proposal 9: A UE capability to be defined per SCS, to indicate the maximum number of supported PDSCHs/PUSCHs per single DCI for SCS 120kHz and 480kHz.

Proposal 10: In the case of increasing the HARQ processes to 32 for SCSs 480kHz and 960kHz, a UE capability should be defined such that X HARQ processes can be supported, and Y of them can do soft combining where X and Y ≥ 16.

Proposal 11: All HARQ-ACK information corresponding to different PDSCHs scheduled by the same DCI to be carried by the same PUCCH.

Proposal 12: Revisit K1 values for SCS 480kHz and 960kHz after the discussions on the PDSCH processing timeline.

Proposal 13: With Alt 1, in the case of time domain bundling of A/N bits corresponding to PDSCHs scheduled by the same DCI into one bit, a single codebook should be defined.

Proposal 14: Allowing different numbers of A/N bits per multi-PDSCH grant, such that for each A/N occasion all the corresponding multi-PDSCH grants will have the same A/N bits, however, from one A/N occasion to another we can allow different number A/N bits per grant

- If time domain bundling is enabled, then the bundling pattern can be changed from one A/N occasion to another.

  - Time-domain bundling patterns to be defined via RRC configuration and the active pattern can be changed by MAC-CE or PDCCH.

Proposal 15:

- Support increasing the field size of the DAI based on RRC configuration to increase the reliability against the missed DCIs. However, the field size increase should be subject to gNB configuration.

- Allow adjusting the resolution of the DAI counter based on the greatest common divisor of the number of the SLIVs, among the rows of the TDRA, i.e., each increment of the DAI indicates that a number of PDSCHs equal to the greatest common divisor has been sent.

Proposal 16: For multi-PDSCH/PUSCH DCI fields enhancements:

- Second TB can be supported for each PDSCH

- MCS for the 2nd TB: This appears only once in the DCI and applies commonly to the second TB of each PDSCH

- NDI for the 2nd TB: This is signaled per PDSCH and applies to the second TB of each PDSCH

- RV for the 2nd TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the second TB of each PDSCH

- CBGTI: Not to be supported for more than one PDSCH/PUSCH for SCS 480kHz and 960kHz

- FDRA optimization can be deprioritized

- VRB-to-PRB mapping and PRB bundling size indicator: to be applied for all granted data allocations by the same DCI

- ZP CSI trigger: to be applied to all the slots granted by the same DCI.

Proposal 17: Consider the impact of RF retuning delay on the frequency hopping when operating over larger SCS

- Frequency hopping discussion can be deprioritized

Proposal 18:  For configuring a TDRA table that supports multi-PDSCH/PUSCH grants with a single DCI, two options can be considered:

- Option 1: each row in the TDRA table specifies explicitly only the slot offset of the first SLIV, i.e., a single value for defining the slot offset of the first allocation, i.e., k0/k2, and define a set of new rules

  - For overlapping SLIVs: the second SLIV to be allocated in the next slot.

  - Allow SLIV ‘0’ to indicate slot level gaps between the adjacent allocations.

- Option 2: each row specifies explicitly the slot offset of each SLIV,

  - Option 2-1: multiple values of k0/k2 equal to the number of the SLIVs

  - Option 2-2: A single value of k0/k2 to indicate the slot offset of the first SLIV and number of parameters (di)’s equals to the number of SLIVs -1, to define the slot offsets between any two adjacent SLIVs

  - Note: Option 2-2 has less configuration overhead

Proposal 19: Define the maximum slot gap between any two SLIVs, it can be either SCS dependent or fixed values for all SCSs.

Proposal 20: Define a maximum allowed span per single DCI as X slots, where X >= 8.

Proposal 21: The TDRA configuration should not allow scheduling more than one PDSCH per slot with a single DCI.

Proposal 22: Introduce new default TDRA PDSCH and PUSCH tables depending on the used SCS, e.g., 960kHz and 480kHz SCS, to be able to schedule all the resources between any two adjacent PDCCH monitoring occasions. The slot offsets in these tables should cover up to the PDCCH monitoring periodicity. For the slots without PDCCH monitoring, L=14 can be considered.

Proposal 23: Support the ability to schedule a single TB to be repeated over multiple allocations and multiple TBs, with no repetitions, using the same DCI format.

- FFS: signaling details and TB size calculations.

Proposal 24: The timeline calculations need to take into account the different cases for PDCCH monitoring, i.e., per-slot or multi-slot. This will require either one of the following

- Timeline is calculated based on the worst case

- gNB and UE applies different processing timeline depending on PDCCH monitoring periodicity

Proposal 25: Introduce new default TDRA PDSCH and PUSCH tables depending on the used SCS, e.g., 960kHz and 480kHz SCS, to be able to schedule all the resources between any two adjacent PDCCH monitoring occasions. The slot offsets in these tables should cover up to the PDCCH monitoring periodicity. For the slots without PDCCH monitoring, L=14 can be considered.

Proposal 26: RAN1 design for 480kHz and 960kHz SCS, should assume a timeline similar to the absolute timeline of 120kHz.

# References

- RP-202925, “Revised WID: Extending current NR operation to 71GHz”, RAN #90e, CMCC

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71GHz”, RAN #86, Intel Corporation

- R1-2005922, “On phase noise compensation for OFDM”, RAN1 #102-e, Ericsson

- ETSI TS 138 212, “Multiplexing and channel coding” V16.2.0 (2020-07)

- R1-2100201	PDSCH/PUSCH enhancments for 52-71GHz band”, RAN #104-e	Huawei, HiSilicon

- R1-2102080 “Summary #4 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, RAN1 #104-e Moderator (LG Electronics).

- R1-2102237 “Discussion summary #3 of 104-e-NR-52-71GHz-05“, RAN1 #104-e, Moderator (vivo)

- R1-2103158 “PDCCH monitoring enhancements”, RAN1 #104-bis-e, Qualcomm

- R1-2104042 “Summary #3 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, RAN1 #104-bis-e Moderator (LG Electronics).

- R1-2105422 “Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, RAN1 #105-e Moderator (LG Electronics)

# Appendix

## CP-OFDM PTRS Evaluations

Fig. 8: ICI compensation: Different block PTRS vs. legacy PTRS with K=4

Fig. 9: ICI compensation: Different block PTRS vs. legacy PTRS with K=4

Fig. 10: Algorithm 1 vs Algorithm 2: CDL-B channel, 20ns delay spread

Fig. 11: Different levels of power boosting: MCS 22, 7 Clusters each with 9 tones (8NZP tones) CDL-B channel, 20ns delay spread

Fig. 12: Different levels of power boosting: MCS 24, 7 Clusters each with 9 tones (8NZP tones) CDL-B channel, 20ns delay spread

## DFT-s-OFDM PTRS Evaluations

Fig. 13: DFT-s-OFDM waveform: TDL-A with DS 5ns, 120kHz SCS, PTRS (L=1), 1 DMRS symbol, Rank 1

## DMRS Evaluations

Fig. 14: New DMRS pattern: TDL-A with DS 20ns, 960kHz SCS, PTRS (K=4, L=1), 1 DMRS symbol, Rank 1

Fig. 15: New DMRS pattern: CDL-B with DS 50ns, 960kHz SCS, PTRS (K=4, L=1), 1 DMRS symbol, Rank 1

Fig. 16: New DMRS pattern: TDL-A with DS 20ns, 480kHz SCS, PTRS (K=4, L=1), 1 DMRS symbol, Rank 1

Fig. 17: New DMRS pattern: CDL-B with DS 50ns, 480kHz SCS, PTRS (K=4, L=1), 1 DMRS symbol, Rank 1

Fig. 18: New DMRS pattern: MCS 16, CDL-B, 960kHz SCS, PTRS (K=4, L=1), 1 DMRS symbol, Rank 2

Fig. 19: New DMRS pattern: MCS 22, CDL-B, 960kHz SCS, PTRS (K=4, L=1), 1 DMRS symbol, Rank 2

Fig. 20: New DMRS pattern: MCS 22, CDL-B, 480kHz SCS, PTRS (K=4, L=1), 1 DMRS symbol, Rank 2