3GPP TSG-RAN WG1 Meeting #106-e	R1-2107563

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	8.8.3

Source:	Ericsson

Title:	Type A PUSCH Repetition for Msg3

# Introduction

In this contribution, we consider the support of Type A Msg3 repetitions agreed in [1] in the following aspects:

- On support of Msg3 PUSCH repetition,

- Mechanism of Msg3 PUSCH repetition initial transmission and retransmission,

- Msg3 PUSCH repetition with frequency hopping,

- Available slot determination for Msg3 repetition,

- RV cycling for Msg3 repetition.

# Discussion

## On support of Msg3 PUSCH repetition

In the RAN1 #105-e meeting, following agreements have been made regarding the early indication of Msg3 PUSCH repetition.

Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

For the shared RO case, mechanisms similar to what we’ve used for 2-step RACH can be used for PRACH preamble grouping for different features. As is known, the total number of preambles used in an RO may be optionally configured in Rel-15/16. Whether the preambles for requesting Msg3 repetition should be outside the total number of preambles configured for Rel-15/16 or should be within the total number of configured preambles should be discussed first.

Proposal 1:

- For the shared RO case,

- Similar mechanisms applied for supporting 2-step RACH can be reused for PRACH preamble grouping for requesting Msg3 repetition.

- Whether the preambles for requesting Msg3 repetition should be outside the total number of preambles configured should be discussed.

- A unified solution of preamble grouping should be supported for requesting Msg3 repetition as well as for indicating other new features in NR Rel-17.

For the details of PRACH resource configuration for the UE to request a Msg3 repetition, RAN1 also needs to decide whether separate PRACH occasions should be introduced. In our understanding, this is needed especially for the case when the preamble resources are not enough. Furthermore, with preamble partitioning currently being used for several features from Rel-16 and not to mention currently discussed RAN2 features, there is a need for flexibility in how the separate PRACH resources should be configured. And without the flexibility to configure separate ROs, there is a serious risk that the feature will be difficult to deploy.

Proposal 2:

  - A separate RO should be supported for requesting Msg3 repetition.

However, for the separate RO case, there could be an RA-RNTI overlapping issue between legacy ROs and new ROs if they’re configured independently.

As is known, the RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211, f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

So when a separate RO overlaps with a legacy RO in the time domain (i.e. it has the same t_id), the RA-RNTI derived based on the formula above may be the same for the RAR in response to the preambles received in these 2 ROs when they’re numbered separately in the frequency domain (i.e. they have the same f_id). This will cause the issue that 2 UEs transmitting with same preamble ID in the 2 ROs may detect 2 RARs, but which one is correct (e.g. which timing advance command received in which RAR that each of the 2 UEs should use for Msg3 transmission) is not clear.

To avoid such RNTI overlapping issues, in 2-step RACH, it was agreed to add a large RNTI offset to RA-RNTI to derive MsgB-RNTI, but it’s hard to do so considering in Rel-17 more features are requiring early PRACH indication. Another way could be that we can restrict the ROs to be only TDMed between Rel-15/16 ROs and Rel-17 ROs. Or we support ROs FDMed with common f_id for RA-RNTI calculation, and the f_id is numbered together among all FDMed ROs for both Rel-15/16 and Rel-17 ROs, but the total number of ROs FDMed should not be more than 8 if we want to keep such restriction as in Rel-15/16.

Similar to the shared RO case, since there are also other features requiring PRACH partitioning with separate ROs, e.g. for indicating a network slicing, for RedCap, or for RA-SDT, it would be good to have an unified solution to address the RA-RNTI overlapping issue.

Proposal 3:

  - For the separate RO case, PRACH configurations should be carefully designed to avoid RA-RNTI overlapping issues, a unified solution for indicating all new features requiring early indication via PRACH transmission in NR Rel-17 is pursued.

In the RAN1 #105-e meeting, the following agreement has been made regarding the criteria for requesting Msg3 repetition.

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. - FFS the determination of the RSRP threshold.

According the agreement, when determining whether Msg3 repetition shall be requested, the UE needs to do an RSRP check, however the details on how to define the RSRP threshold and at which stage during the random access procedure the RSRP checking should be performed are not clear. This may need to be further discussed in RAN2. Some observations and proposals are provided in our RAN2 paper [3].

Proposal 4:

- RAN2 is to discuss the details of how the procedure using the RSRP threshold shall work for determining whether Msg3 repetition shall be requested or not.

## Mechanism to support Msg3 PUSCH repetition

Regarding the mechanism of supporting Msg3 PUSCH repetition, the following agreements have been made in the RAN1 #104bis-e meeting:

Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted. - FFS additionally using MAC RAR for indication. Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted.

In the RAN1 #105-e meeting, the following agreement and working assumption were made:

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetitions of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetitions of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field - The total size of RAR UL grant does not change. - Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not. - FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

In Rel-16, the number of repetitions for a dedicated PUSCH can be configured in the time domain allocation list or semi-statically configured by RRC according to section 6.1.2.1 of 38.214 V16.4.0.

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions-r16 is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions-r16; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1.

To support dynamic repetition of Msg3, a repetition indication (i.e. whether repetition is on or off) and the number of repetitions for Msg3 can be indicated in the SIB1 configured time domain allocation list, and/or in the RAR for initial transmission and/or in DCI for retransmission respectively.

When the repetition is indicated in RAR or DCI, some of the reserved bits or some used bits in RAR or DCI should be used together to indicate the repetition so that the size of RAR and DCI are not extended since a gNB may enable Msg3 repetition but doesn’t know whether the UE supports Msg3 repetition. Also, if the RAR or DCI payload size is different from Rel-16, Rel-16 UEs will not be able to interpret the RAR or DCI, so such a change is not backward compatible.

To minimize changes to DCI (for Msg3 retransmission) or RAR (for Msg3 initial transmission), it is preferred to signal the repetition factor in the TDRA list following the same way as used for PUSCH repetition in NR Rel-16.

Regarding the TDRA table selection, it can be based on the PRACH resource selected by UE. When a UE is detected on PRACH resource requesting Msg3 repetition, the UE should assume the new TDRA table with Msg3 repetition factors should be used, otherwise UE should assume legacy TDRA table is used. Even if the new TDRA table is selected, a repetition factor 1 can be used to disable msg3 repetition.

Proposals 5 to 7:

  - Use the same mechanism for indicating Msg3 repetition for both Msg3 initial transmission and Msg3 retransmission

  - The Msg3 repetition factor is signaled in the TDRA table and the repetition flag can be signaled in DCI or RAR.

  - When a PRACH resource for requesting Msg3 repetition is selected by the UE for PRACH transmission, the UE should assume a new TDRA table with Msg3 repetition factors should be used. Otherwise, the UE will assume a legacy TDRA table is used.

## Msg3 PUSCH repetition with frequency hopping

Inter-slot frequency hopping has been agreed to be supported when Msg3 is repeated in the RAN1 #104-e meeting.

Agreements: Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission. FFS details, e.g., signaling etc.

And in the RAN1 #105-e meeting, following conclusion was made:

Conclusion: - Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously)

Up to NR Rel-16, Msg3 can support intra-slot frequency hopping. However, a drawback of intra-slot FH is that it reduces the amount of filtering (averaging) of the channel estimate in time that can be performed (since such filtering is normally not possible when different frequency-domain resources are used). The reduced filtering has a negative impact on performance.

A better option may therefore be to use a single frequency within a slot and instead make the frequency hops in between slots. Note that for Msg3, each repetition occupies only a single slot, this means frequency hopping is performed between repetitions.

At low speed, inter-slot FH should give similar gain as intra-slot FH, at least as long as the total number of different frequencies is the same. However, and advantage of inter-slot FH is that it could yield better channel estimation performance thanks to allowing channel filtering (averaging) in the time domain over a larger duration (a whole slot instead just part of a slot).

Furthermore, if multiple repetitions are on the same frequency, the slots using the same frequency could be arranged consecutively in order to maximize the potential gains from cross-slot time-domain channel filtering.

The issue that has been discussed in the RAN1 #105-e meeting is whether we should support both intra-slot and inter-slot frequency hopping when repetition is enabled for Msg3.

A comparison between inter-slot and intra-slot FH is shown in Figure 1, in a TDD setting with 4 repetitions, FH over 2 frequencies, and no cross-slot channel filtering. Detailed assumptions are provided in Table 1 in the Appendix. As can be seen, there is a gain of around 1 dB gain from repetition with inter-slot FH compared to repetition with intra-slot FH. Note that we used 4 DMRS symbols for intra-slot hopping, while only the customary 3 DMRS symbols were used for inter-slot hopping. If only 3 DMRS symbols are used also for intra-slot FH (1 DMRS in first hop, 2 DMRS in second, according to table 6.4.1.1.3-6 of 38.211), then intra-slot FH performs even worse because of substantially worse channel estimation in the hop with only 1 DMRS. In [2], when joint channel estimation is enabled, inter-slot FH was also found to perform better than intra-slot FH in the investigated scenario.

Figure 1. Msg3 repetition with inter-slot FH or intra-slot FH.

Observations 1 & 2:

  - Inter-slot frequency hopping allows channel filtering (averaging) in time domain over a larger duration compared to intra-slot frequency hopping.

  - Initial link level results show 1 dB gain from inter-slot hopping over two frequencies compared to repetition with intra-slot FH.

According to above observations, we do not see the need to support intra-slot FH when Msg3 is repeated. On the other hand, when only one FH type is supported in case Msg3 is repeated, the signaling overhead to for indication of FH and repetition will be reduced, which is essential to make backward-compatible DCI/RAR updates.

So, we have following proposal.

Proposal 8:

  - Intra-slot FH is not supported when Msg3 is repeated.

## Available slot determination

In the RAN1 #105-e meeting, the following agreements were reached for counting the repetitions of a Msg3 transmission based on available slots.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0. Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI. Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

Regarding how to use flexible symbols indicated by TDD-UL-DL-Configcommon, according to current specifications, flexible symbols are possible to be used for transmission of Msg3, which means that a slot with consecutive allocated flexible symbols for Msg3 transmission can be treated as an available slot.

Proposal 9:

- Flexible symbols indicated by TDD-UL-DL-Configcommon can be treated as available symbols for available slot determination for Msg3 repetition.

However, the proposal above doesn’t mean that Msg3 shall be actually transmitted on an available slot. For example, when a cancellation indication is received to drop the uplink transmission in one available slot, the Msg3 transmission may or may not be dropped depending on the priority of Msg3 PUSCH repetition, which should be discussed further.

Furthermore, although it is true that based on current specifications, without repetition, Msg3 PUSCH is not expected to have a set of allocated flexible symbols to be indicated as downlink by dynamic SFI, see text in table below, with repetition of Msg3, if some of the available slot for repetitions (not the first repetition) can be expected to be changed to be downlink, the corresponding repetitions will also be dropped.

- a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot

On top of above, when Msg3 repetition collides with other uplink channels, which uplink should have higher priority over other channels should be further discussed.

Proposal 10:

- RAN1 is to further discuss whether Msg3 repetition can be cancelled by dynamic cancellation indication, whether flexible symbols for Msg3 repetition can be dynamically changed to be downlink, and the collision rules when Msg3 repetition collides with other UL channels and signals.

## RV cycling for Msg3 PUSCH repetition

In the RAN1 #105-e meeting, the following agreements have been made with respect to the redundancy version of the first Msg3 repetition for initial or retransmission of Msg3.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission. - The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214. - The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214. - FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Since Msg3 repetition is expected to be based on available slots, the RV determination should be based on the available slots instead of the actual Msg3 repetition transmissions, as the gNB and the UE may have different understandings of the actual repetition transmissions for Msg3.

Proposal 11:

- RV cycling Msg3 PUSCH repetition should be based on available slots.

# Summary

In this contribution, we consider the remaining issues for the support of Type A Msg3 repetitions. We have following observations and proposals based on the discussions.

Observations:

- Inter-slot frequency hopping allows channel filtering (averaging) in time domain over a larger duration compared to intra-slot frequency hopping.

- Initial link level results show 1 dB gain from inter-slot hopping over two frequencies compared to repetition with intra-slot FH.

Proposals:

- For the shared RO case,

  - Similar mechanisms applied for supporting 2-step RACH can be reused for PRACH preamble grouping for requesting Msg3 repetition.

  - Whether the preambles for requesting Msg3 repetition should be outside the total number of preambles configured should be discussed.

  - A unified solution of preamble grouping should be supported for requesting Msg3 repetition as well as for indicating other new features in NR Rel-17.

- A separate RO should be supported for requesting Msg3 repetition.

- For the separate RO case, PRACH configurations should be carefully designed to avoid RA-RNTI overlapping issues, a unified solution for indicating all new features requiring early indication via PRACH transmission in NR Rel-17 is pursued.

- RAN2 is to discuss the details of how the procedure using the RSRP threshold shall work for determining whether Msg3 repetition shall be requested or not.

- Use the same mechanism for indicating Msg3 repetition for both Msg3 initial transmission and Msg3 retransmission

- The Msg3 repetition factor is signaled in the TDRA table and the repetition flag can be signaled in DCI or RAR.

- When a PRACH resource for requesting Msg3 repetition is selected by the UE for PRACH transmission, the UE should assume the new TDRA table with Msg3 repetition factors should be used. Otherwise, the UE will assume a legacy TDRA table is used.

- Intra-slot FH is not supported when Msg3 is repeated.

- Flexible symbols indicated by TDD-UL-DL-Configcommon can be treated as available symbols for available slot determination for Msg3 repetition.

- RAN1 is to further discuss whether Msg3 repetition can be cancelled by dynamic cancellation indication, whether flexible symbols for Msg3 repetition can be dynamically changed to be downlink, and the collision rules when Msg3 repetition collides with other UL channels and signals.

- RV cycling Msg3 PUSCH repetition should be based on available slots.

# References

- RP-211566, “Revised WID on NR coverage enhancements”, China Telecom, 3GPP TSG RAN meeting #92-e, June 14th – 18th, 2021.

- R1-2107561, “Joint Channel Estimation for PUSCH”, Ericsson, 3GPP RAN1 #106-e meeting, August 16th - August 27th, 2021.

- R2-2108273, “On RAN2 impacts for coverage enhancements and Type A PUSCH repetitions for Msg3”, Ericsson, 3GPP RAN2 #115-e meeting, August 16th - August 27th, 2021.

# Appendix

Table 1: Basic setup of LLS for Msg3 PUSCH repetition and frequency hopping

| System | - Carrier frequency 4 GHz - 30 kHz SCS - TDD: DDDSU - Inter-slot FH is over 2 frequencies in 100 MHz BWP (273 PRBs) - 2 PRB allocation - 16 repetitions of repetition Type A (i.e. 4 actual repetitions) - 3 DMRS symbs. per slot for inter-slot FH, 4 DMRS symbs. per slot for intra-slot FH according to Table 6.4.1.1.3-6 in 38.211 V16.4.0 |
| --- | --- |
| Channel | - TDL-C, delay spread 30 ns, 3 km/h |
| Antennas | - 1T4R |
| Receiver | - Practical channel estimation, no cross slot channel estimation |
| Payload | - 7 bytes, 2 PRBs (MCS 0) - DFT-spread OFDM |