3GPP TSG-RAN WG1 Meeting #106-e		R1-2107954

e-Meeting, August 16th – 27th, 2021

Title:	[DRAFT] Relay LS on R16 V2X for PUCCH reporting and for minimum time gap

Response to:	R2-2104475

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	vivo [RAN1]

To:	RAN2

Cc:

Contact person:	Zichao Ji

zichao.ji@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 thanks RAN2 for the LS informing the working assumption and agreement on R16 V2X for PUCCH reporting and for minimum time gap. RAN1 would like to provide answers to the following questions:

Working assumption: “UE assumes that next retransmission(s) of the MAC PDU is required when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources, when sl-CG-MaxTransNum is not reached”

  - Question 1: RAN2 respectfully requests RAN1 to provide feedback on the working assumption above in case of any concern.

    - Answer: RAN1 has no consensus in terms of any concern on RAN2’s working assumption above.

5.22.1.2 TX resource (re-)selection check […] 1> if retransmission of a MAC PDU on the selected sidelink grant has been dropped by either sidelink congestion control as specified in clause 8.1.6 of TS 38.214 or de-prioritization as specified in clause 16.2.4 of TS 38.213 [6], clause 5.4.2.2 of TS 36.321 [22] and clause 5.4.2.2: 2> remove the resource(s) from the selected sidelink grant associated to the Sidelink process, if the resource(s) of the selected sidelink grant is indicated for re-evaluation or pre-emption by the physical layer; 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of a SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];

  - Question 2: RAN2 respectfully requests RAN1 to provide feedback in case of any concern on the MAC specification above.

    - Answer: RAN1 confirms that current MAC specification is not aligned with a prior RAN1 agreement. However, after intensive discussion, RAN1 has no consensus in terms of a subsequent action (e.g., to revert the prior RAN1 agreement, or to request RAN2 to update MAC specification).

2	Actions

To RAN2

RAN1 respectfully asks RAN2 to take the above information into account in related work.

3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #106bis-e			October 11th – 19th, 2021 			e-meeting

TSG RAN WG1 Meeting #107-e			November 11th – 19th, 2021   			e-meeting