3GPP TSG-RAN WG1 #106-e		 	R1- 2108015

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.2.2

Title:	PDCCH Monitoring for NR from 52.6 GHz to 71 GHz

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Agreements:

Previous agreement is modifed as follows:

Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability.

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately

  - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X

  - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group

  - FFS: Further definition of capabilities

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

  - X is the minimum time separation between the start of two consecutive spans

  - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots]

  - Y <= X

  - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary.

  - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots.

  - FFS: Further definition of capabilities

- Alt 3: Use a sliding window of X slots as the baseline to define the new capability.

  - The capability indicates the BD/CCE budget within the sliding window

  - The sliding unit of the sliding window is [1] slot.

  - FFS: Further definition of capabilities

- Specific numbers for X, Y may depend on UE capability and gNB configuration

  - Examples:

    - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

Conclusion:

For 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed.

Agreement:

For 120 kHz SCS in 52.6-71 GHz, the BD/CCE budget is the same as that for 120 kHz in FR2.

In RAN 1 meeting #104bis-e, the following agreements for supporting NR from 52.6 GHz to 71 GHz were made [1]:

In this contribution, we discuss several aspects of the PDCCH design for supporting NR on frequencies from 52.6 GHz to 71 GHz.

# Supporting NR Operation from 52.6 GHz to 71 GHz

Frequency bands above 52.6 GHz have several advantages such as limited interference, wider bandwidth, high-gain beamforming, multi-beam systems, etc. Therefore, use cases like eMBB with high data rate, short range D2D communications with high data rate, wireless display transfer for AR/VR, etc. can be enabled. To support NR from 52.6 GHz to 71 GHz, operation on both licensed and unlicensed spectrum with a range of ultra-wide-bandwidth frequency bands needs to be considered. For example, the entire 57 GHz – 71 GHz frequency range is available in USA, providing 14 GHz of contiguous bandwidth. In addition, the 66 GHz to 71 GHz band was recently opened for licensed use in international telecommunications union (ITU) Regions 1 and 3.

In RAN1 #103e meeting, subcarrier spacings (SCS) of 120 KHz, 240 KHz, 480 KHz and 960 KHz (i.e., μ= 3, 4, 5 and 6 respectively) for NR operation both for licensed and unlicensed bands from 52.6 GHz to 71 GHz has been agreed.  When the larger SCSs/numerologies are introduced, the duration of slot in a subframe will be decreased accordingly. Due to the limited PDCCH processing capability, the number of monitored PDCCH candidates and the number of non-overlapped CCE per slot are expected to be decreased for the higher SCSs/numerologies (e.g., SCS = 480 KHz, 960 KHz, etc.) scenarios in frequency range greater than 52.6 GHz. Therefore, the decreased number of PDCCH candidates and CCEs per slot may limit the scheduling flexibility. Besides, monitoring every slot for PDCCH becomes too frequent and may consume too much UE power in higher frequency range. Besides, link budget reduces roughly by 3dB when the subcarrier spacing doubles. Therefore, PDCCH coverage is degraded when the higher SCSs/numerologies are introduced for NR 52.6 GHz and above.

In the following sections, we discuss the PDCCH monitoring for NR operation from 52.6 GHz to 71 GHz.

## PDCCH monitoring consideration for NR form 52.6 GHz to 71 GHz

Due to the limited PDCCH processing capability, the number of monitored PDCCH candidates and the number of non-overlapped CCE per slot are expected to be decreased for the higher SCSs/numerologies (e.g., SCS = 960 KHz). To meet the scheduling requirement as the lower SCSs/numerologies, one approach is to monitor PDCCH in every slot like Rel-15/16. However, this kind of configuration may consume lots of power for a UE especially with the higher SCSs/numerologies for NR from 52.6 GHz to 71 GHz.

Like Rel-16 URLLC PDCCH monitoring span (X, Y) definition, it can be extended to the mobile broadband (EMBB) service for NR from 52.6 GHz and above with few modifications. Like Alt-2, the PDCCH monitoring span (X, Y) for higher SCS/numerology (e.g., SCS 480KHz, 960 kHz) where the first number X is the number of slots between the beginning of two consecutive monitoring occasions, the second number Y is the number of slots or symbols needs to be monitored in a monitoring occasion.  Rel-16 PDCCH/DCI span, it supports limited span like (X, Y) = (2, 2), (4, 3), and (7, 3). Note in Rel-16, the value of X and Y is based on units of symbols. Therefore, the X and Y supported in Rel-16 may not be suitable for NR from 52.6 GHz and above. For NR from 52.6 GHz and above, the duration per span may be across several slots to meet the scheduling requirement due to the number of PDCCH candidate and nonoverlapping CCEs being reduced per slot. The UE can be configured by gNB to monitor PDCCH with the maximum number of PDCCH candidates and nonoverlapping CCEs defined per slot as in NR Rel-15/16 or defined per span for the maximum number of PDCCH candidates and non-overlapping CCEs defined per span. An example of a PDCCH monitoring span shown in Figure 1. In Figure 1, we assume a configuration of PDCCH monitoring span for SCS = 960 KHz. For this example, let a span (X=8, Y=4) is configured, note the unit for X and Y can be either based on number of slots or symbols. it means there are PDCCHs need to be monitored in Y=4 slots and each PDCCH monitoring occasion are separated by X= 8 slots.

Figure 1: An exemplary PDCCH monitoring span for NR from 52.6 GHz to 71 GHz.

Proposal 1. PDCCH monitoring can be either based on per slot as in Rel-15/16 for lower SCS (e.g., 120 KHz), or per span (multi slots as Alt-2) for NR from 52.6 to 71 GHz.

The UE can be configured by the gNB to monitor PDCCH for the maximum number of PDCCH candidates (MPDCCHmax,span,μ) and nonoverlapping CCEs (CPDCCHmax,span,μ) defined per span like in Rel-16. In each PDCCH monitoring span, the number of PDCCH candidates and nonoverlapping CCE cannot exceed the UE capability. Therefore, UE behavior can be similar to legacy NR specification even when there is a overbooking. For example, the UE and gNB can map PDCCH candidates in each PDCCH monitoring span as the following mapping rules in legacy NR specification: (1) CSS sets are mapped before USS sets; (2) USS sets are mapped in ascending order of the SS set indices, and if the number of PDCCH candidates/CCEs exceeds either of the UE processing limits, etc.

Alt-3 proposes the multi-slot span with the concept of the "sliding window”. The motivation is to limit the PDCCH processing loads on the UE over any sliding window of, say, B slots. The maximum BD/CCEs can be distributed by gNB configuration to (1) all in one slot or (2) over several slots while respecting the maximum capability constraints over any sliding window of B slots. One of the advantages is that the loading can be evenly distributed from gNB perspective. The sliding window can be treated as a time offset (number of slots) configuration for USS from the beginning of span. Also, the multi-slot span with the concept of the "sliding window” can be implemented by the PDCCH monitoring span pattern (X, Y) when the following two conditions are satisfied, i.e., X and Y are defined in terms of slots, and X = Y. Therefore, Alt-3 can be treated as a special case of Alt-2. In addition, the “sliding window” (e.g., the USS stating time-offset) for each UE is configured by gNB and UE just follows the configuration to perform PDCCH monitoring in each PDCCH monitoring span.

Proposal 2. PDCCH monitoring per span with sliding window (i.e., Alt-3) can be up to gNB configuration for NR from 52.6 to 71 GHz.

# Conclusions

In this contribution, we discussed the fundamental aspects of PDCCH monitoring for supporting NR from 52.6 to 71 GHz. The following proposals were made:

Proposal 1. PDCCH monitoring can be either based on per slot as in Rel-15/16 for lower SCS (e.g., 120 KHz), or per span (multi slots as Alt-2) for NR from 52.6 to 71 GHz.

Proposal 2. PDCCH monitoring per span with sliding window (i.e., Alt-3) can be up to gNB configuration for NR from 52.6 to 71 GHz.

# References

- Chairman Notes, RAN1 #104bis-e, April 12th – 20th, 2021.