3GPP TSG RAN WG1 Meeting #106-e	R1-21xxxxx

E-Meeting, August 16th – August 27th, 2021

Agenda Item:	6

Source:	Moderator (Huawei)

Title:	Feature lead summary on 106-e-LTE-6CRs-04

Document for:	Discussion and Decision

# Introduction

This documents provides the summary of discussions on the corresponding email discussion, regarding the proposed CR in [1].

[106-e-LTE-6CRs-04] Email discussion/approval on distinguishing between PUR and SPS PUSCH for eMTC (R1-2108194) – Yubo (Huawei)

- Issue 5: distinguishing between PUR and SPS PUSCH for eMTC

- Discussion and decision by August 18, CR by August 20, final check by August 24

# Discussion

In [1], a correction to distinguish between PUSCH in PUR and SPS PUSCH for eMTC is proposed, with following motivation.

When PUR was introduced, the term “PUSCH transmission using a preconfigured uplink resource” is used to refer to a PUR PUSCH. However, as there is no corresponding MPDCCH for a PUR PUSCH either, the term “PUSCH without a corresponding MPDCCH” also covers the PUR PUSCH. As a result, there is ambiguity between PUR PUSCH and SPS PUSCH in the spec.

A TP is proposed:

=========================Text proposal to TS 36.213==============================

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged part omitted>

For BL/CE UEs, the set of BL/CE UL subframes is indicated as follows

-	If UL resource reservation is enabled for the UE as specified in [11],

-	for PUSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUSCH transmission without a corresponding MPDCCH or preconfigured uplink resource,

-	if the Resource reservation field in the DCI is set to 0, then the set of BL/CE UL subframes corresponds to all uplink subframes during the PUSCH transmission;

-	if the Resource reservation field in the DCI is set to 1, then the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUSCH transmission are reserved in the subframe);

-	for PUCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUSCH transmission without a corresponding MPDCCH,

-	the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUCCH transmission are reserved in the subframe).

<Unchanged part omitted>

For BL/CE UEs, and for a PUSCH transmission starting in subframe n+ k0 without a corresponding MPDCCH or preconfigured uplink resource, the UE shall adjust the PUSCH transmission in subframe(s) n+ki with i = 0, 1, …, N-1, where

-	0≤k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the activation DCI, where are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the activation DCI is using uplink resource allocation type 5,  where  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s), and in case N=1, k0=0;

<Unchanged part omitted>

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the SPS C-RNTI, the UE shall decode the MPDCCH according to the combination defined in Table 8-5B and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. 
The scrambling initialization of this PUSCH corresponding to these MPDCCHs and PUSCH retransmission for the same transport block is by SPS C-RNTI. The scrambling initialization of initial transmission of this PUSCH without a corresponding MPDCCH or preconfigured uplink resource and the PUSCH retransmission for the same transport block is by SPS C-RNTI.

======================End of Text proposal to TS 36.213===========================

Please input your comment on the motivation and TP above:

| Companies | Comments |
| --- | --- |
| Nokia, NSB | We are OK in principle with the changes. Suggest changing the wording to “or using preconfigured uplink resource”. |

# Summary

# References

- R1-2108194	Discussion on distinguishing between PUR and SPS PUSCH for eMTC	Huawei, HiSilicon