3GPP TSG RAN WG1 Meeting #106bis-e	R1-2108727

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.3.1.2

Source:	Huawei, HiSilicon

Title:	CSI feedback enhancements

Document for:	Discussion and Decision

Introduction

Based on the conclusion in RAN1#106e, delta-MCS is precluded from Rel-17. In addition, sub-band CQI reporting has been agreed to be configurable with 2 or 4 bits as subject to UE capability. In this paper we discuss the last remaining issue whether the wideband CQI report can be omitted for sub-band CQI reporting.

Discussion about enhanced sub-band CQI reporting

In RAN1#106e, 4-bits sub-band CQI reporting is supported. The remaining question is whether wideband CQI can be omitted for the reported sub-band or not.

Agreement For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration. - This feature is subject to UE capability - FFS: Whether wideband CQI report can be omitted

Legacy 2-bits D-CQI sub-band reporting

In the Rel-16 specification, the sub-band CSI feedback is based on differential feedback, and a 2-bit delta CQI index over the wideband CQI is reported. Thus, for the legacy 2-bit differential sub-band CQI, also the wideband CQI has to be reported, since the absolute sub-band CQI value is derived from the wideband CQI and the reported differential value for the given sub-band.

Observation 1: For legacy 2-bit D-CQI sub-band reporting, the wideband CQI report is needed to obtain the absolute sub-band CQI which is calculated from the wideband CQI and the reported D-CQI value for the given sub-band.

4-bits CQI sub-band reporting

If 4-bits sub-band CQI is reported, the gNB obtains the absolute sub-band CQI value directly from the 4-bits report, without the need for a wideband CQI reference. From that perspective, the wide-band CQI could be omitted.

On the other hand, it might still be useful to report the wideband CQI in order to preserve some CSI information about sub-bands when there are not sufficient UCI resources available to carry the full payload. In that case, even if sub-band CQI is dropped, the wideband CQI still provides some CSI information.

From our perspective, the overhead saving that would come from omitting the wideband-CQI is not critical for 4-bit sub-band CQI, since in that case 2-bit D-CQI should be used anyway. But also the potential problems when a sub-band CQI is dropped are very limited and can be prevented by gNB implementation. What should be avoided, however, is an over-optimization of this non-critical aspect which would cause unjustified standardization and implementation complexity.

We are open to go either way, but for simplicity we slightly prefer to not omit the wideband CQI report. We are therefore making the following proposal:

Proposal 1: When 4-bit sub-band CQI is configured, for simplicity we have a slight preference to not omit the wideband CQI.

Conclusions

According to the discussion, following observation and proposal are provided:

Observation 1: For legacy 2-bit D-CQI sub-band reporting, the wideband CQI report is needed to obtain the absolute sub-band CQI which is calculated from the wideband CQI and the reported D-CQI value for the given sub-band.

Proposal 1: When 4-bit sub-band CQI is configured, for simplicity we have a slight preference to not omit the wideband CQI.

- Appendix – Previous agreements

From RAN1#102-e:

Agreements:

- CSI feedback enhancement for Multi-TRP transmission is not to be discussed further under IIoT/URLLC enhancement WI

Agreements:

- Baseline assumptions are used as the required minimum to be simulated for the evaluation of candidate CSI enhancement schemes

  - Reuse the assumptions in TR 38.824 and TR 38.901 as a starting point

  - Companies shall report additional parameters (e.g., CSI measurement settings, CSI reporting schemes) used in their evaluation

  - FFS details of baseline assumptions

- Companies can bring additional simulation results with other set(s) of assumptions

Agreements:

- Study/evaluate further on following CSI enhancement schemes in terms of technical benefit, specification and implementation impacts.

  - New triggering methods for A-CSI and/or SRS

  - New reporting based on one or more of the following:

    - Case 1: channel/interference measurement for new CSI reporting, considering aspects such as one or more of the following:

      - Reporting more accurate interference characteristics

      - Reduced CSI feedback overhead (e.g., reporting interference measurement only)

      - Enhanced CSI reporting such as WB/SB CQI

    - Case 2: other measurement (other than channel/interference) for additional information

      - E.g., PDCCH/PDSCH decoding, recommended HARQ RV sequence, etc.

    - It targets to help gNB scheduler for better link adaptation of (re)transmission

  - [Reduced CSI computation time/complexity]

  - [CSI feedback for PDCCH]

  - Other CSI enhancement schemes that enable accurate MCS selection are not precluded

- Detailed assumptions of the proposed CSI enhancement schemes should be provided by the proponent, such as

  - Reporting values

  - Triggering conditions for the reporting

  - Associated measurement resource

  - Uplink resource to be used for the reporting

  - How to use the reported information at the gNB scheduler

  - CSI-RS overhead and CSI reporting frequency

  - CSI reporting latency/timeline

  - Etc.

Agreements:

- Consider Table 1 as baseline assumption for system level simulation for evaluating CSI enhancement schemes

  - The uses cases in Table 1 is for simulation purposes and it does not preclude a CSI enhancement scheme which is beneficial for the other URLLC use cases

- No baseline assumption is used for link level simulation

  - Companies are encouraged to use one of LLS assumption tables in Section A.3 in TR38.824 for any link level simulation

Table 1. Baseline SLS assumption for CSI enhancement schemes in URLLC/IIoT

| Parameters | Values |
| --- | --- |
| Performance metric | Option-1 (section 5.1 of TR 38.824) Additional metrics (it is up to company to bring results with additional metric): - MCS prediction error (e.g., difference of a scheduled MCS and an ideal MCS) - DL/UL signaling overhead - CCDF of latency samples from all UEs - BLER of 1st transmission - Resource utilization - Spectral efficiency |
| Use cases | Following two use cases can be considered for new triggering method and new reporting. Companies are encouraged to evaluate the following cases in descending priority: - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 4ms (200bytes) - Traffic mode: FTP model 3 (100p/s) - Factory automation in TR 38.824 - Reliability: 99.9999 - Latency: 1ms (32bytes) - Traffic mode: Periodic deterministic traffic model with arrival interval 2ms - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 1ms (32bytes) - Traffic mode: FTP model 3 (100p/s) - Assumptions for eMBB and URLLC UEs sharing the same carrier is used (as in A2.5 of TR 38.824) |
| Simulation assumptions | Following simulation assumption is used based on the use case selected: - Rel-15 enabled use case with UMa (Table A.2.4-1 in TR 38.824) - Factory automation at 4GHz (Table A.2.2-1 in TR38.824) with following update: - Channel model is replaced with InF (InF-DH) in TR 38.901 - Companies can bring results with other InF scenarios additionally - Layout is replaced with BS deployment in Table 7.8-7 in TR 38.901 |
| Transmission scheme | Multiple antenna ports Tx scheme - Companies report the details of Tx scheme used |

From RAN1#103-e:

Agreements

- No change of CSI processing time relative to Rel-16 CSI in this WI

- CSI processing time specific to a new CSI reporting quantity/type (if supported) can be studied

Agreement:

- For Case-2 new reporting, continue studying with focus on the new reporting type based on PDSCH decoding for OLLA performance enhancement for initial and re-transmissions of PDSCH.

Agreements:

For Case-1 New reporting, the following candidate schemes have been identified to address the fast interference change over time. Continue studying with focus on the identified schemes below for further study and evaluation.

- Scheme 1a: New reporting quantity based on CQI/SINR statistics, e.g.,

  - CQI/SINR statistics (e.g., mean, variance, etc.)

  - CSI prediction

- Scheme 1b: New reporting quantity of interference statistics (e.g., mean, variance, interference covariance matrix, etc.)

- Scheme 1c: New reporting quantity based on modifying existing reporting format, e.g.,

  - CQI reporting considering the worst subbands

  - Subband CQI granularity enhancement

- Scheme 1d: New reporting quantity related to CSI expiration time

- Scheme 1e: New reporting quantity with partial information update, e.g.,

  - CSI reporting with interference update only

Companies are encouraged to investigate the above schemes, aiming for down-selection in RAN1#104-e

From RAN1#104-e:

Conclusion: Continue evaluation of new reporting Case 1 and Case 2 for the schemes identified in Appendix B of R1-2102131.

- Companies are encouraged to provide their views on each scheme against each criterion in respective Tables in Appendix B.

- Companies are encouraged to provide additional evaluation results for as many schemes as possible, based on assumptions agreed in RAN1#102-e.

- Aim for down-selection at RAN1#104-b-e by taking into account evaluation results and assessment against criteria from Appendix B.

From RAN1#104bis-e:

Conclusion:

For new reporting Case 1, do not consider further the following schemes:

- Case 1-2: CSI prediction

- Case 1-4: Interference covariance matrix

- Case 1-9: Reference wideband CQI excludes worst sub-bands

- Case 1-10: CSI expiration time

Agreements:

For new reporting Case 2, focus study on reporting of delta-CQI/MCS (Case 2-3):

- Note: this delta-CQI/MCS is determined based on UE implementation (for example, using SINR, LLR, raw BER, flipped bits, LDPC iterations, BLEP, # fail parity checks, etc.)

  - Companies are encouraged to provide more details in their analysis

- FFS: Granularity of new report type (e.g. units of CQI or MCS, how many bits)

- FFS: Whether quantity reported is relative to the scheduled MCS

Agreement: Focus study on the following for new reporting Case 1:

- Reporting of new metric, where new metric shall be determined based on network configured channel and interference measurement interval (multiple CMR and/or IMR instances) to enable accurate MCS selection.

  - Downselect by RAN1#105 to at most a single method from the following options:

    - Mean-CQI/SINR and stdev-CQI/SINR (FFS details)

    - CSI based on worst IMR occasion (FFS details)

    - Interference standard deviation (FFS details)

    - Worst-M CQI (FFS details)

  - FFS: Whether network configured channel and interference measurement interval can also be applied to existing CSI type

- Increasing granularity of subband CQI (e.g. 3-bits differential subband CQI or 4-bits full subband CQI).

- Updating only CQI in a report, where CQI is conditioned on a previous instance in which RI/PMI/(CRI) is updated.

  - Applicable for same reporting quantity as R16 for CQI.

  - FFS: Whether network configured channel and interference measurement interval can also be applied

  - FFS: Whether RI/PMI/(CRI) is transmitted in a report where only CQI is updated

  - FFS: whether the CQI processing time can be is reduced compared to Rel-16 CSI processing delay

From RAN1#106-e:

Agreement

For subband CQI reporting with more than 2 bits per subband

- Support 4-bits CQI only

Agreement

For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration.

- This feature is subject to UE capability

- FFS: Whether wideband CQI report can be omitted