3GPP TSG RAN WG1 Meeting #106bis-e	R1-2109151

e-Meeting, October 11th – October 19th, 2021

Agenda Item:	8.17.9

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NB-IoT and LTE-MTC enhancements

Document for:	Discussion and Decision

- Introduction

This contribution discusses the UE features for NB-IoT and LTE-MTC enhancements.

- Rel-17 UE features

In email discussion post RAN1#106-e, a preliminary version is provided for Rel-17 UE features for NB-IoT and LTE-MTC enhancements [1].

For Rel-17 NB-IoT UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH, the need of FDD/TDD differentiation is [Yes] in the preliminary version. Regarding support of 16-QAM, we failed to see the difference between TDD and FDD. Furthermore, for the legacy NB-IoT, the MCS table for unicast NPDSCH and unicast NPUSCH is the same for TDD and FDD. Therefore, the need of FDD/TDD differentiation should be no for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH.

Proposal 1: The need of FDD/TDD differentiation should be ‘no’ for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH.

For Rel-17 LTE-MTC UE feature A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A only, we do not see any difference between unicast PDSCH and PUR PDSCH, regarding the use of maximum DL TBS of 1736 bits. Similarly, the UE capabilities for combining max DL TBS and other features are not needed either. Therefore, we have the following proposal.

Proposal 2: UE capability for combination of max DL TBS and other features is not needed.

- Conclusion

In this contribution, our views on Rel-17 UE features for NB-IoT and LTE-MTC enhancements are provided. The following proposals are made.

Proposal 1: The need of FDD/TDD differentiation should be ‘no’ for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH.

Proposal 2: UE capability for combination of max DL TBS and other features is not needed.

- References

- R1-2108678, Preliminary RAN1 UE features list for Rel-17 LTE, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106-e, Aug., 2021.