3GPP TSG RAN WG1 #106bis-e		                                                                 R1-2109183

e-Meeting, October 11th – 19th, 2021

Title:	[Draft] Reply LS on synchronous operation between Uu and SL in TDD band n79

Response to:	R1-2106430 /R4-2108114

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	CATT, GOHIGH

To:	RAN4

Cc:

Attachment:	-

Contact Person:

Name:	Rui Zhao

E-mail Address:	zhaorui@gohigh.com.cn

Overall description

RAN1 thanks RAN4 for the LS on synchronous operation between Uu and SL in TDD band n79, and would provide response to the following question from RAN4.

There are two options under discussion in RAN4 as below. Option 1: To follow the Rel-16 agreement to align SL transmission timing with DL timing. Option 2: To reconsider SL transmission timing to align with UL timing to mitigate the interference between Uu and SL, i.e. - For sidelink transmissions, - SL transmission timing is aligned with Uplink timing when Uu and sidelink is TDMed/FDMed coexistence in the same band, including TDM coexistence within the same carrier or different carriers. - Otherwise, SL transmission timing is aligned with Downlink timing. RAN4 respectfully ask RAN1 to clarify that is it feasible that RAN4 consider option 2 from RAN1 perspective to define SL transmission timing to align with UL timing when SL is synchronized to a network?

Response to RAN4:

From RAN1 perspective, in Rel-17, it is not feasible to consider option 2 to define SL transmission timing to align with UL timing when SL is synchronized to a network. RAN1 would also like to note that Rel-16 SL provides DL-pathloss based power control and configurable number and location of sidelink symbols in a slot to mitigate SL interference at the gNB.

Actions:

To RAN4:

RAN1 respectfully requests RAN4 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#107-e			11-19 November 2021			E-Meeting

RAN1#108-e			17-25 January 2022   			E-Meeting