3GPP TSG RAN WG1 #106bis-e		    	R1-2109562

e-Meeting, October 11th – 19th, 2021

Agenda item:	8.2.5

Title:	Multi-PDSCH scheduling design for 52.6-71 GHz NR operation

Source:	MediaTek Inc.

Document for:	Discussion

- Introduction

Up to RAN1 #105-e meeting, several aspects of multi-PDSCH have been agreed, e.g., the maximum number of PDSCHs can be scheduled by single DCI is 8, the definition of basic bit-fields to realize the functionality (K1, MCS, RV, NDI,HARQ ID). The following agreements regarding this agenda are reached up to RAN1 #105-e meeting.

Agreement: - For a UE and for a serving cell, scheduling multiple PDSCHs by single DL DCI and scheduling multiple PUSCHs by single UL DCI are supported. - Each PDSCH or PUSCH has individual/separate TB(s) and each PDSCH/PUSCH is confined within a slot. - FFS: The maximum number of PDSCHs or PUSCHs that can be scheduled with a single DCI - FFS: Whether multiple PDSCH scheduling applies to 120 kHz in addition to 480 and 960 kHz - At least for 120 kHz SCS, single-slot scheduling with slot-based monitoring will still be supported as specified in Rel-15/Rel-16 - The followings will not be considered in this WI. - Single DCI to schedule both PDSCH(s) and PUSCH(s) - Single DCI to schedule one or multiple TBs where any single TB can be mapped over multiple slots, where mapping is not by repetition - Single DCI to schedule N TBs (N>1) where a TB can be repeated over multiple slots (or mini-slots) - Note: This does not imply that existing slot aggregation and/or repetition for PDSCH and PUSCH by single DCI is precluded for the serving cell. Agreement: - For a DCI scheduling multiple PDSCHs, HARQ-ACK information corresponding to PDSCHs scheduled by the DCI is multiplexed with a single PUCCH in a slot that is determined based on K1, - where K1 (indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI) indicates the slot offset between the slot of the last PDSCH scheduled by the DCI and the slot carrying the HARQ-ACK information corresponding to the scheduled PDSCHs. - It is noted that granularity of K1 can be separately discussed. - FFS: If needed, further discuss whether or not HARQ-ACK information corresponding to different PDSCHs scheduled by the DCI can be carried by different PUCCH(s) Agreement: For generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following alternatives can be considered to DAI counting and will be down-selected in RAN1#104bis-e. - Alt 1: C-DAI/T-DAI is counted per DCI. - Alt 2: C-DAI/T-DAI is counted per PDSCH. - Alt 3: C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable (e.g., 1, 2, 4, …). - FFS: Codebook generation details - FFS: How to signal DAI values (e.g., increase of DAI bits for Alt 2 and Alt 3) - FFS: Whether to apply time domain bundling of HARQ-ACK feedback Agreement: - The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz. - FFS: Further restrictions for 480 kHz to 4 - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided. - The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8. - FFS: Further restrictions for 120 kHz and 480 kHz SCS - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS Agreement: For a DCI that can schedule multiple PDSCHs, - MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH - HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed) - FFS: - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements Agreement: - For a DCI that can schedule multiple PUSCHs, - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e - FFS: signaling details - Note: Alt 2 does not preclude continuous resource allocation in time-domain. - For a DCI that can schedule multiple PDSCHs, - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI. - FFS: signaling details - Note: This does not preclude continuous resource allocation in time-domain. - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided. Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered, - Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set - Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table - Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table - FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table Agreement: - RAN1 use the absolute time duration for 120 kHz SCS as the upper bound for the discussion of UE processing timelines (not related to PDCCH monitoring) for 480 kHz and 960 kHz SCS for NR operation in 52.6 to 71 GHz - RAN1 strives to reduce the absolute time durations from the upper bound if feasible - FFS: How to derive timeline values - Case by case study - FFS: model-based approach for selected timelines, e.g. exponential models, projection based on log-linear regression, etc. Agreement: - Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling. - Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI. - Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI. Conclusion: For a DCI that can schedule multiple PUSCHs, - CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2. Agreement: - If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH. - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. - If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH. - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. Agreement: For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs), - A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots. - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH - FFS: Details to introduce the gap between PDSCHs or between PUSCHs Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - FFS: details of further pruning of the set of SLIVs - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot - FFS impact of time domain bundling, if supported Agreement: - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. - FFS: - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields Agreement: If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - At least two sub-codebooks are generated for a PUCCH cell group where - The first sub-codebook is for the following cases: - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH - The second sub-codebook is for the following case: - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH - FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group - FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI Agreement: If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly. - The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR. - Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR - This shall not impose additional gNB’s scheduling restriction. - In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on - The number of SLIVs associated with the row indexes in TDRA table - FFS: details - FFS: the case with configuration of CBG retransmission - FFS: the number of sub-codebooks - FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

Our views on Type-1 and Type-2 codebook designs for multi-PDSCH and remaining discussion of multi-PDSCH scheduling features and timeline discussion are as follows.

- Type-2 codebook design

Rel-16 Type-2 codebook construction can be briefly summarized into the following steps assuming same CC scheduling:

Step1: Determine the received PDSCHs which have HARQ bits to report on a PUCCH

Step2: Order the received PDSCH with the principle of frequency domain (CC ID for the received PDSCH) first and time domain (monitoring occasion starting time of the associated DCI) later.

Step3: Find the missing PDSCHs based on C-DAI/T-DAI in the received DCIs and generate the HARQ-ACK bits based on the ordering of PDSCHs.

An important assumption for Rel-16 Type-2 codebook construction is that one DCI can only schedule one PDSCH such that C-DAI/T-DAI information in DCIs can be used to detect the missing PDSCHs. However, this assumption is not valid when multi-PDSCH scheduling is considered and UE can only detect the missing DCIs without knowing the number of PDSCHs scheduled by the missing DCIs if we directly apply Rel-16 C-DAI/T-DAI design for Type-2 codebook  construction, which can cause codebook size misalignment between gNB and UE.

To address the issue of detection of missing DCIs and codebook size misalignment, DAI per DCI is adopted in RAN1 #106e meeting. For the design principle of DAI per DCI, the assumption is to reuse the current C-DAI/T-DAI design in Rel-16 where 2 bits are considered and any up to 3 consecutive missing DCIs can be detected. On top of it, to resolve the codebook size misalignment issue, UE will report a fixed number of HARQ-ACK bits for each missing/detected DCI.  The drawback of this design is the redundancy in the codebook when the fixed number of HARQ-ACK bits for each DCI is larger than the scheduled PDSCHs. However, the codebook size can be managed by introducing CBG-like grouping among the scheduled PDSCH. In fact, this design principle is aligned with the CBG codebook design in Rel-16, where one DCI schedules a TB which consists of several CBs.

Proposal 1: For Type-2 codebook construction based on the principle of DAI per DCI, support the following PDSCH grouping and HARQ-ACK bit reporting to manage the codebook size.

- When a UE is configured with multi-PDSCH scheduling in a cell c, the scheduled PDSCHs from one DCI are grouped into M PDSCH groups based on Rel-15/16 CBG grouping principle

  - M=min⁡(N,C), where N is the maximum number of PDSCH groups per DCI configured by network and C is the number of scheduled PDSCHs in the DCI.

  - Let M1=modC,M, K1=C/M, K2=C/M

  - Each PDSCH group in the first M1 PDSCH groups contains K1 scheduled PDSCHs and each PDSCH group in the remaining PDSCH groups contains K2 scheduled PDSCHs.

  - UE reports one HARQ-ACK bit for each PDSCH group

    - If all PDSCHs within a PDSCH group are decoded correctly, UE reports “ACK”

    - Else, UE reports “NACK”

  - If M<N, UE will append N-M “NACK” bits after the M HARQ-ACK bits from the M TB groups to construct the codebook

- Multi-PDSCH scheduling design related discussion

In RAN1 #104bis-e meeting, there was one discussion point on whether the HARQ-ACK information corresponding to different PDSCHs scheduled by a DCI can be carried by different PUCCHs. In our view, when more than one PUCCH resources are considered, the codebook design should change accordingly. It is not clear to us how to design C-DAI/T-DAI for this feature and we prefer to see a complete solution on this aspect before discussing the multi-PUCCH enhancement.

Proposal 4: The HARQ-ACK information corresponding to the PDSCHs scheduled by a DCI should only be carried by single PUCCH to simplify Type-2 codebook design.

In RAN1 #105e meeting, it has been agreed that CBG configuration is allowed in 120kHz multi-PUSCH scheduling when only one PUSCH is scheduled. In RAN1 #106e meeting, whether the same behavior can be extended to other SCSs and DL was discussed with no conclusion. In our view, the benefit of CBG functionality in a short slot duration of 480kHz and 960kHz is not clear. On the other hand, this feature can increase HARQ codebook size and degrade the feedback transmission reliability. Therefore, we suggest to exclude CBG functionality in 480kHz and 960kHz for both PDSCH and PUSCH scheduling.

We also propose to reuse the same existing PUCCH payload size limit 1706 to transmit UCI information including HARQ-ACK information bits.

Proposal 5: CBG (re)transmission feature for 480kHz and 960kHz is not supported  in FR2-2.

Proposal 6: The UCI information bits including HARQ-ACK information bits should reuse the existing PUCCH payload size limit 1706.

There are FFS points on the multi-PDSCH feature regarding the time gap between scheduled PDSCHs. Particularly, the maximum gap between two consecutively scheduled PDSCHs and the gap between the first scheduled PDSCH and the last scheduled PDSCH are FFS. Based on our understanding, the motivation to introduce such gap is to allow potential DL/UL switching among the scheduled PDSCHs. Therefore, we don’t see the need to optimize the gap between the first scheduled PDSCH and the last scheduled PDSCH. Furthermore, when the gap between the first scheduled PDSCH and the last scheduled PDSCH is large, HARQ ID starvation issue can arise due to the fact that the HARQ ID of the scheduled PDSCHs can’t be released at least till the end of the last PDSCH transmission. In our view, the scheduled M PDSCHs should be contained within at most M consecutive slots to ensure the single shared MCS reflects the channels across the slots where the multi-PDSCH are scheduled and mitigate the HARQ ID starvation issue.

Proposal 7: For multi-PDSCH scheduling, if M PDSCHs are scheduled by a DCI, the M PDSCHs should be contained within at most M consecutive slots

Regarding the number of scheduled PDSCHs within a slot, the benefit to allow multiple PDSCHs within a slot of 480kHz and 960kHz is not clear and we prefer to have at most one scheduled PDSCH within a slot.

Proposal 8: For 480kHz and 960kHz, support at most one scheduled PDSCH within a slot

In Rel-15/16, there are fundamental scheduling restriction to prevent out-of-order scheduling among the PDCCHs and the scheduled PDSCHs and among PDSCHs and PUCCHs, which are described in TS 38.214 as follows:

TS 38.214 Clause 5.1 In a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions … In a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i.

However, the same rules might not be directly applied to multi-PDSCH scheduling. For example, consider the scheduling illustrated in Figure 1 where DCI1 schedules PDSCH 1-1 and PDSCH 1-2 and DCI2 schedules PDSCH 2-1. It can be observed that (DCI1, PDSCH1-1) and (DCI2, PDSCH 2-1) comply with the valid in-order scheduling. On the other hand, the timing relationship between (DCI1, PDSCH1-2) and (DCI2, PDSCH 2-1) is out-of-order, which is not allowed in Rel-15/16. To resolve the issue, one approach is to define out-of-order rules for multi-PDSCH scheduling by listing possible scheduling scenarios and discussing which ones should not be allowed.  However, such discussion might be time consuming and many corner cases might be involved, which is not desirable at this late stage of WI. The other approach can be reusing Rel-15/16 in-order scheduling restriction to any pairs of (scheduling DCI, scheduled PDSCH/PUSCH) and any pair of (scheduled PDSCH, resource for the HARQ-ACK transmission) for multi-PDSCH/PUSCHs scheduling to simplify the design. In that case, the scheduling scenario in Figure 1 is not allowed.

Proposal 9: For multi-PDSCH/PUSCH scheduling, UE doesn’t expect any of the scheduled PDSCHs and the scheduling DCI lead to out-of-order scheduling.

Proposal 10: For multi-PDSCH scheduling, UE doesn’t expect any of the scheduled PDSCHs and the resource for the HARQ-ACK transmission lead to out-of-order scheduling.

Figure 1 Example of multi-PDSCH scheduling for out-of-order discussion

To introduce additional gap, it has been agreed that the scheduled PDSCHs will be dropped when the scheduling is not aligned with TDD configuration. In addition, if the CBG transmission is not supported for multi-PDSCH scheduling for 480kHz and 960kHz when more than one PDSCH are scheduled, the CBGTI field can be used to indicate which scheduled PDSCHs corresponding to a DCI are not transmitted, which can improve gNB scheduling flexibility.

Proposal 11: To improve gNB scheduling flexibility, reinterpret CGBTI field to indicate which scheduled PDSCHs corresponding to a DCI are transmitted/retransmitted.

- Timeline related discussion

The timeline design is highly related to UE implementation and network scheduling configuration. Take N1 as an example. We identify at least the following factors should be considered when N1 is evaluated.

- PDCCH

  - Per-slot monitoring and multi-slot monitoring

  - BD/CCE limit for per slot and multi-slot

  - Same numerology scheduling and mix numerology scheduling (e.g. cross-BWP scheduling, cross-carrier scheduling)

  - Same carrier and Cross-carrier scheduling

  - CA number

- PDSCH

  - PDSCH mapping type (Type A vs. Type B)

  - PDSCH time length

  - Single PDSCH and multi-PDSCH scheduling design

  - associated HARQ procedure and the supported HARQ ID number

- PUCCH

  - PUCCH formats

  - Same or different numerology between PDCCH/PDSCH and PUCCH

- DMRS

  - DMRS configurations

In RAN1 #106e meeting, the basic absolute processing timelines for 120kHz corresponding to N1/N2/N3 was adopted for the processing timelines for 480kHz and 960kHz, respectively. However, there are other terms in the formula of processing timeline which need to be defined to complete the essential processing time requirements. Take PDSCH processing time specified in TS 38.214 as example, where the processing time is specified as  Tproc,1=N1+d1,1+d22048+144×κ2-μ×Tc+Text. d1,1 is the extra PDSCH processing time to accommodate the channel estimation based on DMRS in short PDSCH. Since the unit of d1,1 is defined as symbols, it should follow the same scaling principle as N1 when determining the values of d1,1 for 480kHz and 960kHz.

Proposal 12: For determining the processing timelines for 480kHz and 960kHz, the following parameters are scaled by 4 and 8 for 480kHz and 960kHz, respectively.

- d1,1 , d2 (in PDSCH processing time)

- d2,1 , d2 (in PUSCH preparation time)

Although the absolute time duration is the maintained as 120kHz, it can already achieve lower latency for higher SCSs compared to the latency for 120kHz, which is illustrated by an example of N1 shown in Figure 2, where 1 symbol PDCCH schedules 7 symbols PDSCH and 1 symbol for the HARQ-ACK information are assumed. In the example, it can be observed that the time gap between the end of PDCCH and the beginning of PUCCH in 960kHz is 50us less than the one in 120kHz even the same N1 (20 symbols of 120kHz) is applied, which indicates a reduced latency for HARQ-ACK transmission. Meanwhile, the reduced latency implies a shorter processing time for the preparation of PUCCH. In other words, the absolute processing time duration for 120kHz is quite challenging to meet in 480kHz and 960kHz without improving the time for the whole processing path, e.g., PDCCH detection, DCI parsing, channel estimation, PDSCH decoding, TPC and DFE for PUCCH, etc.. Therefore, further reduction of N1 needs to be justified.

Figure 2 Example of latency comparison between 120kHz and 960 kHz under the same absolute N1 time duration (20 symbols of 120kHz)

Proposal 13: The necessity of advanced processing timelines for 480kHz and 960kHz should be justified and the methodology for evaluation should be discussed together if the advanced processing timelines are needed.

- Conclusion

In summary, we have the following proposals:

Proposal 1: For Type-2 codebook construction based on the principle of DAI per DCI, support the following PDSCH grouping and HARQ-ACK bit reporting to manage the codebook size.

- When a UE is configured for multi-PDSCH scheduling in a cell c, the scheduled PDSCHs from one DCI are grouped into M PDSCH groups

  - M=min⁡(N,C), where N is the maximum number of PDSCH groups per DCI configured by network and C is the number of scheduled PDSCHs in the DCI.

  - Let M1=modC,M, K1=C/M, K2=C/M

  - Each PDSCH group in the first M1 PDSCH groups contains K1 scheduled PDSCHs and each PDSCH group in the remaining PDSCH groups contains K2 scheduled PDSCHs.

  - UE reports one HARQ-ACK bit for each PDSCH group

    - If all PDSCHs within a PDSCH group are decoded correctly, UE reports “ACK”

    - Else, UE reports “NACK”

  - If M<N, UE will append N-M “NACK” bits after the M HARQ-ACK bits from the M TB groups to construct the codebook

Proposal 4: The HARQ-ACK information corresponding to the PDSCHs scheduled by a DCI should only be carried by single PUCCH to simplify Type-2 codebook design.

Proposal 5: CBG (re)transmission feature for 480kHz and 960kHz is not supported

Proposal 6: The UCI information bits including HARQ-ACK information bits should reuse the existing PUCCH payload size limit 1706.

Proposal 7: For multi-PDSCH scheduling, if M PDSCHs are scheduled by a DCI, the M PDSCHs should be contained within at most M consecutive slots

Proposal 8: For 480kHz and 960kHz, support at most one scheduled PDSCH within a slot

Proposal 9: For multi-PDSCH/PUSCH scheduling, UE doesn’t expect any of the scheduled PDSCHs and the scheduling DCI lead to out-of-order scheduling.

Proposal 10: For multi-PDSCH scheduling, UE doesn’t expect any of the scheduled PDSCHs and the resource for the HARQ-ACK transmission lead to out-of-order scheduling.

Proposal 11: To improve gNB scheduling flexibility, reinterpret CGBTI field to indicate which scheduled PDSCHs corresponding to a DCI are transmitted/retransmitted.

Proposal 12: For determining the processing timelines for 480kHz and 960kHz, the following parameters are scaled by 4 and 8 for 480kHz and 960kHz, respectively.

- d1,1 , d2 (in PDSCH processing time)

- d2,1 , d2 (in PUSCH preparation time)

Proposal 13:

- FFS: the necessity of advanced timelines and the methodology for evaluation if advanced timelines are needed

- References