3GPP TSG RAN WG1 #106b-e                                                             R1-2109609

e-Meeting, 11th – 19th October 2021

Source: 	Intel Corporation

Title:	On timing relationship enhancements for NTN

Agenda item:	8.4.1

Document for:	Discussion and Decision

# Introduction

At the 3GPP TSG RAN meeting #86 the new work item (WI) on solutions for NR to support non-terrestrial networks (NTN) was approved in [1]. The NTN WI aims to specify enhancements identified for NR NTN (especially LEO and GEO) with implicit compatibility to support HAPS (high altitude platform station) and ATG (air to ground) scenarios according to the following principles:

- FDD is assumed for core specification work for NR-NTN.

  - NOTE: This does not imply that TDD cannot be used for relevant scenarios e.g. HAPS, ATG

- Earth fixed Tracking area is assumed with Earth fixed and moving cells

- UEs with GNSS capabilities are assumed.

According to 3GPP TR 38.821 [2] changes for NR specification are required to support NTN due to long propagation delays, Doppler effects and moving cells. Multiple features of NR specification should be enhanced to support NTN including timing relationship. In this contribution timing relationships enhancements for NR to support NTN are discussed.

# Discussion

Timing relationships enhancements were discussed in the study item on solutions for NR to support NTN [2]. For NTN it is assumed that large timing advance (TA) is applied by the UE in order to compensate large propagation delay. In order to avoid the case where UE should transmit PUSCH before reception of UL grant for that PUSCH, slot offset from the DCI to the target slot of PUSCH reception should be larger than TA applied by the UE. In order to solve this issue and several similar issues it was agreed to support common slot offset K_offset [3].

Agreement: - Introduce K_offset to enhance the following timing relationships: - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH). - The transmission timing of RAR grant scheduled PUSCH. - The transmission timing of HARQ-ACK on PUCCH. - The CSI reference resource timing. - The transmission timing of aperiodic SRS. - Note: Additional timing relationships that require K_offset of the same or different values can be further identified.

Details of configuration for K_offset value are still under discussion in RAN1. At the RAN1#103-e meeting [4] it was agreed that at least a cell specific K_offset configuration is supported for initial access. Beam specific K_offset configured for initial access is FFS. Based on values specified in table 4.2-2 from [2], the maximum differential delay within a cell is 10.3 ms for GEO and 3.18 ms for LEO at 1200 km. The number of slots within the maximum differential delay is represented in the below table for different subcarrier spacing (SCS).

Table 1. Number of slots to cover the maximum differential delay within a cell

| Orbit\SCS | 15 kHz | 30 kHz | 60 kHz | 120 kHz |
| --- | --- | --- | --- | --- |
| GEO | 11 slots | 21 slots | 42 slots | 83 slots |
| LEO @1200km | 4 slots | 7 slots | 13 slots | 26 slots |

As it can be seen from the above table, maximal number of slots to cover the differential delay within a cell is 83. If difference of beam-specific K_offset relative to cell-specific K_offset is indicated, 7 bits per beam is required. In order to decrease the overhead, reporting of slot offset difference between adjacent beams can be considered. Furthermore, subsampling of value range can be considered in order to further decrease the overhead. Thus, 2 bits per beam can be enough in order to support beam specific K_offset for initial access. In order to maintain the main principles of Rel. 15 system information design, K_offset for all beams should be indicated in the SI transmitted in every beam.

Proposal 1:

- Support beam specific K_offset configured in system information for initial access

  - Support indication of K_offset difference between adjacent beams with up to X bits per beam (e.g. X = 2)

  - K_offset for all beams should be indicated in the SI transmitted in every beam

At the last RAN1 meeting the following agreement was made on K_offset update after initial access.

Agreement: - The UE-specific K_offset can be provided and updated by network with MAC CE. - FFS: UE can be provided and updated by network with a UE-specific K_offset in RRC reconfiguration - FFS: Details on whether and how the two solutions work together

As it can be seen in the agreement above, solution based on MAC CE is adopted for UE-specific K_offset indication. In order to increase efficiency of the MAC CE transmission differential scheme for UE-specific K_offset indication can be considered, where UE is provided with a difference between cell/beam-specific value and UE-specific value. In this case just a small number of bits (< 7 bits) is required to cover the differential delay in the cell in accordance with Table 1. Also, in our view solution with RRC configuration indication is not needed in additiona to MAC CE based solution since it is redundant.

Proposal 2:

- Differential scheme is supported for indication of UE-specific K_offset via MAC CE

  - Difference between cell/beam-specific K_offset and UE-specific K_offset is indicated

  - RRC-based configuration for UE-specific K_offset is not supported

At the RAN1#103-e meeting [4] the following agreement was made in agenda item for enhancements on UL time and frequency synchronization.

Agreement: - In NTN, the network may broadcast - A common timing offset value - FFS details of the common timing offset - FFS: A common timing drift rate - Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: Where: is derived from the User specific TA self-estimation is derived at least from the common timing offset value if broadcasted by the network. The granularity of and whether is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed. - depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2. - is specified in TS 38.211 section 4.1. - Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

In the above agreement it is stated that indication of common timing advance (common TA) is supported. Since the common TA and common slot offset (K_offset) are related to propagation delay, the value of K_offset can be fully or partially derived from common TA value. For example, for deployment with transparent payload common TA can be used in order to compensate feeder link delay, K_offset for this case is used to compensate maximum delay for the deployment which is sum of feeder link delay and maximum service link delay. Thus, K_offset can be represented as sum of common TA (in slots) and common slot offset for service link.

At the RAN1#104b-e meeting the following agreement was made on K_offset configuration in SI.

Agreement: - For determination of cell-specific K_offset in system information, down-select one option from below: - Option 1: Signal one offset value for K_offset - Note: For example, the value is expected to cover the RTT of service link plus the RTT between serving satellite and reference point - Option 2: Signal a first offset value and a second offset value. K_offset is equal to the sum of the two offset values - Note: For example, the first offset value is expected to cover the RTT between serving satellite and reference point or is determined by common TA, and the second offset value is expected to cover RTT of service link

Thus, if option 2 is supported then part of K_offset corresponding to feeder link can be implicitly derived from the indicated Common TA (or vice versa). Such design results in small overhead reduction (up to 1 bit) since feeder link delay is at most half of the full delay. However, there are some other parameters which can be related to the feeder link delay. At the RAN1#105-e meeting the following agreement was made related to the additional slot offset for MAC CE action K_mac.

Agreement: The starts of ra-ResponseWindow and msgB-ResponseWindow are delayed by an estimate of UE-gNB RTT. - The estimate of UE-gNB RTT is equal to the sum of UE’s TA and K_mac. Note 1: The UE’s TA is based on the RAN1#104bis-e agreement on Timing Advance applied by an NR NTN UE given by . The estimate of gNB-satellite RTT is equal to the sum of and K_mac. How to treat and can be further discussed. Note 2: According to the RAN1#104bis-e agreement: When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. Note 3: The accuracy of the estimated UE-gNB RTT with respect to the true UE-gNB RTT can be further discussed. Note 4: Other options of determining the estimate of UE-gNB RTT can be further discussed.

Thus, full delay is equal to the sum of UE’s TA and K_mac. If UE’s TA corresponds to the service link only (i.e. Common TA is equal to 0) then K_mac corresponds to an estimate of feeder link delay. If Common TA corresponds to feeder link delay then K_mac is zero. Thus, only one additional bit a is required to represent the above two scenarios. E.g. if a = 0, K_mac is equal to 0 and Common TA corresponds to feeder link delay; if a = 1, K_mac corresponds to feeder link delay and Common TA is equal to 0. Also, if Common TA is equal to zero, K_offset shall correspond only to service link. Hence, the overhead required to indicate Common TA and K_mac can be reduced as it is represented in the below figure. The Common TA in the analysis represents slot-level part of the common TA; Sub-slot Common TA can be indicated separately.

Figure 1. Explicit (A) and implicit (B) indication of K_offset, Common TA and K_mac

As it can be seen from the above figure, if implicit indication is used around 30% of overhead can be saved for indication of offset, Common TA and K_mac. Thus, we support option 2 in the agreement above, where indication of K_offset can be divided into two parts. Furthermore, we propose to consider determination of K_offset, Common TA and K_mac based on additional bit a as it is described in figure 1.

Proposal 3:

- Indication of K_offset is done using two values for K_offset determination: K_offset_1 and K_offset_2

- K_offset, Common TA and K_mac are determined based on K_offset_1 and K_offset_2 values and value of bit “a” indicated together with K_offset_1 and K_offset_2

  - If a = 0,

    - K_offset = K_offset_1 + K_offset_2,

    - Common TA = K_offset_2, K_mac = 0

  - If a = 1,

    - K_offset = K_offset_1,

    - Common TA = 0, K_mac = K_offset_2

Since update of K_offset is supported, reporting of UE-specific TA can be used in order to determine UE-specific K_offset value. For that use-case the granularity of UE-specific TA reporting can be equal to 1 slot. In order to handle variable UE-specific TA reporting of UE specific TA should be done periodically with certain update rate. For this case other solution can be considered – UE-specific TA can be calculated based on UE location reported to the gNB. In this case changes of UE-specific TA due to satellite movement can be predicted at the gNB without periodical reporting from the UE.

Proposal 4:

- Consider UE-specific TA reporting from the UE to the gNB with slot granularity

  - Reporting of UE location for UE-specific TA calculation can be considered

At the last RAN1 meeting it was agreed to support K_mac indication in system information (i.e. cell-specific or beam-specific value). The optimal configuration of K_mac is common for all UEs in the cell since it is related to feeder link delay, so UE-specific K_mac is not needed. However, due to satellite movement the optimal value for K_mac is changing over time (for non-GEO satellites). Thus, update of K_mac via MAC CE or RRC can be considered. In our view it is not necessary to support update of K_mac value via MAC CE or RRC since additional delays for MAC CE DL action or RA response window due to larger K_mac value are acceptable.

Proposal 5:

- Slot offset for MAC CE DL action (K_mac) indication via RRC or MAC CE is not supported

At the last RAN1 meeting Beam Failure Recovery (BFR) support for NTN was discussed. In our view the benefit of this feature for NTN is not clear since change of satellite beam in NTN can be handled by using other solutions. Beam blockage in NTN is not expected to be so frequent as for terrestrial FR2 deployments.

Proposal 6:

- Support of Beam Failiary Recovery (BFR) is considered as a low priority for NR NTN

# Conclusion

In this contribution aspects related to timing relationships for NTN were discussed. The following proposals were made.

Proposal 1:

- Support beam specific K_offset configured in system information for initial access

  - Support indication of K_offset difference between adjacent beams with up to X bits per beam (e.g. X = 2)

  - K_offset for all beams should be indicated in the SI transmitted in every beam

Proposal 2:

- Differential scheme is supported for indication of UE-specific K_offset via MAC CE

  - Difference between cell/beam-specific K_offset and UE-specific K_offset is indicated

  - RRC-based configuration for UE-specific K_offset is not supported

Proposal 3:

- Indication of K_offset is done using two values for K_offset determination: K_offset_1 and K_offset_2

- K_offset, Common TA and K_mac are determined based on K_offset_1 and K_offset_2 values and value of bit “a” indicated together with K_offset_1 and K_offset_2

  - If a = 0,

    - K_offset = K_offset_1 + K_offset_2,

    - Common TA = K_offset_2, K_mac = 0

  - If a = 1,

    - K_offset = K_offset_1,

    - Common TA = 0, K_mac = K_offset_2

Proposal 4:

- Consider UE-specific TA reporting from the UE to the gNB with slot granularity

  - Reporting of UE location for UE-specific TA calculation can be considered

Proposal 5:

- Slot offset for MAC CE DL action (K_mac) indication via RRC or MAC CE is not supported

Proposal 6:

- Support of Beam Failiary Recovery (BFR) is considered as a low priority for NR NTN

# References

- RP-193234 Solutions for NR to support non-terrestrial networks (NTN)

- 3GPP TR 38.821 V16.0.0 (2019-12); 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #102-e, e-Meeting

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #103-e, e-Meeting