3GPP TSG RAN WG1 Meeting #106bis-e	R1-2110372

e-Meeting, October 11th – October 19th, 2021

Agenda Item:	8.9.3

Source:	Huawei, HiSilicon

Title:	Discussion on RRC parameters for max DL TBS of 1736bits

Document for:	Discussion and Decision

- Introduction

This contribution discusses the RRC parameters for max DL TBS of 1736bits.

- Rel-17 RRC parameters for max DL TBS of 1736bits

In email discussion post RAN1#106-e, there were initial draft of RRC parameters for the WI [1]. The RRC parameter for max DL TBS of 1736 bits is as below:

| NB_IOTenh4_LTE_eMTC6 | max DL TBS for eMTC | 36.213 |  |  |  | ce-enable-dl-max-TBS | New | enable max DL TBS of 1736 bits | Enable/disable max DL TBS of 1736 bits for HD-FDD Cat. M1 UEs in CE mode A | Enable | disable | Per UE | UE specific | 36.331 | Agreement The 1736 bits DL TBS feature is enabled by unicast RRC configuration. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

There were some discussions about whether to introduce parameters on configuration of the maximum DL TBS for multicast and PUR in eMTC. The use of max DL TBS for multicast and PUR has no impact on the interpretation of the DCI, therefore, the eNB can just schedule a TBS larger than 1000 bits and no larger than 1736 bits, as long as the UE supports it (for PUR) or with the UE discarding the DCI if the UE does not support it (for multicast). Therefore, explicit configuration of max DL TBS for multicast and PUR may not be needed.

Proposal 1: Parameters on configuration of the maximum DL TBS for multicast and PUR in eMTC are not needed.

- Conclusion

In this contribution, our views on Rel-17 RRC parameters for max DL TBS of 1736bits are provided. The following proposals are made.

Proposal 1: Parameters on configuration of the maximum DL TBS for multicast and PUR in eMTC are not needed.

- Reference

- R1-2108684, [Post-106-e-Rel17-RRC-09] Summary of email discussion on RRC parameters for Enhancements for NB-IoT and LTE-MTC, Moderator (Huawei), RAN1#106-e, Aug., 16th-27th, 2021.