3GPP TSG RAN WG1 #106bis-e	                                         	   R1-2110510

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.3.3

Source:	Apple Inc.

Title:	Further discussion on UCI multiplexing and power control in Rel-17 URLLC

Document for:	Discussion/Decision

# Introduction

At RAN1 #106bis-e, under UCI multiplexing (c.f. R1-2110547), power control for PUCCH with inter-L1 priority UCI multiplexing was discussed. In Section 3.2.3 of R1-2110547, several options are provided:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, down-select from the options:

Option 1: Use the HP UCI bit number and HP RE number for  ∆_(TF,b,f,c) (i)   calculation

Option 2: Use the LP UCI bit number and LP RE number for  ∆_(TF,b,f,c) (i)   calculation

Option 3: No enhancement.

# Discussion on power control

A key discussion on Rel-17 URLLC power control is about how the spectral efficiency is calculated.

Let the payload including potential CRC bits for HP be NH, let the REs taken by HP UCI be RH, let Qm be the modulation order for PUCCH, then roughly NH/RH⋅Qm≈r1, where r1 is the coding rate for HP UCI(s), and Let the payload including potential CRC bits for LP be NL, let the REs taken by LP UCI be RL, then roughly NL/RL⋅Qm≈r2, where r2 is the coding rate for LP UCI(s).

We can consider several alternatives:

- Option 1: NH/RH≈r1/Qm

- Option 2: NL/RL≈r2/Qm

- Option 3: NH+NL/RH+RL≈1+βr1/1+βr1/r2/Qm, where β=NL/NH.

When LP UCIs are multiplexed over a HP PUCCH, the reliability of HP UCIs should not be compromised. Hence it is very reasonable to take Alt. 1. Now we can check whether Option 3 can approximately achieve Option 1 under most conditions. From our examination as in the attachment, the answer is no.

Note for PUCCH to work properly, inter-cell interference cannot be ignored. For example, if intercell interference fluctuates from multiplexing with HP UCIs only to HP+LP UCIs,  it can be challenging from cell planning/configuration point of view.

Hence we have

Proposal:

The spectral efficiency calculation is based on the payload size of HP UCI(s) and REs for HP UCI(s)

# 3. Conclusions

In this contribution we share our views power control aspect. We have

Proposal:

The spectral efficiency calculation is based on the payload size of HP UCI(s) and REs for HP UCI(s).