## NR Positioning Enhancements

Please refer to RP-210903 for detailed scope of the WI.

Incoming LS(s) related to this work/study item under agenda item 5: R1-2108696, R1-2108697, R1-2108706, R1-2108707, R1-2108711, R1-2108718, R1-2108698.

[106bis-e-R17-RRC-NR-ePos] Email discussion on Rel-17 RRC parameters for positioning enhancement – Ren Da (CATT)

- 1st check point: October 14

- Final check point: October 19

### Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Including DL, UL and DL+UL positioning methods, and UE-based and UE-assisted positioning solutions.

[106bis-e-NR-ePos-01] Email discussion/approval on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays with checkpoints for agreements on October 14 and 19 – Ren Da (CATT)

R1-2108730	Remaining issues of mitigating Rx/Tx timing error	Huawei, HiSilicon

R1-2108878	Positioning accuracy improvement by mitigating timing delay	ZTE

R1-2108975	Discussion on potential enhancements for RX/TX timing delay mitigating	vivo

R1-2109051	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	OPPO

R1-2109224	Further discussion on mitigating UE and gNB Rx/Tx timing errors	CATT

R1-2109283	Discussion on mitigation of gNB/UE Rx/Tx timing errors	CMCC

R1-2109363	Views on mitigating UE and gNB Rx/Tx timing errors	Nokia, Nokia Shanghai Bell

R1-2109490	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	Samsung

R1-2109611	Mitigation of UE and gNB RX/TX Timing Errors	Intel Corporation

R1-2109679	Discussion on mitigating UE and gNB Rx/Tx timing delays	NTT DOCOMO, INC.

R1-2109790	Discussion on mitigating Rx/Tx timing delays	Sony

R1-2110035	Positioning accuracy enhancements under timing errors	Apple

R1-2110088	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays	LG Electronics

R1-2110133	Discussion on accuracy improvements by mitigating timing delays	InterDigital, Inc.

R1-2110187	Remaining Issues for Timing Error Mitigation for improved Accuracy	Qualcomm Incorporated

R1-2110254	Mitigation of RX/TX timing delays for higher accuracy	MediaTek Inc.

R1-2110298	Considerations for mitigation of Tx/Rx Delays	Lenovo, Motorola Mobility

R1-2110349	Techniques mitigating Rx/Tx timing delays	Ericsson

R1-2110391

R1-2110392

R1-2110393

Working assumption:

    - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA.

      - The serving gNB should forward the association information provided by the UE to the LMF.

        - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs

      - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB.

    - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT.

      - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs

      - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF.

    - FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used.

Agreement:

### Make the following modification on the previous agreement made in RAN#106e:

- Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements.

    - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability

  - The TRP can be either a “RSTD” reference TRP or a neighbour TRP

  - FFS: details of the signalling, procedures, and UE capability

  - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different.

  - Note: All RSTD measurements are relative to a single reference timing

- Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements.

  - M = [2, 3, 4, 6, 8] (FFS: other values)

  - FFS: details of the signalling, procedures

  - The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different.

Agreement:

- For mitigating TRP Tx/Rx timing errors for DL+UL positioning, when a gNB reports a gNB Rx-Tx time difference measurement, the gNB can support either or both of the following options:

- Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID

- Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID

- Note: The TRP Rx TEG ID is associated with one UL positioning SRS resource (or more UL positioning SRS resources) corresponding to the Rx time of the gNB Rx-Tx time difference measurement.

- If a TRP Tx TEG ID is reported with a gNB Rx-Tx time difference measurement, the gNB also reports the association of the TRP Tx TEG ID to the DL PRS resource(s) to the LMF under the condition that the TRP has more than one DL PRS resource.

- FFS: how the association of the Tx TEG ID to the DL PRS resource(s) is determined by the TRP and how the association is reported to the LMF.

- FFS: details of the signalling

Proposal:

- For the UE Rx-Tx time difference:

  - Option 1:

    - Subject to UE capability, the UE may report an additional UL Timestamp associated to a UE Rx-Tx measurement, corresponding to the timing of the uplink subframe of a positioning SRS.

    - Add the following to the UE Rx-Tx time difference definition (similar to the definition for HD-FDD UE in TS 36.214):

      - If the UE does not transmit SRS in subframe #j, and if the UE reports an additional timestamp for the positioning SRS associated to the measurement, it shall compensate for the difference in the transmit timing of uplink subframe #j and the transmission timing of the subframe containing positioning SRS.

  - Option 2:

    - Subject to a UE capability, a UE may optionally report Timing Adjustment (TA) change information

      - Option 2A: The TA change information is included in the UE Tx TEG report

      - Option 2B: The TA change information is included in the Rx-Tx measurement report

      - Note: TA change information corresponds to: Tx Timing change with a timestamp that this change occurred.

- Once RAN1 makes the decision to adopt any of the above options, send an LS to RAN4 to check if RAN4 has issues to support RAN1’s decision.

### Accuracy improvements for UL-AoA positioning solutions

[106bis-e-NR-ePos-02] Email discussion/approval on accuracy improvements for UL-AoA positioning solutions, with checkpoints for agreements on October 14 and 19 – Alexey (Intel)

R1-2108731	Remaining issues of UL AoA enhancements	Huawei, HiSilicon

R1-2108879	Accuracy improvement for UL-AoA positioning solutions	ZTE

R1-2108976	Discussion on potential enhancements for UL-AoA method	vivo

R1-2109052	Enhancements for UL AoA Positioning	OPPO

R1-2109225	Further discussion on enhancements for UL-AoA positioning method	CATT

R1-2109364	Views on enhancing UL AoA	Nokia, Nokia Shanghai Bell

R1-2109491	Discussion on accuracy improvements for UL-AoA positioning solutions	Samsung

R1-2109612	Remaining Aspects of NR Positioning UL-AoA Enhancements	Intel Corporation

R1-2109680	Discussion on UL-AoA positioning enhancements	NTT DOCOMO, INC.

R1-2109791	Considerations on enhancements for UL-AoA	Sony

R1-2109863	TRP Rx-ARP information reporting	Fraunhofer IIS, Fraunhofer HHI

R1-2110036	Positioning Accuracy enhancements for UL-AoA	Apple

R1-2110089	Discussion on accuracy improvement for UL-AoA positioning	LG Electronics

R1-2110134	Enhancements for UL-AoA positioning solutions	InterDigital, Inc.

R1-2110188	Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

R1-2110345	Discussion on enhancements for UL AoA positioning	CEWiT

R1-2110350	Enhancements of UL-AoA positioning solutions	Ericsson

R1-2110453

R1-2110454

Agreement:

Association of UL-AOA positioning measurements with gNB ARP is supported in Rel.17.

Agreement:

- For the first arrival path measurements on SRS for positioning resource,

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one-gNB Rx-Tx time difference}

  - FFS additional option: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference}

  - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF

- For the first arrival path measurements on SRS for MIMO resource,

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - FFS: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF

  - Note: The operation of SRS for MIMO is transparent to the UE

### Accuracy improvements for DL-AoD positioning solutions

Including UE-based and network-based (including UE-assisted) positioning solutions.

[106bis-e-NR-ePos-03] Email discussion/approval on accuracy improvements for DL-AoD positioning solutions with checkpoints for agreements on October 14 and 19 – Florent (Ericsson)

R1-2108732	Remaining issues of DL AoD enhancements	Huawei, HiSilicon

R1-2108880	Accuracy improvement for DL-AoD positioning solutions	ZTE

R1-2108977	Discussion on potential enhancements for DL-AoD method	vivo

R1-2109053	Enhancements for DL-AoD positioning	OPPO

R1-2109226	Further discussion on enhancements for DL-AoD positioning method	CATT

R1-2109284	Discussion on DL-AoD enhancements	CMCC

R1-2109346	Discussion on enhancements for DL-AoD positioning	CAICT

R1-2109365	Views on enhancing DL AoD	Nokia, Nokia Shanghai Bell

R1-2109413	Accuracy improvements for DL-AoD positioning solutions	Xiaomi

R1-2109492	Discussion on accuracy improvements for DL-AoD positioning solutions	Samsung

R1-2109613	Solutions for NR Positioning DL-AoD Enhancements	Intel Corporation

R1-2109681	Discussion on DL-AoD positioning enhancements	NTT DOCOMO, INC.

R1-2109792	Considerations on enhancements for DL-AoD	Sony

R1-2109864	DL-AoD positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2110037	Positioning Accuracy enhancements for DL-AoD	Apple

R1-2110090	Discussion on accuracy improvement for DL-AoD positioning	LG Electronics

R1-2110148	Enhancements for DL-AoD positioning solutions	InterDigital, Inc.

R1-2110189	Remaining Issues on Potential Enhancements for DL-AoD positioning	Qualcomm Incorporated

R1-2110256	Accuracy enhancement for DL-AOD technique	MediaTek Inc.

R1-2110299	Discussion on DL-AoD Positioning Enhancements	Lenovo, Motorola Mobility

R1-2110343	Discussion on enhancements for DL-AoD positioning	CEWiT

R1-2110351	Enhancements of DL-AoD positioning solutions	Ericsson

R1-2110460

R1-2110500

R1-2110578

Agreement:

The measured path DL PRS RSRP for ith path delay is defined as the power of the received DL PRS signal configured for the measurement at the ith path delay of the channel response, and

- path DL PRS RSRP for 1st path delay is the power corresponding  to the first detected path

- FFS: Whether the path RSRP measurement is normalized with PRS RSRP.

- FFS: Whether the definition of the ith path delay (other than i=1) is required.

- Note: UE may choose to use a time window to compute path DL PRS RSRP by UE implementation (there is no impact to specifications managed by RAN1 for this)

- Note: This does not imply that the path delay has to be reported in DL-AoD positioning

- Send LS to RAN4 to check the details of the definition and feedback if they identify any update is necessary

Agreement:

The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows:

- For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}.

- For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability

  - FFS: Values of M. Candidate values include {2,4,8,16,24}.

  - FFS: Whether M is always equal to N

- Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps.

- Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation

### Latency improvements for both DL and DL+UL positioning methods

[106bis-e-NR-ePos-04] Email discussion/approval on latency improvements for both DL and DL+UL positioning methods with checkpoints for agreements on October 14 and 19 – Su (Huawei)

R1-2108733	Enhancements to positioning latency improvements	Huawei, HiSilicon

R1-2108881	Discussion on latency reduction for NR positioning	ZTE

R1-2108978	Discussion on latency enhancement for NR positioning	vivo

R1-2109054	Enhancements on Latency Reduction in NR Positioning	OPPO

R1-2109227	Further discussion on latency reduction for NR positioning	CATT

R1-2109255	Discussion on latency improvement for positioning methods	China Telecom

R1-2109285	Discussion on latency improvement for positioning	CMCC

R1-2109366	Views on PHY Latency Reductions	Nokia, Nokia Shanghai Bell

R1-2109414	Latency improvements for both DL and DL+UL positioning method	Xiaomi

R1-2109493	Discussion on latency improvements for both DL and DL+UL positioning methods	Samsung

R1-2109614	Solutions for NR Positioning Latency Reduction	Intel Corporation

R1-2109682	Discussion on latency improvements for both DL and DL+UL positioning methods	NTT DOCOMO, INC.

R1-2109793	Considerations on latency improvements for NR positioning	Sony

R1-2110038	Views on Rel-17 positioning latency reduction	Apple

R1-2110091	Discussion on latency improvements for NR positioning	LG Electronics

R1-2110149	Latency improvements for both DL and DL+UL positioning methods	InterDigital, Inc.

R1-2110190	Remaining issues on Latency Improvements for Positioning	Qualcomm Incorporated

R1-2110257	Physical latency improvement aspects	MediaTek Inc.

R1-2110300	Enhancements for Positioning Latency Reduction	Lenovo, Motorola Mobility

R1-2110352	Latency improvements for both DL and DL+UL positioning methods	Ericsson

R1-2110445

R1-2110446

R1-2110447

Agreement:

Support the following options (in the agreement made in RAN1#106-e) for a new mechanism of MG activation request for the purpose of positioning.

- Option 2: by UE (via UCI or UL MAC CE)

  - Select only one of UCI and UL MAC CE in RAN1#106bis-e

- Option 1: by LMF (via an NRPPa message)

  - Note: This is transparent to the UE

Proposal:

Support the following option (from the agreement made in RAN1#106-e) for a new MG activation procedure to be performed by the gNB.

- Option 2: DL MAC CE

Conclusion:

Potential enhancements to latency reduction with respect to MG sharing with other RRM procedures is up to RAN4 to decide.

Agreement:

For PRS measurement outside MG, support the following Alt. 2 in the working assumption made in RAN1#106-e with the following update of the PRS cell condition.

- Alt. 2: Applicable to all PRS (serving and/or non-serving cell) under conditions to PRS of non-serving cell.

  - The conditions at least include that the Rx timing difference between PRS from the non-serving cell and that from the serving cell is within a threshold

    - The UE is not expected to determine whether the above condition is satisfied by performing measurements and instead can be determined using assistance data

      - FFS: Rx timing difference between PRS from the non-serving cell and that from the serving cell is determined by the expected RSTD and expected RSTD uncertainty.

  - Further discuss the necessity on the following additional conditions

    - When the PRS is higher priority than other channels/signals, for capability 1A and 1B, the PRS from the non-serving cell have to be inside the PRS prioritization window.

    - When the PRS is higher priority than other channels/signals, for capability 2, the PRS from the non-serving cell have to be in the same symbols as the PRS of the serving cell since the serving cell does not know the symbol position of neighbour cell PRS.

Agreement:

- With regards to UE determining the PRS priority with other DL signal/channels within the PRS processing window for PRS measurement outside MG, support the priority indicated by gNB.

  - FFS: What are the other DL signals/channels

- With regards to the PRS processing window for PRS measurement outside MG, at least support the window indicated by gNB.

### Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

[106bis-e-NR-ePos-05] Email discussion/approval on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation with checkpoints for agreements on October 14 and 19 – Ryan (Nokia)

R1-2108734	Remaining issues of NLOS mitigation and multi-path reporting	Huawei, HiSilicon

R1-2108807	NLOS Mitigation Enhancements	FUTUREWEI

R1-2108882	Enhancements on NLOS and Multi-path mitigation for NR positioning	ZTE

R1-2108979	Discussion on potential enhancements for multipath/NLOS mitigation	vivo

R1-2109055	Discussion on multipath/NLOS mitigation for NR positioning	OPPO

R1-2109228	Further discussion on information reporting from UE and gNB for multipath/NLOS mitigation	CATT

R1-2109367	Views on LoS/NLoS Identification and Mitigation	Nokia, Nokia Shanghai Bell

R1-2109415	Potential enhancements for multipath/NLOS mitigation	Xiaomi

R1-2109494	Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Samsung

R1-2109615	Solutions for Mitigation of NLOS Problem for NR Positioning	Intel Corporation

R1-2109683	Discussion on multipath/NLOS mitigation for NR positioning	NTT DOCOMO, INC.

R1-2109794	Considerations on multipath/NLOS mitigation for NR positioning	Sony

R1-2109866	Potential positioning enhancements for multipath/NLOS mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2110039	Views on potential enhancements for NLOS mitigation in Rel-17 positioning	Apple

R1-2110092	Discussion on multipath/NLOS mitigation for positioning	LG Electronics

R1-2110150	Discussion on multipath/NLOS mitigation for positioning	InterDigital, Inc.

R1-2110191	Remaining Issues on Multipath Reporting in NR Positioning	Qualcomm Incorporated

R1-2110301	Remaining issues for NLOS/Multipath Information Reporting	Lenovo, Motorola Mobility

R1-2110344	Discussion on enhancements of multipath/NLOS reporting from UE and gNB 	CEWiT

R1-2110353	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Ericsson

R1-2110435

R1-2110436

Agreement:

For hybrid positioning methods where UL TDOA and multi-RTT are used in addition to UL AoA, support reporting of up to M=8 UL-AoA values per additional path

Proposal:

Support reporting the path RSRP for additional paths as part of the DL, DL+UL, and UL multipath reporting enhancements.

- FFS: Path PRS-RSRP definition

- FFS: Relative or absolute power

Working assumption:

Supported LoS/NLoS indicator values are [0, 0.1, …, 0.9, 1] (in steps of 0.1) with the values corresponding to the likelihood of LoS

- A value of 1 corresponds to LoS and a value of 0 corresponds to NLoS

- Note: A UE does not have to support the 0.1 step size or all the values in the set and can choose to only report from a subset including the subset [0,1]. Whether a UE capability is defined to differentiate between reporting of hard and soft values can be further discussed.

- FFS: Define a capability allowing the UE to indicate whether the UE can support the step size of 0.1 (soft values) or it can only support values in the subset [0, 1] (hard values)

### Others

Including aspects for RAN2-led on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, and RAN2-led methods/measurements/signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions.

[106bis-e-NR-ePos-06] Email discussion/approval on RAN2-led aspects in the Others section with checkpoints for agreements on October 14 and 19 – Alexey (Intel)

R1-2108883	Discussion on items led by RAN2 for NR positioning	ZTE

R1-2108980	Discussion on inactive state positioning and on-demand PRS	vivo

R1-2109056	Discussion on positioning for UE in RRC_INACTIVE and on-demand PRS	OPPO

R1-2109229	Further discussion on on-demand DL PRS and positioning for UEs in RRC_ INACTIVE state	CATT

R1-2109286	Discussion on RAN2-led items for positioning	CMCC

R1-2109347	Discussion on enhancements of  INACTIVE mode positioning and on-demand PRS	CAICT

R1-2109368	Additional views on Inactive Mode Positioning and on-demand PRS	Nokia, Nokia Shanghai Bell

R1-2109416	On-demand PRS and positioning for UE in RRC_INACTIVE state	Xiaomi

R1-2109495	Discussion on on demand positioning and positioning in inactive state	Samsung

R1-2109616	Support of On-demand DL PRS and NR Positioning in RRC_INACTIVE State	Intel Corporation

R1-2109684	Discussion on positioning for UEs in RRC_INACTIVE state	NTT DOCOMO, INC.

R1-2109744	Discussion on INACTIVE state positioning and on-demand PRS	Huawei, HiSilicon

R1-2109795	Considerations on on-demand PRS and positioning in RRC Inactive Mode	Sony

R1-2109867	Considerations for on-Demand PRS and positioning in RRC_INACTIVE state	Fraunhofer IIS, Fraunhofer HHI

R1-2110093	Discussion on other enhancements for positioning	LG Electronics

R1-2110151	On-demand PRS and positioning during INACTIVE mode	InterDigital, Inc.

R1-2110192	Remaining issues on enhancements Related to On Demand PRS And Positioning in RRC Inactive State	Qualcomm Incorporated

R1-2110261	Potential physical layer impact to the RAN2-led topics	MediaTek Inc.

R1-2110302	Discussion on On-Demand PRS and RRC_INACTIVE Positioning	Lenovo, Motorola Mobility

R1-2110354	Further details  for on-demand PRS reception and SRS in RRC_INACTIVE	Ericsson

R1-2110457

Agreement:

- Send LS to RAN2 with the outcome of RAN1 discussion on types of SRS for positioning to be supported by UEs in RRC_INACTIVE state

- From RAN1 perspective, support of semi-persistent SRS for positioning by RRC_INACTIVE UEs is feasible

- It is up to RAN2 to confirm support of semi-persistent SRS for positioning by RRC_INACTIVE UEs and determine necessary signalling details

Agreement:

- For RRC_INACTIVE UEs, SRS for positioning bandwidth, SCS and CP type are configured by RRC and can be different from that of initial UL BWP configured by the system information

Agreement:

For OLPC of SRS for positioning transmission by RRC_INACTIVE UEs,

- Reuse validity criteria for pathloss measurement defined for RRC_CONNECTED UEs in Rel.16

  - FFS on UE fallback behavior (i.e. whether to reuse fallback to pathloss measurement by RRC_INACTIVE UE for the cell, from which the SS/PBCH is received to obtain MIB, is not accurate)

- For spatial relation of SRS for positioning transmission by RRC_INACTIVE UEs,

  - FFS whether to define validity criteria or reuse validity criteria for OLPC pathloss measurement to determine whether spatial relation with configured RS is valid

Agreement:

- The following list of parameters is supported for UE-initiated and LMF initiated on-demand DL PRS request

  - Start/end time of DL PRS transmission

  - DL PRS resource repetition factor

  - Number of DL PRS resource symbols per DL PRS resource

  - DL-PRS CombSizeN

  - Number of DL PRS frequency layers

  - ON/OFF indicator (for LMF initiated request only)

- FFS values for requested on-demand DL PRS parameters and whether parameters are resource-specific, TRP-specific, or PFL-specific