3GPP TSG RAN WG1 #106bis-e		                                   					       												                 R1-2109918

e-Meeting, October 11th – 19th, 2021

Agenda Item:	8.17.14

Source:	Moderator (AT&T)

Title:	Summary of UE features for IoT over NTN

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [106bis-e-R17-UE-features-IoT-NTN-01] during RAN1 #106bis-e. According to the Chairman’s Notes:

[106bis-e-R17-UE-features-IoT-NTN-01] Email discussion UE features for IoT over NTN – Ralf (AT&T) - 1st check point: October 14 - Final check point: October 19

The following was discussed and/or agreed during RAN1 #106bis-e within the scope of [106bis-e-R17-UE-features-IoT-NTN-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 LTE in [1].

# Summary of Contributions Submitted to RAN1 #106bis-e

The following is the moderator’s summary of contributions submitted to RAN1 #106bis-e in this agenda item.

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Support of GNSS-assisted UE for uplink time and frequency synchronization | Components 1. UE derives its position based on its GNSS implementation |  | No | No | Release 17 UE cannot access NTN |  | No | No | The main assumption is that Release 17 NTN UE is equipped with a GNSS | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | The components of “support of GNSS-assisted UE for uplink time and frequency synchronization” and “uplink time pre-compensation” are all necessary for UE to acquire its timing advance for UL synchronization, they can be put under the same feature and with the component as follows: 1. Enhancement for uplink time synchronization a) UE derives its position based on its GNSS implementation. b) UE specific TA calculation in RRC_IDLE/RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. c) UE applies common TA according to the parameters provided by the network (if any) d) For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops. Regarding whether a FG is mandatory or optional, all LTE NB-IoT eMTC over NTN features should be defined as optional with capability signaling. However, for FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b), they can be viewed as the basic features for LTE NB-IoT(eMTC) over NTN since the UEs supporting LTE NB-IoT(eMTC) over NTN would need these features for basic operation including getting access to the network. For UE supporting LTE NB-IoT(eMTC) over NTN, it must indicate these FGs are supported.  For UEs supporting LTE NB-IoT(eMTC) over NTN, they must indicate FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b) are supported. 2. LTE_NBIOT_eMTC_NTN; 2-1; Support of GNSS-assisted UE for uplink time and frequency synchronization Uplink time pre-compensation; Components - UE derives its position based on its GNSS implementation - UE specific TA calculation in RRC_IDLE/RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops.; 2-2 or 2-3; No; No; Release 17 UE cannot access NTN; No; No; The main assumption is that Release 17 NTN UE is equipped with a GNSS. In Rel-17 IoT-NTN, at least support UE which can compute timing advance and frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] | A UE that supports GNSS-assistance for uplink time and frequency synchronization (FG 2-1) would also report/read parameters to/from a serving satellite (FG 2-2 and 2-3). FG 2-6 is about a validity timer for uplink synchronization configured by the network for satellite ephemeris data / common TA parameters that indicates the maximum time the UE can apply the acquired data/parameters without re-acquiring new data/parameters. FG 2-6 can be a component of a merged FG that would include FGs 2-1/2/3/6. |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] | W.r.t FG 2-1, UE may report the validity duration of GNSS, including the GNSS-fix time instant and validity time length, to achieve consensus with eNB. So one more component may be added if this feature is agreed. 2. LTE_NBIOT_eMTC_NTN; 2-1; Support of GNSS-assisted UE for uplink time and frequency synchronization; - Indicate serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) - Serving satellite ephemeris Epoch time is implicitly known; Release 17 UE cannot access NTN if both ephemeris formats are not supported; The main assumption is that Release 17 NTN UE is equipped with a GNSS. |
| Ericsson [6] | We propose to align with the UE features of NR NTN, for which these components are not listed separately |
| Apple [7] | The enhancement on uplink time and frequency synchronization is an important objective for IoT over NTN. It was agreed that UE needs to pre-compensate the time and frequency in its uplink transmissions in either RRC_IDLE state or RRC_CONNECTED state. The calculation of the amount of timing advance and frequency offset for the pre-compensation is based on UE’s GNSS position and serving satellite ephemeris. The serving satellite ephemeris parameters are either position state vector or orbital parameters. The reception of UE’s GNSS position and serving satellite ephemeris serves for the uplink time and frequency synchronization purpose. In our view, these functionalities (i.e., features 2-1, 2-2, 2-3) could be merged as components to the UE feature 2-4 of uplink time pre-compensation or the UE feature 2-5 of uplink frequency/Doppler pre-compensation. Hence, there is no need to define dedicated features 2-1, 2-2 and 2-3.  Remove feature |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] |  |
| SoftBank Corp. [10] | None of GNSS, ephemeris position information and orbital parameters is essential for HAPS operations, even though they are mandatorily required for satellite operations. Thus, the use of terminology “NTN” is not appropriate for this case, and we propose the following:  Add modifications for the “Consequence if ~” column: “Release 17 UE cannot access satellite” |

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Serving satellite ephemeris position state vector | - Indicate serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) |  |  |  | Release 17 UE cannot access NTN |  | No | No | Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED. | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Regarding whether a FG is mandatory or optional, all LTE NB-IoT eMTC over NTN features should be defined as optional with capability signaling. However, for FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b), they can be viewed as the basic features for LTE NB-IoT(eMTC) over NTN since the UEs supporting LTE NB-IoT(eMTC) over NTN would need these features for basic operation including getting access to the network. For UE supporting LTE NB-IoT(eMTC) over NTN, it must indicate these FGs are supported.  For UEs supporting LTE NB-IoT(eMTC) over NTN, they must indicate FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b) are supported. 2. LTE_NBIOT_eMTC_NTN; 2-2; Serving satellite ephemeris position state vector; - Indicate serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s); Release 17 UE cannot access NTN; No; No; Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] | A UE that supports GNSS-assistance for uplink time and frequency synchronization (FG 2-1) would also report/read parameters to/from a serving satellite (FG 2-2 and 2-3). FG 2-6 is about a validity timer for uplink synchronization configured by the network for satellite ephemeris data / common TA parameters that indicates the maximum time the UE can apply the acquired data/parameters without re-acquiring new data/parameters. FG 2-6 can be a component of a merged FG that would include FGs 2-1/2/3/6. There are descriptions in the component column that have not been agreed, and should be at least in square brackets. For example, the quantities indicated in FG 2-2. |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] | W.r.t FG 2-2 and FG 2-3, Release 17 UE can access NTN if at least one of these features is supported. And for FG 2-2, UE should know the serving satellite ephemeris Epoch time implicitly. 2. LTE_NBIOT_eMTC_NTN; 2-2; Serving satellite ephemeris position state vector; - Indicate serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) - Serving satellite ephemeris Epoch time is implicitly known; Release 17 UE cannot access NTN if both ephemeris formats are not supported; Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.. |
| Ericsson [6] | We propose to align with the UE features of NR NTN, for which these components are not listed separately |
| Apple [7] | The enhancement on uplink time and frequency synchronization is an important objective for IoT over NTN. It was agreed that UE needs to pre-compensate the time and frequency in its uplink transmissions in either RRC_IDLE state or RRC_CONNECTED state. The calculation of the amount of timing advance and frequency offset for the pre-compensation is based on UE’s GNSS position and serving satellite ephemeris. The serving satellite ephemeris parameters are either position state vector or orbital parameters. The reception of UE’s GNSS position and serving satellite ephemeris serves for the uplink time and frequency synchronization purpose. In our view, these functionalities (i.e., features 2-1, 2-2, 2-3) could be merged as components to the UE feature 2-4 of uplink time pre-compensation or the UE feature 2-5 of uplink frequency/Doppler pre-compensation. Hence, there is no need to define dedicated features 2-1, 2-2 and 2-3.  Remove feature |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] | Replace ”indicate” with ”receive”. In any case the FG description does not indicate what the UE actually does with the information it receives, consider refinement. |
| SoftBank Corp. [10] | None of GNSS, ephemeris position information and orbital parameters is essential for HAPS operations, even though they are mandatorily required for satellite operations. Thus, the use of terminology “NTN” is not appropriate for this case, and we propose the following:  Add modifications for the “Consequence if ~” column: “Release 17 UE cannot access satellite” Considering the deployment of HAPS, HAPS aircraft also has its ephemeris information. It may be useful for the HAPS UEs to use this information, even though the capability does not need to be mandated. Therefore, the current sentences, “serving satellite ephemeris” should be modified as follows: Feature Group column for 2-2: - Serving satellite/HAPS ephemeris position state vector Components column for 2-2: - Indicate serving Satellite/HAPS position state vector X,Y,Z in ECEF (m) and serving Satellite/HAPS velocity state vector VX,VY,VZ in ECEF (m/s) Note column for 2-2: - Satellite/HAPS ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED. |

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | Satellite orbital parameters | - Indicate ephemeris orbital parameter for the serving satellite: |  | No | No | Release 17 UE cannot access NTN |  | No | No | Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED. | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Regarding whether a FG is mandatory or optional, all LTE NB-IoT eMTC over NTN features should be defined as optional with capability signaling. However, for FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b), they can be viewed as the basic features for LTE NB-IoT(eMTC) over NTN since the UEs supporting LTE NB-IoT(eMTC) over NTN would need these features for basic operation including getting access to the network. For UE supporting LTE NB-IoT(eMTC) over NTN, it must indicate these FGs are supported.  For UEs supporting LTE NB-IoT(eMTC) over NTN, they must indicate FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b) are supported. 2. LTE_NBIOT_eMTC_NTN; 2-3; Satellite orbital parameters; - Indicate ephemeris orbital parameter for the serving satellite:; No; No; Release 17 UE cannot access NTN; No; No; Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] | A UE that supports GNSS-assistance for uplink time and frequency synchronization (FG 2-1) would also report/read parameters to/from a serving satellite (FG 2-2 and 2-3). FG 2-6 is about a validity timer for uplink synchronization configured by the network for satellite ephemeris data / common TA parameters that indicates the maximum time the UE can apply the acquired data/parameters without re-acquiring new data/parameters. FG 2-6 can be a component of a merged FG that would include FGs 2-1/2/3/6. |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] | W.r.t FG 2-2 and FG 2-3, Release 17 UE can access NTN if at least one of these features is supported. 2. LTE_NBIOT_eMTC_NTN; 2-3; Satellite orbital parameters; Indicate ephemeris orbital parameter for the serving satellite:; No; Release 17 UE cannot access NTN if both ephemeris formats are not supported; Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED.. |
| Ericsson [6] | We propose to align with the UE features of NR NTN, for which these components are not listed separately |
| Apple [7] | The enhancement on uplink time and frequency synchronization is an important objective for IoT over NTN. It was agreed that UE needs to pre-compensate the time and frequency in its uplink transmissions in either RRC_IDLE state or RRC_CONNECTED state. The calculation of the amount of timing advance and frequency offset for the pre-compensation is based on UE’s GNSS position and serving satellite ephemeris. The serving satellite ephemeris parameters are either position state vector or orbital parameters. The reception of UE’s GNSS position and serving satellite ephemeris serves for the uplink time and frequency synchronization purpose. In our view, these functionalities (i.e., features 2-1, 2-2, 2-3) could be merged as components to the UE feature 2-4 of uplink time pre-compensation or the UE feature 2-5 of uplink frequency/Doppler pre-compensation. Hence, there is no need to define dedicated features 2-1, 2-2 and 2-3.  Remove feature |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] | Replace ”indicate” with ”receive”. In any case the FG description does not indicate what the UE actually does with the information it receives, consider refinement. |
| SoftBank Corp. [10] | None of GNSS, ephemeris position information and orbital parameters is essential for HAPS operations, even though they are mandatorily required for satellite operations. Thus, the use of terminology “NTN” is not appropriate for this case, and we propose the following:  Add modifications for the “Consequence if ~” column: “Release 17 UE cannot access satellite” |

| 2. LTE_NBIOT_eMTC_NTN | 2-4 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops | 2-1, 2-2 or 2-3 | No | No | Release 17 UE cannot access NTN |  | No | No | In Rel-17 IoT-NTN, at least support UE which can compute timing advance for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 2. LTE_NBIOT_eMTC_NTN; 2-4; Uplink Time pre-compensation; - UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops; 2-1, 2-2 or 2-3; No; No; Release 17 UE cannot access NTN; No; No; In Rel-17 IoT-NTN, at least support UE which can compute timing advance for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN; Mandatory. |
| Samsung [3] | FG 2-7, which indicates the UL time segment during which the pre-compensation is applied, can be deleted and added as a component in FG 2-4 and FG 2-5. |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] | W.r.t FG 2-4, in fact the mentioned common TA estimation should be based on indication of common TA drift rate. Therefore, that component may be added once this approach is confirmed. 2. LTE_NBIOT_eMTC_NTN; 2-4; Uplink Time pre-compensation; UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. UE applies common TA according to the parameters provided by the network (if any) For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops Common TA estimation is based on indication of common TA drift rate.; 2-1, 2-2 or 2-3; No; Release 17 UE cannot access NTN; In Rel-17 IoT-NTN, at least support UE which can compute timing advance for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN Common TA estimation is based on indication of common TA drift rate.. |
| Ericsson [6] | 2. LTE_NBIOT_eMTC_NTN; 2-4; Uplink Time pre-compensation; - UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops; 2-1, 2-2 or 2-3; No; N/A; Release 17 UE cannot access NTN; No; No; In Rel-17 IoT-NTN, at least support UE which can compute timing advance for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN; Mandatory. |
| Apple [7] | In feature 2-6, it is defined of validity timer of UL synchronization. Like features 2-1, 2-2, 2-3, we think this validity timer is mainly used to determine whether the current satellite ephemeris can be used or not, when determining the time advance and frequency offset for the uplink transmissions. The feature 2-6 could serve as a component of features 2-4 and 2-5, without a need to define a dedicated feature 2-6.  Merge feature 2-6 as a component in features 2-4 and 2-5. |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] |  |
| SoftBank Corp. [10] | Considering the deployment of HAPS, the distance between eNB and UE is normally designed to satisfy the TN LTE maximum distance, i.e. less than 100km. It is therefore still possible for every Release-17 UEs (and pre-release-17 UEs as well) to access the NTN (i.e. HAPS) network without 2-4 capability. The current sentence “Release 17 UE cannot access NTN” is not correct and the modification should be applied.  Add modifications for the “Consequence if ~” column: “Release 17 UE cannot access satellite” |

| 2. LTE_NBIOT_eMTC_NTN | 2-5 | Uplink frequency/Doppler pre-compensation | - In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | 2-1, 2-3 or 2-3 | No | No | Release 17 UE cannot access NTN |  | No | No | In Rel-17 IoT-NTN, at least support UE which can compute frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Regarding whether a FG is mandatory or optional, all LTE NB-IoT eMTC over NTN features should be defined as optional with capability signaling. However, for FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b), they can be viewed as the basic features for LTE NB-IoT(eMTC) over NTN since the UEs supporting LTE NB-IoT(eMTC) over NTN would need these features for basic operation including getting access to the network. For UE supporting LTE NB-IoT(eMTC) over NTN, it must indicate these FGs are supported.  For UEs supporting LTE NB-IoT(eMTC) over NTN, they must indicate FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b) are supported. 2. LTE_NBIOT_eMTC_NTN; 2-4; Uplink frequency/Doppler pre-compensation; - In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.; 2-1, 2-2 or 2-3; No; No; Release 17 UE cannot access NTN; No; No; In Rel-17 IoT-NTN, at least support UE which can compute frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] | FG 2-7, which indicates the UL time segment during which the pre-compensation is applied, can be deleted and added as a component in FG 2-4 and FG 2-5. |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] |  |
| Ericsson [6] | 2. LTE_NBIOT_eMTC_NTN; 2-5; Uplink frequency/Doppler pre-compensation; - In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.; 2-1, 2-2 or 2-3; No; N/A; Release 17 UE cannot access NTN; No; No; In Rel-17 IoT-NTN, at least support UE which can compute frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes; Mandatory. |
| Apple [7] | In feature 2-6, it is defined of validity timer of UL synchronization. Like features 2-1, 2-2, 2-3, we think this validity timer is mainly used to determine whether the current satellite ephemeris can be used or not, when determining the time advance and frequency offset for the uplink transmissions. The feature 2-6 could serve as a component of features 2-4 and 2-5, without a need to define a dedicated feature 2-6.  Merge feature 2-6 as a component in features 2-4 and 2-5. |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] |  |
| SoftBank Corp. [10] | Furthermore, the moving speed of HAPS aircraft is rather slow compared with satellites, thus the level of suffered doppler shift is almost same as TN. Therefore, it is not always required for Release 17 UEs (and even pre-release-17 UEs) to equip 2-5 for HAPS deployment scenarios. The current sentence “Release 17 UE cannot access NTN” is not correct as well. Given the arguments above, we propose the following modifications:  Add modifications for the “Consequence if ~” column: “Release 17 UE cannot access satellite” |

| 2. LTE_NBIOT_eMTC_NTN | 2-6 | Validity timer of UL synchronization | - Validity timer of UL synchronization is configured by the network | 2-2 or 2-3 | No | No | UE does not know the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast |  |  |  | A validity duration configured by the network for satellite ephemeris data / Common TA parameters if broadcast which indicates the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast without having acquired new satellite ephemeris / common TA parameters if broadcast. | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | Regarding whether a FG is mandatory or optional, all LTE NB-IoT eMTC over NTN features should be defined as optional with capability signaling. However, for FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b), they can be viewed as the basic features for LTE NB-IoT(eMTC) over NTN since the UEs supporting LTE NB-IoT(eMTC) over NTN would need these features for basic operation including getting access to the network. For UE supporting LTE NB-IoT(eMTC) over NTN, it must indicate these FGs are supported.  For UEs supporting LTE NB-IoT(eMTC) over NTN, they must indicate FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b) are supported. 2. LTE_NBIOT_eMTC_NTN; 2-6 2-5; Validity timer of UL synchronization; - Validity timer of UL synchronization is configured by the network; 2-2 or 2-3; No; No; UE does not know the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast; A validity duration configured by the network for satellite ephemeris data / Common TA parameters if broadcast which indicates the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast without having acquired new satellite ephemeris / common TA parameters if broadcast.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] | A UE that supports GNSS-assistance for uplink time and frequency synchronization (FG 2-1) would also report/read parameters to/from a serving satellite (FG 2-2 and 2-3). FG 2-6 is about a validity timer for uplink synchronization configured by the network for satellite ephemeris data / common TA parameters that indicates the maximum time the UE can apply the acquired data/parameters without re-acquiring new data/parameters. FG 2-6 can be a component of a merged FG that would include FGs 2-1/2/3/6. |
| Intel Corporation [4] | In our view FG 2-6 can be removed until details on UE behavior w.r.t. validity timer are clear. Depending on the RAN1 agreements, validity time can be considered as assistance information for the UE in order to refine system information update periodicity. In this case separate UE feature for validity time may not be needed. |
| ZTE [5] |  |
| Ericsson [6] | 2. LTE_NBIOT_eMTC_NTN; 2-6; Validity timer of UL synchronization; - Validity timer of UL synchronization is configured by the network; 2-2 or 2-3; No; N/A; UE does not know the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast; A validity duration configured by the network for satellite ephemeris data / Common TA parameters if broadcast which indicates the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast without having acquired new satellite ephemeris / common TA parameters if broadcast.; Mandatory. |
| Apple [7] | In feature 2-6, it is defined of validity timer of UL synchronization. Like features 2-1, 2-2, 2-3, we think this validity timer is mainly used to determine whether the current satellite ephemeris can be used or not, when determining the time advance and frequency offset for the uplink transmissions. The feature 2-6 could serve as a component of features 2-4 and 2-5, without a need to define a dedicated feature 2-6.  Merge feature 2-6 as a component in features 2-4 and 2-5. |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] |  |
| SoftBank Corp. [10] |  |

| 2. LTE_NBIOT_eMTC_NTN | 2-7 | UL transmission segment | - UL transmission segment duration is configured by the network | 2-4 and 2-5 | No | No | UE does not know the maximum time during which the applied pre-compensation for (N)RACH or (N)PUSCH for NB-IoT / eMTC shall not be changed by the UE. |  | No | No | The UL transmission segment length for PUSCH / PRACH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE. | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | The feature of “UL transmission segment” contains both “UL transmission segment for NPUSCH” and “UL transmission segment for NPRACH”. And the two feature groups can be expressed with respective compenents as follows, according to the agreement of RAN1#106-e. - UL transmission segment for NB-IoT - Network configures a number of PUSCH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For NB-IoT, repetition unit is . - Network configures a number of RACH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For NB-IoT, repetition unit is P symbol groups. - UL transmission segment for eMTC - Network configures a number of PUSCH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For eMTC, repetition unit is N_slot^UL×T_slot for sub-PRB allocation, where Tslot = 0.5 ms. For full-PRB allocation, repetition unit is one subframe. - Network configures a number of RACH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For eMTC, repetition unit is one preamble including guard period.  Divide “UL transmission segment” into two feature groups, i.e. “UL transmission segment for NB-IoT” and “UL transmission segment for eMTC”. Regarding whether a FG is mandatory or optional, all LTE NB-IoT eMTC over NTN features should be defined as optional with capability signaling. However, for FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b), they can be viewed as the basic features for LTE NB-IoT(eMTC) over NTN since the UEs supporting LTE NB-IoT(eMTC) over NTN would need these features for basic operation including getting access to the network. For UE supporting LTE NB-IoT(eMTC) over NTN, it must indicate these FGs are supported.  For UEs supporting LTE NB-IoT(eMTC) over NTN, they must indicate FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b) are supported. 2. LTE_NBIOT_eMTC_NTN; 2-6a; UL transmission segment for NB-IoT; - Network configures a number of PUSCH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For NB-IoT, repetition unit is . - Network configures a number of RACH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For NB-IoT, repetition unit is P symbol groups. - For eMTC, repetition unit is for sub-PRB allocation, where Tslot = 0.5 ms. For full-PRB allocation, repetition unit is one subframe.; 2-1; No; No; UE does not know the maximum time during which the applied pre-compensation for (N)PUSCH for NB-IoT / eMTC shall not be changed by the UE.; No; No; The UL transmission segment length for PUSCH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. 2. LTE_NBIOT_eMTC_NTN; 2-6b; UL transmission segment for for eMTC; - Network configures a number of PUSCH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For NB-IoT, repetition unit is . - For eMTC, repetition unit is for sub-PRB allocation, where Tslot = 0.5 ms. For full-PRB allocation, repetition unit is one subframe. - Network configures a number of RACH repetition units as the duration of UL transmission segment for UE pre-compensation for PUSCH transmission. - For NB-IoT, repetition unit is P symbol groups. - For eMTC, repetition unit is one preamble including guard period.; 2-1; No; No; UE does not know the maximum time during which the applied pre-compensation for (N)PUSCH for NB-IoT / eMTC shall not be changed by the UE.; No; No; The UL transmission segment length for PUSCH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] | FG 2-7, which indicates the UL time segment during which the pre-compensation is applied, can be deleted and added as a component in FG 2-4 and FG 2-5. |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] |  |
| Ericsson [6] | 2. LTE_NBIOT_eMTC_NTN; 2-7; UL transmission segment; - UL transmission segment duration is configured by the network; 2-4 and 2-5; No; N/A; UE does not know the maximum time during which the applied pre-compensation for (N)RACH or (N)PUSCH for NB-IoT / eMTC shall not be changed by the UE.; No; No; The UL transmission segment length for PUSCH / PRACH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE.; Mandatory. |
| Apple [7] |  |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] |  |
| SoftBank Corp. [10] |  |

| 2. LTE_NBIOT_eMTC_NTN | 2-8 | K_offset | - Configuration of K_offset | 2-4 | No | No | UE does not know the offset to apply for UL transmission |  | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | The feature of “K_offset” and “start of RAR window” are under the scope of the “enhancement on timing relationship” and all the agreed timing relationship should be contained as component as follows: Enhancement on the timing relationship - configuration of K_offset - timing relationship enhancement based on the K_offset. - delaying the starts of ra-ResponseWindow by the estimated RTT This feature should be known by the eNB to align the timing relationship between eNB and UE. Enhancement on the timing relationship is defined as a UE feature for NB-IoT and eMTC respectively. Regarding whether a FG is mandatory or optional, all LTE NB-IoT eMTC over NTN features should be defined as optional with capability signaling. However, for FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b), they can be viewed as the basic features for LTE NB-IoT(eMTC) over NTN since the UEs supporting LTE NB-IoT(eMTC) over NTN would need these features for basic operation including getting access to the network. For UE supporting LTE NB-IoT(eMTC) over NTN, it must indicate these FGs are supported.  For UEs supporting LTE NB-IoT(eMTC) over NTN, they must indicate FG 2-1, 2-2, 2-3, 2-4, 2-5, 2-6a(b) and 2-7a(b) are supported. 2. LTE_NBIOT_eMTC_NTN; 2-8; K_offset; - Configuration of K_offset; 2-4; No; No; UE does not know the offset to apply for UL transmission; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode.; Mandatory. 2. LTE_NBIOT_eMTC_NTN; 2-7a; Enhancement on the timing relationship for NB-IoT; - configuration of K_offset - timing relationship enhancment based on the K_offset for NB-IoT. - delaying the starts of ra-ResponseWindow by RTT; 2-1; No; No; UE does not know the offset to apply for UL transmission. And, due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble.; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. And, start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. 2. LTE_NBIOT_eMTC_NTN; 2-7b; Enhancement on the timing relationship for eMTC; - configuration of K_offset - timing relationship enhancment based on the K_offset for eMTC. - delaying the starts of ra-ResponseWindow by RTT; 2-1; No; No; UE does not know the offset to apply for UL transmission. And, due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble.; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. And, start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] | A definition of K_offset (used in FG 2-8) needs to be agreed. |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] | W.r.t FG 2-8, since UE-specific Koffset in CONNECTED mode can be used due to the update of K_offset, then one more component is added. 2. LTE_NBIOT_eMTC_NTN; 2-8; K_offset; - Configuration of K_offset - Update of K_offset; 2-4; No; UE does not know the offset to apply for UL transmission; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode.. |
| Ericsson [6] | 2. LTE_NBIOT_eMTC_NTN; 2-8; K_offset; - Configuration of K_offset; 2-4; No; N/A; UE does not know the offset to apply for UL transmission; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode.; Mandatory. |
| Apple [7] | In feature 2-8 (), the only component is configuration of . The feature name seems broad, and we prefer to modify it to reflect the feature, e.g., “enhancing timing relationships using a time offset”. Since both cell specific and UE specific are supported for IoT over NTN, we think it is beneficial to mention them in the component of this feature. From UE point of view, the components could be “UE receives cell specific or UE specific ” and “UE applies in timing relationship enhancements”.  In feature 2-8, modify the feature name to “enhancing timing relationships using a time offset” and the components to - UE receives cell specific or UE specific - UE applies in timing relationship enhancements |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] |  |
| SoftBank Corp. [10] |  |

| 2. LTE_NBIOT_eMTC_NTN | 2-9 | Start of RAR window | - estimate of UE-gNB RTT - delaying the starts of ra-ResponseWindow | 2-4 | No | No | Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. |  | No | No | Start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT. | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | The feature of “K_offset” and “start of RAR window” are under the scope of the “enhancement on timing relationship” and all the agreed timing relationship should be contained as component as follows: Enhancement on the timing relationship - configuration of K_offset - timing relationship enhancement based on the K_offset. - delaying the starts of ra-ResponseWindow by the estimated RTT This feature should be known by the eNB to align the timing relationship between eNB and UE. Enhancement on the timing relationship is defined as a UE feature for NB-IoT and eMTC respectively. 2. LTE_NBIOT_eMTC_NTN; 2-9; Start of RAR window; - estimate of UE-gNB RTT - delaying the starts of ra-ResponseWindow; 2-4; No; No; Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble.; No; No; Start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.; Mandatory. 2. LTE_NBIOT_eMTC_NTN; 2-7a; Enhancement on the timing relationship for NB-IoT; - configuration of K_offset - timing relationship enhancment based on the K_offset for NB-IoT. - delaying the starts of ra-ResponseWindow by RTT; 2-1; No; No; UE does not know the offset to apply for UL transmission. And, due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble.; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. And, start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. 2. LTE_NBIOT_eMTC_NTN; 2-7b; Enhancement on the timing relationship for eMTC; - configuration of K_offset - timing relationship enhancment based on the K_offset for eMTC. - delaying the starts of ra-ResponseWindow by RTT; 2-1; No; No; UE does not know the offset to apply for UL transmission. And, due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble.; No; No; The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. And, start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.; Mandatory Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported. |
| Samsung [3] |  |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] |  |
| Ericsson [6] | 2. LTE_NBIOT_eMTC_NTN; 2-9; Start of RAR window; - estimate of UE-gNB RTT - delaying the start of ra-ResponseWindow; 2-4; No; N/A; Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble.; No; No; Start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT.; Mandatory. |
| Apple [7] | In feature 2-9, it is mentioned that the prerequisite feature group is 2-4. We assume that the start of RAR window is based on the transmission of PRACH, which applies the time pre-compensation. In PRACH transmission, the frequency pre-compensation is also applied, and hence, the feature 2-5 should also be the prerequisite feature group for feature 2-9.  the prerequisite feature groups include both 2-4 and 2-5 |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] |  |
| SoftBank Corp. [10] | The current sentence for 2-9 seems to mean that every single NTN access has a large propagation delay, but obviously this does not apply to HAPS.  add modifications for the “Consequence if ~” column: “Due to large propagation delay in some scenarios of NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble” |

| 2. LTE_NBIOT_eMTC_NTN | 2-10 | UE reporting of information about the UE specific TA pre-compensation | - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content | 2-4 | No | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD |  | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report | Mandatory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | 2. LTE_NBIOT_eMTC_NTN; 2-10 2-8; UE reporting of information about the UE specific TA pre-compensation; - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content; 2-4; No; No; UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD; No; No; UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report; Mandatory Optional with capability signalling.. |
| Samsung [3] |  |
| Intel Corporation [4] | According to the rapporteur, it is considered to make certain UE features for IoT- NTN mandatory for the UE without any capability signalling. This make sense if UE is connecting to a IoT-NTN cell. However, if UE is connecting to terrestrial cell it should be clear that the mandatory UE capabilities are not applicable for this case. In order to clarify that a note can be added with the following text “This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported”. |
| ZTE [5] |  |
| Ericsson [6] | 2. LTE_NBIOT_eMTC_NTN; 2-10; UE reporting of information about the UE specific TA pre-compensation; - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content; 2-4; No; N/A; UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD; No; No; UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report; Mandatory. |
| Apple [7] | In feature 2-10 of UE reporting of information about the UE specific TA pre-compensation, both feature 2-4 and feature 2-5 need to be the prerequisite feature groups.  the prerequisite feature groups include both 2-4 and 2-5 |
| Qualcomm Incorporated [8] |  |
| Nokia/Nokia Shanghai Bell [9] | Mark the FG description as FFS, as the issue is still under discussion in the WI. |
| SoftBank Corp. [10] |  |

General Comments

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| Samsung [3] |  |
| Intel Corporation [4] |  |
| ZTE [5] |  |
| Ericsson [6] |  |
| Apple [7] |  |
| Qualcomm Incorporated [8] | In our view, there should be a single primary feature group (separate for eMTC and NB-IoT) for IoT-NTN, which would encapsulate the “minimum necessary components” that a UE capable of supporting this feature must support. This primary feature group is optional with capability signalling. Further feature group(s) may then depend on this parent, to support anything beyond the minimum necessary components. There may be differences in the minimum set of components that a UE needs to support to facilitate communication with an NTN cell, depending on the orbit of the satellites comprising the NTN cell—specifically, whether the satellites are geostationary (GEO) or non-geostationary (e.g., LEO, MEO). For example, all the time/frequency pre-compensation-related aspects are typically more challenging for LEO satellites than for GEO—these may potentially lead to simplified requirements to support GEO, as compared to supporting non-GEO (NGSO) NTNs. RAN1 should discuss whether and when such differentiation may be applicable, and how to capture the same in the feature list.  RAN1 to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs RAN1 should discuss how to determine the applicability of all terrestrial network (TN) features up to Rel16 for the case of NTN. As such, the legacy capabilities may be interpreted as being exclusively applicable for TNs, while a separate set of capabilities corresponding to those features may be introduced for NTN. This may prevent a “feature by feature” determination of applicability to NTNs from the (legacy) feature list for TNs up to Rel16.  RAN1 to discuss the determination of applicability of Terrestrial Network features up to Release 16 to IoT-NTN in Release 17 |
| Nokia/Nokia Shanghai Bell [9] | - All features are listed as essential for IoT over NTN operation, and hence they should be merged together as a single FG. The new combined FG should be optional. - ”Need for the eNB to know if the feature is supported” should be ”yes” - Notes for all FGs can be removed, they are not essential information for specification drafting |
| SoftBank Corp. [10] | Through the NTN WI in Rel-17, we have pointed out many times that satellite and HAPS have different characteristics and RAN1 should be careful when we use the terminologies “NTN” and/or “satellite” because it may imply the inclusion/exclusion of HAPS. RAN1 should make sure that the terminology is correctly used in the feature list in order to avoid giving a wrong message to outside 3GPP.  RAN1 should make sure that the terminology “NTN” and “satellite” are correctly used in the feature list. - NTN includes HAPS: terminology “NTN” implies that the sentence applies to both satellite and HAPS - Satellite excludes HAPS: terminology “satellite” implies that the sentence does not apply to HAPS It is not appropriate to define any features as Mandatory in Release-14 and later for NB-IoT/eMTC cases because UEs are always allowed not to implement. NTN-IoT is no exception to this rule. Furthermore, these features are not always necessary for HAPS as explained earlier. Thus, just saying “Mandatory” is a bit strong and does not address the concerns very well. We propose to apply conditional mandatory for this case, i.e. these futures can be mandatory for Release-17 UE supporting satellite access such as LEO and GEO. Mandatory/Optional column for 2-1 to 2-10, either Alt 1 or Alt 2 should be applied: Alt 1: Optional without capability signaling. For UE supporting satellite access, UE shall support this FG. Alt 2: Optional without capability signaling. For UE supporting satellite band(s), UE shall support this FG. Note: the choice of Alt may be up to other working groups, e.g. RAN4. If satellite access is completely associated with band definition, Alt 2 is a better choice. |

# Discussion/Approval Items during RAN1 #106bis-e

After review of contributions submitted to RAN1 #106bis-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #106bis-e. Companies submitted the following views on the moderator’s proposals.

# FG 2-1: Support of GNSS-assisted UE for uplink time and frequency synchronization

Two companies proposed to remove this FG to align the IoT NTN UE feature list with that for NR NTN. Others are okay to have the row but suggested edits, highlighted in yellow below.

Proposal 2-1: Agree to keep FG 2-1 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column) and name (third column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | [Support of GNSS-assisted UE for] uplink time [and frequency] [FFS: synchronization or pre-compensation] | Components 1. UE derives its position based on its GNSS implementation [2. UE report the validity duration of GNSS] 3. [Merge FGs 2-1/2/3/6 into FG 2-1] [4. UE specific TA calculation in RRC_IDLE/RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris.] [5. UE applies common TA according to the parameters provided by the network (if any)] [6. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops.] | FFS | [No] | No N/A | Release 17 UE cannot access [NTN/ satellite] |  | No | No | The main assumption is that Release 17 NTN UE is equipped with a GNSS [FFS: In Rel-17 IoT-NTN, at least support UE which can compute timing advance and frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN] | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Overall, we think this is not an independent UE feature. The determination of TA based on GNSS location is a component for uplink TA/frequency pre-compensation. We may want to keep the similar feature grouping as in NR NTN. |
| Nokia, NSB | We tend to agree with Apple that this FG is not useful in isolation, which is supported by the fact that most FGs in this AI are “basic” for IOT NTN. Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | The similar structure as NR can be considered. |
| Huawei, HiSilicon | We also think there is no need to define a separate FG. Overall, one may would like to align with NR NTN on UL timing pre-compensation and UL frequency/Doppler pre-compensation. On the differentiation of eMTC and NB-IoT, our understanding is that this is already the case for TN. One may refer to the existing LTE features (R1-2009350). For the component 2/3/4/5/6, they can be removed since they belong to other FGs. |
| CATT | It should be merged with FG 2-4 and 2-5. |
| MediaTek | It is preferable and simpler to adopt same structure for UE feature in NR NTN for IoT NTN for this UE feature, 2-1 can be merged with 2-4 In the Notes column, the “In Rel-17 IoT-NTN, at least support UE which can compute timing advance and frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes.” is not necessary in FFS based on RAN1 agreements. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - With the highlighted changes, FG 2-1 is now very similar to FG 2-4. We propose to merge them into one (which is then split into one for each of eMTC and NB-IoT). Further, 2-2, 2-3 and 2-6 can also be merged into this FG as they are all essential parts of the UL TA pre-compensation. |

# FG 2-2: Serving satellite ephemeris position state vector

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-2: Agree to keep FG 2-2 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Serving satellite ephemeris position state vector | - Indicate serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) - [Serving satellite ephemeris Epoch time is implicitly known] | [No] | No | No N/A | Release 17 UE cannot access [NTN/ satellite] [if both ephemeris formats are not supported] |  | No | No | Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED. | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Overall, we think this is not an independent UE feature. The reading of serving satellite ephemeris position state vector is a component for uplink TA/frequency pre-compensation. We may want to keep the similar feature grouping as in NR NTN. |
| Nokia, NSB | “Indicate serving…” should be “Receive serving…”, as the UE is not really indicating anything here. Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | It should not be part of UE feature. It is just part of how to achieve the pre-compensation. |
| Huawei, HiSilicon | There is no need to have a separate FG for this. Overall, one may would like to align with NR NTN on UL timing pre-compensation and UL frequency/Doppler pre-compensation. |
| CATT | It should be merged with other FGs. |
| MediaTek | It is preferable and simpler to adopt same structure for UE feature in NR NTN for IoT NTN for this UE feature, 2-2 can be merged with 2-4 Add in the notes column “Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. FFS: Whether this starting time is given by predefined rule or it is indicated by the Network” |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - FG 2-1, FG 2-2, FG 2-3, FG 2-4 and FG 2-6 can be merged into one FG (i.e, one for each of eMTC and NB.IoT) as they are all essential parts of the UL TA pre-compensation. |

# FG 2-3: Satellite orbital parameters

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-3: Agree to keep FG 2-3 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | Satellite orbital parameters | - Indicate Receive ephemeris orbital parameter for the serving satellite: |  | [No] | No N/A | Release 17 UE cannot access [NTN/ satellite] [if both ephemeris formats are not supported] |  | No | No | Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED. | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Overall, we think this is not an independent UE feature. The reading of serving satellite orbital parameter is a component for uplink TA/frequency pre-compensation. We may want to keep the similar feature grouping as in NR NTN. |
| Nokia, NSB | Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | It should not be part of UE feature. It is just part of how to achieve the pre-compensation. |
| Huawei, HiSilicon | There is no need to have a separate FG for this. Overall, one may would like to align with NR NTN on UL timing pre-compensation and UL frequency/Doppler pre-compensation. |
| CATT | It should be merged with other FGs. |
| MediaTek | It is preferable and simpler to adopt same structure for UE feature in NR NTN for IoT NTN for this UE feature, 2-3 can be merged with 2-4 |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - FG 2-1, FG 2-2, FG 2-3, FG 2-4 and FG 2-6 can be merged into one FG (i.e, one for each of eMTC and NB.IoT) as they are all essential parts of the UL TA pre-compensation. |

# FG 2-4: Uplink Time pre-compensation

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-4: Agree to keep FG 2-4 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-4 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - [Common TA estimation is based on indication of common TA drift rate] | 2-1, 2-2 or 2-3 | [No] | No N/A | Release 17 UE cannot access [NTN/ satellite] |  | No | No | In Rel-17 IoT-NTN, at least support UE which can compute timing advance for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN [Common TA estimation is based on indication of common TA drift rate] | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Agree on the feature. We do not think component 5 is needed, as it is already indicated in component 3. |
| Nokia, NSB | Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. Component 5 is still under discussion by RAN1. |
| ZTE | Support and component 5 can be merged into 3 |
| Huawei, HiSilicon | Agree to have this FG which is aligned with NR NTN. Agree with Apple that Component 5 is not needed. |
| CATT | Generally agree. Component 5 is still under discussion, we should delay discuss until the conclusion has made in 8.4.2 and 8.15.1. |
| MediaTek | Component 5 is under discussion in RAN1 and seems premature to be included in column component and column Notes. It is preferable and simpler to adopt same structure for UE feature in NR NTN for IoT NTN for this UE feature. The features 2-1, 2-2, 2-3 can be merged with 2-4 as these are pre-requisites for 2-4 UL frequency/Doppler pre-compensation for similar reasons as 2-4 UL time pre-compensation Since 2-4 and 2-5 are both needed for UE pre-compensation, these could be merged as required sub-components in UE time and frequency pre-compensation component combining 2-1,2-2, 2-3, 2-4, 2-5. This would seem simpler. The overall feature is IoT NTN (referred to as Mother feature by Qualcomm on e-mail reflector) and contain a list of the components / sub-components. The other to look at this is that without UE time and frequency pre-compensation UE cannot support IoT NTN feature. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - FG 2-1, FG 2-2, FG 2-3, FG 2-4 and FG 2-6 can be merged into one FG (i.e, one for each of eMTC and NB.IoT) as they are all essential parts of the UL TA pre-compensation. |

# FG 2-5: Uplink frequency/Doppler pre-compensation

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-5: Agree to keep FG 2-5 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-5 | Uplink frequency/Doppler pre-compensation | - In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | 2-1, 2-32 or 2-3 | [No] | No N/A | Release 17 UE cannot access [NTN/ satellite] |  | No | No | In Rel-17 IoT-NTN, at least support UE which can compute frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We agree with the feature. |
| Nokia, NSB | Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | Support and can be similar as NR-NTN |
| Huawei, HiSilicon | Agree to have this FG which is aligned with NR NTN. |
| CATT | Generally support. However, add the Downlink frequency/Doppler pre-compensation as new FG. If network doesn’t perform common frequency pre-compensation in service downlink, UE need perform Downlink frequency/Doppler pre-compensation. |
| MediaTek | It is preferable and simpler to adopt same structure for UE feature in NR NTN for IoT NTN for this UE feature. The features 2-1, 2-2, 2-3 can be merged with 2-5 as these are pre-requisites for 2-5 UL frequency/Doppler pre-compensation for similar reasons as 2-4 UL time pre-compensation Since 2-4 and 2-5 are both needed for UE pre-compensation, these could be merged as required sub-components in UE time and frequency pre-compensation component combining 2-1,2-2, 2-3, 2-4, 2-5. This would seem simpler. The overall feature is IoT NTN (referred to as Mother feature by Qualcomm on e-mail reflector) and contain a list of the components / sub-components. The other to look at this is that without UE time and frequency pre-compensation UE cannot support IoT NTN feature. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - A more detailed description should be provided in the “components”, e.g., - In RRC_IDLE, UE derives its position based on its GNSS implementation - In RRC_IDLE, UE receives serving satellite ephemeris - In RRC_IDLE and RRC_CONNECTED, UE predicts satellite position and velocity - In RRC_IDLE and RRC_CONNECTED, UE calculates UL frequency compensation based on UE position and predicted satellite position and velocity |

# FG 2-6: Validity timer of UL synchronization

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-6: Agree to keep FG 2-6 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-6 | Validity timer of UL synchronization | - Validity timer of UL synchronization is configured by the network | 2-2 or 2-3 | [No] | No N/A | UE does not know the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast |  |  |  | A validity duration configured by the network for satellite ephemeris data / Common TA parameters if broadcast which indicates the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast without having acquired new satellite ephemeris / common TA parameters if broadcast. | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Overall, we think this is not an independent UE feature. The validity timer of UL synchronization is a component for uplink TA/frequency pre-compensation. We may want to keep the similar feature grouping as in NR NTN. |
| Nokia, NSB | Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | Should not be separate feature and merged for the feature of pre-compensation |
| Huawei, HiSilicon | No need to have a separate FG. It can be merged with UL timing/frequency pre-compensation. |
| CATT | It should be merged with other FGs. |
| MediaTek | It is preferable and simpler to adopt same structure for UE feature in NR NTN for IoT NTN for this UE feature. The feature 2-6 can be merged with 2-4 and 2-5 as these are pre-requisites for 2-5 UL frequency/Doppler pre-compensation for similar reasons as 2-4 UL time pre-compensation Since 2-4 and 2-5 are both needed for UE pre-compensation, these could be merged as required sub-components in UE time and frequency pre-compensation component combining 2-1,2-2, 2-3, 2-4, 2-5, 2-6. This would seem simpler. The overall feature is IoT NTN (referred to as Mother feature by Qualcomm on e-mail reflector) and contain a list of the components / sub-components. The other to look at this is that without UE time and frequency pre-compensation UE cannot support IoT NTN feature. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - FG 2-1, FG 2-2, FG 2-3, FG 2-4 and FG 2-6 can be merged into one FG (i.e, one for each of eMTC and NB.IoT) as they are all essential parts of the UL TA pre-compensation. |

# FG 2-7: UL transmission segment

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-7: Agree to keep FG 2-7 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-7 | UL transmission segment | - UL transmission segment duration is configured by the network | 2-4 and 2-5 | [No] | No N/A | UE does not know the maximum time during which the applied pre-compensation for (N)RACH or (N)PUSCH for NB-IoT / eMTC shall not be changed by the UE. |  | No | No | The UL transmission segment length for PUSCH / PRACH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE. | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Agree with the proposal. |
| Nokia, NSB | Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | Support, this is new need comparing to the legacy behavior. |
| Huawei, HiSilicon | Agree to have this FG. |
| CATT | Support. |
| MediaTek | Support. The overall feature is IoT NTN (referred to as Mother feature by Qualcomm on e-mail reflector) and contain a list of the components / sub-components. The other to look at this is that without iming relationship enhancements, UE cannot support IoT NTN feature. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - A more detailed description should be provided in the “components”, e.g., - UE receives configuration of segment duration from the network - During UL repetition, UE updates time and frequency pre-compensation between segments but not within segments |

# FG 2-8: K_offset

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-8: Agree to keep FG 2-8 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-8 | K_offset Enhancing timing relationships using a time offset | - Configuration of K_offset - UE receives cell specific K_offset or UE specific K_offset - UE applies K_offset in timing relationship enhancements | 2-4 | [No] | No N/A | UE does not know the offset to apply for UL transmission |  | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We agree with the proposal. It is also fine to split cell specific Koffset and UE specific Koffset to two separate features. |
| Nokia, NSB | Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | Support, |
| Huawei, HiSilicon | Agree to have the FG. Prefer to have a separate FG on UE-specific K_offset. |
| CATT | Support. |
| MediaTek | Support. Could be merged as a sub-component with 2-9 in one component timing relationship enhancement The overall feature is IoT NTN (referred to as Mother feature by Qualcomm on e-mail reflector) and contain a list of the components / sub-components. The other to look at this is that without timing relationship enhancements, UE cannot support IoT NTN feature. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. |

# FG 2-9: Start of RAR window

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-9: Agree to keep FG 2-9 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-9 | Start of RAR window | - estimate of UE-gNB RTT - delaying the starts of ra-ResponseWindow | 2-4[, 2-5] | [No] | No N/A | Due to large propagation delay in [some scenarios of ] NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. |  | No | No | Start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT. | Mandatory [Optional with capability signalling. For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Agree with the proposal. |
| Nokia, NSB | Propose to merge all basic FGs into a single one. It is not clear why we would need separate FGs for eMTC and NB-IoT, as the network will already have the information on the type of device. |
| ZTE | Same as NR, should be updated as set of feature for overall time relationship. |
| Huawei, HiSilicon | This FG can be merged with FG2-8 which is also related to timing relationship enhancement. |
| CATT | Support |
| MediaTek | Support. Could be merged as a sub-component with 2-8 in one component timing relationship enhancement The overall feature is IoT NTN (referred to as Mother feature by Qualcomm on e-mail reflector) and contain a list of the components / sub-components. The other to look at this is that without timing relationship enhancements, UE cannot support IoT NTN feature. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. - RAN1 has not yet discussed delaying the start of ra-ResponseWindow. |

# FG 2-10: UE reporting of information about the UE specific TA pre-compensation

Several companies propose to remove the feature group, either entirely or to merge it with other FGs.

Proposal 2-10: Agree to keep FG 2-10 with the changes highlighted in red below and continue discussing the details focusing on the components (fourth column)

- Continue to discuss whether this feature group needs to be separate for eMTC and NB-IoT

- Continue to discuss whether the components required to support IoT-NTN over GEO NTNs are different from the components required to support IoT NTN over non-GEO (NGSO) NTNs

| 2. LTE_NBIOT_eMTC_NTN | 2-10 | UE reporting of information about the UE specific TA pre-compensation | - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content | 2-4[, 2-5] | [No] | No N/A | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD |  | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report | Mandatory Optional with capability signalling. [For UEs supporting NB-IoT eMTC NTN, it must indicate this FG is supported] [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Support the proposal. |
| Nokia, NSB | The issue is still under discussion in the IoT-NTN WID, and hence the FG should not be confirmed yet. |
| ZTE | Support |
| Huawei, HiSilicon | Agree to have the FG. There is no need to mandate the support of this feature. |
| CATT | We think the pre-compensation TA is UE’s TA = UE specific TA + common TA, and “UE reporting of information about the UE specific TA” is not only used in pre-compensation. The description needs to be modified. UE reporting of information about the UE specific TA pre-compensation. |
| MediaTek | We have same view as Nokia and Huawei. The issue is still under discussion in the IoT-NTN WID, and hence the FG should not be confirmed yet. If agreed, 2-10 could be merged as a sub-component with 2-8 and 2-9 in one component timing relationship enhancement The overall feature is IoT NTN (referred to as Mother feature by Qualcomm on e-mail reflector) and contain a list of the components / sub-components. The other to look at this is that without timing relationship enhancements, UE cannot support IoT NTN feature. |
| Ericsson | - Separate feature groups should be defined for eMTC and NB-IoT. - We don’t see a need for separation of support for GEO and non-GEO NTN but it can be further discussed. |

# Proposed Baseline for RAN1 #107-e and outcome of RAN1 #106bis-e

Based on the inputs in Section 3, the following is proposed as baseline for RAN1 #107-e and outcome of RAN1 #106bis-e for this agenda item/work item.

Proposed Agreement: Agree the following table, incl. the changes highlighted in red and the yellow highlighting, as baseline for further discussions during RAN1 #107-e

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS implementation [1-1. UE report the validity duration of GNSS] [1-2. UE specific TA calculation in RRC_IDLE/RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris.] [1-3. UE applies common TA according to the parameters provided by the network (if any)] [1-4. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops.] 2. Indicate serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) [2-1. Serving satellite ephemeris Epoch time is implicitly known] 3. Receive ephemeris orbital parameter for the serving satellite 4. UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. 5. UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. 6. UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (if any) 7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops X. [Common TA estimation is based on indication of common TA drift rate] 8. In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 9. in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 10. Validity timer of UL synchronization is configured by the network 11. UL transmission segment duration is configured by the network 12. UE receives cell specific K_offset 13. UE applies K_offset in timing relationship enhancements 14. estimate of UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] |  | [No] | N/A | Release 17 UE cannot access [NTN/satellite] |  | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. FFS: Whether this starting time is given by predefined rule or it is indicated by the network | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Support of GNSS-assisted UE for uplink time and frequency synchronization | Components 1. UE derives its position based on its GNSS implementation |  | No | No | Release 17 UE cannot access NTN |  | No | No | The main assumption is that Release 17 NTN UE is equipped with a GNSS | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Serving satellite ephemeris position state vector | - Indicate serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) |  |  |  | Release 17 UE cannot access NTN |  | No | No | Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED. | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | 2-3 | Satellite orbital parameters | - Indicate ephemeris orbital parameter for the serving satellite: |  | No | No | Release 17 UE cannot access NTN |  | No | No | Satellite ephemeris read on SIB are valid for the duration of sporadic short transmission in RRC_CONNECTED. | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | 2-4 | Uplink Time pre-compensation | - UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. - UE specific TA calculation in RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies common TA according to the parameters provided by the network (if any) - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops | 2-1, 2-2 or 2-3 | No | No | Release 17 UE cannot access NTN |  | No | No | In Rel-17 IoT-NTN, at least support UE which can compute timing advance for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | 2-5 | Uplink frequency/Doppler pre-compensation | - In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. | 2-1, 2-3 or 2-3 | No | No | Release 17 UE cannot access NTN |  | No | No | In Rel-17 IoT-NTN, at least support UE which can compute frequency adjustment for serving link based on its GNSS position and serving satellite ephemeris signalled by the network and apply corresponding timing advance and frequency adjustment in RRC_IDLE and RRC_CONNECTED modes | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | 2-6 | Validity timer of UL synchronization | - Validity timer of UL synchronization is configured by the network | 2-2 or 2-3 | No | No | UE does not know the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast |  |  |  | A validity duration configured by the network for satellite ephemeris data / Common TA parameters if broadcast which indicates the maximum time during which the UE can apply the satellite ephemeris / common TA parameres if broadcast without having acquired new satellite ephemeris / common TA parameters if broadcast. | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | 2-7 | UL transmission segment | - UL transmission segment duration is configured by the network | 2-4 and 2-5 | No | No | UE does not know the maximum time during which the applied pre-compensation for (N)RACH or (N)PUSCH for NB-IoT / eMTC shall not be changed by the UE. |  | No | No | The UL transmission segment length for PUSCH / PRACH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE. | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | 2-82 | K_offset Enhancing timing relationships using a time offset | - Configuration of K_offset 1. UE receives UE specific K_offset 2. UE applies K_offset in timing relationship enhancements | 2-41 | [No] | No N/A | UE does not know the offset to apply for UL transmission |  | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT | Mandatory Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-9 | Start of RAR window | - estimate of UE-gNB RTT - delaying the starts of ra-ResponseWindow | 2-4 | No | No | Due to large propagation delay in NTN the UE RAR is not expected within a few milliseconds like in TN. The UE does not need to monitor the PDCCH for RAN right after transmitting RA Preamble. |  | No | No | Start of ra-ResponseWindow is delayed by an offset. For IoT NTN, no modifications are needed for the calculation in NR NTN for estimate of UE-eNB RTT. | Mandatory |
| 2. LTE_NBIOT_eMTC_NTN | [2-10] | [UE reporting of information about the UE specific TA pre-compensation] | - The exact content of UE reporting of information about the UE specific TA pre-compensation - frequency of the reports - granularity of the reported content | 2-4 | No | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD |  | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report | Mandatory |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | We are OK with the FL proposal. |
| Huawei, HiSilicon | Some detailed comments below - FG 2-1: - Component 1-2 and component 5 are partially overlapped, Suggest to remove component 1-2 and add “RRC_CONNECTED” in component 5 - Component 1-3 and component 1-4 can be removed since they are covered by component 6 and component 7. - Component 2: Indicate serving Satellite position state vector -> receive serving Satellite position state vector - FG [2-10]: the last row can be change to “optional with capability signaling” |
| Qualcomm | - For 2-1 (and indeed, for others, therefore), there should be “FFS: differentiation based on orbits such as LEO/MEO/GEO”. - Something seems strange about “2-10”. Why is it referencing 2-4, when there doesn’t seem to be a 2-4 in the updated table? Also, why is this mandatory? This has to go into 2-1, since this is a “basic” feature needed to support half-duplex UEs (which include all NB-IoT UEs and most eMTC UEs). - We should also have a general understanding that the wording on each of the components may be further refined/edited in upcoming meetings. |
| Ericsson | We are fine with the general structure except that we think there should be separate FG for eMTC and NB-IoT (the FFS about this is fine for now). Detailed comments: 2-1 (Basic IoT over NTN support) Column 4 (Components): - Items 1-2,1-3,1-4 appear to say the same as 4,5,6,7 and could be removed. - Item 2 could be reformulated as “Receive serving Satellite position state vector…” to be more UE-centric. - Item 2-1 could be reformulated as “Derive epoch time of serving satellite ephemeris from implicit epoch time” to be more UE-centric. - Item 10 could be reformulated as “support a validity timer of UL synchronization configured by the network” to be more UE-centric. - Item 11 could be reformulated as “Based on segment duration configuration from the network, UE updates time and frequency pre-compensation between segments but not within segments during UL repetition” to be more UE-centric. - Item 14 could be reformulated as “UE estimates UE-gNB RTT” to be more UE-centric. 2-2 (Enhancing timing relationships using a time offset) Column 13 (Mandatory/optional): Add the following: “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” 2-10 (UE reporting of information about the UE specific TA pre-compensation) Column 4 (Components): Replace current list with e.g. “UE reports information about UE-specific TA pre-compensation” Column 5 (Prerequisite feature groups): Should be 2-1 Column 13 (Mandatory/Optional): - It should be indicated as “Optional with capability signalling”. - Add the following: “For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported” |
| MediaTek | - FG 2-1: (Basic IoT over NTN support) - Same view as Huawei that Component 1-2 and component 5 are partially overlapped. It should be remove component 1-2 and add “RRC_IDLE” instead of “RRC_CONNECTED” in component 5 - 1-3,1-4 overlap 6,7 and could be removed - Item 2 should be ““Receive ..” not “Indicate ..” - Item 2-1 could be revised as “Derive epoch time of serving satellite ephemeris from implicit epoch time known as a reference time defined by the starting time of a DL slot and/or frame ” to be more UE-centric as suggested by Ericsson, and also be more consistent with RAN1#106-e agreement.. - Same view as Ericsson Item 10 and prefer “support a validity timer of UL synchronization configured by the network” to be more UE-centric. - Same view as Ericsson Item 11 and prefer “Based on segment duration configuration from the network, UE updates time and frequency pre-compensation between segments but not within segments during UL repetition” to be more UE-centric”. - Same view as Ericsson Item 14 and prefer “UE estimates UE-gNB RTT” to be more UE-centric. - 2-2 (Enhancing timing relationships using a time offset) - Same view as Ericsson - 2-10 (UE reporting of information about the UE specific TA pre-compensation) - Same view as Ericsson |
| Samsung | Overall the update list looks fine. 2-1: - Separate parameter for NB-IoT and eMTC - [2-1. Serving satellite ephemeris Epoch time is implicitly known] – describe from UE’s perspective. It could be also moved to comments. - 8, 9, 10, 11, 14 – describe from UE’s perspective (formulation given by other companies above looks fine) 2-2: - Column (Components): use “UE reports…” - Column (Comments): use similar descriptions for - For IoT NTN, support cell-specific Koffset configuration for use during initial access. - For IoT NTN, support the use of UE-specific Koffset configuration in CONNECTED mode. |
| Apple | The overall structure looks good. 2-1: Components 1-2, 1-3, 1-4 should be removed as they are duplicated Component 2 should be “receive serving satellite position state vector” 2-2: the Prerequisite feature groups is 2-1 and 2-10 2-10: the Prerequisite feature groups is 2-1 It is not “mandatory”, it is “Optional with capability signaling” |

# Conclusion

After further discussion on the RAN1 email reflector the following was agreed as part of this email discussion:

# References

- R1-2108678, Preliminary RAN1 UE features list for Rel-17 LTE, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2109156, Rel-17 UE features for IoT over NTN, Huawei/HiSilicon

- R1-2109536, UE features for IoT over NTN, Samsung

- R1-2109654, UE features for IOT over NTN, Intel Corporation

- R1-2109851, Discussion on UE feature for IoT-NTN, ZTE

- R1-2109959, On UE features for IoT over NTN, Ericsson

- R1-2110073, Views on IoT over NTN UE Features, Apple

- R1-2110232, UE features for IoT over NTN, Qualcomm Incorporated

- R1-2110276, On UE features for IoT over NTN, Nokia/Nokia Shanghai Bell

- R1-2110309, Considerations on Rel-17 UE features for IoT over NTN, SoftBank Corp.