3GPP TSG RAN WG1 #107-e     	                                       R1-2110983

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Clarification on SL HARQ-ACK for an incomplete PSFCH period

Agenda Item:	7.2.4

Document for:	Draft CR

- Introduction

In this contribution, we discuss the SL HARQ-ACK reporting on PUCCH for an incomplete PSFCH period.

- Discussion

According to the previous agreement, the number of SL slots of a resource pool in a DFN/SFN cycle may not always be multiple of NPSSCHPSFCH, where NPSSCHPSFCH is the PSFCH period, which means there may be an ‘incomplete’ PSFCH period that has less than NPSSCHPSFCH associated PSSCH occasions at the DFN/SFN cycle boundary.

Figure 1. Example of an incomplete PSFCH period

An example of an ‘incomplete’ PSFCH period is shown above. With PSFCH period NPSSCHPSFCH=4 and minimum PSSCH-PSFCH gap K=2, it can be derived that the time gap between a PSFCH occasion and its NPSSCHPSFCH corresponding candidate PSSCH occasions should be within the range [K, NPSSCHPSFCH+K-1]. However, when the number of SL slots in a 10240ms period is not a multiple of NPSSCHPSFCH, the number of PSSCH occasions that are actually associated with the latter PSFCH occasion among the NPSSCHPSFCH candidate PSSCH occasions is less than NPSSCHPSFCH. As illustrated in Figure 1, the PSFCH occasion in red has only NPSSCH_actualPSFCH=1 associated candidate PSSCH occasion.

TS 38.213 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel Set nF=0 – index of a SL slot within an PSFCH period while nF<NPSFCH MA=MA∪j; j=j+1; nF=nF+1; end while

According to the highlighted text in the current spec, NPSSCHPSFCH HARQ-ACK bits should be reported for each PSFCH occasion involved in a type1 SL HARQ codebook to maintain a constant size. Each of the candidate PSSCH occasions at slot n+2, n+3, n+4 are counted twice as associated and non-associated occasions for PSFCH occasion at slot n+6 and PSFCH occasion at n+7 respectively in the set of MA.

TS 38.213 - 16.5.1.1 Type-1 HARQ-ACK codebook in physical uplink control channel The cardinality of the set MA defines a total number M of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions corresponding to the HARQ-ACK information bits. A UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits as ojACK = HARQ-ACK information bit for candidate PSSCH transmission with index j with corresponding PSFCH reception, for 0 ≤j<M, as described in clause 16.5. If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasions.

Clause 16.5 specifies the procedure to derive the HARQ-ACK information bit for PSSCH transmissions associated with a PSFCH occasion based on the PSFCH reception. According to the red text, for PSFCH occasions at slot n+6, the corresponding HARQ-ACK bits for candidate PSSCH occasions at slot n+1~n+4 should be set to the HARQ-ACK information as described in clause 16.5.

However, like the example shown in Figure 1, for PSFCH occasions at slot n+7, it is not clear how to set the corresponding HARQ-ACK bit values for candidate PSSCH occasions at slot n+2~n+4 as these candidate PSSCH occasions are not associated with the PSFCH occasion.

There are two interpretations among companies based on the previous discussion:

- Interpretation 1: TX UE reports NPSSCHPSFCH HARQ-ACK bits for the ‘incomplete’ PSFCH occasion, including NACK for each non-associated PSSCH occasion and HARQ-ACK information derived based on clause 16.5 for each associated PSSCH occasion.

- interpretation 2: TX UE reports NPSSCH_actualPSFCH HARQ-ACK bits for the ‘incomplete’ PSFCH occasion, which means UE only generates HARQ-ACK for the associated PSSCH occasions.

It seems most companies prefer interpretation 1 (i.e., report NACK for the non-associated PSSCH occasions), while some companies commented that interpretation 1 is already covered by the green text. However, from our point of view, the green text cannot be applied to non-associated candidate PSSCH occasions case because the green text is clearly stated for the case where PSSCH is not transmitted due to the absence of DCI, while UE may have successfully received a DCI format scheduling one or more candidate PSSCH occasion at slot n+2, n+3, n+4 and have transmitted PSSCH in the non-associated occasion(s). Consequently, it is clear that the UE behavior is unspecified in this case.

To avoid any ambiguity, we propose the draft CR to clarify the UE behavior based on interpretation 1.

Observation 1. According to the spec, for a PSFCH occasion, there are NPSSCHPSFCH candidate PSSCH occasions, and NPSSCHPSFCH  HARQ-ACK bits should be generated to keep the constant type1 codebook size, where NPSSCHPSFCH is the PSFCH period; However, it is not clear how to determine the HARQ-ACK bit value for an ‘incomplete’ PSFCH period which has <NPSSCHPSFCH associated PSSCH occasion, the text for ‘If the UE does not transmit a PSSCH’ cannot be applied for this case.

Figure 2. Example of interpretation 1 and interpretation 2

Proposal 1. Adopt the draft CR on HARQ-ACK reporting for the ‘incomplete’ PSFCH period, i.e., TX UE generates NACK for each non-associated PSSCH occasion corresponding to an incomplete PSFCH occasion.

- Conclusion

Based on the above discussions, we have the following observations and proposals:

Observation 1. According to the spec, for a PSFCH occasion, there are NPSSCHPSFCH candidate PSSCH occasions, and NPSSCHPSFCH  HARQ-ACK bits should be generated to keep the constant type1 codebook size, where NPSSCHPSFCH is the PSFCH period; However, it is not clear how to determine the HARQ-ACK bit value for an ‘incomplete’ PSFCH period which has <NPSSCHPSFCH associated PSSCH occasion, the text for ‘If the UE does not transmit a PSSCH’ cannot be applied for this case.

Proposal 1. Adopt the draft CR on HARQ-ACK reporting for the ‘incomplete’ PSFCH period, i.e., TX UE generates NACK for each non-associated PSSCH occasion corresponding to an incomplete PSFCH occasion.

- Draft CR on 38.213

16.5.1.1	Type-1 HARQ-ACK codebook in physical uplink control channel

The cardinality of the set MA defines a total number M of occasions for candidate PSSCH transmissions with corresponding PSFCH reception occasions corresponding to the HARQ-ACK information bits. A UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of  OACK HARQ-ACK information bits as ojACK = HARQ-ACK information bit for candidate PSSCH transmission with index j with corresponding PSFCH reception, for  0 ≤j<M, as described in clause 16.5. If the UE transmits a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion but the slot of the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion is not associated with the slot of the corresponding PSFCH reception occasion as described in Clause 16.3, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion. If the UE does not transmit a PSSCH in an occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion, due to the UE not detecting a corresponding DCI format 3_0, the UE generates a NACK value for the occasion for candidate PSSCH transmission with corresponding PSFCH reception occasion.