3GPP TSG RAN WG1 #107-e		    	R1-2111516

e-Meeting, November 11th – 19th, 2021

Source:	Intel Corporation

Title:	NR MBS Group Scheduling for RRC_CONNECTED UEs

Agenda item:	8.12.1

Document for:	Discussion and Decision

# Introduction

In this paper, group scheduling for NR multicast and broadcast services (MBS) for RRC_CONECTED UEs is discussed. The new work item on NR Support of Multicast and Broadcast Services [1] has the following objectives for normative work on NR MBS related to multicast scheduling:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]: - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2] - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

Based on the agreements reached up to RAN1#106bis-e meetings [2][3], in this paper we provide further discussion on group scheduling to enable NR-MBS reception for RRC_CONNECTED UEs.

# Discussion

In this section we provide details on system design considerations for group scheduling for NR MBS including transmission schemes, common frequency resource and control signaling.

## 2.1. 	Common Frequency Resource for MBS Reception

While the details of CFR design for IDLE/INACTIVE mode UEs are still under discussion, RAN1 should strive to achieve a unified CFR design for CONNECTED and IDLE mode UEs. For example, if the default CFR of IDLE mode UEs is agreed to be CORESET#0, then CONNECTED mode UEs may also be configured with same CORESET#0.

- RAN1 should strive for unified CFR for CONNECTED and IDLE mode UEs

When the CFR is configured to be contained within the initial BWP or CORESET#0, there should be no additional specification necessary. The FDRA should be dimensioned based on the starting PRB index and length of the CFR as in the case of active UE BWP. Additionally, when CFR is larger than initial BWP, it can be assumed, that the UE is configured with a BWP larger than the CFR and it is up to the network to switch the UE to an active BWP which contains the CFR.

- The UE does not expect a CFR larger than the initial BWP if the initial BWP is the active BWP of the UE.

The above proposal also ensures are common design when Case A, C and E are supported for IDLE/INACTIVE UEs where, in Case E, the “configured BWP” is a new initial BWP for MBS capable UEs.

While it’s important to provide UE with necessary parameters via SIB or RRC signaling, a default assumption can also be defined where the default the MBS CFR is identical to active unicast BWP for the case when no configuration is provided to the UE. With this default CFR assumption, it is possible in principle to support multicast in a dedicated unicast BWP. However, the FDRA should be further discussed with respect to the reference point since multiple UEs will receive the same GC-PDCCH. Another point for further discussion is how to handle the case when some UEs in the group are provided a CFR configuration while some other UEs are not. One option is to assume that the it is up to the gNB to configure CFR for the other UEs in such a way that it aligns with the unicast BWP of the UE with the default CFR assumption.

- A default CFR identical to active unicast BWP can be defined for UEs when no CFR configuration is provided

## 2.2. 	Support of PTM Schemes for NR MBS

In RAN1#104bis-e [2] and RAN1#104-e  [3] the following agreements were reached for support of different transmission schemes based on different control signaling mechanisms:

Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB. Agreement: The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

PTM Scheme 1 is the baseline scheme that enables group-common transmissions for MBS. PTM Scheme 2 is additionally defined wherein UE specific PDCCH can be used to transmit or retransmit the group-common MBS PDSCH. The utility of PTM Scheme 2 for initial transmission may be limited since scheduling group-common PDSCH using UE-specific PDCCH adds large control overhead. Since PTM scheme 2 uses UE-specific PDCCH, the utility of this scheme becomes apparent only when ACK/NACK based HARQ is supported similar to the case for PTP based retransmission which was agreed in RAN1#104-e. For the NACK only feedback case, the gNB cannot discern which UE within the group has the failed TB and therefore UE specific retransmission may not be ideal since it needs to retransmit the failed TB to the entire group.

- PTM Scheme 2 should be supported when ACK/NACK based HARQ feedback is configured or enabled for the UEs within a group.

Since ACK/NACK based HARQ is supported for RRC_CONNECTED UEs which can receive high QoS MBS transmissions using delivery mode 1 based on RAN2 design, the support of PTP and PTM Scheme 2 simultaneously may not be mandated. Since both schemes are capable of scheduling individually to only UEs which transmitted a NACK, either one of the schemes can be supported and the support can be semi-statically configured via UE-specific RRC signaling.

- Only one among PTP or PTM Scheme 2 can be supported for UE specific retransmission when the initial transmission was based on PTM Scheme 1. The support of PTP or PTM Scheme 2 can be configured by UE-specific RRC signaling. Different UEs in a group can potentially support different retransmission schemes but not both simultaneously.

For the UE to associate the PTM scheme 2 based re-transmission to the initial transmission using PTM Scheme 1, the HARQ process ID can be used as an identifier similar to the PTP based retransmission case. Additionally, based on semi-static configuration of PTP or PTM scheme 2, the UE can identify whether to use C-RNTI (PTP) or the group common RNTI (PTM Scheme 2) to descramble the PDSCH. It is also assumed that the UE does not expect a unicast transmission with the same HARQ process ID as the ongoing MBS transmission.

- The HARQ process ID is used to associate PTM Scheme 2 based retransmission with the initial transmission using PTM Scheme 1. The UE does not expect to receive a unicast transmission using the same HARQ process ID as the ongoing MBS transmission.

In case, PTP and PTM scheme 2 needs to be simultaneously supported from the perspective of a single UE, then the HARQ process ID does not suffice since the UE will need an additional signaling especially for the case of PTP to differentiate it from PTM Scheme 2 since different RNTIs are used to scramble the PDSCH. In this case, an additional bit may be required in DCI to indicate between PTP or PTM Scheme 2. However, since the utility of simultaneously supporting the two schemes is not clear, such options may not be needed in practice.

In RAN1#105e, the following agreement was made with respect to HARQ process management for the case of PTM Scheme 1 and PTP based retransmissions:

Agreement: For HARQ process management, further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

The agreement was based on discussion that the HPN and NDI cannot uniquely differentiate a PTP based retransmission of an MBS TB from a PTP unicast retransmission since the NDI field is not set independently for MBS and unicast. This is indeed a problem when the UE misses a PTM scheme 1 initial scheduling DCI for example, and then tries to combine a PTP retransmission MBS TB with a unicast TB. However, the underlying assumption here is that the same HPN is used for both the transmissions and that the UE has not sent an ACK for the unicast TB yet. In this case, one solution is to mandate a better HARQ process management by ensuring that the network can only release a HPN for allocation to a PTM Scheme 1 based initial transmission only after the unicast transmission is fully completed or timed out. In this case, there would no conflict in HPN between the MBS and unicast transmissions.

- A UE does not expect PTM Scheme 1 based initial transmission or a PTP based retransmission of a MBS TB using a HARQ process number which is in use for an ongoing unicast transmission.

## 2.3. 	PDCCH Related Enhancements for MBS

### CORESET Configuration

The following agreements were reached for NR MBS CORESET configurations:

Agreement: If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options. - Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission. - Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission. - Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission. - Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission. Agreement: The maximum number of CORESETs per BWP is not increased for support of MBS, and the number of CORESETs configured within the CFR is left to gNB implementation.

Since the UE doesn’t support any additional CORESETs for MBS reception, Option 1 from the agreement above should be supported wherein MBS and unicast can be configured in any CORESET which is contained within the configured CFR

- The CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

For PTM Scheme 1, the CORESET for the group-common PDCCH is configured within the MBS frequency region, but for PTP or PTM scheme 2 which uses UE-specific PDCCH, the restriction is not required and the CORESET can be configured within the unicast BWP and outside the MBS frequency region.

- For PTP or PTM scheme 2, the CORESET scheduling MBS (re)transmission can be configured outside the MBS frequency region.

### Search Space Set Configuration

Conclusion: The specification impact of having a new Type-x CSS for GC-PDCCH in RRC_CONNECTED state can be studied and discussed further.

In our understanding, the Type-x CSS is a new CSS type since in the case of overbooking, the Type-x CSS can be treated similar to an existing USS and may be dropped based on the SS set index. Additionally, the same Type-x CSS can also be reused for IDLE/INACTIVE mode UEs. In current draft specification, this is not captured and we propose that Type-x CSS be captured as a new CSS type.

- Type-x CSS is a new CSS type different from Type 3 CSS which can be treated similar to USS in case of PDCCH overbooking.

### BD/CCE Limits

Agreement: The maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS. - FFS whether the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

For determining the BD/CCE limits for MBS, similar methodology as in the of multi-DCI multi-TRP can be re-used for CA capable UEs as mentioned in the FFS above. For UEs without CA capability, the main bullet holds.

- For determining BD/CEE limits for NR MBS in Rel-17, for CA capable UEs, the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs.

### DCI Formats

Since it has been agreed that HARQ feedback can be enabled or disabled by RRC (and also DCI when enabled by RRC), for the case when RRC disables HARQ feedback, certain DCI 1_0 fields may be reserved.

- When HARQ feedback is disabled by RRC, the following fields of DCI format 1_0 can be assumed to be reserved:

  - PUCCH resource Indicator

  - PDSCH-to-HARQ timing indicator

  - TPC command for scheduled PUCCH

  - HARQ Process Number

  - New Data Indicator

  - Redundancy Version

The fields that are unused should be assumed to be reserved. These fields can be utilized in later releases.

- The unused fields in DCI can be reserved

### DCI Size Budget and Alignment

Since DCI formats 1_0 and 1_1 have been agreed to be used, DCI size alignment either one of the 3 scheduling DCIs or to one other group-common DCI may be required based on the total size of the scheduling DCI. For the case of DCI 1_0, the FDRA field should be dimensioned based on the configured CFR. Depending on the size of the CFR with respect to the initial BWP or CORESET#0, the FDRA field may need to be truncated or padded with zeros to align the size of the MBS DCI 1_0 to the unicast DCI 1_0.

If the size of the MBS DCI 1_0 is smaller than the DCI 1_0 associated with CORESET#0 or initial BWP, padding bits may be added to align the size of the MBS DCI to the larger DCI. In another case, if the MBS DCI 1_0 is larger than the DCI 1_0 corresponding to CORESET#0 and smaller than the DCI 1_0 associated with active DL BWP, the DCI size might be aligned to CORESET#0 so that all UEs in the MBS group with varying DL BWP size can align to a common value. In this case the FDRA field of the MBS DCI 1_0 can be truncated to match it to the size of smaller unicast DCI. In another case, DCI 1_0 or 1_1 may be size aligned to a DCI size which is configured by the network to the UEs.

- For DCI 1_0 DCI size alignment can be performed by either zero-padding or truncating the MSBs of the FDRA field, depending on the relative size of the CFR with respect to CORESET#0 or the initial BWP, such that the DCI size aligns with that of unicast DCI format 1_0 corresponding to the CORESET#0 or the initial BWP.

- For DCI format 1_0 and 1_1, the DCI size can be aligned to a size which is configured by the network to the UE.

Agreement: Confirm the working assumption: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS. - FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

For the question of whether the G-RNTI is counted as C-RNTI or as other RNTI when considering the 3+1 DCI size budget, it may depend on the current DCI sizes of the UEs in the active MBS group and it can be up to the network to configure the UEs in the MBS group to either count it as part of the 3 or as part of the other group-common DCIs.

- For DCI size budget of “3+1”, the UE may be configured to align DCI size with either “3” scheduling DCIs or “1” other group-common DCI depending on network implementation.

## 2.4. Semi-Persistent Scheduling

Agreement: If a SPS-config for MBS is configured in CFR, one G-CS-RNTI is associated with the SPS-config. - FFS: Multiple G-CS-RNTIs associated with one SPS-config

For SPS activation for NR MBS, group common PDCCH with CRC scrambled by group RNTI can be used, where the HARQ ID field is set to all 0’s and the RV field is set to 00 for the TB being scheduled. PUCCH resource for carrying HARQ-ACK feedback of SPS PDSCH for MBS may be configured by higher layers via RMSI (SIB1), OSI or RRC signalling. The SPS release can be performed through a similar group common DCI with the HARQ ID set to all 0’s, MCS and FDRA field set to all 1s and RV set to 0. For the SPS release DCI, the UEs can send HARQ feedback through UE-specific (if ACK/NACK feedback is supported) or cell specific (if NACK only feedback is supported) PUCCH resource configured via dedicated RRC signalling.

- For DL SPS configuration for NR MBS

- Group common PDCCH is used for SPS activation with HARQ ID field set to all 0’s and RV field set to 00 for the TB being scheduled

- PUCCH resource for HARQ feedback may be configured via RMSI, OSI or RRC

- For SPS release, similar group common PDCCH can be used with HARQ ID set to all 0s, MCS and FDRA set all 1’s and RV set 0. For SPS release DCI, UE can be configured with PUCCH resource via RRC

- The PUCCH resources for HARQ feedback for SPS PDSCH as well as the SPS release DCI can be UE-specific for ACK/NACK based feedback or a common PUCCH resource can be configured for the case when NACK-only feedback is configured.

For the case of multiple G-CS-RNTIs, multiple SPS configurations can be used since they are possibly mapped to different services.

- Only one G-CS-RNTI is associated with one SPS-config

# Conclusions

In this paper, a high-level view of NR MBS design was presented. Group scheduling and transmission schemes were discussed, and the following proposals are made for further consideration

- RAN1 should strive for unified CFR for CONNECTED and IDLE mode UEs

- The UE does not expect a CFR larger than the initial BWP if the initial BWP is the active BWP of the UE

- A default CFR identical to active unicast BWP can be defined for UEs when no CFR configuration is provided

- PTM Scheme 2 should be supported when ACK/NACK based HARQ feedback is configured or enabled for the UEs within a group

- Only one among PTP or PTM Scheme 2 can be supported for UE specific retransmission when the initial transmission was based on PTM Scheme 1. The support of PTP or PTM Scheme 2 can be configured by UE-specific RRC signaling. Different UEs in a group can potentially support different retransmission schemes but not both simultaneously

- The HARQ process ID is used to associate PTM Scheme 2 based retransmission with the initial transmission using PTM Scheme 1. The UE does not expect to receive a unicast transmission using the same HARQ process ID as the ongoing MBS transmission

- A UE does not expect PTM Scheme 1 based initial transmission or a PTP based retransmission of a MBS TB using a HARQ process number which is in use for an ongoing unicast transmission

- The CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission

- For PTP or PTM scheme 2, the CORESET scheduling MBS (re)transmission can be configured outside the MBS frequency region

- Type-x CSS is a new CSS type different from Type 3 CSS which can be treated similar to USS in case of PDCCH overbooking

- For determining BD/CEE limits for NR MBS in Rel-17, for CA capable UEs, the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs

- When HARQ feedback is disabled by RRC, the following fields of DCI format 1_0 can be assumed to be reserved:

  - PUCCH resource Indicator

  - PDSCH-to-HARQ timing indicator

  - TPC command for scheduled PUCCH

  - HARQ Process Number

  - New Data Indicator

  - Redundancy Version

- The unused fields in DCI can be reserved

- For DCI 1_0 DCI size alignment can be performed by either zero-padding or truncating the MSBs of the FDRA field, depending on the relative size of the CFR with respect to CORESET#0 or the initial BWP, such that the DCI size aligns with that of unicast DCI format 1_0 corresponding to the CORESET#0 or the initial BWP

- For DCI format 1_0 and 1_1, the DCI size can be aligned to a size which is configured by the network to the UE

- For DCI size budget of “3+1”, the UE may be configured to align DCI size with either “3” scheduling DCIs or “1” other group-common DCI depending on network implementation

- For DL SPS configuration for NR MBS

- Group common PDCCH is used for SPS activation with HARQ ID field set to all 0’s and RV field set to 00 for the TB being scheduled

- PUCCH resource for HARQ feedback may be configured via RMSI, OSI or RRC

- For SPS release, similar group common PDCCH can be used with HARQ ID set to all 0s, MCS and FDRA set all 1’s and RV set 0. For SPS release DCI, UE can be configured with PUCCH resource via RRC

- The PUCCH resources for HARQ feedback for SPS PDSCH as well as the SPS release DCI can be UE-specific for ACK/NACK based feedback or a common PUCCH resource can be configured for the case when NACK-only feedback is configured.

- Only one G-CS-RNTI is associated with one SPS-config

# References

- RP-193248, New Work Item on NR Support of Multicast and Broadcast Services, Huawei, RAN#86, Sitges, Spain, December 2019.

- Chairman’s Notes, 3GPP RAN1 106bis-e Meeting, October 2021

- Chairman’s Notes, 3GPP RAN1 106-e Meeting, August 2021

- R1-2104165, LS on broadcast session delivery and MCCH design, 3GPP RAN1#105e, May 2021.

- 3GPP TR 36.890 v13.0.0, Study on Single Cell Point-to-Multipoint Transmission (Release 13)

- 3GPP TS 38.211 v16.2.0, NR Physical Channels and Modulation (Release 16)

- 3GPP TS 38.212 v16.2.0, NR Multiplexing and Channel Coding (Release 16)

- 3GPP TS 38.213 v16.2.0, NR Physical Layer Procedures for Control (Release 16)

- 3GPP TS 38.214 v16.2.0, NR Physical Layer Procedures for Data (Release 16)