3GPP TSG RAN WG1 #107-e			      R1-2112307

e-Meeting, November 11th – 19th, 2021

Agenda Item: 8.7.1.1

Source: MediaTek Inc.

Title: Summary #1 of Paging Enhancements

Document for: Discussion and Decision

# Introduction

In RAN#93-e, there agreed the following guidance for the design of paging early indication (PEI) [1]:

- Support PDCCH-based PEI as the only option • Only essential function for PEI is support • New DCI format • Higher layer configuration, including SS • Details of the procedures of PEI monitoring, and identification of MOs before PO • Only Behv-A (per RAN1#104e agreement) is supported • If TRS availability indication is agreed to be supported in both paging DCI and the DCI format for PEI, same mechanism/principle for TRS availability indication is adopted for the two DCI formats • Supporting TRS availability indication in DCI format for PEI shall not delay the completion of essential functionality of PEI

In RAN1#106-bis-e, the following agreements are made for PEI DCI format, PEI search space configuration, PEI occasion (PEI-O) definition and determination of PEI-O location [2]:

Agreement For NR Rel-17, paging indications to UE subgroups are carried only in PEI. Agreement For PEI, a new DCI format is supported to include at least paging indications to UE group(s)/subgroups of the associated PO(s) - One bit in the DCI payload indicating one UE subgroup of a PO or one UE group/PO - The maximum number of total bits for paging indication field in PEI DCI format is x - One PEI can be configured to indicate up to 4 PO(s) in a PF - FFS whether to supporting map PEI to 3 POs in a PF - FFS: 1 PEI for POs across multiple PFs - FFS: value of x Agreement A PEI occasion (PEI-O) is a set of S consecutive PDCCH monitoring occasions when nrofPDCCH-MonitoringOccasionPerSSB-InPO is not configured - S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 - The K-th PDCCH monitoring occasion for PEI in the PEI-O has the same QCL assumption as that of the K-th PDCCH monitoring occasion for paging in the PO. - Note: QCL reference is SSB - FFS: Determination of the PEI-O location - FFS: Support of unlicensed spectrum operation with nrofPDCCH-MonitoringOccasionPerSSB-InPO configured Agreement CORESET # 0 or commonControlResourceSet in SIB1 can be used for PEI - Note: The number of CORESETs configured for a UE follows the requirement of UE feature 3-1 Agreement Support configuration of a dedicated search space (‘peiSearchSpace’) for PEI - FFS: Configuration details and whether and how to reuse legacy search space sets, including pagingSearchSpace and searchSpaceSetZero Agreement Determination of PEI-O location for a target PO is based on one of the following alternatives: - Alt 1: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO - FFS: The unit and the range of the frame-level offset - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion (e.g., to be the same as those of firstPDCCH-MonitoringOccasionOfPO) - Alt 2: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the L-th SS burst before the first PDCCH monitoring occasion of the target PO. - FFS: the case that a SSB burst overlaps in time with the target PO - FFS: L = 1, 2 or 3 - FFS: Reference the “start” or “end” of the L-th SS burst - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion - Alt 3: The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO. - FFS: The exact definition of the reference time, e.g. the first MO of the target PO, the first MO of the first PO indicated by the PEI, the start of the PF for the target PO - FFS: The unit and the range of the time offset - FFS: Whether any SS burst or TRS burst is needed between PEI-O and PO - Configuration for one PEI indicating multiple POs within a PF should be taken into consideration in the determination of PEI occasion Decide one of the above alternatives or a single merged solution based on the alternatives in RAN1#107-e meeting. FFS: Extension for the case one PEI indicates multiple POs across multiple PFs, if supported

In the following sections, companies’ proposals for the remaining design details will be collected and discussed for the final proposals and decisions for completing PEI specification:

- Section 2: DCI format 2_7 designs

- Section 3: Remaining PEI occasion (PEI-O) related designs

- Section 4: Remaining ‘peiSearchSpace’ related designs

- Section 5: Other remaining issues

# DCI Format 2_7 Designs

Regarding DCI format design, Sections 2.1 to 2.4 are devoted for the following remaining design details:

- The maximum number of bits for paging indication field in DCI format 2_7

- PEI to PO mapping design details, including

  - Whether to support mapping PEI to 3 POs in a PF

  - Whether to support 1 PEI for POs across multiple PFs

- Detailed DCI content design, including DCI bit mapping design for multiple POs

- RNTI for PEI DCI format

## Maximum number of bits for paging indication field in DCI format 2_7

In the following table, companies’ related views and proposals are collected:

| Company | Companies’ views and proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 6: The maximum bits of paging indication field is preferred to be 32. |
| ZTE, Sanechips |  |
| vivo | Proposal 1: The maximum field size for paging indication should be 16bits. |
| TCL |  |
| Spreadtrum | Proposal 1: The maximum number of total bits for PEI PDCCH is 32. |
| CATT | Proposal 3: The maximum number of total bits for paging indication field in PDCCH-based PEI is 40. |
| OPPO | Proposal 2: The maximum number of total bits (x) for paging indication field in PEI DCI format is 32. |
| Sony |  |
| Intel | Proposal 2: The maximum size of the bitmap used in PEI for sub-grouping information is 16. Observation 1: Maximum size of PEI DCI format should be significantly smaller than that for paging DCI format to ensure overall reliability. |
| Xiaomi | Proposal 4: Support PEI-DCI has the same DCI payload as paging DCI, typically 40 bits, and has different RNTI from P-RNTI Proposal 5: Support 8*M bits subgrouping paging indication in PEI, M is the number of associated POs and maximum M=4. |
| CMCC |  |
| Transsion Holdings | Proposal 2: The maximum number of total bits in PEI DCI format is 35 bit. |
| Panasonic |  |
| Samsung | Observation 1: The payload size for PEI PDCCH should be smaller than paging PDCCH in order to achieve target reliability and power saving gain. Proposal 1: Support PEI with UE subgrouping as the only function for paging enhancements in NR Rel-17. Proposal 4: The payload size of the DCI format for PEI, NbitsPEI, can be determined as NbitsPEI=NUE subgroupsPO∙NPOsPEI, where - NUE subgroupsPO is the number of UE subgroups per PO, i.e. subgroupsNumPerPO with applicable values of 1 to 8, - NPOsPEI is the number of POs mapped to one PEI-O, i.e. number-of-PO with applicable values of 1 to 4. - Maximum value for NbitsPEI is 32. |
| Apple | Proposal 1: The size of the PEI DCI is configurable by the gNB, and it should be no larger than the size of paging DCI. |
| Lenovo, Motorola Mobility |  |
| InterDigital | Proposal 1: The maximum number of total bits for paging indication field in the PEI DCI format is 32. |
| LGE |  |
| NTT DOCOMO |  |
| Ericsson | - Observation 2 Configurable PEI DCI with payload size up to 41 bits can satisfy desired performance requirements. - Proposal 1 Similar to paging, for which NW can adapt PDCCH aggregation level based on deployment, aggregation levels 4, 8, and 16 are configurable for PEI. - Proposal 8 PEI DCI payload size is configured through a higher layer broadcast configuration parameter, including a maximum value 41 bits. |
| Qualcomm | Proposal 4: For the mapping between PEI PDCCH and POs - … - Maximum DCI size for PEI PDCCH should guarantee the missed detection performance of PEI is at least one order lower than the paging PDCCH - Network properly configures the number of sub-groups per PO so that number of bits in each PEI PDCCH does not exceed the maximum DCI size |
| MediaTek | Observation 1: From the identified PDCCH settings that can comply with the mandatory performance criterion, Table 1 provides the performance-equivalent settings subject to the same PDCCH code rates. It is observed that 32-bit paging indication field can be accommodated without violating the performance criterion. Proposal 1: Maximum paging indication field size X = 32 bits in PEI DCI format is supported. Table 1: Identified PDCCH settings that can comply with the mandatory performance criterion Identified settings in RAN1#104-bis-e; RPDCCH; Performance-equivalent settings (same RPDCCH). (12 payload bits, 24 CRC bits, AL4); 116; (48 payload bits, 24 CRC bits, AL8). (12 payload bits, 24 CRC bits, AL8); 132; (48 payload bits, 24 CRC bits, AL16). (41 payload bits, 24 CRC bits, AL8); ∼117.75; (41 payload bits, 24 CRC bits, AL8) No change since payload size > 32 bits already. |
| Nokia | Observation: The DCI size for PDCCH-based PEI should be configurable, from 1 to [35] bits. |
| Nordic | Proposal-1: Support only the cases where PEI is mapped to 1,2 or 4 POs of a single PF in R17. Max size of DCI format payload is 34 bits. - Up to RAN2 whether to reuse P-RNTI or introduce PEI-RNTI |

From the above table, we can first obtain the following statistics:

Number of bits for paging indication field:

- 16 bits (2 companies): vivo, Intel

- 32 bits (9 companies): HW & HiSi, OPPO, Xiaomi, Transsion, Samsung, InterDigital, MediaTek, Nokia, Nordic

- 40 bits (1 company): CATT

From the statistics, 32-bit paging indication size looks of majority support. For proponents who propose maximum of 16-bit payload, the major concern is performance for a given AL. Since there is AL value that can be adjusted for performance, 32-bit payload can be accommodated by a double AL value. Therefore, moderator suggests to add a note on AL flexibility and to address the performance consideration.

Regarding CATT proposal of 40 bits, the intention is to keep maximum network flexibility. For ‘subgroupsNumPerPO’ = 8, 40 bits can accommodate up to 5 POs. Since the total PO number in a paging cycle is a power of 2, Moderator would like to suggest considering “up to 32-bit paging indication filed size” first and ask companies’ check whether there is strong use case/PO configuration that requires 40-bit paging indication filed size.

By the above, the following questions are proposed for companies further inputs to .

Q1: Comparing with 32-bit maximum paging indication size, do you see any strong use case/PO configuration that will require a larger maximum paging indication size?

Q2: Please provide your views and/or suggested revision for the following proposal:

Proposal 2.1-1:

The maximum number of total bits for paging indication field in the DCI format 2_7 is 32.

- Note: It is network flexibility to adjust payload size and AL value for reliable paging performance

Table 1: Companies’ views to the above questions

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Q1: No, we don’t see a strong need. Q2: Support Proposal 2.1-1 |
| MediaTek | For Q1: For paging indication field, we think 32 bits will be very sufficient. Given maximum UE subgroup number of 8, accommodating 1/2/4 POs will be common and useful since the number of total POs in a paging cycle is a power of 2. No strong use case for more than 32 bits is identified by us. For Q2: We are supportive of Proposal 2.1-1. It is noticed that maximum payload of 32 bits doesn’t preclude the configuration of 16-bit payload. Having the flexibility for network performance and overhead trade-off is useful for PEI feature to accommodate various use cases. |
| Nordic | Q1: To include TRS validation bit, but could be handled by reducing number of sub-groups. On the other hand, 1 extra bit would not change PDCCH performance. Q2: We suggested up to 34 |
| Samsung | Q1: No. Q2: We don’t have problem with the proposal. But we think it’s better to discuss the maximum payload size of the entire DCI format instead of the size of the field. For the field size, it depends on the maximum number of POs per PEI, which is under discussion in Section 2.2. |
| Panasonic | On Q1, we are fine with 32-bit and open with 40 bits. On Q2, okay with us. |
| CATT | Ericsson’s contribution also supports up to 41 bits payload size of DCI format 2_7 for PEI Q1: The maximum number of bits in DCI format 2_7 defined in the specification is not to limit the flexibility of the network configuration with the consideration of PEI detection performance. Similar to the agreed number of subgroups is also up to 8 although 8 paging subgroups are not most common use cases. Q2: Since UE preparation in decoding PEI and paging DCI are the same, the detection performance of PEI and paging DCI should be the similar. Thus, the maximum payload size of PEI should not be more than that of paging DCI. Our suggestion of Proposal 2.1-1 is as follows, The maximum number of total bits for paging indication field in the DCI format 2_7 should not be more than that of paging DCI. |
| Intel | Q1: In our view, max size for paging indication field should be less than 32 bits Q2: We have concern on using 32 bits for paging indication field. As mentioned by FL and also reported by some companies, AL may need to be doubled for PEI DCI (compared to paging DCI) in some cases to ensure overall reliability. However, AL = 16 may not be usable in some scenario such as when CORESET 0 BW is small. |
| Qualcomm | Q1: No. Q2: We do not think 32 bits are necessary just because it supports up to 8 subgroups for 4 POs in a PF. As observed in companies’ evaluation results, UE sub-grouping provides a small additional power saving gain compared to other sources of power saving gain. There is no need for network to use 8 bits for each PO for all paging configurations. So we are supportive of 16 bits. We agree with Samsung that the maximum total number of bits for PEI DCI should be determined from decoding performance perspective. |
| Sharp | Q1: maximum 32bits for paging indication is ok and the DCI payload can be more than 32bits Q2:ok |
| CMCC | Q1: No Q2: Ok |
| ZTE, Sanechips | It is understood that the maximum payload size is relevant to the number of POs, which also determines the performance of PEI. If the maximum bits for paging information is 32, the payload size will exceed paging DCI if TRS availability information and other information are considered. A potential compromised value is 24. |
| LGE | Q1: No. We think 32 bits are large enough to convey paging indication for PO(s). Q2: We support the proposal 2.1-1. |
| Ericsson1 | Q1: No - 32 bits total for subgroup indication is sufficient. There can be additional fields in the PEI DCI such as TRS availability, reserved bits that should also be allowed for future extension, and for this we propose to allow max PEI DCI size of 41 bits. Q2: Support - The field size and supported ALs should be flexible enough to allow a single PEI to address up to 32 subgroups (8 subgroups per PO x 4 POs) and leave the configuration details including actual DCI payload size to NW implementation. |
| Huawei, HiSilicon | Q1: no. Q2: support the proposal. Regarding some concerns from companies to insist on smaller maximum value, I would say the proposal does not exclude that gNB can configure 16bits paging indication fields. gNB can have the flexibility to configure the field size based on the deployment. 32 bits is also needed when gNB can provide larger AL transmissions. |
| vivo | Q1: No. Even 32-bit maximum paging indication size is too large to guarantee the PEI DCI reliability. Not to mention a larger maximum paging indication size. Surely, there is no essential use case/PO configuration for a larger maximum paging indication size. Q2: We have concern on this proposal. As mentioned by Intel, even if the network can configure a large AL value (that is, AL>=8), the 32bits paging indication field does not satisfy PEI DCI reliability under some cases. Hence, 16bits will be a compromise compared to 32bits and 12bits. For 32-bit payload size, one of the use cases is to contain 4 POs and each with 8 sub-groups. Alternatively, it can be realized by 2 PEIs each payload is 16 bits or 4 PEIs each payload is 8 bits. - In the last meeting, it has agreed the number of POs that a PEI indicates is configurable. Hence, we think the same functionality has been achieved by the alternative way. We don’t need to optimize the 4 POs and each with 8 sub-groups by accommodating 32-bits in a DCI. Some companies claimed that one of the pros of 32-bits compared to 2 PEIs with each payload 16-bits is it can avoid the PDCCH blocking issues, we think it is not always true. The probability two PEIs are simultaneously transmitted can be calculated by multiplying two PO wake-up rate. If the PO rate is 10%, the probability of simultaneously transmitting two PEI in one slot is 1%. We think by extending the payload from 16-bit to 32-bit in order to avoid 1% bad case is not a good design. Meanwhile, even such case occurs, the blocking may occur and may not. |
| DOCOMO | Q1: No. Q2: Support Proposal 2.1-1 |
| Apple | Q1: PEI can also be used to carry e.g. TRS indication, SI update, etc. So we don’t see a need to cap the max size at 32 bits. Q2: Not preferred. As indicated by the note, the size and AL are configurable by the gNB, so the parameters can be chosen properly by the gNB to ensure the performance. We do not see a need to limit the size from spec point of view. In addition, we wonder if we need to discuss this proposal given that we have P2.3-2. What matters for the performance is the DCI size, not how many bits for paging indication. |
| Lenovo/MotM | Q1: No, Q2: Fine with the proposal 2.1-1. |
| Spreadtrum | Q1: No Q2: Yes |
| OPPO | Q1: No. Q2: Support the proposal 2.1-1. |
| TCL | Q1: we support the maximum number of paging indication field in PEI is 32. However, we also proposed 6 bits for TRS availability indication (similar to the reserve bits of paging DCI) and 2 bits for SI/ETWS notification. So we support the total number of maximum bits for PEI DCI is 40, but we also open for total number of maximum bits as 41 bits. Q1: we are fine with proposal 2.1.-1. |
| Transsion Holdings | Q1: No Q2: From my perspective, 32 bit is enough for paging indication field. We agree Samsung’s opinion that we need to discuss the maximum payload size of the entire DCI format instead of the size of the specific field. |
| Nokia(1st round) | Q1: So based on earlier agreement, we should be able to map at least 4 POs (of a PF) to a PEI. Now the detailed discussion in RAN2 is open, but for sake of discussion lets assume that sub-grouping field size can be configured freely between 1 and 8 bits. Thus, 32 bits would suffice for paging sub-grouping indication. However, if we agree to add some additional information, we will introduce dependency between the sub-grouping field size and the other information, which is not very desirable. However, if majority view is 32 bits, we would be fine to consider this as a working assumption, with the condition that the DCI size can be configured independently of the number of POs mapped to PEI. Q2: Following the afore comment we think that it might be best if try to clarify whether we discuss about the DCI format 2_7 size or about the size of the information related to paging. I understand that these were split on purpose (e.g. Proposal 2.3-2) but there is no need to inform the UE about the full size of the paging information field (see our comments in next section), and in context of progress it might be easier to focus on the key aspect which is the maximum size of the DCI. As noted above we also suggest to consider the 32 bits as a working assumption with the aim to conclude during this week: Proposal 2.1-1_NOK: The maximum configurable size of the DCI format 2_7 number of total bits for paging indication field in the DCI format 2_7 is [32]. - Note: It is network flexibility to adjust payload size and AL value for reliable paging performance |
| IDCC | We are ok with the proposal. |

For Q1, quoted below for ease of reference,

Q1: Comparing with 32-bit maximum paging indication size, do you see any strong use case/PO configuration that will require a larger maximum paging indication size?

- Yes (>32-bit paging indication field size): CATT (preference but no specific use case shown)

- No (<= 32-bit paging indication filed size): Xiaomi, MTK, Samsung, Intel (16 bits), QC, Sharp, CMCC, ZTE (24 bits), LGE, Ericsson, Huawei, Panasonic, vivo, DoCoMo, Lenovo/MotM

It is noticed that Q1 is related to “paging indication field size” while there may be some companies think it is related to DCI payload size.

From companies’ feedback, there is no strong use case requiring >32 bits for paging indication field reported by companies.

For Q2, quoted below for ease of reference,

Proposal 2.1-1: The maximum number of total bits for paging indication field in the DCI format 2_7 is 32. - Note: It is network flexibility to adjust payload size and AL value for reliable paging performance

- Yes (11 companies): Xiaomi, MTK, Samsung, Panasonic, Sharp, CMCC, LGE, Ericsson, HW & HiSi. DoCoMo, Lenovo/MotM

- No (6 companies): CATT (maximum ≤ payload of paging DCI), Intel (16 bits), QC (16 bits), ZTE (24 bits), vivo (16 bits), Apple

By the above statistics, directly restricting the maximum size for paging indication field looks not of consensus. Since the overall PEI performance is determined by the payload size, moderator would like to suggest merging this discussion to Section 2.3 for a joint proposal addressing DCI format 2_7 payload size.

## PEI to PO Mapping Design

In the following table, companies’ related views and proposals are collected:

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon | Proposal 7: One PEI can be allowed to associate with POs across PFs, only when the two PFs are two consecutive radio frames in one SSB periodicity. |
| ZTE, Sanechips | - Proposal 1: One PEI configured to indicate POs across multiple PFs should be supported. |
| vivo | Proposal 2: One PEI for POs across multiple PFs should be precluded. - Issue 3: Whether to supporting map PEI to 3 POs in a PF It is up to gNB implementation. For example, when there are two PEI-Os configured in one PEI-F and one PF contains 4 POs, it is possible that one of the PEI-Os indicates 3 POs and the other indicates the rest one PO. |
| TCL | Figure 6 One PEI mapping to 3 POs in a PF Figure 7 One PEI mapping to 3 POs across the PF Proposal 7: Support one PEI mapping to 1,2,3, and 4 PO(s) in a PF and across the PF. |
| Spreadtrum | Proposal 2: Prioritize one PEI indicating 1 or 2 or 4 PO(s) in a PF. Table 1: One PEI indicates up to 4 PO(s) in PF(s) M=1 (1 PEI indicating 1 PO); M=2 (1 PEI indicating 2 POs); M=3 (1 PEI indicating 3 POs); M=4 (1 PEI indicating 4 POs). Ns=1; PEI -> 1 PF*; PEI -> 2 PFs**; PEI -> 3 PFs; PEI -> 4 PFs. Ns=2; PEI -> 1/2 PF; PEI -> 1 PF; PEI -> 3/2 PFs; PEI -> 2 PFs. Ns=4; PEI -> 1/4 PF; PEI -> 1/2 PF; PEI -> 3/4 PFs; PEI -> 1 PFs. *Note: The green front is supported by current agreement. **Note: The blue front is possible cases for 1 PEI indicating POs across PFs. Proposal 3: Support one PEI indicating POs across 2 or 4 PFs for Ns = 1 or 2 respectively. |
| CATT | Figure 3: An example of the scenario of large number of POs Proposal 2: Support one PDCCH-based PEI for POs across multiple PFs. Proposal 4: The POs cross PF associated with one PEI should be supported with maximum number of POs associated with one PEI being 8. Proposal 5: The standard specification should support any number of POs associated with one PEI and any number of paging subgroups within a PO for the flexibility of network configuration. |
| OPPO | Proposal 1: The number of POs (N) indicated by a PEI could be configured independently to the number of paging occasions (Ns) within a paging frame, and the maximum number of POs (Nmax) indicated by a PEI should depend on the size of the DCI format and the number of subgroups of one PO. - support map PEI to 3 POs in a PF; - support one PEI for POs across multiple PFs. |
| Sony |  |
| Intel | Proposal 3: Support 1 PEI to 1 PO association. Proposal 4: One PEI addressing multiple POs across multiple PFs is not supported. |
| Xiaomi | Proposal 6: Support one PEI can only corresponds to POs within one PF. |
| CMCC | Proposal 1. Don’t support mapping 1 PEI to 3 POs in a PF. Proposal 2. Support 1 PEI indicating multiple POs across multiple PFs. |
| Transsion Holdings | Proposal 1: One PEI associated with N POs and N can be the multiple of Ns. |
| Panasonic | Observation 1: To transmit as less PEI as possible is beneficial for gNB to save system overhead. Observation 2: A PEI indicating multiple POs, which are always all the POs within one or multiple PFs, can provide sufficient flexibility for resource allocation. PF level granularity can ease the standardization efforts to finalize Rel.17. |
| Samsung | Observation 2: Restriction that one PEI to indicate up to 4 PO(s) within a PF limits NW’s flexibility to reduce system overhead on PEI transmission when gNB only configures 1 or 2 POs per PF. Proposal 2: Support one PEI for PO(s) across multiple PFs. |
| Apple | Proposal 4: Do not support one PEI indicating multiple POs across multiple PFs. Proposal 5: The number of POs corresponding to one PEI ({1, 2, 4}) is indicated to the UEs. |
| Lenovo, Motorola Mobility | Proposal 2: One PEI maps to only one PF. The number of POs that one PEI indicates is implicitly configured based on the number of POs for a PF. |
| InterDigital |  |
| LGE |  |
| NTT DOCOMO | Proposed 1: Single PEI associated with Single PO or Multi POs within up to 4 POs should be supported. Proposed 2: Do not support 1 PEI for POs across multiple PFs. |
| Ericsson | - Observation 1 In deployments where one-to-many PEI configuration is used, irrespective of number of POs addressed it is simpler to configure PEI offset relative to a single PF as the reference point. - Proposal 6 For one-to-many PEI configuration, up to 32 bits (configurable) are used for addressing UEs in up to 4 consecutive POs associated with the same paging frame. - Proposal 7 PEI one-to-one/one-to-many configuration is done through a higher layer broadcast configuration parameter. The range of the parameter is 1, 2, 4, where 1 indicates one-to-one mapping. One parameter is applicable to all POs of the cell. |
| Qualcomm | Proposal 4: For the mapping between PEI PDCCH and POs - Support mapping PEI to 3 POs in a PF - Note: subset of POs in a PF may have same PEI location based on PEI alignment with SSB burst - Support 1 PEI for POs across multiple PFs - Maximum DCI size for PEI PDCCH should guarantee the missed detection performance of PEI is at least one order lower than the paging PDCCH - Network properly configures the number of sub-groups per PO so that number of bits in each PEI PDCCH does not exceed the maximum DCI size |
| MediaTek | Proposal 2: Mapping 1 PEI to 3 POs in a PF is not supported due to unequal PEI performance across POs - Define a constant RRC parameter, POnumPerPEI, which is a factor of Ns and has value range {1, 2, 4}. Proposal 3: 1 PEI for POs across multiple PFs is not supported - Note: It is network flexibility to set Ns = 2 or 4 for exploiting PEI indication capacity for multiple POs |
| Nokia | Proposal: It should be under network configuration how many POs (e.g. of a PF) map to a PEI. |
| Nordic | Proposal-1: Support only the cases where PEI is mapped to 1,2 or 4 POs of a single PF in R17. Max size of DCI format payload is 34 bits. - Up to RAN2 whether to reuse P-RNTI or introduce PEI-RNTI |

From the above table, the following statistics can be obtained:

Support of 1 PEI for 3 POs:

- Yes (6 companies): vivo, TCL, CATT, OPPO, QC, Nokia,

- No (12 companies): Spreadtrum, Xiaomi, CMCC, Transsion (Ns or multiple of Ns), Panasonic (Ns or multiple of Ns), Apple, Lenovo & Moto (1 to Ns), Ericsson (1, 2 or 4), MTK, Nordic (1, 2 or 4), LGE(Ns), DoCoMo (1, 2 or 4)

Support multiple PFs:

- Yes (11 companies): HW & HiSi (Consecutive PFs), ZTE, TCL, Spreadtrum (2 or 4 PFs conditioned on Ns), CATT (up to 8 POs), OPPO, Transsion, CMCC, Panasonic, QC, Samsung

- No (9 companies): vivo, Xiaomi, Apple, Lenovo & Moto, DoCoMo, Ericsson, MTK, Nordic, LGE

Regarding whether to support 1 PEI for 3 POs in a PF, “Not support” is the majority view. On the other hand, it is observed that proponents of “support” actually refer to the case 1 PEI can be mapped to multiple PFs. Given the dependency,

Q1: Please provide your views on the following proposal and suggested range for POnumPerPEI:

Proposal 2.2-1:

Support mapping a constant number of PO(s) per PEI.

- Include a configurable parameter, ‘POnumPerPEI’, with possible value in the range {1, 2, ….}

Regarding whether to support multiple PFs, proponents of “support” consider the cases with Ns = 1, so that the indication capability is not fully exploited. On the other hand, it is network flexibility in adjusting Ns. In this regard, moderator would like first to check companies’ views in utilizing this network flexibility so that previous agreement for 1 PF may be sufficient:

Q2: To utilize one PEI for multiple POs, do you think network can always increase Ns value of PO configuration? If yes, do you think whether one PEI for up to 4 POs in a PF is sufficient or not?

In previous meeting, proponents of 1-1 PEI-PO mapping argue diverse UE processing timeline when supporting multiple POs. To address their concern, restriction to 1 PF is finally decided. In order not to repeat the debate/argument, moderator would like to check whether restricting the multiple PFs in adjacent radio frames can be a possible way forward:

Q3: If 1 PEI for multiple PFs is to be supported, do you think fine to put some restriction, as Proposal 2.2-2? What is your suggested maximum value on Y?

Proposal 2.2-2:

Support mapping 1 PEI to up to Y PFs in adjacent radio frames

- Y = 2

Table 2: Companies’ views to the above questions

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Q1: If one PEI can only corresponds to multiple POs within a PF, we think it is more simple and direct to let one PEI corresponds to all POs within a PF. We support mapping a constant number of PO(s) per PEI, but prefer to fixed as Ns. Q2: We agree that one PEI for up to 4 POs in a PF is sufficient. Q3: We think fine to put some restriction. And considering Ns=1 as minimum, maximum Y can be 4 to reach 32 bit paging indication |
| MediaTek | For Q1: We support Proposal 2.2-1. Supporting a constant number of PO(s) per PEI is a reasonable way forward, that simplifies the signalling and specification complexity. For the range, we suggest {1, 2, 4} regarding that 1) Total PO number is a power of 2, and 2) 32 bits can accommodate maximum of 8 subgroups for 4 POs, which implies setting maximum of 4 POs ensures no limitation to network UE subgrouping. For Q2: We think network can always increase Ns to 4 in PO configuration. Regarding the concern 4 POs in a PF may be too crowded, we would like to clarify PF is a reference frame and PO start can actually span whole PF interval (which can be up to 160 ms) with symbol granularity. In this regard, we think there is no issue for network to set Ns = 4 for a PF, and current agreement to support 1 PEI for up to 4 POs in PF is also sufficient. For Q3: By the above clarification, we think there is no issue for network to set Ns = 4 for a PF. In this regard, we are not supportive of Proposal 2.2-2 |
| Nordic | Q1: Agree Q2: PEI to 1,2,4 should be supported Q3: We do not support PEI to multiple PF |
| Samsung | Q1: We are supportive of more configuration flexibility on ‘POnumPerPEI’. We are fine with the value of POnumPerPEI’ in the range of 1:4. But we want to clarify that POnumPerPEI can be configured to any of the possible values regardless of the value of Ns. For value of 3, we are OK with it as long as the number of POs per PEI is not restricted to one PF. Q2: No, whether or not gNB can configure a large number of Ns depends on the traffic, or available channel resources in a frame. In general, we think gNB should have the flexibility to configure Ns and PEI independently. So, we support 1 PEI across multiple PFs. Q3: No, we think the configuration of ‘POnumPerPEI’ is sufficient. |
| Panasonic | On Q1, proposal 2.2-1 is okay with us. On Q2, using PEI supporting multiple PFs is not same with adapting the Ns numbers within a PF. The Ns is related to the paging load and the control overhead network would like to take. However, using PEI to cover multiple POs within and across multiple PFs is to reduce the PEI overhead. So we think these two can not be traded off. On Q3, we already defined ‘number-of-PO’, which is the number of POs that a PEI can indicate. The number of PF can be decided based on the paging early indication field bit number, ‘number-of-PO’ and subgroupsNumPerPO. So there is no need to set a limit for number of PFs. |
| CATT | Q1: The specification should support the configuration with flexible number of POs associated with one PEI regardless the Paging frame boundary. Q2: No. The generic scenario is that the network only configures one PO per paging frame. The additional PO within Paging frame was introduced in Rel-15 when there are large number of IDLE/Inactive UEs within a cell. Ns = 4 was added for NR-U in Rel-16. The mapping of PEI to the associated POs should not assume any change of Ns values. Q3: When one PEI associated with multiple POs cross paging frame, the number of PFs should be large enough for the flexibility of network configuration and the efficiency of DCI format 2_7. If paging subgroups are not configured by the network, each bit in the DCI format 2_7 payload is associated with one paging group in a PO. If Y value is very small, the payload size of DCI format 2_7 would be very small, which is large overhead, when paging subgroup is not configured with a given cell. Thus, we suggest that Y value should be at least 8. |
| Intel | Support Q1. Since Ns can be 1, 2, or 4, and we have agreed that PEI can address upto 4 POs, it would be simpler to just adopt Ns as the number of POs addressed by PEI. Q2: Supporting up to 4 POs in a PF is sufficient. Q3: We do not see strong need to support multiple PFs. There is chance of increased paging latency and increased UE power consumption for UEs in the later PFs as consecutive PFs may not be next to each other and also there is concern on payload increase for PEI DCI which may affect reliability. |
| Qualcomm | Q1: we do not support constant or configured number of POs mapped to each PEI. The number of POs should be determined by the timeline relationship. We support the alignment between SSB and PEI. Based on this, the number of POs mapped to a PEI will be determined by the number of POs between two adjacent SSBs. Together with the maximum DCI size, network can determine the maximum number of sub-groups that can be configured for each PO. Q2: yes, we agree network can increase Ns without exceeding the maximum DCI size for PEI by adjusting the number of subgroups per PO. We do not think one PEI mapping to POs only within one PF is sufficient. Again, this should be determined by the timeline relationship. We support alignment between PEI and SSB. Then there can be 2 PFs (all POs) or 3 PFs (including subset of POs) for 20ms SSB periodicity whose POs are mapped to the same PEI. Q3: for typical 20ms SSB periodicity, Y can be 2 or 3 (3 if subset of POs for the first and third PFs are mapped to the PEI, and all POs for the second PF are mapped to the same PEI). So we think the Y value can be implicitly determined without explicit configuration. |
| Sharp | Q1: the value of range can be {1,2,4,8,16} Q2: Support up to 4 POs in a PF Q3: Support onePEI- to-multiple PFs and a PEI’s capacity can be defined to indicated how many PO will be associated |
| CMCC | Q1: We support this proposal, and also prefer to take Ns as the constant number, but the main bullet needs more clarification, especially considering the proposal in Q2/Q3, if we agree one PEI associated multiple PEIs, and one PEI can associate with 4 POs in a PF, we also support the other two cases: one PEI associate 2PFs with Ns=2, and one PEI associates 4 PFs with Ns=1. Q2: Support Q3: As the Comment in Q1/Q2, if we support one PFI associates 4 POs in one PF, why it can not support to associate 4 PFs with Ns=1? |
| ZTE, Sanechips | (1) Proposal 2.2-1, the wording “constant” is misleading as it is configurable according to the sub-bullet. (2)Not quite understand the first question in Q2. We think network can always increase Ns value of PO configuration regardless whether how many POs are associated with PEI. The second question in Q2 may relevant to Q3. We think one PEI can be associated with more than four POs in case that POs are densely configured. (3) If multiple PFs are to be supported, we think it can be up to 4 PFs. |
| LGE | Q1: We prefer to fix Ns PO(s) within a PF are mapped to a PEI. It can be expected that Ns>1 is used when gNB need to configure PO densely. Thus, if one PEI to one PO mapping is configured even when Ns>1, network congestion will be increased and PDCCH blocking issue will be a matter. From this point of view, we see that one PEI mapping to multiple POs should be supported when Ns>1 is configured, and reusing Ns to determine association is a natural choice. Q2: We don’t see any issues for Ns. Also, we think one PEI for up to 4 POs in a PF is sufficient. Q3: Our best preference is not to support one PEI for multiple PF. However, if it is supported, we may need some restriction to keep gap between PEI and POs as same as possible. For example, Y=2 contiguous PFs (e.g. SFN 2n and 2n+1) can be map to one PEI. |
| Ericsson1 | Q1: OK, with 1, 2 or 4 POs per PEI. Q2: Yes, and 1 PEI for 1, 2, 4 POs associated with one PF is sufficient. Given there is the note in 38.304 (NOTE 1: A PO associated with a PF may start in the PF or after the PF.), we understand that the POs are associated with a PF may not be located in the same frame and perhaps this point can clarified. Q3: The focus should be on finalizing the design for already-agreed mechanism (One PEI can be configured to indicate up to 4 PO(s) in a PF) before moving on to additional scenarios. Issue related to Q3 could be deprioritized given this is last meeting to finish RAN1 work and in accordance with RAN plenary guidance. |
| Huawei, HiSilicon | Q1: for the PEI mapping to POs. We some kind of share similar view with Qualcomm. We think one PEI should be able to indicate all POs between two adjacent SS bursts. We don’t want the gNB to transmit multiple PEI DCIs for POs in the same SS burst period. This may have two issues. The first one is the resource overhead of PDCCH. The second issue is if multiple PEI occasions need to be introduced, it would be difficult to guarantee the location of PEI detection is close to SS burst. Q2: we think gNB has flexibility to choose a proper Ns, either a large one or a small one. Also, gNB also has flexibility to configure dense PF or sparse PF. Maybe gNB could have some typical configuration scenarios. But we should consider all the scenarios that R15 specifications support. That means our solution should be extendable and future proof. Q3: we are fine to add some restriction as a compromise. If we only consider typical 20ms SSB periodicity, Y=2 may be enough. We didn’t fully understand Qualcomm’s proposal on Y=3. If this would happen in a 20ms SS burst period, we are open on it. |
| vivo | We are fine with Q1. Besides, regarding whether or not to support of 1 PEI for 3 POs, our view is that if the same number of PO is configured to all PEI, 1, 2, or 4 POs in a PF associated with a PEI will be adequate. Q2: Supporting up to 4 POs in a PF is sufficient. Q3: 1 PEI for multiple PFs should not be adopted given the reasons below. - It can be harmful to the reliability of PEI DCI, since the larger number of PFs one PEI can associate, the more paging indication bits may be needed for PEI DCI. - The time gap between different PFs is uncertain. If one PEI can indicate POs in different PFs, it will be unfair for those UEs in the last associated PF, since more power consumption will be caused to keep awake. Besides, due to the time sensitivity of measurement, the more time gap between SSB measurement and PO, the worse detection performance of paging reception may result in. So far serving cell RRM relax is not agreed, so that in every paging cycle, UE need to monitor SSB. Hence, we do not see any problem that in each paging cycle, a PEI is associated to this and followed by the SSB transmission/reception. However, if serving cell RRM relax is introduced, we think mapping 1 PEI to multiple PF is needed in order to reduce UE wake-up activities. |
| DOCOMO | Q1: Same view Xiaomi. We prefer to fixed as Ns. Q2: We think gNB can always increase Ns up to 4 in PO configuration, and I think 1PEI for up to 4 POs in PF is sufficient. Q3: We do not support PEI to multiple PF. @ Moderator: We don’t support of 1 PEI for 3 POs, we think PEI to 1,2,4 should be supported. Thus, could you revise my position? @DoCoMo: We have revised the statistics. Please kindly check. |
| Apple | Q1: the proposal is not clear to us. Is it limited to the case where 1 PEI is mapped to multiple POs within a PF? If this is the case, we support 1, 2 or 4 POs per PEI. Also, what does “constant” mean? Q2: we think one PEI for up to 4 POs in a PF is sufficient. Q3: not support. |
| Lenovo/MotM | Q1:We support a constant number of POs per PEI, where the constant number of POs per PEI is same as the number of POs per PF. Q2: One PEI for up to 4 POs in a PF is sufficient. Q3: We prefer not to support one PEI for multiple PFs. |
| Spreadtrum | Q1: Yes Q2: No. Ns=1 or 2 has its use case. NW will configuration T, N, Ns and Nsg (subgroup number in a PO) according to the statistic information. No constrains on Ns is necessary to provide the full flexibility for NW. Q3: Acceptable to reduce the standardization efforts. |
| OPPO | Q1: We support the proposal 2.2-1. The parameter ‘POnumPerPEI’ should be configured independently. When the sub-group per PO is small, such as 2 or 4, one PEI could map to 16 or 8 POs. The maximum number of POs in a PF is 4, and there is no agreement whether or not to support 1 PEI for multiple PFs. We think the range of ‘POnumPerPEI’ should be determined later. Q2: No. We think gNB should configure Ns flexibility, there is no necessary to limit gNB always have a large Ns. And we support 1 PEI across multiple PFs. Q3: Similar with Panasonic. The number of PF can be decided based on the paging early indication field bit number, ‘POnumPerPEI’’ and subgroupsNumPerPO. |
| TCL | Q1: we support the range of POnumberPerPEI in the value of {1,2,3,4}. In our view, the range of POnumberPerPEI depends on the Ns value i.e. the number of POs per PF. For instance, if the Ns value is 1, the POnumberPerPEI is 1, if the Ns value is 2 the POnumberPerPEI is 2 and if the Ns value is 4 the POnumberPerPEI can be {3 or 4}. Thus we Q2: No, As mentioned by Samsung that “the number of Ns depends on the traffic or available channel resources”. Thus there is no need that a network can always increase Ns value of POs. Furthermore, we support, 1 PEI mapping for up to 4 POs in a PF and across the PF. Q3: In our view, PEI mapping to POs should be based on the maximum bit’s of paging indication field in the PEI DCI. As one PEI can be mapped for up to 4 POs, so PEI mapping should be define based on number of POs in a PF. In case one PO is configured in each PF. Then one PEI can be used to indicate paging for each PO across 4 adjacent PFs. In case 2 or 4 POs are configured in a PF then one PEI can be used to indicate paging for 2 and 1 PF respectively. Thus we believe that the maximum value of Y should be 4, which may reduce the PEI transmission in each PF when one PO is configured in each PF. |
| Transsion Holdings | Q1: Support, ‘POnumPerPEI’ can be{1,2,4} Q2: Ns and the number of POs associated with PEI should be independently configured Q3: Support one PEI for multiple PFs.As SSB has multiple period, Y should be flexible configured. |
| Nokia(1st round) | Q1: As commented above and in Proposal 2.3-2 it is discussed to have configurable DCI format size, (which is evidently needed for the UE to be able to detect the DCI). In addition to the (configurable) DCI size, UE needs to know the starting location of the sub-grouping field corresponding to the PO (and length of the sub-grouping field) and location of any other relevant fields (if agreed). The total number of POs mapped the PEI DCI format (e.g. as GC-PDCCH) does not matter from UE perspective. Hence, in principle we do not need a parameter to tell the UE what is the number of POs in the PEI (or the size of the paging related information field), but we need to tell the starting location of the PO sub-grouping field. Depending on other design decisions (e.g. SI change indication and ETWS indication support) the starting location of the UE specific field could of course be (integer) function of sub-grouping field length plus offset (if the other fields would be added to the start). Q2/Q3: As discussed the SSB periodicity is configurable (5ms ..160ms) as is paging cycle and number of paging frames. Hence, in some configurations it could be desirable, in order to ensure preferable PEI-PO relation to be able to map POs of different PFs to the PEI. This can be somewhat seen as a different question than the total number of POs that can be mapped to PEI. Also as discussed earlier, if we agree the maximum DCI format 2_7 to be (for example) 32, from (individual) UE perspective actual number of POs that are carried by the PEI is somewhat irrelevant. Acknowledging the fact that we have agreed to support max 4 POs, we would accept that limitation, but would like to consider not restricting the 4 POs to single PF. |
| IDCC | We do not support PEI to multiple PFs. We think up to 4 POs in the same PF is sufficient. |

For Q1, quoted below for ease of reference,

Proposal 2.2-1: Support mapping a constant number of PO(s) per PEI. - Include a configurable parameter, ‘POnumPerPEI’, with possible value in the range {1, 2, ….}

- Yes (14 companies): Xiaomi (Ns), MTK ({1, 2, 4}), Nordic, Samsung (>1 and not restricted ≤Ns), Panasonic, Intel ({1, 2, 4}), CMCC, ZTE (“constant”  “configurable”), LGE (Ns), Ericsson ({1, 2, 4}), Sharp ({1, 2, 4, 8, 16}), vivo, DoCoMo (Ns), Lenovo/MotM (Ns)

- No (3 companies): CATT, QC (#POs between SS bursts), HW & HiSi (#POs between SS bursts)

For Q2, quoted below for ease of reference,

Q2: To utilize one PEI for multiple POs, do you think network can always increase Ns value of PO configuration? If yes, do you think whether one PEI for up to 4 POs in a PF is sufficient or not?

- Yes (11 companies): Xiaomi, MTK, Nordic, Intel, LGE, Ericsson, Sharp, vivo, DoCoMo, Apple, Lenovo/MotM

- Yes for Ns; but prefer up multiple PFs (3 companies): CMCC, ZTE, QC

- No (4 companies): Samsung, Panasonic, CATT, HW & HiSi

For Q3, quoted below for ease of reference,

Proposal 2.2-2: Support mapping 1 PEI to up to Y PFs in adjacent radio frames - Y = 2

- Yes (7 companies): Xiaomi (Y = 4), CMCC (Y = 4), ZTE (Y = 4), LGE (2nd preference with Y = 2), QC (implicit Y), Sharp, (Y depends on PEI’s capacity), HW & HiSi (Y = 2)

- No:

  - Multiple PFs w/o constraint (3 companies): Samsung, Panasonic, CATT

  - No multiple PF (9 companies): MTK, Nordic, Intel, LGE (1st preference), Ericsson (specify 1 PEI for 1 PF as essential functionality), vivo, DoCoMo, Apple, Lenovo/MotM

For companies’ inputs, we have the following observations:

- There is currently no consensus on supporting multiple PFs.

  - Moderator comment:  As per Ericsson’s suggestion, “The focus should be on finalizing the design for already-agreed mechanism (One PEI can be configured to indicate up to 4 PO(s) in a PF) before moving on to additional scenarios”, moderator would also like to suggest proceeding the specification based on what agreed in RAN1 #106-bis-e, instead of pending on multiple PF discussion. The related specification for multiple PF can additional included if there is consensus.

- 14 out of 18 companies think network can adjust Ns configuration so as to exploit PEI indication capacity. But 4 companies think we should not change network PO configuration.

  - Moderator comment: From current PO configurable framework in TS 38.304 and TS 38.331, network can keep the same PO locations while changing to Ns. Please refer to the following illustration that can help clarify the equivalency. The key is the flexible configuration by the IE ‘firstPDCCH-MonitoringOccasionOfPO’. We would like to suggest companies’ check and hope this can clarify why supporting 1 PEI for 1 PF can be sufficient.

    - @CATT: We check R15 TS 38.331 and Ns = 4 is supported since R15

- There are 14 out of 17 companies support a configurable ‘POnumPerPEI’. But 3 companies think we should allow that different PEI can indicate different number of POs.

  - Moderator comment: To address companies’ demand in supporting “different PEI can indicate different number of POs”, it will be useful if CATT/QC/HW&HiSi can provide use case(s) and illustration(s) where “each PEI indicates the same number of POs” cannot work. Since this is the last meeting, a working function will be our priority to complete R17 PEI. By the above, we would like to check companies’ views for the following two questions:

Q4: Do you see any use case(s) or PO setting(s) where “each PEI indicates the same number of POs” cannot function? If yes, it is appreciated if you can also provide the specific configuration/illustration.

Q5: What is your views/suggested revision for Proposal 2.2-1 (r1)?

Proposal 2.2-1 (r1):

Support mapping the same number of PO(s) per PEI.

- Include a configurable parameter, ‘POnumPerPEI’, with possible value in the range {1, 2, 4}

Table 3: Companies’ views to the above question

| Company | Companies’ views |
| --- | --- |

## Detailed DCI Content Design

In the following table, companies’ related views and proposals are collected:

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon | Figure 5. DCI format for PEI transmission. Proposal 4: Paging indication field, TRS availability indication field and SI change/ETWS field are supported by the new DCI format of PEI DCI, i.e. DCI format 2_7, where: - Paging indication field is mandatory if the DCI format is configured in the SIB; - TRS availability indication field are optionally configurable and the bit length follows the agreement in potential TRS occasion, which could be configurable or fixed to 1; - Two bits of SI update and ETWS change notification can be optionally configured in the DCI format; Proposal 5: The DCI format size of PEI DCI is smaller than that of DCI format 1_0, which is: - Atl.1: Explicitly configured by DCI format 2_7; - Alt.2: implicitly determined by the sum of configured fields; Figure 6. Example of 4PFs in one SS burst periodicity and 2POs sharing the same bit in paging indication field in PEI-DCI Proposal 8: For paging indication field of x bits in PEI DCI, the associated sub-group indication bit index of the j-th sub-group in the i-th associated PO is calculated by (i-1)*M+(j-1) mod x, where: - M is configured by subgroupsNumPerPO; - N is the number of total POs in each SS burst periodicity, which is figured out by existing paging configuration and SSB configuration; - x is the length of paging indication field. Proposal 9: For the field length of paging indication field, narrow down from the following alternatives: - Alt.1: the field length of paging indication field is calculated by x=min (M*N, X); - Alt.2: the field length of paging indication field is explicitly configured by higher layer parameter, which is not larger than X; , where: - M is the number of sub-groups per PO, which is configured as a per cell parameter, subgroupsNumPerPO; - N is the number of total POs in each SS burst periodicity, which is figured out by existing paging configuration and SSB configuration. - X is the maximum length of the field to guarantee the reliability of PEI DCI detection, which is specified as 32 or 16 based on the decision by the group. Proposal 10: Use the same principle in paging DCI to indicate availability information for assistance TRS in PEI DCI: - The start location of TRS availability indication field is determined by the length of paging indication field; - The length of TRS availability indication field could be configurable or fixed to 1, based on the conclusion in assistance TRS occasion. |
| ZTE, Sanechips | - Proposal 2: The TRS availability indication conveyed by PEI should be supported. - Proposal 3: The SI change and ETWS information carried by PEI should be considered. Figure 1 Bit structure of PDCCH-based PEI Proposal 4: A bit structure of PEI in Figure 1 can be supported. |
| vivo | Proposal 3: In a PEI DCI, a UE can obtain its paging indication block based on the the associated PO index in a PF and then figure out its corresponding bit according to the index of subgroup notified by the network. Figure 5: An example for the overlapping between multiple PEI-Os in a PEI-frame. Proposal 10: It is necessary for PEI DCI to notify the indicated PO index(es) to avoid the ambiguity e.g., adding 0-2bits PO index(es) indication in PEI DCI. - TRS availability indication This should be decided in AI 8.7.1.2 and then we can just follow the agreements on TRS availibility indication endorsed in RAN plenary #93 [6], which refers to that if TRS availability indication is agreed to be supported in both paging DCI and the DCI format for PEI, same mechanism/principle for TRS availability indication is adopted for the two DCI formats. Proposal 11: SI update indication delivered in PEI DCI should be precluded. |
| TCL | Proposal 1: Consider the following fields in the PEI DCI format with the maximum number of bits: - Subgroup paging Indication field: 32 bits - TRS Availability Indication field: 6 bits - SI change and ETWS notification field: 2 bits |
| Spreadtrum |  |
| CATT | Proposal 6: Short messages are not supported by PDCCH-based PEI. Proposal 13: If one PEI indicates N1 POs and the number of sub-groups per PO is G, the new DCI format of PEI contains N1 groups of bit fields and each bit field of a paging group contains G bits. Proposal 14: A UE determines the corresponding bit location in PDCCH-based PEI based on PO_Index, G and its sub-group number, where PO_Index is the index of PO within multiple POs, and G is the number of sub-groups per PO. Figure 7: The bit fields design of PDCCH-based PEI Proposal 18: TRS availability indication should not be carried in PDCCH-based PEI. |
| OPPO |  |
| Sony |  |
| Intel | Proposal 1: Essential function of PEI includes providing indication for UE paging sub-grouping information only. Proposal 6: DCI payload of PEI can be configurable. Proposal 11: UE may follow TRS availability indication (if agreed) by PEI regardless of whether UE is indicated to monitor PO or not by the same PEI. |
| Xiaomi | Proposal 4: Support PEI-DCI has the same DCI payload as paging DCI, typically 40 bits, and has different RNTI from P-RNTI Proposal 7: PEI will alert the UE to wake up only when paging message is expected. |
| CMCC | Proposal 5. When UE subgrouping is not configured, for one UE which PO’s index is i_s, - If 1 PEI indicating Ns POs in one PF, the i_sth bit in PEI is used to indicate wake up information; - If 1 PEI indicating K*Ns POs in K PFs, the [(SFN*N/T mod K) *Ns+ i_s]th bit in PEI is used to indicate wake up information, where SFN is the UE’s PF frame. Proposal 6. When UE subgrouping is configured, for one UE which PO’s index is i_s and subgroup index is m, where m = 0, 1, … M-1, M is the total number of subgroups in one PO, - If 1 PEI indicating Ns POs in one PF, the [i_s*M+m]th bit in PEI is used to indicate wake up information; - If 1 PEI indicating K*Ns POs in K PFs, the [(SFN*N/T mod K) *Ns+ i_s]*M+m th bit in PEI is used to indicate wake up information, where SFN is the UE’s PF frame. |
| Transsion Holdings |  |
| Panasonic | Proposal 1: In the DCI format for PEI, the early indication of multiple POs within a PF and across multiple PF(s) are both supported. The number of POs are configurable by SIB by granularity of PF(s). Proposal 2: Sub-grouping information and the used bit width are configurable in SIB. Proposal 3: For the DCI format defined for PEI, one or more blocks can be configured for subsequent PO(s) and the TRS availability/unavailability indication. In addition, an upper limit of the DCI format size should be defined. |
| Samsung | Proposal 1: Support PEI with UE subgrouping as the only function for paging enhancements in NR Rel-17. Proposal 5: The bit mapping for PEI can be determined based on UE subgroup index, iPO, and order of the target PO, iPO, such that the (iPO∙NUE subgroupsPO+isub_group+1)th bit in the DCI payload maps to (isub_group+1)th UE subgroup from the (iPO +1)th PO indicated by the PEI, where - iPO = 0, 1, …, number-of-PO -1, - NUE subgroupsPO = 0,1, …, subgroupsNumPerPO -1 |
| Apple | Proposal 1: The size of the PEI DCI is configurable by the gNB, and it should be no larger than the size of paging DCI. Proposal 2: The new DCI format for PEI can be configured to carry SI update and ETWS indication. Proposal 3: The new DCI format for PEI can be configured to carry TRS availability indication. |
| Lenovo, Motorola Mobility |  |
| InterDigital |  |
| LGE | Observation 1: TRS/CSI-RS availability indication via PEI is beneficial in terms of UE power saving and reducing NW overhead. Observation 2: Indicating availability for the TRS/CSI-RS occasion(s) considering beam selectivity manner can reduce the DCI overhead of the PEI. Observation 3: Once the SI change indication is transmitted, repetitions of SI change indication may occur within preceding modification period. Observation 4: Conveying information with regard to SI change indication and/or ETWS/CMAS notification over PEI is beneficial from power saving perspective. Observation 5: Compared to the Alt 1-a, conveying information with regard to SI change indication and/or ETWS/CMAS notification over PEI does not increase the NW overhead. Proposal 1: PEI conveys the following information in addition to UE group/subgroup indication. - TRS/CSI-RS availability indication - Information with regard to SI change indication and ETWS/CMAS notification - If supported, indication of stop PEI Monitoring Proposal 2: The DCI for the PEI contains the bitmap for the UE group and UE sub-group indication. - The number of UE sub-groups for a PO (i.e. NSG) is configured via higher layer - The number of UE groups associated with a PEI is NS, where the NS is a paging parameter that is for determining a number of POs in a PF. - The size of the DCI field for the UE group and UE sub-group indication is NSG*NS. Figure 4 Table 1 Contents; The number of DCI bits; Example (12 bits). UE group and subgroup indication; NSG(# of sub-group for a PO) * NS (# of POs for a PF); 8 bits. TRS/CSI-RS availability indication; [0 or 1 or 2]; 1 bits. SI change indication; 1; 1 bit. ETWS/CMAS notification; 1; 1 bit. Stop PEI monitoring; 0 or 1; 1 bit. Reserved bits; Configurable; 0. Proposal 3: The DCI for the PEI contains - [1 or 2] bits for the TRS/CSI-RS availability indication, if configured. - 2 bits for indicating SI change indication and ETWS/CMAS notification. - 1 bit for stop PEI monitoring indication, if configured. - X bits for reserved bits field, where X is configured via higher layer. Observation 6: The size of the DCI format for the PEI can be aligned with the DCI format 0_0/1_0 when zero bit padding is applied. Observation 7: When the UE only monitors a PEI at a PEI occasion, UE may assume that bits in the padding bit field are frozen bits for PDCCH decoding. Proposal 5: Zero bit padding for the DCI size alignment is applied to the DCI format for the PEI. |
| NTT DOCOMO | Figure1. A solution of how to define the bit fields corresponding to essential function Subgroup Info #1 of PO#1; 1 bit; Information for PO#1. :; :. Subgroup Info # X of PO#1 (※Note1); 1 bit. :; :; :. Subgroup Info #1 of PO# Y; 1 bit; Information for PO#X (※Note2). :; :. Subgroup Info # X of PO# Y; 1 bit. TRS availability indication; Z bit (※Note3); Information for all associated POs. Short Message; 2 bit; Information for all associated POs. Note1: X is the number of subgroups in one PO. Note2: Y is the number of the associated POs. Note3: Z is up to decision in AI 8.7.1.2.. Proposed 3: DCI size for PEI should be implicitly derived from number of POs, number of subgroups, whether or not TRS availability indication and Short Message is included in PEI. Proposed 5: UE should be informed, via SIB, of whether or not TRS availability indication and Short Message are included in PEI. |
| Ericsson | - Observation 3 In deployments where subgrouping is not used, no bits shall be wasted on subgrouping information. - Proposal 2 When subgrouping is not configured in the cell, but PEI is configured, PEI presence invokes all UEs at the addressed PO. - Proposal 3 PEI shall support addressing 2…8 subgroups. The same number of subgroups is applicable to all POs of the cell. - Proposal 4 The number of bits assigned to subgrouping is controlled via the RRC configuration parameter subgroupsNumPerPO with range 1...8, where value 1 means that no subgrouping is configured. - Proposal 5 PEI supports individual addressing of up 8 subgroups belonging in up to 4 consecutive POs associated with the same paging frame. - Proposal 8 PEI DCI payload size is configured through a higher layer broadcast configuration parameter, including a maximum value 41 bits. - Observation 4 Use of reserved bits in paging DCI (as a PDCCH-PEI) in one PO as paging early indication for UEs in one or more groups in other POs can further reduce PEI signalling overhead. - Proposal 10 PEI design should allow the use of reserved bits in paging DCI in one PO as paging early indication for UEs in one or more groups in other POs. - Proposal 11 PEI design should allow potential future size extension. - Proposal 12 TRS presence is configured in PEI. - Proposal 13 The locations of the following information elements within the PEI DCI is configurable via higher layer broadcast configuration: - Subgroups per PO - TRS availability - Proposal 17 ETWS/CMAS/SI updates indication is not included in PEI. In case of SI update or public warning transmission, the NW invokes all UEs so that they find the relevant information in corresponding POs. |
| Qualcomm |  |
| MediaTek | Proposal 4: For the case 1 PEI indicates multiple POs in a PF, - There are POnumPerPEI segment of subgroupsNumPerPO bit(s) in the paging indication field - Given UE’s PO index in the PF is i, 0≤i≤Ns-1, UE’s paging indication lies in the ((i mod PONumPerPEI) + 1)th segment of the paging indication field |
| Nokia | Proposal: The PEI DCI format configuration should provide for each PO the location of the subgrouping field. Subgrouping field and DCI size could be configured in cell specific manner. Proposal: To enable providing additional information, i.e. ‘systemInfoModification’, ‘etwsAndCmasIndication’ and L1 availability indication in PEI, the PEI DCI format configuration should provide the location (and size) for each information field. The location of these fields, if configured, could be at the start of the DCI format so that it can be common for all PEIs/POs. Proposal: Consider further whether PEI could be configured to carry ‘systemInfoModification’ indication bit and/or ‘etwsAndCmasIndication’ indication bit. Inform RAN2 of the RAN1 decision and assumed UE behaviour. Observation: The DCI size for PDCCH-based PEI should be configurable, from 1 to [35] bits. Table 1 Energy consumption saving loss for L1 availability indication in paging DCI as compared to the availability indication being included in the PEI. All numbers in %. [6] L1 availability indication probability; Low SINR; Medium SINR; High SINR. 20 %; -11.4; -7.6; -4.9. 40 %; -22.7; -15.2; -9.8. 60 %; -34.1; -22.8; -14.6. Proposal: Support configuring L1 availability indication in PEI. |
| Nordic | Proposal-1: Support only the cases where PEI is mapped to 1,2 or 4 POs of a single PF in R17. Max size of DCI format payload is 34 bits. - Up to RAN2 whether to reuse P-RNTI or introduce PEI-RNTI |

From the above table, it is observed companies all support simple “bitmap” structure for multiple subgroups and multiple POs. In this regard, moderator would like to check the following two questions:

Q1: What is your view and suggested revision on Proposal 2.3-1

Proposal 2.3-1:

When UE is provided a PO index, iPO, 0≤iPO<POnumPerPEI, among the indicated POs by the target PEI and a subgroup index, iSG, 0≤iSG<subgroupsNumPerPO, UE is required to monitor its PO if  iPO×subgroupsNumPerPO+iSG+1-th bit of paging indication field is set to ‘1’.

Q2: Do you consider subgroupsNumPerPO×POnumPerPEI> maximum paging indication field size, [32]? If yes, do you support circular mapping as proposed by HW & HiSi, as illustrated below:

Regarding DCI payload size, the following statistics can be obtained:

DCI format 2_7 payload size:

- Configurable by network (6 companies): HW & HiSi, Intel, Panasonic, Apple, Ericsson, Nokia,

- Implicitly determined by the sum of configured fields (6 companies): HW & HiSi, ZTE, vivo, TCL, CATT, DoCoMo, Nordic

- Fixed (1 company): LGE (zero-padding to paging DCI payload size)

It is observed that proponents on configurable DCI payload size considered the coexistence with legacy PDCCH. For example, if the DCI format 2_7 payload size is configured to be the same as that of paging DCI, UE blind decoding complexity is reduced if both DCI formats share the same CORESET occasion. Additionally, having reserved bits can also be beneficial for future extension. By the above, moderator would like to invite companies’ views on the following question:

Q3: Please provides your views and/or suggested revision on the following Proposal 2.3-2

Proposal 2.3-2:

Payload size of DCI format 2_7 is configurable in SIB

- No larger than paging DCI size

- The undefined bit(s) is reserved bit(s)

For TRS availability indication, moderator would like to suggest it is up to TRS agenda decision.

For the support of short message, the following statistics can be obtained:

Support of short message (SI change and ETWS):

- Yes (7 companies): HW & HiSi (2 bits), ZTE, TCL (2 bits), Apple, LGE, DoCoMo (2 bits), Nokia (field at start of DCI format)

- No (4 companies): vivo, CATT, Ericsson, Nordic

Proponent of “Support” think it is beneficial to avoid unnecessary wake-up in the preceding modification period. Also the overhead can be shared across the POs indicated by the same PO and 2 bits per PEI look sufficient. In this regard, moderator would like to check companies’ views on including a 2-bit field for SI change and ETWS. For network not desiring to support the feature, we can consider the 2-bit filed configurable.

Q4: Please provides your views and/or suggested revision on the following Proposal 2.3-3

Proposal 2.3-3:

A 2-bit field, one bit for SI change indication and one bit for ETWS indication, can be configured in DCI format 2_7

Table 4: Companies’ views to the above questions

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Q1: Support Proposal 2.3-1. Q2: We are not sure about the need and benefit to support such case. Q3: Support Proposal 2.3-2. |
| MediaTek | For Q1: We are supportive of Proposal 2.3-1. It is the simple ‘bitmap’ design for given parameters of iPO, POnumPerPEI, iSG and subgroupsNumPerPO. If there only supports 1 PEI for 1/2/4 POs in a PF, we also have iPO=is mod POnumPerPEI, where is is the PO index derived from PO determination as per TS 38.304. In this regard, we suggest to include an sub-bullet for specifying Proposal 2.3-1 (Updated) When UE is provided a PO index, iPO, 0≤iPO<POnumPerPEI, among the indicated POs by the target PEI and a subgroup index, iSG, 0≤iSG<subgroupsNumPerPO, UE is required to monitor its PO if iPO×subgroupsNumPerPO+iSG+1-th bit of paging indication field is set to ‘1’. - If 1 PEI is mapped to 1/2/4 POs in a PF, iPO=is mod POnumPerPEI, where is is the PO index derived from PO determination as per TS 38.304 For Q2: Since we support 1 PEI for up to 4 POs in a PF, there will be no such condition with maximum paging indication field size of 32 bits. In this regard, we do not consider this case. For Q3: We are fine with the proposal. Intentionally setting an aligned DCI format size with paging DCI can indeed help reducing UE blind decoding complexity, but we are not keen to such setting since reusing the control resource of PO may not optimize the UE power saving gain. On the other hand, we see the configurable payload size and reserved bits allows future extension of PEI functionality, which is more attractive for us. For Q4: We are supportive of Proposal 2.3-3 since 2-bit indication is a small overhead, and it is useful for UE to avoid dummy PO monitoring what the SI update or ETWS information is acquired but the modification period doesn’t expire. |
| Nordic | Q1: Support Q2: We do not support mapping of PEI to multiple PFs Q3: Proposal should be Payload size of DCI format 2_7 is configurable in SIB - Max size is [X] - The undefined bit(s) is reserved bit(s) Q4: Do not support, only Subgroups and Idle TRS validation |
| Samsung | Q1: We are OK with P2.3-1 Q2: No. We think bitmap based indication is sufficient. Q3: The explicit configuration of the payload size is not needed. It depends on the paging indication field size, which is subgroupsNumPerPO×POnumPerPEI if paging indication field is the only functionality supported. Even if TRS availability indication is supported, we should consider the case only paging indication field is configured. We should prioritize the essential function in the last meeting, while TRS availability indication is not essential. So, we suggest the following revision. Payload size of DCI format 2_7 is subgroupsNumPerPO×POnumPerPEI if only paging indication field is configured configurable in SIB - No larger than paging DCI size - The undefined bit(s) is reserved bit(s) Q4: We cannot accept it. We are not convinced by the gain considering the SI or ETWS indication has a much lower transmission rate than UE paging rate. It’s redundant functionality, which has been supported in paging PDCCH. The new DCI format should only indicate UE to wake up for paging PDCCH reception. |
| Panasonic | On Q1, Proposal 2.3-1 is okay with us. On Q2, it is good to clarify the merit on using circular mapping, as it looks like same effect by adjusting the subgroupsNumPerPO. On Q3, Proposal 2.3-2 is okay with us. On Q4, we are open to discuss proposal 2.3-3 if time allows. |
| CATT | Q1: We are OK with the principle of mapping Q2: We don’t support circular mapping, which have additional complexity without any benefit. Q3 The size of DCI format 2_7 should be implicitly derived from the number of POs per PEI and the number of paging subgroup per PO without specific higher layer signaling. The DCI formats 2_7 to the paging DCI would be distinguished by new PEI-RNTI. Q4: We have strong concern on the proposal since this is not an essential function for PEI, which is the requirements agreed in RAN#93. We can NOT agree on this proposal. |
| Intel | Q1: Support Q2: Do not support Q3: Support main bullet , without the sub-bullets. We have concern on PEI DCI having similar size as paging DCI Q4: SI update, ETWS indication are not essential functions to be indicated by PEI. Do not support |
| Qualcomm | Q1: we support proposal 2.3-1 Q2: we do not think network should configure the number of subgroups across POs indicated by the PEI larger than the maximum number of DCI size of PEI. Network can simply reduce the number of subgroups per PO to avoid this condition. Q3: we do not think network needs to configure the number of used bits within the DCI. Network only needs to configure a DCI size, i.e., maximum number of bits that can be used. UE can derive the actual number of used bits used for paging indication. Q4: do not support. Network can page the UE to receive SIB update and ETWS from the paging PDCCH |
| Sharp | Q1: ok Q2: the benefit is not clear, we don't support it Q3: ok with the proposal Q4: we are open to support it |
| CMCC | Q1: We think this proposal is only workable under the condition of one PEI associating with one PF, the bitmapping of one PEI with multiple PFs need FFS Q2: Don’t support Q3: Fine Q4: Don’t think it is essential to include these 2 bits in PEI. |
| ZTE, Sanechips | (1)We are okay with proposal2.3-1; (2)We don't think we need to consider the case in Q2. Network can flexibly adjust the paging information field according to the number of POs /sub-groups. (3)we are okay to restrict that the PEI payload size should be no larger than paging DCI considering PEI detection performance should be better than paging DCI. However, the second sub-bullet is unclear to us with regard to what it the “undefined bits” (4)Proposal 2.3-3: support. Indication of SI change and ETWS can help UE to skip PO detection to the most degree, which provides the best power saving gain. |
| LGE | Q1: We are fine with proposal 2.3-1 Q2: Sharing indication bit for a multiple UE group is not preferred since it is not aligned with the benefit from the UE subgroup indication. Also, as we commented in a previous section, we support one PEI mapping up to Ns PO(s) within a PF. Q3: Although our view is supporting DCI size alignment for PEI DCI, if we are the only company who support it, we can compromise to move forward. Regarding the proposal 2.3-2, we are fine with the main bullet and the second sub-bullet. As usual DCI design, reserve bits are useful from forward compatibility point of view, and configurable size would be beneficial to allow gNB handle the DCI payload by eliminating unnecessary bits. For the first sub-bullet, these restrictions may not be necessary if a new RNTI is used for PEI. Q4: We support the proposal 2.3-3. By indicating SI change and PWS notification via PEI, UE can avoid unnecessary power consumption by PO monitoring. Also, it should be noted that repeating SI change indication during the modification period can be expected to guarantee UEs acquire updated system information. |
| Ericsson1 | Q1: In principle OK, but may need update – UE is not provided with any PO index (or at least there is no such higher layer parameter). Also given the note in 38.304 (A PO associated with a PF may start in the PF or after the PF.), perhaps using associated might be better than “in”. Suggested updates based on MTK version below. Proposal 2.3-1 (Updated)-E// When UE is provided a PO index, iPO, 0≤iPO<POnumPerPEI, among the indicated POs by the target PEI and a subgroup index, iSG, 0≤iSG<subgroupsNumPerPO, UE is required to monitor its PO if iPO×subgroupsNumPerPO+iSG+1-th bit of paging indication field is set to ‘1’. - If 1 PEI is mapped to 1/2/4 POs in associated with a PF, iPO=is mod POnumPerPEI, where is is the PO index derived from PO determination as per TS 38.304 Q2: No – we are not convinced of the need to support more than 32 paging occasions using single PEI. Q3: Support, including maximum DCI size = 41 bits. As we show in our contribution, we see no issue with allowing maximum 41-bit payload for PEI – actual configuration details can be left up to NW implementation. Q4: Do not support. |
| Huawei, HiSilicon | Q1: We are in generally fine for the sequential mapping by firstly sub-group ID and then PO index. However, we think some condition should be added to consider how to handle the case when subgroupsNumPerPO×POnumPerPEI> maximum paging indication field size in Q2. Maybe we could revise it as: Proposal 2.3-1: When UE is provided a PO index, iPO, 0≤iPO<POnumPerPEI, among the indicated POs by the target PEI and a subgroup index, iSG, 0≤iSG<subgroupsNumPerPO, UE is required to monitor its PO if iPO×subgroupsNumPerPO+iSG+1-th bit of paging indication field is set to ‘1’, if subgroupsNumPerPO×POnumPerPEI≤ maximum paging indication field size . We think it would be better to discuss Q1 and Q2 together. Q2: After reading comments from companies, we think we should explain the benefit/merit of circular mapping. 20ms SS burst period is a typical case. However, gNB has the requirement for gNB side energy saving, which may prolong the SS burs period and broadcast it in SIB, e.g. 40ms, 80ms and 160ms. In this scenario, more POs/PFs may exist in one SS burst period. For example, for 160ms SS burst period, there could be up to 16*4 POs already. This would be already larger than the size of paging DCI. Some companies may argue that multiple PEI DCIs could be transmitted. However, as I replied for Q1 in section 2.2. Multiple PEI DCIs transmission is not acceptable from gNB perspective. Q3: we are fine with it. However, to avoid any confusion in future, maybe “explicitly” should be added before “configurable”. Q4: we support the proposal. Maybe we could add “optionally” before “configured” to make clear that network may or may not configure it based on gNB’s preference. |
| vivo | Q1: We are supportive. Q2: We do not support "subgroupsNumPerPO"×"POnumPerPEI"> maximum paging indication field size, since this can be avoided by the proper network configuration. As such, no need to design circular mapping. Q3: We are partially acceptable for Proposal 2.3-2. The total payload size of DCI format 2_7 can be configurable in SIB. However, the two subbullets should be deleted. And the paging DCI size should be addressed after the decision on the maximum payload for paging indication field. Q4: Not support. As per the agreement endorsed in RAN plenary #93 [6], SI update indication and ETWS indication delivered in PEI should be precluded since they are not an essential function for PEI. |
| DOCOMO | Q1. We are generally fine with the proposal 2.3-1. Q2. We do not consider this case. Q3: We are open to discuss Proposal 2.3-2. Q4: If we agree with Proposal 2.3-2, it means there may be some reserved bits depending on configuration. Short message is only 2-bit indication, thus we think there are cases where it is not burdensome to send this information via PEI. So, we support the proposal 2.3-3. |
| Apple | Q1: Fine with the principle. But what does “When UE is provided a PO index” mean? How does the UE determine the PO index? We think this should be implicitly derived. Q2: No, we do not think it is necessary. Q3: support Q4: support |
| Lenovo/MotM | Q1: Ok with proposal 2.3-1, Q2: Do not support, Q3:Ok with proposal 2.3-2, Q4: Ok with proposal 2.3-3 |
| Spreadtrum | Pending after decision of supporting paging search space for PEI |
| OPPO | If the payload size of PEI can be used to distinguish PEI and paging PDCCH, P-RNTI can be reused. |
| TCL | We support only PEI RNTI. In our view, PEI is a new feature of Rel-17, and using the existing paging RNTI will leads the legacy UE to assume the PEI DCI as paging PDCCH. Therefore, PEI RNTI should be used for PEI. |
| Transsion Holdings | Q1: Support |
| Nokia(1st round) | It is not fully clear if we would have selection (via configuration) two fixed RNTIs (i.e. P-RNTI and PEI-RNTI) or whether the PEI-RNTI would have full flexibility to be configured? In general, we don’t have very strong view, but if we introduce separate RNTI for PEI, it would be probably simples to use it always. I.e. there seem not be any extra benefit to support P-RNTI if PEI-RNTI is introduced. |
| IDCC | We are ok with this proposal. |
| Spreadtrum | Pending after decision of supporting paging search space for PEI |

For Q1, quoted below for ease of reference,

Proposal 2.3-1: When UE is provided a PO index, iPO, 0≤iPO<POnumPerPEI, among the indicated POs by the target PEI and a subgroup index, iSG, 0≤iSG<subgroupsNumPerPO, UE is required to monitor its PO if iPO×subgroupsNumPerPO+iSG+1-th bit of paging indication field is set to ‘1’.

- Yes (18 companies): Xiaomi, MTK, Nordic, Samsung, Panasonic, CATT, Intel, QC, Sharp, CMCC (FFS multiple PF case), ZTE, LGE, Ericsson (with revision), HW & HiSi (with revision), vivo, DoCoMo, Apple (implicit PO index), Lenovo/MotM

- No: N/A

For Q2, quoted below for ease of reference,

Q2: Do you consider subgroupsNumPerPO×POnumPerPEI> maximum paging indication field size, [32]? If yes, do you support circular mapping as proposed by HW & HiSi, as illustrated below:

- Yes (1 companies): HW & HiSi (use case: Ext. SSB period for NW eng. saving)

- No (17 companies): Xiaomi, MTK, Nordic, Samsung, Panasonic (merit not clear), CATT, Intel, QC, Sharp (merit not clear), CMCC, ZTE, LGE, Ericsson, vivo, DoCoMo, Apple, Lenovo/MotM

For Q3, quoted below for ease of reference,

Proposal 2.3-2: Payload size of DCI format 2_7 is configurable in SIB - No larger than paging DCI size - The undefined bit(s) is reserved bit(s)

- Yes (explicit configuration; 15 companies): Xiaomi, MTK, Nordic (with maximum payload specified), Panasonic, Intel (w/o sub-bullet), Sharp, CMCC, ZTE (“undefined bits” unclear), LGE (no 1st sub-bullet), Ericsson, HW & HiSi (“explicitly” configurable), vivo (w/o sub-bullets), DoCoMo, Apple, Lenovo/MotM

- No (implicit info; 3 companies): Samsung, CATT, QC

For Q4, quoted below for ease of reference,

Proposal 2.3-3: A 2-bit field, one bit for SI change indication and one bit for ETWS indication, can be configured in DCI format 2_7

- Yes (9 companies): MTK, Panasonic (if time allows), Sharp, ZTE, LGE, HW & HiSi (“optionally” configured), DoCoMo, Apple, Lenovo/MotM

- No (8 companies): Nordic, Samsung, CATT, Intel, QC, CMCC, Ericsson, vivo,

By the above, the following observations can be obtained:

- There is no consensus for supporting short message in PEI for Rel-17

  - Moderator comment: Since the network can still wake-up all UEs in each paging cycle of modification period, there is no impact to SI update and ETWS although the UE power saving is not optimized. In this regard, moderator would like to support not perusing SI update and ETWS indications in PEI for Rel-17.

- There is no consensus for supporting “circular bit mapping” for paging indication field in PEI for Rel-17

  - Moderator comment: The use case of “circular bit mapping” looks related to Rel-18 topic of network energy saving. In this regard, moderator would like to suggest pursuing this extension in a future release.

- There is consensus on Proposal 2.3-1

  - Moderator comment: We will bring Ericsson revised version to email discussion for consolidating the proposal for online decision. On the other hand, since there is no consensus to define an explicit maximum paging indication field size, moderator would suggest not to include the revision from HW & HiSi in the initial version for email discussion.

- There are 15 out of 18 companies support explicitly configured payload size for DCI format 2_7. But, 3 companies think the size should be implicit information for the field configuration.

  - Moderator comment: (To be updated)

## RNTI for PEI DCI format

In the following table, companies’ related views and proposals are collected:

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon | Proposal 3: A new RNTI, PEI-RNTI, is configured to scramble the CRC of PEI-DCI for idle/inactive mode UEs. |
| ZTE, Sanechips | - Proposal 8: P-RNTI is supported for PEI. |
| vivo | Proposal 4: A new PEI-RNTI is needed for PEI DCI. |
| TCL | Proposal 2: Support a new PEI-RNTI for PEI DCI to avoid any conflict with PDCCH channel and legacy UEs. |
| Spreadtrum |  |
| CATT | Proposal 16: A new RNTI is used to scramble the CRC of PDCCH-based PEI. |
| OPPO | Proposal 3: Whether or not to introduce a new PEI-RNTI should be decided according to the value of x. - If x can be used for UE to distinguish PEI and paging PDCCH, P-RNTI can be reused. |
| Sony |  |
| Intel | Proposal 9: P-RNTI can be used for PEI DCI. |
| Xiaomi | Proposal 4: Support PEI-DCI has the same DCI payload as paging DCI, typically 40 bits, and has different RNTI from P-RNTI |
| CMCC |  |
| Transsion Holdings | Proposal 3: P-RNTI should be supported. |
| Panasonic | Proposal 4: Define a new semi-statically configurable RNTI for Rel.17 PEI. |
| Samsung | Proposal 3: reuse P-RNTI for the DCI format dedicated to PEI. |
| Apple |  |
| Lenovo, Motorola Mobility | Proposal 4: RNTI for a PEI PDCCH is determined based on a PF that is associated with the PEI PDCCH. |
| InterDigital |  |
| LGE | Proposal 4: PEI-RNTI that is used for CRC scrambling for the PEI is configured via higher layer. |
| NTT DOCOMO | Proposed 4: Reuse P-RNTI for PEI if DCI size for PEI smaller than DCI Format 1_0. |
| Ericsson | - Observation 5 Reusing the predefined P-RNTI for PEI detection limits the possibility to extend PEI content in future. - Observation 6 Compared to a predefined RNTI, a configurable RNTI for PEI does not introduce considerable complexity for the UEs. - Proposal 9 For the PEI DCI, the RNTI used for CRC masking is configured via higher layers. |
| Qualcomm |  |
| MediaTek | For topic 5), RNTI for PEI DCI format, future compliance should be considered. If P-RNTI is utilized, there is limited space for future extension in order to avoid DCI size overlapping with paging DCI. Since introducing a new PEI-RNTI has little impact to network and UE, the following is therefore recommended: Proposal 5: A new RNTI, PEI-RNTI, is supported for PEI DCI format |
| Nokia | Observation: The PDCCH-based PEI should be monitored only by IDLE/Inactive UEs. The PEI DCI format CRC could be scrambled with P-RNTI or PEI-RNTI. |
| Nordic | Proposal-1: Support only the cases where PEI is mapped to 1,2 or 4 POs of a single PF in R17. Max size of DCI format payload is 34 bits. - Up to RAN2 whether to reuse P-RNTI or introduce PEI-RNTI |

For the RNTI for DCI format 2_7, the following statistics can be obtained:

- New PEI-RNTI (6 companies): HW & HiSi, vivo, TCL, CATT, Lenovo & Moto, MTK

- P-RNTI (5 companies): ZTE, Intel, Transsion, Samsung, DoCoMo

- Network configuration (P-RNTI or PEI-RNTI; 4 companies): Panasonic, LGE, Ericsson, Nokia,

- Depending on DCI format 2_7 size (3 companies): OPPO, Xiaomi, Nordic (up to RAN2)

In principle, which RNTI to be used depends on DCI payload size. For the case, DCI format 2_7 payload size is set the same as paging DCI for sharing the same CORESET while not increasing UE blind decoding complexity, a different RNTI should be applied. On the other hand, for the scenario, where network prefer dedicated occasion for PEI, the payload size can be configured different from paging DCI and reusing P-RNTI is possible. Since it is related to network implementation, moderator would like to suggest reserving the flexibility to network:

Q1: Please provides your views and/or suggested revision on the following Proposal 2.4-1

Proposal 2.4-1:

Either P-RNTI or new PEI-RNTI can be configured in SIB for PEI

Table 5: Companies’ views to the above questions

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Q1:Support Proposal 2.4-1 |
| MediaTek | We are supportive of 2.4-1. This provides the flexibility for different network configurations as well potential future extension. |
| Nordic | We agree, could be left up to RAN2 |
| Samsung | Q1: we think only one RNTI is enough. We think P-RNTI should be reused if there is no any technical issue. |
| Panasonic | On Q1, we support Proposal 2.4-1. |
| CATT | We believe a new PEI-RNTI should be supported for future proof. A new PEI-RNTI would also avoid any specification of abnormal condition of PEI detection. |
| Intel | There is no need to support two RNTIs for a DCI. We can discuss payload of PEI DCI first and then conclude RNTI issue. Some companies are using future compliance as justification, however, that does not necessarily solve the issue of impact of increased payload on the overall reliability. |
| Qualcomm | Q1: P-RNTI is sufficient given PEI DCI size should be always smaller than that for the paging PDCCH. Although network can configure large AL for PEI but in bad channel condition both paging PDCCH and PEI PDCCH will be configured with the largest AL. |
| CMCC | Ok |
| ZTE, Sanechips | If the payload size of PEI is no larger than paging DCI, P-RNTI can be reused. |
| LGE | Q1: we support the proposal 2.4-1. Also, we are fine with using new PEI-RNTI only. |
| Ericsson1 | Q1: Selection of RNTI should be left for NW configuration. Suggested update is as follows. Either P-RNTI or a separate new PEI-RNTI can be configured in SIB for PEI. |
| Huawei, HiSilicon | We support the proposal, which seems already some compromise. We think the DCI format size cannot be a strong argument to block the introduction of PEI RNTI. Let’s define the PEI functionality as a clear way and future proof. |
| vivo | Only a new PEI-RNTI is needed as a cleaner solution. A new PEI-RNTI is also suitable for TS 38.202 to describe the PEI monitoring behaviour, it can be different to PO. |
| DOCOMO | Same view as Qualcomm and ZTE. |
| Apple | We do not see the need for configurability. We should either use the P-RNTI or a new pre-defined RNTI depending on the outcome of PEI DCI size. |
| Lenovo/MotM | Support |
| Spreadtrum | Pending after decision of supporting paging search space for PEI |
| OPPO | If the payload size of PEI can be used to distinguish PEI and paging PDCCH, P-RNTI can be reused. |
| TCL | We support only PEI RNTI. In our view, PEI is a new feature of Rel-17, and using the existing paging RNTI will leads the legacy UE to assume the PEI DCI as paging PDCCH. Therefore, PEI RNTI should be used for PEI. |
| Transsion Holdings | Q1: Support |
| Nokia(1st round) | It is not fully clear if we would have selection (via configuration) two fixed RNTIs (i.e. P-RNTI and PEI-RNTI) or whether the PEI-RNTI would have full flexibility to be configured? In general, we don’t have very strong view, but if we introduce separate RNTI for PEI, it would be probably simples to use it always. I.e. there seem not be any extra benefit to support P-RNTI if PEI-RNTI is introduced. |
| IDCC | We are ok with this proposal. |
| Spreadtrum | Pending after decision of supporting paging search space for PEI |
| OPPO | If the payload size of PEI can be used to distinguish PEI and paging PDCCH, P-RNTI can be reused. |

For Q1, as quoted below for ease of reference,

Proposal 2.4-1: Either P-RNTI or new PEI-RNTI can be configured in SIB for PEI

- Yes: Xiaomi, MTK, Nordic (can also up to RAN2), Panasonic, CATT, CMCC, ZTE, LGE, Ericsson (with revisioin), HW & HiSi, vivo, Lenovo/MotM

- No : Samsung (P-RNTI only), Intel (P-RNTI only), QC (P-RNTI only), Apple (either P-RNTI-only or PEI-RNTI-only)

# Remaining PEI-O Related Designs

In RAN1#106-bis-e [2], initial agreements related to PEI-O are decided. The following are the remaining design details to be discussed decided in the following subsections:

- Determination of PEI-O location

- Support of unlicensed spectrum operation

- Other PEI-O Related Design Considerations

## Determination of PEI-O Location

The reference agreement in RAN1#106-bis-e is quoted as follows:

Agreement Determination of PEI-O location for a target PO is based on one of the following alternatives: - Alt 1: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO - FFS: The unit and the range of the frame-level offset - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion (e.g., to be the same as those of firstPDCCH-MonitoringOccasionOfPO) - Alt 2: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the L-th SS burst before the first PDCCH monitoring occasion of the target PO. - FFS: the case that a SSB burst overlaps in time with the target PO - FFS: L = 1, 2 or 3 - FFS: Reference the “start” or “end” of the L-th SS burst - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion - Alt 3: The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO. - FFS: The exact definition of the reference time, e.g. the first MO of the target PO, the first MO of the first PO indicated by the PEI, the start of the PF for the target PO - FFS: The unit and the range of the time offset - FFS: Whether any SS burst or TRS burst is needed between PEI-O and PO - Configuration for one PEI indicating multiple POs within a PF should be taken into consideration in the determination of PEI occasion Decide one of the above alternatives or a single merged solution based on the alternatives in RAN1#107-e meeting. FFS: Extension for the case one PEI indicates multiple POs across multiple PFs, if supported

In the following table, companies’ related views and proposals are collected:

| Company | Alt? | Companies’ Views |
| --- | --- | --- |
| Huawei, HiSilicon | Alt 2 | Observation 1: PEI can provide attractive power saving gain only when the PEI occasions are close to a SS burst(s). Table 1 Power saving gain for PEI in different location Power saving gain for 14ms gap between SSB and PO. 1ms gap between the end of first SS burst and PEI; 12.5ms gap between nearest SS burst and PEI (PEI 1ms before PO). 1SS burst for PDSCH T/F tracking; w/o RRM; 17.40%; 6.95%. 2SS bursts for PDSCH T/F tracking; w/o RRM; 30.15%; 5.72%. 3SS bursts for PDSCH T/F tracking; w/o RRM; 39.07%; 4.86%. Observation 2: PEI occasion after the first SS burst among the SS bursts which are required by UE before the reception of paging PDSCH can provide the maximal power saving gain. Figure 4. Alt 2 to define the monitoring occasion for PEI. Proposal 2: Adopt Alt 2 to determine PEI-O location for a target PO, where the first PDCCH monitoring occasion of the PEI-O is provided w.r.t. ‘the end’ of the L-th SS burst before the first PDCCH monitoring occasion of the target PO. - The L-th SS burst is configured by network via System information - L = 1, 2, or 3. - The offset, X, between the first PDCCH monitoring occasion of the PEI occasion and ‘the end’ of the L-th SS burst is not larger than 3ms; - UE is not required to monitor PDCCH PEI during the Y ms prior to the beginning of the target PO, which is used for UE processing of PEI before the PO; Figure 7 Time interval between PEI and PO reception should consider the RF retuning for Redcap UEs. Proposal 12: PEI is transmitted in the MIB-configured initial DL BWP where SSB is transmitted. Proposal 13: The value of minimum time offset between PEI and PO should consider the conclusion in RedCap regarding whether paging reception can be configured in the separate initial DL BWPs or not. |
| ZTE, Sanechips | Alt 3 | Figure 2(b) configuration of PEI associated with POs across PFs - Observation 1: In the scenario where the PEI is configured to indicate paging information across PFs, Alt 1 with one PF-specific offset is not sufficient to determine the PEI occasion. Figure 3 configuration of PEI relative to SSB - Observation 2: For Alt 2, the L-th SSB should be PO-specific in the scenario when one PEI is configured to indicate paging information for multiple POs - Observation 3: The PEI-O location determined by the L-th SSB before the target PO may not bring additional power saving gain. - Observation 4: Alt 3 is the most straightforward and robust to determine the PEI location in all scenarios. Figure 4(b) Configuration of PEI associated with multiple POs across the PF - Proposal 5: Alt 3, i.e., the determination of the PEI-O location by an offset for the target PO should be adopted. - Proposal 6: The occurrence of SSB between the PEI and the PO can be considered to provide a larger power saving gain. |
| vivo | Alt 2 | Figure 3: Power consumption description in High SINR case Proposal 7: Support Alt2 i.e., the first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the L-th SS burst before the first PDCCH monitoring occasion of the target PO. Proposal 8: The gap between PEI-O and the associated PF/PO should contain M SSB bursts, where the value of M can be 1, 2, 3 etc. Figure 4: An illustration for PEI-O related configuration. Proposal 9: To avoid a fragmented PEI-O configuration, support to adopt the following configuration and RRC parameters: - Define PEI-frame and configure the number of PEI-O contained in one PEI-frame. - Configure the number of POs in a PF that one PEI-frame can associate with. - Configure the indication relationship between PEI-Os in a PEI-frame and the associated POs for the PEI-frame. - Configure the list of firstPDCCH-MonitoringOccasionOfPEI-O for each PEI-frame. Each of PEI-Os in a PEI-frame corresponds to a firstPDCCH-MonitoringOccasionOfPEI-O parameter. Figure 5: An example for the overlapping between multiple PEI-Os in a PEI-frame. Proposal 10: It is necessary for PEI DCI to notify the indicated PO index(es) to avoid the ambiguity e.g., adding 0-2bits PO index(es) indication in PEI DCI. |
| TCL | Alt 2 | Figure 3 PEI-O configured 3 SSB earlier than the target PO Observation 1: PEI-O associated to the incoming target PO with no SS burst between PEI and the target PO will let PEI to be used only for paging indication and cannot be used for TRS availability Indication. Observation 2: PEI-O associated to the incoming target PO with at least 3 SSB earlier than the target PO will let PEI to be used for paging indication and TRS availability Indication. Proposal 4: The first PDCCH monitoring occasion of the PEI-O is provided at least 3 SS bursts before the first PDCCH monitoring occasion of the target PO. Figure 5 PEI-O associated to the previous PO for incoming PO with 3 SSBs Observation 3: PEI-O associated to the previous PO for the incoming target PO with at least 3 SSB after the previous PO will let PEI to be used for paging indication and TRS availability indication. Proposal 5: The first PDCCH monitoring occasion of the PEI-O is provided at least 3 SS bursts after the last PDCCH monitoring occasion of the previous PO. Proposal 6: Support Alt2: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the L-th SS burst before the first PDCCH monitoring occasion of the target PO, where L = at least 3 SS bursts. |
| Spreadtrum | Alt 1 | Table 3: The comparisons among Alt-1/2/3 Alt-1; Alt-2; Alt-3. Mechanism; Using PEI_offset to define PEI-F like PF; Using “firstPDCCH…PEI” to defined the PEI-O-specific symbol-level offset; Prior to the L-th SS burst before PEI-O; Using a reference time and a time offset to define the location of PEI-O. Flexibility; Full; TBD; Limited. Signaling overhead; Large; TBD; Small. New parameter; M, PEI_offset, “firstPDCCH…”; TBD; M, the time offset. RAN2 involvement; More due to parameters PEI_offset and “firstPDCCH…PEI”; TBD; Less. Proposal 4: Support Alt-1 for determination of PEI-O location. Proposal 5: If Alt-1 is supported, PEI-F can be determined by PEI_offset like PF, and the starting monitoring occasion of PEI-O can be determined by “firstPDCCH…” like PO. Proposal 6: The number of PO(s) indicated by one PEI should be defined. |
| CATT | Alt1 or Alt3 | Proposal 1: Three SSBs are needed for the UE to perform coherent detection of PDCCH-based PEI before PO. Proposal 7: For the frame-level offset in Alt 1, the unit is radio frame level and the range of the frame-level offset is 0~1. Proposal 8: The unit and range of the configuration for the first PDCCH monitoring occasion in Alt 1 can use the unit and range of firstPDCCH-MonitoringOccasionOfPO as the reference. Proposal 9: For determining the first PDCCH monitoring occasion of the PEI-O in Alt 3, the reference time is the first MO of the first PO indicated by the PEI. Proposal 10: For Alt 3, the first PO indicated by one PEI is the reference time of PO with PO_Index=0. Proposal 11: For Alt 3, the maximum time offset between PEI-O and reference time is the SSB periodicity. And the unit of the time offset is ms or slots. Proposal 12: The PEI-O could be determined by one of the following alternatives: - Alt 1: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO - Alt 3: The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO. - The reference time is the first MO of the first PO indicated by the PEI. |
| OPPO | Alt 1 or Alt 3 | Observation 1: For alt1, when one PEI is associated with multiple POs, if the PF of the first PO indicated by the PEI as the reference point, all the POs indicated by one PEI will have the same reference point and the same configuration of frame-level offset and firstPDCCH-MonitoringOccadionOfPO. But it will cause POs in a PF have different reference point in some cases. Observation 2: For alt1, when one PEI is associated with multiple POs, if the PF of PO indicated by the PEI as the reference point for this PO, all the POs in a PF will have the same reference point. But all the POs indicated by one PEI will have different reference point and configuration of frame-level offset when one PEI can indicate multiple POs across multiple PFs. Proposal 4: For alt1, when one PEI is associated with multiple POs, we prefer the PF of the first PO indicated by the PEI as the reference point. Observation 3: For alt2, when one PEI is associated with multiple POs, if all the POs indicated by one PEI are located behind the same SSB, these POs could have the same configuration of L-th SSB and offset between SSB and PEI. Observation 4: Alt3 determines the location of PEI via one offset between reference time and PEI, while alt1 and alt2 need two-level offset to determine the location of PEI. Observation 5: For alt3, if the reference time is the start of the PF for the target PO / first PO indicated by one PEI, alt3 is very similar to alt1, the difference between two alts is determining the location of PEI via one-level offset or two-level offset. Observation 6: Compare with alt2, alt1 and alt3 could achieve the same result (the L-th SS burst before the first PDCCH monitoring occasion of the target PO) by configuring appropriate offset between PEI and PO. Proposal 5: For alt3, when one PEI is associated with multiple POs, we prefer the start of the PF for the first PO indicated by one PEI as the reference time. Proposal 6: We prefer alt1 and alt3 when determine the location of PEI. - They can achieve the same result of alt2 via appropriate offset between PEI and PO. - When the reference time of alt3 is the start of the PF for the first PO indicated by one PEI, alt3 is a more direct way compare with alt1. |
| Sony | Alt 3 | Proposal 1 – From possible choices in alt 3, support the first monitoring occasion of the target PO or the start of the target PO to be selected as the reference point. Figure 1 – Design of PEI occasions Proposal 2 – Length for the offset can be expressed as PEI-O + L*SSB-periodicity, where L is at least one. The PDCCH monitoring occasion is given in number of slots or subframes and so do the time unit for the offset. Proposal 3 – Support SIB based configuration of PEI, including - a time offset relative to start of an associated PO, considered as reference time, to indicate start of PEI monitoring, - minimum time-gap between last PEI occasion and associated PO. |
| Intel | Alt 3 | Proposal 7: An offset is configured to derive MOs for PEI monitoring with respect to the start of PO or start of PF containing the PO, addressed by the PEI. Proposal 10. The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO, where the reference time can be one of the following: the first MO of the target PO, or the start of the PF for the target PO. |
| Xiaomi | Alt 1 | Proposal 3: Support Alt 1, that is the first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO. |
| CMCC | Alt 1 | Proposal 3. Alt 1 is supported as the determination of PEI-O location. Proposal 4. The PEI frame is determined as: - (SFNPEI + PEI_offset + PF_offset) mod T= (T div N)*(UE_ID mod N) in case 1 PEI indicating multiple POs in 1 PF; - (SFNPEI + PEI_offset + PF_offset) mod T= [T div (N/K)]*[floor(UE_ID/K) mod (N/K)] in case 1 PEI indicating multiple POs in K PFs. |
| Transsion Holdings | Alt 3? | Figure 1 Proposal 6: An offset set and monitoring window can be defined for the PEI-O |
| Panasonic | Alt 3 | Observation 7: To determine the first monitoring occasion of the PEI-O, the specification impact of Alt 1 is larger than Alt 2 and Alt 3, in terms of the number of steps and additional efforts to define one-to-multiple mapping between PEI-F and PFs, if PEI for POs across multiple PFs is supported. Observation 8: To determine the first monitoring occasion of the PEI-O, the Alt 3 can achieve same technical merits with Alt 2 but no need to specify the SS bursts between PEI-O and PO, which can just be gNB implementation. Proposal 7: To adopt Alt 3 that the first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. the first MO of the first PO indicated by the PEI. |
| Samsung | Alt 3 | Observation 3: It is the number of SSBs processed before PEI reception that matters to achieve the target power saving gain of PEI, rather than the timing between SSBs and PEI. The number of SSBs needed before PEI-O depends on channel condition and sleep duration, while the impact of the SSB burst location is negligible. Observations 5: Alt1 requires RAN2 to define a new concept of PEI frame, where multiple PEI-Os can be configured per PEI frame. However, multiple PEI-Os per frame is not needed to support 1:N mapping between PEI-O and POs. Observations 6: Alt2 overcomplicates the issue regarding time offset configuration, where the association or restriction based on SSB bursts are not needed, considering - the number of SSB bursts needed for synchronization/AGC before PEI or PO are up to UE implementation, which can be different among UEs, - a cell-specific configuration of L doesn’t work for all UEs in idle mode, - Rel-17 TRS resources for idle UEs is the PS scheme to address the issue of inappropriate time alignment between SSBs and data reception, which applies to both paging PDCCH and PEI PDCCH. The PEI should only focus on provide early paging indication to target PO. Observations 7: For RedCap UEs with separate initial DL BWP for paging, UE may doesn’t expect to receive SSBs in the separate initial DL BWP. Proposal 8: Support SIB based configuration for a time offset between start frame/slot of PEI-O and start frame/slot of the first PO indicated by the PEI. |
| Apple | Alt 3 | Proposal 6: The first MO for a PEI is provided by a negative offset in unit of symbols w.r.t. the first MO for the first PO indicated by the PEI. Separate offset value is provided for each PEI. |
| Lenovo, Motorola Mobility | Alt 1 | Proposal 3: The first PDCCH monitoring occasion of a PEI-O is provided with respect to a start of a reference frame determined by a frame-level offset to a PF of a target PO (Alt 1). |
| InterDigital | Alt 3 | Proposal 2: For determination of PEI-O location for a target PO, support Alt 3: The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO. Proposal 3: UE monitors the PEI PDCCH in K consecutive PDCCH monitoring occasions (where K is the number of actual transmitted SSBs) in a window according to the configured search space. The starting and ending points of the window are determined with reference to the firstPDCCH-MonitoringOccasionOfPO of the associated PO (or the first PO if multiple POs are indicated). |
| LGE | Alt 1 | Proposal 6: Support Alt 1 for determining the PEI occasion Proposal 7: Define PEI occasion, which is a set of PDCCH monitoring occasions. - The PDCCH monitoring occasions for PEI are determined according to peiSearchSpace and [firstPDCCH-MonitoringOccasionOfPEI] - When SearchSpaceId = 0 is configured for peiSearchSpace, the PDCCH monitoring occasions for PEI are same as for RMSI - When SearchSpaceId other than 0 is configured for peiSearchSpace, the UE monitors the PEI occasion which is configured via higher layer Proposal 8: Define PEI frame as the Radio Frame contains one PEI occasion or starting point of a PEI Occasion. - PEI Frame is determined by offset from the PF |
| NTT DOCOMO | Alt 1 | Proposed 6: Support the first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO (Alt1). |
| Ericsson | Alt 1 | - Observation 7 From the UE power consumption perspective, at average 10% paging rate, the UE can at 90% of the time in idle mode immediately go back to deep sleep after PEI decoding regardless of PEI location with respect to PO. - Observation 8 PEI transmissions should not be restricted to be in conjunction/adjacent to other transmission. - Proposal 15 A separate search space for PEI PDCCH monitoring can reuse the same principle as for paging including a new parameter firstPDCCH-MonitoringOccasionOfPEI. - Proposal 16 PO-specific configuration of the PEI search window follows alt1 from RAN1#106b-e meeting, including an offset frames from the paging frame (PF) associated with the PO ranging at least up to 3 SSBs prior to PF and includes a window of PEI monitoring occasions during which the UE searches for PEI. |
| Qualcomm | Alt 2 | Observation 1: BWP and CORESET for PEI PDCCH monitoring by the RedCap UE may depend on the following design aspect for the RedCap UE: - Whether separate initial DL BWP is adopted for the RedCap UE - Whether paging occasions are configured in RedCap initial DL BWP for the RedCap UE - Whether SSB or TRS is transmitted in the RedCap initial DL BWP. Observation 2: From UE power saving and implementation perspective, it is beneficial if - The gap between PEI and target PO is large enough for UE to finish PEI PDCCH decoding and wake up the hardware before PO - SSB and PEI are received in the same slot. Proposal 1: A non-zero gap between the PEI monitoring occasions across beams and the target PO is needed - The gap is from the end of PEI monitoring occasions to the start of target PO - Note: the gap should be long enough for the UE to process the PEI and to wake up the hardware. Observation 3: SSB and PO alignment can lead to UE power saving. Aligning SSB and PEI can achieve similar power saving effect for Rel-17 PEI design. Observation 4: For determination of PEI location for a target PO, Alt 1 and Alt 3 requires per PF or per PO offset configuration to align the PEI with SSB. Figure 1: Alternatives for location of PEI monitoring occasion Proposal 3: To determine the location of the first PEI PDCCH monitoring occasion, adopt the alignment between PEI and SSB as described in Alt 2 with the following updates - Do not use the SSB burst that overlaps in time with the target PO as reference time for PEI location - There is no need to explicitly define L. The first SSB before the PO that satisfies the minimum gap between PEI and PO is used as the reference time for PEI - Use the “start” of the SSB burst as reference time for PEI. |
| MediaTek | Alt 2 | Observation 2: For UE power saving gain in high SNR case, PEI-O should be placed close/adjacent to an earlier SS burst before PO. Figure 1: PEI-O locations and the corresponding power saving gains (illustrating the case UE not paged) Observation 3: There is buffering penalty to realize UE power saving with overlapped PEI-O and SS burst. The amount of buffering is even larger for a REDCAP UE than an eMBB UE due to its slower processing speed. - Note: if REDCAP UE needs to additionally receive previous SS burst for 1st PEI to avoid buffering, the wake-up energy overhead will cause larger power consumption than legacy UEs in high SNR case. Figure 2: PEI-O location where PDCCH monitoring occasion for PEI can be ahead of its associated SSB Proposal 6: PEI-O location should be close to and after an earlier SS burst before PO Observation 4: PDSCH is more sensitive to CFO than PDCCH. The CFO that allows UE to detect PDCCH-based PEI may not be small enough for reliable detection of paging PDSCH. Proposal 7: Network configuration between PEI-O and PO should allow at last one SS burst or TRS for UE to further reduce CFO for reliable detection of paging PDSCH Proposal 8: For determination of PEI-O location, the following alternative is supported: - Alt 2: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the L-th earlier SS burst before the first PDCCH monitoring occasion of the target PO. - L is configurable with value in {1, 2, 3} - The first PDCCH monitoring occasion of the PEI-O is provided by a time offset from the slot end of the reference SS burst and is configurable with value in {0, 1, 2, …, 63} symbol(s) |
| Nokia | Alt 1 | Observation: The start point of the PEI-O needs to be determined at symbol level. Proposal: Determine the PEI-O configuration based on PF of the target PO. Proposal: Define the reference location for PEI monitoring, PEI frame (PEI-F), based on offset to PF. Offset could be defined in radio frames, and be separate for each PF, with range of 1 to [8] frames. Proposal: Define a PO specific offset, PEI-O, (e.g. ‘firstPDCCH-MonitoringOccasionOfPEI’) in relation to PEI monitoring reference location PEI-F. Offset could be defined in symbols and be separate for each PO. Figure 1. Illustration of PDCCH-based PEI monitoring occasion configuration Observation: If Type0-PDCCH CSS is re-used for PEI monitoring, PEI-PF offset can be used to indicate the PEI monitoring occasion corresponding to the PF/PO. Observation: Minimum time gap between PO and PEI should enable UE to acquire synchronisation for paging message reception. Figure 2 Timeline for SSB, PEI and PO monitoring/reception. Figure 3 Result for PEI duration of 3/2/1 slots (left) and 8 slots (right). The reference is the release 15 paging procedure. Observation: The highest energy saving benefit is achieved when the PEI is located right after the last SSB prior to the PO. Observation: The lowest energy saving benefit is achieved when the PEI is located at an offset after the last SSB, which prevents deep sleep. Observation: The energy saving impact of the SSB-to-PEI offset is small considering overall UE energy consumption. Proposal: Network implementation can decide where network configures the PEI location in time domain. |
| Nordic | Alt 3 | Figure 1 PEI to PO mapping for different combinations Proposal-3: Support Alt3, gNB configures offset in units of frames and range is from [1-4] frames. - For the case when idle TRS periodicity is large >20ms, consider delaying UE’s PF from nominal location to frame after TRS, in order to facilitate power saving. - Map PEI and PO(s) as in Figure 1 - Note: Whether gNB guarantees 1 SSB or iTRS between PEI frame and PO frame is up to gNB implementation |

From the above table, the following statistics can be obtained:

- Alt 1 (10 companies): Spreadtrum, CATT, OPPO, Xiaomi, CMCC, Lenovo & Moto, LGE, DoCoMo, Ericsson, Nokia,

- Alt 2 (5 companies): HW & HiSi, vivo, TCL, QC, MTK

- Alt 3 (10 companies): ZTE, CATT, OPPO, Sony, Intel, Transsion, Panasonic, Samsung, Apple, Nodic

Since Alt 2 is of less support and SS burst start is also a frame start, moderator would like to first check companies’ view on merging Alt 1 and Alt 2. Since proponents of Alt 2 bring technical analyses that shows PEI-O close to SS burst is important for realizing the power saving, particularly useful for high SNR case. Moderator would like to check companies’ views in how to incorporate such benefit in terms of some proper restriction to Alt 1 parameters.

Q1: Please provide your views and/or suggest revision for the possible integration of Alt 1 and Alt 2 that can include the power saving benefits by placing PEI-O close to an earlier SS burst (intention of Alt 2).

- Alt 1: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO - FFS: The unit and the range of the frame-level offset - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion (e.g., to be the same as those of firstPDCCH-MonitoringOccasionOfPO)

- Alt 2: The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the L-th SS burst before the first PDCCH monitoring occasion of the target PO. - FFS: the case that a SSB burst overlaps in time with the target PO - FFS: L = 1, 2 or 3 - FFS: Reference the “start” or “end” of the L-th SS burst - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion

For Alt 3, it is observed per-PO time offsets may be needed for flexible PEI-PO mapping. Also it is also useful if Alt 3 can incorporate part of characteristics of Alt 1 and Alt 2. In this regard, moderator would like to check companies’ views on the following question:

Q2: Please provide your views and/or suggest revision for supporting Alt 3 with

- Per-PO time offsets (and number of time offsets will be POnumPerPEI)

- Charactering a time offset by “number of frame(s)” and “symbol offset” from each PO start to PEI-O start. In this way, number of bits for each time offset can be reduced and the “frame offset” concept of Alt 1/2 can be incorporated.

- Alt 3: The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO. - FFS: The exact definition of the reference time, e.g. the first MO of the target PO, the first MO of the first PO indicated by the PEI, the start of the PF for the target PO - FFS: The unit and the range of the time offset

Table 6 Companies’ views to the above questions

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Q1: From our view, the intention of Alt 2 can be naturally merged in Alt 1. For Alt 1, the offset can be flexibly configured by frame level unit. If the offset if M* SSB periodicity, at least M-1 SSB will be located between PEI and PO. We can agree with the intention of Alt 2, place PEI after and near to a SS burst so that T/F tracking and synchronization can be more power saving. But since we already have TRS for idle, which can be configured very flexibly, to facilitate the same purpose, it would be too restrictive to adopted Alt 2. It is even possible that only TRS, no SS burst, is between PEI and PO. Q2: The disadvantage of Alt 3 is there would be multiple offset if one PEI for multiple PO. And this is main reason we don’t support it. But if Alt 3 is to be supported, we agree with Moderator proposals in Q2. |
| MediaTek | For Q1: If there is no majority support on Alt 2, we think allowing PEI-frame to be set to a frame with SS burst can be one way forward. Alt 1 (updated): The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of a reference frame determined by a frame-level offset to the PF of the target PO - The frame-level offset from PF start to PEI frame (PEI-F) start is in unit of frame and has value range of {0, 1, 2, 3, 4, 5, 6, 7, ,8} - The unit of the configuration for the first PDCCH monitoring occasion w.r.t. PEI-F is symbol and has the following range, depending on SCS setting: SCS setting; Value range (spanning 8 ms in unit of symbol). 15 kHz; {0, 1, …, 111}. 30 kHz; {0, 1, …, 223}. 60 kHz; {0, 1, …, 447}. 120 kHz; {0, 1, …, 895}.. For Q2: For Alt 3, per-PO time offset is a reasonable way forward for UE to identify PEI-O along the determination of PO. Assuming 1 PEI for POs in a PF, we can specify 4 time offsets so that UE can refer its offset according the PO index is. Decoupling the time offset into frame portion and symbol portion is also useful to incorporate Alt 2 design as well as reduce the bit number for flexible time offsets. On the other hand, for the case 1 PEI is only for POs in a PF, we see the resulting signalling overhead is actually larger than Alt 1, where the frame offsets for the 4 POs is a common PEI-F offset. Neverthless, the following updated Alt 3 can also be considered: Alt 3 (updated): The first PDCCH monitoring occasion of the PEI-O is provided by a time offset w.r.t. a reference time for the target PO. - The reference time the start of the PF for the target PO - There are Ns time offset(s) specified as of, i×#symbols per frame+os, i, 0≤i<Ns, - The range of of, i is {0, 1, 2, 3, 4, 5, 6, 7, ,8} - #symbols per frame and the range of os, i depends on SCS setting: SCS setting; #symbols per frame; the range of os, i. 15 kHz; 140; {0, 1, …, 139}. 30 kHz; 280; {0, 1, …, 279}. 60 kHz; 560; {0, 1, …, 559}. 120 kHz; 1120; {0, 1, …, 1119}.. |
| Nordic | For Q2: We are supportive of alternative 3, but we think offset should be only in units of frames. It would be sufficient to have predetermined mapping between MOs of PEI to MOs of POs If some minimal values of distance should be defined, they could be in symbols. |
| Samsung | Q1: We think Alt3 already provide gNB high flexibility to place PEI-O close to SSB burst. In general, we don’t the gap between SSB burst and PEI-O is a problem at all. In the worst case, the nearest SSB burst before PEI-O is one SSB burst period, e.g. 20ms, which is totally fine. The synchronization won’t change within such short period. Also, idle mode TRS resources is the PS scheme to address the issue. Lastly, if companies really think SSB location is an issue, we are fine to not reuse SS set #0. Q2: we don’t see any benefit to support Per-PO time offset. The gaps between POs that indicated by the same PEI are predetermined. Even we support multiple Per-PO time offsets, the value of Per-PO time offset is limited only by the time offset between PEI-O and the first PO from the multiple POs indicated by the same PEI. |
| Panasonic | On Q2, the time offset is defined w.r.t the first MO of the first PO indicated by the PEI. This is a quite clearer solution rather than charactering a time offset from each PO start to PEI-O start. |
| CATT | Q1: We don’t agree to integrate Alt 1 and Alt 2. It is network configuration to determine the distance of PEI occasion to the SSB. We don’t agree on this proposal in particular Alt 2 has least support from companies. Either Alt 1 or Alt 3 standalone would be a good solution. Q2: We have strong concern and can NOT agree on the Per-PO time offset since it requires large higher layer signaling for indicating offset of each PO without any benefit when one PEI associated with multiple POs. |
| Intel | Q1. Do not support. Based on companies positions, we think correct approach should be to merge Alt 1 and Alt 3 and find their common ground (e.g., using start of PF as reference for calculating offset), instead of bringing Alt2 into the picture which has minimal support and needs more specification efforts. Q2: We think per PO offset is possible, although it may require gNB to indicate multiple offsets per PO in a PF. To find a common ground, one way forward is to consider start of PF as reference point for calculating offset. Whether to adopt frame level offset or symbol/slot level offset, that can be further discussed. We suggest to agree the following, by merging Alt 1 and Alt 3 - The first PDCCH monitoring occasion of the PEI-O is provided w.r.t. the start of the PF of the target PO - FFS: The unit and the range of the offset(s) needed to identify the first PDCCH monitoring occasion of the PEI-O - FFS: The unit and the range of the configuration for the first PDCCH monitoring occasion (e.g., to be the same as those of firstPDCCH-MonitoringOccasionOfPO) |
| Qualcomm | Q1: We agree that Alt 1 can achieve Alt 2 effect and provides more flexibility of network configuration. However, we would like to note that such a flexibility is not beneficial for UE power saving and can even hurt UE power saving performance if PEI location is not aligned with SSB location. In typical channel condition, UE just need one SSB to update its tracking loops. In this case, the only essential power saving source is the alignment between PEI and SSB so that UE just wakes up once to receive both SSB and PEI. Without this alignment, the power saving gain is only provided by UE sub-grouping. As indicated by companies’ evaluation result, this power saving gain is very small. Even if some UEs need more than SSB per PO, aligning PEI and SSB still provides essential power saving gain. Therefore, we think Alt 2 should be the design as long as network configures UE to monitor PEI. Because of this, Alt 2 should be the baseline for PEI location determination. Q2: We do not think there is a need to explicitly configure per-PO offset from PO to PEI as Alt 3. Alt 2 can implicitly provide per-PO offset if PEI is aligned with SSB. |
| Sharp | Q1: Alt1 is sufficient, gNB can manage the offset for PEI-paging occasion from SSB by configuring a similar parameter as “firstPDCCHMonitoringOccasionOfPO” Q2: we don't support Per-PO time offsets. the PEI configuration will be configured in SIB for idle/inactive UEs and the configuration per-PO offset will incur significant overhead. |
| CMCC | Q1: prefer Alt 1, gNB can guarantee a proper configuration to avoid the overlapping between SSB and PEI-O to avoid the issue in Alt 2. In addition, if we support one PEI associates with multiple PFs, Alt 2 needs more spec effort, e.g., the values of L-th SS burst are different among POs if the time gap between PF is larger than 20ms SSB periodicity. Q2: Don’t support Alt 3, considering 1 PEI can indicate multiple POs in a PF, Alt 1 is simpler and reduce signalling overhead than Alt 3 which take PF as reference time not each PO. |
| ZTE, Sanechips | Q1: As we commented in our contribution, we think that the SSB processed by UE depends on many factors in implementation, it is not easy to conclude that which SSB that PEI is closely located will be beneficial to all UEs’ energy efficiency. Therefore, we don’t think we need to restrict the PEI location. Meanwhile, with alt 3, NW can configure PEI close to SSB, if needed. So we don't agree to merge alt1 and alt2 into one solution. Q2: We agree that alt1 and alt 3 resemble each other. We believe that alt 3 is the most flexible way for the determination of PEI occasions, which will fit into different PEI-PO mapping relationships. Therefore, we support the above proposal in general. And we are okay to consider some aspects in alt1 and alt2 for the sake of progress, for example the time offset w.r.t. a reference time for the PF of the target PO. In addition, the occurrence of SSB between the PEI and the PO can be considered. |
| LGE | Q1: The intention of Alt 2 can be covered by Alt 1, and it is obvious that Alt 1 provide more flexibility of network configuration. Moreover, it should be noted that SSB periodicity can be set up to 160 ms by higher layer. The gap between PEI-O and PO around 160 ms seems not a proper configuration since it may cause significant paging latency. Thus, we prefer to keep current version of Alt 1. Q3: If a frame level offset is considered for Alt 3, we don't see much difference between Alt 1 and Alt 3. Otherwise if a slot/symbol level offset is considered without frame level offset, signaling overhead will be a matter. Moreover, if one PEI mapping to Ns PO(s) within a PF(or Ns*X PO(s) within a consecutive X PFs) is supported, a frame level offset can be applied to all the POs within a PF(s) simply. |
| Ericsson1 | Q1: Support Alt 1, and the allowed value range for both time offset and the first monitoring occasion should provide sufficiently flexibility for the NW. We support Mediatek proposed compromise Alt 1(updated), perhaps with a bit detailed check on the exact values. As explained in our contribution, Alt 1 enables simpler paging reconfiguration of the gNB by the O&M system compared to if other reference points such as POs are used. For example, the operator does not need to recalculate/reconfigure the PEI occasions in case additional POs are added to the system during runtime. Q2 : We do not see any clear benefit of Alt 3 over Alt 1. |
| Huawei, HiSilicon | Q1: we think Alt.2 should be the baseline for the discussion. All companies provides evaluation results based on the assumption of Alt.2 to provide PEI occasions are close to SS burst. We also would like to comment from UE vendor perspective that the reception duration and light sleep time would be reduced a lot if PEI occasions are close to SS burst, especially for High SINR case. It is not only about synchronization which is mentioned by Samsung. For eMBB UEs, we also observed that most of UEs only need one SS burst. For some RedCap UEs with reduced Rx chains, more SS bursts would be needed. Therefore, we think the close of SS burst and PEI occasions are the key component of PEI feature. Regarding the flexible configurations in Alt.1 and Alt.3, we agree with Qualcomm that this would not provide any power saving gain, which is the most important motivation of this feature. And also, it seems we just design another flexible configured set of POs by Alt.1 and Alt.3, which may have the same situation as that in Rel-15/16 with worse power consumption than LTE. We are open on the merge of Alt.1 and Alt.2, but we think we should target to achieve the Alt.2, which is the basis of the power saving gain we observed. Q2: we don’t think per-PO configuration is needed. One PEI is transmitted per SS burst period. Let’s just define one offset to determine PEI location. |
| vivo | Q1: We agree that the intention of Alt2 should be addressed. Also, it will be fine to integrate Alt 1 and Alt 2. To extract the advantages of both alternatives, we suggest the following integration solution: - Define PEI-frame, and select the system frame which contains SSB as the PEI-frame to address the intention of Alt2 as shown in the below figure. - To avoid the Per-PO time offset, configure the list of firstPDCCH-MonitoringOccasionOfPEI-O for each PEI-frame. Each of PEI-Os in a PEI-frame corresponds to a firstPDCCH-MonitoringOccasionOfPEI-O parameter. - To place PEI-O close to an earlier SS burst, the values of firstPDCCH-MonitoringOccasionOfPEI-O can be configured with respect to the end/start of the SSB contained in the PEI-frame. - FFS: whether the gap between PEI-O and the associated PF/PO should contain M SSB bursts, where the value of M can be 1, 2, 3 etc. Q2: we do not support Alt3 due to the Per-PO time offset. |
| Apple | Q1: we do not think Alt 1 and 2 can be easily merge together. Alt 1 or Alt 3 can be configured in a way to achieve Alt 2, but not guaranteed. We do agree that putting PEI and SSB close together is more efficient for UE power saving, but this can be only achieved if the offset between SSB and PEI is very strictly controlled. It would be good to know the proposed range for the offset in Alt 2. Q2: Alt 1 and 3 are very similar fundamentally. The reason for us to support Alt 3 with the reference time being the first MO of the first PO indicated by the PEI is that the start of the PF for the target PO may not be a good choice because the actual PO can be far from the start of the PF. This would result in a larger range for the offset value. On the other hand, if we use the first MO as the reference time, we only need to determine the maximum offset for PEI compared to the first MO and the range is then well-defined. Moreover, for Alt 3, the time offset does not need to be per PO. It can be configured per PEI, using the first MO of the first PO indicated by the PEI as the reference. In terms of overhead, it is not clear to us why a single offset in unit of symbol would have more overhead compared to a frame offset plus a symbol-level offset. We think they should be the same or similar (a single offset should not be worse). |
| Lenovo/MotM | Q1: We think Alt 1 is sufficient. It is not clear that Alt 2 (attempting to align PEI-O with SSB burst) can provide the best power saving gain. Also, TRS provisioned for idle/inactive mode UEs should be considered. Q2: Not support. It is not clear that per-PO time offset can provide additional power saving gain. |
| Spreadtrum | Q1: We support merge Alt-1 and Alt-2 by putting the restrictions, e.g. guarantee the maximum offset b/w the 1st SS burst and PEI is smaller than a value. We should optimize the case where PEI indicates not to monitor PO, so the maximum offset b/w the 1st SS burst and PEI should be small enough to achieve the promising power saving gain. For the MTK’s version, we slightly prefer to reuse “fisrtPDCCH…PO”, which gives the full flexibility, i.e. firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL, -- Need R Q2: For Alt-3, multiple offsets should be supported at least for 1 PEI indicating 1 PO. We think without frame/symbol level offset mechanism, the absolute values of offset cannot provide the sufficient flexibility, and UE power saving gain will be reduced. So we prefer Alt-1 than Alt-3. |
| OPPO | For alt3, when 1 PEI map to multiple POs, the time offset is defined w.r.t a reference time for the first PO indicated by the PEI. There is no need to configure a time offset from each PO start to PEI-O start. To merge alt1 and alt3, we think the reference time of alt3 could be the start of the PF for the first PO indicated by PEI. And the offset is from the start of PF for the first PO indicated by PEI to the start of PEI-O. |
| TCL | Q1: we support alt2, as it can provide enough time gap between PEI and PO for the case when TRS in configured before the target PO, and PEI can be used to indicate the TRS. In our view, a frame level offset is not as stable option and it can change with the number of configured POs in a PF. On the other hand, in a PO level offset there will be one constant offset between PEI and the target PO. Even in case when 4 POs are configured in a PF, only one PEI will be transmitted and the PEI-O offset can be defined based on the first PO. Q2: Alt3 which focus on multiple offset and summing up the offset. In our view, there is no need of summing up the offset. |
| Transsion Holdings | We support alt 2+alt3. For idle/inactive mode, the SSB period is not always 20ms, but may be 160ms. This causes the number of POs in an SSB period to exceed the maximum number of POs that can be associated with a PEI. Therefore, during one SSB, we may need multiple PEIs. Therefore, we suggest that for the PEI closed to the SSB, the reference point should be SSB, and for other PEIs in the same SSB period, the reference point should be PO |
| Nokia(1st round) | Q1: We would support Alt 1 (as explained in our paper). For the frame level offset, range of {0 ..8} frames could be considered. The symbol level offset (PEI-O) from the reference frame (PEI-F) could have same range and definition as for firstPDCCH-MonitoringOccasionOfPO. As explained in our paper, this offset would need to be in symbol level to enable addressing the symbols in slot as per SS configuration. Q2: If we want to support mapping of PO to different PEIs (in a PF) we think we need PO specific offset. I.e. it should not be required to have all PEI monitoring occasions (corresponding to different POs) to be consecutive. We could consider using re-using the firstPDCCH-MonitoringOccasionOfPO for PEI as well. However as the UL/DL slot pattern periodicity can be 20ms, thus if the frame offset (between PF and PEI-F) is not function of (or aligned to) of UL/DL slot pattern periodicity , the PEI-O would need to be different than firstPDCCH-MonitoringOccasionOfPO. Number of offsets needed would be same as number of POs (similar to paging configuration). The offset from PF to PEI-F could be PF specific, thus does not significantly affect the overhead. |
| IDCC | For Q2, we think offset to the first PO can be used. For Q1, we do not think Alt2 is necessary. Overall, we prefer Alt 3 due to its simplicity. |
| Spreadtrum | Q1: We support merge Alt-1 and Alt-2 by putting the restrictions, e.g. guarantee the maximum offset b/w the 1st SS burst and PEI is smaller than a value. We should optimize the case where PEI indicates not to monitor PO, so the maximum offset b/w the 1st SS burst and PEI should be small enough to achieve the promising power saving gain. For the MTK’s version, we slightly prefer to reuse “fisrtPDCCH…PO”, which gives the full flexibility, i.e. firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL, -- Need R Q2: For Alt-3, multiple offsets should be supported at least for 1 PEI indicating 1 PO. We think without frame/symbol level offset mechanism, the absolute values of offset cannot provide the sufficient flexibility, and UE power saving gain will be reduced. So we prefer Alt-1 than Alt-3. |
| OPPO | For alt3, when 1 PEI map to multiple POs, the time offset is defined w.r.t a reference time for the first PO indicated by the PEI. There is no need to configure a time offset from each PO start to PEI-O start. To merge alt1 and alt3, we think the reference time of alt3 could be the start of the PF for the first PO indicated by PEI. And the offset is from the start of PF for the first PO indicated by PEI to the start of PEI-O. |

## Support of Unlicensed Operation

The related agreement in RAN1#106-bis-e is quoted below:

Agreement A PEI occasion (PEI-O) is a set of S consecutive PDCCH monitoring occasions when nrofPDCCH-MonitoringOccasionPerSSB-InPO is not configured - S is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 - The K-th PDCCH monitoring occasion for PEI in the PEI-O has the same QCL assumption as that of the K-th PDCCH monitoring occasion for paging in the PO. - Note: QCL reference is SSB - FFS: Determination of the PEI-O location - FFS: Support of unlicensed spectrum operation with nrofPDCCH-MonitoringOccasionPerSSB-InPO configured

In the following table, companies’ related views and proposals are collected:

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon |  |
| ZTE, Sanechips |  |
| vivo |  |
| TCL |  |
| Spreadtrum |  |
| CATT | Proposal 17: PDCCH-based PEI can apply the same principle of multiple occasions for paging DCI in unlicensed spectrum. |
| OPPO |  |
| Sony |  |
| Intel |  |
| Xiaomi |  |
| CMCC |  |
| Transsion Holdings |  |
| Panasonic | Proposal 5: For unlicensed spectrum operation, a separate parameter from nrofPDCCH-MonitoringOccasionPerSSB-InPO may be configured for PEI-O to support multiple monitoring occasions for each beam. |
| Samsung | Proposal 9: Reuse the multi-beam operation of PO for PEI-O regardless of nrofPDCCH-MonitoringOccasionPerSSB-InPO configured or not, such that a PEI occasion is a number of consecutive PDCCH monitoring occasions, where - the number of consecutive PDCCH monitoring occasions, N, equals to the number of PDCCH MOs per PO - the K-th PDCCH monitoring occasion for PEI in the PEI-O has the same QCL assumption as that of the K-th PDCCH monitoring occasion for paging in the PO |
| Apple |  |
| Lenovo, Motorola Mobility |  |
| InterDigital | Proposal 4: PEI is supported in unlicensed spectrum: A PEI-O is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPEI-O if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. |
| LGE |  |
| NTT DOCOMO |  |
| Ericsson |  |
| Qualcomm |  |
| MediaTek |  |
| Nokia | Proposal: Multiple monitoring occasions per actually transmitted SSB is supported when nrofPDCCH-MonitoringOccasionPerSSB-InPO is configured. |
| Nordic |  |

There is no objection to the following principle: PEI can apply the same principle of multiple occasions for paging DCI in unlicensed spectrum. In this regard, moderator would like to check companies’ views in the following proposal:

Q1: Please provides your views and/or suggested revision on the following Proposal 3.2-1

Proposal 3.2-1:

A PEI-O is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPEI-O if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S.

Table 7: Companies’ views to the above question

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Agree with the proposal. |
| MediaTek | We are supportive of Proposal 3.2-1 which is analogous to PO. |
| Nordic | Agree |
| Samsung | We don’t think explicit configuration of nrofPDCCH-MonitoringOccasionPerSSB-InPEI-O is needed. nrofPDCCH-MonitoringOccasionPerSSB-InPO can be reused for PEI-O. There should be one to one mapping between PEI PDCCH MOs and paging PDCCH MO. We suggest to replace “nrofPDCCH-MonitoringOccasionPerSSB-InPEI-O” by “nrofPDCCH-MonitoringOccasionPerSSB-InPO” |
| Panasonic | We support this proposal. |
| CATT | We are OK with the proposal. |
| Intel | Support |
| Qualcomm | We are fine with proposal. Agree with Samsung that the number of PMOs per PO needs not to be explicitly configured for PEI. |
| CMCC | Ok |
| ZTE, Sanechips | Okay to reuse the principle of PO determination in NR-U. |
| LGE | It seems like our view for the shared spectrum channel access is not captured. We are generally fine with supporting ‘S*X’ consecutive PDCCH monitoring occasions. However, we would like to emphasize that ‘stopPagingMonitoring’ can be indicated using short message field in paging DCI to prevent unnecessary PDCCH monitoring at the PO where UE receive the indication when nrofPDCCH-MonitoringOccasionPerSSB-InPO is configured. From the power saving perspective, similar mechanism should be supported for the PEI. Otherwise, UE may need to monitor the entire PDCCH monitoring occasions of PEI-O regardless of PEI detection results, which is against the purpose of power saving feature. From this point of view, we need to consider how to indicate “stop PEI monitoring” when S>1 repetition is configured for a PEI-O. |
| Ericsson1 | OK. |
| Huawei, HiSilicon | Similar view with Samsung. It is not clear whether nrofPDCCH-MonitoringOccasionPerSSB-InPEI-O is configured only for unlicensed spectrum. Maybe we just reuse the parameter of PO. |
| vivo | OK. |
| Apple | Agree |
| Lenovo/MotM | Ok with the proposal |
| OPPO | Support. |
| TCL | Agree with the proposal. |
| Transsion Holdings | Support |
| Nokia(1st round) | We are OK with the proposal in principle, and agree with Samsung that paging parameter can be re-used. |
| IDCC | Support. |
| OPPO | Support. |

For Q1, as quoted below for reference,

Proposal 3.2-1: A PEI-O is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPEI-O if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S.

- Yes: Xiaomi, MTK, Nordic, Panasonic, CATT, Intel, Ericsson, vivo, Apple, Lenovo/MotM

- No (unless nrofPDCCH-MonitoringOccasionPerSSB-InPO): Samsung, QC, HW&HiSi, LGE (need ‘stopPagingMonitoring’),

## Other PEI-O Related Design Considerations

In the following table, companies’ related views and proposals are collected:

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon |  |
| ZTE, Sanechips |  |
| vivo |  |
| TCL |  |
| Spreadtrum |  |
| CATT |  |
| OPPO |  |
| Sony |  |
| Intel |  |
| Xiaomi |  |
| CMCC |  |
| Transsion Holdings | Figure 2 Proposal 7: PEI should support multiple beams |
| Panasonic |  |
| Samsung |  |
| Apple |  |
| Lenovo, Motorola Mobility |  |
| InterDigital |  |
| LGE |  |
| NTT DOCOMO |  |
| Ericsson |  |
| Qualcomm |  |
| MediaTek |  |
| Nokia | Proposal: Network flexibility to choose in which cells/beams paging is sent, should be maintained and applied also to PEI. Proposal: To enable/disable broadcast beam specific PEI, bit map could be used to indicate the SSBs to which the PEI is active. Proposal: The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. Proposal: Determine the valid PDCCH-PEI monitoring occasions from the search space configuration (e.g. ‘pagingSearchSpace’ or ‘peiSearchSpace’) based on monitoring occasion timing indicated by PEI-F and PEI-O (e.g. ‘firstPDCCH-MonitoringOccasionOfPEI’), slot configuration and number of actually transmitted SSBs. |
| Nordic |  |

# Remaining ‘peiSearchSpace’ related designs

In RAN1#107-e, there agrees to specify ‘peiSearchSpace’ details for PEI:

Agreement Support configuration of a dedicated search space (‘peiSearchSpace’) for PEI - FFS: Configuration details and whether and how to reuse legacy search space sets, including pagingSearchSpace and searchSpaceSetZero

The following sub-sessions are for addressing the remaining issues:

- Whether and how to reuse ‘pagingSearchSpace’ and ‘searchSpaceSetZero’

- Other dedicated specification for ‘peiSearchSpace’

## Whether and How to Reuse ‘pagingSearchSpace’ and ‘searchSpaceSetZero’

In the following table, companies’ related views and proposals are collected:

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon | Figure 1 SearchSpaceZero configuration result in PDCCH monitoring occasion before the corresponding SSB Proposal 1: peiSearchSpace can be configured to refer to an existing search space set if PDCCH monitoring occasions of the search space set to monitor PEI appearing after the corresponding SS burst. |
| ZTE, Sanechips |  |
| vivo | Proposal 5: Not support to reuse searchSpaceSetZero as PEI search space. And support to reuse pagingSearchSpace as PEI search space when pagingSearchSpace ID>0. |
| TCL |  |
| Spreadtrum | If search space zero is used for PEI and the same mechanism is applied to PEI-O, there could be collision b/w PO and the corresponding PEI in a frame. In our view, search space zero used by RAR/paging can be regarded as “fallback” search space when the time domain resource for CSS is lack. For paging search space, it is more flexible than search space zero due to SIB1 configuration. As the commonality of search space zero and paging search space, they are both used for scheduling data, so there are symbols in between monitor occasions reserved for scheduled data. However, this is not true for PEI search space. Therefore, we are reserved our viewpoint on supporting using search space zero or paging search space for PEI, but we are open to discuss it. |
| CATT | Proposal 15: Reusing searchSpaceSetZero and pagingSearchSpace for PDCCH-based PEI is supported. |
| OPPO |  |
| Sony |  |
| Intel | Proposal 5: Dedicated search space of PEI may include paging search space or search space zero. |
| Xiaomi | Proposal 1: Paging search space should be reused to transmit PEI. Proposal 2: Support search space 0 for PEI search space. Proposal 8: If PEI is transmitted in paging search space, UE can directly try to decode the paging DCI in the same PO carrying PEI, to get the short messages. |
| CMCC |  |
| Transsion Holdings | Proposal 4: Reusing pagingSearchSpace with SearchSpaceId> 0. Proposal 5: SearchSpaceZero cannot be supported. |
| Panasonic | Proposal 6: The dedicated search space (‘peiSearchSpace’) for PEI is optionally configured and added in the commonSearchSpaceList of PDCCH-ConfigCommon. If not configured, peiSearchSpace can share same search space ID with pageSearchSpace or searchSpaceSetZero. |
| Samsung | Observations 4: PEI PDCCH is no different from paging PDCCH regarding synchronization requirement based on SSB, there is no problem to reuse SS#0 for PEI. gNB has the flexibility to configure a search space set ID with applicable value of 0, 1, 2, 3, for peiSearchSpace. Proposal 6: Support configuration of peiSearchSpace with applicable values of 0, 1, 2, or 3. by SIB. Proposal 7: Support same CORESET for peiSearchSpace and pagingSearchSpace without explicit/dedicated configuration of CORESET for peiSearchSpace. |
| Apple | Proposal 7: peiSearchSpace can be configured to be search space set #0 for multiplexing pattern 1 if O is not equal to 0. - Further discuss whether peiSearchSpace can be configured to be search space set #0 for multiplexing pattern 2 and 3. |
| Lenovo, Motorola Mobility |  |
| InterDigital | Proposal 5: Support configuration of pagingSearchSpace and searchSpaceSetZero as PEI search space. |
| LGE |  |
| NTT DOCOMO |  |
| Ericsson |  |
| Qualcomm | Proposal 2: Do not support direct reuse of pagingSearchSpace for PEI PDCCH monitoring including the case pagingSearchSpace is same as searchSpaceSetZero - It is fine to derive the PEI search space set from the pagingSearchSpace by separately determining time domain location of PEI monitoring occasion so that there is a gap between PEI monitoring occasion and PO. Observation 5: If searchSpaceSetZero is not configured as pagingSearchSpace, i.e., PEI and paging PDCCH are configured in different search space sets, it is possible for the network to configure PEI such that there is a minimum gap between the PEI location and the PO. Proposal 5: If searchSpaceZero is used for PEI PDCCH monitoring, only index 1 of Table 13-11 is supported for FR1 and only indices 1 and 6 of Table 13-12 are supported for FR2. Figure 2: SS/PBCH block and CORESET multiplexing patterns |
| MediaTek | Observation 5: Network can spare PDCCH resources for PEI from reduced number of POs. The increased PO paging rate has limited impact (<1% increment) to legacy idle/inactive UE power consumption since it is dominated by UE wake-ups before PO, and the wake-ups are irrelevant to PO paging rate. Figure 5: How network can improve idle/inactive UE power saving with less PDCCH resources for paging Observation 6: Subject to the same or less overall PDCCH resource overhead for paging, the monitoring occasions provided by pagingSearchSpace or searchSpaceSetZero may lead to inferior UE power saving gain. Proposal 9: PEI applies dedicated search space set to optimize PEI-O location for idle/inactive UE power saving - Reference to legacy pagingSearchSpace or searchSpaceSetZero is not supported |
| Nokia | Proposal: Support also searchSpaceSetZero for PEI monitoring. |
| Nordic | Proposal-2: peiSearchSpace can be zero or non-zero (i.e. one of up to 4 common SS sets configured by commonSearchSpaceList). |

From the above table, the following statistics can be obtained:

- Support searchSpaceSetZero and pagingSearchSpace with ID = 0:

  - Yes (11 companies; 3 of them suggest conditional support): HW & HiSi (if after SS burst), CATT, Intel, Xiaomi, Panasonic, Samsung, Apple (pattern 1 with O≠0), InterDigital, QC (some selected patterns), Nokia, Nordic

  - No (4 companies): vivo, Spreadtrum, Transsion, MTK,

- Support reference to other legacy common search space configuration with ID > 0:

  - Yes (9 companies; 1 of them suggest conditional support): HW & HiSi (if after SS burst), vivo, CATT, Intel, Xiaomi, Transsion, Panasonic, Samsung, Nordic

  - No (4 Companies): Spreadtrum, InterDigital, QC, MTK,

From the statistics, it looks restricting to some patterns for SS#0 can give good consensus and potentially address the concern of objecting companies. In this regard, moderator would like to check the following question first:

Q1: Please provides your views and/or suggested revision on the following Proposal 4.1-1

Proposal 4.1-1:

If searchSpaceZero is used for PEI PDCCH monitoring, only index 1 of Table 13-11 is supported for FR1 and only indices 1 and 6 of Table 13-12 are supported for FR2.

For reference to other legacy common search space set configuration, the main concern from the opponents is whether the configurations can be useful for power saving. To address opponent companies’ concern, moderator suggests to add a note for Proposal 4.1-2:

Q2: Please provides your views and/or suggested revision on the following Proposal 4.1-2

Proposal 4.1-2:

peiSearchSpace can reference to one of up to 4 common SS sets configured by commonSearchSpaceList.

Note: Reference legacy common SS set configuration may not necessarily optimize UE power saving

Table 8: Companies’ views to the above question

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Q1: We can understand the intention of Proposal 4.1-1. But we believe whether to apply certain searchspaceZero for PEI should leave to gNB’s judge(whether to prioritize resource efficiency over power saving ). If the nearest SSB is a little far away from PEI, UE may have to wake up to synchronize on SSB first, and then fall into sleep for a short period and wake up again to monitor PEI, which would cost some power on the two state transition periods. but as a whole, there would still be power saving gain in this case. And Type #0 CSS, which is composed by search space 0 and CORESET 0, is a default and very important CSS, all the search space in RRC idle mode, Type #0A/#1/#2 CSS, if not configured, can all use Type #0 CSS by default. So gNB may very possible only configure Type #0 CSS in idle mode to save resource overhead, and CSS for PEI should also be able to reuse Type #0 CSS. From our understanding, it should be left to gNB implementation to decide whether to configure Type #0 CSS(which includes search space 0) for PEI for the purpose of saving resource overhead at the cost of some extra power. And gNB can always avoid such search space 0 mentioned above configured as PEI search space if gNB decide to save the power of the two state transition periods. Q2: Support the proposal. |
| MediaTek | For Q1: We are not supportive of Proposal 4.1-1 since O value of FR1 should at least be restricted. But if the restriction cause undesired specification effort, we suggest not to support SS#0 for simplicity. For Q2: We can support Proposal 4.1-2 given SS#0 is excluded. The following is the suggested revision: Proposal 4.1-2 (updated): peiSearchSpace can reference to one of up to 4 common SS sets configured by commonSearchSpaceList and with SearchSpaceId > 0. Note: Referencing legacy common SS set configuration may not necessarily optimize UE power saving |
| Nordic | Q1: We do not really understand why restrictions are needed, but would be fine also not to support SS#0 Q2: Support |
| Samsung | Q1: We are not clear how the restriction helps to resolve any issue. We don’t see the need to restrict gNB’s flexibility to reuse SS#0. Compared with the proposed restriction, we prefer to not support SS#0 as a compromise. Q2:OK. |
| Panasonic | On Q1, we understand the motivation that the PEI PDCCH monitoring occasions are desired to be later than the location of SSB. However, we do not think such restriction is necessary. As UE may wake up to receive multiple SSBs depending on the channel condition and different implementations, the benefit is marginal. On Q2, we are okay with proposal 4.1-2. |
| CATT | Q1: We can NOT agree to have any restriction when SearchSpaceZero is used. Q2: We are OK with the proposal. |
| Intel | Q1: Do not support such restriction. If there is MO before SSB, that can still be used by buffering at the UE. Moreover, some companies are assuming one SSB maybe sufficient for PEI detection regardless of SINR conditions. We do not agree with this statement, since UE may need to prepare for worst case in terms of #SSBs processing before PEI DCI. Q2: Support main bullet. We do not see the need for the note |
| Qualcomm | Q1: we support the proposal 4.1-1 for multiplexing pattern 1 with the limited indices. We support multiplexing pattern 2 and 3 for FR2 unconditionally. Q2: the note is confusing. The purpose of PEI is to optimize the power saving gain, then network should do its best to configure such a SS set for PEI monitoring. Besides, pagingSearchSpace should be precluded because it assumes one PEI to one PO mapping and provides minimal power saving gain. There has been an agreement that sub-grouping indication is only carried by PEI but not paging PDCCH. Then receiving PEI at the same location as paging PDCCH should be precluded too as it also causes potential collision with the paging PDCCH. |
| Sharp | Q1: we cannot see the benefits for the restriction Q2: OK |
| CMCC | Q1: Don’t support to adding such kind of restriction. Q2: Ok |
| ZTE, Sanechips | (1) We don't think the restriction in search space zero is needed. In addition to search space 0, we think there is no harm to reuse paging search space set. Note it does not imply the PEI is overlapping with PO, since the PO is also determined by the location of paging frame, the configuration of first MO for each PO. (2)According to the observation in Rel-16 UE PS TR, the PS gain from reduction of blind detection is minimal, hence, we think the note is proposal 4.1-2 is not needed. Besides, we think it is better to replace “reference” with “be configured” to make it clear. |
| LGE | Q1: We do not think such a restriction would be required. It should be gNB choice whether and how to configure PEI-O. If configured PEI-O is not preferred for a UE side, UE can skip the PEI and monitors a PO directly. We do prefer to support search space set zero without any restriction. Q2: We are fine with proposal 4.1-2, but we may not need the note. |
| Ericsson1 | Q1: No – such restrictions are not needed. In fact, dedicated PEI search space that was agreed is enough - there is sufficient flexibility for NW to place the PEI SS at desired locations. Q2: Such CSS set can be used for PEI. The same CSS set for PEI and other RNTIs (P/SI/RA) can be reused if they have the same payload size i.e. PEI size of 41 bits. |
| Huawei, HiSilicon | Q1: we think proposal 4.1-1 seems not enough. Our preference is PEI monitoring occasion is close and after the SS burst, considering UE needs time to process SS burst for Time/frequency retuning. We are fine to agree SS#0 supported with the above restriction. Q2: we don’t agree but can be fine if we do some restriction for SS#0. |
| vivo | Q1: In addition to the concern raised by Huawei, in some configurations for searchSpaceSetZero as defined in clause 13 in TS 38.213 [4], there will be some overlaps in time domain between multiple PDCCH MOs corresponding to different SSB as illustrated in Figure 2 (assuming the SCS for both SSB and CORESET#0 is 30KHz). As such, UE will be forced to blindly select one QCL assumption to do PEI detection. Due to this ambiguity, the MDR of PEI will be impacted and may not be guaranteed, however, this goes against with our previous agreements for the MDR of PEI (i.e., the MDR of PEI should be much lower than that of paging PDCCH) endorsed in RAN1 #104e [2]. Figure 2: The overlapping in time domain between multiple PDCCH MOs. Given the facts aforementioned, some proper restrictions for reusing searchSpaceSetZero are needed aiming to address these concerns. However, these restrictions for reusing searchspacezero including Proposal 4.1-1 will make things become very complicated. At this very late stage, we suggest to not support to reuse searchSpaceSetZero as PEI search space. Q2: The intention of the Note in proposal 4.1-2 conflicts with that of the main-bullet. From our opinions, we agree that reusing common SS sets except for pagingsearchspace cannot optimize UE power saving. However, rregarding pagingSearchSpace, in order to reduce resource overhead in some cases, it could be desirable to reuse it as PEI search space as long as pagingSearchSpace ID>0, considering the similar characteristics of beam operations and monitoring manner for paging PDCCH and PEI. |
| Apple | Q1: we are supportive of introducing some restrictions for using searchSpaceZero for PEI. However, the reason for the proposed restriction is not clear to us. Q2: OK. |
| OPPO | Q1: We don't think the restriction in search space zero is needed. Q2: We are OK with the proposal. |
| Nokia(1st round) | Q1: We don’t support these restrictions. We think it is best that we either agree to support the Type0-PDCCH CSS unconditionally or preclude it, and would have preference to the former. Q2: We would be OK with the proposal (wording changes depending on the SS#0 support conclusion). We don’t see that using pagingSearchSpace would imply one to one mapping for PEI and PO. If the PEI-O offset from PEI-F point to same monitoring occasions, the PEI could be shared between POs. |
| IDCC | We think gNB should be able to configure searchspacezero. Restriction is not needed, gNB can configure accordingly. But if majority of companies support the restriction, we are ok with it. |

For Q1, as quoted below for ease of reference,

Proposal 4.1-1: If searchSpaceZero is used for PEI PDCCH monitoring, only index 1 of Table 13-11 is supported for FR1 and only indices 1 and 6 of Table 13-12 are supported for FR2.

- Yes: QC (FR2 can be unconditional),

- No:

  - Further restriction needed: MTK, HW & HiSi, Apple, vivo (no SS#0)

  - No need of restriction: Xiaomi, Nordic (also fine with no SS#0), Samsung (also fine with no SS#0), Panasonic, CATT, Intel, Sharp, CMCC, ZTE, LGE, Ericsson,

For Q2, as quoted below for ease of reference,

Proposal 4.1-2: peiSearchSpace can reference to one of up to 4 common SS sets configured by commonSearchSpaceList. Note: Reference legacy common SS set configuration may not necessarily optimize UE power saving

- Yes: Xiaomi, Nordic, Samsung, Panasonic, CATT, Intel (w/o note), Sharp, CMCC, ZTE (w/o note), LGE (w/o note), Ericsson,

- No: QC, MTK (unless SearchSpaceId > 0), HW & HiSi (unless SearchSpaceId > 0), vivo (unless SearchSpaceId > 0)

## Other Dedicated Specification for ‘peiSearchSpace’

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon |  |
| ZTE, Sanechips | - Proposal 7: The aggregation levels and candidates of Type0/0A/2-PDCCH CSS can be reused for PDCCH-based PEI. |
| vivo | Proposal 6: Support to keep the same AL configuration of PEI search space with that of legacy CSS set. |
| TCL | Proposal 3: The AL of PDCCH based PEI is configurable. |
| Spreadtrum |  |
| CATT |  |
| OPPO |  |
| Sony |  |
| Intel | Proposal 8: CCE ALs and numbers of candidates for PDCCH CSS set for PEI monitoring are as follows: CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. |
| Xiaomi |  |
| CMCC |  |
| Transsion Holdings |  |
| Panasonic |  |
| Samsung |  |
| Apple | Proposal 8: The maximum number of PDCCH candidates for PEI is 4, 2 and 1 for AL 4, 8 and 16, respectively. |
| Lenovo, Motorola Mobility |  |
| InterDigital |  |
| LGE |  |
| NTT DOCOMO |  |
| Ericsson |  |
| Qualcomm |  |
| MediaTek | Observation 7: PEI-O is likely dedicated for PEI use since - PEI transmission should be of high priority to secure paging performance which will impact PDCCH of legacy UE multiplexed in PEI-O - If PEI is not transmitted, network can reuse the resource for PDSCH of legacy UEs by leveraging dynamic CORESET-wise rate-matching over PEI-O resources Proposal 10: For PEI monitoring, up to 2 AL candidate values is supported |
| Nokia |  |
| Nordic |  |

From the above table, the following statistics can be obtained:

- The aggregation levels and candidates of Type0/0A/2-PDCCH CSS can be reused for PDCCH-based PEI.

- Yes (4 Companies): ZTE, vivo, Intel, Apple,

- No (2 companies): TCL (configurable), MTK (up to 2 ALs)

From the statistics, companies tend to reuse current setting for Type0/0A/2-PDCCH CSS. On the other hand, for network deployment where PEI utilizes dedicated occasions (not sharing with legacy PDCCH), it is reasonable to allow UE to perform less blind decoding. In this regard, moderator would like to check if the following proposal can work for all companies:

Q1: Please provides your views and/or suggested revision on the following Proposal 4.2-1

Proposal 4.2-1:

The aggregation levels and candidates of Type0/0A/2-PDCCH CSS are the default configuration for PEI monitoring. When dedicated ‘peiSearchSpace’ setting is applied, a subset of the aggregation levels and candidates can be optionally configured.

Table 9: Companies’ views to the above question

| Company | Companies’ views |
| --- | --- |
| Xiaomi | Fine with the Proposal. |
| MediaTek | We can accept Proposal 4.2-1 since network can avoid unnecessary UE blind decoding complexity when PEI-O is not shared with legacy PDCCH (but can be released for PDSCH of legacy UEs via dynamic CORESET-wise rate-matching) |
| Nordic | Q1: Agree |
| Samsung | Q1: The configuration of CCE AL should be up to gNB implementation. The don’t support the restriction to limit gNB’s flexibility. |
| Panasonic | We do not support this proposal and think it is good to keep the AL options more flexible for network to control the performance and overhead, |
| CATT | Q1: We don’t see the need and can NOT agree to have subset of ALs for peiSearchSpace in the specification, which increase the signaling overhead without any benefit. |
| Intel | Support as follows, without the second sentence. The aggregation levels and candidates of Type0/0A/2-PDCCH CSS are also adopted for the default configuration for PEI monitoring. When dedicated ‘peiSearchSpace’ setting is applied, a subset of the aggregation levels and candidates can be optionally configured. |
| Qualcomm | Fine with the proposal, but does not agree with the “subset” of AL and candidates part. |
| CMCC | Ok |
| ZTE, Sanechips | As we commented before, according to the observation in Rel-16 UE PS TR, the PS gain from reduction of blind detection is minimal. Reusing the AL and candidates of Type0/0A/2-PDCCH CSS provides more flexibility. There is no need to further restrict the number of candidates. Intel’s revision looks okay to us. |
| LGE | We prefer to support using ALs and candidates of Type0/0A/2-PDCCH CSS and support Intel’s modified version. We do not see strong need for additional configuration. |
| Ericsson1 | Configuration of BD/AL should be part of the PEI SS configuration. If default (from Type0/0A/2-PDCCH CSS) is referring to the BD/CCE table 10.1-1, perhaps that can be explicitly captured in the proposal. |
| Huawei, HiSilicon | The configuration of CCE AL should be up to gNB implementation. We agree with Samsung on this. |
| vivo | We are fine with the Intel’s modification version of proposal 4.2-1. |
| Apple | The proposal is not clear to us. If peiSearchSpace points to a SS also used for other purposes (e.g., Type0/0A/2-PDCCH CSS), the question is whether we directly reuse the BD/AL configuration for that SS, or we put some upper limitation on what UE needs to decode for PEI. If peiSearchSpace points to a SS not used for any other purpose (i.e., dedicated for PEI), the actual BD/AL is up to gNB configuration, and the question is also the upper limit. So for the two cases, we think the issue is the same, i.e., defining the max limit of BDs for each AL. |
| OPPO | Support the proposal. |
| TCL | The configuration of CCE AL should be up to gNB implementation. We agree with Samsung and Huawei |
| Nokia(1st round) | To clarify, is the intention that AL and candidate numbers according to Table 10.1-1 in 38.213 would be supported, but leave the option for the network to further limit to sub-set of these? We are fine to align the AL/candidates with Type0/0A/2, but don’t see significant benefit for further restrictions. |
| IDCC | We are ok with this proposal. |

For Q1, quoted below for ease of reference,

Proposal 4.2-1: The aggregation levels and candidates of Type0/0A/2-PDCCH CSS are the default configuration for PEI monitoring. When dedicated ‘peiSearchSpace’ setting is applied, a subset of the aggregation levels and candidates can be optionally configured.

- Yes: Xiaomi, MTK, Nordic, Intel (w/o subset restriction), QC (w/o subset restriction), CMCC, ZTE (w/o subset restriction), LGE (w/o subset restriction), vivo (w/o subset restriction),

- No: Samsung (SS configuration), CATT (no subset restriction), Ericsson (SS configuration), HW & HiSi (SS configuration), Apple (defining the max limit of BDs for each AL?)

# Other Remaining Issues

| Company | Companies’ Views |
| --- | --- |
| Huawei, HiSilicon | Proposal 11: The agreements and progress in RedCap need to be carefully considered in PEI discussion to ensure PEI utilization on RedCap UE, as required by RedCap WID. |
| ZTE, Sanechips |  |
| vivo |  |
| TCL |  |
| Spreadtrum |  |
| CATT |  |
| OPPO |  |
| Sony |  |
| Intel |  |
| Xiaomi |  |
| CMCC |  |
| Transsion Holdings |  |
| Panasonic |  |
| Samsung |  |
| Apple | Proposal 9: Support separate PO configurations for UEs supporting sub-grouping and UEs not supporting sub-grouping. |
| Lenovo, Motorola Mobility | Proposal 1: Support a PEI search space configuration in an RRC release message for an inactive mode UE. |
| InterDigital |  |
| LGE | Proposal 9: The UE assumes that the same information on UE group/subgroup indication, SI change indication and ETWS/CMAS notification are repeated in all transmitted beams in a PEI occasion. Proposal 10: If the PEI convey availability indication only for the TRS/CSI-RS occasion(s) in the same beam direction, the UE assumes that the information on the TRS availability indication can be different in each transmitted beam in a PEI occasion. If the PO configuration at the separate initial DL BWP is supported, how to configure the PEI occasion for the RedCap UEs should be discussed as well. |
| NTT DOCOMO |  |
| Ericsson | - Proposal 14 If PEI is supported/used by the cell, its configuration is provided via broadcast system information. The configuration is applicable to all POs of the cell for RRC Idle/Inactive UEs and may be updated through existing SI update mechanisms. - Proposal 18 In case of collision with other activity, rendering the UE unable to perform PEI decoding (for example when entering the cell in-between the PEI and PO occasion), the UE shall for that PO fall back to regular PO monitoring as per legacy procedures. - Proposal 19 RAN1 to discuss other scenarios (e.g., fluctuating channel conditions) for which the UE falls back to regular PO reception as per legacy procedures. |
| Qualcomm | Proposal: To avoid UE blind detection of the PEI transmit power, network should configure the transmit power of the PEI or UE can assume a fixed power offset between the PEI and another signal, e.g., SSS on the same beam, paging PDCCH on the same beam. |
| MediaTek |  |
| Nokia | Proposal: RAN1 discuss configuration of more than one PEI per PO to accommodate different device preferences. |
| Nordic | Proposal-4: Consider introducing wide-band PDCCH DMRS transmitted in an entire CORESET configured by SIB1 or MIB during PEI monitoring occasions to facilitate sequence-based detection of PEI presence and/or to facilitate PDCCH DMRS for consequent finer-synchronization. - narrow-band precoding can be assumed in CORESET#0, i.e. no change from R15/R16. Regarding RedCap issues, we believe that Redcap AI still needs to conclude first on where UE camps (which initial DL BWP/CORESET#0) in IDLE, depending on that we may need to revisit. For example, if RedCap UE is allowed to camp only on CORESET#0 by MIB, there should be no issues. Regarding whether and how to support separate PO configuration, we believe it would be beneficial e.g. in case of PO determination would be different for R17 UEs supporting iTRS, as proposed in Proposal-3. Transmit power of PEI should be the same as for other CSS PDCCH. |

# Summary

While the statistics are still being updated per latest companies’ inputs, the following proposals are discussed in email thread “[107-e-NR-R17-PowSav-01]” and will be suggested for online discussion:

The first proposal is related to unlicensed operation and is analogous to the description for PO in TS 38.304. In particular the sub-bullet is analogous to the following description:

TS38.304 (R16) “If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.”

Proposal 3.2-1 (r2):

A PEI-O is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for PEI in the PEI-O corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S.

- If X > 1, when the UE detects a PEI within its PEI-O, the UE is not required to monitor the subsequent PEI-O(s) associated with the same PO

The second proposal is related to paging indication field structure and the corresponding PO monitoring behaviour. It is related to Section 2.3:

Proposal 2.3-1 (r2)

- When UE is provided a subgroup index, iSG, 0≤iSG<subgroupsNumPerPO, UE checks the corresponding paging indication from iPO×subgroupsNumPerPO+iSG+1-th bit of the paging indication field.

  - If one PEI is mapped to 1/2/4 POs associated with a PF, iPO=is mod (#PO(s) mapped to the PEI), where is is the PO index derived from PO determination as per TS 38.304

  - FFS the determination of iPO if one PEI is agreed to be associated with multiple PFs

- UE is required to monitor PO only if the corresponding paging indication value is set to ‘1’

- FFS: whether and how to support subgroupsNumPerPO * POnumber within a SS burst period can be larger than the length of paging indication field in the PEI DCI

# Reference

- RP-212611, “Moderator’s summary for discussion [93e-18-PowSav-WI]: Final Round”, Moderator (CMCC), RAN#93-e

- “Draft Report of 3GPP TSG RAN WG1 #106b-e”, RAN1#106-bis-e meeting, online available@ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_106b-e/Report

- RP-211452, “Status report for WI UE Power Saving Enhancements for NR”, Rapporteur (MediaTek), RAN#92-e

- R1-2110671, “Summary #5 of paging enhancements”, Moderator (MediaTek), RAN1#106-bis-e

- R1-2110837, “Paging enhancements for UE power saving in IDLE/inactive mode”, Huawei, HiSilicon

- R1-2110839, “Analysis on power consumption for IDLE mode UE	”, Huawei, HiSilicon

- R1-2110937, “Discussion on power saving enhancements for paging”, ZTE, Sanechips

- R1-2110939, “Additional simulation results of UE power consumption in RRC idle and inactive state”, ZTE, Sanechips

- R1-2111023, “Remaining issues on paging enhancements for idle/inactive mode UE power saving”, vivo

- R1-2111025, “Discussion on paging grouping”, vivo

- R1-2111062, “Paging enhancements for idle/inactive UE power saving ”, TCL Communication Ltd.

- R1-2111103, “Discussion on potential paging enhancements for UE power saving”, Spreadtrum

- R1-2111266, “Paging enhancement for UE power saving”, CATT

- R1-2111268, “Multiple POs associated with one PEI”, CATT

- R1-2111325, “Further discussion on Paging enhancements for power saving”, OPPO

- R1-2111404, “Remaining issues on potential paging enhancement”, Sony

- R1-2111504, “On remaining aspects of paging early indication design”, Intel Corporation

- R1-2111582, “Paging enhancement for power saving”, Xiaomi

- R1-2111617, “Discussion on paging early indication design”, CMCC

- R1-2111674, “Discussion on PEI Design”, Transsion Holdings

- R1-2111675, “On paging enhancement”, Panasonic

- R1-2111747, “Discussion on paging enhancements”, Samsung

- R1-2111884, “Paging enhancements for idle/inactive-mode UE”, Apple

- R1-2111945, “Paging enhancement for UE power saving”, Lenovo, Motorola Mobility

- R1-2111960, “Discussion on paging enhancements for UE power saving”, InterDigital, Inc.

- R1-2112060, “Discussion on potential paging enhancements”, LG Electronics

- R1-2112116, “Discussion on paging enhancement”, NTT DOCOMO, INC.

- R1-2112149, “Design of Paging Enhancements”, Ericsson

- R1-2112226, “Paging enhancements for idle/inactive UE power saving”, Qualcomm

- R1-2112228, “Aligning Paging Occasions to SSB for UE idle and inactive mode power saving”, Qualcomm

- R1-2112308, “Remaining design details of paging early indication”, MediaTek Inc.

- R1-2112370, “Open items on paging enhancements for UE power saving”, Nokia, Nokia Shanghai Bell

- R1-2112379, “On  paging early indication”, Nordic Semiconductor ASA