3GPP TSG RAN WG1#107bis-e    		                                         R1-2200278

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.7.2

Source:	Spreadtrum Communications

Title:	Discussion on power saving techniques for connected-mode UE

Document for:	Discussion and decision

# Introduction

In the past meetings, power saving techniques for connected-mode UE and the corresponding signaling design were discussed. In this contribution, we discuss the power saving techniques and the corresponding signaling design.

# Power saving techniques

It is common understanding that there are mainly two power saving techniques for PDCCH monitoring reduction in connected mode, such as SSSG switching and PDCCH skipping for a duration.

It can be observed that these two techniques are similar. In our previous evaluation [1], these two techniques have the similar power saving gain. From the agreement in RAN1#104, these two techniques should be both supported.

## Rel-16 SSSG switching technique for unlicensed operation

### Wakeup indication and fallback timer

In Rel-16 SSSG switching technique for unlicensed operation, DCI format 2-0 can be used for triggering switching from “dense” SSSG outside COT to “sparse” SSSG inside COT. For explicit triggering, there is 1 toggle bit for SSSG switching in DCI format 2-0.

It should be noted that DCI format 2-0 is very important for Rel-16 unlicensed operation, since COT information, frequency location information, guard-band information and SSSG switching information are all conveyed by DCI format 2-0. If DCI format 2-0 is missed by UE, the whole COT will be out of order. Therefore, gNB should guarantee the reliability of DCI format 2-0 in Rel-16 unlicensed operation.

To mitigate the impact of miss-detection of DCI triggering switching from the “dense” SSSG to the “sparse” SSSG, the fallback timer is specified.

### State machine

Rel-16 SSSG switching technique for unlicensed operation has an “elegant” state machine. There are two states: the first one is the monitoring of the “dense” SSSG outside COT, and the second one is the monitoring of the “sparse” SSSG inside COT. For explicit triggering, the 1 bit in DCI format 2-0 can trigger transition between the two states. For implicit triggering, detection of DCI in the “dense” SSSG can trigger transition from the first state to the second state. In our view, gNB should guarantee the reliability of triggering PDCCH, otherwise, the whole state machine will be out of order.

Furthermore, in CA case, such state machine will operate in each activated SCell. It will complicate the UE implementation. However, it was not be addressed in R16 NR-U, since CA is not so necessary in sub-6GHz unlicensed band (UE can using 100MHz BWP to achieve the target data rate).

### Application delay

The application delay is defined in Rel-16 SSSG switching technique for unlicensed operation. In our view, at least (may include bandwith/guard-band adaptation time), the pipeline switching is assumed in Rel-16 SSSG switching technique for unlicensed operation.

## Rel-17 SSSG switching technique for power saving

For SSSG switching, power saving gain comes from the switching from the dense PDCCH monitoring occasions in an SSSG to the sparse PDCCH monitoring occasions in another SSSG. The “dense” and the “sparse” SSSG can be called as “normal” SSSG and “dormant” SSSG.

### Wakeup indication and fallback timer

Wakeup indication and fallback timer are similar as that of Rel-16 SSSG switching technique. As the conclusion of Rel-16 power saving study item (in RAN1_AH_1901), the wakeup indication should have the target miss detection rate equal to 0.1%.

Agreements: The performance evaluation of the power saving signal/channel should target the miss detection at X% and the false alarm rate at Y% with the following aspects identified for the proposed power saving signal/channel - The target of miss detection X% and the false alarm rate at Y% as baseline for evaluation - For power saving signal/channel for wake-up purpose, X=[0.1] and Y=[1] - For power saving signal/channel for go-to-sleep purpose, X=[1] and Y=[0.1]

However, unfortunately, there was no discussion on the miss detection rate of the wakeup indication for Rel-16 SSSG switching technique in NR-U discussion. In the context of NR-U discussion, COT-related information is more important than power saving function.

To avoid the out-of-control due to low reliability- of L1 signaling, the wakeup indication for Rel-17 SSSG switching technique should be assisted by the fallback timer.

### State machine

It is common understanding that the target miss detection rate for wakeup indication is 0.1%. If the target miss detection rate is far to be met by L1 signaling, the power saving gain will be zero (even negative) or scheduling delay will be too large.

Furthermore, if gNB and UE do not align in their own state machine due to miss detection and false alarm, UE may have the similar experience of radio link failure, since a series of PDCCH is missed. It may not be critical for small cell (e.g. in NR-U), but definitely critical for macro cell in term of user experience.

Moreover, in CA case, the state machine will operate in each activated SCell. It will complicate the UE implementation, since SCell dormancy mechanism may launch a state machine already.

Therefore, Rel-17 SSSG switching should not rely on the state machine with L1 signaling.

### Application delay

For SSSG switching, if the full-blown SSSG switching is supported like Rel-16 SSSG switching, UE should switch the pipeline dynamically, and the application delay is needed for the pipeline switching. In Rel-17 power saving, the “dormant” SSSG still has almost full flexibility (fully depends on gNB configuration). So, the application delay could be that defined in Rel-17 SSSG switching.

Proposal 1: The application delay for Rel-17 SSSG switching technique can be defined.

### Rel-16 SSSG switching is not compatible to Rel-17 SSSG switching

We would like to re-emphasize that Rel-16 SSSG switching is not compatible to Rel-17 SSSG switching.

Design purpose

For unlicensed operation, the purpose of SSSG switching in Rel-16 NR-U is to adapt different PDCCH monitoring characteristics inside COT and outside COT. Outside COT, UE can monitor the mini-slot level PDCCH to avoid missing scheduling or COT information when gNB gains the channel. Inside COT, UE can monitor the slot level PDCCH to reduce PDCCH monitoring. However, the purpose of SSSG switching for Rel-17 power saving is to control UE to monitor the “dormant” SSSG when packet is not arriving.

Observation 1: The purpose of Rel-16 SSSG switching is different from that of Rel-17 SSSG switching.

Explicit triggering

For explicit triggering, DCI format 2-0 is used for unlicensed operation. DCI format 2-0 may not be suitable for licensed operation, since the triggering bit in DCI format 2-0 is only used for unlicensed operation together with other bits like bits informing COT structure. It is general view that DCI format 2-0 is used for SFI in licensed operation, and gNB should transmit DCI format 2-0 only if SFI is changed. Moreover, the UE-specific like indication should be addressed in group-common DCI to support indicating a specific UE, like DCI format 2-6.

Observation 2: For explicit triggering, DCI format 2-0 may not be suitable for Rel-17 SSSG switching.

Implicit triggering

Rel-16 SSSG switching for NR-U has defined the implicit triggering, but Rel-17 SSSG switching has excluded the implicit triggering.

Observation 3: Implicit indication has been excluded from Rel-17 SSSG switching.

## Rel-17 PDCCH skipping technique for power saving

For Rel-17 PDCCH skipping technique, power saving gain comes from skipping PDCCH monitoring occasions for a duration.

### Wakeup indication and fallback timer

There is no wakeup indication in Rel-17 PDCCH skipping technique, and fallback time is necessary.

### State machine

Rel-17 PDCCH skipping technique has no state machine.

### Application delay

For Rel-17 PDCCH skipping technique, the application delay can be zero, since there is no pipeline switching at UE side.

Proposal 2: The application delay for Rel-17 PDCCH skipping technique is zero.

## Some thoughts on the state machine

In Rel-16, RAN2 had discussed PDCCH skipping and summarized in [2]. As summary, RAN2 had consensus that PDCCH skipping has smaller time scale as DRX operations and PDCCH skipping is independent of MAC operations.

Observation 1: DCI-based PDCCH skipping and DRX operate on different time scales. Observation 2: DCI-based PDCCH skipping and DRX have different functionality. Observation 3: A UE is expected to operate in different power states when in DRX-OFF and when executing a DCI-based PDCCH skipping, however this is not RAN2 expertise and also depends on UE implementation. Observation 4: DCI-based PDCCH skipping does not duplicate the DRX functionality. Proposal 1: DCI-based PDCCH skipping is supported in Rel-16. Proposal 2: DCI-based PDCCH skipping has no effect on the drx-onDurationTimer. Proposal 3: DCI-based PDCCH skipping has no effect on the drx-InactivityTimer. Proposal 4: DCI-based PDCCH skipping has no effect on the drx-RetransmissionTimerUL. Proposal 5: DCI-based PDCCH skipping has no impact on the drx-RetransmissionTimerDL. Proposal 6: DCI-based PDCCH skipping has no impact on the ra-ContentionResolutionTimer. Proposal 7: DCI-based PDCCH skipping does not trigger the MAC entity to exit DRX Active Time during an SR procedure. Proposal 8: DCI-based PDCCH skipping has no impact on MAC. Proposal 9: The UE applies the PDCCH skipping during DRX Active Time irrespective of the event that triggered the Active Time. Proposal 10: MAC has no impact on DCI-based PDCCH skipping.

In other words, PDCCH skipping is pure PHY control, which is not so reliable. It was stated in LTE TS 36.300. As suggested in 36.300, the state machine should be performed by RRC.

Table B.1-1: Summary of the difference between MAC and RRC control MAC control; RRC control. Control entity; MAC; RRC. Signalling; PDCCH; MAC control PDU; RRC message. Signalling reliability; ~ 10-2 (no retransmission); ~ 10-3 (after HARQ); ~ 10-6 (after ARQ). Control delay; Very short; Short; Longer. Extensibility; None or very limited; Limited; High. Security; No integrity protection No ciphering; No integrity protection No ciphering; Integrity protection Ciphering. The main difference between MAC and RRC control lies in the signalling reliability. Due to the signalling reliability, signalling involving state transitions and radio bearer configurations should be performed by RRC. Basically, all signalling performed by RRC in UTRA should also be performed by RRC also for E-UTRA.

Therefore, the state machine for PHY layer control should be simple in general design.

In NR, only BWP switching and Rel-16 SSSG switching for NR-U has the state machine for PHY layer control. For BWP switching, it has large time scale, even larger than DRX operations, and it can be protected by HARQ feedback. For Rel-16 SSSG switching for NR-U, we think it is suitable for small cell and DCI format 2-0 can be configured reliably. Further, Rel-16 SSSG switching for NR-U is not commercialized in real world. Hence, BWP switching and Rel-16 SSSG switching for NR-U cannot be taken as a reference for the state machine.

## Comparison

We compare the above three techniques in the following table.

Table 1: Comparison of the above three techniques

|  | Rel-16 SSSG switching technique for unlicensed operation | Rel-17 SSSG switching technique for power saving | Rel-17 PDCCH skipping technique for power saving |
| --- | --- | --- | --- |
| Wakeup indication and fallback timer | Explicit triggering: 1 bit wakeup indication; Fallback timer | The wakeup indication should be assisted by the fallback timer; Fallback timer | No wakeup indication; Fallback timer |
| State machine | State machine with two states | Rel-17 SSSG switching should not rely on the state machine with L1 signaling | No state machine |
| Application delay | Non zero (at least for pipeline switching) | Non zero (for pipeline switching) | Zero |

# Signaling design

## Striving common design of signaling

It was agreed in RAN1#104e [3] to strive for a common design to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration.

Agreements: - Strive for a common design for DCI based PDCCH monitoring adaptation in active time for an active BWP to support functionalities inclusive of both SSSG switching and PDCCH skipping for a duration. - Details FFS

Then, it was agreed in RAN1#104e [3] that two alternatives of signalling design can be considered.

Agreements: - The following alternatives can be considered for DCI based PDCCH monitoring adaptation in active time for an active BWP for power saving - Alt 1: Enhancement of Rel-16 SSSG switching to support PDCCH monitoring adaptation including skipping for a duration - Alt 2a: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and SSSG switching - Alt 2b: Enhancement of DCI(s) utilized for Rel-16 power saving adaptation for supporting both skipping PDCCH monitoring for a duration and PDCCH monitoring periodicity adaptation - Others not precluded

It should be noted that both Alt 1 and Alt 2a can support both PDCCH skipping for a duration and SSSG switching.

In RAN1#105e, it seems that companies cannot achieve a common design of signalling, and cannot achieve a consensus for down selection from Alt 1 and Alt 2a.

## Two understandings of common design

The first understanding

Some companies think specifying one RRC configuration is common design. For example, if one RRC configuration within SSSG switching framework is specified, the “dormant” SSSG (or “empty” SSSG) should be defined to emulate PDCCH skipping. An example for the first understanding can be described in details:

- RRC: Configure {the “dormant” SSSG, the “normal” SSSG}

- DCI: Trigger SSSG#0 or SSSG#1

The second understanding

In our view, a set of two RRC configurations is also common design. gNB can choose one of two RRC configuration for a given UE. In this sense, RAN1 can still design two frameworks (SSSG switching and PDCCH skipping) in parallel, which correspond to two separate UE capabilities. An example for the second understanding can be shown as follows:

- RRC:

  - Configure { the “dormant” SSSG, the “normal” SSSG }, if UE supports Rel-17 SSSG switching, or

  - Configure {no PDCCH skipping, PDCCH skipping for a duration}, if UE supports Rel-17 PDCCH skipping

- DCI:

  - Trigger the “dormant” SSSG or the “normal” SSSG, if { the “dormant” SSSG, the “normal” SSSG } is configured

  - Trigger non PDCCH skipping or PDCCH skipping for a duration, if {no PDCCH skipping, PDCCH skipping for a duration} is configured

From RAN1#105e discussion and conclusion, the above two understandings are both applicable. In our view, the second understanding can give more flexible signalling design. Otherwise, we have to let the SSSG switching emulate PDCCH skipping.

## Scheduling DCI

In RAN1#105e [4], it was agreed that PDCCH skipping technique can be supported by the scheduling DCI (DCI format x-1 and DCI format x-2).

Agreement: - PDCCH schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping for a duration is supported. - At least DCI format(s) 1-1, 0-1, 1-2 and 0-2 can be used for the indication(s)

In RAN1#106e [5], it was agreed that at most 2 bits indication in self-scheduling DCI can be specified.

Agreement - At most 2 bit indication in self-scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) can be specified for triggering the PDCCH monitoring adaptation in a single cell - FFS: the bit size of the indication is configurable - FFS: bit mapping to the PDCCH monitoring behaviour - FFS: details of indication of multiple cells case

In RAN1#107-e [6], the above agreement was update to address the new indication field in scheduling DCI.

Agreement The following agreement in RAN1#106-E is updated as follows, Agreement(extracted from RAN1#106-E) - At most 2 bit indication in self-scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) can be specified for triggering the PDCCH monitoring adaptation in a single cell - FFS: the bit size of the indication is configurable - FFS: bit mapping to the PDCCH monitoring behaviour - FFS: details of indication of multiple cells case - A new indication field in scheduling DCI is used for indicating PDCCH monitoring adaptation

### Supporting PDCCH skipping technique

For supporting PDCCH skipping technique, there were two alternatives of signalling design agreed in RAN1#105e [4] shown as follows.

Agreement: - At least one of Alt 1 and Alt 2 is supported, to be decided in RAN1#106, - Alt 1: Supporting SSSG switching to emulate PDCCH skipping functionality, - Alt 1-1: by an ‘empty’ SSSG which no SS set(s) is configured for the ‘empty’ SSSG, UE does not monitoring PDCCH on the ‘empty’ SSSG, - Alt1-2: by a ‘dormant SSSG’ which may have associated SS sets, and monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration is supported. - FFS details, including - e.g., joint / separate indication of SSSG switching and PDCCH skipping - Determination of the duration(s) for PDCCH skipping, e.g., - by RRC signaling, - by DCI indication - Implicitly, to the end of C-DRX active time

For Alt-1, some companies prefer to use SSSG switching signalling framework to emulate PDCCH skipping technique.

For Alt-2, some companies prefer to use “simple” or “native” PDCCH skipping signalling framework to support PDCCH skipping technique.

In our view, SSSG switching has many different aspects from PDCCH skipping. Moreover, UE capabilities for SSSG switching and PDCCH skipping are very different and needs to be separate in the spec. PDCCH skipping has lower UE complexity. There is no need to use SSSG switching signaling framework to support a separate capability (PDCCH skipping). Therefore, we support Alt-2.

In RAN1#106e discussion, companies cannot achieve consensus on Alt-1 or Alt-2. Therefore, companies agreed to separate the PDCCH monitoring behaviours for PDCCH skipping and Rel-17 SSSG switching.

Agreement Select either package 1 or package 2 Package 1 · UE behavior after receiving PDCCH indication of monitoring adaptation can be one of the followings, - Working Assumption: Beh 1: PDCCH skipping is not activated - Beh 1A: PDCCH skipping means stopping PDCCH monitoring for a duration X - FFS the possible values for X - FFS: Whether and how to support more than one skipping duration(s) - FFS: whether to continue monitoring PDCCH scrambled by C-RNTI for Type 0/1/1A/2 CSS or not - Beh 2: stop monitoring SS sets associated with SSSG#1 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#0 (legacy behaviour) - Beh 2A: stop monitoring SS sets associated with SSSG#0 and SSSG#2 (if confirmed) and monitoring of SS sets associated to SSSG#1 (legacy behaviour) - Working Assumption: Beh 2B(if confirmed): stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2 (if confirmed) · Note: The number of supported SSSG is left to UE feature discussion. · FFS: UE capability of supported UE behaviors · Indication of Beh 1A when SSSG(s) are not configured is supported. · Working assumption: Indication of Beh 1A for current SSSG when two SSSG(s) are configured is supported · FFS: Indication of Beh 1A when three SSSG(s) (if supported) are configured · Y bits is configured for scheduling DCIs (i.e., DCI format 1-1/0-1/1-2/0-2) indicating PDCCH schedules data and also PDCCH monitoring adaptation - FFS how the UE behavior(s) defined above mapping to Y bits § Note: at most Y = 2 · Working Assumption at most 3 SSSGs is supported to be configured. - FFS: whether or how SSSG can be configured to be monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - FFS: whether or how non-default SSSG to another non-default SSSG · FFS details of timer(s) for switching between SSSG(s) - UE fallbacks to default SSSG (i.e., SSSG#0) after timer expiration. - R16 timer for SSSG switching and the corresponding behavior is as baseline · FFS whether the timer(s) is configured per SSSG, or per BWP or other approaches. · FFS whether the skipping duration(s) is configured per SSSG, per BWP, or other approaches. · FFS PDCCH monitoring adaptation indicated by non-scheduling DCI · PDCCH based monitoring adaptation is limitedapplied to USS and type-3 CSS. Agreement Package 1 in above agreement is selected.

In RAN1#106bis-e [7], supporting Behaviour 1 and supporting Behaviour 1A for current SSG were confirmed.

Confirm the four working assumptions (extracted from package 1) Working assumption(extracted from package 1): Beh 1: PDCCH skipping is not activated Working assumption(extracted from package 1): Indication of Beh 1A for current SSSG when two SSSG(s) are configured is supported

Also, it was agreed that UE can be RRC configured with multiple values of skipping duration.

Agreement For Beh 1A, - The UE can be configured to be indicated by DCI a value of X slots (i.e., skipping duration) among multiple RRC configured values by scheduling DCIs indicating PDCCH schedules data - The bits for indicating PDCCH monitoring adaptation also indicating skipping duration. Details FFS

Finally, Case 1 for codepoints mapping to Behaviour 1 and Behaviour 1A were agreed.

Agreement The bit mapping of DCI indication PDCCH monitoring adaptation is as follows, - For Case 1 (i.e., PDCCH skipping), the following is supported - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=1 - ‘0’ is Beh 1 and ‘1’ is Beh 1A - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors if M=2 or 3 - ‘00’ is Beh 1 - ‘01’ is Beh 1A with skipping duration 1 - ‘10’ is Beh 1A with skipping duration 2 - ‘11’ is Beh 1A with skipping duration 3 if M=3, reserved if M=2

### Supporting SSSG switching technique

In RAN1#105e [4], it was agreed that SSSG switching technique can be supported by the switching between two SSSGs.

Agreement: At least SSSG#0 and SSSG#1 switching is supported for Rel-17 SSSG switching indicated by PDCCH scheduling data and/or timer.

The further agreements for SSS switching is omitted for page limit.

In RAN1#106bis-e [7], supporting 3 SSSGs and supporting Behaviour 2B were confirmed.

Confirm the four working assumptions (extracted from package 1) Working assumption(extracted from package 1): At most 3 SSSGs is supported to be configured for PDCCH monitoring adaptation. Working assumption(extracted from package 1): Beh 2B: stop monitoring SS sets associated with SSSG#0 and SSSG#1 and monitoring of SS sets associated to SSSG#2.

And then Case 2 and Case 3 were agreed.

Agreement The bit mapping of DCI indication PDCCH monitoring adaptation is as follows, - For Case 2 (i.e., 2 SSSG switching) , the following is supported - 1-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘0’ is Beh 2 and ‘1’ is Beh 2A - For Case 3 (i.e., 3 SSSG switching) , the following is supported - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 2B - [‘11’ is reserved]

### Supporting joint indication for PDCCH skipping and SSSG switching technique

In RAN1#106bis-e [7], the joint indication for PDCCH skipping and SSSG switching technique was agreed as Case 4.

Agreement The bit mapping of DCI indication PDCCH monitoring adaptation is as follows, - For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , the following is supported - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors, - FFS details bit mapping - FFS: For Case 5 (i.e., 3 SSSG switching and skipping) - 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 2B - ‘11’ is Beh 1A - FFS Timer behavior when Beh 1A is indicated - Note: The UE can be configured to be indicated by DCI a value of X slots (i.e., skipping duration) among M RRC configured values by scheduling DCIs indicating PDCCH schedules data - FFS whether to restrict Skipping duration to be shorter than SSSG initial timer value - FFS whether the configuration is same or different for DCI format x_1 and DCI format x_2

In RAN1#106bis-e [7], the details of Case 4 for 1 bit and 2 bits was defined, and Case 5 was excluded.

Agreement - For Case 4 (i.e., 2 SSSG switching with PDCCH skipping) , one of the following is supported - Alt a: 2-bit in scheduling DCI is supported to indicate PDCCH monitoring adaptation UE behaviors - If M = 1, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A (stay in current SSSG) - [‘11’ is reserved] - If M = 2, then - ‘00’ is Beh 2 - ‘01’ is Beh 2A - ‘10’ is Beh 1A for duration T1 (stay in current SSSG) - ‘11’ is Beh 1A for duration T2 (stay in current SSSG) - Note: for ‘00’ and ‘01’, UE performs Beh 1(i.e., does not perform PDCCH skipping ) - Note: By a single DCI indication, a UE is not expected to simultaneously perform two behaviors of SSSG switching and PDCCH skipping. - FFS SSSG switching timer behavior, and timer value related to the value of T1/T2

## Non-scheduling DCI with C-RNTI scrambling

In Rel-16, SCell dormancy can be supported by the non-scheduling DCI with C-RNTI scrambling (i.e. SCell dormancy case 2 like), i.e. DCI format x_0 and x_1 with the disabled frequency resource allocation. With such non-scheduling DCI, UE can be indicated to switch to dormant BWP in SCell(s) without being scheduled. It has the following benefits.

- It can carry large number of DCI bits.

- It can support the HARQ feedback for the indicating PDCCH shown below (TS 38.213).

A UE is expected to provide HARQ-ACK information in response to a detection of a DCI format 1_1 indicating SCell dormancy after N symbols from the last symbol of a PDCCH providing the DCI format 1_1.

So, we support the non-scheduling DCI with C-RNTI scrambling that indicates PDCCH monitoring adaptation by both SSSG switching and PDCCH skipping.

Proposal 3: Support the non-scheduling DCI with C-RNTI scrambling that indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping.

## DCI format 2-0

In our view, DCI format 2-0 is not mandatory for licensed operation. On the other hand, SFI in DCI format 2-0 usually has large periodicity in real deployment. Therefore, it may not be suitable for Rel-17 SSSG switching or Rel-17 PDCCH skipping.

## DCI format 2-6

In Rel-16, DCI format 2-6 can carry the wakeup indication. Whether it can carry the PDCCH monitoring adaptation indication is questionable. Firstly, DCI format 2-6 should be reliable (i.e. with low miss detection rate) so that it cannot carry too many DCI bits. Secondly, the use case that gNB want to start onDurationTimer but also want UE skips PDCCH monitoring at the beginning may be a corner case. Therefore, it may not be suitable for Rel-17 SSSG switching or Rel-17 PDCCH skipping.

# Conclusion

In the contribution, we have the following proposals.

Rel-17 SSSG switching technique

Proposal 1: The application delay for Rel-17 SSSG switching technique can be defined.

Rel-17 PDCCH skipping technique

Proposal 2: The application delay for Rel-17 PDCCH skipping technique is zero.

Non-scheduling DCI with C-RNTI scrambling

Proposal 3: Support the non-scheduling DCI with C-RNTI scrambling that indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping.

# Reference

- R1-2100815, “Discussion on power saving techniques for connected-mode UE”, Spreadtrum, RAN1#104e, Jan. 25th - Feb. 5th, 2021.

- R2-1908072, “Report on [105bis#27][NR/Power Saving] – PDCCH skipping”, CATT, RAN2#106, May 13th - 17th, 2019.

- 3GPP RAN1#104e Chairman’s Notes, Jan. 25th - Feb. 5th, 2021.

- 3GPP RAN1#105-e Chairman’s Notes, May 10th - 27th, 2021.

- 3GPP RAN1#106-e Chairman’s Notes, August 16th - 27th, 2021.

- 3GPP RAN1#107-e Chairman’s Notes, November 11th - 19th, 2021.

- 3GPP RAN1#106bis-e Chairman’s Notes, Oct. 11th - 19th, 2021.