3GPP TSG RAN WG1 #107b-e		                                    		   R1-2200522

e-Meeting, January 17th – 25th, 2022

Agenda Item:	8.8.3

Source:	InterDigital, Inc.

Title:	Type A PUSCH repetitions for Msg3

Document for:	Discussion and Decision

# Introduction

This contribution addresses the following remaining issues for Type A PUSCH repetitions for Msg3:

- Repetition indication for msg3 re-transmission

- Repetition for CFRA PUSCH

The contribution proposes to that 8 MCS index can be indicated in the case of msg3 retransmission. It also proposes to confirm the working assumption on repetition for PUSCH scheduled by RAR in case of CFRA.

# Repetition indication for msg3 re-transmission

In RAN1#107-e, RAN1 agreed to use MCS information field for indication of the number of repetitions for msg3 initial transmission. RAN1 also agreed that 4 MCS index values can be signaled. As the MCS field of the random access response grant comprises 4 bits, the field can potentially indicate any combination of one out of 4 MCS indices and one out of 4 repetition factors.

For the indication of the number of repetitions for msg3 retransmissions, RAN1 agreed to use the same mechanism as for msg3 initial transmission. However, in this case the MCS field is included in the DCI and has a size of 5 bits. The following options can be envisioned for the use of the additional bit:

- Doubling the number of MCS values to 8

- Doubling the number of repetition values to 8

- Additional combinations of MCS values and repetition values

- Reserving

In the above, option c) offers would incur lots of additional SIB overhead which may not be warranted by the benefits of the additional flexibility. Option (d) is of course possible but either (a) or (b) would increase scheduling flexibility from network perspective and seems better. In particular, option (a) would allow the network to recover some flexibility in selection of MCS value that was lost for the signaling of the initial transmission and is therefore preferred. As for the initial transmission, the set of additional MCS values can be configurable in SIB1 with default values.

Proposal 1: When the UE applies msg3 repetition, the MCS information field for msg3 re-transmission can indicate up to 8 MCS index values.

Proposal 2: Four additional candidate MCS index values can be configured by SIB1 for msg3 retransmission, with default values MCS 4~7 if field is absent.

# Repetition for CFRA PUSCH case

In RAN1#107-e, RAN1 made a working assumption to support repetition for a PUSCH scheduled by RAR UL grant also for the case of CFRA PUSCH. The working assumption can be confirmed only if no additional RAN1 specification impact is identified and if no optimization specific for CFRA PUSCH is supported.

Supporting repetition for CFRA PUSCH scheduled by RAR UL grant may be beneficial for example in case of handover when coverage-limited (e.g. between two large cells or when the UE is shadowed). In this case, the UE may receive a dedicated preamble in the handover command, and it can then apply repetition on the PUSCH scheduled by RAR. Assuming that the network knows the UE capability, the UE can interpret the RAR as indicating repetitions whenever a dedicated preamble is provided (CFRA) and its RACH is configured for msg3 repetition.

Proposal 3: Confirm the following working assumption:

Working assumption

- support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH.

  - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions.

  - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced.

  - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition.

  - No additional RAN1 specification impact

Note: UE reports Msg3 repetition capability after initial access.

Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

# Conclusion.

This contribution proposes the following:

Proposal 1: When the UE applies msg3 repetition, the MCS information field for msg3 re-transmission can indicate up to 8 MCS index values.

Proposal 2: Four additional candidate MCS index values can be configured by SIB1 for msg3 retransmission, with default values MCS 4~7 if field is absent.

Proposal 3: Confirm the following working assumption:

Working assumption

- support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH.

  - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions.

  - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced.

  - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition.

  - No additional RAN1 specification impact

Note: UE reports Msg3 repetition capability after initial access.

Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

# References

[1] R1-2112836,	“Feature lead summary #4 on support of Type A PUSCH repetitions for Msg3”, Moderator (ZTE Corporation).

# Appendix

Agreements from RAN1#107-e

Agreement

- Flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition.

- Note: whether and how to introduce other potential mechanisms to use the flexible symbols are separately discussed.

- Note: The Rel-15/16 rules are reused for collision handling between Msg3 PUSCH transmission and a CORESET for Type0-PDCCH CSS set indicated to a UE by pdcch-ConfigSIB1 in MIB in a set of flexible symbols indicated by tdd-UL-DL-ConfigurationCommon.

Conclusion

- There is no consensus to additionally introduce explicit indication to indicate whether or not flexible slots/symbols configured via TDD-UL-DL-Configcommon are available for Msg3 repetition.

Agreement

- RV cycling for Msg3 PUSCH repetition is based on transmission occasions on available slots.

Agreement

For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules.

- The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused.

- The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case intra-slot FH is disabled is reused.

- The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused.

Agreement

- For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted.

  - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent.

  - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}.

Agreement

For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted.

FFS: [8] MCS index to be used for Msg3 re-transmission

Agreement

- Reuse legacy collision handling rule between Msg3 PUSCH transmission and downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated.

- Note: there is no specification impact.

Working assumption

- support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH.

  - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions.

  - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced.

  - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition.

  - No additional RAN1 specification impact

Note: UE reports Msg3 repetition capability after initial access.

Note: The working assumption can be confirmed only if no additional RAN1 specification impact nor optimization specific for CFRA PUSCH.

R1-2112836	Feature lead summary #4 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

R1-2112558	Feature lead summary #3 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

R1-2112557	Feature lead summary #2 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

R1-2112556	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3	Moderator (ZTE Corporation)

Agreements from RAN1#106b-e

NR_cov_enh – Msg3 repetition (From AI 5)

R1-2108712	LS on Msg3 repetition in coverage enhancement	RAN2, ZTE

Response LS needed. To be handled under agenda item 8.8.3.

The Draft LS of v01 is endorsed in principle.

Final LS R1-2110585 is endorsed.

Working Assumption

Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission.

- Alt 1: If TDRA information field is chosen, Option 2 is supported.

  - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]}

- Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows.

  - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values.

    - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]}

Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195).

RAN1 thinks at least the number of preambles per SSB per RO for request of Msg3 repetition, i.e., CB-PreamblesPerSSB, is needed. It’s up to RAN2 whether to indicate the start of preamble index for request of Msg3 repetition with shared RO.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195).

- From RAN1 perspective, there is no need to separately configure the following legacy RACH parameters configured in RACH-ConfigCommon for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

- prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- totalNumberOfRA-Preambles

- ssb-perRACH-OccasionAndCB-PreamblesPerSSB

- FFS: rsrp-ThresholdSSB

- rsrp-ThresholdSSB-SUL

- prach-RootSequenceIndex

- msg1-SubcarrierSpacing

- restrictedSetConfig

- msg3-transformPrecoder

Conclusion

There is no consensus to additionally support intra-slot frequency hopping for Msg3 PUSCH with repetition in Rel-17.

Note: intra-slot FH is supported when a UE is scheduled Msg3 PUSCH without repetition.

Agreement

Include the following into the reply LS to R1-2108712(R2-2109195)

 From RAN1 perspective, it can be beneficial to separately configure rsrp-ThresholdSSB for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

Agreement

If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping.

Agreement

The Rel-15/16 Msg3 PUSCH collision handling rules are reused for transmission of Msg3 PUSCH repetition in an available slot.

 FFS whether collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated is an exceptional case, i.e., Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17.

 FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s)

Agreements from RAN1#106-e

Agreement

Do NOT support fallback RAR UL grant in 2-step RACH for indicating Msg3 repetition.

Agreement

The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition.

Working Assumption

Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission.

- Alt 1: If TDRA information field is chosen, introducing a new configurable TDRA table including the repetition factors.

  - The new TDRA table is configured by SIB1, with selecting one of the two options below.

    - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor.

    - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor.

  - If a new TDRA table is not configured, the legacy default TDRA table is used, and repetition factor K=1 is applied.

    - K=1.

- Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows.

  - X MSB bits of the MCS information field are used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

- Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below.

  - Option 1: X LSB bits of the TPC information field are used for repetition indication.

    - FFS the value of X.

    - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set.

  - Option 2: A predefined TPC command table with including repetition factor K is introduced.

    - FFS details.

Agreement

Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions.

- Option 1:

- When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition.

  - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.

- Option 2:

- When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB.

  - FFS details, e.g. implicit or explicit indication or predefined.

  - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field.

- When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

Agreement

- Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition.

- FFS whether to support other values, e.g., 8.

- Note: K=1 is supported and how to support K=1 is FFS.

Agreement

- The available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, no other additional Rel-16 signals/signalings will be considered.

- If a symbol for Msg3 repetition in a slot overlaps with SSB transmission [FFS:N Gap symbols after SSB], the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot.

Agreement

Do not support TBoMS for Msg3 in Rel-17 coverage enhancement WI.

Agreements from RAN1#105-e

R1-2106007

Update on 5/22:

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold.

- FFS the determination of the RSRP threshold.

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options.

- Option 1: Use the same mechanism as supported for Msg3 initial transmission.

- Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission.

- The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214.

- The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214.

- FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Agreement:

- For requesting Msg3 PUSCH repetition, support the following:

  - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs.

    - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

    - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH).

  - FFS whether or not to additionally support one (& only one) more option:

    - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

    - E.g., Option 3: Use separate RO, which include

      - the separate RO configured by a separate RACH configuration index from legacy UE, and

      - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated.

Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon.

- A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols.

  - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition.

  - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

Working assumption:

- Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission

  - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission.

    - TDRA information field with introducing a new TDRA table including the repetition factors.

    - MCS information field

    - TPC information field

    - CSI request information field

    - FDRA information field

- The total size of RAR UL grant does not change.

- Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not.

- FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc.

Conclusion:

- Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously)

Final summary in R1-2106247

Agreements from RAN1#104b-e

R1-2103829

Agreement: For Msg3 PUSCH repetition,  support the following modified Option 2-1.

- Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.).

- Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual

- FFS details if any.

Update on 4/16

Agreements: For the determination of RV for Msg3 PUSCH repetition,

- RV of the first repetition is determined in the same way as legacy.

  - Use RV 0 for the first repetition of Msg3 PUSCH initial transmission.

  - Use a dynamically indicated RV id via DCI 0_0 with CRC scrambled by TC-RNTI for the first repetition of Msg3 PUSCH re-transmission.

- FFS determination of the RV sequence.

Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted.

- FFS additionally using MAC RAR for indication.

Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted.

Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3.

- FFS: the determination of available slots.

Agreements from RAN1#104-e

Agreements:

- For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below.

  - Option1: UL grant scheduling Msg3.

    - FFS details.

    - FFS fallbackRAR UL grant.

    - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported.

  - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI

    - FFS details.

  - Option3: SIB1 only

- Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively

Agreements:

- For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below.

Option1: DCI format 0_0 with CRC scrambled by TC-RNTI.

FFS details.

Any modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI.

Option2: Can be determined based on the repetition number  for  Msg3 initial transmission

Agreements:

Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission.

FFS details, e.g., signaling etc.

Agreements:

For Msg3 PUSCH repetition,  the following options are considered, aiming for down-selection in RAN1#104b-e:

- Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request,

- A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- FFS details if any.

- Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request,

- gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

- For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition

- For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc.

- gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition.

- FFS details if any.

- Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

- A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions.

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission.

- FFS details if any.

- Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions,

  - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions.

  - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways)

- Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

  - FFS details if any.

- Other options are not precluded.