3GPP TSG RAN WG1 #109-e	                          			      R1-2204143

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.1.2

Source:	LG Electronics

Title:	Potential CSI enhancement for high/medium UE velocities and coherent JT

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss potential CSI enhancement for high/medium UE velocities and for coherent JT. Since specification impact and functionality are different for high speed UE and for CJT, we first discuss potential CSI enhancement for high/medium UE velocities in Section 2.1 and potential CSI enhancement for CJT in Section 2.2

Discussion

Potential CSI enhancement for high/medium UE velocities

According to Rel-18 MIMO WID [1], two different approaches for potential CSI enhancement for high speed UE are listed up; one is type II codebook enhancement and the other is UE reporting of time domain channel properties such as Doppler shift or Doppler spread via TRS. The purpose of these two approaches is to help gNB make better link level adaptation for time varying channel due to UE’s high mobility.

Specifically, an enhanced type II codebook with time domain compression can provide gNB with precoding information for multiple time instances, instead of a single time instance which is vulnerable to channel aging, so that gNB can track and predict time varying channel direction and finally schedule better MCS and precoder. On the other hand, on top of legacy CSI, by calculating and reporting Doppler information via TRS, UE can help gNB to compensate outdated CSI with Doppler information, so that gNB can predict time varying channel direction and decide better MCS and precoder.

In this way, the two approaches have a different specification impact but share the same motivation, purpose and goal. So it makes sense to focus on essential enhancement without duplication. Therefore, in our view, it is reasonable to down select one of the two approaches considering advantage and disadvantage, performance gain, and implementation complexity.

Observation 1: Two approaches for potential CSI enhancement for high speed UE, i.e., type II codebook enhancement and reporting time domain channel property, have a different specification impact but share the same motivation, purpose and goal.

Proposal 1: Consider to down select one of the two approaches, which is captured in Observation 1, by taking into account advantage and disadvantage, performance gain, and implementation complexity.

In comparison with the two approaches, type II codebook enhancement has more potential specification impact and we focus the type II codebook enhancement and list up potential issues in this section in the following order: channel measurement, time instances the new codebook represents, TD (time domain) basis.

- Channel measurement for type II codebook enhancement:

The type II codebook enhancement with TD compression requires multiple channel measurement at UE side, so it is worth taking a look into how legacy UE conducts multiple channel measurement. According to current specification, time domain channel/interference measurement restriction can be enabled/disabled by gNB. If it is enabled, UE measures the latest CMR/IMR no later than CSI reference resource, therefore single measurement is used to estimate channel in CSI reference resource.  If it is disabled, UE can measure multiple CMR/IMR instances no later than CSI reference resource and it is up to UE implementation how to use those multiple measurements to estimate channel in CSI reference resource. With this unrestricted measurement configuration, legacy UE is already able to use multiple measurements to calculate CSI. If codebook enhancement with TD compression is introduced in Rel-18, this unrestricted measurement configuration is necessary for UE to calculate PMI for multiple time instances.

Observation 2: type II codebook enhancement with TD compression requires multiple channel measurement at UE side.

Observation 3: If timeRestrictionForChannelMeasurements is not configured, UE can use multiple channel/interference measurements to calculate CSI with enhanced codebook based on TD compression.

Observation 4: If timeRestrictionForChannelMeasurements is configured, UE can use only single channel/interference measurement to calculate CSI, so enhanced codebook with TD compression cannot be used.

Whether multiple channel measurement is possible does not only depend on time domain measurement restriction, as explained above, but also depend on periodicity of CMR/IMR. In case of AP CMR/IMR, only single measurement is possible since there is only one CMR/IMR instance without periodicity. On the other hand, P/SP CMR/IMR can provide UE with multiple chances for channel/interference measurement. However, since minimum periodicity is 4 slots according to current specification, it should be studied whether it provides sufficiently frequent measurement instances for high speed UE. If it is too sparse, it may decrease PMI accuracy with poor TD compression. If it is too dense, it may increase CSI-RS overhead.

Observation 5: AP CMR/IMR, only single measurement is possible since there is only one CMR/IMR instance without periodicity.

Proposal 2: Study whether legacy P/SP CSI-RS provides sufficient measurement time instances for enhanced type II codebook with TD compression.

- Multiple time instances that enhanced type II codebook with TD compression represents:

Like legacy type II codebook with FD compression representing PMIs for multiple sub-bands, new type II codebook with TD compression represents PMIs for multiple time instances, so how to determine these multiple time instances should be discussed. The legacy CSI including PMI/RI/CQI represents channel for a single time instance which is called CSI reference resource. However, type II codebook with TD compression should be able to provide PMI not only in CSI reference resource but also in other slots/symbols. Basically, these time instances can be defined either after CSI reference resource or before CSI reference resource, which Figure 1 and 2 shows, respectively, and it has different impacts on UE implementation.

Figure 1. PMIs for multiple time instances no earlier than CSI reference resource

In Figure 1, UE calculates legacy PMIref_rsc based on DL channel in CSI reference resource, which can be derived from periodic CMRs no later than CSI reference resource. More importantly, UE calculates PMIref_rsc+τ and PMIref_rsc+2τ based on DL channel in CSI reference resource+τ slot and in CSI reference resource+2τ slot, which is predicted from periodic CMRs no later than CSI reference resource. Finally, PMIref_rsc, PMIref_rsc+τ and PMIref_rsc+2τ are compressed to reduce PMI feedback overhead by using enhanced type II codebook with TD compression. Since this codebook includes PMIs in future, which requires UE to predict DL channel after CSI reference resource, it is robust to channel aging and gNB can use it for better link adaptation.

Figure 2. PMIs for multiple time instances no later than CSI reference resource

In Figure 2, UE calculates legacy PMIref_rsc in the same way as we explained above in Figure 1. However, unlike Figure 1, UE calculates PMIref_rsc-τ and PMIref_rsc-2τ based on DL channel in CSI reference resource-τ slot and in CSI reference resource-2τ slot, which is measured from periodic CMRs no later than CSI reference resource. Note that Figure 2 shows an example in which periodic CMRs are in CSI reference resource, CSI reference resource-τ slot and CSI reference resource-2τ slot. Finally, PMIref_rsc, PMIref_rsc-τ and PMIref_rsc-2τ are compressed to reduce PMI feedback overhead by using enhanced type II codebook with TD compression. Since this codebook includes PMIs in past, it does not require UE to predict further DL channel, which has less complexity than UE in Figure 1. However, PMIref_rsc-τ and PMIref_rsc-2τ are staler than legacy PMIref_rsc so it is questionable how to use them for better link adaptation at gNB side and potential gain.

Proposal 3: Study how to configure additional multiple time instances that enhanced type II codebook represents including whether they are after CSI reference resource or before CSI reference resource.

- TD basis for type II codebook enhancement:

As we described in Figure 1 and 2, UE calculates multiple PMIs for multiple time instances in order to cope with channel aging effect due to high mobility.  However, it increases PMI feedback payload linearly as multiple time instances increases. Even if those PMIs share the same SD basis and FD basis, at least different coefficient values for each PMI, which occupies a dominant portion of the payload, needs to be reported. Therefore, time domain compression using TD basis for the coefficient values can be beneficial for feedback overhead reduction. Considering legacy type II codebook, in which SD basis and FD basis are independently selected based on DFT matrix, TD basis also can be selected independently from SD/FD basis. This is simple and straight forward way but not the best way in performance aspect. For example, beam direction of each SD basis can be fully/partially aligned with or opposite to UE’s moving direction, so each SD basis experience the different level of time varying channel. If different TD basis can be selected for different SD basis considering this aspect, it helps to increase codebook accuracy.

Proposal 4: Study how much feedback overhead can be saved taking advantage of TD compression and performance benefit of SD/FD basis specific TD basis selection.

Potential CSI enhancement for coherent JT

Non-coherent JT for PDSCH is supported with S-DCI based MTRP and M-DCI based MTRP in legacy NR specification. Since two TRPs transmit different layers at the same time, there is no need to consider coherency between the signals the two TRPs transmit. On the other hand, for coherent JT, two TRPs transmit the same layer like SFN transmission but, in order to make constructive sum of the two signal, the two TRPs need to apply precoder with proper co-phase rotation. In case of Type I MP codebook, this co-phase component per panel is already supported, but Type II codebook is not.  There are two different approaches on how to feedback this co-phase component in Rel-18; the first approach is to add this component in the Type II codebook, similar with Type I multi-panel codebook and the second approach is to add co-phase component as new reporting quantity on top of RI/PMI/CQI for MTRP transmission.

Observation 6: There are two different approaches to report per TRP/panel co-phase component to support CJT;

- the first approach is to add the co-phase component in the Type II codebook

- the second approach is to add the co-phase component as new reporting quantity, on top of RI/PMI/CQI for MTRP transmission.

In the first approach with Type II codebook enhancement, the legacy Type II codebook needs to be extended for multiple TRPs/panels. Specifically, dimension of each SD basis vector should be extended to the sum of CSI-RS ports of multiple TRPs and different SD basis can be selected per TRP considering different DL channel and antenna configuration for each TRP. In addition, there are many other component that can be extended such as TRP specific SCI, FD basis, quantization level for coefficient, and so on. As many codebook components are extended in TRP specific manner, codebook accuracy increases but, as drawback, feedback overhead also increases.

In addition, the first approach accompanies CSI-RS enhancement since aggregated DL channels from multiple TRPs are measured from CSI-RS. Again, we can consider two different approaches for the CSI-RS enhancement; one is to measure aggregated channel from a single CSI-RS resource and another is to measure each TRP’s channel from different CSI-RS resources and aggregate them. In a single CSI-RS resource based approach, since different TRPs transmit different port groups of the CSI-RS resource, port group wise TCI state should be indicated. In addition, according to WID, the maximum number of CSI-RS ports per resource remains the same as in Rel-17 so there is limitation that sum of CSI-RS ports across MTRP cannot be over 32. On the other hand, in multiple CSI-RS resource based approach, channel measurement can be done in legacy way but UE should aggregate channels measured from the multiple CSI-RS resources.

Observation 7: Type II codebook enhancement to support CJT accompanies CSI-RS enhancement for aggregated channel measurement and there are two different approaches;

- the first approach is to measure aggregated DL channel from a single CSI-RS resource in which different TRP transmit different port group.

- the second approach is to measure DL channel for each TRP from multiple CSI-RS resources and aggregate the measured channel.

Instead of codebook enhancement, another approach to support CJT is to add new reporting quantity, e.g. co-phase component per TRP, in Rel-17 MTRP CSI. For Rel-17 MTRP CSI, CMR pair is configured and UE reports 2 RIs, 2 PMIs and combined CQI assuming NCJT transmission. This MTRP CSI framework can be reused for CJT CSI with limited specification impact. Specifically, gNB configures CMR pair for CJT in the same ways as Rel-17 and UE reports co-phase component per TRP in addition to 2 RIs, 2 PMIs and combined CQI assuming CJT transmission, instead of assuming NCJT transmission. Unlike Type II codebook enhancement, this approach does not have an impact on CSI-RS, channel measurement, TCI indication, and codebook, which seems much simpler and straight forward way to support CJT.

Proposal 5: Study the two approaches to support CJT, which is captured in Observation 6, in terms of CSI overhead, performance and specification impact.

Conclusion

In this contribution, we discuss potential CSI enhancement for high/medium UE velocities and for coherent JT and propose the followings.

Potential CSI enhancement for high/medium UE velocities:

Observation 1: Two approaches for potential CSI enhancement for high speed UE, i.e., type II codebook enhancement and reporting time domain channel property, have a different specification impact but share the same motivation, purpose and goal.

Proposal 1: Consider to down select one of the two approaches, which is captured in Observation 1, by taking into account advantage and disadvantage, performance gain, and implementation complexity.

Observation 2: type II codebook enhancement with TD compression requires multiple channel measurement at UE side.

Observation 3: If timeRestrictionForChannelMeasurements is not configured, UE can use multiple channel/interference measurements to calculate CSI with enhanced codebook based on TD compression.

Observation 4: If timeRestrictionForChannelMeasurements is configured, UE can use only single channel/interference measurement to calculate CSI, so enhanced codebook with TD compression cannot be used.

Observation 5: AP CMR/IMR, only single measurement is possible since there is only one CMR/IMR instance without periodicity.

Proposal 2: Study whether legacy P/SP CSI-RS provides sufficient measurement time instances for enhanced type II codebook with TD compression.

Proposal 3: Study how to configure additional multiple time instances that enhanced type II codebook represents including whether they are after CSI reference resource or before CSI reference resource.

Proposal 4: Study how much feedback overhead can be saved taking advantage of TD compression and performance benefit of SD/FD basis specific TD basis selection.

Potential CSI enhancement for coherent JT:

Observation 6: There are two different approaches to report per TRP/panel co-phase component to support CJT;

- the first approach is to add the co-phase component in the Type II codebook

- the second approach is to add the co-phase component as new reporting quantity, on top of RI/PMI/CQI for MTRP transmission.

Observation 7: Type II codebook enhancement to support CJT accompanies CSI-RS enhancement for aggregated channel measurement and there are two different approaches;

- the first approach is to measure aggregated DL channel from a single CSI-RS resource in which different TRP transmit different port group.

- the second approach is to measure DL channel for each TRP from multiple CSI-RS resources and aggregate the measured channel.

Proposal 5: Study the two approaches to support CJT, which is captured in Observation 6, in terms of CSI overhead, performance and specification impact.

- References

[1] RP-213598	New WID: MIMO Evolution for Downlink and Uplink		Samsung