3GPP TSG RAN WG1 #109-e		                                    		   R1-2204159

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.4.1

Source:	InterDigital, Inc.

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, the contents of SID for Rel. 18 were agreed [1]. It was agreed to study AIML based positioning. The descriptions in the SID related to use cases and evaluation assumptions are as follows.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

In this contribution, we discuss evaluation assumptions and initial set of evaluation results for fingerprinting-based positioning.

# Evaluation assumptions

## Simulation assumptions

In Table A1 in Appendix, we provide simulation parameter configuration for AIML based positioning. In Section 3, simulation results for 2 use cases have been presented namely, 1) NLOS Identification and 2) fingerprinting-based positioning. The purpose of performance evaluation of NLOS identification is to examine characteristics of different AI models.

For AIML based positioning simulations, we divide the positioning procedure in the following two phases:

Training Phase: during which ground truth/output is known along with input (measurements) to the model. During training phase, we use training data to train the model and validation data to verify the accuracy of trained model during training phase.

Inference Phase: during which input(measurements) are provided to the trained model to predict the output. During deployment phase, the input data provided to the model has been referred as test data which hasn’t been exposed to the model during training phase.

An agreement on the simulation assumptions for AIML based positioning needs to be made. One of the practical use  cases for positioning is an IIoT scenario where workers or AGVs (Automatic Guided Vehicle) need to be located [2]. The starting point for evaluation assumptions may be the evaluation assumptions for the IIoT scenario assumed in TR 38.857 [2]. In particular, NLOS heavy setting such as DL (Dense clutter Low BS) or DH (Dense clutter High BS) defined in TR 38.901 [3] scenario can be considered in the evaluation assumption to evaluate the accuracy performance of AIML based positioning.

Proposal 1: Use IIoT scenarios (e.g., InF-DL or InF-DH) from TR 38.901 as one of the evaluation scenarios for AIML based positioning

In addition, in system level simulation, UEs dropped in the convex hull of the factory floor in IIoT scenarios tend to benefit from higher positioning accuracy, compared to UEs located close to corners of the factory floor, since more TRPs that are visible. Due to lack of visibility of TRPs, the UEs located in the corner of the factory tends to suffer from dilution of precision. High accuracy should be guaranteed for the entire floor and accuracy for UEs located in the corner of the factor floor should be evaluated in the study. Thus, we make the following proposal :

Proposal 2: Evaluate performance of the UEs located in the corner of the factory floor in IIoT scenarios

Finally, latency require for positioning should be evaluated. Since the AIML-based positioning requires training the AIML model, the latency required for training the AIML should be evaluated. In addition, the speed the AIML model can generate a predicated UE location, i.e., prediction rate, should also be evaluated.

Proposal 3: Evaluate latency required for AIML based positioning which includes time required to train AIML models and rate at which the AIML models can generate predicted UE locations

## Data Set generation procedure for NLOS Identification

For NLOS link identification, UEs are dropped in the deployment area as described in Table A1. During the training phase, input measurements are collected at each UE from all TRPs. All the measurements are combined and provided to the model along with NLOS flag (0 or 1 indicator per UE-TRP link). During the inference phase, another set of UEs are dropped in the same scenario and the trained model has been used to predict the NLOS flag by providing collected input measurements to the model. We use classification models to solve NLOS identification problem. Parameters associated with NLOS Identification use case are described in Table A2.

## Dataset generation procedure for fingerprinting-based positioning

During training phase of single TRP fingerprinting, input measurements are collected at each UE from a single TRP. The UE measures RSRP values from different beams (with different Azimuth angles) of the same TRP. All RSRP measurements and UE location information are used to train the ML (Machine Learning) model. We use a convolutional neural network for training. During inference generation, a separate set of UEs are dropped in the same scenario and trained model has been used to predict UE’s position by providing collected RSRP measurements to the ML model.

During the training phase of multi TRP fingerprinting, input measurements are collected at each UE from TRPs deployed in the area (scenario parameters are specified in Table A1). Each UE trains the common ML model using RSRP fingerprints. Multi TRP fingerprint and UE location are used to train the ML model. A fully connected neural network is used to train the model for multi TRP fingerprinting based positioning.  During inference procedure, a separate set of UEs are dropped in the same scenario and trained model has been used to predict UE’s position by providing the collected RSRP measurements to the ML model. In Table A3, parameters assumed for performance evaluation of fingerprinting-based positioning are shown.

# Evaluation results

## Evaluation results: LOS/NLOS detection

In this section, we present accuracy of ML based NLOS identification. Performance of different classification model is listed in Table 1. To evaluate the accuracy of the model, we use following KPIs:

- True Positive Rate (TPR) implies number of samples predicted correctly to the total available sample per class (e.g. LOS, NLOS).

- Prediction speed signifies the complexity of each model based on number of observations/prediction a model can make per second, , normalized by the prediction speed required for SVM

- Training time is the time taken by the model to train a set of training data, normalized by the training time required for SVM

Table 1 : NLOS detection accuracy comparison for different classification models

| Model Type | TPR (NLOS) | TPR (LOS) | Prediction speed | Training time |
| --- | --- | --- | --- | --- |
| SVM | 93.1% | 80.1% | 1 (baseline) | 1 (baseline) |
| Random forest | 93.2% | 78.5% | 68 (normalized by SVM speed) | 0.13 (normalized by SVM training time) |
| Weighted KNN | 99.5% | 85.7 % | 0.35 (normalized by SVM speed) | Training time: 0.075 (normalized by SVM training time) |

From Table 1, compared to SVM, random forest based AI model requires less training time. However, the rate the random-forest AI model generates location estimates is slower than the SVM. Weighted KNN based AI model requires less training time and faster prediction speed compared to SVM. In addition, weighted KNN based AI yields slightly better detection accuracy in terms of LOS/NLOS prediction.

## Evaluation results: Fingerprinting based positioning for DL-based positioning

In Table 2, 3D accuracy of fingerprinting-based positioning, measured during the inference phase, is compared against that of DL-TDOA. For DL-TDOA, results are generated based on simulation parameters shown in Table A1. Positioning accuracy is measured in terms of root mean square error of predicted position against UE’s actual position. In the evaluation, the InF-SH (Sparse clutter and High BS) scenario was assumed.

Table 2 : Accuracy of fingerprinting-based positioning(3D)

| Positioning Method | 3D Positioning accuracy (meter) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50% ile | 67% ile | 80% ile | 90% ile |
| DL-TDOA, InF-SH | 0.8752 | 1.6067 | 2.0653 | 2.6520 |
| Single TRP Fingerprinting, InF-SH | 0.8339 | 1.1663 | 1.5744 | 2.2419 |
| Multi TRP Fingerprinting, InF-SH | 0.9379 | 1.2043 | 1.4405 | 1.7532 |

From the simulation results summarized in Table 2 and Figure 1, at the 90% ile CDF, multi-TRP based fingerprinting positioning yields approximately 0.5 m improvement over single-TRP based fingerprinting method. Compared to DL-TDOA, multi-TRP and single-TRP based fingerprinting methods yield 0.9m and 0.4m improvement over DL-TDOA. The performance of multi-TRP can be further improved by increasing the amount of data.

Observation 1 : Multi-TRP based fingerprinting yields approximately 0.5m and 0.4m improvement over single-TRP based fingerprinting and DL-TDOA, respectively, at CDF=0.9.

Figure 1 CDF plot of single/multi fingerprinting positioning method and DL-TDOA techniques

# Conclusion.

In this contribution, the following proposals and observation are made.

Proposal 1: Use IIoT scenarios (e.g., InF-DL or InF-DH) from TR 38.901 as one of the evaluation scenarios for AIML based positioning

Proposal 2: Evaluate performance of the UEs located in the corner of the factory floor in IIoT scenarios

Proposal 3: Evaluate latency required for AIML based positioning which includes time required to train AIML models and rate at which the AIML models can generate predicted UE locations

Observation 1: Multi-TRP based fingerprinting yields approximately 0.5m and 0.4m improvement over single-TRP based fingerprinting and DL-TDOA, respectively, at CDF=0.9.

# Reference

[1] 3GPP, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” RP-213599, RAN#94e, Dec. 2021.

[2] TR 38.857, “Study on NR Positioning Enhancements,” ver. 1.1.0, March, 2021.

[3] TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz,”, ver. 17.0.0, March, 2022.

# Appendix

Table A1 : IIoT scenario system parameters

| Parameter | Values |
| --- | --- |
| Carrier frequency, GHz | 3.5GHz |
| Bandwidth, MHz | 100MHz |
| Subcarrier spacing, kHz | 30kHz |
| Channel model | InF-SH |
| Hall size | 120(L) x 60(W) m, D – 20 m |
| BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. |
| Room height | 10 m |
| Number of floors | 1 |
| Clutter parameters: {density , height ,size } | InF-SH - {20%, 2m, 10m} |
| UE model parameters |  |
| UE noise figure, dB | 9dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| Network synchronization | Fully synchronized |
| UE/gNB RX and TX timing error | 0 ns |
| UE horizontal drop procedure | Uniformly distributed over the horizontal evaluation area within convex hull of horizontal BS deployment |
| UE antenna height | 1.5 m |
| gNB model parameters |  |
| Total gNB TX power, dBm | 24 dBm |
| gNB noise figure, dB | 5dB |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 |
| gNB antenna radiation pattern | Single sector, 3 sector (120 degrees apart from each other) – Note 1 |
| gNB antenna height | 8 m |

Table A2 : Data Set generation assumptions for NLOS Identification

| Parameter | Values |
| --- | --- |
| Training input measurements | power-delay characteristics of the channel, channel cluster parameters (e.g., delay, power) |
| Output of AIML model | NLOS flag (1 or 0) |
| Size of total dataset | 144000 (Number of UE-TRP links) |
| Size of Validation dataset (training phase) | 7200 (Number of UE-TRP links, 5% of total dataset) |
| Size of dataset used training | 136800 (Number of UE-TRP links, 95% of total dataset) |
| Size of test dataset | 18000 (Number of UE-TRP links) |

Tabe A3 : Data Set generation assumptions for fingerprinting-based positioning

| Parameter | Values |
| --- | --- |
| Single TRP fingerprinting |  |
| Training input measurements | Per beam RSRP (12 beams per TRP) |
| Output of AIML model | UE position |
| Number of TRPs | 1 |
| Deployment area | 20m by 20m area around the TRP’s location |
| Size of total dataset (fingerprint) | 10000 UEs |
| Size of dataset used for model training (training phase) | 7000 UEs (70% of total dataset) |
| Size of Validation dataset (training phase) | 1000 UEs (10% of total dataset) |
| Size of test dataset (inference phase) | 2000 UEs (20% of total dataset) |
| ML model | Convolutional neural network (Kernel Size=9) |
| Multi TRP fingerprinting |  |
| Training input measurements | Per beam RSRP from multiple TRPs (54 RSRP values, 3 beams per TRP) |
| Output of AIML model | UE position |
| Number of TRPs | 18 |
| TRP locations | As specified in Table A1 |
| UE deployment area | Convex hull of TRP locations |
| Size of total dataset (fingerprint) | 20000 UEs |
| Size of dataset used for model training (training phase) | 14000 UEs (70% of total dataset) |
| Size of Validation dataset (training phase) | 2000 UEs (10% of total dataset) |
| Size of test dataset (inference phase) | 4000 UEs (20% of total dataset) |
| ML model | Fully connected neural network (Number of layers=3) |