3GPP TSG RAN WG1 #109-e		R1- 2204222

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.7

Source:	Apple Inc.

Title:	Views on UE features for Rel-17 UE power saving

Document for:	Discussion/Decision

# Introduction

The latest UE feature list is provided in R1-2202928 after RAN1#108-e discussion. We provide our views on one of the remaining issues for R17 UE power saving features in this contribution.

# R17 UE Power Saving Features

FG 29-2

For FG 29-2, there was extensive discussion related to the support of L1 indication in PEI (DCI format 2_7), e.g., whether a UE reporting 29-2 should always support it.

Our view is that a UE supports L1 indication in DCI format 2_7 only if it reports both 29-1 (which includes the UE capability for receiving DCI format 2_7) and 29-2. If a UE reports 29-2 but not 29-1, the UE supports L1 indication in paging DCI only. It has been argued many times during the WI discussion that the basic feature is the support of paging DCI, and the UE may not support PEI. It was also argued that the indication in paging DCI is absolutely necessary because there are UEs that do not support PEI.

In this sense, we think 29-1 should not be the prerequisite for 29-2. Otherwise, there is no point for the network to transmit TRS availability indication in both paging DCI and DCI format 2_7, and transmitting it in one of the DCIs would be sufficient as the UE supports both anyway.

A few different alternatives had been discussed how to capture the intention:

- Alt 1: add “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1” in the note

  - Alt 1a: Alt 1 evolved into the following sentence for the note “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7”. Compared to Alt 1, this is considered to provide more flexibility in UE implementation. A UE may not support the full FG 29-1, but it still has the flexibility to implement DCI format 2_7 to support TRS indication in DCI format 2_7. Even though this is not our first preference, we are also fine with it.

- Alt 2: introduce a new FG 29-2a for the support of L1 indication in DCI format 2_7, and FG 29-1/29-2 are the prerequisites for 29-2a.

  - Alt 2 had been discussed in RAN1#108-e, and many companies prefer not to do the split.

We are generally fine with any of these alternatives, because the intention is sufficiently captured by any of them, i.e., not forcing all the UE supporting 29-2 to also support L1 indication in DCI format 2_7.

For the 2nd component of FG 29-2, it would be better to split into 2 parts, especially given that we have a note mentioning DCI format 2_7.

Proposal: Adopt the following changes for FG 29-2:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via paging DCI 3. Support receiving L1 indication for TRS availability via DCI format 2_7 |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N/A | N/A | N/A | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |