3GPP TSG-RAN WG1 Meeting #109-e	R1-2204908

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.11.3

Source:	Huawei, HiSilicon

Title:	Discussion on XR-specific capacity and power issues based on SA2 outcome

Document for:	Discussion and Decision

Introduction

In RAN1 Rel-17 study item “Study on XR Evaluations for NR”, progress has been made on many aspects, e.g. applications, KPIs, evaluation assumptions/methodologies, and performance evaluations. The results have been summarized in TR 38.838 [1]. For RAN Rel-18, a new SID “Study on XR Enhancements for NR” have been approved in RAN#94e with the following areas: XR-awareness in RAN, XR-specific power saving, and XR-specific capacity improvements [2]. The details are as follows:

(Copied from RP-213587 [2]) - 4 Objective - 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

Note that there is also a new SID on “Study on architecture enhancement for XR and media services” for SA2 Rel-18 [3], which has been started earlier this year in SA2#149e meeting. The outcomes have been summarized in TR 23.700-60 [4], where the main key issues have been identified. Solutions for solving these key issues are also proposed and discussed. As noted in TR 23.700-60 [4], some of the key issues may need coordination with RAN WGs. Meanwhile, RAN Rel-18 XR study also needs coordination with SA2, especially the aspects of XR-awareness in RAN. Therefore, in this contribution, we discuss the outcomes of recent SA2 study (some Key Issues in SA2 TR 23.700-60 are copied to Appendix) and the potential enhancements of XR-awareness in RAN, which are beneficial for XR capacity improvement and power saving.

Discussion

In this section, we discuss the potential enhancements for improving XR capacity, including QoS aspects, application awareness operation, and XR-specific resource allocation and scheduling enhancements.

View on Key Issue #4 (PDU Set integrated packet handling) in SA2 TR

Motivation

As identified in Key Issue #4 of SA2 outcome [4], PDU Set integrated packet handling is a key factor that can be considered to enhance the resource efficiency and promote user experience. This key issue is proposed to study PDU Set integrated packet handling in 5G network, in which the group of packets belonging to the same PDU Set will be handled in an integrated manner. The definition of a PDU Set is copied as follows:

(below is copied from SA2 TR 23.700-60) PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g. a frame or video slice for XRM Services), which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing.

Limited by the maximum transmission unit (MTU) of a network, e.g. 1500 bytes for Internet network, one XR video frame/slice may be segmented into one or multiple packets before being provided to the network for transmission. Generally, an XR video frame/slice can only be decoded and reconstructed correctly in media layers if all its associated packets have been correctly received. For XR/media services, such a group of packets are used to carry payloads of a media unit (e.g. a frame, video slice), forming a PDU Set. SA2 has sent an LS on QoS support with PDU Set granularity [5] to SA4 to confirm the definition of PDU Sets and SA4 confirmed that “the current PDU Set definition and the anticipated use for an application may map to and benefit some use cases and operation modes for media delivery. In particular, it should be possible that the media application layer can make use of the PDU Set definition and define the appropriate information units that are mapped to a PDU set” in the reply LS [6].

However, the dependency/correlation among the packets belonging to the same PDU Set is not known by the network (e.g. core network or RAN). Consequently, when these correlated packets arrive at the network, it may not be able to transmit the packets of a PDU Set within the required latency. On the other hand, if one or more packets of a PDU Set have already been lost and there is no chance to complete the transmission in the required latency, the network may still try to send the subsequent packets as it has no idea they are associated. In short, without considering PDU Set integrity, the quality of XR service cannot be guaranteed and the capacity cannot be enhanced due to potential unnecessary transmission. While if the PDU Set integrity is considered, the system performance for XR can be further improved.

This aspect has been considered in XR evaluation in RAN1 to some extent. For example, according to the evaluation methodology for capacity [1] (copied below), efficient data dropping is considered in Option 2 to improve the XR performance. The non-transmitted part of an XR frame may be discarded at the transmitter if any part of the PDU Set has lost or exceeded the delay budget.

(below is copied from TR 38.838) Packet Discarding Once communication commence between the UE and gNB, an XR/CG packet is deemed in error (i.e., lost) when it has exceeded the PDB, such that it will be added to the PER counting. It is up to company to report the details for the handling of packet which has exceeded the PDB, e.g. - Option 1: The packet exceeding the delay is still delivered to the other side - Option 2: The packet (including the non-transmitted part) is discarded at the transmitter (at the gNB for DL packets and at the UE for UL packets) - Other options are not precluded

Observation 1: The PDU Set integrated packet handling is important to enhance the system capacity and promote user experience for XR services.

Potential enhancements

To support PDU Set integrity, enhancements can also be done from E2E perspective. The QoS mechanism can consider including PDU Set level parameters, e.g. the PDU Set error rate, PDU Set delay budget etc. This is helpful for the network to decide whether to continue the subsequent data transmission or to discard. Coordination between source and 5G core network to identify which packets belonging to one PDU Set is also beneficial for satisfying XR service requirement. Such information of grouped packets can then be indicated to the RAN. RAN can benefit from this PDU Set integrated transmission to have efficient radio resource management, e.g. efficient packet dropping.

Proposal 1: PDU Set integrated packet handling is studied to further improve the XR system performance.

View on Key Issue #5 (Differentiated PDU Set Handling) in SA2 TR

Motivation

As identified in Key Issue #5 of SA2 outcome [4], different PDU sets can also carry different content, e.g. I/B/P frames, slices/tiles within an I/B/P frame, etc., and thus have different importance. It is anticipated that the 5GS QoS framework will be enhanced to support different QoS handling for PDU Set. The Key Issue #5 is proposed to support differentiated QoS handling considering different importance of PDU Sets e.g. by treating packets (i.e. PDUs) belonging to less important PDU Set(s) differently to reduce the resource wasting. Similarly, RAN1 Rel-17 study also identified that there can be multiple data streams with different traffic characteristics and QoS requirements in both DL and UL for a given XR application, for example, I-frame and P-frame, FOV stream and omnidirectional stream, video/depth and pose/control, video stream and audio stream [1]. These data streams actually have different importance, some of them directly dominate the end user experience, while some of them do not. However, in the current 5G QoS framework, the multiple data streams belonging to the same XR service are transmitted over the same QoS flow. Consequently, these data streams cannot be identified and thus the network will always treat them equally, which results in over-protection for non-important packets and potential waste of radio resources, causing low capacity. Even if the application layer may already split the important and non-important parts into different QoS flows, the 5G network has no idea whether these QoS flows have such association and will treat packets over multiple QoS flows independently. This may result in inefficient scheduling or priority handling of the packets transmission from the same source. In this case the service quality cannot be guaranteed.

Observation 2: Existing 5G QoS mechanism cannot well support different QoS requirements for a certain XR service with multiple data streams, resulting in low system capacity.

Potential enhancements

To accommodate the characteristic of multiple data streams in XR services, a layered QoS mechanism can be considered as illustrated in Figure 1, where two QoS flows are considered for illustration purpose. In the existing mechanism, each data flow may be mapped to one QoS flow respectively but cannot identify the correlation between these two QoS flows. Some E2E mechanism can be considered, e.g. the core network can identify these two QoS flows are associated with the same XR source by interaction with the application layer, and then informs such an association to the gNB. The gNB can therefore use this information to improve the scheduling mechanisms, e.g. efficient packet dropping and differentiated priority handling. Specifically, there may be some dependence between these two data flows from the same source. If the packets from one flow is lost, the packets from another flow may not be useful, which can be dropped to save radio resource or handled with lower priority.

Figure 1. E2E layered QoS mechanism

Proposal 2: Coordinated transmission of multiple data streams per XR application and differentiated priority PDU Set handling are studied to enhance the capacity.

View on Key Issue #8 and #9 (XR-awareness in RAN for power saving)

The objective of Key Issue #8 and Key Issue #9 in [4] is to study whether and how to improve power management schemes like C-DRX to achieve the best trade-off among key performance indicators like latency and device battery lifetime and how to support trade-off of throughput/latency/reliability and power consumption. SA2 has sent an LS to RAN1 in R1-2203219 [7] and asked RAN1 and RAN2 to clarify which type of information should be provided to the RAN for power saving enhancements for XR applications. We have a companion paper [8] to discuss this LS. According to our discussion in [8], some of the potential enhancements of C-DRX and PDCCH monitoring, may require assistance information from the CN or application, e.g. frame rate or traffic arrival interval/periodicity, jitter distribution related parameters, and PDU set related information. Therefore, we have the following proposal:

Proposal 3: The following information can be provided to the RAN for power saving enhancements for XR applications.

- Frame rate or traffic arrival interval/periodicity

- Jitter distribution related parameters, including range, STD (standard deviation)

- PDU set related information, including the relationship between packets belonging to the same PDU set and relationship between packets and the PDU set.

Conclusions

In this contribution, we discuss some of the key issues of SA2 outcome that may need coordination with RAN. The potential enhancements to address these issues are discussed with the following observations and proposals:

Observation 1: The PDU Set integrated packet handling is important to enhance the system capacity and promote user experience for XR services.

Observation 2: Existing 5G QoS mechanism cannot well support different QoS requirements for a certain XR service with multiple data streams, resulting in low system capacity.

Proposal 1: PDU Set integrated packet handling is studied to further improve the XR system performance.

Proposal 2: Coordinated transmission of multiple data streams per XR application and differentiated priority PDU Set handling are studied to enhance the capacity.

Proposal 3: The following information can be provided to the RAN for power saving enhancements for XR applications.

- Frame rate or traffic arrival interval/periodicity

- Jitter distribution related parameters, including range, STD (standard deviation)

- PDU set related information, including the relationship between packets belonging to the same PDU set and relationship between packets and the PDU set.

- References

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”, Dec. 2021.

RP-213587, “Study on XR Enhancements for NR”, 3GPP TSG RAN Meeting #94e, Dec. 6 - 17, 2021.

SP-211610, “Study on architecture enhancement for XR and media services”, 3GPP TSG SA Meeting #SP-94E, Dec. 14 - 20, 2021.

3GPP TR 23.700-60, “Study on XR (Extended Reality) and media services”, Feb. 2022.

R1-2203016, “LS on QoS support with PDU Set granularity”, 3GPP TSG RAN WG1#109-e, May 9-20, 2022.

S4-220505, “LS Reply on QoS support with PDU Set granularity”, 3GPP TSG SA4 Meeting #118-e, April 6-14, 2022.

R1-2203219, “LS on UE Power Saving for XR and Media Services”, RAN1#109-e, May 9 – 20, 2021.

R1-2204926, “Discussion on LS from SA2 on UE Power Saving for XR and Media Services”, RAN1#109-e, May 9 – 20, 2021.

- Appendix: Some Key Issues copied from SA2 TR 23.700-60

- 5.4	Key Issue #4: PDU Set integrated packet handling

- 5.4.1	Description

In current 5GS, the QoS Flow is the finest granularity of QoS differentiation in the PDU Session. The 5G QoS characteristics is determined by the 5QI. This implies that each packet in a QoS flow is treated according to the same QoS requirements.

For XR/media services, a group of packets are used to carry payloads of a PDU Set (e.g. a frame, video slice/tile).

In media layer, packets in such a PDU Set are decoded/handled as a whole. For example, the frame/video slice may only be decoded in case all of the packets carrying the frame/video slice are successfully delivered. A frame within a GOP (Group of Pictures) can only be decoded by the client in case all frames on which that frame depends are successfully received. Hence the groups of packets within the PDU Set have inherent dependency on each other in media layer. Without considering such dependencies between the packets, 5GS may perform a scheduling with low efficiency. For example, the 5GS may randomly drop a packet but try to deliver other packets of the same frame/video slice which are useless to the client and thus waste radio resources.

Also audio samples, haptics applications or remote control operations may benefit if the 5GS considers the PDU Set characteristics.

If such dependency between packets of a PDU Set (e.g. a frame/video slice) can be considered, it's possible to enhance the efficiency and promote user experience.

This key issue proposes to study PDU Set integrated packet handling in 5G network, in which the group of packets belongs to a same PDU Set will be handled in an integrated manner. The key issue includes the following aspects:

-	Which types of PDU Set (e.g. frame/video slice) shall be supported for PDU Set integrated packet handling by 5G network.

NOTE 1:	Coordination with SA WG4 is possibly needed.

-	What information should be provided to the 5GS regarding PDU Set for integrated packet handling, and how such information should be provided.

-	How the 5GS identifies that a PDU belongs to a specific PDU Set.

NOTE 2:	It is assumed that some header information necessary for the identification of PDUs and it is assumed that the corresponding information is not encrypted.

-	Whether and how to enhance the QoS model and policy control for PDU Set integrated packet handling.

-	Whether and how to perform the PDU Set integrated packet handling in the UE, RAN and/or UPF.

-	What information needs to be provided to the RAN and/or UPF to support PDU Set integrated packet handling in both the downlink and uplink direction.

-	Whether and how provide information to the UE for PDU Set integrated packet handling.

NOTE 3:	Coordination with RAN WGs may be needed.

- 5.5	Key Issue #5: Differentiated PDU Set Handling

- 5.5.1	Description

XR/media services are characterized by high data rate and low latency. In this release, it is anticipated that the 5GS QoS framework will be enhanced to support different QoS handling for PDU Set. PDU Sets can carry different content, e.g. I/B/P frames, slices/tiles within an I/B/P frame, etc.

This key issue proposes to support differentiated QoS handling considering different importance of PDU Sets e.g. by treating packets (i.e. PDUs) belonging to less important PDU Set(s) differently to reduce the resource wasting. The key issue includes the following aspects:

-	How does the 5GS identify the PDUs of one PDU Set.

-	How does the 5GS determine the importance information for a given PDU Set and/or dependency information between different PDU Sets.

-	Which network entity needs the importance/dependency information associated with the PDU Set and how it receives it.

-	Whether and how to enhance the QoS model and policy control for the importance/dependency information associated with a given PDU Set.

-	Definition of the importance/dependency information enabling differentiated PDU Set handling.

NOTE 1:	RAN related aspects will be coordinated with RAN WGs.

NOTE 2:	Aspects related to how PDU Sets may depend on each other will be coordinated with SA WG4.

- 5.8	Key Issue #8: Enhancements to power savings for XR services

- 5.8.1	Description

The objective of this Key Issue is to study whether and how to improve power management schemes like CDRX to achieve the best trade-off among key performance indicators like latency and device battery lifetime.

Solutions for this Key Issue rely on the enhancements to power saving schemes to support XR services studied during RAN1 SI on XR evaluation in Release 17 and under investigation in Release 18 and builds on top of those features.

For this Key Issue the following areas should be studied:

-	Which information (e.g. XR/media traffic characteristics, traffic pattern and statistics), if any, is needed by the RAN from the CN and/or the UE to enhance power management (i.e. CDRX). If needed, how such information is collected.

NOTE:	This Key issue requires collaboration with RAN WGs.

- 5.9	Key Issue #9: Trade-off of QoE and Power Saving Requirements

- 5.9.1	Description

The more efficient codec requires more power consumption and higher reliability. Also, the higher throughput and lower latency budget also requires more power consumption and more network resource. Hence, trade-off of QoE and device power consumption needs to be considered.

This key issue aims at addressing the following points for XRM:

-	How to support trade-off of throughput/latency/reliability and power consumption (e.g. device battery life).

NOTE:	Coordination with RAN WGs may be needed for the above bullet.