3GPP TSG RAN WG1 #110bis-e	R1-2208363

e-Meeting, October 10th - October 19th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation of SL positioning

Agenda item:	9.5.1.1

Document for:	Discussion and Decision

# 1	Introduction

At RAN#94, a new study item “Study on expanded and improved NR positioning” (FS_NR_pos_enh2) was approved ‎[1]; the WID was updated in [2]. One of the objectives is relevant for the present agenda item:

 Study solutions for sidelink positioning considering the following: [RAN1, RAN2]  Scenario/requirements o Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage o Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 o Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) o Spectrum: ITS, licensed  Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]  Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].  Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] o Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] o Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] o Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] o Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#98 to see if sufficient information is available for this review.

In this contribution, we provide initial evaluation results for SL positioning.

# Evaluation Methodology

In the previous RAN1 meeting, there have been the following agreements made:

Agreement For SL positioning evaluation in IIOT use case, companies should report how to drop anchor UEs and how to select anchor UEs Agreement Adopt the tables in section 3 of R1-2207606 as templates to collect SL positioning simulation results from each company. Agreement For SL positioning evaluation purpose, the following assumptions are further adopted - Companies should report whether SL-PRS and other SL signals are FDMed or not FDMed, and whether other SL signals are present - Adopting system level simulations (rather than the link level simulations) as the baseline tool - For SL positioning evaluation in highway scenario or urban grid scenario, the performance metrics can include absolute horizontal accuracy, relative horizontal accuracy, ranging with distance accuracy, and ranging with direction accuracy (optionally). - In highway and urban grid scenarios, companies can further consider other UE types, e.g. pedestrian UE or VRU devices. Agreement In the evaluation, relative positioning or ranging is performed between two UEs within X m, where X value(s) are reported by companies, and companies should also report the minimum distance used in the evaluations for each use case. The assumption used for X will be included in the TR for each set of results.

In addition to the agreements listed above, the Earlier Agreements section below lists the agreements reached in the RAN1 #109-e meeting [3][4].

# Evaluation Results

Good progress was made at RAN1#109 and RAN1#110 to agree on simulation assumptions and scenarios. In this contribution we will present the evaluation results for the IIoT use case regarding the accuracy of absolute positioning. To produce the results, the agreed assumptions are followed unless specifically mentioned otherwise. The detailed parameters used for the simulations are reported in Table 2.

For this initial investigation, we focused on SL-only positioning in an InF scenario where UEs are randomly dropped in the hall. For computing the position of each target UE, the following steps are followed:

- A fixed number of LoS anchor nodes is selected by choosing the ones with smallest ToA to the target UE.

- The RSTD measurements are made using Ts/4 level of granularity (oversampling 4x).

- The SL TDOA method (DL-TDOA-like, where anchors transmit SL PRS to the target UE) is applied to the RSTD measurements and the Taylor Series expansion method (based on the least squares algorithm) is used to solve the positioning equations, as described in [13].

In order to select how many anchor UEs to use for positioning, in Figure 1 we show the CDF of the horizontal absolute positioning error for a different number of anchor UEs with the InF-SH channel model. It can be noticed that performance improves as the number of anchors increases and we meet the requirement of set A (1m@90%) with 10 anchor nodes or more. However, using more than 12 anchors does not bring a relevant performance gain. Consequently, we chose to use 12 anchors in the remaining part of the evaluations.

Figure 1 – CDF of horizontal positioning error for a different number of anchor UEs with the InF-SH channel model.

In Table 1, we evaluate the SL TDOA method in scenarios InF-SH and InF-DH, for two different SL PRS bandwidths of 40 MHz and 100 MHz. We report the 50th, 67th, 80th, and 90th percentiles of horizontal absolute positioning error, and whether Set A and Set B requirements are met for each case.

Table 1 - Simulation results for IIOT for absolute positioning - horizontal accuracy.

| Case ID and brief description | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case #1, BW=40MHz, InF-SH | 0.398 | 0.622 | 0.996 | 1.594 | No, 80.1% of UEs satisfying the target positioning accuracy requirement | No, 20.6% of UEs satisfying the target positioning accuracy requirement |
| Case #2, BW=100MHz, InF-SH | 0.196 | 0.279 | 0.488 | 0.834 | Yes | No, 51.5% of UEs satisfying the target positioning accuracy requirement |
| Case #3, BW=40MHz, InF-DH | 0.348 | 0.508 | 0.696 | 1.076 | No, 88.4% of UEs satisfying the target positioning accuracy requirement | No, 24.9% of UEs satisfying the target positioning accuracy requirement |
| Case #4, BW=100MHz, InF-DH | 0.192 | 0.265 | 0.462 | 0.811 | Yes | No, 53.9% of UEs satisfying the target positioning accuracy requirement |

As could be seen from Table 1, Set A requirements (error less than 1m@90%) can be met only with 100 MHz of SL PRS bandwidth in both InF-SH and InF-DH scenarios, using the positioning technique described above. Whereas Set B requirements (error less than 0.2m@90%) cannot be achieved.

Observation 1: For both IIoT InF-SH and InF-DH absolute positioning with SL-only SL-TDOA, the accuracy of less than 1m@90% (requirement Set A) can be achieved only with 100 MHz bandwidth, while the accuracy of less than 0.2m@90% (requirement Set B) cannot be achieved.

# Conclusions

In this contribution we presented initial evaluation results for SL positioning and made the following observations:

Observation 1: For both IIoT InF-SH and InF-DH absolute positioning with SL-only SL-TDOA, the accuracy of less than 1m@90% (requirement Set A) can be achieved only with 100 MHz bandwidth, while the accuracy of less than 0.2m@90% (requirement Set B) cannot be achieved.

# References

- RP-213588, “Study on expanded and improved NR positioning”

- RP-222616, “Study on expanded and improved NR positioning”

- R1-2205227, “Summary #1 of [109-e-R18-Pos-03] Email discussion on evaluation of SL positioning”

- R1-2205228, “Summary #2 of [109-e-R18-Pos-03] Email discussion on evaluation of SL positioning”

- 3GPP TR 37.885: "Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR"

- 3GPP TR 36.885: "Study on LTE-based V2X Services"

- 3GPP TR 36.843: “Study on LTE Device to Device Proximity Services; Radio Aspects”

- 3GPP TR 38.845: “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases”

- 3GPP TR 38.901: “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”

- 3GPP TR 38.855: “Study on NR positioning support”

- 3GPP TR 38.857: “Study on NR Positioning Enhancements”

- “A Simple and Efficient Estimator for Hyperbolic Location”, Y. T. Chan and K. C. Ho

# Appendix

## Simulation Assumptions

Table 2 - Simulation assumptions.

| Scenario parameters |  |  |
| --- | --- | --- |
| Carrier frequency, GHz |  | 3.5GHz |
| Bandwidth, MHz |  | 40MHz, 100MHz |
| Subcarrier spacing, kHz |  | 30kHz |
| PHY/link level abstraction |  | Explicit simulation of all links, individual parameters estimation is applied. SL signals other than SL PRS are not present. |
| UE RX and TX timing error |  | No synchronization error. |
| Channel model |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: 300x150 m InF-DH: 120x60 m |
|  | Room height | 10m |
| Penetration loss |  | 0dB |
| Number of floors |  | 1 |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} |
| UE model parameters |  |  |
| UE noise figure, dB |  | 9dB – Note 1 |
| UE TX power, dBm |  | 23dBm – Note 1 |
| UE antenna configuration |  | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) |
| UE antenna radiation pattern |  | Omni, 0dBi |
| UE horizontal drop procedure |  | 180 UEs are randomly distributed in the simulation area |
| UE antenna height |  | 1.5m |
| UE mobility |  | 3km/h |
| Min UE-UE distance (2D), m |  | 1m |
| Positioning parameters |  |  |
| Positioning method |  | Taylor Series (based on least squares algorithm) |
| Description of Measurement Algorithm |  | Thresholding 0.5, (Oversampling x4) |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) |  | Comb-6, 6 symbol NR PRS |
| Number of anchor nodes |  | 12 |
| Selection of anchor nodes |  | LoS anchor nodes with smallest ToA to target UE |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |  |

# Earlier Agreements

Agreement

For SL positioning evaluation in highway and urban grid scenarios, UE dropping option A defined in section 6.1.2 of TR 37.885 is used, i.e.

	UE dropping option A is used for the highway scenario:

o	Vehicle type distribution: 100% vehicle type 2.

o	Clustered dropping is not used.

o	Vehicle speed is 140 km/h in all the lanes as baseline and 70 km/h in all the lanes optionally.

	UE dropping option A is used for the urban grid scenario:

o	Vehicle type distribution: 100% vehicle type 2.

o	Clustered dropping is not used.

o	Vehicle speed is 60 km/h in all the lanes.

o	In the intersection, a UE goes straight, turns left, turns right with the probability of 0.5, 0.25, 0.25, respectively.

Agreement

For SL positioning evaluation in highway and urban grid scenarios, antenna model follows the description in TR 37.885 section 6.1.4.

	Vehicle UE option 1 is the baseline (Vehicle UE antenna is modelled in Table 6.1.4-8 and 6.1.4-9 in TR 37.885)

	Vehicle UE option 2 (two panels) can be optionally selected by companies

Agreement

For SL positioning evaluation in highway and urban grid scenarios, channel model follows description in TR 37.885 section 6.2.

Agreement

- ●	For SL positioning evaluation in highway and urban grid, the following simulation parameters are used for FR1

Evaluation parameters for SL positioning in FR1

| Parameters | Urban grid for eV2X | Highway for eV2X |
| --- | --- | --- |
| Carrier frequency | Uu : 4 GHz SL: 6 GHz | Uu : 2 GHz or 4GHz SL: 6 GHz |
| BS Tx power | Macro BS: 49dBm | Macro BS: 49dBm |
| UE Tx power | Vehicle UE or UE type RSU: 23dBm | Vehicle UE or UE type RSU: 23dBm |
| BS receiver noise figure | 5dB | 5dB |
| UE receiver noise figure | 9 dB |  |

Agreement

	For SL absolute positioning evaluation in highway scenario, the following options are supported

○	Alt 1 as optional: BS and UE-type RSU deployment follows TR 36.885, where wrap around method of 19*3 hexagonal cells with 500m ISD in Figure A.1.3-3 of TR 36.885 section A.1.3 is used.

○	Alt 2 as baseline: BSs are disabled, UE-type RSUs are uniformly located with 200m spacing on both sides of highway symmetrically.

-	Optional: staggered/unsymmetrical UE-type RSU distribution like

	For SL absolute positioning evaluation in urban grid scenario, BS and UE-type RSU deployment follows the description in TR 36.885 section A.1.3.

○	Companies can provide additional BS/ UE-type RSU deployment, e.g. additional UE-type RSUs are added to UE-type RSU deployment in TR 36.885

Note: For absolute positioning in highway, Alt 1 is assumed for evaluation of joint Uu/SL positioning, Alt 2 is assumed for evaluation of SL only positioning.

Agreement

	For evaluation of relative positioning or ranging in highway scenario

○	BSs are disabled,

○	UE type RSU may be disabled (as baseline) or enabled (as optional)

-	If enabled, UE-type RSUs are uniformly located with 200m spacing on both sides of highway symmetrically.

□	Optional: staggered/unsymmetrical UE-type RSU distribution like

	For evaluation of relative positioning or ranging in urban grid scenario

○	BSs are disabled, or enabled (companies should report their assumption)

○	UE type RSU may be disabled or enabled (companies should report their assumption)

-	If enabled, UE type RSU deployment follows the description in TR 36.885 section A.1.3.

-	If enabled, companies can provide additional RSU deployment, e.g. additional RSUs are added to RSU deployment in TR 36.885

Agreement

- The following performance metrics for SL positioning accuracy evaluation is defined:

    - For relative and absolute positioning

        - horizontal accuracy

        - vertical accuracy

    - For ranging

        - Ranging for distance, i.e. accuracy of distance

        - Ranging for angle, i.e. accuracy of angle

- Companies are required to output

    - The percentiles of positioning accuracy error including 50%, 67%, 80%, 90% of UEs,

        - FFS others

    - And the CDF of positioning accuracy error

- Performance metrics other than positioning accuracy, such as PHY/end-to-end latency, are up to companies

Agreement

- For absolute positioning evaluation, anchor UEs’ locations are known

    - In the evaluation of SL only positioning

        - Anchor UEs are used to locate target UEs

    - In the evaluation of Joint Uu/SL positioning

        - Both BS and anchor UEs are used to locate target UEs

- In the evaluation, relative positioning or ranging is performed between two UEs within X m

    - FFS X which can be different for different scenarios, e.g. highway, urban grid, etc.

    - Companies can consider to provide simulation results based on multiple X values

- Positioning method should be reported by companies.

Agreement

For SL positioning evaluation,

- The existing pattern and sequence of DL-PRS or positioning SRS can be reused as baseline for evaluation purpose.

    - Companies should provide the description if other pattern and sequence are evaluated,

    - AGC settling time is considered by companies

- Explicit simulation of all links, individual parameters estimation is applied. Companies should provide description of applied algorithms for estimation of signal location parameters.

- As baseline for absolute positioning, sidelink anchors location coordinates are perfectly known.

    - Uncertainty in the sidelink anchors location coordinates can be considered by companies

- As baseline, Perfect synchronization between network and anchor UEs in the evaluation is assumed.

    - Network synchronization error and timing errors defined in TR 38.857 Table 6-1 can also be optionally used by companies for Synchronization between BS and BS, between BS and anchor UEs, and between anchor UEs.

Agreement

- For SL positioning evaluation, simulation bandwidths of 10, 20, 40 and 100 MHz in FR1 can be used.

- For SL positioning evaluation, simulation bandwidths of 100, 200 and 400MHz in FR2 can be used.

Agreement

For SL positioning evaluation, V2X use case with highway and urban grid scenarios defined in TR 37.885 is supported.

- The road configuration for urban grid and highway provided in TR 37.885 Annex A is reused

Agreement

- For SL positioning evaluation of Public safety use cases

    - Companies should provide detailed simulation assumptions including selected scenarios, channel models, center frequency, UE drop models, etc.

        - Evaluation methodology on channel model of TR 36.843 is reused,

            - Reuse the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with modification: Each component of channel model reuses what is specified in TR 38.901

        - Anchor UE height should be reported by companies, e.g. anchor UE height is the same as TRP

    - The performance metrics at least include absolute positioning accuracy and ranging with distance accuracy. Optionally, relative positioning accuracy or ranging with angle accuracy.

- For SL positioning evaluation of Commercial use cases

    - Companies should provide detailed simulation assumptions including selected scenarios, channel models, center frequency, UE drop models, etc.

        - Evaluation methodology on channel model of TR 36.843 is reused,

            - Reuse the parameters of “Channel models” specified in Section A.2.1.2 of TR 36.843 with modification: Each component of channel model reuses what is specified in TR 38.901

        - Anchor UE height should be reported by companies, e.g. anchor UE height is the same as TRP

    - The performance metrics at least include absolute positioning accuracy and ranging with distance accuracy. Optionally, relative positioning accuracy or ranging with angle accuracy

Agreement

For SL positioning evaluation for IIOT use cases, InF-SH and/or InF-DH defined in TR 38.857 are used

Agreement

For SL positioning evaluation on indoor factory scenarios, companies can select one of the following options for UE-2-UE channel model

- Option 1: BS-2-UE channel model defined in TR 38.901 is revised

  - The UE parameters in the channel model defined in 38.901, e.g. UE height, antenna model, transmit power are used to replace gNB’s corresponding parameters.

    - Anchor UE height should be reported by companies, e.g. anchor UE height is the same as TRP.

- Option 2: D2D channel mode from 36.843 A.2.1.2 is used

Agreement

For SL positioning evaluation on IIOT use case, the performance metrics at least include absolute accuracy and relative accuracy.

- FFS how to select anchor UEs/RSU for absolute positioning, e.g. 20 anchor UEs/RSU are randomly deployed in the simulation area