3GPP TSG RAN WG1 #110bis-e	                                                                           R1-2208660

e-Meeting, October 10th – 19th, 2022

Source:	vivo

Title:	Discussion on XR specific power saving enhancements

Agenda Item:	9.10.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #110 meeting, the following agreements on R18 XR specific power saving enhancement were agreed [1]:

Conclusion Conclude that “SFN wraparound mismatch” is a RAN2 issue. It can be left to RAN2 to address. RAN1 does not further study it. Agreement RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity Conclusion RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA. Conclusion All the proposed PDCCH monitoring adaptation/reduction schemes including those for jitter handling need to be compared against the Rel-17 PDCCH monitoring adaptation which is to be used as performance reference. Conclusion UE transmission and reception alignment for Issue 3-1 is deprioritized for power saving in Rel-18 XR SI. Conclusion RAN1 does not assume dynamic switch of different XR video data rates or frame rates for Rel-18 XR power saving study before further input is provided by SA. For future meetings Companies are encouraged to account the enhancement of CDRX to align with XR traffic periodicity in their further evaluations for XR power saving enhancements. Conclusion: - Companies are requested to use the Excel sheet attached with TR 38.838 in RP-213652 for recording the simulation results that are provided in their contributions.

In this contribution, as per the above conclusions and agreement, we further discuss the potential techniques for XR specific power saving enhancement, followed by our evaluation results.

- Clarification on the jitter assumption for R18 XR study

Currently how to provide jitter information to RAN is being discussed in SA2. If jitter statistics (e.g. range of jitter) can be provided by SA2, they will be useful to RAN for UE power savings, e.g. in configurating DRX on duration and start offset.

The detailed value of jitter range could be based on SA4 input. In R17 XR study, the jitter model was discussed in RAN1 #104-e and #104bis-e meetings based on the trace and configuration files provided in the LS from SA4 #111e-meeting [2]. The jitter distribution for R17 XR study derived from P-trace files by SA4 (S4aV200634 [3]) is shown in Table 1.

Table 1. Jitter distribution for R17 XR study derived from P-trace files by SA4 (S4aV200634)

|  | VR 2-1 | VR 2-3 |
| --- | --- | --- |
| Mean (ms) | 0 | 0 |
| STD (ms) | 2.77 | 2.76 |
| Min (ms) | -5.39 | -5.39 |
| Max (ms) | 5.08 | 5.45 |
| Jitter range (ms) | [-4.1, 4.3] | [-4.1, 4.43] |
| jitter range = [5%-tile in CDF, 95%-tile in CDF] ms |  |  |

According to the analysis on the traces from SA4 [2], for R17 XR evaluation, the following jitter distribution has been agreed to model and we here quote the Table 5.1.1.2-1 in TR 38.838 [4].

Table 5.1.1.2-1: Statistical parameters for jitter (TR 38.838)

| Parameter | unit | Baseline value for evaluation | Optional value for evaluation |
| --- | --- | --- | --- |
| Mean | ms | 0 |  |
| STD | ms | 2 |  |
| Truncation range | ms | [-4, 4] | [-5, 5] |

In SA4, XR traffic characteristics are being further studied in the ongoing SI FS_XRTraffic and a LS [5] on Status Update on XR Traffic was sent to RAN1 in SA4#113-e meeting.

And therefore, we notice that in the attached Permanent Document (S4-210614) from SA4 in the LS [5], the XR video traces and configurations files in SA4 study were updated. Since RAN1 already made agreement on adopting statistics model for the packet and jitter for XR evaluation by RAN1 #104-e, the updated of traces and configurations in the LS [5] was not taken into account for Rel-17 XR SI. In R18 XR SI, XR specific power saving and capacity improvement techniques to accommodate XR traffic characteristics including jitter will be studied. It is necessary to consider the realistic XR traffic characteristics based on SA4’s latest input to study XR-specific techniques.

We did some analyses based on the traces and configurations files according to the latest permanent document of TR26.926 [5]. Figure 1 shows the timing of V-trace, S-trace and P-trace according to SA4’s input.

Figure 11. Example of timing of V-trace, S-trace and P-trace

- Point A: Time stamp of a frame in V-trace

- Point B: Time stamp of the first slice for the frame in S-trace

- Point C: Time stamp of the last slice for the frame in S-trace

- Point D: Time stamp of the first IP packet for the frame in P-trace

- Point E: Time stamp of the last IP packet for the frame in P-trace

Figure 2 shows the distribution of delay from the frame generation time to the time when its last IP packet arrives at the RAN side, i.e., Point E – A for each frame, and Table 2 shows the corresponding distribution of jitter. It can be observed that based on the latest trace files, the maximum jitter is approximately ranging from -18ms to +18ms depending on the configurations, as shown in Appendix C. Considering the range truncated by 5%-tile and 95%-tile in CDF, the stable jitter range is around [-8 ms, +8ms], which is larger than the jitter range (i.e. [-4ms, 4ms]) agreed in R17 XR SI.

Figure 2. The statistic by using the P-trace data from SA4

Table 24. Jitter distribution deriving from the latest P-trace files provided by SA4

| Parameter | VR 2-1 | VR 2-2 | VR 2-3 | VR 2-4 | VR 2-5 | VR 2-6 | VR 2-7 | VR 2-8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mean (ms) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| STD (ms) | 4.96 | 5.00 | 4.85 | 4.89 | 5.31 | 4.02 | 5.19 | 5.18 |
| Min (ms) | -15.89 | -16.00 | -11.87 | -11.96 | -18.95 | -14.40 | -16.48 | -16.18 |
| Max (ms) | +11.27 | +10.30 | +10.10 | +10.51 | +18.69 | +8.08 | +11.59 | +11.13 |
| Jitter range (ms) | [-8.39, +7.89] | [-8.42, +7.91] | [-8.26, +7.69] | [-8.30, +7.77] | [-8.74, +8.42] | [-7.05, +6.24] | [-8.82, +8.19] | [-8.79, +8.20] |
| jitter range = [5%-tile in CDF, 95%-tile in CDF] ms |  |  |  |  |  |  |  |  |

It can be seen that the jitter range derived based on the latest P-trace files is different from that used for R17 XR study derived from P-trace files by SA4 (S4aV200634 [3]). By further checking the trace and configuration files, it seems that there are different parameters in the configurations, which may result in the differences between the latest P-trace and the previous ones.

We note that the traces from SA4 are generated based on typical configurations. Hence, it is reasonable to consider the XR traffic characteristics based on the latest P-trace files.

As analysed above, jitter model in accordance with the latest SA4’s inputs in TR 26.926 [5] needs to be considered for R18 XR study, e.g., [-8ms, +8ms]. We performed evaluation on the power saving schemes using both the R17 jitter the larger jitter range assumptions, as shown in the section 4. It is observed that there is negligible capacity loss with larger jitter range, and there could be different power saving gains with different jitter ranges. Details of evaluation can be found in section 4.

Observation 1: With larger jitter range, there is negligible capacity loss.

Proposal 1: For R18 XR study, jitter model in accordance with the SA4’s inputs in TR 26.926 should be considered, e.g. jitter range of [-8ms, +8ms].

- C-DRX enhancement for non-integer traffic periodicity

In the last e-meeting, a solution for enhancement of CDRX to align with XR traffic periodicity was agreed to be identified. Due to the regularity of XR traffic pattern, it is feasible enough to align DRX cycle with the non-integer traffic periodicity via semi-static approach. It should be noted that the higher layer signalling based DRX enhancements need to be studied in conjunction with RAN2. However, DCI-based CDRX adjustment seems to have no additional benefit for solving non-integer traffic periodicity. To make matters worse, adopting DCI-based approach to solve the non-integer periodicity issue will give rise to more control signalling overhead.

Observation 2: DCI-based approach as solution to non-integer periodicity issue will introduce more control signalling overhead and spec impact.

Additionally, in the previous meeting discussion, many companies held the view that the biggest benefit to study DCI-based CDRX adjustment is to solve both non-integer traffic periodicity issue and jitter issue. But, in accordance with the conclusion agreed in the last meeting, RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA. That is, DCI-based DRX adjustment is unavailable for jitter handling for the reason of unpredictable jitter. Having said that, the study on DCI-based CDRX adjustment should be deprioritized for R18 XR.

Proposal 2: Support semi-static based DRX enhancements to accommodate the non-integer XR traffic periodicity.

  - Semi-static alignment between C-DRX and XR traffic periodicity

According to the XR traffic frame rate, we can convert the unit of FPS into the unit of millisecond. Taking the typical 60FPS as an example, by simplifying the numerator and denominator to interprime, there appears an integer value which is multiple of traffic periodicity, i.e., 50ms. During the 50ms, three traffic bursts (e.g., frame) will arrive.

60 frames1000 ms=3 frames50 ms for 60FPS

As shown in Approach 1 of Figure 3, a DRX cycle set/pattern can be configured by the network. Within a DRX cycle set, there can be multiple DRX cycles with same or different periodicities. For example, there are three DRX cycles and the length of them are {16ms, 17ms, 17ms}, respectively. This DRX pattern can be repeated throughout the time domain. Besides, in Approach 2 of Figure 3, multiple DRX configurations can be another way to solve the mismatch issue. The length of DRX cycle for all the DRX configurations are the same and equal to an integer value, e.g. 50ms. And the difference between the multiple DRX configurations is only the values of drx-startoffset, which aims to match the three frames’ arrival time within per integer DRX cycle. In addition, another potential approach shown in Approach 3 of Figure 3 is to configure three DRX ondurations per integer DRX cycle to match every three frames’ arrival. Actually, both these approaches can solve the mismatch issue and achieve the same power saving effect, but different spec modification workload may cause. In our view, RAN2 should be in charge of the specific spec design for this part.

Figure 3. Three DRX configuration approaches to solve the mismatch issue

Observation 3: The following potential semi-static solutions to align DRX cycle with XR traffic periodicity can be considered.

- Approach 1: Configure DRX cycle set/pattern and each DRX cycle set contains multiple DRX cycles e.g., {16ms, 17ms, 17ms}. And apply the DRX cycle set cyclically in the time domain.

- Approach 2: Multiple DRX configurations with different drx-StartOffset values.

- Approach 3: Single DRX configuration and one DRX cycle can contain multiple DRX ondurations.

- C-DRX and PDCCH monitoring enhancement for jitter handling

According to the conclusions in the RAN1#110 meeting, RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA. Hence, the potential jitter handling can only depend on the statistics jitter information.

In this section, based on the assumption of statistics jitter information, the following jitter handling schemes are discussed.

- Existing R17 PDCCH monitoring adaptation

- Low power-wake up signal (LP-WUS) based jitter handling

- Other potential jitter handling schemes

  - R17 PDCCH monitoring adaptation

Leveraging R17 search space set group (SSSG) switching indication, as depicted in Figure 4, to handling jitter, UE can be switched to the default SSSG with sparse PDCCH monitoring periodicity via the expiration of searchSpaceSwitchTimer-r17 or explicitly SSSG switching indication. Once UE detect a scheduling PDCCH (which implicitly indicate the arrival of the traffic burst), UE can automatically switch to dense SSSG for data transmission. After that, a PDCCH skipping indication can be indicated to UE at a proper time. In this sense, the existing R17 SSSG switching indication can  reduce the unnecessary power consumption caused by jitter.

Figure 4. Jitter handling method based on R17 PDCCH monitoring adaptation

Observation 4: R17 PDCCH monitoring adaptation can reduce unnecessary power consumption caused by jitter.

  - LP-WUS based jitter handling

In order to totally reduce the unnecessary power consumption caused by jitter, replacing PDCCH monitoring with LP-WUS detection before data arrival can be studied, so that the UE will be on-demand waked up only when LP-WUS is detected.

As shown in Figure 5 (b), after aligning C-DRX cycle to XR traffic periodicity, to further reduce the unnecessary PDCCH monitoring during the jitter range (covered by DRX onduration), LP-WUS monitoring occasions (MOs) need to be configured within DRX active time. And UE will firstly detect LP-WUS on the LP-WUS MOs from the beginning of the DRX onduration. When XR traffics arrive at the network side, LP-WUS will be sent to wake up the UE. After detecting the LP-WUS, the UE just start to monitor PDCCH. Compared to the existing R17 search space set group (SSSG) switching depicted in Figure 5 (a), by replacing the PDCCH monitoring on the sparse SSSG with LP-WUS detection, UE will have the opportunity to maintain longer sleep therefore achieve power savings.

Figure 5. Jitter handling method based on LP-WUS

- Performance evaluation for LP-WUS scheme

As mentioned above, we further perform the system level evaluations for R17 PDCCH monitoring adaptation and LP-WUS schemes based on the assumption detailly provided in Appendix B. Besides, more detailed configurations for each schemes and the full simulation results can be founded in Appendix A.

For the application of LP-WUS in RRC connected mode, the design target of power consumption for LP-WUS monitoring should be no larger than half of that for PDCCH monitoring, i.e. the power consumption during the LP-WUS monitoring state should be 50 power units/slot or less. The specific design for LP-WUS and receiver structure can be discussed further, including using main receiver or separate receiver for LP-WUS monitoring. Taking the implementation of LP-WUS detection by separate receiver as an example, the power consumption of LP-WUS receiver can be in the micro Watt magnitude according to the investigation of LP-WUS receiver based on IEEE standard 802.11ba [6]. Due to the ultra-lightweight power of LP-WUS receiver, the power consumption of main receiver accounts for most of the total power consumption during LP-WUS monitoring. From latency perspective, due to the stringent PDB of XR traffic, the main receiver should not be completely powered off. Instead, it needs to be kept in micro or light sleep state during LP-WUS monitoring, so the total relative power for LP-WUS monitoring will be accordingly close to the relative power of micro or light sleep state i.e., 20 or 45 with no or less wake-up latency.

Figure 6. Power saving gain for R17 PDCCH monitoring adaption and LP-WUS schemes

Figure 7. System capacity for R17 PDCCH monitoring adaption and LP-WUS schemes

As shown in Figure 6, compared to always-On scheme, with configuring the enhanced DRX for non-integer traffic periodicity, R17 PDCCH monitoring adaptation scheme can obtain 10.98% ~ 19.28% power saving gain, while LP-WUS monitoring schemes with main radio enters micro sleep during LP-WUS monitoring can achieve 25% power saving gains.

Comparing with R17 PDCCH monitoring adaptation scheme, the LP-WUS monitoring schemes can additionally reduce {6%~14%} UE power consumption with no capacity loss as shown in Figure 7. This is because R17 PDCCH monitoring adaptation scheme is unable to totally avoid the unnecessary PDCCH monitoring due to the usage of sparse SSSG before data arrival.

It can be observed that the larger jitter range, the more unnecessary power consumption will be caused by R17 PDCCH monitoring adaptation scheme. The reason for that is the larger the jitter range is equivalent to the greater uncertainty of XR traffic arrival time, thus UE has to do more unnecessary PDCCH monitoring instead of saving power. On the contrary, for LP-WUS/WUR scheme, the jitter range does no impact on the UE power consumption actually.

Observation 5: Due to the usage of sparse SSSG before data arrival, there are still some unnecessary PDCCH monitoring that cannot be completely avoided by R17 PDCCH monitoring adaptation.

Observation 6: The power saving gain by R17 PDCCH monitoring adaptation will decrease with the increase of jitter range.

Observation 7: Compared to R17 PDCCH monitoring adaptation scheme, LP-WUS based jitter handling scheme can provide up to 14% additional power saving gain with no capacity loss.

Proposal 3: Capture LP-WUS as one candidate solution for jitter handling solutions in TR 38.835.

  - Others jitter handling schemes

As per the feature lead summary given based on the discussion of last meeting [7], there are another two potential jitter handling schemes, two-stage CDRX scheme proposed by [8] and additional On-Duration scheme proposed by [9]. In the following, we perform the evaluations for these two schemes and compared them with the existing R17 PDCCH monitoring adaptation scheme respectively. The simulation assumptions are provided in Appendix B.

- Performance evaluation for Two-stage CDRX scheme

Table 3 provides the performance comparison of the two-stage CDRX proposed by [8] and R17 PDCCH monitoring adaptation scheme. Compared to the existing R17 PDCCH monitoring adaptation, two-stage CDRX scheme will result in 1.84% ~ 5.48% additional UE power consumption due to the lack of flexible PDCCH monitoring adaptation. Besides, two-stage CDRX also obtains a little worse capacity than R17 PDCCH monitoring adaptation in high load case.

| Power saving scheme | DRX configuration (Cycle_ODT_IAT) ms | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Power saving gain (%) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AlwaysOn | - | 5 | 10 | 100% | - |  |
| R17 PDCCH monitoring adaptation | 16.67_10_4 | 5 | 10 | 100% | 23.36% | Note1 |
|  | 16.67_10_4 | 5 | 10 | 100% | 25.39% | Note2 |
| Two-stage CDRX | Outer CDRX: 16.67_10_4 Inner CDRX: 4_2_4 | 5 | 10 | 100% | 21.52% | Note3 |
| AlwaysOn | - | 10 | 10 | 92.50% | - |  |
| R17 PDCCH monitoring adaptation | 16.67_10_4 | 10 | 10 | 92.22% | 19.28% | Note1 |
|  | 16.67_10_4 | 10 | 10 | 88.52% | 21.84% | Note2 |
| Two-stage CDRX | Outer CDRX: 16.67_10_4 Inner CDRX: 4_2_4 | 10 | 10 | 86.89% | 16.36% | Note3 |
| Note1: doing PDCCH monitoring every 2 slots in sparse SSSG before data arrive Note2: doing PDCCH monitoring every 4 slots in sparse SSSG before data arrive Note3: 4ms CDRX cycle and 2ms ODT for inner CDRX |  |  |  |  |  |  |

Table 3. Evaluation results in FR1 DL Indoor Hotspot with 30Mbps traffic model

Observation 8: Compared to the existing R17 PDCCH monitoring adaptation scheme, two-stage CDRX scheme has slightly worse performance results in terms of power saving gain and capacity.

- Performance evaluation for additional On-Duration scheme

Table 4 provide the performance comparison between the additional On-Duration scheme proposed by [9] and R17 PDCCH monitoring adaptation scheme. Compared to the existing R17 PDCCH monitoring adaptation, additional On-Duration scheme will cause 4.6% additional UE power consumption due to the lack of flexible PDCCH monitoring adaptation. Besides, the additional On-Duration scheme also obtains a little worse capacity than R17 PDCCH monitoring adaptation in high load case.

Table 4. Evaluation results in FR1 DL Indoor Hotspot with 30Mbps traffic model

| Power saving scheme | DRX configuration (Cycle_ODT_IAT) ms | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Power saving gain (%) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AlwaysOn | - | 5 | 10 | 100% | - |  |
| R17 PDCCH monitoring adaptation | 16.67_10_4 | 5 | 10 | 100% | 23.36% | Note1 |
| Additional On-Duration | 16.67_4_4 | 5 | 10 | 100% | 18.73% | Note2 |
| AlwaysOn | - | 10 | 10 | 92.50% | - |  |
| R17 PDCCH monitoring adaptation | 16.67_10_4 | 10 | 10 | 92.22% | 19.28% | Note1 |
| Additional On-Duration (4ms) | 16.67_4_4 | 10 | 10 | 91.49% | 14.68% | Note2 |
| Note1: doing PDCCH monitoring every 2 slots in sparse SSSG before data arrive Note2: Additional onduration length is 4ms |  |  |  |  |  |  |

Observation 9: Compared to the existing R17 PDCCH monitoring adaptation scheme, additional On-Duration scheme has slightly worse performance results in terms of power saving gain and capacity.

- PDCCH skipping and interaction with HARQ retransmission

In R17 power saving WI, due to limited time, PDCCH skipping interaction with HARQ retransmission was not supported. And as per the last meeting’s agreements for R17 power saving, if the network indicates a PDCCH skipping, the UE performs PDCCH skipping immediately regardless of whether drx-ReTransmissionTimer is started. That is, the UE does not resume PDCCH monitoring for retransmission during the whole skipping duration. Obeying this principle, for power saving purpose, one implementation as depicted in Option 1 of Figure 8 is that the network indicates PDCCH skipping in the DCI that schedules the initial PDSCH transmission of the last DL packet of an XR traffic burst, thus the UE will immediately skip PDCCH until the start of the next DRX onduration. By this way, if there is any NACK feedback due to decoding failure, the network cannot schedule the corresponding retransmission until the PDCCH skipping ends at the UE side. This will obviously cause come capacity impact because the failed packet without retransmission opportunities will likely be discarded due to exceeded PDB. To satisfy the capacity requirement, as shown in Option 2 of Figure 8, another implementation is that the network indicates PDCCH skipping after all the HARQ-ACK processes of transmissions have been completed. Since the PDCCH skipping indication can only be transmitted by scheduling DCI, the network has to schedule a ‘dummy PDSCH’ with the expectation of NACK from the UE. Although the capacity could be guaranteed by this way, plenty of unnecessary PDCCH monitoring will be caused accordingly as well.

Observation 10: The existing R17 PDCCH skipping indication will cause either capacity loss or unnecessary PDCCH monitoring.

Figure 812. Potential network implementation of PDCCH skipping indication using Rel-17 specification

Figure 9. Example of PDCCH skipping and interaction with HARQ retransmission

In order to achieve a better trade-off between power saving and capacity by adopting PDCCH skipping, it is necessary to tackle HARQ retransmission occurring in PDCCH skipping duration.

Two potential solutions for the interaction between PDCCH skipping and HARQ retransmission are provided in the following:

- Solution 1: As depicted in Figure 9, if there is any NACK feedback during the skipping duration, UE is required to resume the PDCCH monitoring for corresponding retransmission within the drx-retransmissiontimer.

- Solution 1-1: On top of solution 1, the network can dynamically enable or disable the HARQ interaction for the indicated PDCCH skipping, according to the remaining PDB, the importance of the transmission or the probability of decoding failure (e.g. link quality).

- Performance evaluation for PDCCH skipping and interaction with HARQ retransmission

The system level simulation results of PDCCH skipping and interaction with HARQ retransmission scheme and two implementations of R17 PDCCH skipping scheme are presented in Figure 10. Compared with the two implementations of R17 PDCCH skipping scheme, up to 13% additional power saving gain or expected capacity increase can be obtained by the HARQ interaction with PDCCH skipping. Hence, it is verified that much better trade-off between power saving and capacity can be achieved by it.

Figure 10. Power saving gain of VR/AR 30Mbps in InH scenario

Observation 11: Compared with the existing R17 PDCCH skipping indication, PDCCH skipping with HARQ retransmission interaction can obtain 13% additional power saving gain or expected system capacity increase.

Proposal 4: Support the PDCCH skipping with HARQ retransmission interaction by the following solutions:

- Solution 1: If there is any NACK feedback during the skipping duration, UE is required to resume the PDCCH monitoring for corresponding retransmission within the drx-retransmissiontimer.

- Solution 1-1: On top of solution 1, the network can dynamically enable or disable the HARQ interaction for the indicated PDCCH skipping, according to the remaining PDB, the importance of the transmission or the probability of decoding failure (e.g. link quality).

In additional, by adopting the schemes of both PDCCH skipping interaction with HARQ retransmission and LP-WUS jitter handling, 40.08%~43.84% and 36.07%~38.47% total power saving gains can be obtained in low load and high load, respectively, and without capacity loss. The detailed results are presented in the Table I in Appendix A.

Observation 12: By adopting the schemes of both PDCCH skipping interaction with HARQ retransmission and LP-WUS jitter handling, 40.08%~43.84% and 36.07%~38.47% total power saving gains can be obtained in low load and high load, respectively, and without capacity loss.

- Text proposal of TR 38.835

=========================Start of Text proposal of TR 38.835==============================

…Omitted part…

5.2	 Power Saving Techniques

5.2.1 Techniques for handling jitter

Potential PDCCH monitoring enhancement techniques for handling jitter in this study are summarized as follows,

- PDCCH monitoring adaptation enhancements based on Rel-17 PDCCH skipping and sparse/dense SSSG switching

The UE performs sparse PDCCH monitoring during the active time. Once a scheduling DCI is detected, the UE switches to dense PDCCH monitoring. In case indicated by gNB, e.g. by DCI scheduling the last packet of a burst, the UE will skip the PDCCH monitoring until the possible arrival time of the next burst.

- Low power wake-up signal (LP-WUS) triggered PDCCH monitoring

The UE performs LP-WUS detection during active time or outside active time, and triggers PDCCH monitoring once LP-WUS is received. The UE power consumption of LP-WUS monitoring is lower than that of PDCCH monitoring (e.g. 50% reduction) with limited wake-up latency.

5.2.2 Techniques for PDCCH skipping and interaction with HARQ retransmission

Potential enhancement techniques for PDCCH skipping and interaction with HARQ retransmission are summarized as follows,

- Solution 1: If there is any NACK feedback during the skipping duration, UE is required to resume the PDCCH monitoring for corresponding retransmission within the drx-retransmissiontimer.

- Solution 1-1: On top of solution 1, the network can dynamically enable or disable the HARQ interaction for the indicated PDCCH skipping, according to the remaining PDB, the importance of the transmission or the probability of decoding failure (e.g. link quality).

Annex A:

Evaluation Methodology

Rel-17 evaluation methodology for XR power saving captured in [4] is used as the baseline evaluation methodology for UE power saving evaluation of Rel-18 XR.

…Omitted part…

=========================End of Text proposal of TR 38.835==============================

Proposal 5: Capture the above text proposal into R18 XR TR 38.835.

- Conclusion

In this contribution, we provide our views on XR specific power saving enhancements with the following observations and proposals:

Observation 1: With larger jitter range, there is negligible capacity loss.

Observation 2: DCI-based approach as solution to non-integer periodicity issue will introduce more control signalling overhead and spec impact.

Observation 3: The following potential semi-static solutions to align DRX cycle with XR traffic periodicity can be considered.

- Approach 1: Configure DRX cycle set/pattern and each DRX cycle set contains multiple DRX cycles e.g., {16ms, 17ms, 17ms}. And apply the DRX cycle set cyclically in the time domain.

- Approach 2: Multiple DRX configurations with different drx-StartOffset values.

- Approach 3: Single DRX configuration and one DRX cycle can contain multiple DRX ondurations.

Observation 4: R17 PDCCH monitoring adaptation can reduce unnecessary power consumption caused by jitter.

Observation 5: Due to the usage of sparse SSSG before data arrival, there are still some unnecessary PDCCH monitoring that cannot be completely avoided by R17 PDCCH monitoring adaptation.

Observation 6: The power saving gain by R17 PDCCH monitoring adaptation will decrease with the increase of jitter range.

Observation 7: Compared to R17 PDCCH monitoring adaptation scheme, LP-WUS based jitter handling scheme can provide up to 14% additional power saving gain with no capacity loss.

Observation 8: Compared to the existing R17 PDCCH monitoring adaptation scheme, two-stage CDRX scheme has slightly worse performance results in terms of power saving gain and capacity.

Observation 9: Compared to the existing R17 PDCCH monitoring adaptation scheme, additional On-Duration scheme has slightly worse performance results in terms of power saving gain and capacity.

Observation 10: The existing R17 PDCCH skipping indication will cause either capacity loss or unnecessary PDCCH monitoring.

Observation 11: Compared with the existing R17 PDCCH skipping indication, PDCCH skipping with HARQ retransmission interaction can obtain 13% additional power saving gain or expected system capacity increase.

Observation 12: By adopting the schemes of both PDCCH skipping interaction with HARQ retransmission and LP-WUS jitter handling, 40.08%~43.84% and 36.07%~38.47% total power saving gains can be obtained in low load and high load, respectively, and without capacity loss.

Proposal 1: For R18 XR study, jitter model in accordance with the SA4’s inputs in TR 26.926 should be considered, e.g. jitter range of [-8ms, +8ms].

Proposal 2: Support semi-static based DRX enhancements to accommodate the non-integer XR traffic periodicity.

Proposal 3: Capture LP-WUS as one candidate solution for jitter handling solutions in TR 38.835.

Proposal 4: Support the PDCCH skipping with HARQ retransmission interaction by the following solutions:

- Solution 1: If there is any NACK feedback during the skipping duration, UE is required to resume the PDCCH monitoring for corresponding retransmission within the drx-retransmissiontimer.

- Solution 1-1: On top of solution 1, the network can dynamically enable or disable the HARQ interaction for the indicated PDCCH skipping, according to the remaining PDB, the importance of the transmission or the probability of decoding failure (e.g. link quality).

Proposal 5: Capture the above text proposal into R18 XR TR 38.835.

# References

- Chairman’s Notes of RAN1 #109-e v0.3.0, RAN1 #109-e, May 9 - 20, 2022.

- R1-2101765, LS to on XR-Traffic Models, SA4 #111e, Nov.11-20, 2020.

- S4aV200634, 	[FS_XRTraffic] Traces and Configurations for VR2, CG and AR2, Qualcomm Incorporated, Mediatek.

- 3GPP TR 38.838 V17.0.0 (2021-12), Study on XR (Extended Reality) Evaluations for NR (Release 17).

- R1-2104023, LS on Status Update on XR Traffic, RAN1 #104bis-e meeting, E-meeting, 12 – 20 April 2021.

- 802.11ba-2021 - IEEE Standard for Information Technology--Telecommunications and Information Exchange between Systems--Local and Metropolitan Area Networks-Specific Requirements--Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications - Amendment 3: Wake-Up Radio Operation.

- R1-2207833, Final Moderator Summary on XR power saving, Qualcomm Incorporated, RAN1 #110, 22 – 26 August 2022.

- R1-2205916, Discussion on power saving enhancements for XR, Ericsson, RAN1 #110, 22 – 26 August 2022.

- R1-2206328, Discussion on XR specific power saving techniques, OPPO, RAN1 #110, 22 – 26 August 2022.

# Appendix A – Evaluation results for power saving schemes

Based on the power saving schemes analyzed above, we further provide our detailed simulation results in the form of table in this section. For the system-level simulation of the power saving mechanism, the power saving gain and the system capability should be jointly taken into consideration, and the detailed evaluation methodology and traffic model are defined in TR38.838 [4]. The DL single-stream video of VR/AR application with 60FPS, 30Mbps/45Mbps is evaluated for the FR1 indoor hotspot scenario. The overall simulation assumptions and traffic models used can be found in Appendix B.

AlwaysOn (i.e., without adopting any power saving schemes) is assumed as the baseline scheme. Besides, the R15/16 DRX configuration (i.e., R15/16 DRX) is also evaluated for reference. Due to non-integer XR traffic periodicity, to guarantee system capacity performance, the extremely long length of drx-onDurationTimer should be configured. As we mentioned before, to align non-integer DRX cycle with traffic arrival, the enhanced DRX scheme is configured. After configuring the enhanced DRX, R17 PDCCH monitoring adaptation scheme and LP-WUS scheme are also evaluated for jitter handling, under different jitter range. In additional, PDCCH skipping and interaction with HARQ retransmission scheme which can further reduce unnecessary PDCCH monitoring is also simulated. The corresponding simulation results are listed in Table I.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is also configured and the length of DRX onduration is equal to the jitter range.

- PDCCH skipping is indicated in the DCI scheduling the initial PDSCH transmission or scheduling a dummy PDSCH after all the HARQ-ACK processes of transmissions have been completed.

For LP-WUS monitoring scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing the LP-WUS monitoring.

- Based on the analysis in Section 4.3, in simulation, the total relative power for LP-WUS monitoring is assumed as 45 power units.

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring is set to 1 slot.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is also configured and the length of DRX onduration is equal to the jitter range.

For PDCCH skipping and interaction with HARQ retransmission scheme, the key assumptions are introduced below:

- Based on R17 PDCCH skipping adaptation scheme.

- PDCCH skipping indication is carried by DCI that schedules the last initial HARQ transmission of the data in one traffic period or schedules HARQ retransmission.

- if there is any NACK feedback during the skipping duration, UE is required to resume the PDCCH monitoring for corresponding retransmission within the drx-retransmissiontimer.

Table I. Power consumption results in FR1 DL Indoor Hotspot with 30Mbps traffic model

| Power saving scheme | DRX configuration (Cycle_ODT_IAT ms) | avg # UEs/ cell = N1 | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell = N1 | Power saving gain (%) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| AlwaysOn | - | 5 | 10 | 100% | - | Note1 |
| R15/16 DRX | 16_14_4 | 5 | 10 | 100% | 3.67% | Note1 |
|  | 10_8_4 | 5 | 10 | 100% | 5.72% | Note1 |
|  | 4_3_1 | 5 | 10 | 100% | 4.63% | Note1 |
|  | 16_8_4 | 5 | 10 | 11.67% | 19.71% | Note1 |
|  | 10_5_2 | 5 | 10 | 78.33% | 15.41% | Note1 |
|  | 10_4_2 | 5 | 10 | 52.22% | 22.17% | Note1 |
| Enhanced DRX | 16.67_8_4 | 5 | 10 | 100% | 13.05% | Note1 |
| R17 PDCCH monitoring adaptation | 16.67_8_4 | 5 | 10 | 100% | 23.36% | Note1,8 |
|  | 16.67_8_4 | 5 | 10 | 100% | 18.73% | Note2,8 |
|  | 16.67_8_4 | 5 | 10 | 100% | 14.64% | Note3,8 |
|  | 16.67_8_4 | 5 | 10 | 100% | 11.44% | Note4,8 |
| LP-WUS scheme | 16.67_8_4 | 5 | 10 | 100% | 30.21% | Note1,5,8,9 |
|  | 16.67_8_4 | 5 | 10 | 100% | 28.89% | Note2,5,8,9 |
|  | 16.67_8_4 | 5 | 10 | 100% | 28.29% | Note3,5,8,9 |
|  | 16.67_8_4 | 5 | 10 | 100% | 29.23% | Note4,5,8,9 |
|  | 16.67_8_4 | 5 | 10 | 100% | 43.84% | Note1,6,9 |
|  | 16.67_8_4 | 5 | 10 | 100% | 40.08% | Note3,6,9 |
| Interaction with HARQ retransmission | 16.67_8_4 | 5 | 10 | 100% | 37.13% | Note1,7 |
|  | 16.67_8_4 | 5 | 10 | 100% | 26.98% | Note3,7 |
| AlwaysOn | - | 10 | 10 | 92.50% | - | Note1 |
| R15/16 DRX | 16_14_4 | 10 | 10 | 91.81% | 3.46% | Note1 |
|  | 10_8_4 | 10 | 10 | 91.25% | 5.10% | Note1 |
|  | 4_3_1 | 10 | 10 | 91.68% | 4.03% | Note1 |
|  | 16_8_4 | 10 | 10 | 2.78% | 18.21% | Note1 |
|  | 10_5_2 | 10 | 10 | 45.00% | 13.10% | Note1 |
|  | 10_4_2 | 10 | 10 | 22.50% | 18.70% | Note1 |
| Enhanced DRX | 16_8_4 | 10 | 10 | 91.94% | 10.08% | Note1 |
| R17 PDCCH monitoring adaptation | 16.67_8_4 | 10 | 10 | 92.22% | 19.28% | Note1,8 |
|  | 16.67_8_4 | 10 | 10 | 92.16% | 14.96% | Note2,8 |
|  | 16.67_8_4 | 10 | 10 | 91.01% | 10.98% | Note3,8 |
|  | 16.67_8_4 | 10 | 10 | 91.01% | 8.94% | Note4,8 |
| LP-WUS scheme | 16.67_8_4 | 10 | 10 | 92.22% | 25.10% | Note1,5,8,9 |
|  | 16.67_8_4 | 10 | 10 | 92.20% | 24.08% | Note2,5,8,9 |
|  | 16.67_8_4 | 10 | 10 | 91.11% | 24.11% | Note3,5,8,9 |
|  | 16.67_8_4 | 10 | 10 | 91.11% | 25.93% | Note4,5,8,9 |
|  | 16.67_8_4 | 10 | 10 | 92.22% | 38.47% | Note1,6,9 |
|  | 16.67_8_4 | 10 | 10 | 92.10% | 36.07% | Note3,6,9 |
| Interaction with HARQ retransmission | 16.67_8_4 | 10 | 10 | 92.22% | 32.18% | Note1,7 |
|  | 16.67_8_4 | 10 | 10 | 92.20% | 22.93% | Note3,7 |
| Note1: jitter range = [-4, +4]ms Note2: jitter range = [-6, +6]ms Note3: jitter range = [-8, +8]ms Note4: jitter range = [-10, +10]ms Note5: with R17 PDCCH skipping indication Note6: with R17 PDCCH skipping indication and interaction with HARQ retransmission Note7: based on SSSG switching (doing PDCCH monitoring every 2 slots in sparse SSSG before data arrive) and R17 PDCCH skipping Note 8: PDCCH skipping is indicated in the DCI that schedules a dummy PDSCH after all the HARQ-ACK processes of transmissions have been completed Note 9: The total relative power for LP-WUS monitoring is 45 |  |  |  |  |  |  |

# Appendix B – Simulation assumptions

Table II. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table III.  The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| Packet delay budget (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 |  |
| Jitter Range (ms) | [-4, 4] |  |

# Appendix C –VR2: XR Split Rendering

The following is a quote from the latest SA4’s inputs in permanent document of TR26.926 [5].

| Configuration | Basic Content Parameters |
| --- | --- |
| VR2-1 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-2 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |
| VR2-3 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, 1500 byte max packet size packets |
| VR2-4 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s CBR with window 1 frame, buffer sent at same time, unlimited packet size |
| VR2-5 | 1 slice per eye buffer, every 8th frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-6 | 8 slices per eye buffer, 1 slice per frame is intra coded, 30Mbit/s capped VBR with window 200ms, buffers sent interleaved, 1500 byte max packet size |
| VR2-7 | 8 slices per eye buffer, 1 slice per frame is intra coded, ++/*45Mbit/s capped VBR with window 200ms, buffer sent at same time, 1500 byte max packet size |
| VR2-8 | 8 slices per eye buffer, 1 slice per frame is intra coded, 45Mbit/s capped VBR with window 200ms, buffer sent at same time, unlimited packet size |