3GPP TSG RAN WG1 #110bis-e                                                              R1-2208852

e-meeting, October 10th – 19th, 2022

Source:	OPPO

Title:	Evaluation methodology and preliminary results on AI/ML for beam management

Agenda Item:	9.2.3.1

Document for:	Discussion

Introduction

The SID [1] of artificial intelligent (AI) and machine learning (ML) for NR air interface was agreed in RAN#94e meeting. The initial set of use cases including beam management was selected as followings

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

In this contribution, we present our evaluations on spatial and/or temporal domain beam prediction.

Evaluations and discussions

Use case of beam management

According to the SID, the AI/ML beam management can be categorized into the following sub use cases, but not limited to

- Beam prediction in spatial domain for overhead/latency reduction

- Beam prediction in time domain for overhead/latency reduction

- Beam selection accuracy improvement

In this contribution, we evaluate the 1st two sub use cases, e.g. BM-Case1 and BM-Case2 as defined in RAN1#109e, with Top-K Tx-Rx beam pair(s) prediction. Moreover, the dense urban scenario with SLS has been agreed as the baseline in the agreements below. Additionally, we evaluate BM-Case1 for the indoor hotspot (InH) scenario as supplementary results in Appendix II.

Agreement

- For dataset construction and performance evaluation (if applicable) for the AI/ML in beam management, system level simulation approach is adopted as baseline

  - Link level simulation is optionally adopted

Agreement

- At least for temporal beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

- For spatial-domain beam prediction, Dense Urban (macro-layer only, TR 38.913) is the basic scenario for dataset generation and performance evaluation.

  - Other scenarios are not precluded.

Dense urban scenario for BM-Case1

Conventionally, NW broadcasts up to 64 SSBs periodically at FR2 for DL beam sweeping. Deployed with either single panel or multiple panels, UEs sweep all its Rx beams (typically 4 Rx beams) per panel to search the best DL beam pair(s). To fulfill the P1 procedure specified in Rel.15, large DL overhead and latency would be expected.

Intuitively, both NW and UE apply DL Tx and Rx beams respectively with certain pattern. For instance, NW carries out analog beamforming in both vertical and horizontal direction, e.g. 8 beams in horizontal domain and 4 beams in vertical domain (depicted in Figure 2). The combinations of vertical beam and horizontal beam can be seen by UE as total 32 DL Tx beams. Then the number of DL beam pairs in Set A is 128 (32 Tx beams and 4 Rx beams) as illustrated in Figure 1.

Assuming the beamforming pattern can be somehow learnt by neural network (NN), then only a limited number of DL measurement would be enough for NN to infer the quality of all DL beam pair links. The benefit comes from that a large portion of DL measurement of beam pairs can be replaced by beam prediction in spatial domain. This can be called as super-resolution problem.

One may wonder how the Tx beams for UE to measure are selected, i.e. part of Set B selection. We show our selection on Tx beams in Figure 2 where the highlighted circles in both vertical and horizontal domain. They are the most “welcomed” Tx beams among UEs within a cell. The selected Tx beams represent a spatial pattern of beamforming which are widely adopted by UEs. Surely, other Tx beams selection can be applied as well with certain rules and probably with even better performance. We didn’t try all the possibilities, and the intention here is just to show that this typical selection is workable.

- : DL beam sweeping procedure where for traditional beam sweeping 32 Tx beams and 4 Rx beams per panel and for AI/ML beam prediction 8 Tx beams and 4 Rx beams per panel

- : Only a subset of DL beams measured among all DL beams

Dense urban scenario for BM-Case2

For temporal domain beam prediction, the basic assumption can be found in Figure 3, where UE measures DL beam pairs in K time instances and predict beam pairs for next F continuous instances. For example, K = 4 and F = 4. Different from BM-Case1, the time domain beam prediction can be applied when UE is with low/medium mobility, e.g. 30km/h as agreed as baseline. To increase the prediction accuracy for moving UE, spatial consistency should be modeled as well.

- : Temporal domain beam prediction where K = 4 measurements instance and F = 4 prediction instance

AI/ML model training and inference

Data set generation

In our evaluation, the data set are generated by SLS as two parts. The 1st part is a subset of measured beam pairs (highlighted yellow in Figure 10) and the corresponding L1-RSRP. This can be used as input of the NN for training purpose. The 2nd part are the best (with highest L1-RSRP) beam pair index(es) among all the beam pairs and their L1-RSRP, which can be seen as labels to the NN model.

With the labelled data, NN can see which beam pair(s) is/are the best one(s) and then learn the relation or pattern between the limited measurements (as input) and the best one(s) (as output). The limited measurement refers to Set B for both BM-Case1 and BM-Case2. In other words, the Tx and Rx beam pattern in time or space domain can be somehow educated to the NN. Then even with a small portion of actual measurement in time and/or space domain, NN is able to infer which beam pair(s) are most likely to be optimal with the highest L1-RSRP.

Neural network models

For BM-Case1, we adopt two separate DNN (Deep Neural Network) models which are depicted in Figure 4. The inputs to both AI/ML models are the subset of DL measurements, i.e. L1-RSRP values of measured beam pairs in an order. Since the beam pair indexes of Set B is sequentially coupled with the model as inputs, the beam pair indexes can be implicitly captured by the NN models.

Specifically, the AI/ML model 1 is to predict the Top-K beam pair index(es) and the DNN structure with 3 hidden layers is depicted in Figure 5. The AI/ML model 2 is to predict the best L1-RSRP(s). These two AI/ML models are trained separately and the prediction of both models may not be perfectly aligned. Note that the best L1-RSRP predicted by AI/ML model 2 may not always correspond to the Top-K beam pair(s) predicted by AI/ML model 1. There could be a case that the predicted beam pair index is not correct, but the predicted L1-RSRP is quite close to the actual L1-RSRP of the genie-aided beam pair.

- : NN models to predict best beam pair index and the highest L1-RSRP separately

- : Beam pair prediction (model 1) implementation by using DNN model

For BM-Case2, we adopt the concatenation of LSTM (Long-Short Term Memory) + DNN models to predict Top-K beam pair(s) in temporal domain. As one may know, the LSTM model is good at handling the inference in time domain, i.e. remembering key information and forgetting non-essential information. Since both LSTM + DNN are well known in ML field and to save space, we omit the details of LSTM + DNN implementations in this contribution. For the combination of BM-Case1 and BM-Case2, i.e. spatial and temporal beam prediction, we also apply the LSTM + DNN model.

Beam pattern of Set B

In RAN1#110, the following agreement on Set B was achieved. In this contribution, we adopt Option 1 where Set B is fixed across training and inference phases. Compared with Option 2, fixed Set B may provide more stable performance on spatial and/or temporal beam prediction. Dynamically changing the input of model would require larger scale of training to adapt to variants of Set B. There seems no valid evidence that the performance can be boosted, thanks to such flexibility.

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

    - FFS on the beams of Set B

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference)

    - FFS on fixed or variable number of beams (pairs)

    - FFS on the details

  - Other options are not precluded.

  - FFS on the number of beams (pairs) in Set B

  - Note: This does not preclude the alternative that Set B is different from Set A.

- Apply fixed Set B across training and inference phases for BM-Case1 and BM-Case2.

Input of AI/ML models

In RAN1#110, the input of AI/ML models were intensively discussed and the “beam” types can be categorized into 3 different alternatives, e.g. 1) Tx beam prediction, 2) Rx beam prediction and 3) Tx-Rx beam pair prediction.

Agreement

For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams:

- Alt.1: DL Tx beam prediction

- Alt.2: DL Rx beam prediction

- Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

Note1: DL Rx beam prediction may or may not have spec impact

In our view, we would like to give Tx-Rx beam prediction the 1st priority, since this function can address beam prediction for both NW and UE side and perfectly aligned with P1 beam sweeping procedure defined in NR Rel.15. With such setting, either NW or UE would not necessarily to carry out the 2nd stage of beam sweeping to find proper corresponding Tx beam or Rx beam for the predicted Rx beam or Tx beam.

For Tx beam prediction, it aligned well with legacy NR beam measurement, reporting and indication. But as for the corresponding Rx beam, there are still at least two options. One option is to assume a best Rx beam, but how to determine such best Rx beam seems lack of clarity; The other option is to assume a fixed Rx beam which aligns with P2 beam sweeping for Tx beam refinement.

For Rx beam prediction, first we would like to note the Rx beam implementation is up to UE, there seems no strong reason for NW to predict UE’s Rx beam only. Not to mention that UE has to report its measurement on Set B. Hence, it seems Rx beam prediction would be more proper to be implemented at UE side. For instance, UE measures reduced NZP-CSI-RS resource set with Repetition ON, and then predicts the best Rx beam for the fixed Tx beam. In our understanding, the use case for this type of beam prediction would be very limited.

- For BM-Case1 and BM-Case2, at least support beam pair prediction (Alt.3) as the key feature of representative sub use cases.

Another key aspect of the model input is what to be input for training and inference. The inputs of AI/ML model for BM-Case1 and BM-Case2 were discussed as in the following conclusions. It seems most of alternatives include L1-RSRP measurement based on Set B as input, whereas the other one (i.e. Alt.3 in the 1st conclusion) includes CIR only.

Conclusion

Regarding the sub use case BM-Case1, further study the following alternatives for AI/ML input:

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt.2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companions in the discussion:  Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc.

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: CIR based on Set B

- Alt.4: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

Conclusion

Regarding the sub use case BM-Case2, further study the following alternatives of measurement results for AI/ML input (for each past measurement instance):

- Alt.1: Only L1-RSRP measurement based on Set B

- Alt 2: L1-RSRP measurement based on Set B and assistance information

  - FFS: Assistance information. The following were mentioned by companies in the discussion:, Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx  beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam pointing angles beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.) , increase ratio of L1-RSRP for best N beams, UE orientation information

    - Note: The provision of assistance information may be infeasible due to the concern of disclosing proprietary information to the other side.

- Alt.3: L1-RSRP measurement based on Set B and the corresponding DL Tx and/or Rx beam ID

- Note1: It is up to companies to provide other alternative(s) including the combination of some alternatives

- Note2: All the inputs are “nominal” and only for discussion purpose.

One issue is whether DL Tx and/or Rx beam ID should be input to NN model. It can be input in either explicit or implicit way. For explicit method, these beam IDs are directly used as inputs, along with L1-RSRP. As for implicit method, only L1-RSRP are input with proper order. Since the AI/ML model should be trained with fixed order of L1-RSRP as input, during inference phase the AI/ML model is still element-wise sensitive. In other words, each beam measurement from Set B should be put into the right position of the input tensor.

- The input of AI/ML model for beam prediction are element-wise sensitive, therefore the L1-RSRPs of Tx and/or Rx beams in Set B should be input in proper order.

Hence, with proper order of L1-RSRPs measurement of Set B, the Tx and/or Rx beam ID(s) seems not really necessary to be input.

- For BM-Case1 and BM-Case2, suggest to adopt L1-RSRP measurement based on Set B as input of AI/ML model.

In our evaluations in Section 2.4, it appears that the input based only on L1-RSRP can provide good enough performance in the aspects of beam prediction accuracy and L1-RSRP difference. It seems that any additional assistance information would only bring marginal performance benefits.

To sort various of the assistance information, we understand some of them relates to analog beam implementation at both sides, e.g. Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, and some information may trigger proprietary concern at UE side, e.g. UE position information, UE direction information, UE orientation information, etc. Hence, we intend to reuse the beam reporting content in NR, e.g. L1-RSRP and its beam index, which surely has no issues of exposing beamforming implementation and proprietary information.

- For the assistance information of BM-Case1 and BM-Case2, suggest to

  - Justify the performance benefits if assistance information

  - Study whether assistance information would expose beamforming implementation and proprietary information at NW or UE.

KPIs of AI/ML models

In legacy (prior to Rel.18) beam management, L1-RSRP has been widely-used as beam selection metric. With its features of simplicity and computation-friendly, it would be proper to serve as the metric for AI/ML-based beam prediction as well. In RAN1#109e, the following agreement was reached to evaluate the performance of spatial/temporal domain beam prediction.

Agreement

     To evaluate the performance of AI/ML in beam management, further study the following KPI options:

o   Beam prediction accuracy related KPIs, may include the following options:

  Average L1-RSRP difference of Top-1 predicted beam

  Beam prediction accuracy (%) for Top-1 and/or Top-K beams, FFS the definition:

     Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

     Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

  CDF of L1-RSRP difference for Top-1 predicted beam

  Beam prediction accuracy (%) with 1dB margin for Top-1 beam

     The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam “whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam”

  The definition of L1-RSRP difference of Top-1 predicted beam:

     the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

  Other beam prediction accuracy related KPIs are not precluded and can be reported by companies.

o   System performance related KPIs, may include the following options:

  UE throughput: CDF of UE throughput, avg. and 5%ile UE throughput

  RS overhead reduction at least for spatial-domain beam prediction at least for top-1 beam:

       1-N/M,

o   where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement

o   where (FFS) M is the total number of beams

o   Note: Non-AI/ML approach based on the measurement of these M beams may be used as a baseline

       FFS on whether to define a proper value for M for evaluation.

  Other System performance related KPIs are not precluded and can be reported by companies.

o   Other KPIs are not precluded and can be reported by companies, for example:

  Reporting overhead reduction: (FFS) The number of UCI report and UCI payload size, for temporal /spatial prediction

  Latency reduction:

  (FFS) (1 – [Total transmission time of N beams] / [Total transmission time of M beams])

       where N is the number of beams (with reference signal (SSB and/or CSI-RS)) in the input beam set required for measurement

       where M is the total number of beams

  Power consumption reduction: FFS on details

Beam prediction accuracy

Beam prediction accuracy based on L1-RSRP was established as a metric to be compared with baseline scheme. In system-level, CDF of throughput can be evaluated as a final check on performance. Moreover, overhead/latency reduction (main purpose of beam prediction) can be straightforwardly calculated.

However, there are options on how to measure the beam prediction accuracy. In our evaluation, Top-K beam pair(s) can be predicted which may or may not include the genie-aided best beam(s). If the genie-aided best beam is within the predicted Top-K beam(s), then a simple beam sweeping (among Top-K predicted beams) procedure can be triggered to find the best genie-aided beam. Hence, we tend to apply Option 2 in above agreement to evaluate the beam prediction accuracy.

- For beam prediction accuracy, adopt Option 2 (beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”) for AI/ML beam prediction.

Regarding L1-RSRP difference, it was defined in above agreement as the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam. But some AI/ML model implementation can predict the corresponding L1-RSRP of the Top-K predicted beam pair(s). If the inference is carried out by NW, then it’s necessary for NW to know the predicted L1-RSRP. Hence, we think there should be another L1-RSRP difference as KPI.

- Study another definition of L1-RSRP difference of Top-1 predicted beam

- The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

Overhead and latency reduction

For RS overhead reduction, the percentage of reduction should be evaluated with proper calculation. The basic rule to calculate the overhead reduction at least for BM-Case1 in above agreement is as 1-N/M, where N is the number of beams (with reference signal (SSB and/or CSI-RS)) required for measurement, i.e. the size of Set B; and M is the total number of beams, i.e. the size of Set A.

Apparently, the formula 1-N/M can somehow capture a rough evaluation on how much overhead can be saved by beam prediction in spatial domain. But to accurately evaluate the overhead reduction, we should take the whole beam prediction procedure into account. For instance, after the Top-K predicted beam(s) inferred by the model, it also depends on whether there is follow-up beam sweeping procedure.

One reason to carry out the 2nd stage of beam measurement is that for Tx/Rx beam prediction, the Top-K predicted beam(s) are not within Set B, therefore the corresponding Rx/Tx beam may be not available at UE side or NW side. However, this additional beam sweeping is not applicable for Tx-Rx beam pair prediction. To consider this factor more clearly, the exact beam setting should be clarified during discussion.

Another reason for the 2nd stage of beam measurement is to search for the best beam among Top-K (K>1) predicted beams. If the model is not quite confident on which one is the best predicted beam, then Top-K (K>1) can be inferred and verified via follow-up beam measurement focusing on the predicted K beams.

- For BM-Case1, study how to accurately capture the overhead, considering beam measurement on Set B and potential follow-up measurement after beam prediction.

As for BM-Case2, the temporal domain factor should be added based on that of spatial domain beam prediction. According the following agreement made in RAN1#109e, the overhead reduction should consider the T1 duration (measurement on Set B) and T2 duration (prediction among Set A).

If we could put the potential 2nd stage beam sweeping procedure aside, then the basic rule to calculate overhead reduction can be 1 - (T1*N)/((T1+T2)*M). If the size of Set B equals to that of Set A, i.e. N = M, then this rule can be further simplified as 1 – T1/(T1+T2).

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

- For BM-Case2, study how to capture the overhead reduction, considering the T1 duration (measurement on Set B) and T2 duration (prediction among Set A).

Baseline schemes

For the selection of baseline schemes, in RAN1#109e, the following agreements were reached for both spatial domain and temporal domain beam prediction. In our understanding, one of the key benefits of beam prediction is to significantly reduce overhead/latency. Hence, we think it’s fine to choose the baseline with best performance in the sense of beam selection.

Agreement

- For spatial-domain beam prediction, further study the following options as baseline performance

  - Option 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping)

    - FFS CSI-RS/SSB as the RS resources

  - Option 2: Select the best beam within Set A of beams based on the measurement of RS resources from Set B of beams

    - FFS: Set B is a subset of Set A and/or Set A consists of narrow beams and Set B consists of wide beams

    - FFS: how conventional scheme to obtain performance KPIs

    - FFS: how to determine the subset of RS resources is reported by companies

  - Other options are not precluded.

Agreement

- For temporal beam prediction, further study the following options as baseline performance

  - Option 1a: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources or all possible beams from Set A of beams at the time instants within T2

  - Option 2: Select the best beam for T2 within Set A of beams based on the measurements of all the RS resources from Set B of beams at the time instants within T1

    - Companies explain the detail on how to select the best beam for T2 from Set A based on the measurements in T1

  - Where T2 is the time duration for the best beam selection, and T1 is a time duration to obtain the measurements of all the RS resource from Set B of beams.

    - T1 and T2 are aligned with those for AI/ML based methods

  - Whether Set A and Set B are the same or different depend on the sub-use case

  - Other options are not precluded.

- For spatial domain beam prediction, select the best beam within Set A via exhaustive beam sweeping (Option 1) as baseline.

- For temporal domain beam prediction, select the best beam for T2 within Set A via exhaustive beam sweeping (Option 1a) as baseline.

Evaluation results

BM-Case1: Spatial domain beam prediction

In Table 1, we summarize the evaluation results from Top-1 to Top-4 spatial domain beam prediction. It is obvious that the more beam predicted, the higher probability that Top-K predicted beams include the genie-aided best beam. For this evaluation, from Top-1 to Top-4, the beam prediction accuracy increases from 79.15% to 97.95%. In addition, the gap between ideal L1-RSRP of predicted beam pair and that of genie-aided best beam pair is quite narrow, i.e. smaller than 0.5dB. In terms of SE, the SE of UE with Top-1 predicted beam is very close to the baseline scheme which is based exhaustive search to find the best beam pair.

Since there are total 128 beam pairs and UEs only measure 32 beam pairs for spatial domain beam prediction, the rough overhead reduction or latency reduction can be up to (1 – 32 beams / 128 beams) = 75%.

- BM-Case1 performance

|  | Baseline | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- | --- |
| Beam pair prediction accuracy |  | 79.15% | 92.45% | 95.90% | 97.95% |
| Avg. L1-RSRP estimate error |  | 0.41dB | 0.15dB | 0.07dB | 0.04dB |
| Spectrum efficiency (bits/s/Hz) | 1.534 | 1.500 | 1.519 | 1.529 | 1.534 |

- Spatial domain beam prediction can yield beam prediction accuracy (at least 80%) while overhead/latency reduction rate is 75%.

- The system level metric, i.e. spectrum efficiency or throughput, is not sensitive to the L1-RSRP difference introduced by spatial domain beam prediction.

Next, one may see the CDF of L1-RSRP gap of the Top-1 predicted beam as in Figure 6. To have a clear view on the CDF, we only count the L1-RSRP with wrong beam prediction.

- : CDF of the L1-RSRP gap for the Top-1 predicted beam

- For 80% of the incorrect spatial domain beam prediction cases, the L1-RSRP difference can be kept within 2dB.

Finally, let’s provide the CDF of spectrum efficiency (S.E. in bits/s/Hz) as system level performance for BM-Case1. In Figure 7, the spectrum efficiency of baseline scheme and Top-1 to Top-4 beam prediction are shown and one may find the performance are quite close.

We also note that as for scenario of dense urban (macro-layer only) with 200m ISD @FR2, a portion of UEs located close to cell edge (around 35%) are not well served due to high pathloss, and their S.E. are nearly zero.

- : S.E. CDF of baseline (upper bound) and Top-1 to Top-4

Moreover, since the beam predication accuracy is promising as shown in Table 1 (at least 80%) and L1-RSRP error is very marginal (lower than 1dB), the performance on S.E. of Top-1 to Top-4 are very closed to that of the upper bound baseline in Figure 7. We find the hint that if both beam prediction accuracy and L1-RSRP error are good, the comparison on system-level results, i.e. S.E. or throughput may only provide neglectable insight. So it is reasonable for us to focus on the KPI directly related to L1-RSRP.

- When beam prediction accuracy is high (at least 80%) and L1-RSRP difference is small (within 1 dB), the system-level performance, i.e. spectrum efficiency or throughput, may only provide non-essential insight, therefore focusing on L1-RSRP for beam prediction would be good enough.

BM-Case2: Temporal domain beam prediction

As for BM-Case2, the evaluation assumption can be found in appendix Table 6. The UE measures K = 4 continuous time instances from Set B which is the same as Set A, and predicts Top-K (from Top-1 to Top-4) in the upcoming F = 1/2/4 time instances. Apparently, from Table 2, one may find that given Top-K predicted beam pair(s), the beam prediction accuracy decreases from F = 1 to F = 4. The reason lies in the fact that the higher the time gap between measurement and prediction, the higher probability that the prediction is not accurate. Similar observation can be found in Table 3 for L1-RSRP prediction error. But anyway, the performance seems promising (higher than 90.0%) from Top-2 to Top-4 prediction.

- Beam prediction accuracy for BM-Case2

| Beam pair prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 87.1% | 97.3% | 98.9% | 99.5% |
| F = 2 | 82.8% | 95.8% | 98.2% | 99.3% |
| F = 4 | 77.1% | 93.7% | 97.3% | 98.8% |

- L1-RSRP prediction error for BM-Case2

| Avg. RSRP prediction error (dB) | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 0.166 | 0.049 | 0.022 | 0.011 |
| F = 2 | 0.287 | 0.058 | 0.023 | 0.02 |
| F = 4 | 0.539 | 0.126 | 0.043 | 0.026 |

- Temporal domain beam prediction can provide beam prediction accuracy (at least 77%) while overhead/latency reduction can be up to 50% (for the case of K = 4 and F = 4).

- Beam predication accuracy slightly decreases from 87.1% to 77.1% (the case of Top-1) when F increases from 1 to 4, but strongly increases from 77.1% to 98.8% (the case of F = 4) when predicted beam number increases from Top-1 to Top-4.

In Figure 8, the CDF of L1-RSRP difference is presented. Even for the worst case (Top-1, F = 4), 80% of L1-RSRP prediction error is lower than 3.5dB. Note here we only calculate the L1-RSRP for the case when beam prediction in time domain is incorrect.

- : CDF of L1-RSRP gap of Top-1 predicted beam in 1/2/4 time instance(s)

- For 80% of the incorrect temporal domain beam prediction cases, the L1-RSRP difference is lower than 3.5dB which may not strongly impact the spectrum efficiency.

Combination of spatial and temporal domain beam prediction

In this section, we evaluation the use case of spatial and temporal domain beam prediction by taking the model of BM-Case2 as basis. Accordingly, we shrank the size of Set B (previously equal to that of Set A in BM-Case2, i.e. 128 beam pairs) to that of BM-Case1 (i.e. 32 beam pairs) to obtain the beam prediction functions in both spatial and temporal domain.

For K = 4 measurement instances, the beam prediction accuracy and L1-RSRP prediction error can be found in Table 4 and Table 5, respectively. The overhead reduction for the case of K = 4 and F = 4 can be calculated as 1 – (32 beam pairs * 4 measurement instances) / (128 beam pairs * (4 measurement instances + 4 prediction instances)) = 87.5%.

- Beam prediction accuracy for combined BM-Case1 and BM-Case2 (when K = 4 measurement instances)

| Beam prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 85.7% | 97.0% | 98.4% | 99.5% |
| F = 2 | 82.8% | 95.7% | 98.1% | 99.3% |
| F = 4 | 74.4% | 93.1% | 97.0% | 98.8% |

- L1-RSRP prediction error for combined BM-Case1 and BM-Case2 (when K = 4 measurement instances)

| Avg. RSRP prediction error in dB | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 0.200 | 0.057 | 0.030 | 0.021 |
| F = 2 | 0.316 | 0.077 | 0.042 | 0.023 |
| F = 4 | 0.731 | 0.15 | 0.062 | 0.030 |

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 74.4%) while overhead/latency reduction can be up to 87.5% (for the case of K = 4, F = 4 and Set B = 32 beam pairs, Set A = 128 beam pairs).

Moreover, when compared with that of BM-Case2 with same setting but different Set B, the beam prediction accuracy and avg. L1-RSRP error of combined spatial and temporal domain beam prediction only slightly decrease.

For K = 8 measurement instances, the beam prediction accuracy and L1-RSRP prediction error can be found in Table 6 and Table 7, respectively. Similarly, the overhead reduction for the case of K = 8 and F = 8 can be calculated as 1 – (32 beam pairs * 8 measurement instances) / (128 beam pairs * (8 measurement instances + 8 prediction instances)) = 87.5% which is the same as the case of K = 4 and F = 4.

- Beam prediction accuracy for combined BM-Case1 and BM-Case2 (when K = 8 measurement instances)

| Beam prediction accuracy | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 84.4% | 96.7% | 98.8% | 99.7% |
| F = 2 | 80.6% | 95.8% | 98.3% | 99.3% |
| F = 4 | 74.4% | 92.3% | 96.0% | 98.4% |
| F = 8 | 64.5% | 84.4% | 92.9% | 96.2% |

- L1-RSRP prediction error for combined BM-Case1 and BM-Case2 (when K = 8 measurement instances)

| Avg. RSRP prediction error in dB | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- |
| F = 1 | 0.192 | 0.036 | 0.011 | 0.006 |
| F = 2 | 0.353 | 0.067 | 0.024 | 0.014 |
| F = 4 | 0.675 | 0.174 | 0.081 | 0.034 |
| F = 8 | 1.697 | 0.512 | 0.145 | 0.078 |

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 64.5%) while overhead/latency reduction can be up to 87.5% (for the case of K = 8, F = 8 and Set B = 32 beam pairs, Set A = 128 beam pairs).

- For spatial and temporal domain beam prediction, the longer gap between measurement on Set B and prediction among Set A, the deeper performance loss can be observed.

Generalization issues

In this section, we evaluate the property of model generalization for BM-Case1. Specifically, under UMa scenario, we assume the beam prediction model is trained with Set B (32 beam pairs (8T4R)) and Set A (128 beam pairs (32T4R)). Next, the inference was carried for 3 different testing sets.

For Case 1, we use it as a baseline scheme where the training set and testing set are the same type, i.e. both are UMa scenario with 128 beam pairs (32T4R) as Set A and 32 beam pairs (8T4R) as Set B;  For Case 2, we only change the scenario of testing set from UMa to UMi while keeping other factors the same; For Case 3, Set A of testing set is modified with half number of Rx beams (i.e. from 4 to 2); For Case 4, Set A of testing set is changed with a quarter number of that of training set (i.e. 32T4R to 16T2R). We have to note that only for the last 2 cases, where Set A of testing set is with 32T2R or 16T2R, the corresponding Set B (subset of Set A in our evaluations) should be cut accordingly. In Case 3 and Case 4, Set B of testing set is composed of 16 beam pairs (i.e. 8T2R).

In Table 8 and Table 9, from Case 1 through Case 4, the beam prediction accuracy and avg. L1-RSRP difference are shown, respectively. One may find that by comparing Case 1 and Case 2, changing environment (training from UMa and inferencing for UMi) would not deteriorate the performance of beam prediction. One of the explanations would be that the model trained under UMa scenario is more adaptable to UMi scenario. For Case 3 (a half number of Rx beams) and Case 4 (a half number of Tx beams and a half number of Rx beams), we observe slight performance loss due to the generalization issue. The more dramatic change from training set to testing set, the more performance loss can be observed. But when the number of Top-K predicted beams increases to 4, the beam prediction accuracy can be higher than 95% associated with neglectable RSRP difference.

In addition, we would like to mention that to make the AI/ML model workable between training phase and inference phase, simple pre-processing and post-processing techniques could be applied. For example, at the input end of the model, if the size of Set B is changed from 8T4R (training phase) to 8T2R (inference phase), then zero-padding or 1-on-1 replica can be added on 8T2R to fill the input of 8T4R. That’s the pre-processing part. At the output end of the model, if the size of Set A is changed from 32T4R (training phase) to 16T2R (inference phase), post-processing, such as grouping, should be applied to determine rough beam pair prediction (16T2R) from the inference of fine beam pair(s) (32T4R).

- Beam prediction accuracy where the Set B/Set A of training set are UMa_8T4R/UMa_32T4R

| Beam prediction accuracy | Testing Set (Set B) | Testing Set (Set A) | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | UMa_8T4R | UMa_32T4R | 79.2% | 92.5% | 95.9% | 98.0% |
| Case 2 | UMa_8T4R | UMi_32T4R | 80.7% | 94.5% | 97.4% | 98.6% |
| Case 3 | UMa_8T2R | UMa_32T2R | 76.9% | 91.8% | 96.2% | 97.9% |
| Case 4 | UMa_8T2R | UMa_16T2R | 64.8% | 85.1% | 91.9% | 95.1% |

- Avg. L1-RSRP difference in dB where the Set B/Set A of training set are UMa_8T4R/UMa_32T4R

| Avg. L1-RSRP difference in dB | Testing Set (Set B) | Testing Set (Set A) | Top-1 | Top-2 | Top-3 | Top-4 |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 | UMa_8T4R | UMa_32T4R | 0.411 | 0.155 | 0.071 | 0.041 |
| Case 2 | UMa_8T4R | UMi_32T4R | 0.236 | 0.055 | 0.022 | 0.016 |
| Case 3 | UMa_8T2R | UMa_32T2R | 0.608 | 0.201 | 0.079 | 0.043 |
| Case 4 | UMa_8T2R | UMa_16T2R | 2.014 | 0.706 | 0.308 | 0.184 |

- Thanks to generalization capability of well-trained AI/ML model, changing scenario from UMa to UMi may not necessarily deteriorate the beam prediction performance.

- Changing beam pair configuration on Set B and Set A from training phase to inference phase would slightly lower the beam prediction performance.

- When more predicted beam pairs are provided by AI/ML model, e.g. Top-K = 4, the beam selection accuracy can be up to 95% and avg. L1-RSRP difference can be lower than 1dB.

- Study the techniques of pre-processing at model input and post-processing at model output to enable the generalization capability of AI/ML model.

Conclusion

In this section, allow us to repeat our observations and proposals

- Apply fixed Set B across training and inference phases for BM-Case1 and BM-Case2.

- For BM-Case1 and BM-Case2, at least support beam pair prediction (Alt.3) as the key feature of representative sub use cases.

- For BM-Case1 and BM-Case2, suggest to adopt L1-RSRP measurement based on Set B as input of AI/ML model.

- For the assistance information of BM-Case1 and BM-Case2, suggest to

  - Justify the performance benefits if assistance information

  - Study whether assistance information would expose beamforming implementation and proprietary information at NW or UE.

- For beam prediction accuracy, adopt Option 2 (beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”) for AI/ML beam prediction.

- Study another definition of L1-RSRP difference of Top-1 predicted beam

- The difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

- For BM-Case1, study how to accurately capture the overhead, considering beam measurement on Set B and potential follow-up measurement after beam prediction.

- For BM-Case2, study how to capture the overhead reduction, considering the T1 duration (measurement on Set B) and T2 duration (prediction among Set A).

- For spatial domain beam prediction, select the best beam within Set A via exhaustive beam sweeping (Option 1) as baseline.

- For temporal domain beam prediction, select the best beam for T2 within Set A via exhaustive beam sweeping (Option 1a) as baseline.

- Study the techniques of pre-processing at model input and post-processing at model output to enable the generalization capability of AI/ML model.

- The input of AI/ML model for beam prediction are element-wise sensitive, therefore the L1-RSRPs of Tx and/or Rx beams in Set B should be input in proper order.

- Spatial domain beam prediction can yield beam prediction accuracy (at least 80%) while overhead/latency reduction rate is 75%.

- The system level metric, i.e. spectrum efficiency or throughput, is not sensitive to the L1-RSRP difference introduced by spatial domain beam prediction.

- For 80% of the incorrect spatial domain beam prediction cases, the L1-RSRP difference can be kept within 2dB.

- When beam prediction accuracy is high (at least 80%) and L1-RSRP difference is small (within 1 dB), the system-level performance, i.e. spectrum efficiency or throughput, may only provide non-essential insight, therefore focusing on L1-RSRP for beam prediction would be good enough.

- Temporal domain beam prediction can provide beam prediction accuracy (at least 77%) while overhead/latency reduction can be up to 50% (for the case of K = 4 and F = 4).

- Beam predication accuracy slightly decreases from 87.1% to 77.1% (the case of Top-1) when F increases from 1 to 4, but strongly increases from 77.1% to 98.8% (the case of F = 4) when predicted beam number increases from Top-1 to Top-4.

- For 80% of the incorrect temporal domain beam prediction cases, the L1-RSRP difference is lower than 3.5dB which may not strongly impact the spectrum efficiency.

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 74.4%) while overhead/latency reduction can be up to 87.5% (for the case of K = 4, F = 4 and Set B = 32 beam pairs, Set A = 128 beam pairs).

- Spatial and temporal domain beam prediction can provide beam prediction accuracy (at least 64.5%) while overhead/latency reduction can be up to 87.5% (for the case of K = 8, F = 8 and Set B = 32 beam pairs, Set A = 128 beam pairs).

- For spatial and temporal domain beam prediction, the longer gap between measurement on Set B and prediction among Set A, the deeper performance loss can be observed.

- Thanks to generalization capability of well-trained AI/ML model, changing scenario from UMa to UMi may not necessarily deteriorate the beam prediction performance.

- Changing beam pair configuration on Set B and Set A from training phase to inference phase would slightly lower the beam prediction performance.

- When more predicted beam pairs are provided by AI/ML model, e.g. Top-K = 4, the beam selection accuracy can be up to 95% and avg. L1-RSRP difference can be lower than 1dB.

References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94e, Dec. 2021

- TS 38.215 v17.1.0 Physical layer measurements

- TS 38.214 v17.1.0 Physical layer procedures for data

- Appendix I: evaluation assumptions

- Evaluation assumptions for dense urban (both BM-Case1 and BM-Case2)

| Parameter | Value |
| --- | --- |
| Scenario | Dense urban (macro layer only), 21 cells |
| Frequency setting | 30GHz carrier & 120kHz SCS & 80MHz BW |
| BS antenna config | [Mg Ng M N P] = [1 1 4 8 2] baseline |
| UE antenna config | [Mg Ng M N P] = [1 2 1 4 2] baseline |
| BS beamforming | 32 Tx beams (8 horizontal & 4 vertical) |
| UE beamforming | 4 Rx beams (4 horizontal & 1 vertical) |
| UE rotation | OFF |
| Beam selection metric | L1-RSRP |
| Traffic model | Full buffer |
| Performance metric |  |
| L1-RSRP | Top-K beam pairs with maximum L1-RSRPs |
| Spectrum efficiency | Final output of SLS |

- Evaluation assumptions for BM-Case1 (dense urban)

| Model parameter | Values |
| --- | --- |
| Model selection | DNN with 3 hidden layers |
| Input | 32 beam pairs (8 Tx beams and 4 Rx beams) |
| Output | Predicted Top-K beam pairs (K = 1, 2, 3, 4) |
| Training set and mode | 40000, offline |
| Spatial domain specific | Values |
| UE speed | 3km/h |
| UE mobility | OFF |
| Spatial consistency | OFF |

- Evaluation assumptions for BM-Case2 (dense urban)

| Model parameter | Values |
| --- | --- |
| Model selection | LSTM + full connection |
| Input | 128 beam pairs (32 Tx beams and 4 Rx beams) K = 4 measurement instance |
| Output | Predicted Top-K beam pairs (K = 1, 2, 3, 4) in F time instances (F = 1, 2, 4). Each time instance is with granularity 100ms. |
| Training set and mode | 20000 (in case of K=4 and F=1), offline |
| Time domain specific | Values |
| UE speed | 30km/h |
| UE trajectory | Option 2: linear trajectory with random direction change, and bounce back when hitting cell boundary |
| Spatial consistency | Procedure A in TR 38.901 |

- Appendix II: evaluation for indoor hotspot scenario

- Assumption for BM-Case1

In our initial evaluation (before RAN1#109e), we focused on the sub use case of beam prediction in spatial domain for overhead and latency reduction.

The illustration on traditional beam selection and AI/ML beam prediction in spatial domain can be found in Figure 10. For traditional beam sweeping, there are 64 Tx beams, 4 Rx beams, and then the total number of beam pairs, i.e. Set A, to measure can be up to 256. For AI/ML beam prediction, only a subset of all beam pairs (e.g. 8 Tx beams highlighted as yellow) are actually measured by UE with all Rx beams as Set B. Then the measurement number can be calculated as 32 beam pairs (8 Tx beam * 4 Rx beam). Correspondingly, the overhead and latency can be significantly compressed as 1/8 (32 beam pairs / 256 beam pairs).

- : DL beam sweeping procedure where for traditional beam sweeping 64 Tx beams at NW and 4 Rx beams per panel and for AI/ML beam prediction 8 Tx beams and 4 Rx beams per panel

- : Only a subset of DL beams measured among all DL beams

- Indoor hotspot scenario

| Parameter | Value |
| --- | --- |
| Scenario | TR38.901: Indoor hotspot, 12 sites, 3 cells per site |
| SCS | 120kHz |
| Bandwidth | 40MHz |
| Carrier Frequency | 30GHz |
| Tx power | 20dBm |
| BS antenna config | [Mg Ng M N P] = [1 1 4 16 2] |
| UE antenna config | [Mg Ng M N P] = [1 1 1 4 2] |
| BS Tx beam pattern | 64 Tx = 16 horizontal * 4 vertical Azimuth angle = [-16*pi/32, -14*pi/32, …, 0, …, 12*pi/32, 14*pi/32] Zenith angle = [2*pi/8, 3*pi/8, 4*pi/8, 5*pi/8] (azimuth, zenith) = (0, pi/2) is the direction perpendicular to the array |
| BS mech. tilting | 20 degree |
| BS height | 3m |
| UE Rx beam pattern | 4 Rx per panel = 4 horizontal * 1 vertical UE panels deployment [-90, 0, 90], assumed the best panel is selected Azimuth angle = [-3*pi/8, 1*pi/8, pi/8, 3*pi/8] Zenith angle = [pi/2] |
| UE velocity | 3km/h |
| UE height | 1.5m |
| Beam selection method | L1-RSRP |
| Training set | 39600 samples = 12 sites * 3 cells per site * 100 UE per cell * 11 drops |

- Evaluations for BM-Case1

Given the previously introduced data set and NN model setting, we would like to share our initial evaluation results in Table 8. In this table, we compare the AI/ML based prediction with traditional beam selection, where the latter applies no interpolation, but just select the best measured beam pair with highest L1-RSRP among the actual measurements.

In the 2nd column of Table 8, we compare the best beam selection/prediction rate for both AI/ML beam prediction and traditional beam selection. One may see that the correct best beam pair prediction rate of AI/ML could be around 80% and the traditional beam selection has a smaller chance, i.e. round 24% to find the best beam pair.

In the 3rd and 4th column of Table 8, two performance metrics on L1-RSRP error are calculated. L1-RSRP prediction error is the difference between actual (real) L1-RSRP of the best beam pair and the predicted L1-RSRP. L1-RSRP selection error is the difference between actual (real) L1-RSRP of the best beam pair and the actual (real) RSRP of the selected beam pair.

When the best beam pair is predicted right by the NN model, its L1-RSRP prediction error is 1.02dB. That is very close to the quantization step for L1-RSRP beam reporting (e.g. 1dB step size) in TS 38.214 [3]. Note that when the best beam pair is correctly selected, then the L1-RSRP selection error (between the actual L1-RSRP of the best beam pair and the actual R1-RSRP of selected best beam) is 0dB, since in this case, they are of the same beam pair.

The false beam pair selection rate of AI/ML based prediction is round 20%, and the L1-RSRP prediction error is around 2.5dB. Intuitively, if the NN cannot predict the right beam pair, then it would be harder to predict the best L1-RSRP within very small error. Moreover, in the cases of false beam prediction, the L1-RSRP selection error is more than 5dB, which reflects the difference between the actual (real) L1-RSRP of best beam pair and actual (real) L1-RSRP of the predicted beam pair.

As for traditional beam selection, since there is no interpolation/prediction, the L1-RSRP prediction error is not applicable. Hence, we only average the L1-RSRP selection error of traditional beam selection which could be up to 11dB. The reason of such huge gap lies in the fact that UE can only pick up the best beam within 1/8 beam pair of all candidate beam pairs.

- Performance based on L1-RSRP

|  | Correct beam prediction rate | Mean of L1-RSRP prediction error | Mean of L1-RSRP selection error |
| --- | --- | --- | --- |
| AI/ML-Right selection | 80.35% | 1.02dB | 0dB |
| AI/ML-False selection | 19.65% | 2.47dB | 5.49dB |
| Traditional-Right selection | 23.94% | / | 0dB |
| Traditional-False selection | 76.06% | / | 11.16dB |

With above analysis on performance, we would like to share our observations.

- For InH scenario, AI/ML beam prediction in spatial domain can yield relatively high correct prediction rate (around 80%) while using only a small portion (1/8) of DL measurement.

- For InH scenario, when AI/ML beam prediction is correct, the L1-RSRP prediction error is acceptably small (around 1dB); otherwise (AI/ML beam prediction incorrect), the L1-RSRP prediction error would increase slightly (up to 2.5dB).

In Figure 12, we show the CDF of both predicted L1-RSRP and actual (real) L1-RSRP when the AI/ML beam prediction is correct. Statistically speaking, thanks to the technique of AI/ML, these CDFs are closed to each other. It aligns well with average L1-RSRP prediction error (1.02dB as shown in Table 8).

In Figure 13, we show the CDFs when false beam pair selection/prediction happens. When AI/ML model 1 (introduced in Figure 4) predicts an incorrect best beam pair, the CDF of its predicted L1-RSRP (generated by AI/ML model 2 separately) stays not far away from that of actual (real) L1-RSRP of the best beam pair. This curve is marked as “AI-predicted L1-RSRP”.

As for the curve marked as “AI-selected L1-RSRP”, it refers to the actual (real) L1-RSRP of the wrongly-predicted best beam pair. This gap between actual L1-RSRP of AI/ML prediction and actual L1-RSRP of the best beam pair is around 5dB. This performance also matters, since it reflects actual L1-RSRP performance gap of the best beam predicted wrongly by NN.

As we explained previously, the traditional sub-set beam selection scheme has a large gap when compared with the best beam. The reason lies in the fact that the traditional beam selection only has partial beam pair candidates. Note that in this contribution, we only select the very basic traditional beam selection scheme. To make more meaningful comparison, we may need to study more advanced traditional scheme, e.g. beam selection with linear interpolation.

- : CDF of actual (real) L1-RSRP and predicted L1-RSRP when the beam prediction is correct

- : CDF of actual (real)/selected/predicted L1-RSRP when the beam prediction is incorrect