3GPP TSG RAN WG1 #110bis-e	                              R1-2208904

e-Meeting, October 10th – 19th, 2022

Agenda item:		9.2.4.2

Source:		LG Electronics

Title:			Other aspects on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

Introduction

In RAN1#110 meeting, the agreements on sub use cases and potential specification impact of AI/ML for positioning were made as follows.

Agreement For characterization and performance evaluations of AI/ML based positioning accuracy enhancement, the following two AI/ML based positioning methods are selected. - Direct AI/ML positioning - AI/ML assisted positioning - Note 1: the selection does not intend to provide any indication of the prospects of any future normative project. - Note 2: further discussion (including selection of other sub use cases and/or down selection of selected sub use cases) are not precluded based on performance evaluation and potential specification impact study results Conclusion Defer the discussion of prioritization of AI/ML positioning based on collaboration level until more progress on collaboration level discussion in agenda 9.2.1. Agreement Regarding data collection for AI/ML model training, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - Ground truth label determination (e.g., based on UE/PRU/TRP measurement/report) - Partial and/or noisy ground truth label - Signaling for data collection - Other aspects are not precluded Agreement Regarding AI/ML model monitoring and update, to study and provide inputs on potential specification impact at least for the following aspects of AI/ML based positioning accuracy enhancement - AI/ML model monitoring performance metrics - Condition of AI/ML model update - Reference signals and measurement feedback/report - Other aspects are not precluded Agreement Study aspects in terms of potential benefit(s) and requirement(s)/specification impact(s) of AI/ML model training and inference in AI/ML for positioning accuracy enhancement considering at least - UE-side or Network-side training - UE-side or Network-side inference - Note: model inference at both UE and network side is not precluded where proponent(s) are encouraged to clarify their AI/ML approaches Note: companies are encouraged to clarify aspects of their proposed AI/ML approaches for positioning when AI/ML model training and inference are not performed at the same entity Conclusion To use the following terminology defined in TS 38.305 when describe their proposed positioning methods - UE-based - UE-assisted/LMF-based - UE provides some information to the LMF which calculates the final position - NG-RAN node assisted - gNB can provide some information to LMF that determines the final location - Note: companies are required to clarify their positioning method(s) when their approaches do not fall in one of the above

In this contribution, we discuss on other aspects on AI/ML for positioning accuracy enhancements such as representative sub use-cases that are worth considering and related potential specification impacts in terms of RAN1 point of view.

Discussions on potential sub use cases and their specification impact

Based on above agreements, we firstly discuss model monitoring performance metric for AI/ML based positioning accuracy enhancement in the following sub-section 2.1. In sub-section 2.2, we suggest two sub use cases (i.e. LOS/NLOS indication, PRU prediction) on the perspective of positioning accuracy enhancement and the corresponding potential specification impact.

Model monitoring performance metric

For AI/ML based positioning accuracy enhancement, the offline-trained AI/ML model is delivered to UE or NW for utilizing it to actual operating environment. In this case, mismatch between the offline training data and the field data is inevitable and the output performance may be degraded when the AI/ML model is applied. To address this problem, how to update and/or fine-tune the AI/ML model can be considered.

Regarding the AI/ML model monitoring, the intermediate performance which is related to output performance also needs to be taken into account. As an intermediate performance, it would be estimated LOS/NLOS identifier, TDoA, AoA and so on. In other words, it means 2-step monitoring performance metric where the output performance degradation of the AI/ML model is originated from whether the poor intermediate performance or from discrepancy on environment, scenario and the assumption of the offline trained AI/ML model compared to actual operating environment. Based on this monitoring method, the detailed condition is further investigated to perform fine-tuning or update of the AI/ML model.

Proposal #1: Consider AI/ML model fine-tuning or update based on model monitoring performance metric by taking into account the intermediate performance and output performance jointly.

Accuracy improvements

In Rel-17 positioning enhancements, reporting of path specific measurement and providing LOS/NLOS indicator for first arrival path were agreed since LOS/NLOS identification is critical factor for accuracy performance. However, the indication has been without definition and it relies on UE implementation. In this sense, LMF does not know the reliability of the indication and then, it may be difficult to utilize it to improve the performance accuracy. Therefore, it should be required to improve the reliability of LOS indication information and to improve the performance in terms of positioning accuracy and resource efficiency.

Observation #1: In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE).

To improve the reliability of LOS/NLOS information, it can be based on AI/ML approach at UE or LMF side. When applying the AI/ML method at UE-side, the LOS/NLOS probability value for first arrival path and/or the reliability information such as probability can be derived based on the UE capability and various parameters related to LOS estimation such as UE antenna configurability, capability of PRS measurement, AI/ML model for LOS estimation, indication of PRU, the number of PRS measurement, noise variance, etc. For LMF-side AI/ML, based on the measurement results and some assistance information (e.g. LOS probability and/or reliability information) from UE and/or LMF, LOS/NLOS identification can be achieved.

Proposal #2: Consider assistant information including LOS probability and/or reliability information for the AI/ML based LOS/NLOS identification.

Based on the LOS/NLOS related information from the above, NW can collect/process the LOS/NLOS identification at a specific location on coordinate system for supporting UEs. Through this, it is possible to determine which PRS resource is most likely to be LOS for a specific UE in the form of a boundary between specific regions on the coordinate as shown in Figure 1 below. Then, the PRS configuration for the location of the UE can be adaptively configured via the above information.

Figure 1. Example of LOS/NLOS probability and the corresponding PRSs on coordinate system supporting UEs

Proposal #3: Consider PRS priority configuration based on AI/ML based LOS/NLOS indication.

In the Rel-17 positioning enhancement, PRU (positioning reference unit) was introduced by utilizing the exactly known position of a specific UE or gNB (i.e. reference UE or gNB) at LMF for accuracy improvement. Regarding a UE as PRU, there is a limitation since the UE generally has mobility and the location is correspondingly changed. If the LMF can predict the UE location with mobility based on AI/ML method, it would be possible that which UE can be used as a PRU. Then, the LMF can use the UE dynamically as PRU to calculate/estimate the position of target UE.

Observation #2: When LMF can predict UE location with mobility, it is possible that which UE can be used as PRU, the LMF can use the UE dynamically as PRU to calculate the position of target UE.

For LMF-based PRU prediction, based on the measurement report of a normal UE as an input data set of the AI/ML algorithm, it is possible to predict/determine the output whether the normal UE is utilized as a PRU or not at the LMF side. If the UE is determined as the PRU, the position of target UE can be estimated/calculated with the PRU. Meanwhile, for UE-based PRU prediction, it is based on AI/ML model in the UE-side with providing assistance information for AI/ML model at LMF. Depending on the AI/ML model output, the UE can report identification information as PRU and/or assistance information, e.g. PRU expiration time, de-registration request on PRU, etc.

With these consideration, the potential specification impact can be related signaling on measurement report or information on deciding the UE as the PRU. For example, duration and the amount of distance change in duration for sensor based information and/or RSRP based on RAT dependent method can be considered as measurement or assistant information. Based on AI/ML model and assistant information, predicted value of the UE location with the corresponding time instance of the future, PRU related information such as PRU indication, the probability of the conditions, etc. can be obtained.

Proposal #4: Consider PRU prediction on LMF-/UE-side based on measurement report in addition to PRU identification and/or assistance information utilized for PRU determination.

Conclusion

In this contribution, we discussed about several aspects on AI/ML for positioning enhancement, and our observations and proposals are summarized below:

Observation #1: In Rel-17, LOS/NLOS indication for first path can be reported but the detailed algorithm is up to UE implementation (reliability issue per UE).

Observation #2: When LMF can predict UE location with mobility, it is possible that which UE can be used as PRU, the LMF can use the UE dynamically as PRU to calculate the position of target UE.

Proposal #1: Consider AI/ML model fine-tuning or update/transfer based on model monitoring performance metric by taking into account the intermediate performance and output performance together.

Proposal #2: Consider assistant information including LOS probability and/or reliability information for the AI/ML based LOS/NLOS identification.

Proposal #3: Consider PRS priority configuration based on AI/ML based LOS/NLOS indication.

Proposal #4: Consider PRU prediction on NW-/UE-side based on measurement report in addition to PRU identification and/or assistance information utilized for PRU determination.