3GPP TSG RAN WG1 #110bis-e		                                                                  R1-2208911

e-meeting, October 10th – 19th, 2022

Title:	Draft Reply LS on IUC with non-preferred resource set

Response to:	R1-2205728/R2-2206314

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	[RAN1], CATT, GOHIGH

To:	RAN2

Cc:

Attachment:	-

Contact Person:

Name:	Rui Zhao

E-mail Address:	zhaorui@gohigh.com.cn

Overall description

RAN1 thanks RAN2 for the LS asking UE-B’s behavior with non-preferred resource set when UE-B does not perform sensing in the associated resource pool. RAN1 would provide responses to the following questions from RAN2.

In RAN2#118-e meeting, RAN2 discussed the inter-UE coordination scenario in which UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set, e.g. UE B performs mode 2 random resource selection, etc. For this case, RAN2 has the following questions: Q1: Is the scenario described above a valid scenario or not? Q2: If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not? Q3: If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

RAN1 response for Q1：

From RAN1’s perspective, it is a valid scenario in which UE-B need perform random selection after receiving non-preferred resource set, because UE-B may not perform sensing or may has insufficient sensing results due to power saving.

RAN1 response for Q2：

The answer of RAN1 is Yes. Since the non-preferred resource set means the resource set that is not desired by UE-A, so non-preferred resource set should be excluded in random selection procedure to avoid selecting resource(s) within the non-preferred resource. Otherwise, it is not aligned with the agreement and design principle of non-preferred resource set.

RAN1 response for Q3:

In order to have a consistent normative text, non-preferred resource set exclusion specified in PHY layer is RAN1’s preference, where the received non-preferred resource set will be directly excluded from the initial candidate resource set after Step 1) of TS 38.214 Section 8.1.4 to form available resource set for random selection.

Actions:

To RAN2:

RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

Date of Next TSG-RAN1 Meetings:

RAN1#110bis-e			10th -19th Oct. 2022				E-Meeting

RAN1#111			14th -18th Nov. 2022				Toulouse, France