3GPP TSG RAN WG1 #110bis-e	R1-2209007

e-Meeting, October 10th – 19th, 2022

Agenda item:	8.2

Source: 	Fujitsu

Title:	Remaining issues of multi-PDSCH/PUSCH scheduling via a single DCI

Document for:		Discussion and decision

- Introduction

In this contribution, we would like to present some discussions about some remaining issues of Type-1 HARQ-ACK codebook with time domain bundling.

- Discussion

For multi-PDSCH scheduling, there may be only a single valid PDSCHs among multiple scheduled PDSCHs. Meanwhile, Type-1 HARQ-ACK codebook can be configured with time domain bundling. In this case, for the candidate PDSCH reception occasion m corresponding to the multiple scheduled PDSCHs, it is straight forward that the value of HARQ-ACK information bit should be equal to the value of HARQ-ACK information bit for the single valid PDSCH among the multiple scheduled PDSCHs. However, the pseudo-code in TS 38.213 has not correctly captured how to generate the HARQ-ACK information bit for the single valid PDSCH among multiple scheduled PDSCHs.

Observation 1: The current pseudo-code for Type-1 HARQ-ACK codebook generation with time domain bundling fails to capture the case of single valid PDSCH among multiple PDSCHs scheduled by a single DCI.

There are two different interpretations on the“a PDSCH associated with occasion m” as highlighted below.

TS 38.213, h20: … Set m=0 – index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update while m<Mc if enableTimeDomainHARQ-Bundling is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, scheduled by the DCI format on serving cell c; ….

- Interpretation 1: “a PDSCH associated with occasion m”is a PDSCH scheduled in the corresponding DL slot of occasion m, and the corresponding DL slot of occasion m is the DL slot where the last SLIV locates for determining occasion m. Under this interpretation, an example of HARQ-ACK codebook generation is given in Figure 1, assuming a single TDRA row and a single DCI scheduling multiple valid PDSCHs for simplicity.

Figure 1. Example of HARQ-ACK codebook generation for Interpretation 1

- Interpretation 2: “a PDSCH associated with occasion m”is a PDSCH of which the corresponding HARQ-ACK information is mapping to occasion m. Under this interpretation, an example of HARQ-ACK codebook generation is given in Figure 2, assuming a single TDRA row and a single DCI scheduling multiple valid PDSCHs for simplicity.

Figure 2. Example of HARQ-ACK codebook generation for Interpretation 2

How to update the pseudo-code to capture the case of single valid PDSCH among multiple scheduled PDSCHs depends on which interpretation we assume.

Observation 2: There are two possible interpretations on the “a PDSCH associated with occasion m”. How to update the pseudo-code to capture the case of single valid PDSCH among multiple scheduled PDSCHs depends on which interpretation we assume.

  - Interpretation 1: “a PDSCH associated with occasion m”is a PDSCH scheduled in the corresponding DL slot of occasion m, and the corresponding DL slot of occasion m is the DL slot where the last SLIV locates for determining occasion m.

  - Interpretation 2: “a PDSCH associated with occasion m”is a PDSCH of which the corresponding HARQ-ACK information is mapping to occasion m.

Since Interpretation 2 is the common understanding of the majority companies as per the discussions in RAN1#110 meeting, we provide a draft CR based on Interpretation 2.

Observation 3: Interpretation 2 is the common understanding according to the discussions in RAN1#110 meeting.

Proposal 1: For Type-1 HARQ-ACK codebook generation with time domain bundling, to capture the case of single valid PDSCH among multiple PDSCHs scheduled by a single DCI, adopt the CR in [1] which is based on Interpretation 2.

- Conclusion

According to the discussions above, we have the following observations and proposals for Type-1 HARQ-ACK codebook with time domain bundling.

Observation 1: The current pseudo-code for Type-1 HARQ-ACK codebook generation with time domain bundling fails to capture the case of single valid PDSCH among multiple PDSCHs scheduled by a single DCI.

Observation 2: There are two possible interpretations on the “a PDSCH associated with occasion m”. How to update the pseudo-code to capture the case of single valid PDSCH among multiple scheduled PDSCHs depends on which interpretation we assume.

  - Interpretation 1: “a PDSCH associated with occasion m”is a PDSCH scheduled in the corresponding DL slot of occasion m, and the corresponding DL slot of occasion m is the DL slot where the last SLIV locates for determining occasion m.

  - Interpretation 2: “a PDSCH associated with occasion m”is a PDSCH of which the corresponding HARQ-ACK information is mapping to occasion m.

Observation 3: Interpretation 2 is the common understanding according to the discussions in RAN1#110 meeting.

Proposal 1: For Type-1 HARQ-ACK codebook generation with time domain bundling, to capture the case of single valid PDSCH among multiple PDSCHs scheduled by a single DCI, adopt the CR in [1] which is based on Interpretation 2.

References

[1] R1-2209006, Correction on Type-1 HARQ-ACK codebook determination in TS 38.213, RAN1#110bis-e meeting, e-Meeting, October 10th – 19th, 2022