3GPP TSG RAN WG1 #110bis-e                                       	       R1-2209294

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.5.2.3

Source:	Xiaomi

Title:                     Discussion on Low Power High Accuracy Positioning

Document for:	Discussion and Decision

Introduction

In RAN- 94 e-meeting, the SI of Rel-18 positioning [1] is approved, and the objectives on improved accuracy, integrity, and power efficiency can be seen as below:

- Improved accuracy, integrity, and power efficiency:

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]:

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we focus on the discussion about LPHAP (Low Power High Accuracy Positioning).

Power consumption evaluation

For the evaluation parameters for the LPHAP evaluation, following agreements are archived in RAN1-109 e-meeting [2].

Agreement

- Adopt the following parameters as the common evaluation parameters for the LPHAP evaluation:

  - Frequency range: FR1 (baseline); FR2 (optional)

  - SCS: 30kHz for FR1 (baseline); 120kHz for FR2 (optional)

  - BW of the DL PRS and UL SRS pos: 100MHz;

  - Single-sample measurement per position fix (baseline); 4-sample measurement per position fix (optional)

  - UE mobility: up to 3km/h

- Note: It is up to each company to provide detailed power model and evaluation results on power consumption in FR2.

Agreement

Adopt the following periodicity of DL PRS / UL SRS for positioning in the baseline evaluation of Rel-17 RRC_INACTIVE positioning:

- 1 DL PRS / UL SRS for positioning occasion per N I-DRX cycle(s);

  - Candidate values of N to evaluate is 1 and 8 for I-DRX cycle of 1.28s;

    - Note: Individual company may consider either one or both in the evaluation.

  - Candidate value of N to evaluate is 1 for I-DRX cycle of 10.24s.

Agreement

- The I-DRX configuration is included in the baseline evaluation of Rel-17 RRC_INACTVIE positioning.

  - Note: This does not preclude the case where no I-DRX cycle nor paging is considered in the evaluation of potential solutions to maximize the battery life.

- Adopt the following I-DRX cycle to evaluate:

  - 1.28s (baseline); 10.24s (optional).

Agreement

- Adopt the following reference configuration and assumption for DL PRS to define the power consumption model for DL PRS measurement:

  - 1 Number of PFL;

  - 8 DL PRS resources per slot are measured;

  - DL PRS instance of smaller than or equal to 1 slot duration;

- Adopt the following table as the power consumption model for DL PRS measurement (derived from Table 22 in TR38.840):

| N: Number of TRPs for DL PRS measurement | Synchronous case (baseline) |  | Asynchronous case (optional) |  |
| --- | --- | --- | --- | --- |
|  | FR1 (baseline) | FR2 (optional) | FR1 | FR2 |
| N=4 (baseline) | 120 | 195 | 140 | 255 |
| N=8 (optional) | 150 | 225 | 170 | 285 |

Agreement

Adopt the following power consumption model common for the baseline evaluation of Rel-17 RRC_INACTIVE state positioning. TR38.840

| Power State | Relative power |
| --- | --- |
| PDCCH-only (PPDCCH) | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 120 |
| SSB proc. (PSSB) | 50 |
| UL | 250 (0 dBm) 700 (23 dBm) |
| (Optional) PRACH | [210] |
| (Optional) BWP switching | [50] |
| (Optional) Intra-frequency RRM measurement (Pintra) | [60] (synchronous case, N=8, measurement only; Pintra, meas-only) [80] (combined search and measurement; Pintra, search+meas) |
| (Optional) Inter-frequency RRM measurement (Pinter) | [60] (measurement only per freq. layer; Pinter, meas-only) [150] (neighbor cell search power per freq. layer; Pinter, search-only) Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |

Agreement

Adopt the following power consumption model for UL SRS for positioning transmission.

| Power State | Relative power |
| --- | --- |
| SRS | 210 (baseline); 700 (optional) |

Following agreements are archived in RAN1-110 meeting [3].

Agreement

In the LPHAP evaluation, adopt the following model to convert the relative power unit to the battery life:

- Alt. 1: battery life is used as the metric to identify the gap

  - K is an implementation factor, K = 1 (baseline); K = 0.5, 2, 4 (optional)

- Note: The definition of the notations will be captured in the updates of TR.

- Note: The voltage is assumed to be the same for the reference device and the LPHAP device.

Agreement

- In the LPHAP evaluation, adopt the following example parameter values in the conversion model to evaluate the battery life:

  - For the reference device in the conversion model:

| C1 (mAh) | T1 (hour) | X | reference traffic type |
| --- | --- | --- | --- |
| 4500 | 12 | 20% | FTP (model 3) |

  - For the LPHAP device, consider 2 types in the conversion model:

| LPHAP device | C2 (mAh) | T2req (month) |
| --- | --- | --- |
| Type A (baseline) | 800 | 6~12 |
| Type B (optional) | 4500 | 6~12 |

- Note: As the reference device and LPHAP device characteristics, and therefore the parameter values of the model for determining battery life, is dependent on implementation factors, manufacturer, design options and cost options, it is up to individual company to evaluate the optional K values, and report the corresponding parameter values.

Agreement

In the LPHAP evaluation, adopt the example value of relative power unit of the reference device P1 = 50 to further align the battery life among companies.

Agreement

For the purpose of LPHAP evaluation, an ultra-deep sleep state is considered. The following options of the power consumption model of the ultra-deep sleep state can be further discussed:

  - Option 1:

    - The relative power unit: 0.015

    - Additional transition energy: 2000

    - Total transition time: 400ms

  - Option 2:

    - The relative power unit: 0.01

    - Additional transition energy: 450;

    - Total transition time: 25ms

    - FFS: restrictions in processing associated with option 2 after the UE comes out of ultra-deep sleep state

  - Notes: the values above can be further discussed

Agreement

For option 1 in the agreement above, the value of additional transition energy is changed to “a value between 2000 and 20000”. FFS which value.

Agreement

For the purpose of LPHAP evaluation, the following assumptions on eDRX configuration and/or paging reception can be optionally considered:

  - The eDRX cycle to evaluate: 20.48s; 30.72s;

  - For paging reception:

    - 1 paging occasion is included in one eDRX cycle

    - 10% paging rate

  - No paging reception can be optionally evaluated;

  - 1 DL PRS and/or UL SRS for positioning occasion per 1 eDRX cycle

    - Minimizing the gap between PRS measurement, SRS transmission and/or measurement reporting with paging monitoring in time domain can be evaluated.

UE-based DL positioning

For the UE-based DL positioning, following agreements are archived in RAN1-109 e-meeting.

Agreement

- For DL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning:

  - For the UE-based DL positioning,

    - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle;

    - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%;

    - DL PRS measurement with 0.5 ms duration;

    - (Optional) BWP switching with [1] ms duration;

    - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration;

Based on the evaluation parameters agreed in RAN1-109 e-meeting and RAN1-110 meeting, an example of the simplest procedure of PRS measurement with deep sleep state in 1 I-DRX cycle of 1280ms can be seen in Figure 1. And based on Figure 2, Table 1 provides the evaluation cases and assumptions, and Table 2 and Table 3 provide the UE power consumption result for each evaluation case for UE-based DL positioning.

Figure 1, the procedure of PRS measurement

Table 1: Evaluation cases and assumptions for UE-based DL positioning

| Evaluation assumption | [Case ID#1], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#2], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#3], [FR1], [UE-based DL positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep state | Deep sleep state | Deep sleep state |
| DRX cycle | 1 I-DRX cycle: 1280ms | 1 I-DRX cycle: 1280ms | 1 eDRX cycle: 20.48s |
| paging reception | Without paging | Without paging | Without paging |
| RS periodicity | 1 I-DRX cycle | 8 I-DRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |

| Evaluation assumption | [Case ID#4], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#5], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#6], [FR1], [UE-based DL positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Ultra sleep state (option 2) | Ultra sleep state (option 1) | Ultra sleep state (option 1) |
| DRX cycle | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 30.72s |
| paging reception | Without paging | without paging | without paging |
| RS periodicity | 1 eDRX cycle | 1 eDRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |

Table 2: UE power consumption result for each evaluation case for UE-based DL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#1 | SSB | 50 | 4 | 1 | 4 | 200 | 5.14 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 3.08 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 5.14 |
|  | Deep sleep transition | 450 | 40 | 2 | 80 | 900 | 23.13 |
|  | Deep sleep | 1 |  | 1 | 2471 | 2471 | 63.51 |
|  | Total (every power cycle) |  |  |  | 2560 | 3891 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.52 |  |
|  | Battery life (in month) |  |  |  | 351 hours = 14.6 days = 0.5 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#2 | SSB | 50 | 4 | 8 | 32 | 1600 | 5.28 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.40 |
|  | paging | 50 | 4 | 8 | 32 | 1600 | 5.28 |
|  | Deep sleep transition | 450 | 40 | 16 | 640 | 7200 | 23.77 |
|  | Deep sleep | 1 |  | 8 | 19775 | 19775 | 65.27 |
|  | Total (every power cycle) |  |  |  | 20480 | 30295 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.48 |  |
|  | Battery life (in month) |  |  |  | 360 hours = 15 days = 0.5 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#3 | SSB | 50 | 4 | 1 | 4 | 200 | 0.47 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.28 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 0.47 |
|  | Deep sleep transition | 450 | 40 | 2 | 80 | 900 | 2.13 |
|  | Deep sleep | 1 |  | 1 | 40871 | 40871 | 96.64 |
|  | Total (every power cycle) |  |  |  | 40960 | 42291 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.03 |  |
|  | Battery life (in month) |  |  |  | 549 hours = 23 days = 0.8 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#4 | SSB | 50 | 4 | 1 | 4 | 200 | 10.94 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 6.57 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 10.94 |
|  | Ultra sleep transition | 450 | 50 | 2 | 100 | 900 | 49.25 |
|  | ultra sleep | 0.01 |  | 1 | 40851 | 408.51 | 22.3 |
|  | Total (every power cycle) |  |  |  | 40960 | 1828.51 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.045 |  |
|  | Battery life (in month) |  |  |  | 11852 hours = 493 days = 16.4 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#5 | SSB | 50 | 4 | 1 | 4 | 200 | 3.91 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.35 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 3.91 |
|  | Ultra sleep transition | 2000 | 800 | 2 | 1600 | 4000 | 78.27 |
|  | ultra sleep | 0.015 |  | 1 | 39351 | 590.27 | 11.55 |
|  | Total (every power cycle) |  |  |  | 40960 | 5110.27 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.125 |  |
|  | Battery life (in month) |  |  |  | 4266 hours = 178 days = 5.9 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#6 | SSB | 50 | 4 | 1 | 4 | 200 | 3.69 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.22 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 3.69 |
|  | Ultra sleep transition | 2000 | 800 | 2 | 1600 | 4000 | 73.84 |
|  | ultra sleep | 0.015 |  | 1 | 59831 | 897.47 | 16.57 |
|  | Total (every power cycle) |  |  |  | 61440 | 5417.47 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.088 |  |
|  | Battery life (in month) |  |  |  | 6060 hours = 252 days = 8.4 months |  |  |

Table 3: Summary for UE power consumption results for UE-based DL positioning

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| [Case ID#1], [Rel-17 ] | 1.52 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#2], [Rel-17 ] | 1.48 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#3], [potential enhancements ] | 1.03 | 0.8 | No (5.2 months) | No (11.2 months) |
| [Case ID#4], [potential enhancements ] | 0.045 | 16 | Yes | Yes |
| [Case ID#5], [potential enhancements ] | 0.125 | 5.9 | No (0.1 months) | No (6.1 months) |
| [Case ID#6], [potential enhancements ] | 0.088 | 8.4 | Yes | No (3.6 months) |

- Observation 1: For UE-based DL positioning

- Case ID#4 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 2) can meet the target requirement 12 months.

- Case ID#5 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can almost meet the target requirement 6 months.

- Case ID#6 with 1 eDRX cycle (30.72s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can meet the target requirement 6 months.

  - If the value of additional transmission energy changed to more than 3000, it will not be able to meet the target requirement 6 months.

UE-assisted DL positioning

For the UE-assisted DL positioning, following agreements are archived in RAN1-109 e-meeting.

Agreement

- For DL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning:

  - For the UE-assisted DL positioning,

- SSB proc. with 2 ms duration and the periodicity of I-DRX cycle;

- Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%;

- DL PRS measurement with 0.5 ms duration;

- CG-SDT with 1ms duration and the periodicity of positioning interval;

      - RRCRelsease after the CG-SDT can be optionally included with [1] ms duration;

- (Optional) BWP switching with [1] ms duration;

- (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration;

- (Optional) RA-SDT (e.g., including CORSET0 + SIB1, PRACH, RAR, Msg 3/4/5) in case of CG-SDT is unavailable;

Based on the evaluation parameters agreed in RAN1-109 e-meeting and RAN1-110 meeting, an example of the simplest procedure of PRS measurement and report with deep sleep state in 1 I-DRX cycle of 1280ms can be seen in Figure 2. And based on Figure 2, Table 4 provides the evaluation cases and assumptions, and Table 5 and Table 6 provide the UE power consumption result for each evaluation case for UE-assisted DL positioning.

Figure 2, the procedure of PRS measurement and report

Table 4: Evaluation cases and assumptions for UE-assisted DL positioning

| Evaluation assumption | [Case ID#7], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#8], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#9], [FR1], [UE-assisted DL positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep state | Deep sleep state | Deep sleep state |
| DRX cycle | 1 I-DRX cycle: 1280ms | 1 I-DRX cycle: 1280ms | 1 eDRX cycle: 20.48s |
| paging reception | Without paging | Without paging | Without paging |
| RS periodicity | 1 I-DRX cycle | 8 I-DRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT per I-DRX cycle | CG-SDT per 8 I-DRX cycle | CG-SDT per eDRX cycle |
| implementation factor K | 1 | 1 | 1 |

| Evaluation assumption | [Case ID#10], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#11], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#12], [FR1], [UE-assisted DL positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Ultra sleep state (option 2) | Ultra sleep state (option 1) | Ultra sleep state (option 1) |
| DRX cycle | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 30.72s |
| paging reception | Without paging | without paging | without paging |
| RS periodicity | 1 eDRX cycle | 1 eDRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT per eDRX cycle | CG-SDT per eDRX cycle | CG-SDT per eDRX cycle |
| implementation factor K | 1 | 1 | 1 |

Table 5: UE power consumption result for each evaluation case for UE-assisted DL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#7 | SSB | 50 | 4 | 1 | 4 | 200 | 4.83 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.90 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 4.83 |
|  | CG-SDT | 250 | 2 | 1 | 2 | 250 | 6.04 |
|  | Deep sleep transition | 450 | 40 | 2 | 80 | 900 | 21.74 |
|  | Deep sleep | 1 |  | 1 | 2469 | 2469 | 59.65 |
|  | Total (every power cycle) |  |  |  | 2560 | 4139 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.62 |  |
|  | Battery life (in month) |  |  |  | 329 hours = 13.7 days =0.5 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#8 | SSB | 50 | 4 | 8 | 32 | 1600 | 5.24 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.39 |
|  | paging | 50 | 4 | 8 | 32 | 1600 | 5.24 |
|  | CG-SDT | 250 | 2 | 1 | 2 | 250 | 0.82 |
|  | Deep sleep transition | 450 | 40 | 16 | 640 | 7200 | 23.57 |
|  | Deep sleep | 1 |  | 8 | 19773 | 19773 | 64.74 |
|  | Total (every power cycle) |  |  |  | 20480 | 30543 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.49 |  |
|  | Battery life (in month) |  |  |  | 358 hours = 15 days =0.5 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#9 | SSB | 50 | 4 | 1 | 4 | 200 | 0.47 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 0.28 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 0.47 |
|  | CG-SDT | 250 | 2 | 1 | 2 | 250 | 0.59 |
|  | Deep sleep transition | 450 | 40 | 2 | 80 | 900 | 2.12 |
|  | Deep sleep | 1 |  | 1 | 40869 | 40869 | 96.07 |
|  | Total (every power cycle) |  |  |  | 40960 | 42539 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.04 |  |
|  | Battery life (in month) |  |  |  | 513 hours = 21 days = 0.7 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#10 | SSB | 50 | 4 | 1 | 4 | 200 | 9.62 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 5.77 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 9.62 |
|  | CG-SDT | 250 | 2 | 1 | 2 | 250 | 12.03 |
|  | Ultra sleep transition | 450 | 50 | 2 | 100 | 900 | 43.30 |
|  | ultra sleep | 0.01 |  | 1 | 40849 | 408.49 | 19.65 |
|  | Total (every power cycle) |  |  |  | 40960 | 2078.49 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.051 |  |
|  | Battery life (in month) |  |  |  | 10458 hours = 436 days = 14.5 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#11 | SSB | 50 | 4 | 1 | 4 | 200 | 3.73 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.24 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 3.73 |
|  | CG-SDT | 250 | 2 | 1 | 2 | 250 | 4.66 |
|  | Ultra sleep transition | 2000 | 800 | 2 | 1600 | 4000 | 74.62 |
|  | ultra sleep | 0.015 |  | 1 | 39349 | 590.24 | 11.01 |
|  | Total (every power cycle) |  |  |  | 40960 | 5360.24 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.131 |  |
|  | Battery life (in month) |  |  |  | 4071 hours = 170 days = 5.7 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#12 | SSB | 50 | 4 | 1 | 4 | 200 | 3.53 |
|  | PRS measurement | 120 | 1 | 1 | 1 | 120 | 2.12 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 3.53 |
|  | CG-SDT | 250 | 2 | 1 | 2 | 250 | 4.41 |
|  | Ultra sleep transition | 2000 | 800 | 2 | 1600 | 4000 | 70.58 |
|  | ultra sleep | 0.015 |  | 1 | 59829 | 897.44 | 15.84 |
|  | Total (every power cycle) |  |  |  | 61440 | 5667.44 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.092 |  |
|  | Battery life (in month) |  |  |  | 5797 hours = 242 days = 8.1 months |  |  |

Table 6: Summary for UE power consumption results for UE-based DL positioning

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| [Case ID#7], [Rel-17 ] | 1.62 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#8], [Rel-17 ] | 1.49 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#9], [potential enhancements ] | 1.04 | 0.7 | No (5.3 months) | No (11.3 months) |
| [Case ID#10], [potential enhancements ] | 0.051 | 14.5 | Yes | Yes |
| [Case ID#11], [potential enhancements ] | 0.131 | 5.7 | No (0.3 months) | No (6.3 months) |
| [Case ID#12], [potential enhancements ] | 0.092 | 8.1 | Yes | No (3.9 months) |

- Observation 2: For UE-assisted DL positioning

- Case ID#10 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 2) can meet the target requirement 12 months.

- Case ID#11 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can almost meet the target requirement 6 months.

- Case ID#12 with 1 eDRX cycle (30.72s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can meet the target requirement 6 months.

  - If the value of additional transmission energy changed to more than 3000, it will not be able to meet the target requirement 6 months.

UL based positioning

For the UL positioning, following agreements are archived in RAN1-109 e-meeting.

Agreement

- For UL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning:

  - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle;

  - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%;

  - UL SRS for positioning transmission with 0.5 ms duration;

  - (Optional) BWP switching with [1] ms duration;

  - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration;

Based on the evaluation parameters agreed in RAN1-109 e-meeting and RAN1-110 meeting, an example of the simplest procedure of SRS transmission with deep sleep state in 1 I-DRX cycle of 1280ms can be seen in Figure 3. And based on Figure 3, Table 7 provides the evaluation cases and assumptions, and Table 8 and Table 9 provide the UE power consumption result for each evaluation case for UL positioning.

Figure 3, the procedure of SRS transmission

Table 7: Evaluation cases and assumptions for UL positioning

| Evaluation assumption | [Case ID#13], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#14], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#15], [FR1], [UL based positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep state | Deep sleep state | Deep sleep state |
| DRX cycle | 1 I-DRX cycle: 1280ms | 1 I-DRX cycle: 1280ms | 1 eDRX cycle: 20.48s |
| paging reception | Without paging | Without paging | Without paging |
| RS periodicity | 1 I-DRX cycle | 8 I-DRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |

| Evaluation assumption | [Case ID#16], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#17], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#18], [FR1], [UL based positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Ultra sleep state (option 2) | Ultra sleep state (option 1) | Ultra sleep state (option 1) |
| DRX cycle | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 30.72s |
| paging reception | Without paging | without paging | without paging |
| RS periodicity | 1 eDRX cycle | 1 eDRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |

Table 8: UE power consumption result for each evaluation case for UL positioning

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#13 | SSB | 50 | 4 | 1 | 4 | 200 | 5.02 |
|  | SRS transmission | 210 | 1 | 1 | 1 | 210 | 5.28 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 5.02 |
|  | Deep sleep transition | 450 | 40 | 2 | 80 | 900 | 22.6 |
|  | Deep sleep | 1 |  | 1 | 2471 | 2471 | 62.07 |
|  | Total (every power cycle) |  |  |  | 2560 | 3981 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.56 |  |
|  | Battery life (in month) |  |  |  | 342 hours = 14.2 days = 0.5 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#14 | SSB | 50 | 4 | 8 | 32 | 1600 | 5.2 |
|  | SRS transmission | 210 | 1 | 1 | 1 | 210 | 0.69 |
|  | paging | 50 | 4 | 8 | 32 | 1600 | 5.2 |
|  | Deep sleep transition | 450 | 40 | 16 | 640 | 7200 | 23.5 |
|  | Deep sleep | 1 |  | 8 | 19775 | 19775 | 64.55 |
|  | Total (every power cycle) |  |  |  | 20480 | 30633 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.50 |  |
|  | Battery life (in month) |  |  |  | 356 hours = 15 days = 0.5 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#15 | SSB | 50 | 4 | 1 | 4 | 200 | 0.47 |
|  | SRS transmission | 210 | 1 | 1 | 1 | 210 | 0.49 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 0.47 |
|  | Deep sleep transition | 450 | 40 | 2 | 80 | 900 | 2.1 |
|  | Deep sleep | 1 |  | 1 | 40871 | 40871 | 95.88 |
|  | Total (every power cycle) |  |  |  | 40960 | 42629 |  |
|  | Slot-averaged power unit |  |  |  |  | 1.04 |  |
|  | Battery life (in month) |  |  |  | 513 hours = 21 days = 0.7 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#16 | SSB | 50 | 4 | 1 | 4 | 200 | 9.2 |
|  | SRS transmission | 210 | 1 | 1 | 1 | 210 | 9.7 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 9.2 |
|  | Ultra sleep transition | 450 | 50 | 2 | 100 | 900 | 41.5 |
|  | ultra sleep | 0.01 |  | 1 | 40851 | 408.51 | 18.84 |
|  | Total (every power cycle) |  |  |  | 40960 | 2168.49 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.053 |  |
|  | Battery life (in month) |  |  |  | 10062 hours = 419 days = 14 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#17 | SSB | 50 | 4 | 1 | 4 | 200 | 3.85 |
|  | SRS transmission | 210 | 1 | 1 | 1 | 210 | 4.04 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 3.85 |
|  | Ultra sleep transition | 2000 | 800 | 2 | 1600 | 4000 | 76.92 |
|  | ultra sleep | 0.015 |  | 1 | 39351 | 590.27 | 11.35 |
|  | Total (every power cycle) |  |  |  | 40960 | 5200.27 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.127 |  |
|  | Battery life (in month) |  |  |  | 4199 hours = 175 days = 5.8 months |  |  |

| Evaluation case | Power states | Relative power unit | Duration (in slots) | Instances | Sum Durations (in slots) | Relative power | Power ratio |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case ID#18 | SSB | 50 | 4 | 1 | 4 | 200 | 3.6 |
|  | SRS transmission | 210 | 1 | 1 | 1 | 210 | 3.8 |
|  | paging | 50 | 4 | 1 | 4 | 200 | 3.6 |
|  | Ultra sleep transition | 2000 | 800 | 2 | 1600 | 4000 | 72.63 |
|  | ultra sleep | 0.015 |  | 1 | 59831 | 897.47 | 16.3 |
|  | Total (every power cycle) |  |  |  | 61440 | 5507.47 |  |
|  | Slot-averaged power unit |  |  |  |  | 0.090 |  |
|  | Battery life (in month) |  |  |  | 5926 hours = 247 days = 8.2 months |  |  |

Table 9: Summary for UE power consumption results for UL positioning

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| [Case ID#13], [Rel-17 ] | 1.56 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#14], [Rel-17 ] | 1.5 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#15], [potential enhancements ] | 1.04 | 0.7 | No (5.3 months) | No (11.3 months) |
| [Case ID#16], [potential enhancements ] | 0.053 | 14 | Yes | Yes |
| [Case ID#17], [potential enhancements ] | 0.127 | 5.8 | No (0.2 months) | No (6.2 months) |
| [Case ID#18], [potential enhancements ] | 0.09 | 8.2 | Yes | No (3.8 months) |

- Observation 3: For UL positioning

- Case ID#16 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 2) can meet the target requirement 12 months.

- Case ID#17 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can almost meet the target requirement 6 months.

- Case ID#18 with 1 eDRX cycle (30.72s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can meet the target requirement 6 months.

  - If the value of additional transmission energy changed to more than 3000, it will not be able to meet the target requirement 6 months.

Potential enhancement

Ultra-deep sleep state

According to the above evaluation results on the power consumption for each positioning method, Ultra-deep sleep state can reduce the power consumption to archive the battery life for LPHAP device. Thus we prefer to support defining Ultra-deep sleep state for LPHAP device.

- Proposal 1: Support to define Ultra-deep sleep state for LPHAP device.

DRX based positioning

Also based on the above evaluation results on the power consumption for each positioning method, eDRX cycle can reduce the power consumption to archive the target battery life for LPHAP device. Thus it is better to configure eDRX cycle with 20.48s and 30.72s for LPHAP device.

- Proposal 2: eDRX cycle with 20.48s and 30.72s can be configured to archive the target battery life for LPHAP device.

During the discussion of power saving WI in R17, for paging with DRX cycle, new DCI format 2_7 is agreed for notifying the paging early indication (PEI) and TRS availability indication for one or more UEs, the principle can be shown in Figure 1. It means for one PO, if the early indication indicate the UE not need to wake up for monitoring PO, UE will not wake up during the indicated PO. Thus power consumption can be reduced. But it is possible that the PRS or the SRS is overlapped with non-wake up POs. In order to satisfy the requirement of accuracy and latency for positioning for INACTIVE and IDLE UE, the PRS in non-wake up PO may be needed to be measured and the SRS in non-wake up PO may be needed to be transmitted.

Figure 4, Principle of PEI

- Proposal 3: Study SRS transmission or PRS measurement in PO indicated not necessary to wake up by DCI format 2_7.

UL positioning enhancement

In our understanding, the UL positioning methods can reduce more power consumption than DL positioning methods since UE only needs to transmit positioning SRS for UL positioning methods. The enhancements on UL positioning methods can be considered to meet the low power consumption requirements.

In Rel-17, the UL positioning methods were introduced for RRC Inactive state, but the positioning SRS configuration for RRC inactive will be released when UE performs cell reselection, the network should provide the new SRS configuration to UE, and it will lead more UE power consumption since the additional signaling exchange procedures are needed. Therefore, if positioning SRS configuration provided by network can still be used by the UE when UE performs the cell reselection, the UE power consumption can be reduced.

According to the use case, positioning interval is 15s to 30s, thus UE may only need to send SRS every 15s to 30s. For periodic SRS configuration, the longer period of SRS can satisfy the requirements of the use case and it can also reduce UE power consumption,

- Proposal 4: The positioning SRS con be configured per cell group for UE power consumption reduction.

If the cell group changes, the positioning SRS will be not valid. In this case, UE in RRC_ INACTIVE need to request the new SRS configuration from gNB. Thus, random access can be used for SRS configuration request.

- Proposal 5: Study SRS configuration request by random access.

Conclusion

In this contribution, we discussed about LPHAP (Low Power High Accuracy Positioning). Based on above discussion, we provide the following observations and proposals.

Table 1: Evaluation cases and assumptions for UE-based DL positioning

| Evaluation assumption | [Case ID#1], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#2], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#3], [FR1], [UE-based DL positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep state | Deep sleep state | Deep sleep state |
| DRX cycle | 1 I-DRX cycle: 1280ms | 1 I-DRX cycle: 1280ms | 1 eDRX cycle: 20.48s |
| paging reception | Without paging | Without paging | Without paging |
| RS periodicity | 1 I-DRX cycle | 8 I-DRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |
| Evaluation assumption | [Case ID#4], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#5], [FR1], [UE-based DL positioning], [LPHAP device type A] | [Case ID#6], [FR1], [UE-based DL positioning], [LPHAP device type A] |
| Sleep state | Ultra sleep state (option 2) | Ultra sleep state (option 1) | Ultra sleep state (option 1) |
| DRX cycle | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 30.72s |
| paging reception | Without paging | without paging | without paging |
| RS periodicity | 1 eDRX cycle | 1 eDRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |

Table 4: Evaluation cases and assumptions for UE-assisted DL positioning

| Evaluation assumption | [Case ID#7], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#8], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#9], [FR1], [UE-assisted DL positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep state | Deep sleep state | Deep sleep state |
| DRX cycle | 1 I-DRX cycle: 1280ms | 1 I-DRX cycle: 1280ms | 1 eDRX cycle: 20.48s |
| paging reception | Without paging | Without paging | Without paging |
| RS periodicity | 1 I-DRX cycle | 8 I-DRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT per I-DRX cycle | CG-SDT per 8 I-DRX cycle | CG-SDT per eDRX cycle |
| implementation factor K | 1 | 1 | 1 |
| Evaluation assumption | [Case ID#10], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#11], [FR1], [UE-assisted DL positioning], [LPHAP device type A] | [Case ID#12], [FR1], [UE-assisted DL positioning], [LPHAP device type A] |
| Sleep state | Ultra sleep state (option 2) | Ultra sleep state (option 1) | Ultra sleep state (option 1) |
| DRX cycle | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 30.72s |
| paging reception | Without paging | without paging | without paging |
| RS periodicity | 1 eDRX cycle | 1 eDRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT per eDRX cycle | CG-SDT per eDRX cycle | CG-SDT per eDRX cycle |
| implementation factor K | 1 | 1 | 1 |

Table 7: Evaluation cases and assumptions for UL positioning

| Evaluation assumption | [Case ID#13], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#14], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#15], [FR1], [UL based positioning], [LPHAP device type A] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep state | Deep sleep state | Deep sleep state |
| DRX cycle | 1 I-DRX cycle: 1280ms | 1 I-DRX cycle: 1280ms | 1 eDRX cycle: 20.48s |
| paging reception | Without paging | Without paging | Without paging |
| RS periodicity | 1 I-DRX cycle | 8 I-DRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |
| Evaluation assumption | [Case ID#16], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#17], [FR1], [UL based positioning], [LPHAP device type A] | [Case ID#18], [FR1], [UL based positioning], [LPHAP device type A] |
| Sleep state | Ultra sleep state (option 2) | Ultra sleep state (option 1) | Ultra sleep state (option 1) |
| DRX cycle | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 20.48s | 1 eDRX cycle: 30.72s |
| paging reception | Without paging | without paging | without paging |
| RS periodicity | 1 eDRX cycle | 1 eDRX cycle | 1 eDRX cycle |
| M-sample | 1 | 1 | 1 |
| RRM measurement | No | No | No |
| BWP switching | No | No | No |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | No | No | No |
| implementation factor K | 1 | 1 | 1 |

Table 10: Summary for UE power consumption results

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| [Case ID#1], [Rel-17 ] | 1.52 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#2], [Rel-17 ] | 1.48 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#3], [potential enhancements ] | 1.03 | 0.8 | No (5.2 months) | No (11.2 months) |
| [Case ID#4], [potential enhancements ] | 0.045 | 16 | Yes | Yes |
| [Case ID#5], [potential enhancements ] | 0.125 | 5.9 | No (0.1 months) | No (6.1 months) |
| [Case ID#6], [potential enhancements ] | 0.088 | 8.4 | Yes | No (3.6 months) |
| [Case ID#7], [Rel-17 ] | 1.62 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#8], [Rel-17 ] | 1.49 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#9], [potential enhancements ] | 1.04 | 0.7 | No (5.3 months) | No (11.3 months) |
| [Case ID#10], [potential enhancements ] | 0.051 | 14.5 | Yes | Yes |
| [Case ID#11], [potential enhancements ] | 0.131 | 5.7 | No (0.3 months) | No (6.3 months) |
| [Case ID#12], [potential enhancements ] | 0.092 | 8.1 | Yes | No (3.9 months) |
| [Case ID#7], [Rel-17 ] | 1.56 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#8], [Rel-17 ] | 1.5 | 0.5 | No (5.5 months) | No (11.5 months) |
| [Case ID#9], [potential enhancements ] | 1.04 | 0.7 | No (5.3 months) | No (11.3 months) |
| [Case ID#10], [potential enhancements ] | 0.053 | 14 | Yes | Yes |
| [Case ID#11], [potential enhancements ] | 0.127 | 5.8 | No (0.2 months) | No (6.2 months) |
| [Case ID#12], [potential enhancements ] | 0.09 | 8.2 | Yes | No (3.8 months) |

- Observation 1: For UE-based DL positioning

- Case ID#4 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 2) can meet the target requirement 12 months.

- Case ID#5 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can almost meet the target requirement 6 months.

- Case ID#6 with 1 eDRX cycle (30.72s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can meet the target requirement 6 months.

  - If the value of additional transmission energy changed to more than 3000, it will not be able to meet the target requirement 6 months.

- Observation 2: For UE-assisted DL positioning

- Case ID#10 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 2) can meet the target requirement 12 months.

- Case ID#11 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can almost meet the target requirement 6 months.

- Case ID#12 with 1 eDRX cycle (30.72s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can meet the target requirement 6 months.

  - If the value of additional transmission energy changed to more than 3000, it will not be able to meet the target requirement 6 months.

- Observation 3: For UL positioning

- Case ID#16 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 2) can meet the target requirement 12 months.

- Case ID#17 with 1 eDRX cycle (20.48s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can almost meet the target requirement 6 months.

- Case ID#18 with 1 eDRX cycle (30.72s) and Ultra sleep state (Option 1 with 2000 as additional transmission energy) can meet the target requirement 6 months.

  - If the value of additional transmission energy changed to more than 3000, it will not be able to meet the target requirement 6 months.

- Proposal 1: Support to define Ultra-deep sleep state for LPHAP device.

- Proposal 2: eDRX cycle with 20.48s and 30.72s can be configured to archive the target battery life for LPHAP device.

- Proposal 3: Study SRS transmission or PRS measurement in PO indicated not necessary to wake up by DCI format 2_7.

- Proposal 4: The positioning SRS con be configured per cell group for UE power consumption reduction.

- Proposal 5: Study SRS configuration request by random access.

References

- RP-213588 Revised SID on Study on expanded and improved NR positioning

- Chairman notes of RAN1-109 e-meeting

- Chairman notes of RAN1-110 meeting