3GPP TSG RAN WG1 #110bis-e			                        R1-2209310

e-Meeting, October 10th – 19th, 2022

Source: 	CMCC

Title:	Remaining issues on HARQ-ACK feedback for multicast

Agenda item:	8.12

Document for:	Discussion & Decision

- Introduction

In this contribution, some remaining issues on HARQ-ACK feedback for multicast will be discussed.

- Remaining issues on NACK-only based feedback for multicast

2.1 Codebook for NACK-only

The following agreement is achieved in RAN1#109-e meeting regarding how to count and order the HARQ-ACK bits for NACK-only for Alt4, and the detailed solution for Case 2 and Case 3 needs further down-selection.

Agreement

For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, further down-selection on the following cases/options in the next meeting:

- Case 2: for the case of all UEs configured with the same set of G-RNTIs

- Opt2-1: support Alt4 for this case

  - Opt2-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values.

  - Opt2-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs

- Opt2-2: does not support Alt4 for this case.

- Case 3: for the case of different UEs configured with different G-RNTIs

- Opt3-1: support Alt4 for this case

  - Opt3-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values.

  - Opt3-1-2: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs.

- Opt3-2: does not support Alt4 for this case.

Regarding Case 2, Opt 2-1-1 follows the legacy unicast DAI counting rule, but compared with unicast if the last DCI of any G-RNTI is missed, gNB and UE are not aligned with the count and order of TBs which causes the last DCI missing issue is more serious that unicast. For Opt 2-1-2, the additional C-DAI* can mitigate the last DCI missing issue and provides the similar performance as unicast.

Regarding Case 3, we think this case should also be supported which gives gNB more flexible to configure RNTIs for different UEs compared with case 2. As the PUCCH resource indication, as the illustration in FL summary [2], network may retransmit TBs for undifferentiated cases, but this issue can be avoided by gNB’s proper configuration using Opt 3-1-1. For example, assuming UE1 receives TB1, TB2 for G-RNTI 1 and UE2 receives TB1, TB2 for G-RNTI 2, gNB can configure orthogonal PUCCH resources for G-RNTI 1 and G-RNTI 2. Although the HAQR-ACK value may be the same for two UEs, gNB can also differentiate which TB is decoded incorrectly via the detection of orthogonal PUCCH resources. For Opt 3-1-2, the usage of C-DAI* should be combined with legacy C-DAI and UE needs to feedback ACK values for unreceived TBs.

In a summary, we think Opt 2-1-1 for Case 2 and Opt 3-1-1 for Case 3 are feasible which don’t need additional RRC signalling design or UE features compared with Opt 2-1-2 and Opt 3-1-2 and can be supported in maintenance phase.

Proposal 1. For addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, Opt 2-1-1 and Opt 3-1-1 are supported.

2.2 PRI indication in NACK-only

Agreement

For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows:

- The mapping is applied to both a single G-RNTI

  - FFS: multiple configured G-RNTIs cases.

- FFS: how to order the TBs, whether/how to account for missed or non-scheduled TBs

- FFS whether/how PRI is used for indication.

The another remaining issue is the PRI indication in NACK-only based HARQ-ACK feedback.

We have defined two NACK-only modes, which NACK-only mode 1 is converted into ACK/NACK based for more than one TB and NACK-only mode2 is the PUCCH resource selection based on PDSCH decoding result. For NACK-only mode 1, the PRI is used to indicate the PUCCH resource similar to ACK/NACK based HARQ-ACK feedback. For NACK-only mode 2, since the PUCCH resources is based on the PDSCH decoding result, the PRI is useless and can be ignored.

Proposal 2. For PRI included in DCI format 4_1/4_2:

- if the G-RNTI is configured with NACK-only mode1, PRI indicates the PUCCH for the HARQ-ACK feedback;

- if the G-RNTI is configured with NACK-only mode2, PRI is ignored.

- Conclusions

In this contribution, some remaining issues on HARQ-ACK feedback for multicast, and the following proposals are made.

Proposal 1. For addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, Opt 2-1-1 and Opt 3-1-1 are supported.

Proposal 2. For PRI included in DCI format 4_1/4_2:

- if the G-RNTI is configured with NACK-only mode1, PRI indicates the PUCCH for the HARQ-ACK feedback;

- if the G-RNTI is configured with NACK-only mode2, PRI is ignored.

- References

- Chairman’s notes RAN1#109-e meeting

- R1-2207714, FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs, Moderator (Huawei)