3GPP TSG RAN WG1 Meeting #110bis-e		R1-2209772

e-Meeting, October 10th – 19th, 2022

Agenda Item: 9.9.2

Source: MediaTek Inc.

Title: On multi-carrier UL Tx switching scheme

Document for: Discussion and Decision

# Introduction

A work item on multi-carrier enhancements was approved in RAN#94-e with the following objective on UL Tx switching ‎[1]:

- Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

In this contribution, we discuss the pros and cons of UL Tx switching on more than two carriers and provide views on the UE’s procedures to facilitate the implementation of such feature.

# UL Tx switching in R18

UL Tx switching between two configured bands was introduced in R16 and further enhanced in R17 to improve the UL performance by allowing the 2Tx to be used any one of the two bands with UL MIMO. However, 2TX UE can be configured with at most 2 UL bands, which only can be changed by RRC reconfiguration, and UL Tx switching can only be performed across the 2 configured UL bands. Allowing the network to configure the UE with more than 2 bands and trigger the UL Tx switching dynamically could enhance the system performance be allowing more UL transmission opportunities.

In this section, we discuss the possible configurations for UL Tx switching in R18.

Switching period: Unlike R16/R17 UL Tx switching, in R18 the Tx switching will be among more than two bands. This will require more complex implementation on the UE side and, larger switching period might be required compared to R16/R17 switching period.

RAN4 has provided the following response regarding the required switching period for R18 UL Tx switching across 3 or 4 bands ‎[2].

RAN1 Question: RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on potential increase of switching period and UE’s complexity in case of UL Tx switching across 3 or 4 bands in comparison to 2 bands. RAN4 Answer: Agreement: On the length of switching period: - For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL. - The length of switching period is applied per band pair for each band combination. - For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17. - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured. - For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17. On the UE complexity: RAN4 has not identified any technical difficulty for UE to prevent realizing Tx switching across 3 or 4 bands. RAN4 would like to recommend the UE memory sharing issue to be further discussed in RAN1 if necessary.

From RAN4 response, the following can be concluded:

- The existing set of switching periods (i.e., {35 us, 140 us, 210 us}) can be reused.

- The switching period is per band pair, and it can be different for 1Tx-2Tx switching and 2Tx-2Tx switching.

Supported number on switching cases: For option#2 of Tx switching where simultaneous transmission on the two bands is supported, the possible case for the 3 bands scenario is provided in the table below. Supporting the switching between all these cases could be challenging for most of the UEs. Also, from network operation perspective, switching between all these cases may not be crucial for the UL performance. Hence, it is essential to allow the UE to report which Tx switching mode (switchedUL or dualUL) is supported for any band combination.

Table 1: The possible cases for UL Tx switching among 3 bands.

|  | Antenna Tx Mode | #antenna ports for UL |
| --- | --- | --- |
| Case 1 | 1T+1T+0T | (1P+0P+0P), (1P+1P+0P), (0P+1P+0P) |
| Case 2 | 1T+0T+1T | (1P+0P+0P), (1P+0P+1P), (0P+0P+1P) |
| Case 3 | 0T+1T+1T | (0P+1P+0P), (0P+1P+1P), (0P+0P+1P) |
| Case 4 | 2T+0T+0T | (2P+0P+0P), (1P+0P+0P) |
| Case 5 | 0T+2T+0T | (0P+2P+0P), (0P+1P+0P) |
| Case 6 | 0T+0T+2T | (0P+0P+2P), (0P+0P+1P) |

- For UL Tx switching among 3/4 bands, the required switching period is reported separately from R16/R17 switching period.

- Reuse the existing set for switching periods {35 us, 140 us, 210 us}.

- The switching period is reported per band pair.

- For each band pair, the switching period can be reported separately for 1Tx-2Tx and 2Tx-2Tx switching.

- For each band pair, the switching period can be reported separately for “2 bands” and “3/4 bands” switching.

- The supported Tx switching option (switchedUL or dualUL) is reported per band pair.

In RAN1#110, the following Working Assumption was reached on the options for Tx switching between 3 or 4 bands.

Working Assumption - If Rel-18 UL Tx switching is supported, following switching mechanism is considered as baseline for the Rel-18 UL Tx switching across 3 or 4 bands - Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via dynamic grant and/or RRC configuration for UL transmission - RAN1 will support one or more of following complexity reduction options, considering at least the potential additional preparation time, additional interruption time, and RF complexity for certain switching cases/patterns, if Rel-18 UL Tx switching is supported based on Alt.1, and companies are encouraged to investigate options with striving for down-selection at RAN1#110bis-e. - Option 1: UE is allowed to support only some of concurrent UL cases (band pairs) - FFS: at least one band pair should be supported as in Rel-17 - FFS: for both 3 and 4 bands cases or only for 4 bands case - FFS: potential capability/RRC signaling - Option 2: UE is allowed to support 2 ports transmission only on some of bands out of configured bands for UL Tx switching - FFS: at least two bands should support up to 2 Tx as in Rel-17 - FFS: for both 3 and 4 bands cases or only for 4 bands case - FFS: for both switched UL and dual UL cases or only for dual UL case - FFS: whether/how to reuse or extend existing capability/RRC signaling - Option 3: UE is allowed with more preparation procedure time (or interruption time) only for some specific switching cases/patterns - FFS: specific switching cases/patterns where more preparation procedure time (or interruption time) is necessary, e.g., switching patterns not existed in Rel-17 - FFS: how long preparation procedure time and/or interruption time is necessary, and whether RAN4 involvement is necessary - FFS: whether/how to report/indicate the specific switching cases/patterns and/or value(s) of preparation procedure time (or interruption time) - FFS: what is the definition of preparation procedure time or interruption time, including whether interruption happens during the preparation procedure time and whether it includes switching period - FFS: whether/how long minimum interval between two succeeding UL Tx switching is necessary - Option 4: UE is allowed to support only some of band pairs for tx switching - FFS: at least one band pair should be supported as in Rel-17 - FFS: for both 3 and 4 bands cases or only for 4 bands case - FFS: for switched UL and/or dual UL - FFS: potential capability/RRC signaling - Other options are not precluded

Option#1 and Option#2 should be supported as they are aligned with what is assumed for R16/R17 UL Tx switching.

Option#3 can be supported to allow some UEs report longer processing time for R18 UL Tx switching. However, the extra Tx switching time should be applicable to the switching cases (among the 3 or 4 bands) as there is no difference among these cases from PUSCH preparation time perspective.

Regarding Option#4, it is not clear how this could help with UE complexity. For example, for UL Tx switching among 3 bands (A, B, C), if the UE reports the following:

- SwitchedUL (A, B),

- SwitchedUL (A, C),

- No UL Tx switching (B, C),

- 2 ports transmission on A, B & C.

This implies the following is not supported (Please see Table 1 for the cases definition):

- Direct switching between Case#1 & Case#2

- Direct switching between Case#1 & Case#6

- Direct switching between Case#5 & Case#6

However, switching from Case#1 to Case#2 can be achieved by the following switching sequence: Case#1  Case#3  Case#2. Similar thing can be achieved with the other “not supported” switching cases. Thus, Option#4 doesn’t help in reducing the UE complexity and it only add longer switching procedure.

- Option#4 doesn’t offer UE complexity reduction, and it will require two switching gaps in some scenarios, which degrades the UL performance.

- For UL Tx switching among 3/4 bands:

- Support Option#1 and Option#2.

- Do not support Option#4.

- Consider Option#3 with the following modification: “UE is allowed with more preparation procedure time (or interruption time) only for some specific all switching cases/patterns”.

# Conclusion

In this contribution, we discussed UL Tx switching for R18 and we have the following observation and proposals:

- Option#4 doesn’t offer UE complexity reduction, and it will require two switching gaps in some scenarios, which degrades the UL performance.

- For Proposal 1: For UL Tx switching among 3/4 bands, the required switching period is reported separately from R16/R17 switching period.

- Reuse the existing set for switching periods {35 us, 140 us, 210 us}.

- The switching period is reported per band pair.

- For each band pair, the switching period can be reported separately for 1Tx-2Tx and 2Tx-2Tx switching.

- For each band pair, the switching period can be reported separately for “2 bands” and “3/4 bands” switching.

- The supported Tx switching option (switchedUL or dualUL) is reported per band pair.

- For UL Tx switching among 3/4 bands:

- Support Option#1 and Option#2.

- Do not support Option#4.

- Consider Option#3 with the following modification: “UE is allowed with more preparation procedure time (or interruption time) only for some specific all switching cases/patterns”.

# References

- RP-213577, “New WID on Multi-carrier enhancements”, DOCOMO, RAN#94-e, December 2021.

- R4-2214464, “Reply LS on UL Tx switching across 3 or 4 bands”, China Telecom, RAN4#104-e, Aug. 2022.