3GPP TSG RAN WG1 #110bis-e	R1-2210069

e-Meeting, October 10th – 19th, 2022

Source:	Panasonic

Title: 	Discussion on network verified UE location for NTN

Agenda Item:		9.11.2

Document for:	Discussion and decision

# Introduction

The Rel.18 work item on Non-Terrestrial Network (NTN) [1] has been started with the following scope:

- Coverage enhancement

- NR-NTN deployment in above 10 GHz bands

- Network verified UE location

- NTN-TN and NTN-NTN mobility and service continuity enhancements

On network verified UE location, the study phase in RAN has been finished and TR 38.882 on the necessity of network verified UE location was created [2]. The following is the scope of study in the working groups.

The study in [RAN2,RAN1,RAN3], which will study and evaluate solutions for the network to verify UE reported location information, shall consider the following aspects: - The scenario of single satellite (or HAPS) in view by the UE at a time is considered with higher priority. - Multiple satellite (or HAPS) in view by the UE may be considered if time allows - Assume that the UE is attached to a network (so that its context has been set up in the network) for the purpose of positioning - Different solutions or positioning methods for NGSO, GSO or HAPS are not precluded - When considering solutions based on positioning methods, existing 3GPP defined RAT dependent positioning methods shall be considered as baseline. Other methods are not precluded. - Solutions using existing NG-RAN architecture and procedures shall be considered

In this document, we discuss network verified UE location for NR NTN.

# Discussion

It is described in TR 38.882 that relying only on the GNSS based location information reported by the UE is not considered reliable by SA3-LI due to, e.g., intentional (e.g., maliciously tampering by user or by 3rd party) or unintentional (e.g., interference) causes. Therefore, network verification of UE location is necessary. For the study, the UE location information is considered verified if the reported UE location is consistent with the network-based assessment to within 5-10 km (similar to terrestrial network macro cell size).

In the following, we describe a method based on multiple RTT measurements (see Figure 1) which allows for very accurate UE location verification.

| - NTN with single LEO satellite and multiple timing | b) Satellite positions during one RTT measurement |
| --- | --- |
| Figure 1: Positioning with multiple transmission points |  |

## Location estimation based on non-linear least-squares minimization

Trilaterion is a method for positioning based on the solution of a non-linear system of equations. A receiver is provided with data telling it when the data was emitted. Since the receiver knows when the data was received, it can also derive the propagation time, and thus knows the distance between itself and the transmitter, e.g., a satellite. With signals from four transmitters a receiver can uniquely (up to measurement noise) identify the receiver’s position in space.

Let x=x,y,zT denote the receiver location, pi=pi,x,pi,y,pi,zTthe transmitter position, Ti the propagation delay, and di the distance between transmitter and receiver

x-pi=Tic0=di

With four transmitters in sight, we have a system of four equations

x-p0=d0x-p1=d1x-p2=d2x-p3=d3

We can estimate the UE location x by iteratively minimizing the sum of least residual squares

x=argminx x-p0-d02+x-p1-d12+x-p2-d22+x-p3-d32

The algorithm for iterative minimization is known as Gauss-Newton method. It is a multi-dimensional generalization of the Newton method for finding the root of a scalar-valued function and can be formulated as follows:

Jxk+1-xk=-fxk

We define the residual functions fix=x-pi-di. The Jacobian matrix J comprises the gradients (∇fix) as row vectors of the residual functions fix stacked on top of each other.

J=∇f0x,∇f1x,∇f2x,∇f3xT

fx=f0x,f1x,f2x,f3(x)T

For one iteration, we can solve the equation system Jv=-f(xk) with standard methods of linear algebra for vector v and obtain an update of the location estimate via xk+1=xk+v.

### Method 1

The method above determines the UE location as the intersection of four spheres. The underlying assumption is that the transmitters are either stationary, like in a terrestrial network, or that UE is provided with the position of the transmitter at the time instant when the data was emitted, like in GPS. For NTN, we consider RAT-dependent techniques, i.e., we do not rely on GPS. If we consider multiple measurements from a single satellite, then the satellite movement must be accounted for – in particular, for fast moving LEO satellites. The way we model this is to say that during one Round-Trip-Time the satellite moves between positions on its orbit, say pi and qi for the i-th measurement. In other words, when the i-th RTT-measurement is started the satellite is at position pi and when the RTT-measurement is finished the satellite is at position qi. Then the NTN-adjusted function describing the distance between satellite and UE becomes

x-pi+x-qi=RTTi∙c0

This is the equation for an ellipse, which is defined as the set of points whose sum of distances to the focal points is constant. Conceptually, we can use this function as a basis for the Gauss-Newton method described above. Before that we must account for the rotation of Earth, however, since during one RTT not only the satellite moves but also Earth.

The necessary modifications can best be understood when the problem is reduced to two dimensions, i.e., we consider the equatorial plane only. Afterwards the extension to three dimensions is fairly straightforward.

The satellite position at time t is denoted as

pSATt=R+hexpj2πtTSAT

The UE location is denoted as

pUEt=Rexpj2πtTE+φUE

Tsat is the orbital period of the satellite, and TE is Earth’s sidereal rotation period (~24h). The static phase φUE allows to position UE within the cell. Based on these functions we denote the propagation dekay in Dowlink and Uplink as τi,0 and τi,1 for the i-th RTT-measurement and they are computed based on these implicit equations

τi,0=pSATt+i∆-pUEt+i∆+τi,0c0

τi,1=pUEt+i∆+τ0-pSATt+i∆+τi,0+τi,1c0

RTTt+i∆=τi,0+τi,1

Note that the RTT(t+i∆) depends on the location of the UE pUEt+i∆+τ0 at time t+i∆+τ0. For multiple RTT measurements at subsequent time instances t, t+∆ etc. the UE location is different for each RTT measurement and must be accounted for.

τi,0=R+hexpj2πt+i∆TSAT-Rexpj2πt+i∆+τi,0TE+φUEc0=R+hexpj2πt+i∆TSATexp-j2πi∆+τi,0TE-Rexpj2πtTE+φUEc0=pSAT(t+i∆)exp-j2πi∆+τi,0TE-pUEtc0

τi,1=Rexpj2πt+i∆+τi,0TE+φUE-R+hexpj2πt+i∆+τi,0+τi,1TSATc0=Rexpj2πtTE+φUE-R+hexpj2πt+τi,0+τi,1TSATexp-j2πi∆+τi,0TEc0=pUE(t)-pSATt+τi,0+τi,1exp-j2πi∆+τi,0TEc0

Based on a single RTT measurement, the satellite knows its positions pSATt+i∆ and pSATt+i∆+τ0+τ1 and the corresponding RTT(t+i∆). To determine the UE location pUE(t) in this two-dimensional scenario three measurements are needed. In practice, for three dimensions in space four measurements are required. To compute the location based on the Gauss-Newton method, the satellite positions for each RTT-measurement must be de-rotated with the term exp-j2πi∆+τi,0TE, i.e., by a phase which is equivalent to the propagation delay τ0 in the downlink from satellite to UE plus the time difference ∆ between two RTT-measurements. However, only the sum of τ0 and τ1 are known as RTT(t+i∆). We can reasonably assume that both delays are similar. Hence, with τ0≈τ1 we assume that τi,0≈RTTt+i∆2.

τ0≈pSAT(t)exp-j2πi∆+RTTt+i∆2TE-pUEtc0

τ1≈pUE(t)-pSATt+τ0+τ1exp-j2πi∆+RTTt+i∆2TEc0

Thus, the residual function for the Gauss-Newton method reads

pSAT(t)exp-j2πi∆+RTTt+i∆2TE-pUEt+pUE(t)-pSATt+τ0+τ1exp-j2πi∆+RTTt+i∆2TE≈c0RTT(t+i∆)

Conclusion

For the i-th RTT-measurement we define pi=pSATti and qi=pSATti+τ0+τ1 and RTTi=RTTt+i∆.

piexp-j2πi∆+RTTi2TE-pUEt+pUE(t)-qiexp-j2πi∆+RTTi2TE≈c0RTTi

The extension to three dimensions can be done by replacing the rotation factor exp-j2πi∆+RTTi2TE with a suitable rotation matrix. For that, the cross-product of pi and qi yields on orthogonal vector around which we can rotate both vectors on the plane which contains them both. The corresponding technique is known as “Rodrigues rotation”.

### Method 2

Since the RTT is comprised of UL and DL, it is in principle possible to estimate the UE location with only two RTT measurements for the two-dimensional setup. That is, we can assume that the delay over UL and DL equal roughly half of the RTT:

τi,0=pSAT(t)exp-j2πi∆+RTTi2TE-pUEt+i∆c0≈RTTi2

τi,1=pUE(t)-pSATt+τ0+τ1exp-j2πi∆+RTTt2TEc0≈RTTi2

Hence, with two measurements we can establish four equations and thus uniquely solve for the UE location. By analogy, for the three-dimensional case three RTT measurements are sufficient. In practice, this method is more error-prone since it suffers from an ill-conditioned system of equations. The problem is that the satellite positions at pSAT(t) and pSATt+τ0+τ1 are quite similar. Thus, the two equations above describe two circles of the same radius, RTTi2, which are almost congruent.

## Preliminary simulation results

We consider LEO600 and LEO1200 satellites in a circular orbit, providing a cell with a beam footprint RBF=500km (center-to-edge). The minimum elevation angle under which the satellite is visible is 10°. UE and satellite are uniformly distributed within cell and visibility area of the satellite. RTT-measurements are performed in steps of ∆ seconds. That is, for Method 1 we record the satellite positions p0,q0,p1,q1, p2,q2 corresponding to the time instances t, t+∆, and t+2∆. For Method 2, we record the satellite positions p0,q0,p1,q1 corresponding to the time instances t and t+∆.

### Three measurements (Method 1)

Figure 2 depicts the CCDF of the error magnitude between the measured and the actual UE location. As a reminder, the only assumption underlying Method 1 is that the DL delay equals half the RTT, τi,0≈RTTi/2 . We observe that positioning the order of millimeters is possible, but note, that channel impairments have been ignored here.

| LEO600 | LEO1200 |
| --- | --- |
| Figure 2: Simulation results for Method 1 (three RTT-measurements) |  |

### Two measurements (Method 2)

Figure 3 depicts the CCDF of the error magnitude between the measured and the actual UE location for Method 2. Unlike Method 1, only two RTT-measurements are performed. We recognise a sensitivity of the result with respect to the RTT-measurement interval ∆. That is, the smaller ∆, the larger the expected error. We explained this above by an ill-conditioned equation system, i.e., the closer subsequent measurements are in time, the more similar the geometry of the underlying ellipses become. However, if the system design allows for sufficiently large measurement intervals, Method 2 is a viable option.

| LEO600 | LEO1200 |
| --- | --- |
| Figure 3: Simulation results for Method 2 |  |

### Performance with timing measurement error

In the FL summary R1-2207631 [4], it was proposed to evaluate the timing measurement error based on a truncated normal distribution. In the following, we have added a truncated normally distributed error to the RTT measurements with zero mean and a standard deviation of both 10ns and 100ns, which roughly corresponds to displacements of +/-3m and +/-30m. The results in Figure 4 (three RTT measurements) and Figure 5 (two RTT measurements) clearly illustrate that UE location determination is negatively affected, although the error remains under the limits of 5 to 10 km. We observe that increasing the interval ∆ between RTT measurements, i.e., the measurement window, improves the performance of the localization algorithm.

A comparison of Figure 4 and Figure 5, i.e., between three RTT measurements and two RTT measurements, should be done carefully, i.e., three measurements with an interval of Delta=2 s should be compared to two measurements with Delta=4 s, so that the total measurement interval is the same. With this caveat in mind, the performance of both approaches seem to be on par. We conjecture that three RTT-measurements mean in principle better performance, but this is offset by the presence of extra error term during the localization compared to the alternative. Whereas two measurements mean in principle worse performance but also one error term less. It is premature to decide on the number of required RTT measurements, and RAN1 should carefully consider these aspects.

Figure 4: Effect of RTT-measurement error on UE location accuracy for three RTT measurements (Method 1)

Figure 5: Effect of RTT-measurement error on UE location accuracy for two RTT measurements (Method 2)

Observation 1: Multi-RTT with a set of equations adjusted to the NTN-environment is a viable method to determine UE location with a verification accuracy of 5 to 10 km and with a single satellite.

Proposal 1: Adopt Multi-RTT as a method for network-based UE location verification.

Observation 2: A measurement window in the order of seconds may be required to achieve the required accuracy.

Observation 3: Multi-RTT can be modified such that three measurements (instead of four) are sufficient to determine UE location in three dimensions, but this requires a sufficiently large interval between subsequent RTT-measurement.

Observation 4: The interval between RTT-measurements has a stronger impact on the accuracy of UE location estimation than the number of RTT-measurements.

Proposal 2: RAN1 should carefully consider the number of required RTT-measurements for multi-RTT.

# Conclusion

In this document, we discussed issues on network verification of UE location for NTN. We observe and propose the following:

Observation 1: Multi-RTT with a set of equations adjusted to the NTN-environment is a viable method to determine UE location with a verification accuracy of 5 to 10 km and with a single satellite.

Observation 2: A measurement window in the order of seconds may be required to achieve the required accuracy.

Observation 3: Multi-RTT can be modified such that three measurements (instead of four) are sufficient to determine UE location in three dimensions, but this requires a sufficiently large interval between subsequent RTT-measurement.

Observation 4: The interval between RTT-measurements has a stronger impact on the accuracy of UE location estimation than the number of RTT-measurements.

Proposal 1: Adopt Multi-RTT as a method for network-based UE location verification.

Proposal 2: RAN1 should carefully consider the number of required RTT-measurements for multi-RTT.

# Reference

- RP-220953, “WID: NR NTN (Non-Terrestrial Networks) enhancements”, Thales

- TR38.882 V18.0.0

- TR38.821 V16.0.0

- R1-2207631, “FL Summary #4: Network verified UE location for NR NTN”, RAN1#110