3GPP TSG RAN WG1 #110bis-e R1-2210156 e-Meeting, October 10th – 19th, 2022 CR-Form-v12.0. CHANGE REQUEST. 38.213; CR; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on HARQ-ACK feedback for PDSCH scheduled by DCI format 4_1. Source to WG:; Lenovo. Source to TSG:; R1. Work item code:; NR_MBS-Core; Date:; 2022-09-30. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In current spec, for a given G-RNTI, when RRC configures presence of HARQ-ACK feedback enabling/disabling indication in the DCI format with CRC scrambled by the G-RNTI, this enabling/disabling HARQ-ACK feedback indication is only included in DCI format 4-2 with CRC scrambled by the G-RNTI. Hence, UE behavior is not clear upon reception of a PDSCH scheduled by DCI format 4-1 since HARQ-ACK feedback enabling/disabling indication is not included in DCI format 4-1. Since the principle of unconfigurable fields in DCI format 4-1 should be maintained, default UE behavior is required for the UE to always assume the HARQ-ACK feedback for PDSCH scheduled by DCI format 4-1 with CRC scrambled by a G-RNTI is enabled if RRC configures presence of HARQ-ACK feedback enabling/disabling indication in DCI format 4-2 with CRC scrambled by the G-RNTI.. -. Summary of change:; Add UE behaviors that if a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, the UE provides the HARQ-ACK information for PDSCH receptions scheduled by the multicast DCI format 4-1 associated with the G-RNTI or the G-CS-RNTI.. Consequences if not approved:; UE behavior on HARQ-ACK feedback is not clear upon reception of a PDSCH scheduled by DCI format 4-1.. Clauses affected:; 18. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR.. <Unchanged part omitted> # 18 Multicast Broadcast Services **********************<irrelevant part omitted>************************************************ A UE can be configured per G-RNTI or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI or G-CS-RNTI, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI, the UE provides the HARQ-ACK information for PDSCH receptions scheduled by the multicast DCI format 4-1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions based on an indication by the multicast DCI format 4_2 associated with the G-RNTI or the G-CS-RNTI [4, TS 38.212]. If a UE would multiplex multicast HARQ-ACK information according to the second HARQ-ACK reporting mode with multicast HARQ-ACK information according to the first HARQ-ACK reporting mode, or unicast HARQ-ACK information, or CSI reports in a first PUCCH or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH when the UE is provided moreThanOneNackOnlyMode, the UE considers that the UE would transmit the second PUCCH using any PUCCH resource from the PUCCH resources associated with the second HARQ-ACK reporting mode when all values of the HARQ-ACK information are 'ACK'. If a UE is provided multiple G-RNTIs or G-CS-RNTIs, a configuration for a HARQ-ACK codebook type applies to all G-RNTIs or G-CS-RNTIs. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE generates a Type-1 HARQ-ACK codebook as described in clauses 9.1.2, 9.1.2.1, and 9.1.2.2. If a UE is provided pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE generates a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. **********************<irrelevant part omitted>************************************************

<Unchanged part omitted>