3GPP TSG-RAN WG1 Meeting #110bis-e	  R1-2210366

e-Meeting, October 10 – 19, 2022

Agenda Item:	9.2.2.1

Source:	Moderator (Huawei)

Title:	Summary#2 of [110bis-e-R18-AI/ML-02]

Document for:	Discussion and Decision

Introduction

In RAN#94-e plenary meeting, a new SID on AI/ML for air-interface was approved for Rel-18 [1], where the use cases include CSI feedback enhancement, and evaluations would be performed for this use case:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. …… For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. …… Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. - Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This document summarizes the key issues discussed under agenda item 9.2.2.1 based on the views in [2]-[29], and aims to discuss a set of issues for the evaluation of the AI/ML-based CSI feedback enhancements in RAN1#110.

1st round email discussions

Before going to the discussions in the following sections, companies are invited to type in the contact person information into the following table.

Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point of contact | Email address |
| --- | --- | --- |
| Samsung | Ameha | amehat.abebe@samsung.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| Lenovo | Vahid Pourahmadi Jianfeng Wang | vpourahmadi@lenovo.com wangjf20@lenovo.com |
| ZTE | Lun Li | li.lun1@zte.com.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Intel | Victor Sergeev | victor.sergeev@intel.com |
| Ericsson | Mattias Frenne | Mattias.frenne@ericsson.com |
| Futurewei | Baoling Sheen | bsheen@futurewei.com |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |
| AT&T | Isfar Tariq Salam Akoum | isfar.tariq@att.com salam.akoum@att.com |
| CMCC | Yuhua Cao | caoyuhua@chinamobile.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |

Generic issues on evaluation methodology

Summary of views from companies

### 2.1-1: Remaining issues of the EVM table

Traffic model

- FTP model 1: Huawei, Hisilicon, Samsung, vivo, Nokia, ZTE, OPPO, Intel, Panasonic

  - Huawei, Hisilicon: For CSI feedback enhancement evaluation, use FTP model 1 with packet size 0.5 Mbytes as baseline. Results of full buffer traffic are not precluded.

  - Samsung: For the evaluation of the AI/ML based CSI feedback enhancement consider the following traffic models, FTP model 1 as baseline, Full buffer as optional

  - Nokia: Adopt the FTP 1 traffic model for system level simulation evaluation of throughput performance of AI/ML-based CSI feedback enhancement

  - ZTE: Full buffer traffic model can be optionally taken for calibration purpose and SI conclusions should be based on evaluation results of FTP traffic models

  - OPPO: For traffic model, use FTP model 1 as the baseline and companies are encouraged to provide full buffer results for SLS throughput comparison

  - Intel: FTP model 1 with packet size 0.5 Mbytes is considered as a baseline. Full buffer traffic model is not precluded for evaluations

  - Panasonic: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, for ‘traffic model’, full buffer model is not precluded at least for the purpose of calibration and/or comparing results, while the conclusions for the use cases in the SI should be drawn based on FTP model

- Full buffer

  - Full buffer is not precluded/optional: Huawei, Hisilicon, Samsung, OPPO, Xiaomi, ZTE, Intel

  - Full buffer as an option: Qualcomm

    - Qualcomm: Full-buffer traffic model should also be considered as one option for the evaluation of AI/ML-based CSI feedback enhancement

- FTP model 3: ZTE

Channel estimation

Issue 1: If ideal DL channel estimation is considered (which is optional), whether ideal channel estimation is applied for dataset construction, or performance evaluation/inference

- Option 1: Both dataset construction and performance evaluation/inference: Huawei, Hisilicon, Xiaomi, NTT DOCOMO, Intel, Panasonic

  - Huawei, Hisilicon: For the evaluation of the AI/ML-based CSI feedback enhancement, ideal channel estimation is used for both dataset construction and inference if ideal DL channel estimation is optionally considered

  - Xiaomi: ideal channel estimation can be applied for dataset construction and taken into the EVM for the purpose of calibration and/or comparing intermediate results

  - NTT DOCOMO: Consider the following dataset for the evaluation: Alt.1 if ideal channel estimation is applied for the input calculation in model inference

  - Intel: Ideal channel estimation is assumed for dataset construction for training

  - Panasonic: In the evaluation of the AI/ML based CSI feedback enhancement, for ‘channel estimation’, if ideal DL channel estimation is optionally considered, ideal channel estimation is used for both dataset construction and inference

- Option 2: Ideal channel for dataset construction, realistic channel for demodulation/inference (under realistic CE): Qualcomm, OPPO, ETRI

  - Qualcomm: For the evaluation of AI/ML-based CSI feedback enhancement, use ideal channel to construct datasets for training as a starting point

  - OPPO: For channel estimation, use realistic channel estimation as the baseline for eventual SLS performance evaluation in inference stage and ideal channel estimation for dataset construction in AI/ML model training stage

  - ETRI: For channel estimation, use realistic channel estimation for dataset construction for AI/ML model training for final evaluation

- Option 3: Whether the ideal or realistic channel estimation is applied for dataset construction is up to companies: Samsung,

- Other

  - Xiaomi: Ideal channel estimation is optional adopted besides realistic channel estimation for performance evaluation

  - China Telecom: For the evaluation of the AI/ML based CSI feedback enhancement, take ideal channel estimation as baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

  - China Telecom: Realistic channel estimation may be used as baseline for system-level performance evaluation (e.g., SE, UPT, etc.).

Issue 2: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

- Option 1: Use the ideal channel as target CSI for intermediate KPI calculation, Huawei, Hisilicon, Qualcomm, ZTE, Xiaomi, Fujitsu, China Telecom, Panasonic

  - Huawei, Hisilicon: For the evaluation of the AI/ML-based CSI feedback enhancement, ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

  - Qualcomm: For the evaluation of AI/ML-based CSI feedback enhancement, when computing intermediate KPIs, use the ideal channel to determine the target CSI

  - ZTE: Intermediate results should be calculated based on ideal channel and AI/ML model output at least for calibration purpose

  - Xiaomi: if the realistic channel estimation is used, ideal channel is still used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

  - Fujitsu: The vectors from the ideal channel matrix are suggested to be used in the formula of the squared generalized cosine similarity (SGCS)

  - China Telecom: Ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

  - Panasonic: In the evaluation of the AI/ML based CSI feedback enhancement, for ‘channel estimation’, if realistic DL channel estimation is considered, use the target CSI from realistic channel estimation and use output CSI from the realistic channel estimation

- Option 2: Realistic channel estimation is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation Samsung, NTT DOCOMO

  - NTT DOCOMO: Consider the following dataset for the evaluation: Alt.5 if realistic channel estimation is applied for the input calculation in model inference

  - NTT DOCOMO: Take target CSI as CSI which AI model is trained to output (Label in training dataset). Ordering issue can be avoided in SGCS calculation

Other views

- Ideal channel is used as a target CSI for intermediate results calculation for AI/ML output CSI from ideal channel estimation Samsung

Traffic load (RU)

- Samsung: 50/70 % for SU/MU-MIMO with rank adaptation, 20% for SU-MIMO with rank adaptation, companies are encouraged to report the MU-MIMO utilization.

Simulation bandwidth

- Samsung: Simulation bandwidth: 10MHz for 15KHz SCs as a baseline.

MIMO scheme

- Samsung: SU/MU-MIMO with rank adaptation as a baseline

- OPPO: For SLS evaluation and calibration: Evaluate and calibrate rank 1 and rank 2 with MU-MIMO in the first stage. Evaluate and calibrate rank 3 and rank 4 with SU-MIMO in the second stage

Other views

- OPPO: For intermediate KPI and SLS throughput evaluation, focus on R16 CSI enhancement as the baseline in current stage

  - FFS: R17 CSI enhancement considering downlink/uplink channel reciprocity as the baseline

### 2.1-2: Metrics

Two remaining issues from the last meeting include the preference of the Cosine Similarity and Rank>1 case for calculating Cosine Similarity. In addition, some more views on intermediate KPIs, eventual KPIs, UE capability related KPIs are also raised by companies for this meeting.

SGCS

- Granularity of SGCS

  - Ericsson: Intermediate KPI performance curves differ among companies for the Type-II baseline, which should not be the case

  - Ericsson: Use 1 RB as frequency granularity when ground truth is computed in the intermediate KPI

- Rank>1 case

  - Method 1: Average over all layers: Nokia, MediaTek, OPPO, CAICT, ETRI

    - Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

SGCS=E1N×Ki=1Nj=1K||wijHwij ||||wij||  ||wij||2

    - Nokia: all of the metrics have a similar relationship to throughput. Since the overall mean SGCS (Method 1) combines all layers into a single metric, we prefer this method compared to the per layer SGCS (Method 3).

    - MediaTek: For training a layer-common models for rank>1 channel, the training loss should be simply averaged across the layers.

    - OPPO: For intermediate KPI, use SGCS as the evaluation metric for calibration. Average all ranks for rank > 1

    - CAICT: For SGCS calculation for rank>1, method 1(average over all layers) is proposed as baseline

    - ETRI: For intermediate KPI for rank>1, use averaged SGCS over all layers as baseline and each layer’s SGCS as optional

  - Method 2: Weighted average over all layers: Samsung, CATT, Fujitsu

    - Samsung: For the evaluation of the AI/ML based CSI feedback enhancement, calculate SGCS for rank>1 as weighted average over all ranks

SGCS=E1N×Ki=1NjK(λij)2j=1K(λij)2||wijHwij ||||wij||  ||wij||2

where λij is an eigenvalue of the channel covariance matrix corresponding to wij.

Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples.

    - Fujitsu: The KPI for the intermediate evaluation on AI/ML model performance for CSI feedback enhancement when the rank of the channel matrix is larger than 1 is the normalized weighted SGCS

SGCS=E1Ni=1Nj=1Kλij2j=1Kλij2wijHwij2 wij2wij2,

where λij is the j-th eigenvalue of the squared matrix GGH at resource unit i. The vector wij is the eigenvector associated with the eigenvalue λij, i=1,2,…,N, and j=1,2,…,K. The matrix G is the channel matrix, and the superscript H is the Hermitian operator. The vector wi is the reconstruction of the compressed wi.

    - CATT: For the evaluation of AI/ML based CSI feedback enhancement, as the intermediate KPI for rank>1 cases, either or both of the following two methods are selected: Method 2: Weighted average over all layers as follows, where λij is an eigenvalue of the channel covariance matrix corresponding to wij, wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples:

SGCS=E1Ni=1NjKλijj=1Kλij||wijHwij ||||wij||  ||wij||2

  - Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer): Huawei, Hisilicon, CATT, Apple, ZTE, CMCC, Intel

    - Huawei, Hisilicon: Method 3 can provide the insights for per layer, e.g., the accuracy of AI/ML based CSI compression has different effects on different layers.

    - Apple: since the cosine similarity is to provide insight into the AI performance per layer, reporting cosine similarity per layer would provide the complete picture

    - ZTE: Method 3 can be used to analyze intermediate performance gains from different layers, which actually influence the eventual system throughput. Method 3 can be adopted as a baseline for SCGS calculation when rank>1

    - CATT: For the evaluation of AI/ML based CSI feedback enhancement, as the intermediate KPI for rank>1 cases, either or both of the following two methods are selected: Method 3: SGCS is separately calculated for each layer.

    - Intel: SGCS is supported as intermediate metrics for rank 1. For rank > 1 throughput results can be used or per-layer SGCS can be provided

  - Other views

    - Panasonic: SGCS calculation for rank > 1 should depend on how training is carried out and whether rank/layer is from AI/ML or from conventional identification

Other intermediate KPIs

- Alt.1: Realized relative SNR (RRSNR)

  - Note: Nsb is the number of subbands, L is the number of layers, Hnsb~[NRx×NTx] is the complex channel matrix for subband nsb, wnsb,l is the precoding vector, σnsb,l is the l-th largest singular value

RRSNRL≔ E1L Nsb l=1Lnsb=1Nsbσnsb,l2l=1Lnsb=1NsbHnsbwnsb,l2wnsb,l2

- Alt.2: Equivalent MSE:

Equivalent MSE dB=10*log102-2*GCS

- Alt.3: Chordal distance (for rank >= 1): Qualcomm

Chordal Distance = 1Ni=1NK-ViHVi F2

  - Note: ||.||F represents the Frobenius norm. When rank is 1, the chordal distance formula reduces to 1-SGCS.

- Alt.4: Numerical spectral efficiency gap (for rank >= 1): Qualcomm

Numerical SE Gap =  1Ni=1NSEVi, Hi-SE(Vi, Hi)

  - Note: SE(.) denotes the numerical spectral efficiency function which may be defined as follows, where γ denotes the SNR:

SEV,H=log2detI+γHVVHHH

  - Qualcomm: Chordal distance and numerical spectral efficiency gap metrics are not affected by a mismatch in the order of the eigenvectors between the target CSI and output CSI

- Alt.4A: Numerical spectral efficiency ratio/relative achievable rate (RAR) (for rank >= 1): Qualcomm, Ericsson, Lenovo

  - Qualcomm

Numerical SE Ratio =  i=1NSE(Vi, Hi)i=1NSEVi, Hi

    - Note: SE(.) denotes the numerical spectral efficiency function which may be defined as follows:

SEV,H=log2detI+γHVVHHH

where γ denotes the SNR. Numerical SE ratio has the benefit that it is normalized.

  - Qualcomm: Adopt at least one of the numerical spectral efficiency ratio, numerical spectral efficiency gap, and the chordal distance as an intermediate KPI for rank >= 1.

  - Ericsson

RARγ,L=EH1Ff=1FlogdetI+γPfHHfHHfPfEH1Ff=1FlogdetI+γPfHHfHHfPf

    - Hf~[NRx×NTx] is the complex channel matrix for RB f

    - F is the total number of RBs,

    - Pf~[NTx×L] is the reported precoding matrix for RB f,

    - Pf~[NTx×L] is the optimal (SVD-based) precoding matrix for RB f,

    - L is the number of MIMO layers the KPI is evaluated for,

    - γ is the SNR-value;

γ=100EH1Ff=1FσmaxHf

    - and the outer expectation EH is taken over a distribution of MIMO channels.

  - Ericsson: As an intermediate KPI, adopt the Relative Achievable Rate (RAR) as defined above, evaluated with γ=100EH1Ff=1FσmaxHf

  - Lenovo: As one intermediate KPI, to evaluate the efficiency of the estimated precoders, we suggest to use relative achievable rate (RAR) defined as:

RARγ=EHi1Bj=1Bk=1Klog21+γHijTvijk2 EHi1Bj=1Bk=1Klog21+γHijTvijk2

where γ is the SNR and channels are normalized, i.e.,  E|Hi|=1 is assumed to be normalized. For simplification, RARγ can report at γ=1, i.e.,

RAR1=EHi1Bj=1Bk=1Klog21+HijTvijk2 EHi1Bj=1Bk=1Klog21+HijTvijk2

Note also, if the metric is intended for comparison between different implementations which use the same “H”, then the denominator can be removed (as it will be the same between different models).

- Alt.5: Precoder error (e.g. cosine similarity between ideal and reconstructed precoders):

- Alt.6: Normalized Expected Directional Gain (NEDG): Lenovo

- NEDG=EHi1Bj=1Bk=1KHijTvijk2 EHi1Bj=1Bk=1KHijTvijk2

- Alt.7: GCS in the log scale: Mavenir

  - GCS in the log scale should be used as the intermediate KPI during the first evaluation stage (i.e., AI/ML model evaluation) since it reflects the eventual KPI better compared to other intermediate KPI candidates.

- Other intermediate results are optional:

  - Huawei, HiSilicon: For the evaluation of the AI/ML-based CSI feedback enhancement, other intermediate KPIs other than SGCS and NMSE can be optionally considered and reported by proponent companies

  - Samsung: RRSNR as intermediate KPI may be advantageous on providing better emulation of the MU-MIMO DL throughput performance than GCS in some receiver assumptions. If the UE performs inter-layer interference nulling, this advantage is lost.  This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG

  - Samsung: In MU-MIMO scenario, when the gNB do not directly apply the reconstructed precoder, RRSNR does not give a practical advantage in terms of emulating the DL MU-MIMO throughput performance than SGCS. This observation holds to other proposed intermediate KPIs such as numerical spectral efficiency gap and NEDG

  - Samsung: For the evaluation of the AI/ML based CSI feedback enhancement, deprioritize discussion on additional intermediate KPIs

Eventual KPIs

- Throughput per complexity unit: MediaTek

  - MediaTek: To appreciate low-complex, yet high-performing, AI/ML model designs, a KPI measuring throughput per complexity unit would be beneficial.

    - We suggest defining new KPIs incorporating both throughput and complexity to appreciate AI/ML models with a balanced design, such as Throughput per FLOPs, Throughput per MACs, or Throughput per number of trainable parameters

- CSI feedback overhead

  - NVIDIA: The CSI feedback overhead can be measured by number of feedback bits and/or compression ratio

Capability/complexity related KPIs

- Processing complexity

  - Xiaomi: Number of model parameters is selected between AI/ML model size and number of AI/ML parameters as metric of model complexity

  - NVIDIA: AI/ML model complexity and computational complexity should not be regarded as a roadblock to the adoption of AI/ML based algorithms for CSI feedback enhancements

    - Increasing hardware performance can support successively more complex AI/ML models. For example, GPU inference performance has improved by 317x in 8 years (2012-2020), more than doubling each year.

Price for model training/updating

- Number of training samples

  - MediaTek: Number of training samples to reach a certain performance KPI can itself be used as a KPI to quantify trainability of AI/ML models

Benchmark for evaluation results comparison

- Type I CB is not necessary to be taken as a baseline for performance evaluation ZTE, LG

  - ZTE: Type I CB is not necessary to be taken as a baseline for performance evaluation

  - LG: Type I CSI is not preferred as a baseline for the AI/ML based CSI reporting

Other views on metrics

- NVIDIA: The CSI feedback accuracy can be measured by intermediate KPIs such as GCS/SGCS and/or NMSE

- NVIDIA: Evaluate system throughput (e.g., average and 5-percentile throughput) performance to assess the system performance gains from the reduced CSI feedback overhead and/or improved CSI feedback accuracy.

### 2.1-3: Generalization

Some specific methods to assess the generalization performance in evaluations are brought up by companies.

- Issue A: Methodology for verifying generalization

Generalization verification from fine-tuning perspective (Case 2A)

- Huawei, HiSilicon: Case 2A is different from Case 3 on training dataset construction and testing dataset construction

- Huawei, HiSilicon: Case 2A should also be considered for the methodology for generalization of AI/ML-based CSI feedback:

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

- Samsung: For generalization performance evaluation of AI-based CSI feedback enhancement, Case2A can be viewed as part of Case 1-3 depending on factors such as:  the relative size of fine-tuning dataset, learning rate, or the order of dataset samples for training or fine-tuning

- Samsung: For generalization performance evaluation of AI-based CSI feedback enhancement, focus on the agreed three cases, i.e., Case 1, Case 2 and Case 3, to simplify the evaluation. The case described as Case 2A and other possible sub-cases can be studied later with lower priority

- ZTE: Case 2A can be listed as an independent case for verifying model generalization

  - Case 2A also differs from Case 3 that Case 2A only focuses on the performance in the second Scenario/Configuration. However,  Case 3 takes the performance of both Scenarios/Configurations into account

- LG: For AI/ML model generalization, Case 2A and Case 3 can be merged

Other views on generalization verification:

- Nokia: Include generalization tests from scenarios or configurations outside the training data set

- ZTE: Case 3 (AI model trained with mixed dataset) can be utilized to study AI generalization as a starting point.

- MediaTek: For generalization study, the mixed datasets should be subject of further discussions to determine the exact contribution of each sub-dataset into the mixed one according to real-world settings.

- MediaTek: Considering the potential number of mixing dimensions, offer public datasets to facilitate study on generalization aspects of AI/ML models

- CMCC: The generalization capability of AI model across different configurations/ scenarios could be evaluated

- CMCC: The solution to improve the generalization capability of AI model across different configurations/ scenarios could be further studied

- Fujitsu: The KPIs for the generalization/scalability of an AI/ML model should be further studied. In particular, both the performance-related KPI and the complexity-related KPI are suggested to be used.

- Fujitsu: It is suggested that the comparison of the model complexity of baseline and generalization schemes can be taken as a KPI. As an example, the ratio of the model complexity of the baseline approach over that of the generalization scheme.

- FUTUREWEI: In AI/ML model generalization across different scenarios/configurations for spatial-frequency domain CSI compression, further study the applicable generalization mechanism(s) that can be applied to different scenario/configuration combinations.

Findings on generalization Case 1/2/3

  - InterDigital: The AI/ML models generalize well under Case I and they considerably outperform CSI Type II at lower overhead when tested in the same Scenario/Configuration used in training

  - InterDigital: The AI/ML models generalize well under Case II, and they outperform CSI Type II at lower overhead when tested in a different Scenario/Configuration as used in training

  - InterDigital: The trained AI/ML Model ABC on mixed datasets generalizes well when tested under each individual dataset. The generalized model provides substantial savings in memory and still outperforms CSI Type II at lower overhead

  - InterDigital: The AI/ML Model ABC and Model C generalize well when tested on UMa samples only (dataset A)

  - InterDigital: The SGCS seems to be a good and computationally cheap intermediate KPI that can uncover important generalization aspects of AE models.

  - Fraunhofer: Global CsiNet still outperforms Type II CSI for medium-to-high total overhead bits, however, it has worse performance than CsiNet

Then the second issue is what specific inference configuration/scenario should be assumed for verification.

- Issue B: Scenarios (dimensions of the input/output of the AI/ML model are unchanged) to verify generalization

A number of companies evaluated the generalization under varying scenarios.

- Various deployment scenarios (UMa/UMi/InH): Huawei, HiSilicon, vivo, Samsung, Xiaomi, ZTE, OPPO, CATT, CMCC, NTT DOCOMO, Intel, FUTUREWEI, InterDigital, NVIDIA, Spreadtrum

Findings on generalization verification

  - Huawei, HiSilicon: For an AI/ML model trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa/UMi) for inference (i.e., generalization Case 2), its performance may be degraded compared to inference under Scenario#A (i.e., generalization Case 1), but mixing the dataset over the three scenarios for AI/ML model training (i.e., generalization Case 3) is helpful to improve the generalization

  - Huawei, HiSilicon: For an AI/ML model trained under Scenario#A (e.g., InH) dataset but applied to Scenario#B (e.g., UMa) for inference, its performance can be improved by applying fine-tuning (i.e., generalization Case 2A) using a relatively small dataset from Scenario#B.

  - vivo: For a generic model (non-optimized for a specific area/cell), AI model performance does not degrade when generalized from UMi to UMa

  - vivo: For a generic model (non-optimized for a specific area/cell) AI model can generalize across different scenarios with a mixed dataset. A reasonable mixing ratio can provide better performance for each scenario

  - Samsung: GCS degradation in cross-deployment-scenario, i.e., Casse 2, can be alleviated by mixing various datasets

  - Samsung: For the generalization, mixing various configuration(s)/scenario(s) is a viable option to alleviate the degradation of the generalization performance

  - ZTE: Case 3(AI model trained with mixed deployment scenarios) shows good generalization performance for various deployment scenarios.

  - Xiaomi: AI/ML model shows good generalization performance in different scenarios

  - OPPO: For different scenarios, the SGCS degradation is slight (about 1%~3%) when training set and testing set are mismatching

  - OPPO: For different scenarios, training on mixing dataset can improve the generalization performance of AI/ML model

  - CATT: For the generalization of AI/ML based CSI feedback, the following is observed

    - For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small

    - For applying AI/ML model in InH, the AI/ML model trained based on a dataset collected in UMa/UMi slightly outperforms the AI/ML model trained based on a dataset collected in InH

    - For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH. Training the AI/ML model with mixed data of UMa and InH can alleviate the performance gap

  - NTT DOCOMO: Generalization performance of AI/ML model under the tested scenarios/configurations (various deployment scenarios and various carrier frequencies) is good

  - Intel: If dataset with both UMa and UMi channel models is used for training (Case 3) then performance loss is marginal comparing to training and testing on aligned dataset (Case 1)

  - Intel: Autoencoder trained on a dataset with InH channels significantly outperforms an autoencoder trained on a dataset with UMa channels for inference on InH channels

  - CMCC: The AI model trained with mixed dataset across various scenarios might have some performance loss comparing with dedicated model. (Note: A mixed dataset of 50K samples from CDL-C-30 and 50K samples from CDL-C-300 is generated as the training dataset.)

  - FUTUREWEI: For AI/ML model generalization across different deployment scenarios in spatial-frequency domain CSI compression sub use case, our experiments showed

    - The AI/ML model trained using dataset generated for UMa 2GHz CANNOT directly generalize to UMi 2GHz, and the AI/ML model trained using dataset generated for UMa 4GHz CANNOT generalize to UMi 4GHz.

    - The AI/ML model trained using dataset generated for UMi 2GHz CANNOT directly generalize to UMa 2GHz, and the AI/ML model trained using dataset generated for UMi 4GHz CANNOT generalize to UMa 4GHz.

    - The performance results from mixed datasets show significant improvement over the naïve transfer approach between Umi and Uma scenarios in both 2GHz and 4GHz carrier frequencies.

  - InterDigital: Both UMa and UMi channel samples distributions seem to be close to each other as one ML model trained on mixed samples yield the same result as having two ML models trained separately on each channel model

  - NVIDIA: Evaluation results show that it is much easier for the autoencoders to compress CSI in CDL-C than in dense urban scenario, as the link level channel model CDL-C has fixed angle values and represents only a single channel realization while the system level channel in the dense urban scenario is much more sophisticated

  - NVIDIA: Evaluation results show the autoencoders trained in the sophisticated dense urban scenario perform well in CDL-C, illustrating the generalization capability of the AI/ML models

  - Spreadtrum: The AI based CSI compression and recovery trained under different scenarios can also achieve better SGCS performance than Rel-16 eType II codebook

Preliminary trends observed by more than one company

  - AI model generalizes well from UMa to UMi and vice versa. It is relatively difficult to generalize from UMa or UMi to InH. Huawei, HiSilicon, Samsung, ZTE, CATT, InterDigital

    - Samsung: AI model generalizes well from UMa to UMi and vice versa. It is relatively difficult to generalize from UMa or UMi to InH.

    - ZTE: The AI model trained with the scenario of UMa/UMi shows good generalization performance for InH dataset, while the AI model trained with the scenario of InH shows generalization degradation for UMa/UMi dataset.

    - CATT: For applying AI/ML model in UMa/UMi, the performance difference between training the AI/ML model with dataset of UMa and training the AI/ML model with dataset of UMi is small. For applying AI/ML model in UMa, compared to the AI/ML model trained in UMa, significantly performance loss can be seen by the AI/ML model trained in InH.

    - InterDigital: Both UMa and UMi channel samples distributions seem to be close to each other as one ML model trained on mixed samples yield the same result as having two ML models trained separately on each channel model

- Various outdoor/indoor UE distributions for UMa/UMi: Huawei, HiSilicon, vivo, Samsung, Qualcomm, Nokia, Intel

Findings on generalization verification

  - Huawei, HiSilicon: For generalization over indoor/outdoor UE distribution ratios,

    - AI/ML model trained by any indoor/outdoor UE distribution ratio shows similar performance on the outdoor testing dataset.

    - With the decrease of the indoor channel ratio for the training dataset, the trained AI/ML model brings decreased performance on the indoor testing dataset.

  - vivo: For a generic model (non-optimized for a specific area/cell), AI model trained in complicated channel environment (more indoor users) has good generalization ability of simple channel environment (more outdoor users)

  - Samsung: For AI/ML based CSI compression, the AI model generalizes well from indoor channel to outdoor channel

  - Qualcomm: Training on a dataset constructed by mixing the datasets of multiple scenarios enables the same ML model to perform well during inference in each of the scenarios

  - Intel: For an autoencoder trained on UMa dataset with Indoor/Outdoor, LoS/NLoS UEs, similar performance is observed for LoS and NLoS channels while small performance gain (0.05 GCS difference) is observed for Outdoor UEs comparing to Indoor UEs

Preliminary trends observed by more than one company

  - For generalization Case 2, AI model generalizes well for training with indoor and inference with outdoor, while a poor generalization performance is observed for the other way around. Huawei, HiSilicon, Samsung, Qualcomm

  - Qualcomm: Study mechanisms to monitor the performance of the AI/ML model to detect whether the data observed during inference is outside the distribution of the dataset used to train the model

- Various carrier frequency: vivo, NTT DOCOMO, FUTUREWEI, InterDigital

Findings on generalization verification

  - vivo: AI model performance does not degrade when a generic model (non-optimized for a specific area/cell) trained for a frequency is applied to another frequency

  - NTT DOCOMO: Generalization performance of AI/ML model under the tested scenarios/configurations (various deployment scenarios and various carrier frequencies) is good

  - FUTUREWEI: For AI/ML model generalization across different frequencies in spatial-frequency domain CSI compression sub use case, our experiments showed

    - The AI/ML model trained using dataset generated for UMa 2GHz CAN generalize to UMa 4GHz, and the AI/ML model trained using dataset generated for UMa 4GHz CAN generalize to UMa 2GHz without significant performance difference.

    - The AI/ML model trained using dataset generated for UMi 2GHz CAN generalize to UMi 4GHz, and the AI/ML model trained using dataset generated for UMi 4GHz CAN generalize to UMi 2GHz without significant performance difference.

    - The performance results from mixed datasets do not show significant difference compared to naïve transfers for cross frequency scenarios between 2GHz and 4GHz in both UMi and UMa scenarios.

  - FUTUREWEI: When considering spatial-frequency domain CSI compression, in some cases, AI/ML model trained using dataset from one scenario can generalize to another scenario (e.g., at least between 2GHz and 4GHz), while in some other cases, AI/ML model trained using dataset from one scenario cannot generalize to another scenario (e.g., at least between UMa and UMi).

Preliminary trends observed by more than one company

  - AI model trained under one carrier frequency generalizes well over other carrier frequencies (e.g., from the set of 2GHz, 3.5GHz, 4GHz, 5.5GHz)

- Various antenna spacing: vivo

Findings on generalization verification

  - vivo: For a generic model (non-optimized for a specific area/cell), there is obvious performance loss for antenna spacing mismatch of training data

- Various antenna virtualization (TxRU mapping): Huawei, HiSilicon, vivo,

Findings on generalization verification

  - Huawei, HiSilicon: For generalization over various TxRU mapping methods including (8,8,2,1,1,2,8) and (2,8,2,1,1,2,8), the AI/ML model trained by the mixed datasets show moderate performance on each of the TxRU mapping methods.

  - vivo: For a generic model (non-optimized for a specific area/cell), SGCS performance of AI model may degrade slightly from 128 antennas with virtualization to 32 antennas without virtualization. While the SE performance may degrade heavily due to the less antennas

  - vivo: For a generic model (non-optimized for a specific area/cell), in the case of 32 antennas,  AI model trained with 32 antennas may have similar SE performance compared with AI model trained with 128 antennas and settled in the case of 128 antennas, which is needed to be further studied

- Various LOS/NLOS: Samsung

  - Samsung: For AI/ML based CSI compression, the AI model generalizes well from LOS channel to NLOS channel and vice versa

- Effect of pre-processing vivo

  - Issue: Performance loss for antenna spacing mismatch of training data

  - Solution: Pre-processing to achieve a small AI models with spatial domain and frequency domain compression

  - Findings: For a generic model (non-optimized for a specific area/cell), the influence of mismatch of training data may be reduced by pre-processing

- Issue C: Configurations (dimensions of the input/output of the AI/ML model are different, i.e., scalability needed) to verify generalization

A number of companies evaluated the generalization under varying configurations.

- General views

  - Fujitsu: The baseline for the evaluation of the generalization/scalability of an AI/ML model over various configurations should be further studied. The baseline is suggested to be the performance of the two-sided AI/ML model composed by an encoder and a decoder only, without pre-processing/post-processing. Other potential baseline schemes can be further studied.

  - Fujitsu: For the generalization/scalability of an AI/ML model over various configurations, it should be clarified how an AI/ML model is defined.

    - Alt. 1. Pre-/post-processing are precluded.

    - Alt. 2. Pre-/post-processing are included, i.e., a unique AI/ML model in the sense that the backbone model is unique.

    - Alt. 3. One encoder vs. multiple decoders, or vice versa.

  - Fujitsu: The pre-/post-processing are suggested to be counted as part of an AI/ML model for the generalization/scalability of an AI/ML model over various configurations.

- Various bandwidths/frequency granularities: vivo, Qualcomm, OPPO, Xiaomi, ZTE, CMCC, InterDigital,

  - Issue: Different frequency granularity or different ports number can cause different input dimension of AI model

  - Solution 1: Pre-processing to angle-delay domain. E.g., precoders of each subband can be transformed into angle-delay domain with selected beam and delay. With the projection from space-frequency domain to angle-delay domain, the dimension of input data is specific to the number of selected beams and delays vivo

  - Solution 2: Training with mixed variable subband configurations. E.g., training the AI model using random subband patterns in addition to the full subband case. Qualcomm

  - Solution 3: Adapt the subband size according to BW size (to keep the input dimension unchanged). E.g., 4RBs per sub-band for 10MHz and 8RBs per sub-band for 20MHz. ZTE,

  - Solution 4: Zero padding. Xiaomi, OPPO, CMCC

    - OPPO: the AI/ML model trained on dataset from Configuration#A with 13 sub-band can be inferenced/tested on dataset from Configuration#B with 8 sub-band and zero-padding on the sub-band domain

    - CMCC: we train AI model with the samples composed of the eigenvectors of 12 subbands and apply this AI model to test the samples composed of the eigenvectors of 8 subbands. To achieve better generalization performance, we will pre-process the samples composed of the eigenvectors of 8 subbands before inputting these samples into generation part, i.e., we will perform padding zero at the end of each sample

  - Solution 5: Restoration. ETRI

    - We use Transformer network architecture for the restoration NN in the Decoder. Transformer network is one of Recurrent Neural Network (RNN) and RNNs can process variable length of data sequences. The restoration NN in Decoder gets (reconstructed) eigenvectors of subbands as an input sequence and puts restored eigenvectors of subbands as an output sequence.

Findings on generalization verification

  - vivo: Pre-processing with delay domain and spatial domain compression can solve different input dimensions caused by various frequency granularities and port numbers

  - vivo: Frequency domain can be compressed more than spatial domain

  - vivo: Pre-processing with delay domain and spatial domain compression can solve different input dimensions caused by various frequency granularities and port numbers

  - Qualcomm: Training with mixed variable subband configurations achieve robust performance across all possible subband configurations including arbitrary number of subbands and arbitrary subband patterns

  - Qualcomm: Training with mixed variable subband configurations outperforms specific training with specific subband configuration

  - ZTE: AI/ML approaches can achieve good generalization performance for the case that the training/validation dataset and testing dataset are generated with different bandwidth configurations

  - Xiaomi: AI/ML model shows good generalization performance in different sizes of subband and subband number

  - ETRI: For operation of AI/ML model over various bandwidths (or subband sizes), it is required that the AI/ML model to support variable sizes of input and output

  - ETRI: The PCA based AI/ML for CSI compression can be designed to support various bandwitdth (or subband sizes).

  - ETRI: In evaluation of AI/ML for CSI compression over various bandwidths, both PCA based AI models trained using datasets of smaller bandwidth (Case 2) and mixed datasets (Case 3) achieve almost same performance of the model trained using the dataset of target configuration (Case 1) in channel with low delay spread (30ns).

  - ETRI: In evaluation of AI/ML for CSI compression over various bandwidths, a PCA based AI models trained using mixed datasets (Case 3) achieve almost same performance of the model trained using the dataset of target configuration (Case 1) in channel with high delay spread (300ns).

- Various CSI feedback payloads: Huawei, HiSilicon, vivo, Qualcomm, Xiaomi, CATT, OPPO, CMCC, Fujitsu, InterDigital

  - Issue: Different payload can cause different output dimensions of AI model.

  - Solution 1: Payload truncation.

    - vivo: the output of the encoder is cut out from the beginning to the specific payload length

    - Xiaomi: different feedback payloads are obtained by cutting off the tail of the maximum 240 bit. The loss function is the average results of different decoder parts

    - Xiaomi: RAN1 study pre-processing mechanisms for the input of decoder to  improve the AI model generalization performance on various feedback payloads

    - OPPO: When the AI/ML model trained on Configuration#A is adopted on Configuration#B, the first LB bits can be reserved, and the latter LA-LB bits can be truncated during the interface feedback. Then, the truncated  LA-LB bits can be regarded as default 0 or 1 for the decoder input.

    - CMCC: The dimensions of the output of generation part is designed based on the maximum feedback bits, and before outputting from the generation part, some extra bits will be dropped

      - Case 1: one joint encoder and 2 separate decoders of which the number of feedback bits are 32 and 48 bits.

      - Case 2: one joint encoder and 3 separate decoders of which the number of feedback bits are 32, 48 and 120 bits.

    - Fujitsu: Specifically, the unique encoder and three decoders form three pairs of two-sided AI/ML models. The input of the three decoders are the truncations of the output of the encoder. The loss function of the entire training is the summation of the loss functions of the three pairs of two-sided AI/ML models, which is used for backpropagation.

  - Solution 2: Different payload configurations are trained at the same time Qualcomm

    - Option 1-two payload configurations are considered (i.e., encoder output dimension = 32 and 64) and are trained at the same time;

    - Option 2-training using contiguous patterns with random number of subbands, and the number of subbands are randomly generated;

    - Option 3-Similar to Option 2 but arbitrary subband pattern is considered in the training

  - Solution 3: Adaptation layer CATT, Fujitsu

    - CATT: fully-connected layers are used for both down-sampling (DS-x block) and up-sampling (US-x block). At the inference phase, only one branch is activated according to the configured payload.

    - Fujitsu: In the case of Fig. 4, the truncators are replaced by FCLs, and each of the FCL is trained together with its corresponding pair of two-sided AI/ML models. At the inference stage, a proper decoder is used based on the desired payload… It turns out that a universal decoder is possible if three additional FCLs are added between the FCLs and the decoder, as shown in Fig. 5.

Findings on generalization verification

  - vivo: Study payload generalization with payload truncation as baseline.

  - vivo: Payload truncation can be used to release payload generalization of AI models

  - Qualcomm: Smaller number of subbands can achieve comparable results to the larger number of subbands with half of reporting payload

  - Xiaomi: The generalized AI model does not work well on various CSI feedback payloads, especially for a small number of feedback payloads

  - OPPO: The scalability performance of AI/ML model for various antenna ports and CSI feedback payloads can be improved by trained on mixing datasets

  - CATT: Compared with a family of layer-common AI/ML models, the scalable AI/ML model (SCsiNet) can achieve a similar performance and can significantly reduce storage memory and model transferring overhead

  - CATT: Compared to Rel-16 Type II codebook based CSI feedback, 3%~11% throughput improvement under the same CSI feedback payload can be achieved by proposed scalable AI/ML model

  - CMCC: The unified AI model of one common encoder and multiple specific decoders performs well across different number of feedback bits.

  - Fujitsu: Generalization of payload can be achieved by a framework of a single encoder and multiple decoders with proper pre-processing/post-processing

  - Fujitsu: Various payload configurations can be supported in a unified two-sided AI/ML model with proper pre-processing/post-processing for CSI compression

  - Fujitsu: The performances of the generalization of payload are very similar in terms of the SGCS, no matter whether the structure of the AI/ML model is composed by a single encoder and a single decoder with proper pre-processing/post-processing, or a single encoder and multiple decoders with proper pre-processing/post-processing.

  - Fujitsu: The performance of the generalization of payload, in terms of the SGCS, are very similar to the baseline scheme, where the two-sided AI/ML model is composed by an encoder and a decoder only, without pre-processing/post-processing

  - InterDigital: Increasing the AE feedback size from 256 bits to 384 bits does not provide throughput performance benefits, while the corresponding SGCS results showed significant improvement when increasing the feedback size

- Various Tx/Rx antenna port layouts: Qualcomm, ZTE, Apple, CATT, OPPO,

  - Solution 1A: training a common AI model using mixed data set of 2x8, 4x4 and 2x4 antenna configurations. Qualcomm

  - Solution 1B: Case1: training on [8,2,2], testing on [8,2,2]; Case2: training on [8,2,2], testing on [4,4,2]; Case3: training on mixed dataset of [8,2,2] and [4,4,2], testing on [8,2,2]/[4,4,2]. Apple

  - Solution 2: Zero padding to 32 ports (and dataset mixing). ZTE, OPPO

    - OPPO: the AI/ML model trained on dataset from Configuration#A with 32 port can be inferenced/tested on dataset from Configuration#B with 16 port and zero-padding on the antenna port domain

  - Solution 3: Adaptation layer CATT

    - CATT: Fully connected layers are used for linear pre-transforming (LPT-x block) and linear transforming (LT-x block) for the purpose of unifying input/output dimensions and probability distribution of eigenvectors from different port numbers.

Findings on generalization verification

  - Qualcomm: Training with mixed antenna configuration achieves robust performance across all antenna configurations in the training

  - Qualcomm: Training with mixed antenna configuration outperforms specific training with specific antenna configuration

  - ZTE: Case 3(AI model trained with mixed configurations of antenna port numbers) shows good generalization performance for various antenna port numbers

  - ZTE: AI model trained with the configuration of 32 antenna ports can maintain performance for 16 antenna ports.

  - Apple: For generalization study case 2, when the autoencoder is trained in UMa with [8 2 2] antenna port layout, and test with [4 4 2] antenna port layout, large performance loss is observed

  - Apple: For generalization study case 3, when the autoencoder is trained in mixed dataset with [8 2 2] and [4 4 2] antenna port layout, and test with [4 4 2] antenna port layout, similar performance is observed as case 1

  - OPPO: The scalability performance of AI/ML model for various antenna ports and CSI feedback payloads can be improved by trained on mixing datasets

  - CATT: Compared with a family of layer-common AI/ML models, the scalable AI/ML model (SCsiNet) can achieve a similar performance and can significantly reduce storage memory and model transferring overhead

- Various ranks/layers: Huawei, HiSilicon, vivo, Samsung, Apple, MediaTek, Xiaomi

  - Issue: Different payload can cause different output dimension of AI model.

  - Solution: Layer common model. E.g., a unified AI/ML model is trained and applied for each layer to perform individual inference. Huawei, HiSilicon, vivo, Samsung, MediaTek Xiaomi

Elaboration

    - Option 4-A: A unified AI/ML model is trained with the dataset for layer 1 (Case 1/2) only; Samsung, Apple

    - Option 4-B: A unified AI/ML model is trained with the mixed dataset for layer 1 and layer 2 (Case 3) Samsung, Xiaomi

Findings on generalization verification

  - vivo: Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model.

  - vivo: For rank > 1 cases, the precoder of each layer can be compressed by a common AI model.

  - Samsung: For AI/ML based CSI compression, the following observations were made for generalization performance across layer 1 and layer 2,

    - AI model generalizes well from layer 1 to layer 2 thus a layer-common model can be used

    - Training an AI model with the mixed dataset consisting of layer 1 and layer 2 doesn’t help it rather degrades the SGCS performance as compared to training an AI model with a single dataset (layer 1 or layer 2).

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, re-usage of AI/ML models (case 2 of generalization) at layer level causes 1.29% GCS degradation

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, re-usage of AI/ML models (case 2 of generalization) at rank level causes 1.13% GCS degradation

  - MediaTek: It is feasible to generalize a layer-common AI/ML model across ranks with negligible performance loss

  - MediaTek: a unified AI/ML model (generalization case 3) not only does not degrade the feedback accuracy, but it also achieves 0.46% higher GCS accuracy compared to the dedicated AI/ML models for both layers

  - MediaTek: a unified AI/ML model (generalization case 3) shows 5.8% higher GCS accuracy for EVs of layer 0 compared to those belonging to layer 1. The similar trend has also been observed among the dedicated AI/ML models

  - Xiaomi: AI/ML model shows good generalization performance in various rank number

- Overall proposals on generalization

  - Qualcomm: For the evaluation of generalization of AI model to variable configurations, consider the following in data set generation:

    - For subband generalization, generate N>=1 random patterns (either contiguous or non-contiguous) for each data sample in the training set. The full subband pattern can be used in addition.

    - For antenna configuration generalization, mix data sample generated based on M antenna configuration with equal proportion.

    - Same configuration in the testing set and training set

  - OPPO: Focus on the same input and output CSI dimension with different configuration(s)/scenario(s) for generalization performance evaluation. Focus on different input and output CSI dimensions with different configuration(s) for scalability performance evaluation

    - E.g., different numbers of antenna ports, different number of sub-bands and different CSI feedback payloads

  - OPPO: Suggest to construct some typical datasets with aligned scenarios/configuration(s) to draw the conclusion on generalization performance.

    - Companies to report the details of utilized scenarios/configurations in the current stage

  - OPPO: Suggest to construct some typical datasets with aligned configuration(s) to draw the conclusion on scalability performance

    - Companies to report the details of utilized methods and configurations in the current stage

  - OPPO: For scalability evaluation, zero-padding, clipping and truncation can be considered for pre-processing and post-processing.

- Overall findings on generalization

  - vivo: For a generic model (non-optimized for a specific area/cell), AI models perform well in generalization of carrier frequency, channel scenario, indoor/outdoor ratio

  - vivo: For a generic model (non-optimized for a specific area/cell), AI models perform bad in antenna spacing and antenna virtualization, which can be further studied

As the other direction as opposed to achieving generalization, one company raised to adopt overfitting Per-cell (region) model with small size. In Moderator’s understanding, this is a different direction of studying AI/ML models as opposed to applying a generalized model with potentially large model size or requiring large training dataset. The principle of the per–area/cell/region model is to train a much smaller and overfitting model with smaller training dataset and apply it to a specific area, while on the other hand it may potentially not be applicable to other areas.

Per-area/cell/region model vivo

- Model size/structure

  - vivo: Under ray tracing channel model, even with simple (e.g., one layer fully connected structure) and small scale (e.g., ~200kB size) model, per-cell (region) model could achieve the near optimal gain

  - vivo: Under field dataset, performance of simple model structure, e.g., one hidden layer full-connected encoder, is good enough for typical per single cell or multiple cell operations.

- Overhead of model update

  - vivo: If the model structure of CSI generation part is simple(e.g., one-layer MLP), overhead of the model updating procedure will be very small (probably less than 100kB)

  - vivo: Further study the model update for per-cell (region) models

- Data collection

  - vivo: Further study the data collection for per-cell (region) models (Cell ID/sector ID or some other information that could represent the collecting area; there could be some concerns on user privacy as some of the assistance information during data collection is sensitive)

- Channel model

  - vivo: Support to add map-based hybrid channel model in 38.901 as one of the optional channel models in EVM table, where the map can be generated based on open data set or based on per-company proposed ones

- Findings on performance

  - vivo: Based on initial field test results, per-cell (region) models can provide more than 30%~50% improvement on SCGS of AI models.

  - vivo: Under ray tracing channel model, Per-cell (region) model could achieve near-optimal CSI compression performance at the matched area/environment, which obviously surpasses that of general models trained on data collected from a variety of situations

  - vivo: Under field dataset, Per-cell (region) model demonstrate very good performance compared to Rel-16 Type II codebook.

### 2.1-4: Dataset related issues

Channel model for dataset

- vivo: Study the performance and overhead of per-cell (region) model transfer in CSI compression

- vivo: Support to add map-based hybrid channel model in 38.901 as one of the optional channel models in EVM table, where the map can be generated based on open data set or based on per-company proposed ones

Ground-truth CSI labels Huawei, Hisilicon, Ericsson, Fujitsu

For evaluating the overhead of ground-truth CSI labels, some company mentioned that the method of compression/quantization for the ground-truth CSI labels will impact the overhead evaluation. In addition, if the AI/ML model is trained at NW, it may need to collect the ground-truth CSI labels fed back from UEs via air interface. Based on these observations, it is raised to study how to quantize the ground-truth CSI via air interface.

- Huawei, Hisilicon: Companies are encouraged to report the methods of how to feedback measured ground-truth CSI, e.g., compression/quantization method, etc., for evaluating the overhead of dataset delivery via air interface

  - Floating compression, e.g., Float32

  - Rel-16 TypeII CB-like method with new parameters

- Fujitsu: In order to achieve a low-overhead dataset transferring in separate training, the codebook-based quantization approach should be further studied

  - However, the overhead for dataset transfer is large. To reduce the overhead due to the dataset transfer, the dataset should be carefully designed in such a way that the space of the channel vectors is well represented by a set of small size. To begin with, Rel-16 e-type II codebook can be used for this purpose. The dataset can also be constructed by using the framework of codebook generation, but not restricted to the parameters given in the technical specification.

- Fujitsu: The choice of parameters for the codebook-based dataset should be studied, for the purpose of finding an optimal configuration where the performance and overhead tradeoff is achieved

- Findings on overhead of dataset

  - Huawei, Hisilicon: the average overhead per hour for Float32 compression is only 5.9 MB if data collection period is 1 week, and it can be reduced to 238 KB if training dataset is quantized by Rel-16 TypeII CB with new parameters.

  - Huawei, Hisilicon: For AI/ML-based CSI compression on spatial-frequency domain, compared with the training dataset quantized by Float32 format, the training dataset quantized by Rel-16 TypeII CB with new parameters can provide only minor performance margin (<0.7%) but reduces 96% overhead of training dataset delivery

  - Ericsson: High-resolution FD+SD basis vector preprocessing of the channel significantly reduces the channel dimension without losing important information for SU- and MU-MIMO transmissions

  - Fujitsu: It is observed that there is a significant overhead reduction of transferring a codebook-based dataset than a dataset composed by channel vectors of floating-point numbers for separate training. So, it is worth to study codebook-based quantization method in order to achieve a low-overhead dataset transferring in separate training

  - Fujitsu: The parameters of the codebook weight differently in the SGCS between the quantized codebook vectors and the ground-truth right singular vectors of the channel matrix

Source of dataset

- MediaTek: Considering the potential number of mixing dimensions, offer public datasets to facilitate study on generalization aspects of AI/ML models

- OPPO: Companies are encouraged to disclose their utilized dataset(s) and reference model(s)

  - FFS: to establish common dataset(s) and/or reference model(s) for performance calibration and drawing final conclusions.

- NVIDIA Companies are encouraged to contribute real data to develop and evaluate AI/ML based algorithms for CSI feedback enhancement

1st round email discussions

### 2.2-1: Remaining issues of the EVM table

- Issue#2-1 (Closed) Traffic model

As a controversial issue remaining from the 109-e/110 meetings, whether the full buffer can be optionally considered or taken as the baseline as the same as FTP model, has been discussed by companies for this meeting.

From the inputs of this meeting, a majority of companies are in favor of taking FTP as baseline, while selecting full buffer as an optional choice. In addition, more companies have provided FTP results which conduct to some observations on the impact of traffic load (RU) to the gains of AI/ML-based solutions (see “Impact of traffic load/RU” in 3.1). So let’s try if we can agree on the following proposal.

Proposal 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

| Traffic model | FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. Full buffer model is not precluded at least for the purpose of calibration and/or comparing results, while the conclusions for the use cases in the SI should be drawn based on FTP model. |
| --- | --- |

| Support/Can accept | MediaTek, Fujitsu, OPPO, CATT, Xiaomi, vivo, Lenovo, NTT DOCOMO, LG, ZTE, NVIDIA |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| MediaTek | Agree. FTP mode 1 is more general as baseline for simulating burst traffic conditions and can better reflect the reality. For AI based architecture (or data-based architecture), it is more robust if we use real traffic models to compare users' throughput. Moreover, RAN1 MIMO also adopt FTP mode 1 as the baseline EVM, so we can follow it. |
| Samsung | Ok. |

- Issue#2-2 (Medium) Channel estimation- Whether ideal channel estimation is applied for dataset construction, or performance evaluation/inference

As a remaining issue from the last meeting, if ideal channel is adopted (which is optional), whether the ideal channel estimation results is applied for dataset construction, or performance evaluation/inference needs to be discussed.

Based on the inputs from companies, it looks more companies are in favor of Option1, but the number of companies providing their attitudes is still limited, so a question is raised in the first round. Different from the last meeting, Option 3 is added. It is Moderator’s thinking that if this issue is still controversial after this meeting, then Option 3 would be the solution by default.

Question 2.2.1: If ideal DL channel estimation is considered (which is optional), whether ideal channel estimation is applied for only dataset construction, or also performance evaluation/inference:

- Option1: Ideal channel estimation is used for both dataset construction and inference

- Option2: Ideal channel estimation is used for dataset construction, while realistic channel estimation is used for inference

- Option3: Whether the ideal or realistic channel estimation is applied for dataset construction and performance evaluation/inference is up to companies

- Option4: Other

| Option 1 | Support/Can accept | NTT DOCOMO, NVIDIA, Futurewei, Spreadtrum, Panasonic, Xiaomi, vivo, CMCC, Qualcomm, Fujitsu |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | MediaTek,OPPO,ETRI, CAICT, InterDigital, CATT, LG, Lenovo |
|  | Object/Concern |  |
| Option 3 | Support/Can accept | vivo, Fujitsu |
|  | Object/Concern |  |
| Option 4 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| MediaTek | While the ideal channels can be used for calibrating AI/ML models, the final evaluation needs to target realistic channels to avoid any performance mismatch in the design and deployment phases. |
| OPPO | We prefer to use ideal channel estimation for dataset construction. In inference stage, ideal channel can be used for calibration, while the final conclusion should be drawn based on realistic channel estimation. |
| ZTE | From our view, ideal channel estimation can be applied for dataset construction and calibration on intermediate KPIs. For the eventual SLS evaluation, the final conclusion should be drawn based on realistic channel estimation. |
| Intel | In our view ideal channel estimation can be applied for dataset construction for training. For inference, we think both ideal and non-ideal channel estimation can be considered. |
| Futurewei | We prefer Option 1. As the moderator described in the proposal “If ideal DL channel estimation is considered (which is optional)”, ideal channel estimation can be used in the evaluation using intermediate KPIs for the purpose of verifying AI/ML model performance. For AI/ML model, typically consistent/same data distribution in training and testing is assumed. |
| InterDigital | It doesn’t make sense that the ideal channel estimation is assumed for inference as it is not realistic assumption and it will lead to performance mismatch when it is deployed as also mentioned by MediaTek. |
| Xiaomi | We prefer Option1. We think the training dataset and testing dataset should be consistent, i.e., Ideal channel estimation is used for both dataset construction and inference. |
| LG | We have the same view with InterDigital. |
| CMCC | We prefer Option1. If ideal channel estimation is adopted optional to see the upper bound of AI/ML based method, we prefer the training and inference dataset is generated based on same assumption. |
| Qualcomm | In our view, “Ideal DL channel estimation is optionally considered” refers to the inference. We support option 1. Separately, in our view, dataset construction should also be based on ideal channel as a starting point to separate the dataset generation/training from the system-level evaluation. (Request to moderator: In the moderator summary in Section 2.1-1 above, Qualcomm is marked as supporting Option 2. Please update this to Option 1). |

- Issue#2-3 (High priority) Channel estimation-Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

As another FFS issue related to channel estimation and has been discussed in the last two meetings, whether the target CSI should consider ideal channel even under the realistic channel estimation (with the logic that this ideal CSI can be regarded as the upper bound) has been discussed in the contributions of this meeting.

For this issue, Option 1 in below is the most favorite option (7 companies), while 2 companies prefer Option 2. DCM [18] brought up that ordering issue may happen when the inference input is realistic CSI while the target CSI is ideal, which will cause performance underestimation. To Moderator’s understanding, if the ideal channel is used as the target CSI, it is equally applied to the intermediate KPI calculation for both AI/ML and non-AI/ML baseline (e.g., legacy eType II CB), so the underestimation may not impact the relative performance gain in the end. Hence, let’s first try to see if Option 1 is also acceptable to Option 2 proponents.

Proposal 2.2.2: In the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, if realistic DL channel estimation is considered, regarding how to calculate the intermediate KPI of CSI accuracy, Option 1 of the following is preferred:

- Option1: Use the target CSI from ideal channel and use output CSI from the realistic channel estimation

- Option2: Use the target CSI from realistic channel estimation and use output CSI from the realistic channel estimation

- Option3: Companies to report the target CSI is from ideal channel or from the realistic channel estimation

- Option4: Other

| Support/Can accept | MediaTek, OPPO, CATT, ETRI, Xiaomi, vivo, Lenovo, NTT DOCOMO, ZTE, Ericsson, Futurewei, CAICT, InterDigital, New H3C, Spreadtrum, AT&T, Panasonic, CMCC, Qualcomm |
| --- | --- |
| Object/Concern | Fujitsu |

| Company | View |
| --- | --- |
| MediaTek | In our opinion, ideal channels as the target CSI reveals an AI/ML model’s real capability in compressing CSI, and its robustness against noise can be reported separately. We prefer option 3. |
| Samsung | We believe Option2 is the reasonable option. Given this topic has been discussed for two meetings now and the majority of companies support Option 1, we can live with Option 1. |
| Fujitsu | Our view is that the target CSI and the output CSI should be computed from the same type of channel estimation, either both ideal or both realistic. So we propose Option 4 to be as follows: Use the target CSI from realistic channel estimation and use output CSI from the realistic channel estimation, or, Use the target CSI from ideal channel estimation and use output CSI from the ideal channel estimation. |
| ETRI | In our view, using target CSI from ideal channel estimation is more appropriate to see the eventual accuracy of the CSI feedback. |
| vivo | Whether using ideal channel or realistic channel estimation does not impact evaluation results. We should not spend too much time on it, and simply follow up the majority view to make the agreement. |
| Lenovo | For intermediate results, we believe option 1 should be okay. |
| NTT DOCOMO | Thanks for the clarification. If it is common understanding that the intermediate KPI is to evaluate the relative performance with baseline, we are fine with the proposal. |
| LG | We also think option 2 is more reasonable than option 1. |
| ZTE | For calibration on intermediate results, ideal target CSI can be used. |
| NVIDIA | Agree with Fujitsu that target CSI and the output CSI should be computed from the same type of channel estimation, either both ideal or both realistic. |
| Intel | In our view Option 1 should be considered. |
| Futurewei | We are ok with Option 1 or Option 2. It is more likely Option 2 is close to data collection situation in the deployment phase, so our preference is Option 2 while Option 1 is ok for study purpose. |
| CAICT | Similar with as vivo. |
| Spreadtrum | We prefer option 2. The same type of channel estimation should be used for target CSI and the output CSI. |
| AT&T | We would prefer option 2 as the same channel model should be used for both target and output CSI. |
| CMCC | Since it is just intermediate KPI, Option1 is fine for us. |
| Qualcomm | To measure the accuracy of the CSI for a given channel realization, the output CSI should be compared with the CSI corresponding to the true channel realization. Using the ideal channel for the target CSI allows to capture any gain resulting from the ML model learning to output the CSI of the true channel based on the realistic channel. |

### 2.2-2: Metrics

- Issue#2-4 (High priority) SGCS calculation granularity

The calculation of SGCS is an average of SGCS values over all frequency units and all samples. E.g., in the following formula where rank=1, N is the number of frequency units, wi is the eigenvector of the target CSI at frequency unit i, wi is the output vector at frequency unit i,  E{∙} denotes the average operation over multiple samples.

SGCS=E1Ni=1N||wiHwi ||||wi||  ||wi||2

It has been raised by Ericsson [6] that the granularity of the SGCS has to be aligned over companies to better align the results. As per Moderator’s review, some companies used RB as the granularity while some companies used subband as the granularity. Therefore, a question is brought up to see if it is needed to align the granularity of SGCS calculation, and if so, what is the frequency granularity.

Question 2.2.2: In the evaluation of the AI/ML based CSI feedback enhancement, for the calculation of SGCS and NMSE, what should be the granularity of the frequency unit for averaging operation?

- Option1: 1 RB

- Option2: 1 Subband

- Option3: Other

| Option 1 | Support/Can accept | OPPO, Ericsson, Qualcomm |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | Samsung,OPPO, ETRI, NVIDIA, Ericsson, Futurewei, CAICT, InterDigital, New H3C, Spreadtrum, MediaTek, Xiaomi, vivo ,CATT, CMCC, Lenovo, Fujitsu, ZTE |
|  | Object/Concern |  |
| Option 3 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| Samsung | We slightly prefer subband based comparison for the calculation of NMSE and SGCS. The RB based comparison could be marginally more accurate (finer frequency units) but it may require alignment in the assumed interpolation scheme for the baseline based on Rel-16 CB. Companies may report the value they considered for numberOfPMI-SubbandsPerCQI-Subband=1,2. |
| OPPO | Both option 1 and option 2 can be considered. However, since option 2 has been widely adopted currently, we think it is better to use option 2 for intermediate KPI calibration. |
| Intel | In our view Option 1 shall be assumed since it is not clear whether there is a subband concept for the AI/ML CSI. |
| Ericsson | If subband is agreed then we also need to agreed the number of RB in a subband otherwise we are back at square one. |
| Futurewei | We prefer using Option 2 for intermediate KPI calculation. |
| Xiaomi | We prefer Option 2. Considering the baseline is Type II codebook, it is better to use similar concept with the baseline. |
| vivo | We think the granularity of SGCS needs to be the same as PMI report. |
| CATT | We can follow the subband definition in legacy CSI report. |
| CMCC | Prefer Option2 as it is more aligned with legacy codebook. |
| ZTE | We prefer option 2 for calibration since option 2 is widely adopted for evaluation among companies. If option 1 is considered, maybe how to align align the interpolation scheme from the granularity of subbands to RBs needs further clarification. |

- Issue#2-5 (High priority) SGCS calculation for rank>1

Another FFS issue is, when Cosine Similarity is selected as the intermediate KPI, then for rank>1 case, how to obtain the intermediate KPI by calculating the Cosine Similarity. There are three methods raised in the agreement of the last meeting, where the benefit of each method is given as per the inputs from companies for this meeting. For Method 2, in particular, there are 3 different opinions on how to perform the weighted average, as also listed in below.

- Method 1: Average over all layers - 5 companies

  - Pros: simple; all of the metrics have a similar relationship to throughput

SGCS=E1N×Ki=1Nj=1K||wijHwij ||||wij||  ||wij||2

Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the jth output vector of the output CSI of resource unit i. N is the total number of resource units. E{∙} denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers - 3 companies

  - Pros1: The normalized weighted SGCS gives a larger number than the SGCS;

  - Pros2: The SGCS decreases as the rank of the channel matrix increases.

  - Pros3: Instead of averaging over ranks evenly in the GCS formula, the stronger ranks whose GCS are larger are more heavily weighted than the weaker ranks in the normalized weighted GCS.

  - Method 2-1: 2 companies

SGCS=E1N×Ki=1NjK(λij)2j=1K(λij)2||wijHwij ||||wij||  ||wij||2

  - Method 2-2: 1 company

SGCS=E1Ni=1NjKλijj=1Kλij||wijHwij ||||wij||  ||wij||2

Note: wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer) - 6 companies

  - Pros: can provide the insights for per layer, e.g., the accuracy of AI/ML based CSI compression has different effects on different layers

However, it has been argued by Ericsson [6], and Lenovo [14], that SGCS is sensitive to exact definition of the ground truth in a way that is not physically relevant, e.g., the ordering of the eigenvalues and the frequency granularity, for rank>1 cases, while some of the raised intermediate KPIs above are not affected by a mismatch in the order of the eigenvectors between the target CSI and output CSI.

[R1-2206883, RAN1#110] For example, imagine SU-MIMO transmission over a channel with two distinct main clusters with approximately equal strength. The ordering of the precoding vectors for these clusters does not matter from a DL throughput perspective. However, the GCS and SGCS KPIs need to be applied in a layer-by-layer manner using the “correct” order. For example, single-layer transmission using the precoding vector for “wrong” cluster will result in almost the same throughput, but GSC, SGSC, GCS_LM, and SGCS_LM will report low values. Hence, this vector-swapping example illustrates a problem with the naïve (S)GCS-based metrics.

But it is Moderator’s first impression that such disorder issue can be resolved with AI/ML model design, e.g., input/output type, or with some processing when calculating the SGCS statistics, e.g., order the output eigenvectors by performing the correlation operation with the original eigenvectors of the corresponding layer. E.g., for the output eigenvectors of w1, w2, w3, w4, their orders can be determined by performing the correlation with the original input eigenvectors of w1, w2, w3, w4; in other words, the SGCS for 4 layers are the largest 4 values of the 16 combinatorial SGCS values.

So, a question is raised in advance to collect the views from companies, that whether SGCS is sensitive or immune to the eigenvector disorder issue.

Question 2.2.3: In the evaluation of the AI/ML based CSI feedback enhancement, if SGCS is adopted as the intermediate KPI for the rank>1 situation, whether and in what case there will be disorder of the output eigenvectors (e.g., the layer index of the output eigenvector of the AI/ML model does not match the layer index of the corresponding input eigenvector)?

| Yes | Lenovo, Qualcomm |
| --- | --- |
| No | OPPO |

| Company | View |
| --- | --- |
| Moderator | For the proponent companies who think the disorder issue occurs for rank>1, please elaborate the details (input type, output type, whether any processing in simulation can resolve/alleviate the issue). For the opponent companies who think the disorder issue does not exist if SGCS is adopted, clarifications are also welcome. |
| Samsung | We also agree with FL’s view that this problem can be alleviated by reordering the output vectors while computing the SGCS. |
| OPPO | From our understanding, the disorder issue does not exist. If the layer-specific/common AI/ML model is utilized, the correct CSI output can be obtained with the input CSI of corresponding layer. For rank-specific AI/ML model, the CSI eigenvector for each layer can also be extracted correctly from the output of AI/ML model. |
| NTT DOCOMO | If the correlation operation is performed to determine the order before the SCGS calculation, the disorder issue can be avoided. |
| Intel | Ordering of the SVD vectors can be corrected for the intermediate metrics calculation per layer. |
| MediaTek | We think it is a design problem and not a general case. So, should not be discussed as a general issue. On the other hand, layer-common models do not have this problem. |
| CATT | In addition to FL’s comments, we also doubt that it is a usual case where two spatial paths have almost equal strength. |
| CMCC | Tend to agree with FL. Some post-processing can be helpful to alleviate disorder issue. |
| Lenovo | Why we might have disordered eigenvectors: Consider a two-sided model that gets raw CSI as the input and reconstruct the raw CSI. In this case we can determine K eigen-vectors from the input CSI. After feedback and channel reconstruction, at the gNB we can again determine K eigen-vectors from the reconstructed CSI. Note that, here there is no clear mapping between which of the output eigne-vectors should go with which of the input vectors. This ambiguity is more visible in case of imperfect reconstruction, or having eigen values not well separated. Note 1: We acknowledge that if we feed the model directly with eigenvectors (not raw CSI), there will be no eigne-vector matching issue. Note 2: The idea presented by NTT-DOCOMO may result in a not accurate similarity measure. To illustrate assume that raw input channel with the input eigen vectors of V1 and V2 resulted in V1^hat and V2^hat. The correct cosine similarity now should be evaluated using < V1, V1^hat> and < V2, V2^hat> Now assume that due to low feedback bits (imperfect reconstruction), the estimated eigen-vectors are not that accurate so the < V1, V1^hat> and < V2, V2^hat> are not close to 1. In this case, it could be possible that < V1, V2^hat> becomes larger than < V1, V1^hat> and based on NTT-DOCOMO proposal we pair V1 and V2^hat. In this case, the cosine similarity will be calculated based on < V1, V2^hat> and < V2, V1^hat> which is incorrect. In short, the eigenvalue matching that results to highest SGCS is not always the correct matching. |
| Qualcomm | The SGCS formula clearly shows that it is sensitive to the order assigned to the output CSI vectors relative to the order of the target CSI vectors. A common way to order the target CSI vectors is based on the eigen value. The moderator has proposed one method to order the output CSI vectors – based on correlation. Different companies may use a different method to derive the order, resulting in different KPI value for the same output. Moreover, the physical meaning associated with ordering based on correlation is not clear. For example, if two of the output CSI vectors are identical, the rank and therefore the capacity will be limited. However, the correlation or SGCS of this vector with target CSI vectors may be high. The RAR metric discussed in Issue 2-6 addresses these concerns. |
| Fujitsu | We agree with the FL’s view on this issue. |
| ZTE | Agree with OPPO. |

Proposal 2.2.3 (on hold): For the evaluation of the AI/ML based CSI feedback enhancement, if the SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, consider both the SGCS calculation/extension methods of Method 3 and a down-selected method between Method 1, Method 2:

- Method 1: Average over all layers

- Method 2: Weighted average over all layers

SGCS=E1N×Ki=1NjK(λij)2j=1K(λij)2||wijHwij ||||wij||  ||wij||2

where wij is the jth eigenvector of the target CSI at resource unit i and K is the rank. wij is the  jth output vector of the output CSI of resource unit i. N is the total number of resource units.  E. denotes the average operation over multiple samples. λij is an eigenvalue of the channel covariance matrix corresponding to wij.

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- FFS the down selection between Method 1 and Method 2

- Issue#2-6 (High priority) Other intermediate KPIs

As discussed in the last meeting, there are other intermediate KPIs each of which is supported by a limited number of companies. To this meeting, the following intermediate KPI is supported by 3 companies, so it is brought up for further discussions (formula borrowed from Ericsson [6]).

- Alt.4A: Relative achievable rate (RAR) – 3 companies

RARγ,L=EH1Ff=1FlogdetI+γPfHHfHHfPfEH1Ff=1FlogdetI+γPfHHfHHfPf

    - Hf~[NRx×NTx] is the complex channel matrix for RB f

    - F is the total number of RBs,

    - Pf~[NTx×L] is the reported precoding matrix for RB f,

    - Pf~[NTx×L] is the optimal (SVD-based) precoding matrix for RB f,

    - L is the number of MIMO layers the KPI is evaluated for,

    - γ is the SNR-value;

γ=100EH1Ff=1FσmaxHf

and the outer expectation EH is taken over a distribution of MIMO channels.

As for the value of γ, there are different views from the 3 proponent companies, so for this option, the value of γ is moved to FFS.

Question 2.2.4: For the intermediate KPI for evaluating the accuracy of the AI/ML output CSI, except for SGCS and NMSE which has been agreed as the baseline metrics, which of the following option do you prefer?

- Option 1: Relative achievable rate (RAR),

RARγ,L=EH1Ff=1FlogdetI+γPfHHfHHfPfEH1Ff=1FlogdetI+γPfHHfHHfPf

where Hf~[NRx×NTx] is the complex channel matrix for RB f

F is the total number of RBs,

Pf~[NTx×L] is the reported precoding matrix for RB f,

Pf~[NTx×L] is the optimal (SVD-based) precoding matrix for RB f,

L is the number of MIMO layers the KPI is evaluated for,

γ is the SNR-value;

  - FFS the value of γ used for RAR calculation

- Option 2: Other intermediate KPIs are not considered as baseline for metrics, and can be optionally considered and reported by companies.

- Option 3: Other

| Option 1 | Support/Can accept | Ericsson, Lenovo, Qualcomm |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | Samsung,OPPO, NTT DOCOMO, NVIDIA, Futurewei, CAICT, New H3C, Spreadtrum, AT&T, MediaTek, Xiaomi, vivo, CATT, CMCC, ZTE |
|  | Object/Concern |  |
| Option 3 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | For the proponents of Option 1, please also elaborate your views on how to set γ to express RAR. |
| Samsung | As we discussed in our Tdoc, the additional advantages of using the channel matrix and reported precoders to compute intermediate KPIs, e.g., in terms of capturing inter-layer interference, emulating SINR, etc, are not applicable if the gNB does not directly apply the reported precoders. Moreover, considering additional intermediate KPIs may pose difficulties in fair comparison and drawing conclusions. |
| OPPO | We think that the intermediate KPI is proposed only for calibration between companies. Too many metrics are not helpful for calibration. |
| Intel | We prefer to focus on SGCS and throughput for the AI/ML CSI evaluations. |
| Ericsson | The RAR can be used by proponents as the single optional intermediate KPI. |
| Futurewei | We do not recommend using many intermediate KPIs. Companies may end up using various sets of KPIs which may make the comparison more difficult. |
| MediaTek | The final KPI is the most important indicator to measure the AI/ML performance. Therefore, we suggest not to consider many intermediate KPIs. |
| CMCC | We are open for other intermediate KPIs, but it should be optional. |
| Lenovo | - When rank>1, we will have several SGSC for each layer. To be able to compare different schemes, however, we need to have a scalar metric. The proposed metric is therefore can be used for such evaluation when ranke>1 and as we discussed it lso to resolve the eigenvalue mismatch problem. - The proposed metric gives a more realistic expected gain compared to metrics presented in Proposal 2.2.3. It is due to the fact that it considers the effect of selected precoder mismatch for that actual channel realization (not just angle between the two precoders). In response to Samsung point: Your point is valid and the gNB might want to select another precoder other than what is fed back by the UE. So our intension here is not to determine the actual rate that will be transmitted by the gNB. Here the intension is just to show what can be achieved if the gNB uses the feedback precoder (not forcing the gNB to use that precoder). - In a general case, γ can be set to different values so we can observe the performance of the model in different SNRs. However, to have a single metric we can set γ to a constant number as 1. |
| Qualcomm | The formula in option 1 is not sensitive to the relative ordering of the target CSI vectors and the output CSI vectors. Moreover, it reflects the impact of the error in the precoder on the spectral efficiency. As discussed before, SGCS-based metrics are sensitive to the ordering of the target CSI vectors relative to the ordering of the output CSI vectors. |
| Fujitsu | As an intermediate KPI, the SGCS (including normalized weighted SGCS for the case of rank>1) is enough for evaluation purposes. We think that we do not need to have these capacity-oriented metrics, which can be finally reflected in the throughput performance. |
| ZTE | We think too many intermediate KPIs are not helpful for calibration among companies. |

- Issue#2-7 (High priority) Benchmark for evaluation results comparison

In the 109-e and 110 meeting, it has been agreed that companies to report Rel-16 or Rel-17 TypeII CB is used as baseline in the evaluation, while there is still a FFS issue on whether Type I CB should be also considered as a candidate benchmark.

Agreement For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows Baseline for performance evaluation; Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation.

From the inputs of this meeting, there are 2 companies believe it is not necessary to take Type I CB as a baseline. Based on the inputs of this meeting and the replies of the 110 meeting, the following proposal is provided.

Proposal 2.2.4: In the evaluation of the AI/ML based CSI feedback enhancement, for “Baseline for performance evaluation” in the EVM table, Type I Codebook can be optionally considered for comparing AI/ML schemes up to companies

- Note: Type II Codebook is baseline as agreed

| Support/Can accept | MediaTek, Fujitsu, OPPO, CATT, ETRI, vivo, Lenovo, NTT DOCOMO, LG, NVIDIA, Ericsson, Futurewei, CAICT, InterDigital, New H3C, Spreadtrum, AT&T, Panasonic, CMCC, Qualcomm (comment below), Fujitsu, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Samsung | Ok. |
| Ericsson | If Type I is included then Type II baseline should also be included (so that the best possible baseline is used at all time). Note that Type I outperforms Type II for some cases. |
| Qualcomm | Using Type I codebook by itself as a baseline with result in the over-estimation of gain from AI/ML-based CSI feedback. To avoid this, if Type I is considered, then the better choice between Type I and Type II should be used as the baseline. |

### 2.2-3: Generalization verification

- Issue#2-8 (High priority) Case 2A for generalization verification

In the last meeting, 3 cases are agreed for generalization verification methodology. Still there is one FFS on whether Case 2A (fine-tuning) should be also considered for generalization verification. From the inputs of companies for this meeting, 2 companies are in favor of considering Case 2A as a generalization case with the views that Case 2A provides different performance from Case 1/3, while 2 companies are against it by feeling Case 2A is one special case of Case 1/3.

| Proponents | Views |
| --- | --- |
| Huawei, Hisilicon | For Case 3, besides training, the testing dataset for the AI/ML model is based on datasets from both Scenario#A and Scenario#B, so the trained AI/ML model can adapt to the channel characteristics of both scenarios; while for Case 2A, the fine-tuning procedure, including the validation/testing, only takes into account the dataset from Scenario#B, so the model after fine-tuning can well match Scenario#B but may lose the match with Scenario#A to some extent. |
| ZTE | Case 2A also differs from Case 3 that Case 2A only focuses on the performance in the second Scenario/Configuration. However, Case 3 takes the performance of both Scenarios/Configurations into account |

| Opponents | Views |
| --- | --- |
| Samsung | One can see this case as Case 1 as from the testing perspective the model is last trained/finetuned by the same scenario as the scenario the test is conducted. One can also view Case2A as part of Case 3. In the above example, the fine-tuning training can basically mix the training dataset. The mixed training dataset in Case 3 can be applied in both interleaved manner (dataset samples from multiple scenarios/configurations interleaved in the training) or ordered manner (dataset samples from one scenario/configuration applied one after the other). |
| LG | in the generalization perspective, Case 2A can be included in Case 3. Therefore, Case 2A and Case 3 can be merged. |

The following question is then raised to collect views from more companies to see if Case 2A should be considered as an individual case for generalization verification.

Question 2.2.5: For the generalization verification of CSI feedback enhancement, whether Case 2A should be classified as an individual case for generalization verification?

- Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B

| Support/Can accept | Futurewei, vivo, Lenovo, ZTE |
| --- | --- |
| Object/Concern | Ericsson, MediaTek, OPPO, Xiaomi |

| Company | View |
| --- | --- |
| Intel | In our view Case 1, Case 2 and Case 3 are enough for generalization performance verification. |
| Ericsson | This seems to be a test of fine tuning and not of generalization verification. |
| Futurewei | We think Case 2A can be considered as an individual case. The difference between Case 3 and Case 2A is that Case 3 assumes datasets from 2 or multiple scenarios/configurations are available to train the AI/ML model from the beginning while Case 2A typically assumes that only the trained model (for scenario/configuration A) is available while the original (full) dataset used in training the first model is not available, and the labelled data from the new scenario/configuration (e.g., scenario/configuration B) is limited. Thus, the trained model (even though for a different scenario/configuration) is fine-tuned using the data from the new scenario/configuration in order to reuse the originally trained model in the new scenario/configuration. |
| CAICT | Same view as Intel. |
| MediaTek | Case2A is the subset of Case3 and we believe Case2A is not a category of generalization, it is a fine-tuning technique to obtain objective of Case3. No need to be introduced as a generalization case. |
| OPPO | For generalization verification, Case 2A with fine-tuning should be studied with low priority. Before considering Case 2A, the details of fine-tuning should be clarified. For example, offline or online fine-tuning should be considered? And the size of dataset, time of fine-tuning may also influence the performance of fine-tuning. |
| Xiaomi | We share similar view with Intel. |
| vivo | We believe the Case 2A is an independent case, and should be classified as an individual case. |
| CATT | We do not have strong views on how to mark/number this case. But we are OK if this case can be explicitly listed since it focuses on the capability of fine-tuning, targeting at one specific scenario. |
| Lenovo | Yes, we think this case can be considered as an independent case. |
| Fujitsu | It is mentioned in the main bullet in the agreement that: The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: We wonder whether the model before and after update/finetuning can be counted as an AI/ML model or not? We think that this should be clarified. |
| ZTE | From our view, Case 2A can be considered as an individual case. For Case 2 and Case 3, once model training is finished, the parameters of AI/ML model are fixed. Then, the trained AI/ML model is tested for generalization. However, Case 2A is different from Case 2 and Case 3 since it entails parameter modification. Case 2A aims at addressing the generalization problem of Case 2 via fine-tuning with the data from the new scenario/configuration. In addition, the fine-tuned data may be much less than the data in the mixed dataset for Case 3. |
| Samsung | We understand the intention of Case2A as fine-tuning can serve as one way of solving the generalization issue. However, We think it is better if RAN1 first focuses on studying the three agreed cases and get back to this case later if time allows. |

- Issue#2-9 (Medium) Per-area/cell/region model

Besides the statistical channel model which has been agreed as the baseline for evaluation, one company also evaluated a ray-tracing channel model, and evaluated with the dataset collected from field [5], to simulate the performance of AI/ML for per-area, with the observations that AI/ML can achieve much better performance under a per-area usage (as the channel characteristics are less diverse). It is Moderator’s understanding that it is meaningful to look into the performance of per-area AI/ML model though, it will consume time to align the EVM for the ray-tracing channel model (e.g., the digitized map), calibrating the ray-tracing model, or align the dataset from field on the other hand. Therefore, it may be up to companies to submit the results for information, as agreed in 109-e.

Agreement - Use 3gpp channel models (TR 38.901) as the baseline for evaluations. - Note: Companies may submit additional results based on other dataset than generated by 3GPP channel models

Question 2.2.6: For the evaluation of CSI enhancements, do you think the evaluations based on ray-tracing channel models (other than the statistic channel models as agreed) or based on field dataset can be considered up to companies?

| Support/Can accept | NTT DOCOMO, NVIDIA, CAICT, MediaTek, Panasonic, OPPO, vivo, CATT, CMCC, Lenovo, Samsung |
| --- | --- |
| Object/Concern | Ericsson, InterDigital, Qualcomm |

| Company | View |
| --- | --- |
| Intel | It is not clear for us what are consequences of this proposal. Are we going to study new channel model based on ray tracing or specify dataset for AI/ML study? This specific discussion may take a lot of time to converge in RAN1. |
| Ericsson | Companies can submit anything they want, but keep in mind that whether these results are included in the TR is another discussion. For TR inclusion there need to be able to draw a conclusion from similar/aligned results from multiple companies. |
| InterDigital | Agree with Ericsson |
| MediaTek | At present, the topic is still in SI. Therefore, companies are welcome to study and submit the results of different channel models. |
| OPPO | The evaluations on ray-tracing channel models or field dataset may show larger performance gain with per-area AI/ML model. Companies can provide these results if possible. |
| Xiaomi | We think the statistic channel model based on TR 38.901 is sufficient. |
| vivo | According to our evaluation, per-cell (region) model can achieve near-optimal CSI compression performance at the matched area/environment, which obviously surpasses that of general models trained on data collected from a variety of situations. This case should be captured with further detailed discussion. The field data, in addition, could make the performance investigation more concrete and realistic. |
| LG | Agree with Ericsson. |
| Lenovo | Results specially based on field dataset could be very useful for evaluation purposes. So, it is desirable if companies optionally can provide such results. |
| Qualcomm | Alignment across companies on datasets from the field or a ray-tracing model may take a lot of time in RAN1. |
| Fujitsu | We share a similar view to that of Ericsson for this question. |
| ZTE | We think evaluation on the field dataset can be deferred to discuss. |
| Samsung | We are ok if companies submit their evaluation results. |

### 2.2-4: Others

Question 2.2.7: Do you think there are additional high priority issues or EVM parameters which are generic to all sub use cases and have not been discussed/captured in previous sub-sections?

| Company | View |
| --- | --- |
| ZTE | From our view, we can add the comparison of throughput gains between AI approaches and the ideal eigenvector feedback(performance upbound). Since the eType II performance is different from companies and it is not good enough for throughput, and we can compare the AI performance with the ideal situation to see there is how much room existed for AI to improve. |

Specific evaluation methodology for CSI compression sub use case

Summary of views from companies

### 3.1-1: Sub use cases evaluated by companies

CSI compression in spatial-frequency domain

- Findings on spatial-frequency domain CSI compression

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 7%-10.3% gain on rank 1 under full buffer traffic.

    - 8.9%-14.8% gain on rank 2 under full buffer traffic.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 2.2%-2.5% gain on rank 1 under FTP traffic and 50% RU.

    - 3.2%-7.8% gain on rank 1 under FTP traffic and 70% RU.

    - 4.8%-8.8% gain on rank 1 under FTP traffic and 80% RU.

    - 5.4%-10.1% gain on rank 2 under FTP traffic and 50% RU.

    - 7.2%-14.4% gain on rank 2 under FTP traffic and 70% RU.

    - 7.5%-14.5% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain can outperform Rel-16 Type II codebook with

    - 2.2%-7% gain on rank 1 under FTP traffic and 50% RU.

    - 6.4%-14.6% gain on rank 1 under FTP traffic and 70% RU.

    - 7.1%-16.3% gain on rank 1 under FTP traffic and 80% RU.

    - 4.3%-13.3% gain on rank 2 under FTP traffic and 50% RU.

    - 8.1%-18% gain on rank 2 under FTP traffic and 70% RU.

    - 9.1%-23.7% gain on rank 2 under FTP traffic and 80% RU.

  - Qualcomm ML CSF provides UL PMI overhead gains of 53% over eType2, at an average throughput of 15 Mbps, and 40% at 15.7 Mbps. At a given UL PMI overhead of around 134 bits, ML CSF provides 9% average throughput gain. At the cell-edge, the gain in overhead is roughly 30% for a throughput of 2.5 Mbps. At an overhead of around 134 bits, ML CSF provides a cell-edge throughput gain of 9%.

  - ZTE: AI based CSI reconstruction completely outperforms the Rel-16 eType II in term of SGCS for rank=1. With the same feedback overhead, AI based CSI recovery can obtain 7%-8% SGCS gains

  - ZTE: AI based CSI reconstruction achieves very little UPT gain for the case of rank=1 and less than 1% average UPT gain over Rel-16 eTypeII with the same feedback overhead

  - ZTE: AI based CSI reconstruction with Case 1 method (single-layer model input and single-layer model output) shows performance gains in SGCS per layer over the Rel-16 eType II for rank =2.

  - ZTE: With maximum rank up to 2 under the same feedback overhead, AI based CSI reconstruction has about 4%-6% average UPT gain over the Rel-16 eType II under the case of 50% RU and 5%-8.5% average UPT gain can be obtained by AI based CSI recovery under the case of 70% RU.

  - ZTE: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for maximum rank up to 2.

  - ZTE: The SGCS performance for AI/ML-based CSI compression presents positive correlation with the throughput performance for up to rank=4.

  - ZTE: AI based CSI reconstruction shows performance gain in SGCS per layer over the Rel-16 eType II for rank =3/4.

  - MediaTek: On average over evaluation settings with 100~300 bits of CSI feedback, AI/ML models achieve 5.43% GCS gain over eType II codebook in terms of CSI feedback accuracy. The GCS gain ranges from 3.75% to 6.47% for 100~300 bits of CSI feedback.

  - MediaTek: At 0.85 GCS, the AI/ML model is able to approximately reduce feedback overhead by 36%.

  - Apple: Transformer based AE can achieve better SGCS performance comparing to type II codebook

  - Xiaomi: AI based CSI enhancement shows 10% performance gain on average SE comparing with eType2 codebook

  - CATT: Compared to Rel-16 Type II codebook based CSI feedback, obvious performance gain can be achieved by CSI feedback with proposed scalable AI/ML model for rank=1, 2, 3, 4:

    - SGCS can be improved by 0.02~0.22 under the same CSI feedback payload;

    - Payload can be saved by 30%~60% bits under the same SGCS.

  - NTT DOCOMO AI-based CSI feedback could obviously improve the eventual performance over Rel-16 Type II codebook:

    - Rank 1: 5~17% gain for 5%-ile UPT, 18~30% gain for average UPT

    - Rank 2: 44~85% gain for 5%-ile UPT, 27~63% gain for average UPT

  - Intel: ML based Autoencoder can outperform Rel-16 eType II codebook for Rank-1 case in almost all overhead regimes for InH and Dense Urban Macro deployments

  - CMCC: Compared with traditional codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same or similar number of feedback bits.

    - With the same or similar number of feedback bits, AI based approach could obtain 4%~40% performance gain over traditional codebook in the square of generalized cosine similarity

  - CMCC: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits when achieving the same CSI or higher accuracy

    - With similar performance in the square of generalized cosine similarity, AI based approach could reduce 30%~60% feedback bits

  - CMCC: Compared with traditional codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits when achieving the same CSI or higher accuracy

  - InterDigital: AE based CSI compression achieves about 63% feedback overhead reduction and 13% improvement in the mean throughput relative Rel-15 Type II

  - CAICT: From preliminary results, AI based spatial-frequency domain CSI compression shows good SGCS performance at least for rank=1

  - China Telecom: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same feedback bits.

  - China Telecom: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits with the similar CSI accuracy.

  - ETRI: With an Autoencoder using a previously developed neural network structure, CsiNet, there are significant improvements in terms of SGCS compared to the baseline (eTypeII) in CSI compression sub use case.

  - Fraunhofer: The performance of NN are strongly dependent on the architecture of the network and the parameters of the scenario e.g., compression rate. Therefore, for each system parameters there is a specific optimal structure

  - Fraunhofer: The trained AI-based method outperforms Type II CSI when the compression rate is not small

  - Mavenir: GCS in the log scale reflects the post-equalization SINR better than other intermediate KPI candidates such as GCS in the linear scale, SGCS in the linear scale, and NMSE (both in the log and linear scales)

  - Mavenir: Post-processing SINR performance with the AE-based precoder is better compared to a system with the 5G NR Type I-based precoder. As a result, the deviation from the benchmark SINR, obtained through ideal SVD, is lower when the AE-based precoder is utilized.

  - Mavenir: The AE-based precoder provides a higher post-equalization SINR and throughput compared to the 5G NR Type I-based precoder, considering the same amount of feedback bits.

  - Mavenir: The throughput performance gap between the AE-based precoder and the 5G NR Type I-based precoder is more significant in the low SNR regime, and the gap shrinks as SNR increases

  - Spreadtrum: The AI based CSI compression and recovery can achieve better SGCS performance and lower feedback bits cost than Rel-16 eType II codebook

- Preliminary trends observed by more than one company

  - Different effects of AI/ML on different layers: The benefit of AI/ML method is more obvious in higher rank CSI compression due to better effect . Huawei, Hisilicon, ZTE, Apple, OPPO, Ericsson

    - Huawei, Hisilicon: For the rank=2 case, AI/ML-based CSI compression on spatial-frequency domain can achieve more gains over the Rel-16 Type II on the 2nd layer than on the 1st layer.

    - ZTE: AI/ML approaches show better throughput performance with heavy traffic load for up to rank=2.

    - ZTE: AI based CSI reconstruction shows larger performance gains in layer 3/4 than layer 1/2 in terms of SGCS with the assumption of the same feedback overhead of each layer.

    - ZTE: The case of rank>1 should be prioritized in later discussion.

    - Apple: AI based approach achieve higher gain in SGCS for higher rank compared to type II codebook

    - OPPO: Compared to rank 1 achieving 5%~8% SGCS gain and 1%~3% SLS throughput gain, AI based CSI feedback achieves 8%~16% SGCS gain and 4%~10% SLS throughput gain for rank 2. Compared with rank 1, the AI based CSI feedback has larger performance gain in rank 2.

    - Ericsson: The presented AE has an overhead that is similar to Rel16 Type-II ParComb1. For rank 1 and rank 2 transmissions the performance is expected to be close to ParComb1, but for rank 3 and rank 4 the performance is increased, almost to that of ParComb 3

  - Performance of different layers: 1st layer has higher SGCS than later layers Huawei, Hisilicon, Nokia, ZTE, MediaTek CATT

    - Huawei, Hisilicon: For the rank=2 case, the 1st layer achieves higher SGCS than that of the 2nd layer as the eigenvectors of the 2nd layer are more sparse

    - Nokia We find that the first layer has a significantly higher GCS than the second layer.

    - MediaTek: For rank-2 channels in Dataset 1-Dataset 3, EVs of layer 0 are more correlative across frequency and antenna domains compared to EVs of layer 1.

  - Impact of CSI payload size: Compared to higher feedback overhead, AI based CSI feedback achieves larger gain with lower feedback overhead.

    - OPPO: Compared to higher feedback overhead achieving 1% for rank 1 and 4% for rank 2 SLS throughput gain, AI based CSI feedback achieves larger SLS throughput gain with lower feedback overhead, about 3% for rank 1 and 10% for rank 2

  - Impact of traffic load/RU: AI/ML approaches show better throughput performance with heavy traffic load/full buffer. Huawei, Hisilicon, ZTE, OPPO, NTT DOCOMO

    - Huawei, Hisilicon: AI/ML-based CSI compression on spatial-frequency domain can achieve more obvious gains over the Rel-16 Type II codebook on higher RU

    - ZTE: With maximum rank up to 2 under the same feedback overhead, AI based CSI reconstruction has about 4%-6% average UPT gain over the Rel-16 eType II under the case of 50% RU and 5%-8.5% average UPT gain can be obtained by AI based CSI recovery under the case of 70% RU.

    - ZTE: For up to rank=4, with the same feedback overhead, AI based CSI reconstruction has about 4%-13% average UPT gain over the Rel-16 eType II under the case of 50% RU and 6%-16% average UPT gain can be obtained by AI based CSI reconstruction under the case of 70% RU.

    - ZTE: AI/ML approaches show better throughput performance with heavy traffic load for up to rank=4.

    - OPPO: Compared to FTP model achieving 1%~3% SLS throughput gain for rank 1, AI based CSI feedback achieves larger SLS throughput gain for full buffer model about 3%~6%.

    - NTT DOCOMO: The higher performance could be obtained in the higher traffic load, since there is more possibility for the scheduler to match the UEs for MU transmission which can be achieved by the finer CSI feedback

  - Effects on cell edge UEs: AI/ML can achieve more gains for cell edge UEs (than average). Huawei, Hisilicon, NTT DOCOMO, Mavenir

    - Huawei, Hisilicon: AI/ML-based CSI compression on spatial-frequency domain can achieve more obvious gains over the Rel-16 Type II codebook for cell edge UEs

    - NTT DOCOMO: the cell edge UE could benefit more from AI/ML model since the reduction of inter-UE interference affects the SINR more compared to cell center UE

    - Mavenir: The throughput performance gap between the AE-based precoder and the 5G NR Type I-based precoder is more significant in the low SNR regime, and the gap shrinks as SNR increases

CSI compression in temporal-spatial-frequency domain: Huawei, Hisilicon, CAICT

Solution description (from Huawei, Hisilicon): Different from the CSI prediction in time domain which predicts a future CSI with an AI/ML model, the CSI compression involving time domain compresses CSI with an AI/ML model by taking into account the historical CSI information, and the feedback CSI does not include predicted future CSI.

- Sub use case categorization

  - Huawei, Hisilicon: Categorize temporal-spatial-frequency domain CSI compression (AI-SFT) as a specific AI/ML solution under spatial-frequency domain CSI compression and capture the evaluations results of AI-SFT into the spatial-frequency domain CSI compression results

  - CAICT: The evaluation for CSI compression in spatial-frequency domain and in temporal-spatial-frequency domain could share the same evaluation framework

- AI/ML backbone

  - Huawei, Hisilicon: LSTM+Transformer

- Findings on temporal-spatial-frequency domain CSI compression

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of cell average throughput, AI/ML-based CSI compression on spatial-frequency domain involving temporal domain compression can outperform Rel-16 Type II codebook with

    - 18.3%-25.4% gain on rank 1 under full buffer traffic.

    - 23.3%-30.2% gain on rank 2 under full buffer traffic.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of mean UPT, AI/ML-based CSI compression on spatial-frequency domain involving temporal domain compression can outperform Rel-16 Type II codebook with

    - 7.7%-14.9% gain on rank 1 under FTP traffic and 80% RU.

    - 16.6%-28.6% gain on rank 2 under FTP traffic and 80% RU.

  - Huawei, Hisilicon: With the same overhead of CSI feedback and in terms of 5% UPT, AI/ML-based CSI compression on spatial-frequency domain involving temporal domain compression can outperform Rel-16 Type II codebook with

    - 10%-28.4% gain on rank 1 under FTP traffic and 80% RU.

    - 17%-39.2% gain on rank 2 under FTP traffic and 80% RU.

Joint/chained CSI prediction and CSI compression: Samsung, vivo

Solution description (from Samsung): Approach 1 gNB-side prediction (CSI decompression followed by prediction) and Approach 2 UE-side prediction (prediction followed by CSI compression)

- AI/ML backbone

  - Samsung: Bi-LSTM

- Findings on joint/chained CSI prediction and CSI compression

  - Samsung Adding the time-domain in the CSI compression domains, i.e., CSI compression in spatial-frequency-time domains, achieves a higher compression, i.e., further reduction in CSI report overhead, as compared to spatial-frequency domain CSI compression

  - Samsung AI-based CSI compression incurs a multiple order of increase in the computational complexity (measured in terms of number of FLOPs) as compared to CSI computation based on Rel-16 eType II codebook.

    - The increase in FLOPs is invariant with respect to the number of time-domain CSI measurements considered in the compression, i.e., same for spatial-frequency-domain and spatial-frequency-time-domain compression.

    - The model size remains in the same range for spatial-frequency-domain and temporal-spatial-frequency -domain compression

  - Samsung Significant gain is observed for UE-side joint CSI prediction and compression as compared to Rel-16 CB reporting without prediction.

    - Most of the performance gain is attributed to CSI prediction as compared to compression.

    - The prediction based on full channel matrices outperforms prediction based on eigenvectors.

  - Samsung gNB-side prediction performs well as UE-side prediction if the UE-side eigenvectors are available at the gNB side.

    - The UE-side eigenvectors can be compressed and reported with overhead that scales up with the reported rank

Other views

- AT&T: Finalize the EVM for the representative sub-use case to be considered for the CSI feedback enhancement

- NVIDIA: AI/ML based algorithms for CSI compression (e.g., using autoencoders) should be selected as a sub-use case for evaluation

### 3.1-2: AI/ML training methods

As per the agreement of the last meeting in 9.2.2.2., there are 3 training types to obtain/align the CSI generation part at UE and the CSI reconstruction part at the gNB during the training procedure. From the evaluation perspective, it is Moderator’s understanding that the method/solution for Type 2 and Type 3 may need to be clarified to better align the results; in addition, the price for Type 1/2/3 may need to be somehow evaluated as part of metrics and reported for better look into the pros/cons for each Type.

- FUTUREWEI: When considering various training types applicable for spatial-frequency domain CSI compression, companies are encouraged to share the performance and overhead related to how the proposed method handles scalability issue in real deployment scenario

- Huawei, HiSilicon: Companies are encouraged to report the following evaluation metrics for evaluation of the corresponding training types.

  - For Type1 (Joint training of the two-sided model at a single side/entity)

    - Companies to report the contents of model transfer (structure and/or parameters) and the overhead of model transfer

    - Companies to report the metric to evaluate inference compatibility between AI/ML model and UE, e.g., in terms of inference latency

  - For Type 2 (Joint training of the two-sided model at Network side and UE side, respectively)

    - Interaction approach and necessary information that is exchanged between Network side and UE side during the joint training, e.g., gradients, dataset, etc.

    - Whether the adopted AI/ML models for the Network part and the UE part are subject to the same or different structure(s).

    - Support of one common CSI reconstruction part at Network to multiple CSI generation parts of different UEs

    - Support of one common CSI generation part at UE to multiple CSI reconstruction parts of different Networks

    - Overhead of the exchanged information.

  - For Type 3 (Separate training at Network side and UE side, respectively),

    - Interaction approach and necessary information that is exchanged between Network side and UE side during the separate training, e.g., dataset.

    - Whether the adopted AI/ML models for the Network part and the UE part are subject to the same or different structure(s).

    - Support of one common CSI reconstruction part at Network to multiple CSI generation parts of different UEs

    - Support of one common CSI generation part at UE to multiple CSI reconstruction parts of different Networks

    - Overhead of the exchanged information.

- Lenovo: In order to define the model training characteristics and the relevant evaluation parameters, companies are encouraged to report the quadruple of (Lifecycle stage, Training type, Training entity, Dataset type) for the proposed AI/ML scheme, such as in Table 1.

Table 1 Training Stages of an AI/ML model.

| Lifecycle stage | Type 1 | Type 2 | Type 3 | Training entity | Dataset type: Simulated dataset, Offline Field dataset, Online (near-real time) dataset |
| --- | --- | --- | --- | --- | --- |
| Initial Training |  |  |  |  |  |
| Fine Tuning |  |  |  |  |  |
| Model Update |  |  |  |  |  |

- Lenovo: Based on the selected quadruple for the proposed AI/ML scheme (Lifecycle stage, Training type, Training entity, Dataset type), companies are encouraged to report related metrics

  - (examples from the text: Overhead of model transfer for the UE-part, Overhead of training data transmission/ground-truth CSI transmission, Latency of ground-truth CSI transmission, Complexity of model training on UE, Latency of model training, etc.)

- Descriptions/Findings for Type 1

- vivo: Study the performance and overhead of per-cell (region) model transfer in CSI compression

- MediaTek: Use the performance of matched/unmatched encoder-decoder trained via joint training strategy as the baseline for evaluation of other training strategies

- NTT DOCOMO: Overhead: Bytes of model, and Number of parameters

- NTT DOCOMO: Type 1 training procedure should be considered as the baseline considering the training overhead.

- China Telecom: The number of AI/ML parameters can be used as baseline, AI/ML model size is useful and may be used to consider the overhead of AI/ML model transmission

Findings

- NTT DOCOMO: Type 1 training procedure requires much less overhead than the others

- Descriptions/Findings for Type 2

- vivo: Information that should be exchanged between network and UE during joint training for training collaboration type 2 includes: 1) forward propagation results; 2) back propagation results; 3) training datasets; 4) other information such as learning rate and quantization/dequantization method.

vivo: the whole procedure contains three main steps: 1) UE computes the forward- propagation result on CSI generation model based on collected data, and sends the (last layer) forward-propagation result together with the input data to gNB; 2) gNB completes the remaining forward-propagation computation based on the received forward-propagation result, computes loss function based on the received data, and back propagates through CSI reconstruction part to acquire the gradients on the first layer of CSI reconstruction model. The backward propagation results, i.e., the gradients on the first layer of CSI reconstruction model, will be then sent back to the corresponding UE. UE completes the remaining back propagation procedure for CSI generation part based on the received gradients. 3) UE and gNB update CSI generation/reconstruction part based on the exchanged information. The above procedure will be repeated each batch, until the whole training procedure ends.

- MediaTek: If joint training is adopted, the least requirements of the training are disclosing gradient by gNB, disclosing latent vector by UE, and sharing a common dataset by both sides

- MediaTek: Report UE’s gain/loss and gNB’s gain/loss separately for unmatched encoder-decoder pairs in any training strategy

- NTT DOCOMO: Overhead: Bytes of gradients for FP, Bytes of gradients for BP, and Bytes of ground truth data

- Different structures between NW part and UE part (NW and UE are not aware of the AI/ML model of each other) Qualcomm, vivo, MediaTek,

  - Solution description: 2 cases are raised by companies in the evaluations

    - Case 1 (baseline): Same backbone/structure at NW ane UE

    - Case 2: Different backbones/structures between NW and UE, e.g., NW with Transformer and UE with CNN, or the other way around

  - MediaTek: Define a mechanism/threshold to identify and avoid certain vulnerable pairings of encoders and decoders

Findings

  - Qualcomm: Type 2 offline training of the UE-side model and NW-side model is feasible even if the ML model structure of the UE-side and NW-side models are not matched.

  - MediaTek: UE and gNB vendors equally suffer from the performance loss of the unmatched pairs in the joint training. In average for all unmatched pairs, UE losses 2.23% performance and gNB losses 2.26%

  - MediaTek: Overall, joint training on all pairs caused 1.68% performance loss

- One common CSI reconstruction part at NW to multiple CSI generation parts of different UEs vivo, Qualcomm, MediaTek

Solution description

  - vivo: Study the potential specification impacts of supporting one common CSI reconstruction part to multiple CSI generation parts of different UEs for training collaboration type2.

vivo: 1) the involved UEs compute their local forward-propagation results based on their local collected data (there could be some problems here, which will be discussed later.) and report them to gNB. The reported content contains not only the result of forward propagation but also the labels for loss function computation, which is paired with the forward-propagation results in reporting. 2) gNB computes the loss function as well as the gradients for back-propagation for each UE respectively and transmits the gradients to each UE. Note that different UEs’ gradients are computed based on their reported information, which are generally different for different UEs. 3) Each part of models completes the back-propagation procedure and updates the weights according to their gradients.

Findings

  - vivo: One common CSI reconstruction part could be trained to match multiple CSI generation parts of different UEs in training collaboration type 2 at the cost of some performance loss

  - vivo: Considering one common CSI reconstruction part matching three CSI generation parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduces from 0.075 to 0.052, i.e., losing about 30% performance gain.

  - Qualcomm: It is feasible to use Type 2 offline training to train a common NW-side model together with separate UE-side models without any performance impact when compared to training a separate NW-side model for each UE-side model

  - MediaTek: In overall, multi-encoder training strategy have inferior performance compared to joint and separate training strategies

  - MediaTek: In pair-to-pair comparison with joint training strategy, multi-encoder training strategy causes 0.7% performance loss

  - MediaTek: Employing multi-encoder training strategies, UEs and gNBs lose ~2.4% performance in average

- Multiple CSI reconstruction parts of different NWs to one common CSI generation parts at UE vivo, MediaTek

Solution description

  - vivo:.

1) UE computes the forward-propagation result based on the local data, which will be transmitted to all involved networks. 2) Involved networks compute the loss and the back-propagation results based on the reported information. Back-propagation results are then sent to UE. Note that the sent back-propagation results should be kept in the same order with forward-propagation results. 3) Finally, involved CSI reconstruction/generation parts update their own weights according to the computed gradients.

  - MediaTek: Assign higher priority to multi-decoder training compared to multi-encoder training strategy

Findings

  - vivo: One common CSI generation part could be trained to match multiple CSI reconstruction parts of different networks in training collaboration type 2 at the cost of some performance loss

  - vivo: Considering one common CSI generation part matching three CSI reconstruction parts, SGCS performance gain of AI/ML models over Rel-16 Type II codebook reduce from 0.075 to 0.061, i.e., losing about 19% performance gain.

  - MediaTek: In single-encoder multi-decoder training strategy, we observe 0.3% performance loss compared to the joint training. Also, degradation for both UE and gNB vendors are ~2.0%.

- Overhead calculation

  - vivo: Overhead in information exchange for training collaboration type 2 grows linearly with the number of iterations at training stage

    - Huawei, HiSilicon, vivo Overhead ≈ # of epoch*(forward-propagation information + back-propagation information + input data)

- Other views

  - MediaTek: In both multi-decoder and multi-encoder training strategies, matched encoder-decoder pairs may promote themselves at the cost of degrading the performance of unmatched encoder or decoder

  - MediaTek: Discuss how to calculate a joint loss to avoid adverse bias toward the matched pairs

- Descriptions/Findings for Type 3

- NTT DOCOMO: Overhead: Bytes of dataset, Bytes of encoder, and Bytes of decoder

The detailed methods for Type 3 include two directions

- vivo: For discussion purpose, define the interaction approaches for separate training via “active” and “passive” side, where “active” side actively shares the input/output data for model at “passive” side. The interaction approaches for separate training include two options: 1) UE is the active side and network is the passive side; 2) network is the active side and UE is the passive side.

- Direction 1: Sequential training starting with Network side training (NW-first training) Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT

Solution description

  - Huawei, HiSilicon: Further clarify the steps of performing training Type 3

    - Step1: Network trains the Network side CSI generation part (which is not used for inference) and the Network side CSI reconstruction part jointly

    - Step2: Network shares UE side with the dataset including the input (original CSI) and output (CSI feedback) of the Network side CSI generation part

    - Step3: UE side trains the UE side CSI generation part based on the dataset shared by Network

  - MediaTek: If gNB-first separate training is adopted, the least requirements of training are disclosing latent vector by gNB and sharing a common dataset by both sides

  - MediaTek: In the gNB-first separate training strategy, gNB should inform UE vendor at least about the type of its dropped encoder’s architecture

  - CATT: Option 1 (Separate encoder training): Training encoder A+ decoder A based on initial training dataset #A of {Channel}. Then based on encoder A, obtaining training dataset #B of {Channel, target CSI}, and training encoder B based on training dataset #B, with “Channel” as the input and “target CSI” as the output.

Findings

  - MediaTek: In gNB-first separate training, matched pairs not only do not experience performance loss, but they also reach a gain compared to joint training.

- Direction 2: Sequential training starting with UE side training (UE-first training) vivo, Qualcomm MediaTek, CATT

Solution description

vivo Step 1: The encoder is trained firstly at UE or a server at UE side using collected dataset0. Specifically, a complete model containing both encoder and decoder is trained and then the encoder is picked out for separate training. The decoder obtained in step 1 is termed as decoder0. Step 2: UE passes dataset1 into encoder to obtain the encoded feature1, and combines the dataset1 (encoder input) and encoded feature1 (encoder output) into the exchanging dataset, i.e., the encoder output serves as the label of encoder input. Step 3: UE transmits the exchanging dataset to gNB. Step 4: gNB utilizes the exchanging dataset to train the decoder via supervised learning. The decoder obtained in this step is termed as decoder1. Step 5: Test the SGCS of joint inference of encoder and decoder based on dataset2.

  - MediaTek: If UE-first separate training is adopted, the least requirements of the training are disclosing latent vector by UE and sharing a common dataset by both sides

  - MediaTek: In the UE-first separate training strategy, UE should inform gNB about the type of its architecture.

  - MediaTek: Give higher priority to UE-first separate training if separate training is adopted as the main training framework

  - CATT: Option 2 (Separate decoder training): Training encoder A+ decoder A based on initial training dataset #A of {Channel}. Then based on encoder A, obtaining training dataset #B of {Channel, target CSI}, and training decoder B based on training dataset #B, with “target CSI” as the input and “Channel” as the output.

Findings

  - MediaTek: UE-first separate training does not necessarily reach an inferior performance compared to joint training. Matched pairs experience 1.2% improvement

- Different structures between NW part and UE part (NW and UE are not aware of the AI/ML model of each other) Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT, CMCC

  - Solution description: 4 cases are raised by companies in the evaluations

    - Case 1 (baseline): Same backbone/hyperparameters/quantization method at NW ane UE Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT

    - Case 2: Same backbone and quantization method at NW and UE, but different hyperparameters (e.g., different number of layers) between NW and UE Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT

    - Case 3: Same quantization method at NW and UE, but different backbones between NW and UE (e.g., one side is Transformer, the other side is CNN/ResNet) Huawei, HiSilicon, vivo, Qualcomm, MediaTek, CATT

    - Case 4: Different backbones (and hyperparameters)/quantization methods between NW and UE vivo

  - Solution description-different dataset size: simulation results with dataset #B with only half size of dataset #A CATT, CMCC

Findings

  - Huawei, HiSilicon: For AI/ML-based CSI compression on spatial-frequency domain, there is only minor margin (<0.5%) between the SGCS of the separate training and the SGCS of the joint training even when the UE-side CSI generation part has a different structure or backbone with the Network side CSI generation part.

  - vivo: With the assumption that the model structure is aligned from the two sides, when the number of exchanged data samples is large enough (e.g., similar to the number of samples utilized in joint training), separate training could achieve near-joint training performance

  - vivo: If the model structure is not aligned (e.g., dequantization method at decoder and the quantization method in encoder could not match), there will be an obvious performance loss compared with that in case where the dequantization and quantization method are matching

  - Qualcomm: Type 3 offline training of the UE-side model and NW-side model is feasible and has a similar performance as compared to Type 1 training of the two-sided model.

  - MediaTek: UE-first separate training does not degrade the performance of unmatched pairs (w.r.t. joint training). It shows a negligible improvement of 0.93%

  - MediaTek: In UE-first separate training, UE and gNB both experience 0.7% performance loss compared to their matched pairs which are trained via joint training strategy

  - MediaTek: In the UE-first separate training strategy, unmatched decoders may struggle to leverage the latent features provided by a pre-trained TF-based encoder

  - MediaTek: In gNB-first separate training, UE and gNB respectively experience 1.52% and 1.42% performance loss compared to their matched designed trained via joint training strategy

  - MediaTek: Unlike UE-first separate training strategy, gNB-first separate training strategy degrades the performance of unmatched pairs

  - MediaTek: In the gNB-first separate training strategy, unmatched encoders may fail to replace the gNB’s TF-based encoder (cannot establish similar mapping from CSI to latent space). A significant degradation of -12.05% is observed in the performance for such pairings.

  - CATT: For separate training for AI/ML based CSI compression, compared to joint training

    - Performance loss is tiny when both the UE side and network side use transformer based AI/ML model;

    - Performance loss is obvious when one of the UE side and network side uses transformer based AI/ML model and the other one uses ResNet based AI/ML model.

  - CATT: For separate training, compared to dataset #B has the same size as dataset #A, minor performance loss can be seen for dataset #B has half size of dataset #A

  - CMCC: With large enough dataset samples at UE side, separate training could achieve similar SGCS as joint training

  - CMCC: When the number of dataset samples at UE side decreases, the SGCS of separate training will also decrease.

  - CMCC: When the generation part at UE side and the reconstruction part at network side have the same AI algorithms or model structures, to ensure separate training achieve similar SGCS as joint training, the requirement of number of dataset samples at UE side is much lower than the requirement when the AI algorithm or model structure is different between UE side and network side.

- One common CSI reconstruction part at NW to multiple CSI generation parts of different UEs Huawei, HiSilicon, vivo, Qualcomm,

  - Solution description: (For sequential training starting with UE side training) One gNB could collect paired input/output data from multiple UEs and then train one CSI reconstruction part based on a mixed dataset of all collected data. vivo, Qualcomm

Findings

  - Huawei, HiSilicon: One common CSI reconstruction part to multiple CSI generation parts of different UEs is naturally supported for the sequential training starting with Network side training, as the dataset generated by one Network side CSI reconstruction part can be delivered to multiple UEs to train multiple CSI generation parts independently.

  - vivo: One common CSI reconstruction/generation part could be trained to match multiple CSI reconstruction/generation parts of different UEs in training collaboration type 3 at the cost of some performance loss

  - vivo: Performance of one common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts of different networks/UEs is affected by the amount of exchanged data from each network/UE

  - Qualcomm Type 3 (separate) offline training of a shared NW-side decoder model for multiple UE-side encoder models is feasible and has a similar performance as compared to Type 1 training of a separate NW-side model for each UE-side model

- Multiple CSI reconstruction parts of different NWs to one common CSI generation part at UE Huawei, HiSilicon, vivo,

  - Solution description: (For sequential training starting with Network side training) UE uses a mix of two datasets generated by two different Networks to train a common CSI generation part Huawei, HiSilicon

Findings

  - vivo: For sequential training starting with UE side training, it is trivial to realize one common CSI generation part to multiple CSI reconstruction parts of different networks, since it is natural for UEs to broadcast the input/output of the same CSI generation part to multiple gNBs

  - Huawei, HiSilicon: For AI/ML-based CSI compression on spatial-frequency domain, there is only minor margin (<0.5%) between the SGCS of the separate training and the SGCS of the joint training when the separate training considers multiple CSI reconstruction parts of different Networks to one common CSI generation part at UE.

- Other findings related w.r.t. the combinatorial issue

  - vivo: Performance loss in supporting common CSI generation/reconstruction part to multiple CSI reconstruction/generation parts gets worse as the number of supported UEs/networks increases

- Alignment on vector quantization/dequantization

Solution description Qualcomm

  - Approach 1: UE server trains the encoder without quantization and shares the dataset (z, Vtarget); Vector quantizer (VQ) is trained with the decoder at NW-side training entity

  - Approach 2: UE server trains the encoder with quantization;

Findings

  - Qualcomm: Training UE encoder without quantization and generating the separate training based on this encoder may lead to some performance degradation compared to encoder training with quantization

  - Qualcomm: Separate training with VQ for multiple vendors achieves almost the same performance as Type 1 training

### 3.1-3: AI/ML model settings

AI/ML model settings for rank>1

Some companies have discussed the issue on how to set up AI/ML models for multiple ranks situation. Some companies analyzed detailed methods, which can be summarized as follows:

- Option1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference. Huawei, Hisilicon, CATT,

  - MediaTek: rank-specific designs are not suited for UE vendors as stacking layers, to process them jointly, will drastically increases AI/ML models’ complexity.

  - CATT: For AI/ML based CSI feedback, the overheads of CSI feedback for rank 3 and rank 4 are expected to be comparable to rank 2. Therefore layer specific AI/ML model (i.e., Alt 2) or rank specific AI/ML model (i.e. Alt 3) should be considered.

- Option2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference. Huawei, Hisilicon, Ericsson, [Lenovo]

  - Ericsson: Layer specific and rank common approach (for rank group 1-2 and 3-4 respectively) used in the evaluations. AE1/2 for layer 1/2 of rank1&2, AE3/4/5/6 for layer 1/2/3/4 of rank 3&4

  - Lenovo: [Scheme 2 Joint encoding?] Use training dataset to train a two-part model, M, which is optimized to transmit the first two largest eigenvector of the channel matrix at the same time

- Option3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference. CATT, Xiaomi, ZTE, Ericsson, Lenovo

  - MediaTek: Given the possible number of scenarios and configurations, it is infeasible to train and deploy a dedicated AI/ML model for each. In fact, it is the objective of generalization efforts to avoid dedicated designs like layer-specific and rank-specific AI/ML models.

  - CATT: For AI/ML based CSI feedback, the overheads of CSI feedback for rank 3 and rank 4 are expected to be comparable to rank 2. Therefore layer specific AI/ML model (i.e., Alt 2) or rank specific AI/ML model (i.e. Alt 3) should be considered.

  - Ericsson: Since the AE is layer specific it, it is also trained per layer.

  - Lenovo: [Scheme 1 Separate encoding?] since we wanted to compare the performance with scheme 2, we have selected scheme 1 as an upper bound of the separate encoding scheme.

- Option4 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference. Huawei, Hisilicon, vivo, Samsung, MediaTek, Xiaomi, Apple, ZTE, Lenovo

Solution

  - Huawei, Hisilicon: For the evaluation of the AI/ML-based CSI compression sub use cases with rank >=1, companies are encouraged to report the specific option adopted for AI/ML model settings to adapt to ranks/layers. Option 4 is adopted in our evaluation

  - MediaTek: Between layer-specific and layer-common AI/ML models settings for rank>1, down select layer-common AI/ML models.

  - ZTE: For rank>1, two cases on model input/output can be considered for intermediate KPIs and eventual performance evaluation as a starting point.

    - Case 1: Single layer in model input and single layer in model output

    - Case 2: Multiple layers in model input and multiple layers in model output

  - CATT: For each layer-common AI/ML model, the AI/ML model is trained for a specific payload and port with mixed eigenvectors from all 4 layers

  - Lenovo: [Scheme 1 Separate encoding?] Especially, for scheme 1, it is desirable two train one single model that works for both the first and the second eigen vector.

  - Lenovo: Performance of the Joint and Separate embedding methods should be evaluated

Findings

  - vivo: Rank generalization with per-layer model can achieve similar SGCS with half model size compared with per-rank model

  - Samsung: AI model generalizes well from layer 1 to layer 2 thus a layer-common model can be used

  - MediaTek: Layer-common AI/ML model respectively achieves 1.18% and 1.55% SGCS gain for layer 0 and layer 1 of rank-2 channels compared to layer-specific AI/ML models.

  - ZTE: Case 1(single-layer model input and single-layer model output) can achieve better performance than Case 2 (multi-layer model input and multi-layer model output).

  - Xiaomi: the generalized AI model (Option4) has even better performance than the baseline (Option3)

AI/ML models adopted in evaluations

- Huawei, Hisilicon: Transformer, LSTM+Transformer, CNN,

- vivo: Transformer, CNN, MLP

- Ericsson: ResNet

- Qualcomm: Transformer, CNN

- Samsung: vision transformer (ViT), CNN

- Nokia: ResNet

- OPPO: EVCsiNet-T

- Xiaomi: Transformer

- Apple: CNN

- CATT: Transformer, ResNet

- Intel: ACRNet

- ZTE: Transformer

- CMCC: EVCsiNet,Transformer

- FUTUREWEI: CNN

- ETRI: Modified CsiNet, Transformer

- InterDigital: CSI-Net

- Spreadtrum: Transformer

- Mavenir: CSI-Net

- Fujitsu: CNN

- NVIDIA: CNN

- China Telecom: CNN

- Fraunhofer: CsiNet

Findings on capability-related KPI

- Samsung: The number of FLOPs to perform the AE operations is much larger than eType II

Input/output CSI format

Option 1: Raw channel matrix. [Ericsson], FUTUREWEI, NVIDIA, InterDigital, Fraunhofer

- NVIDIA: Both autoencoders with raw channel matrix as input and autoencoders with eigenvector(s) of raw channel matrix as input should be evaluated

Option 2: Eigenvector. Huawei, Hisilicon, CATT, vivo, Samsung, Nokia, MediaTek, OPPO, Xiaomi, Apple, ZTE, CMCC, NVIDIA, ETRI, Google, Mavenir, China Telecom, Spreadtrum

- Huawei, Hisilicon: To align with legacy CSI codebook, eigenvector is preferred as the input CSI and the target CSI applied for training/inference under AI/ML-based CSI feedback.

- Google: Study the input of CSI compression based on the eigenvectors of the raw channel with a wideband precoder selected as SD basis, e.g. HW1.

Option 2A: Legacy-like PMI (e.g., Type I-like, Type II-like CSI). Intel, vivo

- Intel: The eigenvectors are then transformed to the angular-delay domain

- vivo: Same as eType II, precoders of each subband can be transformed into angle-delay domain with selected beam and delay.

Quantization method for the output of the encoder:

- Huawei, Hisilicon, Xiaomi, ZTE, FUTUREWEI, Spreadtrum: vector quantization

- Samsung, CATT, Nokia, NTT DOCOMO, Intel, CMCC, InterDigital: uniform (scalar) quantization/individual values quantization

  - Intel: A uniform B-Bit quantizer is used in the AE. The FC layer outputs K channels where K=⌊2NSNT/β⌋. The quantizer uniformly quantizes each of the K inputs to produce KB feedback bits.

  - InterDigital: The encoder output is passed through a tanh layer to restrict the range of the encoder output for uniform quantization. The quantization operation is included during the training so that the encoder and decoder can learn appropriate weights while taking into account the quantization impact

Pre-processing/Post-processing

- Pre-processing to angular-delay domain as model input

  - vivo: Pre-processing with delay domain compression can solve the generalization of bandwidth and subband number.

  - MediaTek: The data samples in a sparse domain (e.g., delay-beam domain) lack a strong autocorrelation but still can be effectively compressed

  - Intel: The SLS channel H is generated in the space-frequency domain and is converted to an input matrix V of size 2×NS×NT in the angular-delay domain.

### 3.1-4: Other EVM for CSI compression

CSI payload alignment

- Huawei, Hisilicon: For the performance comparison between AI/ML-based CSI compression and legacy TypeII codebook, use tables in section 6.3.2.1.2 of TS 38.212 to calculate CSI payload.

- Huawei, Hisilicon: For the evaluation of AI/ML-based CSI compression and legacy TypeII codebook, following two options can be considered for further down-selection:

  - Option 1: Use the CSI payload of the maximum rank as the final CSI payload.

  - Option 2: Use the weighted average CSI payload over ranks considering rank distribution as the final CSI payload

- Qualcomm: For the evaluation of the AI/ML based CSI compression sub use case, the CSI feedback overhead should be computed based on the rank indicated by the UE

  - Option 1: Calculate the payload size assuming the maximum possible rank.

  - Option 2: Calculate the payload size assuming the rank indicated by the UE.

- Apple: RAN1 consider to calibration type II codebook SGCS, to properly evaluate the gain of AI based approach.

- Apple: To achieve fair comparison with Type II CSI feedback, companies should align the maximum feedback overhead for Type II, the maximum number of non-zero coefficients for Type II. Companies should report the average number of non-zero coefficients for Type II, rank distribution of PDSCHs for both Type II and autoencoder

  - the average Type II feedback overhead (in bits) at configuration c is given by  r=14Bc,r-Nc,r-M(c,r⋅7Rc,r, where B(c,r)  is the maximum feedback overhead (in bits) of Type II for configuration c and rank r, c=1,2,...,8, r=1,2,3,4, N(c,r) is the maximum number of non-zero coefficients, Mc,r,r=1,2,3,4 is the average number of actual non-zero coefficients at different ranks at the chosen Type II configuration, R(c,r), r=1, 2, 3, 4 is the rank distribution of PDSCHs at a given offered load.

  - The AI/ML rank distribution can be also reported, so the average feedback overhead is r=14ArR'r where A(r) is the number of feedback bits at a given rank for an autoencoder, and R'(r) is the rank distribution at a given offered load

- CAICT: CSI payload calculation for R16 type II should be aligned between different companies and CSI payload for different AI model schemes could be reported by different companies

Template for simulation results collection

- Huawei, Hisilicon: The following initial template is considered for companies to report the evaluation results.

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
| Common description | Input type |  |  |
|  | Output type |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset size | Train/k |  |  |
|  | Test/k |  |  |
| Gain for intermediate KPIs | SGCS |  |  |
|  | NMSE |  |  |
|  | [Others] |  |  |
| Gain for eventual KPI | Mean UPT |  |  |
|  | 5% UPT |  |  |

- FUTUREWEI: When reporting AI/ML model generalization evaluation results for CSI feedback enhancements, companies are encouraged to align the reporting attributes and format as depicted in Table 2.2-1.

Table 2.2-1: Performance evaluation report attributes

| Training type | Model input type | Model output type | CSI payloads | Training scenario / config. | Testing scenario/ config. | Dataset size |  | Perf. KPIs | Other overhead | Mechanism applied |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Train | Test |  |  |  |

CSI data construction in evaluations

To have an accurate test dataset, we should make sure that for a given UE not only the test samples but also the samples close in time with the test samples are not present in the training data. This issue has been discussed in the last meeting, and some companies raised it for this meeting.

- ZTE: For CSI dataset construction, training dataset and inference dataset should be separated in different drops at least for calibration purpose

- Fraunhofer: Further studies on the link-level based on block error rate (BLER) and system-level are required for comparison between AI-based schemes and Type II CSI. Offline overhead and coordination between the UE and gNB for the parameters of the encoder and the decoder should be studied as well

1st round email discussions

### 3.2-1: AI/ML training methods

In the last meeting, 3 training collaborations have been agreed. Companies provided the solutions to achieve Type 2/3, and the assumptions on how to perform evaluation under the realistic network, e.g., different model structures between NW and UE, how to achieve one NW to multiple UEs, and how to achieve one UE to multiple NWs, etc.

Therefore, the issues in this section is to be discussed to study/evaluate the feasibility and performance of training Type 2/3 in the realistic network.

- Issue#3-1 (High priority) Definition for Type 2 training procedure

To align/clarify the basic procedure for Type 2 training, the following question is raised. 4 companies evaluated the performance of Type 2, and 3 companies discussed the overhead report of the Type 2 training method.

Question 3.2.1: For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered:

- For each FP/BP loop,

  - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side

  - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side

  - Step 3: UE side trains the CSI generation part based on the BP information from NW side

- Note: the dataset between UE side and NW side is aligned, FFS overhead report.

| Support/Can accept | NTT DOCOMO, NVIDIA, CAICT, MediaTek, OPPO, vivo, CATT, LG, CMCC, Lenovo (comment) |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Intel | It is not clear for us what is the additional information from this agreement we have for the simulations. If we want to compare Type 1 and Type 2 then the performance will be the same if quantization of FP/BP info is the same. |
| Ericsson | Agree with Intel, its questionable what is new here? What is precluded? |
| MediaTek | In the evaluation of type 2, we usually assume the proprietary solution. Thus, UE doesn’t know NW part model and NW doesn’t know UE part model. It is different from the evaluation for type 1. |
| Panasonic | If the companies already have common understanding on the procedure of Type 2 training, the agreement would not be necessary. If not, we think clarification is beneficial. |
| CATT | Seems like a conclusion to confirm the understanding. |
| CMCC | It is helpful for us to have common understanding on Type 2 training. |
| Lenovo | It is worthy to elaborate on the definition of the UE side and the gNB side. We suggest adding a note as: Note: The UE side can be a single UE or a node representing one (a few) UEs. The gNB side can be a single gNB or a node representing one (a few) gNBs |
| Qualcomm | The need for such an agreement is not clear. |
| ZTE | Agree with Intel. |

- Issue#3-2 (High priority) Multi-vendor training for Type 2 training

To NW vendors, it is desirable to support one NW part to Multi-UE parts rather than training tremendous models based on a combinatorial manner. To UE vendors, it is also desirable to support one UE part to Multi-NW parts. Therefore, it may be beneficial to evaluate whether/how much is the potential performance degradation compared to the ideal case of one NW part to one UE part. 3 companies have provided the evaluations for one NW to Multi-UE or one UE to Multi-NW, while it is still unclear whether/how to achieve Multi-NW to Multi-UE, so a FFS is added by Moderator.

Therefore, the following question is raised to evaluate the performance of multi-vendors.

Question 3.2.2: For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), the following evaluation cases are considered for multi-vendors,

- Case 1 (baseline): Type 2 training between one NW part model to one UE part model

- Case 2: Type 2 training between one NW part model and M>1 separate UE part models

  - FFS different backbones/structures between the UE part model and the NW part model

  - FFS different backbones/structures over the M UE part models

- Case 3: Type 2 training between one UE part model and N>1 separate NW part models

  - FFS different backbones/structures between the UE part model and the NW part model

  - FFS different backbones/structures over the N NW part models

- FFS N NW part models to M UE part models

| Support/Can accept | NTT DOCOMO, NVIDIA, CAICT, MediaTek, Panasonic, OPPO, vivo , CATT, LG, CMCC, Lenovo (with comment), Qualcomm (comment below), Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Companies are also welcome to share your views on the FFS part. |
| Apple | It seems most evaluation so far focus on target/output CSI is eigenvectors, layer specific. Since there are different types of input/output such as channel versus eigenvector, pre/post processing, rank-specific model or rank-common model, layer specific versus layer common, how one NW/UE model is trained for multiple UE/NW models? |
| OPPO | We agree to study, but with some concerns required to be clarified. Firstly, the definition of backbone and structure are not clear. Secondly, from our understanding, the AI/ML model structure for UE part model and NW part model must be different since both their input and output are different. Therefore, we think the expression of Case 2 and Case 3, including the FFS parts are not appropriate. And if M UE part models are different, there must be some UE part models are different from NW side. |
| Lenovo | In case2, it is important to determine if the UE parts are different in statistics or not. Due to distributed setting, this will result in different gains. So, we suggest adding: Case2: FFS: Effect of different/same statistics between datasets of different UE sides We also suggest adding a note as: Note: The UE side can be a single UE or a node representing one (a few) UEs. The gNB side can be a single gNB or a node representing one (a few) gNBs Note: By statistics of the dataset, we mean, if they are from the same Scenario#X/Configuration#X and maybe more accurately if they are all indoor UEs or some outdoor and some indoor UEs. |
| Qualcomm | Regarding the FFS in Case 2 and Case 3, we do not see the need to discuss model structure choices as part of evaluation cases. Model details should be left to companies. |
| Samsung | Agree with Apple. We would like to capture whether this training strategy requires alignment in the input/output domain. Moreover, companies may have to report which of the following options they considered. - Option 1: The training in Case 2 or Case 3 is performed in a single training session for all UE and NW parts, e.g., model updates for UE-NW pairs are interlaced per each training dataset batch. - Option 2: The training in Case 2 or Case 3 is performed in separate training sessions per UE-NW pair, i.e., UE-NW pairs are trained sequentially. Option 1 and Option 2 have different requirements. Option 1 requires training of M+1 or N+1 parts in a single training session. |

- Issue#3-3 (High priority) Definition for Type 3 training procedure-NW-first

5 companies have discussed/evaluated the NW-first Type 3 training (i.e., sequential training starting with NW side training), and 3 companies discussed the overhead report of the Type 3 training method. To align the basic procedure for NW-first Type 3 training, the following question is raised. For the FFS part, there may be two approaches on whether the shared CSI feedback is the output after quantization or the output before quantization.

Question 3.2.3: For the evaluation of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training:

- Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly based on dataset#1 (original CSI)

- Step2: After NW side training is finished, NW side shares UE side with the dataset#2 including the input (same original CSI of dataset#1) and output (CSI feedback) of the NW side CSI generation part

  - FFS whether the output (CSI feedback) is the input or output of the quantization

- Step3: UE side trains the UE side CSI generation part based on the dataset#2, using the CSI feedback in dataset#2 as labels for the original CSI

- FFS overhead report

| Support/Can accept | NTT DOCOMO, NVIDIA, Futurewei, CAICT, MediaTek, OPPO, vivo , CATT, CMCC |
| --- | --- |
| Object/Concern | Lenovo, Samsung (see comment) |

| Company | View |
| --- | --- |
| Intel | Do we really need to agree on the specific training algorithm for evaluations? We are open to discuss training implementations for the TR. For evaluations we can simply agree to disclose the training algorithm assumed for Type 3 for each company. |
| Futurewei | We think the procedures specified in the proposal can be an option. There may be other options feasible. Thus, we suggest rewording the sentence before the steps to: “For the evaluation of Type 3 (Separate training at NW side and UE side), the following procedure is considered as an option for the sequential training starting with NW side training:” |
| Panasonic | We are fine with Futurewei’s update. |
| OPPO | Agree with Futurewei’s update. |
| ETRI | We share a same view with Intel. Companies can provide details of the training. |
| LG | Agree with Intel and ETRI. |
| Lenovo | We are generally okay with the proposal but the second step can be more general for now as there could be different ideas. For example, to reduce overhead the gNB side may decide not to send the “input” back to the UE and just report the desired output for the samples that have been already sent to the gNB in dataset1. So, we suggest: - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly based on dataset#1 (original CSI) Step2: After NW side training is finished, NW side shares UE side with a set of information that is used by the UE to be able to train the CSI generation part. - the dataset#2 including the input (same original CSI of dataset#1) and output (CSI feedback) of the NW side CSI generation part - FFS whether the output (CSI feedback) is the input or output of the quantization - Step3: UE side trains the UE side CSI generation part based on the received information. dataset#2, using the CSI feedback in dataset#2 as labels for the original CSI We also suggest adding a note as: Note: The UE side can be a single UE or a node representing one (a few) UEs. The gNB side can be a single gNB or a node representing one (a few) gNBs |
| Qualcomm | Not sure why we need this proposal. Companies can report the procedure used during evaluation. In general, there may be many variations, and aligning the details is not required and may consume a lot of time. |
| Fujitsu | We share a similar view to that of Intel. We think the current definition of type 3 training collaboration in the (RAN#110) agreement is enough for evaluation purposes, and there is no need to restrict the training algorithm to be the one written in Question 3.2.3. |
| ZTE | Generally agree with Futurewei’s update, and we suggest rewording as “For the evaluation of Type 3 (Separate training at NW side and UE side), the following procedure is considered as an examplary procedure for the sequential training starting with NW side training:” |
| Samsung | Similar comment as FUTUREWEI. Separate training can be realized by sharing/common reference model. We can name this as Type 3-1 to make it specific. |

- Issue#3-4 (High priority) Definition for Type 3 training procedure-UE-first

4 companies have discussed/evaluated the UE-first Type 3 training (i.e., sequential training starting with UE side training), and 3 companies discussed the overhead report of the Type 3 training method. To align the basic procedure for UE-first Type 3 training, the following question is raised.

Question 3.2.4: For the evaluation of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training:

- Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly based on dataset#1’ (original CSI)

- Step2: After UE side training is finished, UE side shares NW side with the dataset#2’ including the input (CSI feedback) and label (same original CSI as dataset#1’) of the UE side CSI reconstruction part

  - FFS whether the input (CSI feedback) is the input or output of the dequantization

- Step3: NW side trains the NW side CSI reconstruction part based on the dataset#2’, using the original CSI in dataset#2’ as labels for the CSI feedback

- FFS overhead report

| Support/Can accept | NTT DOCOMO, NVIDIA, Futurewei, CAICT, MediaTek, OPPO, vivo, CATT, CMCC |
| --- | --- |
| Object/Concern | Lenovo |

| Company | View |
| --- | --- |
| Intel | Same comment as for Question 3.2.3. |
| Futurewei | Like our comments for the NW side first training procedure specified in Question 3.2.3, the procedures specified in the proposal can be an option. There may be other options feasible. Thus, we suggest rewording the sentence before the steps to: “For the evaluation of Type 3 (Separate training at NW side and UE side), the following procedure is considered as an option for the sequential training starting with UE side training:” The descriptions for Step 2 and Step 3 are a bit not clear, and we suggest modifying the wording to: - Step2: After UE side training is finished, UE side shares NW side with the dataset#2’ including the output of the UE side CSI generation part input (CSI feedback) and label (same original CSI as dataset#1’) of the UE side CSI reconstruction part - FFS whether the input (CSI feedback) is the input or output of the dequantization - Step3: NW side trains the NW side CSI reconstruction part based on the dataset#2’, using the output of the UE side CSI generation part in dataset#2’ as the input and the original CSI in dataset#2’ as labels for the CSI feedback |
| Panasonic | We are fine with Futurewei’s update. |
| OPPO | Agree with Futurewei’s update. |
| ETRI | Same comment as for Issue #3-3 (Question 3.2.3). |
| LG | Same comment as for Question 3.2.3. Companies can provide details of the training. |
| CMCC | Support in principle. But we are not sure the “label” in dataset#2’ should be the same original CSI as dataset#1’ or just the output of UE side CSI reconstruction part. |
| Lenovo | It is essentially the same comment that we had for the previous case. So we suggest the following update: - Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly based on dataset#1’ (original CSI) - Step2: After UE side training is finished, UE side shares NW side with a set of information that is used by the UE to be able to train the CSI reconstruction part. the dataset#2’ including the input (CSI feedback) and label (same original CSI as dataset#1’) of the UE side CSI reconstruction part - FFS whether the input (CSI feedback) is the input or output of the dequantization - Step3: NW side trains the NW side CSI reconstruction part based on the on the received information. dataset#2’, using the original CSI in dataset#2’ as labels for the CSI feedback We also suggest adding a note as: Note: The UE side can be a single UE or a node representing one (a few) UEs. The gNB side can be a single gNB or a node representing one (a few) gNBs |
| Qualcomm | Not sure why this proposal is needed. |
| Fujitsu | We have the same comment as for Question 3.2.3. |
| ZTE | Like our comments on Question 3.2.3, we suggest rewording as “For the evaluation of Type 3 (Separate training at NW side and UE side), the following procedure is considered as an expmplary procedure for the sequential training starting with UE side training:” |
| Samsung | Similar comment as FUTUREWEI. Separate training can be realized by sharing/common reference model. We can name this as Type 3-1 to make it specific. |

- Issue#3-5 (High priority) Different model structures between NW to and UE for Type 3

As the spirit of the separate training is to reserve the proprietary of the AI/ML model, it is then desirable to evaluate the realistic case where NW side and UE side do not have aligned AI/ML structures. 6 companies have discussed/evaluated the Type 3 training with different structures between NW and UE. 2 companies have evaluated different dataset sizes between NW and UE. In addition, one company raised to consider different quantization/dequantization methods between NW and UE, but it is Moderator’s understanding that the quantization/dequantization between NW and UE has to be anyhow aligned for the inference purpose, so this assumption is put to FFS.

Question 3.2.5: For the evaluation of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations:

- Case 1 (baseline): Fully aligned AI/ML model backbone (e.g., CNN, Transformer, etc.) and structure (e.g., number of layers) between NW and UE

- Case 2: Same backbone but different structures between NW and UE

- Case 3: Different backbones between NW and UE

- FFS different dataset sizes between NW and UE

- FFS different quantization/dequantization methods between NW and UE

| Support/Can accept | NTT DOCOMO, NVIDIA, CAICT, MediaTek, Panasonic, OPPO, vivo, CATT, CMCC, Lenovo(with comment), Fujitsu, ZTE, Samsung |
| --- | --- |
| Object/Concern | Qualcomm |

| Company | View |
| --- | --- |
| Moderator | Companies are also welcome to share your views on the FFS part. |
| Futurewei | Depending on UE’s capability and capacity, the AI/ML or NN model complexity may be very different and there may be many architecture variations. We suggest adding the following: FFS: number of different AI/ML model architectures on UE and/or NW side to be studied |
| Apple | Same clarification as 3.2.2. It seems most evaluation so far focus on target/output CSI is eigenvectors. Since there are different types of input/output such as channel versus eigenvector, pre/post processing, rank-specific model or rank-common model, layer specific versus layer common, how one NW/UE model is trained for multiple UE/NW models? Are we aligning target CSI first? |
| OPPO | We agree to study, but from our understanding, Case 1 as the baseline is not a good choice, since the AI/ML model structure consists of many details besides the number of layers, and why we have to have fully aligned AI/ML model structure between NW and UE? |
| Lenovo | We are okay with the mentioned cases. However, one important case that should be investigated in Type3 is on the performance of the trained model when the statistics of the UE sides’ datasets are different. We note that, in practice, it is very possible to have UEs with different channel statistics So suggest to add these cases as well: Case 4: Same statistics between datasets of different UE sides Case 5: Different statistics between datasets of different UE sides Note: By statistics of the dataset, we mean, if they are from the same Scenario#X/Configuration#X and maybe more accurately if they are all indoor UEs or some outdoor and some indoor UEs. |
| Qualcomm | The proposal could be simplified to two cases – matched structure and mismatched structure. We do not see the need for finer categorization of the cases. |
| Fujitsu | We support this proposal in general. We suggest a new FFS regarding the pre-processing and post-processing: For the evaluation of Type 3 (Separate training at NW side and UE side), the following cases are considered for evaluations: - Case 1 (baseline): Fully aligned AI/ML model backbone (e.g., CNN, Transformer, etc.) and structure (e.g., number of layers) between NW and UE - Case 2: Same backbone but different structures between NW and UE - Case 3: Different backbones between NW and UE - FFS different sizes of datasets sizes exchanged between NW and UE - FFS different quantization/dequantization methods between NW and UE - FFS: pre-processing and post-processing methods between NW and UE. |
| Samsung | Agree with Apple. We can name this as Type 3-1. |

- Issue#3-6 (High priority) One NW to Multi-UEs for Type 3 training

Similar to the analysis for Type 2, it is desirable to support one NW part to Multi-UE parts, and evaluate whether/how much is the potential performance degradation compared to the ideal case of one NW part to one UE part. 3 companies have discussed/evaluated the one NW part to Multi-UE parts. The assumptions for evaluation are then raised in the following question.

Question 3.2.6: For the evaluation of Type 3 (Separate training at NW side and UE side) with the sequential training starting with UE side training, the following evaluation cases are considered for one NW to Multi-UEs

- Case 1 (baseline): Type 3 training between one NW part model to one UE part model

- Case 2: Type 3 training between one NW part model and M>1 separate UE part models

  - FFS different backbones/structures between UE part model and the NW part model

  - FFS different backbones/structures over the M UE part models

- Note: the sequential training starting with NW side training can naturally support one NW to Multi-UEs

| Support/Can accept | NTT DOCOMO, NVIDIA, CAICT, MediaTek, Panasonic, OPPO, vivo, CATT, CMCC, Lenovo(with comment), Qualcomm (comment below), Fujitsu, ZTE, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | Same view as above questions. |
| Lenovo | As we explained in response to the previous question, it is important to investigate the performance when the statistics of the UE sides’ datasets are different. So, we suggest adding: Case2: FFS: Effect of different/same statistics between datasets of different UE sides Note: By statistics of the dataset, we mean, if they are from the same Scenario#X/Configuration#X and maybe more accurately if they are all indoor UEs or some outdoor and some indoor UEs. |
| Qualcomm | Regarding the FFS in Case 2, we do not see the need to discuss model structure choices as part of evaluation cases. Model details should be left to companies. |
| Samsung | Similar to the above, we can name this as Type 3-1. |

- Issue#3-7 (High priority) One UE to Multi-NWs for Type 3 training

Similarly, the evaluation on the feasibility of one UE to Multi-NWs are also beneficial. 2 companies have discussed/evaluated the one UE part to Multi-NW parts. The assumptions for evaluation are then raised in the following question.

Question 3.2.7: For the evaluation of Type 3 (Separate training at NW side and UE side) with the sequential training starting with NW side training, the following evaluation cases are considered for one UE to Multi-NWs

- Case 1 (baseline): Type 3 training between one NW part model to one UE part model

- Case 2: Type 3 training between one UE part model and N>1 separate NW part models

  - FFS different backbones/structures between UE part model and the NW part model

  - FFS different backbones/structures over the N NW part models

- Note: the sequential training starting with UE side training can naturally support one UE to Multi-NWs

| Support/Can accept | NTT DOCOMO, NVIDIA, CAICT, MediaTek, Panasonic, OPPO, vivo, CATT, CMCC, Lenovo(with comment), Qualcomm (comment below), Fujitsu, ZTE, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | Same view as above questions. |
| Lenovo | As we explained in response Question 3.2.5, it is important to investigate the performance when the statistics of the UE sides’ datasets are different. So, we suggest adding: Case2: FFS: Effect of different/same statistics between datasets of different UE sides Note that the question may seem that we have only one UE, however, since we have many gNBs there should be other UEs that are also connected to these gNBs. Note: By statistics of the dataset, we mean, if they are from the same Scenario#X/Configuration#X and maybe more accurately if they are all indoor UEs or some outdoor and some indoor UEs. |
| Qualcomm | Regarding the FFS in Case 2, we do not see the need to discuss model structure choices as part of evaluation cases. Model details should be left to companies. |
| Samsung | Similar to the above, we can name this as Type 3-1. |

### 3.2-2: AI/ML model settings

- Issue#3-8 (Medium) AI/ML model settings for rank>1

In this meeting, more companies have provided views on the issue on how to set up AI/ML models for multiple ranks situation. The candidates include 4 options: rank specific, rank common, layer specific, and layer common. For each option, there are some inputs, so the intention is not to do the down selection at this meeting, but to encourage companies to report the AI/ML solution for better understanding of other companies.

In addition, for the generalization/scalability analysis, some companies mentioned Option 4 can naturally achieve the scalability over rank values without additional pre-processing of the model input. For other options, proponents may provide whether/how to perform the generalization/scalability over rank values.

Proposal 3.2.1: For the evaluation of the AI/ML based CSI compression sub use cases with rank >=1, companies are encouraged to report the specific option adopted for AI/ML model settings to adapt to ranks/layers, and whether additional processing is needed to achieve generalization/scalability over rank numbers for the reported option.

- Option1 (rank specific): Separated AI/ML models are trained per rank value and applied for corresponding ranks to perform individual inference.

- Option2 (rank common): A unified AI/ML model is trained and applied for adaptive ranks to perform inference.

- Option3 (layer specific): Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference.

- Option4 (layer common): A unified AI/ML model is trained and applied for each layer to perform individual inference.

- Other options not precluded.

- FFS further down selection for the above options

| Support/Can accept | NTT DOCOMO, NVIDIA, Futurewei, CAICT, MediaTek, Panasonic,OPPO, Xiaomi, vivo, CATT, Lenovo, Fujitsu, Samsung |
| --- | --- |
| Object/Concern | Ericsson |

| Company | View |
| --- | --- |
| Ericsson | This is not aligned with the options discussed in agenda 9.2.2.2 |
| MediaTek | Our preference is to have a unified AI/ML model for all layers (Option 4). |
| OPPO | From our perspective, Option 1, Option 3 and Option 4 can be considered. The Option 2 with rank common AI/ML model seems difficult to realize when considering the scalability problem. For Option 1 and Option 3, multiple AI/ML model should be saved at UE/gNB side. For Option 4 with layer common AI/ML model, only one AI/ML model is required, and the performance is near to the Option 3 with layer specific AI/ML model by training on mixing datasets including multiple layers without additional pre-processing to achieve generalization/scalability issues. |
| Xiaomi | We support this proposal and prefer Option 4. |
| CATT | Although we are fine with such categorization here, it is noted that 6 alternatives is proposed in 9.2.2.2. It will be better if we can align the categorization first between these two agendas. |
| LG | Ok with the proposal and Option 4 is preferred. |
| Qualcomm | Not sure why this proposal is needed. How to support rank>1 is implementation issue, it is up to companies to report or not report what they do. |
| ZTE | From our understanding, option1, option 3 and option 4 can be considered, while we think option 2 may be difficult to realize due to the adaption to all ranks. For option 4 with layer common, AI/ML model performance is near to option 3 and it can reduce the model parameters. In addition, we are not clear about why this sentence whether additional processing is needed to achieve generalization/scalability over rank numbers for the reported option is added here, since we think it may be discussed for generalization. |
| Samsung | Ok |

- Issue#3-9 (High priority) Type of input/output for AI/ML model

From the inputs for this meeting, 5 companies shows interests on taking raw channel matrix as inputs, while 17 companies prefer to consider eigenvector as the input of the model. 2 companies discussed some further pre-processing, such as angular-delay domain converting to the eigenvectors. It seems both raw channel matrix and eigenvectors (and its further converting) are workable for the evaluations, and there is no other input types raised by companies, so the following proposal is provided. For the pros/cons of the two input types, companies can cross-check based on evaluations results.

Proposal 3.2.2: For the evaluation of the AI/ML based CSI compression sub use cases, both of the following types of AI/ML model input are considered for evaluations

- Raw channel matrix

- Eigenvector(s), including further pre-processing of the eigenvector(s), e.g., angular-delay domain converting

| Support/Can accept | NTT DOCOMO, NVIDIA, Ericsson, Futurewei, CAICT, New H3C, MediaTek, Pansonic OPPO, ETRI, CATT, LG, CMCC, Lenovo, Qualcomm, Fujitsu, ZTE, Samsung |
| --- | --- |
| Object/Concern | vivo |

| Company | View |
| --- | --- |
| Ericsson | It needs to be clarified whether RAW channel matrix indeed is the TX x RX matrix per RB (i.e. the channel matrix obtained if SRS based reciprocity is used in TDD) |
| Futurewei | For Rel-18, we believe both input types can be included in the study. |
| Apple | Same angular-delay domain converting can be to channel matrix as well. |
| MediaTek | “Including further pre-processing of the raw channel matrix, e.g., angular-delay domain converting” should be added to raw channel matrix too. |
| OPPO | We are both okay, but slightly prefer use eigenvector(s) as the input, since it can be directly compared with Rel-16 eType II baseline. We are also not clear about what is the output when using raw channel matrix as the input, raw channel or eigenvector? |
| Xiaomi | We share similar view with Apple, suggest the following update. For the evaluation of the AI/ML based CSI compression sub use cases, both of the following types of AI/ML model input are considered for evaluations - Raw channel matrix, including further pre-processing of the channel matrix, e.g., angular-delay domain converting - Eigenvector(s), including further pre-processing of the eigenvector(s), e.g., angular-delay domain converting |
| vivo | We prefer the eigenvectors for the fair comparison with eType II codebook. The raw channel matrix can be low propriety or precluded for current study. |
| ZTE | We agree with this proposal, and we can use the wording in agenda 9.2.2.2: - Option 1: Raw Channel matrix - 1a: raw channel is in frequency domain - 1b: raw channel is in time domain - Option 2: Precoding matrix - 2a: The precoding matrix is a group of eigenvectors - 2b: The precoding matrix is an eType II-like PMI. |
| Samsung | We have the same understanding as Ericsson for raw channel matrix. It might be helpful to clarify this |

### 3.2-3: Other EVM for CSI compression

- Issue#3-10 (High priority) Payload size alignment

As discussed in the last meeting, a majority of companies believe it is beneficial to align the CSI payload size. From the inputs for this meeting, 3 companies provided the options on how to align the CSI payload for legacy Type II and AI/ML based CSI compression. Therefore, the following question is raised, with the intention of down selecting one option. Option 1 may be simpler but may not accurately reflect the actual CSI overhead as it is overestimated to the CSI of the maximum rank; Option 2 is more accurate it may be hard to align a single CSI payload value over different schemes so interpolation may be needed.

In addition, Apple [24] raised that companies are also to report the maximum feedback overhead (in bits) B(c,r), the maximum number of non-zero coefficients N(c,r), and the average number of actual non-zero coefficients for per rank Mc,r, so that the Type II payload size can be accurately calculated as

r=14Bc,r-Nc,r-M(c,r)⋅7Rc,r

To Moderator’s initial feeling, the detailed parameters of Type II may not necessarily be reported as companies are obliged to guarantee the accuracy of the calculated Type II payload size. So this proposal is tentatively moved to FFS in the question in below.

Question 3.2.8: For the CSI payload size calculation for AI/ML-based CSI compression as well as the legacy TypeII codebook, what is your preference on the following two options?

- Option 1: Payload size is calculated based on the maximum rank.

- Option 2: Payload size is calculated as the weighted average of CSI payload per rank and the distribution of ranks.

- Option 3: Other

- FFS whether companies are to report maximum feedback overhead, maximum number of non-zero coefficients, and average number of non-zero coefficients for Type II

| Option 1 | Support/Can accept | CAICT,OPPO, vivo, CATT, LG |
| --- | --- | --- |
|  | Object/Concern |  |
| Option 2 | Support/Can accept | NTT DOCOMO, MediaTek, Xiaomi, CMCC, Qualcomm (comment below), Samsung |
|  | Object/Concern |  |
| Option 3 | Support/Can accept |  |
|  | Object/Concern |  |

| Company | View |
| --- | --- |
| Moderator | Companies are also welcome to share your views on the FFS part. |
| Intel | Option 1 as for MIMO CSI. |
| Apple | We would like to emphasize that calculating type II overhead using max size per configuration is not accurate, and pessimistic for traditional codebook. With higher configuration, UE report CSI less than max number allowed in many cases. Option 2 is weighted average of max based on rank distribution, but still do not count the case when less than max is used to report. The following formula can be used to calculate true CSI overhead in the SLS simulation. r=14Bc,r-Nc,r-M(c,r)⋅7Rc,r |
| OPPO | Option 1 is enough for performance calibration. |
| Xiaomi | Support option 2 in principle, the detailed calculation method need further study. |
| CATT | Option 1 can make the comparison with non-AI/ML approach simpler, since it is widely used in legacy MIMO. |
| LG | Option 1 is enough |
| CMCC | Prefer Option 2, at least for AI based compression. |
| Lenovo | If throughput-overhead tradeoff is used as the metric, Option 1 should be supported since overhead increases with throughput. On the other hand, if SGCS is considered as a metric, Option 2 should be supported |
| Qualcomm | We support Option 2 with a clarification: regarding the rank to be used, in our view, the CSI feedback overhead should be computed based on the rank reported by the UE. The rank scheduled by the gNB should not be used for this purpose. Regarding the FFS, the following was already agreed in the EVM table in RAN1#109-e: “Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics.” |
| ZTE | We have a question about payload size. For intermediate KPIs, some companies offer the overhead calculation based on the maximum rank. However, for the eventual simulation evaluation, the payload of UEs may not base on the maximum rank and the overhead calculation is different from the overhead for intermediate KPIs. Therefore, how to address the unmatched payload size between intermediate KPI and eventual KPI needs further discussion. |
| Samsung | It is possible to count the reported payload bits at each CSI report based on Rel-16 and report the average. That, of course, will be more accurate and resembles Option 2. We sympathize with companies which prefer Option 1 as that is more customary in MIMO studies given the uplink resources are assigned based on the largest expected payload size (in case of report with rank adaptation). |

- Issue#3-11 (Medium) Template for simulation results collection

Two companies discussed the template to describe the AI/ML model and collect simulation results [1][2]. A question is raised accordingly to collect views from companies on the template.

Question 3.2.9: To facilitate the description of the AI/ML model and collection of the evaluation results for the CSI compression use case, please provide your views on the following template which is taken a starting point

- FFS the generalization description and results

|  |  | Source 1 | … |
| --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
| CSI reconstruction part | AL/ML model backbone |  |  |
|  | Pre-processing |  |  |
|  | Post-processing |  |  |
|  | FLOPs/M |  |  |
|  | Parameters/M |  |  |
| Generic description | Input type of CSI generation part |  |  |
|  | Output type of CSI reconstruction part |  |  |
|  | Quantization /dequantization method |  |  |
| Dataset size | Train/k |  |  |
|  | Test/k |  |  |
| Gain for intermediate KPIs | SGCS |  |  |
|  | NMSE |  |  |
|  | [Others] |  |  |
| Gain for eventual KPI | Mean UPT |  |  |
|  | 5% UPT |  |  |
| CSI payload | Payload 1 |  |  |
|  | Payload 2 |  |  |
|  | Payload 3 |  |  |
| FFS others |  |  |  |

| Company | View |
| --- | --- |
| Futurewei | We appreciate FL’s suggestion on using a template to capture the evaluation results from companies which is aligned with our view as described in [1]. We think the attributes in the table should include those related to generalization, by adding: - Training scenario/configuration - Testing scenario/configuration - Generalization mechanism/approach applied |
| OPPO | Using a template to capture the evaluation results from companies is helpful in this SI. We have following suggestions based on the template: 1. The SGCS and eventual SLS UPT performance gain are highly related to various traffic models (including FTP model 1 or full buffer), RUs, CSI payloads and rank numbers, so it seems not convenient to depict the details using this template. 2. For generalization description, an independent template is better. 3. For scalability issue with different configurations changing the input/output dimensions, also use an independent template, and add the ‘scalability mechanism’ to describe how to deal with the scalability issue. |
| vivo | We support to clarify a normal template. For the table above, we think it is suitable to type 1 training. For type 2/3 training, the multi encoders and multi decoders, the specific quantization/dequantization of encoders and decoders needs to be considered too. |
| CATT | FL’s template is a nice starting point. When filling the table, companies may do necessary adjustment based on their design, e.g. when a family of AI/ML model is used for scalability, and even part of the model is shared among this family of models ([11]). |
| LG | We agree with oppo’s comment in the first bullet. And, what is the meaning of payload 1/2/3?? |
| Lenovo | We believe it is a bit early for this discussion. |

- Issue#3-12 (High priority) Format of ground-truth CSI samples

The report of the ground-truth CSI may be needed for some procedures such as training, monitoring, etc., and the dataset delivery including the ground-truth CSI may also be needed for training Type 2/3. For evaluating the overhead of ground-truth CSI labels and the potential performance degradation due to the quantization, some companies mentioned that the methods of format/quantization method for the ground-truth CSI need to be studied [2][6][12]; in addition, they also come up with specific methods for obtaining high resolution CSI ground-truth labels, including float32, and codebook based compression, such as Rel-16 Type II-like CB with new parameters than legacy.

Therefore, a question is raised in below to evaluate the overhead and potential performance degradation.

Question 3.2.10: For evaluating the overhead of data samples and performance impact in the CSI compression, do you think high resolution quantization methods for ground-truth CSI are to be studied, e.g., including at least the following options

- High resolution scalar quantization, e.g., Float32, Float16, etc.

- High resolution codebook quantization, e.g., R16 Type II-like method with new parameters

  - FFS new parameters

| Support/Can accept | MediaTek, OPPO, vivo, CATT, CMCC, Fujitsu, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| NTT DOCOMO | If joint training is performed at UE side and model delivery from UE to NW is performed, data collection at NW side is unnecessary. Then, We think how to collect the data at NW side should be discussed after the conclusion of training procedure. |
| OPPO | Agree to study this issue, both in Type 2/3 and model monitoring procedures with ground-truth label transmission. |
| vivo | Support to study. Some other methods may not be precluded. |
| CATT | Ground-truth label is not only critical for training, but also useful for model monitoring. We support to study. |
| CMCC | OK to study. We think if it is offline training, the delivery of ground-truth CSI may not have spec impact. But if it is for evaluating the overhead of data delivery, it can be studied. |
| Lenovo | We are Okay with this proposal but maybe postponed for now. |
| Qualcomm | The impact of quantization of data in datasets on the performance of the AI/ML models can be studied with lower priority. Since data collection is not a time-critical process, the collection does not require the use of air-interface. It can even be done in a proprietary manner opportunistically without much impact to network resources. Therefore, the overhead associated with the data collection and the need to study ways to quantize the data seem to be second-order concerns. |
| ZTE | We think this issue can be deprioritized since the evaluation for 3.2-1, 3.2-2 may bring much workload to be discussed and studied. |
| Samsung | First bullet shall be a baseline. |

### 3.2-4: Generalization to the CSI compression use case

- Issue#3-13 (Medium) Scalability evaluations- input dimensions

A number of companies evaluated the generalization under varying configurations, and provided some elaborations on how to achieve the scalability.

For scalability over different bandwidths, 1 company raised to consider converting to angle-delay domain so that the dimensions can be fixed to the selected beams/delays rather than bandwidths [5]. 3 companies evaluated zero padding [8][17][18], where zeros are padded at the end of the sample before going into the encoder.

For scalability over different antenna ports, 2 companies raised to adopt zero padding [3][8], while 1 company raised to adopt adaptation layer in the AI/ML model [11], where additional fully connected layers are used for adaptation.

In addition, Fujitsu [12] discussed the general principle to consider the baseline for evaluating scalability without pre-processing/post-processing, and as per the understanding of Moderator, this baseline may be classified as Case 1; while the pre-/post-processing as mentioned above can be considered as the scalability solutions, which is applicable to Case 2/3 as per the understanding of Moderator.

Therefore, a question is raised in below. As clarifications:

- The intention is to better align the methodology over companies on the generalization/scalability, so the scalability solutions Option 1/2/3 are not to be down selected at the moment (whether to make down selection in future depends on the feasibility justification with more inputs).

- Not clear whether Case 2 can still achieve moderate performance to a different dimension even when the preprocessing is performed, so Case 2 is marked as FFS, and companies are welcome to provide your views on whether there is need to evaluate Case 2.

Question 3.2.11: For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input dimensions of CSI generation part (e.g., different bandwidths/frequency granularities, or different antenna ports), what is your view to the following elaborations to Case 1/2/3?

- Case 1: The AI/ML model is trained based on training dataset from a fixed dimension X1 (e.g., a fixed bandwidth/frequency granularity, or number of antenna ports), and then the AI/ML model performs inference/test on a dataset from the same dimension X1.

- FFS Case 2: The AI/ML model is trained based on training dataset from a single dimension X1, and then the AI/ML model performs inference/test on a dataset from a different dimension X2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…, or Xn.

- Note: For Case [2]/3, the solutions to achieve the scalability between Xi and Xj, are reported by companies, including, e.g.,

  - Option 1: Pre-processing to angle-delay domain

  - Option 2: Zero padding

  - Option 3: Additional adaptation layer in AI/ML model

  - Note: the above options are for better understanding of the solutions rather than down-selection in the EVM at the moment

- FFS Case 2A

| Support/Can accept | Futurewei (with comments below), MediaTek, OPPO, vivo, CATT, CMCC, ZTE, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Futurewei | Depending on the assumptions and expectation of the generalization, companies may choose different approaches. Case 1 may be a default case and model switching may be needed in case different input dimensions of CSI generation part is needed. For Case 2 and Case 3, they depend on data availability assumption and implementation choice in the deployment phase. For the note, including examples is ok, but using “option” is not needed. We suggest just indicating the 3 as examples (i.e., via pre-processing to angle-delay domain, zero padding, additional adaptation layer(s) in the model) and removing the “Option” wording. |
| OPPO | Agree. At this moment, all cases should be evaluated. Case 1 is the upper baseline and Case 2 can be regarded as the lower baseline. Companies are encouraged to provide more details on how to perform option 1-3 or other solutions to deal with this scalability issue. |
| CATT | Support. And we see a ‘FFS Case 2A’ at the end. Does it mean the Case 2A (*fine-tuning) in the agreement for ‘generalization evaluation’? If so, we’d better make it clear, otherwise it seems suggesting an unknown sub-case under the ‘FFS Case 2’ in THIS proposal. |
| Lenovo | We believe we can have another case as below. This is very similar to case3 with a slight modification at the end. We agree with this option, - Case 4: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of X1, X2,..., Xn, and then the AI/ML model performs inference/test on a single dataset subject to the dimension of X1, or X2,…,Xn, X(n+1), X(n+2),… , X(n+k). This way we let the model learn from the set of dimensions and also we are testing its generalizability. |
| Qualcomm | This proposal may make things complicated. Such details can be left to companies to report. |
| Fujitsu | We think that only Case 3 needs to be evaluated. Our view is that the scalability of configuration more like an implementation issue, which is different from generalization of scenarios. The various configurations are basically pre-known for two-sided model that should have been optimized during its training stage with various configurations. The Case 2A is not clear to us. |
| Samsung | Ok. |

- Issue#3-14 (Medium) Scalability evaluations-output dimensions

Similar to the question to the scalability evaluation of input dimensions, the scalability evaluation to output dimensions of the CSI generation part are also brought up in below.

To achieve the scalability over different CSI payload sizes, 5 companies adopted payload truncation [5][8][12][17][18], where the output of the encoder is cut out from the beginning to the specific payload length; 2 companies adopted adaptation layer, where additional fully-connected layers are used for down-sampling (DS-x block) and up-sampling [11][12].

Similarly, a question is raised in below for the scalability of output dimensions.

Question 3.2.12: For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different output dimensions of CSI generation part (e.g., different CSI payloads), what is your view to the following elaborations to Case 1/2/3?

- Case 1: The AI/ML model is trained based on training dataset from a fixed output dimension Y1 (e.g., a fixed payload), and then the AI/ML model performs inference/test on a dataset from the same output dimension Y1.

- FFS Case 2: The AI/ML model is trained based on training dataset from a single output dimension Y1, and then the AI/ML model performs inference/test on a dataset from a different output dimension Y2.

- Case 3: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn.

- Note: For Case [2]/3, the solutions to achieve the scalability between Yi and Yj, are reported by companies, including, e.g.,

  - Option 1: Payload truncation

  - Option 2: Additional adaptation layer in AI/ML model

  - Note: the above options are for better understanding of the solutions rather than down-selection in the EVM at the moment

- FFS Case 2A

| Support/Can accept | Futurewei (with comments below), MediaTek, OPPO, vivo, CATT, CMCC, Samsung |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Futurewei | Like our comments for 3.2.11, depending on the performance, companies may choose different approaches. Case 1 may be a default case and model switching may be needed in case different payload size/dimension is needed. For Case 2 and Case 3, they depend on data availability and implementation choice in the deployment phase. For the note, including examples is ok, but using “option” is not needed. There may be other options that are not discussed in companies’ contribution yet, e.g., by using some assistance information which can be part of the input from the beginning or added later which is different from both options mentioned. We suggest just indicating the 2 as examples (i.e., via payload truncation, additional adaptation layer(s) in the model) and removing the “Option” wording in both. |
| OPPO | Agree. At this moment, all cases should be evaluated. Case 1 is the upper baseline and Case 2 can be regarded as the lower baseline. Companies are encouraged to provide more details on how to perform option 1,2 or other solutions to deal with this scalability issue. |
| CATT | Same minor question as the previous one for ‘FFS Case 2A’ in the last bullet. |
| Lenovo | We believe we can have another case as below. This is very similar to case3 with a slight modification at the end. We agree with this option, - Case 4: The AI/ML model is trained based on training dataset by mixing datasets subject to multiple dimensions of Y1, Y2,..., Yn, and then the AI/ML model performs inference/test on a single dataset of Y1, or Y2,…, or Yn, Y(n+1), Y(n+2),… , Y(n+k). This way we let the model learn from the set of dimensions and also we are testing its generalizability. |
| Qualcomm | This proposal may make things complicated. We do not see the need for such an agreement. Such details can be left to companies to report. |
| Samsung | Ok. |

- Issue#3-15 (Medium) Scalability evaluations-number of CSI generation parts / CSI reconstruction parts

For the companies who evaluated the scalability performance, 3 companies evaluated one encoder with adaptive payload sizes to multiple decoders each of which is subject to a fixed size [5][12][18], with Figure 1 as an example, while some others considered a unified pair of one encoder with one decoder, both with payload size adaptation, with Figure 2 as an example.

Figure 1

Figure 2

A question is then raised to collect views from companies to determine the evaluation cases, i.e., which alternative(s) do you prefer as the baseline case(s) for evaluation.

Question 3.2.13: For evaluating the generalization/scalability over various configurations for CSI compression, to achieve the scalability over different input/output dimensions, which of the following alternative(s) should be considered as the evaluation cases?

- Alt.1: One CSI generation part with scalable input dimensions to M separate CSI reconstruction parts each with fixed output dimensions

- Alt.2: M separate CSI generation parts with fixed output dimensions to one CSI reconstruction part each with scalable input dimensions

- Alt.3: A unified pair of CSI generation part with scalable input dimensions and CSI reconstruction part with scalable output dimensions

| Support/Can accept | OPPO |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | Prefer Alt.3 as the baseline, since it provides best scalability with a unified CSI generation part and reconstruction part. |
| vivo | We prefer Alt 1 and Alt 3 in consideration of the complexity of UE side, and we think the other Alts can be FFS. |
| CATT | In our view, the baseline is M separate CSI generation parts and M corresponding separate CSI reconstruction parts each with fixed input/output dimensions (quite un-scalable one). We prefer Alt.3 here, but we are open for other alternatives. |
| Lenovo | All options can be studied for now |
| Qualcomm | There is no need to agree on these alternatives. It can be left to companies to report. |
| Fujitsu | We share a similar view to that of CATT. The purpose of evaluation is to compare the performance and complexity of Alt 3 with the baseline (proposed by CATT). |
| ZTE | We think it is up to companies to report. |

### 3.2-5: Others

Question 3.2.14: For the evaluation of the AI/ML based CSI compression sub use cases, what other aspects related with evaluations do you think is necessary to be discussed?

| Company | View |
| --- | --- |

Specific evaluation methodology for CSI prediction sub use case

Summary of views from companies

### 4.1-1: Sub use cases evaluated/supported by companies

CSI prediction on time domain: Huawei, Hisilicon, vivo, Samsung, Nokia, ZTE, MediaTek, Apple, OPPO, Intel, Google, NVIDIA, Sharp, AT&T, [China Telecom]

- vivo: To ensure the enhancement of CSI at both low and high-speed scenarios, study AI/ML for time domain CSI prediction with high priority

- AT&T: Finalize the EVM for the CSI prediction sub-use case for CSI feedback enhancements

- China Telecom: If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE

- Google: Study the CSI prediction based on the following options:

  - Option 1: CSI prediction is deployed in NW side

  - Option 2: CSI prediction is deployed in UE side

- Google: For NW-based CSI prediction, study the input based on the channel eigenvectors or channel, where the channel eigenvectors should be based on the CSI reported by Rel-16 codebook.

- NVIDIA: AI/ML based CSI prediction should be selected as a sub-use case for evaluation

- NVIDIA: The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. Both should be evaluated to understand the potential gains of performing CSI prediction at gNB side vs. UE side

- Sharp: Consider RNN series machine learning model (RNN, LSTM,GRU.etc) for historical CSI prediction

- Sharp: In order to shorten the developing period. At least until machine learning approach for NR air interface starting as work item, method regarding model implementation are decide by companies

- Sharp: Parameter settings are decide by the companies. We encourage the companies to report multiple combinations for better observation

- Sharp: In order to clarify and explain the result. Companies are encourage to replicate the procedure and observe the results

- Sharp: In order to clarify and explain the result. XAI techniques should also being introduced into agenda 9.2.

- Findings on CSI prediction involving temporal domain

  - Huawei, Hisilicon: AI/ML-based CSI prediction outperforms the baseline without CSI prediction in terms of GCS

  - vivo: Without CSI prediction, using AI/ML based CSI compression, there exist significant spectral efficiency loss at least for moderate and high-speed scenarios

  - vivo: The AI-based CSI prediction can make up the spectral efficiency loss caused by channel aging

  - vivo: The AI-based CSI prediction outperforms the non-AI based one

  - vivo: The advantages of AI prediction over AR-based non-AI prediction:

  - Higher accuracy;

  - Less CSI-RS and feedback overhead;

  - Fewer historical CSIs, i.e., shorter measurement window;

  - Flexibility of predicting time;

  - Nokia: Performance results for CSI prediction using the agreed evaluation methodology conditions continue to indicate promising performance for this use case

  - Nokia: CSI prediction using AI/ML provides substantial performance gain at 5 and 10 ms prediction times compared with zero-order hold

  - ZTE: AI-based CSI prediction completely outperforms the non-AI method

  - ZTE: The prediction accuracy of AI-based approach drops seriously when the predicted time becomes longer due to the channel aging. However, AI-based CSI prediction can still maintain its performance for a long time

  - MediaTek: Depending on the requirements on CSI prediction, for example the required prediction length, AI/ML-based solutions provide superior performance compared to classical non-AI based methods

  - Apple: LSTM based AI model achieves more than 10dB gain for CSI prediction use case

  - Intel: There is a significant performance difference for non-AI/ML-based prediction with different parameters N (number of measurements used to derive predictor) and P (prediction order)

  - NVIDIA: Evaluation results show that AI/ML based CSI prediction significantly outperforms the baseline case without prediction (sample-and-hold)

CSI prediction on frequency domain: Samsung,

Solution description (from Samsung): A gNB can configure a UE to send it CSI reports for an inactive bandwidth part (BWP). The UE can use received DL CSI-RS on an active BWP and then perform AI-based CSI extrapolation to infer CSI on the inactive BWP.  The gNB can then decide whether to configure the UE to switch to the inactive BWP, depending on the CSI reports for the active and inactive BWPs.

- Findings on CSI prediction on frequency domain

  - Samsung: AI-based CSI frequency extrapolation can be enhanced by utilizing additional CSI-RS observations in the band to be extrapolated, which amounts to AI-based CSI frequency extra(inter)-polation..

### 4.1-2: AI/ML model settings

AI/ML model adopted in evaluations

Huawei, Hisilicon: FCN

vivo: 2D-FCN

Nokia: RNN / LSTM

Samsung: 3D-CNN+ResNet (time domain), TestNet (frequency domain), Bi-LSTM (joint CSI prediction & CSI compression)

MediaTek: 3D-CNN/3D-CNN-Res

- MediaTek: For AI/ML-based CSI prediction, a CNN-based AI/ML model can be applied for training

- MediaTek: For AI/ML-based CSI prediction, residual neural networks architecture can be applied to enhance the accuracy of prediction

Apple: LSTM

NVIDIA: CNN

Input/output CSI type

Three candidate options are raised by companies for analysis.

Option 1: Raw channel matrix vivo, Samsung, Nokia, ZTE, MediaTek, Apple, NVIDIA

- vivo: the raw historic channel in PRBs is considered as AI-input, and the raw channel in PRBs in scheduling delay is predicted as AI-output

- Samsung: In particular, the objective is to predict the next 3-D CSI sample H, which has dimensions of K x Nt x Nr.

- Nokia: Estimated downlink channel H, k time instance

- ZTE: For AI-based CSI prediction, the raw channel matrix on PRBs as model input/output can be considered for intermediate evaluations at least for calibration purpose

- MediaTek: each CSI instance is a complex-valued matrix with dimensions NRX×NTX×NF, where NRX and NTX are the numbers of RX and TX antennas, respectively, and NF is the number of elements in the frequency dimension, which could be on subcarrier or PRB level

Option 2: Eigenvector Huawei, Hisilicon, OPPO

- Huawei, Hisilicon: the input of the CSI predictor includes k historic eigenvectors which are obtained from the k historic CSI-RS, respectively

- OPPO: Suggest use eigenvector as the input and output of AI/ML model for CSI prediction sub use case.

Option 3: CSI feedback information

Pre/post-processing

- MediaTek: When the UE receives the CSI-RS signal, it will perform the post-processing through descrambling, channel estimation, etc. Then, the UE can obtain the channel information, which can be CIR (channel impulse response) or CFR (channel frequency response) of CSI-RS

Observation window

- EVM needed/Companies to report the observation window (the number of historical CSI inputs): Huawei, Hisilicon, vivo, Nokia, MediaTek, Apple, Intel, NVIDIA

  - Huawei, Hisilicon: The observation window is the latest k=4 observation instances with 5 slots distance to each other

  - vivo: The CSI prediction (both the AI-based and non-AI method) is with 15 historical CSIs as the input

  - Nokia: The LSTM is trained to do a prediction of one time-step ahead after watching the last three time-steps

    - Time domain: We have a channel sample time of 0.5 ms and use either 20 or 30 time-domain input samples.

    - Frequency domain: Either 16 (1.44 MHz) or 100 (9 MHz) input frequency domain CSI reference signals

  - Nokia: Support high number of frequency-domain and of time-domain channel samples for training and inference of NNs for channel prediction

  - ZTE: We simulate 4 historical CSIs and 10 historical CSIs as the input of AI model to predict 3 CSIs in future occasions, respectively

  - ZTE: The number of observation window and prediction window need to be discussed at least for calibration purpose

  - MediaTek: In simulation, observation window includes [1, 2, 3,…,N] observation instances, where time distance between observation instances is 1ms/4ms/5ms

  - MediaTek: Use 15ms as a baseline observation window length and other candidates are not precluded

  - MediaTek: The tradeoff between the observation length and prediction length should be further studied

  - Apple: We used eight time-domain samples to predict the next 1 or 2 channel response

  - Intel: The parameter N corresponds to number of consecutive slots used for measurements, P corresponds to autoregressive model order. N=20, 50, 200.

  - NVIDIA: The raw channel matrices of the four latest CSI-RS measurement instances are used as the AI/ML model input. The raw channel matrices are associated with the first PRB

- Company findings on the Observation window:

  - Nokia: Channel prediction performance improves significantly as the observation bandwidth increases and as the time step between measurements decreases.

  - Nokia: The complexity due to higher number of time domain and frequency domain channel samples increases only moderately

  - ZTE: The prediction accuracy of AI-based approach improves with the increasing number of historical CSIs as model input.

  - ZTE: The prediction accuracy of AI-based approach presents positive correlation with the number of historical CSIs as model input

  - MediaTek: AI/ML-based CSI prediction will have more benefits than non-AI based prediction for longer time distances of OW and PW

  - MediaTek: If we want to predict the CSI at the same time, observe more intensive CSI-RS can obtain better prediction results

Prediction window

- EVM needed/Companies to report the prediction window: Huawei, Hisilicon, vivo, Nokia, ZTE, MediaTek, Apple, Intel, NVIDIA

  - Huawei, Hisilicon: The prediction window is 1 future slot

  - vivo: the scheduling delay is 4ms

  - Nokia: NMSE over the prediction time or, alternatively, cosine similarity in case of PMI prediction

  - ZTE: We simulate 4 historical CSIs and 10 historical CSIs as the input of AI model to predict 3 CSIs in future occasions, respectively

  - MediaTek: In simulation, prediction window includes 10ms, 15ms, and 20ms

  - Apple: 5ms or 10ms

  - Intel: The parameter N corresponds to number of consecutive slots used for measurements, P corresponds to autoregressive model order. P=3, 5, 7.

  - NVIDIA: The AI/ML model output is the predicted raw channel matrix at 4 ms ahead

Other views

- AT&T: For the AI/ML based CSI prediction sub use cases, companies are encouraged to report the details of their models for evaluation, including:

  - Location of model (gNB or UE)

  - The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

  - The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

  - The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

  - Observation and prediction windows size

  - Data pre-processing/post-processing

  - Loss function

  - Others are not precluded

### 4.1-3: EVM for CSI prediction

Carrier frequency

- Samsung: 4GHz

- vivo: 3GHz, 4GHz

- MediaTek: 3GHz, 2GHz, 3.5GHz

UE distribution

- LG: If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, O2I car-penetration loss in TR 38.901 should be taken into account for evaluation.

  - O2I car-penetration loss in TR 38.901 should be taken into account for evaluation of CSI prediction. The exact model is 	 where  is the basic outdoor path loss,  μ = 9, and σP = 5.

KPI

- Intermediate KPI

  - SGCS: Huawei, Hisilicon, Samsung, Nokia, Intel

    - Intel: SGCS in this case corresponds to the average SGCS for prediction for 1,2,…,10 slots per PRB

  - NMSE: vivo, Samsung, MediaTek, Nokia, Apple, NVIDIA

- The overhead includes both reference signal and feedback overhead: Nokia

  - One of the desired effects of CSI prediction is to reduce not only the number of CSI feedback occasions, but also the number of CSI-RS transmissions required to meet a certain performance level: Nokia

- Other KPIs

  - Inference latency, e.g., Time (ms) between CSI-RS transmission to transmission of the predicted CSI Nokia

  - Processing complexity Nokia, AT&T

  - AT&T: For the evaluation of the AI/ML based CSI prediction sub use cases, the inference complexity in the ‘Evaluation Metric’ includes: Computational complexity of model inference: FLOPs; Computational complexity for pre- and post-processing; Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte)

  - Observation time, e.g., Minimum channel observation time needed, e.g., minimum number of CSI-RS measurements needed Nokia

Throughput

  - AT&T: For the evaluation of the AI/ML based CSI prediction sub use cases, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT

- Other views

  - Nokia: Adopt specific KPIs related to channel prediction such as throughput, overhead, channel prediction horizon, observation time, complexity, and execution latency

  - LG: If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, at least followings need to be further discussed

    - Spatial consistency

    - Reasonable baseline scheme

    - Measurement / prediction window

Benchmark for evaluation results comparison

- Option 1: Nearest historical CSI (sample-and-hold): Huawei, Hisilicon, vivo, Samsung, Nokia, ZTE, MediaTek, Apple, NVIDIA

  - Huawei, Hisilicon: For the evaluation of AI/ML-based CSI prediction sub use cases, the nearest historical CSI can be taken as a baseline for the benchmark for performance comparison, while other non-AI/ML-based CSI prediction benchmarks can be also reported by companies

  - vivo: For AI/ML for time domain CSI prediction, nearest historical CSI (sample-hold without prediction) and other non-AI CSI prediction method (e.g., auto-regression) can be used as the baseline.

  - Nokia: Adopt the Rel-16 eType II codebook without prediction as the baseline for performance comparisons of ML-based CSI prediction, considering both throughput and overhead, where the overhead includes both reference signal and feedback overhead.  Kalman filtering can also be considered as a performance baseline

  - ZTE: Non-AI baseline for CSI prediction: Nearest historical CSI (sample-and-hold)

- Option 2: Kalman filtering: Nokia, OPPO

  - Nokia: The Kalman filter uses an autoregressive model of order three and updates the state-space equations for the prediction.

  - OPPO: Suggest to use a public non-AI CSI prediction (e.g. Kalman filtering, MMSE filtering, etc.) as baseline for evaluation

- Option 3: auto regression (AR): vivo, MediaTek

  - vivo: For AI/ML for time domain CSI prediction, nearest historical CSI (sample-hold without prediction) and other non-AI CSI prediction method (e.g., auto-regression) can be used as the baseline.

  - MediaTek: The number of FLOPs to perform the AI/ML-based (CNN) prediction is less than the auto-regression-based prediction with order-10

- Option 4: linear combination Samsung, Google

  - Samsung: In particular, to predict the next CSI sample Hn+1, we computed a linear combination of the current and previous CSI samples Hn and Hn-1, respectively.  Thus, we obtained a prediction Hpred,n+1 = αHn+βHn-1.  For each UE speed, we used a numerical search to determine the weights α and β.

  - Google: The baseline for CSI prediction should be the CSI based time-domain interpolation for the channel or channel eigenvectors

- Option 5: Companies to report the non-AI prediction solution Apple, Intel

  - Apple: For CSI prediction using one sided model, companies to report the traditional method used for prediction

  - Intel: Baseline non-AI/ML based channel prediction method which is used for comparison with AI/ML based prediction method is disclosed by each company

Parallel study of R18 MIMO CSI prediction

- vivo: The study of AI/ML based CSI prediction is independent with the R18 MIMO since Rel-18 MIMO’s focus is codebook design for feedback while CSI prediction algorithm itself is out of the scope of Rel-18 MIMO.

- ZTE: Evaluation assumptions for AI-based CSI prediction can reuse agreed assumptions in Rel-18 MIMO for Type-II codebook refinement under high/medium velocities as much as possible

- MediaTek: AI/ML-based CSI prediction for transmit precoding enhancement should use the outcome of the CSI enhancement objectives in 3GPP WI as a classical benchmark solution for performance evaluation

- AT&T: Use same baseline for AI/ML CSI prediction as in R18 MIMO CSI enhancement

Modeling of the UE mobility/trajectory

- Option 1: No explicit trajectory modeling (Reflected by Doppler shift which can refer to Section 7.5 of TR 38.901). Huawei, Hisilicon, ZTE vivo

  - Huawei, Hisilicon: For the evaluation of CSI prediction in temporal domain, no explicit trajectory modeling and spatial consistency modeling is considered as a starting point, i.e., UE mobility reflected by Doppler shift

- Option 2: Linear trajectory (refer to mobility modeling for intra-cell mobility scenarios for Rel-17 Multi-beam enhancement)

- Option 3: Spatial consistency modeled in Section 7.6.3 of TR38.901. LG

  - LG: If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, at least followings need to be further discussed: Spatial consistency

CSI-RS periodicity

- vivo: The choice of CSI-RS periodicity (especially the baseline parameters) should depend on the speed and carrier frequency

  - The performance of AI-based CSI prediction is highly related with the CSI-RS periodicity. 5ms CSI-RS periodicity (chosen as baseline in #110) is too large for the case of 4GHz carrier frequency and 30km/h or higher speed

- MediaTek: The UE speed will affect the tradeoff between CSI prediction length and CSI-RS periodicity

  - For CSI prediction, the Doppler effect and the coherence time are critical factors for AI/ML model’s prediction accuracy

  - When the coherence time is less than the CSI-RS periodicity, the CSI prediction performance will degrade rapidly

- Samsung: CSI-RS periodicity of 5 ms

### 4.1-4: Generalization study

Generalization methodology

- Nokia: Consider specialized AI/ML models based on one or few generalized AI/ML models to achieve highest channel prediction performance with minimum number of AI/ML model versions

  - Nokia: With a specialized AI/ML model we refer to an AI/ML model which has been overfitted to the current radio channel evolution of the UE by using the channel estimates of the last 50 ms, 100 ms, or several hundreds of ms as a data set for fine tuning the AI/ML model.

Findings on fine-tuning of CSI prediction

  - Nokia: Additional fine tuning of the CSI prediction model can substantially improve the performance of the CSI prediction for certain UEs

- AT&T: For the evaluation of the AI/ML based CSI prediction sub use cases, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

  - The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

  - The configuration(s)/ scenario(s) for testing/inference

  - Other details are not precluded

Generalization over frequency PRBs

- vivo: Firstly, the AI model is trained using the data only collected from 1-st PRB. Then, the trained model is directly inferred on the 10-th, 20-th, 30-th, 40-th and 50-th PRB to evaluate the generalization performance

  - Findings: The generalization of AI-based CSI prediction with respect to PRBs is good. The generalization performance across frequency domain should be studied.

- MediaTek: Further study the trade-off between single RB and joint RBs

Findings

  - For CSI prediction, the AI/ML model trained upon a certain RB (or sub-band) can be generalized and performed inference on other RBs (or sub-bands).

  - For CSI prediction, the AI/ML model trained on a certain RB (or SB) can be generalized to other RBs (or SBs).

  - Compared with training at single RB, more complex models need to be considered when training at multiple RBs, otherwise the performance cannot be improved

  - The AI/ML model trained on joint RBs can be generalized and inferenced on other joint RBs

- NVIDIA: AI/ML training and inference are performed on the first PRB. Next, we use the AI/ML model trained on the first PRB to carry out inference/testing on different PRBs.

  - Findings: The AI/ML based CSI prediction model trained on a certain PRB can be generalized to perform inference on other PRBs

Generalization over UE speeds

- vivo: the trained model is directly inferred on the validation data with the UE speed of 10, 15, 20 and 30km/h to evaluate the generalization performance.

- vivo: The generalization performance across speeds should be studied

  - Findings: The generalization of AI-based CSI prediction with respect to PRBs is good

- Samsung: UE speed of {10, 20, 30} km/h

  - Case 1: Training dataset was equally split between data where the UE speed was set to 1) 10 km/h and 2) 20 km/h; testing dataset was set to 10 km/h. Case 2: Training dataset was equally split between data where the UE speed was set to 1) 20 km/h and 2) 30 km/h; testing dataset was set to 20 km/h. Case 3: Training dataset was equally split between data where the UE speed was set to 1) 20 km/h and 2) 30 km/h; testing dataset was set to 30 km/h.

  - Findings: AI-based CSI prediction can yield performance benefits over a range of UE speeds, compared to a conventional CSI predictor that computes a linear combination of the current and previous CSI samples

- MediaTek: UE speed is leveraged for evaluation with 10, 20, 30, 60, 120km/h

  - Case 1/2: Training at 30km/h, inference at 10/20/30/60/120km/h; Training at 120km/h, inference at 10/20/30/60/120km/h

  - Case 3: Training at mixed [10, 20, 30, 60, 120] km/h, inference at 10/20/30/60/120km/h

Findings

  - For CSI prediction, the AI/ML model trained on a certain speed may not be generalized to other speeds

  - Using mixed datasets over UE speed for AI/ML model training is helpful to improve the generalization

Generalization over carrier frequency

- MediaTek: Observe the performance changes under different carrier frequency values (including 2, 3 and 3.5GHz)

  - Case 1/2: Training at 3GHz, inference at 2GHz/3GHz/3.5GHz

  - Case 3: Training at mixed [2GHz, 3GHz, 3.5GHz], inference at 2GHz/3GHz/3.5GHz

Findings

  - For CSI prediction, the AI/ML model trained on a certain carrier frequency may not be generalized on other carrier frequencies

  - Using mixed datasets over Doppler frequency for AI/ML model training is helpful to improve the generalization

Generalization over deployment scenarios

- MediaTek: Observe the performance changes under different carrier frequency values (including 2, 3 and 3.5GHz)

  - Case 1: Training at UMa, inference at UMa; Training at UMi, inference at UMi

  - Case 2: Training at UMa, inference at UMi; Training at UMi, inference at UMa

  - Case 3: Training at mixed [UMa, UMi], inference at UMa / UMi

Findings

  - For CSI prediction, the AI/ML model trained on a certain deployment (e.g., UMa/UMi) can be generalized and performed inference on other deployment (e.g., UMa/UMi)

1st round email discussions

### 4.2-1: EVMs for CSI prediction

For the CSI prediction, some EVMs are raised on top of the currently agreed generic EVM table.

- Issue#4-1 (High priority) UE distribution

One FFS issue of the conclusion of the last meeting is whether O2I car penetration loss is assumed.

Conclusion If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation, - 100% outdoor UE is assumed for UE distribution. - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

One company discussed this FFS issue, with the view that the car penetration loss needs to be modeled. As a background information, R18 MIMO also modeled car penetration in 38.901 if the UE is inside the vehicle. Therefore, the following proposal is provided.

Proposed conclusion 4.2.1: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, for the outdoor UEs, add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles.

| Support/Can accept | MediaTek, Samsung, OPPO, CATT, Lenovo, LG, ZTE, NVIDIA, Ericsson, CAICT, New H3C, AT&T, CMCC, Qualcomm, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| vivo | Whether adding O2I or not does not change the evaluation consequence due to the large scale factor which does not impact the channel characteristic. For the sake of simplicity, we slightly don't prefer to add O2I. |
| Intel | Support the proposal |
| MediaTek | Agree with vivo. |

- Issue#4-2 (High priority) Modeling of UE mobility/trajectory

One issue that has been discussed in the last meeting is whether to model the UE trajectory and spatial consistency. It seems a majority view that UE trajectory is not needed for modeling.

For spatial consistency, as a background, R18 MIMO has spatial consistency modeling (Spatial consistency procedure A/B with 50m decorrelation distance from TS 38.901 is used), but some view from companies is that for the CSI case, if the UE trajectory is not modeled, spatial consistency may not be as essential as in PoS use cases. In this regard, a FFS is added to collect more views of companies.

Proposed conclusion 4.2.2: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, no explicit trajectory modeling is considered for evaluation, i.e., UE mobility is reflected by Doppler shift.

- FFS: Spatial consistency procedure A/B with 50m decorrelation distance from 38.901

| Support/Can accept | MediaTek, Samsung, OPPO, CATT, vivo, LG, ZTE, Ericsson, CAICT, New H3C,AT&T, CMCC, Lenovo |
| --- | --- |
| Object/Concern | Qualcomm (comment below) |

| Company | View |
| --- | --- |
| Moderator | Companies are also welcome to share your views on the FFS part. |
| OPPO | We agree to no explicit trajectory modeling. We also think that the spatial consistency procedure is not required. |
| CATT | In beam management use case, spatial consistency and UE trajectory is proper since change of Top-K beam(s) is usually geographical. But unlike beam management, CSI prediction concerns more on the varying of channel itself. Modeling Doppler shift should be enough. |
| Intel | Support the proposal |
| MediaTek | Doppler shift is the key indicator for CSI prediction. Therefore, we agree that explicit trajectory modeling is not required. |
| CMCC | We think Doppler shift is enough. Modeling trajectory and spatial consistency is not needed. |
| Qualcomm | We support adopting spatial consistency procedure. If adopted, then the parameters associated with the clusters would evolve over time. Hence, we suggest to remove the following part in the wording: “, i.e., UE mobility is reflected by Doppler shift.” |

- Issue#4-3 (High priority) Benchmark CSI prediction scheme for performance comparison

As R18 MIMO has not discussed the baseline extrapolation algorithm for CSI prediction (linear, AR, filtering, etc.), and how to perform the prediction is up to implementations, there seems to be no need to make agree-upon benchmark non-AI/ML algorithm for R18 AI/ML evaluations. From the inputs of companies, 8 companies prefer to use the sample-and-hold CSI, while 2 companies suggested to report the non-AI/ML benchmark up to companies. Therefore, the following proposed conclusion is provided.

Proposed conclusion 4.2.3: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, the nearest historical CSI can be taken as a baseline for the benchmark of performance comparison, and other non-AI/ML based CSI prediction benchmarks for performance comparison are reported by companies.

| Support/Can accept | MediaTek, Samsung, OPPO, CATT, vivo, ZTE, NVIDIA, Ericsson, CAICT, New H3C, AT&T, CMCC |
| --- | --- |
| Object/Concern | Lenovo, LG, InterDigital, Qualcomm |

| Company | View |
| --- | --- |
| OPPO | We agree. It is better to provide other non-AI/ML based CSI prediction benchmarks besides the nearest historical CSI baseline by companies. |
| CATT | Empirically, using nearest historical CSI will lead to quite bad performance, which may exaggerate the benefit of AI/ML-based approach. But if no fast consensus can be reach on better non-AI/ML approach, this seems to be the only choice as baseline. Other benchmarks are up to companies. |
| Lenovo | Comparison only with only sample-and-hold might show over-estimate for the potential AI/ML. We suggest having some non-AI prediction methods as well. |
| LG | Agree with Lenovo |
| Intel | Agree with Lenovo |
| Ericsson | Support. This is how a baseline is defined in Rel.18 MIMO WI. In practice, before the Rel.19 WI is agreed, RAN can compare the X% gain over Rel.16 baseline of non-AI based prediction (Rel.18 MIMO WI) with Y% gain of AI based CSI prediction ober Rel.16 baseline (Rel.18 AI SI). |
| InterDigital | We have a strong concern to make this simple option as a baseline and lower the bar to show the gain of CSI prediction. Rel-18 MIMO CSI prediction should be the baseline or whatever functionality is added in Rel-18 for CSI prediction can be used with implementation-based AI/ML could be also considered as the baseline. |
| AT&T | We agree with Ericsson. |
| MediaTek | Use sample-and-hold as baseline method to make performance alignment between each company easier. However, it’s welcome and recommend companies provide other non-AI based results. |
| CMCC | We are OK with using the nearest historical CSI as the baseline if we cannot reach fast consensus on better non-AI/ML approach. And the ground-truth CSI in the predicted slot can also be adopted as the upper bound performance of CSI prediction. |
| Qualcomm | Agree with Lenovo |

- Issue#4-4 (High priority) Intermediate KPI calculation

The intermediate KPIs include SGCS and NMSE as already agreed. One company raised that if the number of prediction instances is larger than 1, then it may consider the average SGCS over the predicted instances [22]. This may be a similar logic with the average over frequency units as discussed in Issue#2-4. Therefore, a proposal is made in below.

Proposed conclusion 4.2.4: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, and if the AI/ML model outputs multiple predicted instances, the SGCS/NMSE is calculated as the average value over the predicted instances.

| Support/Can accept | Samsung, MediaTek, OPPO |
| --- | --- |
| Object/Concern | vivo, |

| Company | View |
| --- | --- |
| Moderator | Please also provide your view on whether the AI/ML model with multiple predicted instances output would be considered for evaluation. |
| Ericsson | Too early to discuss |
| MediaTek | Agree. Same as CSI compression, we can use SGCS/NMSE as intermediate KPIs. |
| vivo | We prefer dedicated SGCS/NMSE for each predicted instance. This is because the averaged NMSE/SGCS may be dominated by the nearest or the farthest predicted instance. |
| CATT | Although we support SGCS/NMSE as intermediate KPI, we tend to agree with vivo. Such simple averaging in time domain may lead to information loss. |
| CMCC | Tend to agree with vivo. |

- Issue#4-5 (Medium) Overhead reduction due to CSI prediction

The CSI feedback overhead has been agreed as one KPI in the EVM table. One company raised to also consider reference overhead for the CSI prediction case, with the point that the CSI-RS resources can be reduced compared to non-AI/ML schemes to achieve the same system performance. To Moderator’s understanding though, the ratio of CSI-RS overhead is minor in the frame, so reducing the CSI-RS overhead may not benefit lot to the overall DL system performance. A question is then provided to collect views from companies.

Question 4.2.1: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, do you think the overhead reduction of the reference signals (e.g., CSI-RS) should be taken as a KPI?

| Support/Can accept |  |
| --- | --- |
| Object/Concern | vivo |

| Company | View |
| --- | --- |
| Ericsson | Too early to discuss |
| Lenovo | We believe it is better to only focus on feedback reduction for now |
| Qualcomm | Before such an agreement, the definition of the overhead reduction should be discussed. |
| Samsung | Agree with Ericsson |

### 4.2-2: AI/ML model settings

- Issue#4-6 (High priority) Type of input for AI/ML model

From the inputs of companies, 7 companies preferred raw channel matrix as the input of the AI/ML model, and 2 companies preferred eigenvectors. Similar to the input type for CSI compression case, it is not intended to make down-selection but rather to support both of the input types. The proposal is made in the following accordingly.

Proposed conclusion 4.2.5: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, both of the following types of AI/ML model input are considered for evaluations:

- Raw channel matrix

- Eigenvector(s)

| Support/Can accept | Samsung, NVIDIA, AT&T, MediaTek, OPPO, vivo, CATT, CMCC, Lenovo, Qualcomm, ZTE |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| OPPO | Both okay, but using raw channel matrix as the input/output may require very larger training dataset. |
| ZTE | Agree with OPPO. |

- Issue#4-7 (High priority) Observation window/prediction window

As discussed in the last meeting, the observation window and the prediction window are encouraged to be reported by companies for better understanding of AI/ML. From the inputs of this meeting,

Upd Proposed conclusion 4.2.6: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, companies are encouraged to report the assumptions on the:

- Companies are encouraged to report the assumptions on the Observation window, including number/time distance of historic CSI measurements as the input of the AI/ML model, and

- Companies to report the assumptions on the Prediction window, including number/time distance of predicted CSI as the output of the AI/ML model

| Support/Can accept | MediaTek, Samsung, OPPO, CATT, vivo, Lenovo (comment), LG, ZTE, NVIDIA, CAICT, New H3C, AT&T, CMCC, Qualcomm (comment below) |
| --- | --- |
| Object/Concern |  |

| Company | View |
| --- | --- |
| Fujitsu | We think it is too early to discuss this issue at this stage. |
| Lenovo | We agree that companies are encouraged to report assumption on “Observation window” However, for each scheme the “Prediction window” should be reported otherwise there will be no way to compare the results. |
| Moderator | @Fujitsu this issue has been started for discussion since 109-e, so I would not feel it is a new issue. @Lenovo Changes are made in the updated version |
| Lenovo | Looks good to us. |
| Qualcomm | The wording should clarify that this is for evaluation purpose. Specifically, the observation window is left up to UE implementation. |

### 4.2-3: Others

Question 4.2.2: For the evaluation of the AI/ML based CSI prediction sub use cases, what other aspects related with EVM do you think is necessary to be discussed?

| Company | View |
| --- | --- |
| vivo | The choice of CSI-RS periodicity (especially the baseline parameters) should depend on the speed and carrier frequency. The performance of AI-based CSI prediction is highly related with the CSI-RS periodicity. 5ms CSI-RS periodicity (chosen as baseline in #110) is too large for the case of 4GHz carrier frequency and 30km/h or higher speed. |
| Qualcomm | The CSI feedback mechanism after prediction should be reported, e.g., what CSI codebooks or AI/ML based CSI feedback. |

Specific evaluation methodology for other sub use cases

1st round email discussions

Question 5.1.1: For the evaluation of the AI/ML based CSI feedback enhancement, is there any other sub use case that you think is necessary for EVM discussion and have not been discussed/captured in previous sections?

| Company | View |
| --- | --- |

Potential proposals for GTW

Proposals for 10 October GTW

- Issue#2-1 (High priority) Traffic model

As a controversial issue remaining from the 109-e/110 meetings, whether the full buffer can be optionally considered or taken as the baseline as the same as FTP model, has been discussed by companies for this meeting.

From the inputs of the 1st round discussions, a vast majority of companies support the following proposal, i.e., FTP model 1 is the baseline and used to draw SI conclusions, and full buffer is not precluded for calibration/comparing.

Proposal 2.2.1: For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Traffic model’ in the baseline of EVM is captured as follows:

| Traffic model | FTP model 1 with packet size 0.5 Mbytes as a baseline Other FTP model is not precluded. Full buffer model is not precluded at least for the purpose of calibration and/or comparing results, while the conclusions for the use cases in the SI should be drawn based on FTP model. |
| --- | --- |

| Support/Can accept | MediaTek, Fujitsu, OPPO, CATT, Xiaomi, vivo, Lenovo, NTT DOCOMO, LG, ZTE, CAICT, New H3C |
| --- | --- |
| Object/Concern |  |

- Issue#2-3 (High priority) Channel estimation-Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

As another FFS issue related to channel estimation and has been discussed in the last two meetings, whether the target CSI should consider ideal channel even under the realistic channel estimation (with the logic that this ideal CSI can be regarded as the upper bound) has been discussed in the contributions of this meeting.

A majority of companies are supporting Option 1, i.e., when the realistic channel is used, the intermediate KPI is calculated by ideal channel as the target CSI, with the logic that the ideal channel can be regarded as an upper bound. 1 company still prefer Option 2, i.e., aligning the calculation with both realistic channels.

Proposal 2.2.2: In the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, if realistic DL channel estimation is considered, regarding how to calculate the intermediate KPI of CSI accuracy, Option 1 of the following is preferred:

- Option1: Use the target CSI from ideal channel and use output CSI from the realistic channel estimation

- Option2: Use the target CSI from realistic channel estimation and use output CSI from the realistic channel estimation

- Option3: Companies to report the target CSI is from ideal channel or from the realistic channel estimation

- Option4: Other

| Support/Can accept | MediaTek, OPPO, CATT, ETRI, Xiaomi, vivo, Lenovo, NTT DOCOMO, ZTE, Intel, CAICT, New H3C |
| --- | --- |
| Object/Concern | Fujitsu |

- Issue#2-7 (High priority) Benchmark for evaluation results comparison

In the 109-e and 110 meeting, it has been agreed that companies to report Rel-16 or Rel-17 TypeII CB is used as baseline in the evaluation, while there is still a FFS issue on whether Type I CB should be also considered as a candidate benchmark.

From the inputs of the 1st round, a vast majority of companies support the following proposal.

Proposal 2.2.4: In the evaluation of the AI/ML based CSI feedback enhancement, for “Baseline for performance evaluation” in the EVM table, Type I Codebook can be optionally considered for comparing AI/ML schemes up to companies

- Note: Type II Codebook is baseline as agreed

| Support/Can accept | MediaTek, Fujitsu, OPPO, CATT, ETRI, vivo, Lenovo, NTT DOCOMO, LG, Intel, CAICT, New H3C |
| --- | --- |
| Object/Concern |  |

- Issue#4-1 (High priority) UE distribution

One FFS issue of the conclusion of the last meeting is whether O2I car penetration loss is assumed. As a background, R18 MIMO has adopted O2I car penetration loss modeling, so it may be simple to reuse that EVM.

From the inputs of the 1st round discussion, most companies support the following proposal.

Proposed conclusion 4.2.1: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, for the outdoor UEs, add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles.

| Support/Can accept | MediaTek, Samsung, OPPO, CATT, Lenovo, LG, ZTE, Intel, CAICT, New H3C |
| --- | --- |
| Object/Concern |  |

- Issue#4-2 (High priority) Modeling of UE mobility/trajectory

One issue that has been discussed in the last meeting is whether to model the UE trajectory and spatial consistency. It seems a majority view that UE trajectory is not needed for modeling.

For spatial consistency, as a background, R18 MIMO has spatial consistency modeling (Spatial consistency procedure A/B with 50m decorrelation distance from TS 38.901 is used), but some view from companies is that for the CSI case, if the UE trajectory is not modeled, spatial consistency may not be as essential as in PoS use cases.

Proposed conclusion 4.2.2: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, no explicit trajectory modeling is considered for evaluation, i.e., UE mobility is reflected by Doppler shift.

- FFS: Spatial consistency procedure A/B with 50m decorrelation distance from 38.901

| Support/Can accept | MediaTek, Samsung, OPPO, CATT, vivo, LG, ZTE, Intel, CAICT, New H3C |
| --- | --- |
| Object/Concern |  |

- Issue#4-7 (High priority) Observation window/prediction window

As discussed in the last meeting, the observation window and the prediction window are encouraged to be reported by companies for better understanding of AI/ML. From the inputs of the 1st round discussions, a number of companies can accept the following proposal, while 1 company thinks it is too early to discuss, and 1 company thinks the prediction window ‘should’ be reported. To clarify, the observation/prediction window issue has been discussed since 109-e meeting, and the proposal is not to determine the assumptions but let companies to report. The proposed conclusion is modified as follows.

Proposed conclusion 4.2.6: If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, companies are encouraged to report the assumptions on the:

- Companies are encouraged to report the assumptions on the Observation window, including number/time distance of historic CSI measurements as the input of the AI/ML model, and

- Companies to report the assumptions on the Prediction window, including number/time distance of predicted CSI as the output of the AI/ML model

| Support/Can accept | MediaTek, Samsung,OPPO, CATT, vivo, Lenovo (comment), LG, ZTE, Intel, CAICT, New H3C |
| --- | --- |
| Object/Concern |  |

- References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, December 6-17, 2021.

- R1-2208366	Continued discussion on evaluation of AI/ML for CSI feedback enhancement FUTUREWEI

- R1-2208429	Evaluation on AI/ML for CSI feedback enhancement  Huawei, HiSilicon

- R1-2208521	Evaluation on AI for CSI feedback enhancement	ZTE

- R1-2208547  Discussion on evaluation on AIML for CSI feedback enhancement  Spreadtrum Communications, BUPT

- R1-2208634	Evaluation on AI/ML for CSI feedback enhancement  vivo

- R1-2208729	Evaluations on AI-CSI	Ericsson

- R1-2208769	Evaluation on AI/ML for CSI feedback enhancement China Telecom

- R1-2208850	Evaluation methodology and preliminary results on AI/ML for CSI feedback enhancement  OPPO

- R1-2208878  On Evaluation of AI/ML based CSI  Google

- R1-2208899  Discussions on evaluation on AI/ML for CSI feedback enhancement	LG Electronics

- R1-2208967	Evaluation on AI/ML for CSI feedback enhancement  CATT

- R1-2209011  Evaluation on AI/ML for CSI feedback enhancement  Fujitsu

- R1-2209047  Evaluation for CSI feedback enhancements  Intel Corporation

- R1-2209120  Evaluation on AI/ML for CSI feedback  Lenovo

- R1-2209131  Discussion on evaluation methodology and KPI on AI/ML for CSI feedback enhancement  Panasonic

- R1-2209230  Some discussions on evaluation on AI-ML for CSI feedback	 CAICT

- R1-2209277	Discussion on evaluation on AI/ML for CSI feedback enhancement  xiaomi

- R1-2209328  Discussion on evaluation on AI/ML for CSI feedback enhancement  CMCC

- R1-2209367  Evaluation of ML for CSI feedback enhancement  Nokia, Nokia Shanghai Bell

- R1-2209386  GRU for Historical CSI Prediction  Sharp

- R1-2209400  Evaluation on AI/ML for CSI feedback enhancement  ETRI

- R1-2209506  Evaluation on AI/ML for CSI feedback enhancement  MediaTek Inc.

- R1-2209548  Evaluation of AI/ML based methods for CSI feedback enhancement  Fraunhofer IIS, Fraunhofer HHI

- R1-2209576  Evaluation on AI/ML for CSI feedback  Apple

- R1-2209625  Evaluation of AI and ML for CSI feedback enhancement  NVIDIA

- R1-2209640	Evaluation on AI/ML for CSI feedback enhancement  InterDigital, Inc.

- R1-2209652  Evaluation on AI/ML for CSI Feedback Enhancement  Mavenir

- R1-2209722	Evaluation on AI ML for CSI feedback enhancement	Samsung

- R1-2209794	Discussion on AI/ML for CSI feedback enhancement	AT&T

- R1-2209896  Discussion on evaluation on AI/ML for CSI feedback enhancement  NTT DOCOMO, INC.

- R1-2209976  Evaluation on AI/ML for CSI feedback enhancement  Qualcomm Incorporated

- Appendix I: Agreement list

### Agreements of the 109-e meeting

Agreement

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline

- Link level simulation is optionally adopted

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for the calibration purpose on the dataset and/or AI/ML model over companies, consider to align the parameters (e.g., for scenarios/channels) for generating the dataset in the simulation as a starting point.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, ideal DL channel estimation is optionally taken into the baseline of EVM for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.)

- Note: Eventual performance comparison with the benchmark release and drawing SI conclusions should be based on realistic DL channel estimation.

- FFS: the ideal channel estimation is applied for dataset construction, or performance evaluation/inference.

- FFS: How to model the realistic channel estimation

- FFS: Whether ideal channel is used as target CSI for intermediate results calculation with AI/ML output CSI from realistic channel estimation

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, companies can consider performing intermediate evaluation on AI/ML model performance to derive the intermediate KPI(s) (e.g., accuracy of AI/ML output CSI) for the purpose of AI/ML solution comparison.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, Floating point operations (FLOPs) is adopted as part of the ‘Evaluation Metric’, and reported by companies.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, AI/ML memory storage in terms of AI/ML model size and number of AI/ML parameters is adopted as part of the ‘Evaluation Metric’, and reported by companies who may select either or both.

- FFS: the format of the AI/ML parameters

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information.

- At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following table is taken as a baseline of EVM

- Note: the following table captures the common parts of the R16 CSI enhancement EVM table and the R17 CSI enhancement EVM table, while the different parts are FFS.

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, FFS 2GHz or 4GHz as a baseline |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz for 2GHz, 30kHz for 4GHz |
| Simulation bandwidth |  | FFS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | FFS |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption (i.e., whether the CSI-RS transmission is UE-specific or not and take that into account for overhead computation) |
| Traffic model |  | FFS |
| Traffic load (Resource utilization) |  | FFS |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) FFS whether/what other indoor/outdoor distribution and/or UE speeds for outdoor UEs needed |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic as a baseline FFS ideal channel estimation |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | FFS |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, as a starting point, take the intermediate KPIs of GCS/SGCS and/or NMSE as part of the ‘Evaluation Metric’ to evaluate the accuracy of the AI/ML output CSI

- For GCS/SGCS,

  - FFS: how to calculate GCS/SGCS for rank>1

  - FFS: whether GCS or SGCS is adopted

- FFS other metrics, e.g., equivalent MSE, received SNR, or numerical spectral efficiency gap.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if LLS is preferred, the following table is taken as a baseline of EVM

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

- FFS: other parameters and values if needed

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| Nt | 32: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Nr | 4: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel model | CDL-C as baseline, CDL-A as optional |
| UE speed | 3kmhr, 10km/h, 20km/h or 30km/h to be reported by companies |
| Delay spread | 30ns or 300ns |
| Channel estimation | Realistic channel estimation algorithms (e.g. LS or MMSE) as a baseline, FFS ideal channel estimation |
| Rank per UE | Rank 1-4. Companies are encouraged to report the Rank number, and whether/how rank adaptation is applied |

Agreement (modified by May 23rd post)

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The training dataset of configuration(s)/ scenario(s), including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- The detailed list of configuration(s) and/or scenario(s)

- Other details are not precluded

Note: Above agreement is updated as follows

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, study the verification of generalization. Companies are encouraged to report how they verify the generalization of the AI/ML model, including:

- The configuration(s)/ scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios

- The configuration(s)/ scenario(s) for testing/inference

- Other details are not precluded

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the details of their models, including:

- The structure of the AI/ML model, e.g., type (CNN, RNN, Transformer, Inception, …), the number of layers, branches, real valued or complex valued parameters, etc.

- The input CSI type, e.g., raw channel matrix estimated by UE, eigenvector(s) of the raw channel matrix estimated by UE, etc.

  - FFS: the input CSI is obtained from the channel with or without analog BF

- The output CSI type, e.g., channel matrix, eigenvector(s), etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the following parameters are taken into the baseline of EVM

- Note: The 2nd column applies if R16 TypeII codebook is selected as baseline, and the 3rd column applies if R17 TypeII codebook is selected as baseline.

  - Additional assumptions from R17 TypeII EVM Same consideration with respect to utilizing angle-delay reciprocity should be considered taken for the AI/ML based CSI feedback and the baseline scheme if R17 TypeII codebook is selected as baseline

  - FFS baseline for potential sub use cases involving CSI enhancement on time domain

- Note: the baseline EVM is used to compare the performance with the benchmark release, while the AI/ML related parameters (e.g., dataset construction, generalization verification, and AI/ML related metrics) can be of additional/different assumptions.

  - The conclusions for the use cases in the SI should be drawn based on generalization verification over potentially multiple scenarios/configurations.

- FFS: modifications on top of the following table for the purpose of AI/ML related evaluations.

| Parameter | Value (if R16 as baseline) | Value (if R17 as baseline) |
| --- | --- | --- |
| Frequency Range | FR1 only, 2GHz as baseline, optional for 4GHz. | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Simulation bandwidth | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz. | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. Above 15kHz is replaced with 30kHz SCS for 4GHz |
| MIMO scheme | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU | SU/MU-MIMO with rank adaptation. Companies are encouraged to report the SU/MU-MIMO with RU |
| Traffic load (Resource utilization) | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. | 20/50/70% Companies are encouraged to report the MU-MIMO utilization. |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if SLS is adopted, the ‘Baseline for performance evaluation’ in the baseline of EVM is captured as follows

| Baseline for performance evaluation | Companies need to report which option is used between - Rel-16 TypeII Codebook as the baseline for performance and overhead evaluation. - Rel-17 TypeII Codebook as the baseline for performance and overhead evaluation. - FFS: Whether Type I Codebook can be optionally considered at least for performance evaluation |
| --- | --- |

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’ for rank>1 cases, companies to report the GCS/SGCS calculation/extension methods, including:

- Method 1: Average over all layers

  - Note:  is the eigenvector of the target CSI at resource unit i and K is the rank. is the  output vector of the output CSI of resource unit i.  is the total number of resource units.  denotes the average operation over multiple samples.

- Method 2: Weighted average over all layers

  - Note: Companies to report the formula (e.g., whether normalization is applied for eigenvalues)

- Method 3: GCS/SGCS is separately calculated for each layer (e.g., for K layers, K GCS/SGCS values are derived respectively, and comparison is performed per layer)

- Other methods are not precluded

- FFS: Further down-selection among the above options or take one/a subset of the above methods as baseline(s).

### Agreements of the 110 meeting

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

Agreement

The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

  - Note: Companies to report the ratio for dataset mixing

  - Note: number of the multiple scenarios/configurations can be larger than two

- FFS the detailed set of scenarios/configurations

- FFS other cases for generalization verification, e.g.,

  - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, if the GCS/SGCS is adopted as the intermediate KPI as part of the ‘Evaluation Metric’, between GCS and SGCS, SGCS is adopted.

Agreement

For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point:

- Various deployment scenarios (e.g., UMa, UMi, InH)

- Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10)

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc.

- Companies to report the selected scenarios for generalization verification

Conclusion

If the AI/ML based CSI prediction sub use cases is to be selected as a sub use case, consider CSI prediction involving temporal domain as a starting point.

Agreement

For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point:

- Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband)

- Various sizes of CSI feedback payloads, FFS candidate payload number

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc.

- Companies to report the selected configurations for generalization verification

- Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

Conclusion

For the evaluation of the AI/ML based CSI feedback enhancement, for ‘Channel estimation’, it is up to companies to choose the error modeling method for realistic channel estimation and report by willingness.

- Note: It is not precluded that companies use ideal channel to calibrate

Agreement

For the evaluation of the AI/ML based CSI feedback enhancement, the throughput in the ‘Evaluation Metric’ includes average UPT, 5%ile UE throughput, and CDF of UPT.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, companies are encouraged to report the specific quantization/dequantization method, e.g., vector quantization, scalar quantization, etc.

Agreement

For the evaluation of the AI/ML based CSI compression sub use cases, the capability/complexity related KPIs, including FLOPs as well as AI/ML model size and/or number of AI/ML parameters, are to be reported separately for the CSI generation part and the CSI reconstruction part.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, a one-sided structure is considered as a starting point, where the AI/ML inference is performed at either gNB or UE.

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, for evaluation,

- 100% outdoor UE is assumed for UE distribution.

  - FFS: whether to add O2I car penetration loss per TS 38.901 if the simulation assumes UEs inside vehicles

- UE speed is assumed for evaluation with 10, 20, 30, 60, 120km/h

  - Note: Companies to report the set/subset of speeds

- 5ms CSI feedback periodicity is taken as baseline, while other CSI feedback periodicity values can be reported for the EVM

Conclusion

If the AI/ML based CSI prediction sub use case is to be selected as a sub use case, companies are encouraged to report the details of their models for evaluation, including:

- The structure of the AI/ML model, e.g., type (FCN, RNN, CNN,…), the number of layers, branches, format of parameters, etc.

- The input CSI type, e.g., raw channel matrix, eigenvector(s) of the raw channel matrix, feedback CSI information, etc.

- The output CSI type, e.g., channel matrix, eigenvector(s), feedback CSI information, etc.

- Data pre-processing/post-processing

- Loss function

- Others are not precluded