3GPP TSG RAN WG1 #110bis-e	  	R1-2210416

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.3.1

Source: 		Moderator (CMCC)

Title: 	Summary#3 on evaluation on NR duplex evolution

Document for:	Discussion/decision

# 1st set of stable proposals for email approval

#### Updated proposal 2-5-2a (Stable):

Remove square bracket for the traffic load and update the high traffic load from ~50% to ≥50% (i.e., low (<10%), medium (20%-40%) and high (≥50%)) in previous agreement made in RAN1#110.

#### Updated proposal 2-5-3a (Stable):

For SBFD deployment case 3-2, reuse the traffic model assumptions of SBFD deployment case 1 as much as possible.

- For comparison between baseline legacy TDD network (i.e., legacy TDD for both Layer 1 and Layer 2) and SBFD deployment case 3-2 (i.e., legacy TDD for Layer 1 and SBFD for Layer 2), the packet arrival rates are kept the same for each layer.

- The UL traffic load and DL traffic load can be independently selected for each layer.

#### Updated proposal 2-5-4a (Stable):

For SBFD deployment case 4, reuse the traffic model assumptions of SBFD deployment case 1 as much as possible.

- For comparison between baseline legacy TDD network (i.e., legacy TDD for both Operator#1 and Operator#2) and SBFD deployment case 4 (i.e., legacy TDD for Operator#1 and SBFD for Operator#2), the packet arrival rates are kept the same for each operator.

- The UL traffic load and DL traffic load can be independently selected for each operator.

#### Initial proposal 2-7-1 (Stable):

Confirm the working assumption for gNB-gNB channel model and gNB-UE channel model made in RAN1#110.

#### Initial proposal 2-7-2 (Stable):

Confirm the working assumption for UE-UE channel model made in RAN1#110.

#### Initial proposal 2-7-3 (Stable):

Adopt the following gNB-UE O2I building penetration loss model:

- Indoor office: penetration loss is not modelled.

- Percentage of high loss and low loss building type for Urban Macro / Dense Urban [refer to table 5B of ITU M.2412]:

  - 80% low-loss model

  - 20% high-loss model

  - Note: The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; otherwise the building type is high loss [refer to section 5.3.3 of ITU M.2412].

- FFS for 2-layer Scenario B

#### Updated proposal 2-7-4a (Stable):

Adopt the following table for gNB-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | gNB-UE channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 d2D-in - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Outdoor UE: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 - Both Car penetration (for outdoor UE) and O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901d2D-in - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. |
| Fast fading parameters | FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 |

#### Updated proposal 2-7-5a (Stable):

Adopt the following table for gNB-gNB channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | gNB-gNB channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). - For Indoor factory layer: InF in TR 38.901 (hUE =3m). - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901d2D-in - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |
| Fast fading parameters | FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - For Indoor factory layer: InF in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP |

#### Updated proposal 2-7-6a (Stable):

Adopt the following table for UE-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor).

|  | UE-UE channel model for 2-layer Scenario B |
| --- | --- |
| Large-scale channel parameters | FR1: - Outdoor UE to Outdoor UE: - Option 1: A.2.1.2 in TR36.843 (*) - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - For Indoor factory layer: InF in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-13 in TR38.802d2D-in |
| Fast fading parameters | FR1: - Outdoor UE to Outdoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - For Indoor factory layer: InF in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802. |  |

#### Initial proposal 2-8-2 (Stable):

For comparison between legacy TDD and SBFD, companies should report the assumption of BS transmit power on DL slots and SBFD slots in SBFD operation.

- For calibration purpose, assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission.

#### Initial proposal 2-8-3 (Stable):

For SBFD Deployment Case 4, different power levels in adjacent carriers can be simulated and it is up to company to report the power levels.