3GPP TSG RAN WG1 Meeting #110bis-e	                                       R1-22xxxxx

e-Meeting, October 10th – 19th, 2022

Agenda item:	7.2

Source:	Moderator (Sharp)

Title:	Summary of [110bis-e-NR-R16-08] Discussion on the definition of defer duration and conditions of new back-off

WI:	NR_unlic-Core

Document for:	Discussion and Decision

Introduction

This document is a summary of the following email discussion,

[110bis-e-NR-R16-08] Discussion on the definition of defer duration and conditions of new back-off by Oct 14 – TBD (Sharp)

Summary of inputs in relation to [110bis-e-NR-R16-08]

RAN1#110

The following was agreed in RAN1#110 meeting, see [1],

Agreement - TP#2, TP#3, TP#4 and TP#5 in R1-2007919 are endorsed in principle and provided to the specification editors for their next update of the Rel-16 and Rel-17 specifications.

As per decision for email discussion [Post-110-AlignmentCR-37.213], R1-2208306 [2] from the Editor of TS 37.213 (capturing TP#3 and TP#5 in R1-2007919 [3]) was agreed.

During email discussion [Post-110-AlignmentCR-37.213], some concerns were raised on TP#2 in R1-2007919 [3] (reproduced below for convenience).

- TP#2 for TS 37.213 <Unchanged parts are omitted> ### 4.1.1 Type 1 DL channel access procedures <Unchanged parts are omitted> The defer duration Td consists of duration Tf=16us immediately followed by mp consecutive sensing slot durations Tsl, and Tf includes a sensing slot duration Tsl at start of Tf. <Unchanged parts are omitted>.

RAN1#110bis-e

R1-2209800 [4] was submitted to RAN1#110bis-e, where the following observations and proposal in relation to TP#2 in R1-2007919 [3] were made,

Observation 1: The definition of “defer duration” has been implemented in accordance with RAN1 agreements in both TS 36.213 and TS 37.213. Observation 2: The intention of the sentence “if the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration Td” in clause 4.1.1 of TS 37.213 was as follows, - “the channel has to be continuously idle for at least initial defer duration immediately before the transmission”. Proposal: Capture Observation 2 in R1-2209800 as a conclusion in Chair’s note.

Discussion

Round 1

The case of no sensing slot being idle in a defer duration

As pointed out in R1-2209800 [4], there is a sentence in clause 4.1.1 of TS 37.213 (“if the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration Td”) that seems to conflict with the definition of defer duration, i.e.

- On one hand, by definition, a defer duration always starts with an idle sensing slot;

- On the other hand, if the condition “the channel has been sensed to be not idle during any of the sensing slot durations of a defer duration Td” is satisfied, then there is no idle sensing slot in the corresponding defer duration.

In R1-2209800 [4], it was proposed to conclude in RAN1 that the actual intention of the above mentioned sentence in clause 4.1.1 of TS 37.213 was that “the channel has to be continuously idle for at least initial defer duration immediately before the transmission”, see Observation 2 in R1-2209800 [4].

- Q1: Do you agree to capture Observation 2 in R1-2209800 as a conclusion in RAN1#110bis-e chair’s notes?

| Company | Answer (Yes/No) | Comment |
| --- | --- | --- |

Other issues (if any)

- Q2: If you think there is any other issue(s) that should be discussed as part of [110bis-e-NR-R16-08], please specify. Otherwise please skip this question.

| Company | Comment |
| --- | --- |

Summary

[TBD]

References

- RAN1#110 Chairman’s notes.

- R1-2208306, “Rel-16 editorial corrections for TS 37.213”, Ericsson, RAN1#110.

- R1-2207919, “Summary of comments on R1-2206935 miscellaneous corrections”, Moderator (Sharp), RAN1#110.

- R1-2209800, “On definition of defer duration and conditions of new back-off”, Sharp, Ericsson, RAN1#110bis-e.

- R1-2206935, “Miscellaneous corrections to TS 37.213”, Sharp, RAN1#110.